3GPP TSG-RAN WG4 Meeting # 113	                                       R4-2417891

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda Item: 7.24.2.2

Source: MediaTek Inc.

Title: Discussion on event triggered L1 report for LTM

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of CSI-RS L1 measurements.

# Discussion

## 2.1 known cell definition in CSI-RS based L1 measurement

Last meeting, RAN4 agreed that RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells, while there are still several FFS regarding the known cell definition as shown below. In the next, we would like to discuss the FFSs one by one.

#### Issue 2-1: CSI-RS based L1 measurement for known/unknown cell - Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells #### Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

The divergent part of the first bullet in known cell definition is what L3 measurement refers to, SSB based L3 measurement only or both SSB and CSI-RS based L3 measurement. In our understanding, the first bullet is to guarantee that UE has detected the cell and acquired the rough timing & frequency of the cell. From this point of view, no matter which RS UE has performed L3 measurement, SSB or CSI-RS, as long as UE performs L3 measurement recently, it is enough. In another way, we don’t think it makes any difference to limit to SSB based L3 measurement or not. To perform CSI-RS based L3 measurement, UE is supposed to perform L3 measurement on the associated SSB. So it is sufficient to limit to SSB based L3 measurement only.

Proposal 1: Open to limit to SSB based L3 measurement only in the first condition of known cell definition in CSI-RS based L1-RSRP measurement.

The second bullet is to make sure the associated SSB of the CSI-RS is detectable. As proposed in Proposal 6 in 2.3, CSI-RS for L1 measurement shall be Type-D QCL to SSB for L1 measurement in FR2-1.

Proposal 2: Keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

The third bullet is to guarantee the channel quality of CSI-RS is not too bad. Regarding the third bullet, we prefer to use a side condition. CSI-RS is not used for detection. It is not accurate to use the wording “Detectable”.

Proposal 3: The third condition of known cell definition in CSI-RS based L1-RSRP measurement is: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB.

## 2.2 Measurement procedure of CSI-RS based L1 measurement

In the past two meetings, RAN4 had extensive discussions on the measurement procedure of CSI-RS based L1 measurement and whether to define requirements for CSI-RS resources with repetition on or off. Last meeting, the following tentative agreement was reached. The key issue to discuss is whether step 2 can be skipped in FR2-1. In this part, we would like to provide our views on this issue.

#### Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Pending on whether step 2 can be skipped and whether CSI-RS resources are configured with repetition ON or OFF, there will be four cases as shown in Table 1.

Table 1

| Case | Pre-requisite SSB measurements before measuring CSI-RS | CSI-RS repetition | Note | Whether to sweep beams when measuring CSI-RS | Rx beam type for measuring CSI-RS and receiving target PDSCH | Whether to define requirement |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | SSB based L3 measurement only | OFF | Step 2 is skipped | No | Rough beam | No |
| 2 |  | ON |  | Yes | Fine beam | No |
| 3 | SSB based L1 measurement | OFF | Step 2 is kept | No | Fine beam | Yes |
| 4 | SSB based L1 measurement | ON | Step 2 is kept Corresponding to step 4 as the below procedures of serving cell. This case is not included in the three steps in the tentative agreement. | Yes | Fine beam | No |

Before discussing the four cases, we would like to recall CSI-RS based L1 measurement procedures of serving cells in FR2-1, i.e.,

  - Step 1: SSB based L3 measurements

  - Step 2: SSB-based L1 measurements (P-1 in Fig. 1)

    - Tx-Rx beam pair refinement based on SSB is performed.

    - After step 2, NW determines the best SSB and UE determines the best Rx beam.

  - Step 3: CSI-RS based L1 measurements on resources with repetition off (P-2 in Fig. 1)

    - The parameter repetition of CSI-RS resources is set to OFF. Beams of CSI-RS are supposed to be finer than that of SSB.

    - UE is supposed to use the same Rx beam as the associated SSB to measure the CSI-RS.

    - NW uses these CSI-RS resources with repetition OFF to refine Tx beam.

  - Step 4: CSI-RS based L1 measurements on resources with repetition on (P-3 in Fig. 1)

    - The further refined Tx beam in step 3 is used at NW side.

    - UE is supposed to refine the Rx beam further.

Fig. 1

The P-1 to P-3 procedures designed in R15 are to refine both Tx beam at gNB and Rx beam at UE with the least overhead. In our understanding, case 2 in Table 1, i.e., skip SSB based L1 measurement and CSI-RS are configured with repetition ON, is to use CSI-RS with repetition ON in P-1. This will lead to larger overhead. As the example shown in Table 2, the overhead will be 8 to 340 (128x8/3) times of the overhead using the legacy P-1/2 procedure. That is why SSB is used in P-1 but not CSI-RS in serving cell. Of course, considering there will be UEs under different SSB beam, the overhead to transmit CSI-RS with repetition ON for every SSB beam would be very high. We wonder whether it is worthy to do CSI-RS based L1 measurement on neighbor cell with so much overhead, considering the gain is marginal compared to SSB based L1 measurement.

Table 2

|  | P-1 | P-2 | P-3 |
| --- | --- | --- | --- |
| Using SSB in P-1 | No additional overhead. Reuse the SSB for L3 measurement | - 3 additional CSI-RS transmissions to complete a round of Tx beam further refinement for a single UE. - 128 additional CSI-RS transmissions are needed for the worst case | As far as we know, P-3 is not used in the real field. |
| Using CSI-RS in P-1 | 128 (Tx beam sweeping) x 8 (Rx beam sweeping) additional CSI-RS transmissions to complete a round of Tx-Rx pair refinement. | No need. | No need. |
| Note: Assume there are 64 SSB beams and 128 CSI-RS beams at NW side, and 8 Rx beams at UE side. Assume the number of CSI-RS beams associated to a single SSB is 3 as shown in Fig.1. |  |  |  |

If the intention of skipping step 2 is to go to P-2 directly without P-1 (case 1), only a rough Rx beam as L3 measurement will be used. We don’t think there is any gain compared to SSB based L1 measurement. In addition, the motivation to support L1-RSRP measurement on neighbor cell is to use fine beam directly after cell switch, Case 1 is not aligned with the goal. So we propose not to support case 1.

Proposal 4: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1.

Proposal 5: Considering that there is no gain compared to SSB based L1 measurement, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition OFF in FR2-1.

The next case we would like to discuss is Case 4 in Table 1, which is corresponding to P-3 in Fig.1. In this case, UE has already performed L1 measurement on the associated SSB (with fine beam already), it seems unnecessary to further use CSI-RS with repetition ON to re-fine the Rx beam again. Even if such an opportunity is provided, we doubt whether its extra Rx beamforming gain can justify the additional RS overhead for transmitting CSI-RS with repetition ON.

The last one is case 3, i.e., further Tx beam refinement after SSB based L1 measurement. In this case, UE has performed L1 measurement on the associated SSB, UE can use the same fine Rx beam as SSB to measure CSI-RS resources. Tx beam can be further refined. The same fine Rx beam can be used for PDSCH reception afterwards. Although we don’t think there is much gain even in case 3, we can define requirements for this case only since CSI-RS based L1 measurement is already in the scope.

Proposal 6: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, only consider the following procedure:

- Step 1: SSB based L3 measurements

- Step 2: SSB-based L1 measurements and reports on the same candidate cell

- Step 3: CSI-RS based L1 measurements on the same candidate cell

  - Note: The CSI-RS in Step 3 should be Type-D QCL with the SSB in Step 2 and configured with repetition OFF.

## 2.3 Whether to define requirements for RTD>CP case

Single FFT is assumed for multiple cell measurements for CSI-RS based L3 measurement. For SSB based L1 measurement, RTD>CP case is supported with UE capability. Last meeting, RAN4 agreed that “For CSI-RS based L1-RSRP measurement on neighbor cell, define the measurement period and accuracy requirements based on Density=3 with (≥) 48 PRBs”. If to support RTD>CP case for CSI-RS based L1 measurement, another FFT is needed to cover as wide as 48PRBs, while SSB only occupies 20PRBs. The cost will be much higher to support RTD>CP for CSI-RS compared to SSB. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement. We propose not to support RTD>CP case for CSI-RS based L1-RSRP measurement.

Proposal 7: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

## 2.4 Definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement

In the past two meetings, RAN4 discussed whether and how to define intra-frequency for CSI-RS L1 measurement. Regarding the definition of intra-frequency for CSI-RS based L1 measurement, now there are several options according to the WF.

- Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BWP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

We are not strongly against to define intra-frequency and inter-frequency for CSI-RS based L1 measurement, but think it may be not practical to follow the definition of CSI-RS based L3 measurement. We think Huawei had given an elaborate analysis on why it is not practical to follow the definition of CSI-RS based L3 measurement in R4-2415841 last meeting. The key problem of following L3 is which CSI-RS resource of the serving cell to be used as the reference to determine the L1 measurement intra-frequency or inter-frequency. For L3 measurement, CSI-RS resources are included in MeasObjectNR. In each MO, the CSI-RS resources should have the same CSI-RS center frequency, bandwidth and density. With the indication of “ServingcellMO”, it is easy to determine which MO is intra-frequency and which is inter-frequency.

However, different from L3 CSI-RS measurement, there is no concept of “ServingcellMO” for L1 measurement. In other words, there is no specific CSI-RS resource to be used as a reference to determine CSI-RS based intra-f and inter-f L1 measurement. A straightforward thinking is to introduce one. But different CSI-RS resources of the serving cell may have varied center frequency and BW, and the center frequency of CSI-RS resources configured in the serving cell may change with the active BWP. It is not so straightforward to introduce a reference CSI-RS resource for intra-frequency and inter-frequency definition.

Moreover, we don’t think it really matters to introduce a reference CSI-RS resource for the definition. Take SSB based L1-RSRP measurement as an example, RAN4 defined the requirements for intra-f without gap, inter-f without gap and inter-f with MG. As well known, the requirements for SSB within active BWP, i.e., inter-f without gap and intra-f without gap, are exactly the same. Similarly, for CSI-RS based L1-RSRP measurement, even we define intra-f and inter-f based on some reference CSI-RS in serving cell, the requirements of intra-f without gap and inter-f without gap will be the same. It may be better to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. If the majority in RAN4 prefer to call “CSI-RS based L1-RSRP measurement within active BWP” intra-frequency as in the above Option 5. We can live with it too.

Proposal 8: Prefer to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. We can compromise to call “CSI-RS based L1-RSRP measurement within active BWP” intra-frequency if it is the majority view.

## 2.5 Whether to define requirements for CSI-RS outside active BWP

To perform CSI-RS based L1-RSRP measurement outside active BWP, gap shall be configured. Typical MGL are 6ms, 3ms, 4ms, 5.5ms and 1.5ms. To minimize the RS overhead, one CSI-RS resource is used for both serving and neighboring cell measurements, e.g., CSI-RS resources for L1-RSRP measurement on neighbor cell for UE#1 are in fact the resources for L1 measurement of serving cell for UE#2. For serving cell, there is no specific restriction in time domain on configurations of CSI-RS resources for L1-RSRP measurement. It makes difficult to ensure all the CSI-RS of the same frequency layer are within a single gap window. To solve this issue, for CSI-RS based L3 measurement, all CSI-RS resources in the same MO are confined in the same 5ms window. If to support CSI-RS based L1-RSRP measurement outside active BWP, similar restriction is also necessary. If so, it may pose a strong limitation on CSI-RS configuration. Generally, MG is configured for SSB based L3 measurement. SSB of each cell are overlapped or partially overlapped with MG. Due to measurement restriction in FR2, UE is not supposed to perform SSB based L3 measurement and CSI-RS based L1 measurement simultaneously. So the reasonable configuration is that SSB symbols and CSI-RS symbols for L1 measurement in FR2 are not overlapping. As shown below, most of the slots are occupied by SSB and only the white ones are not occupied by SSB. And SSB takes most of the symbols in the slots that contain SSB. If not to use concurrent gap, NW shall squeeze all the CSI-RS resources to the remaining symbols. It seems difficult to confine SSB and CSI-RS of the same frequency layer in the same 5ms window. In other words, it is difficult to use one gap to cover both SSB based L3 or L1 measurement and CSI-RS based L1 measurement, so concurrent gap is needed.

Also, to support CSI-RS based L1-RSRP measurement outside active BWP, it is necessary to have a definition for frequency layer, as different frequency layers should be measured TDM-ly. If there is no such a definition, it will be difficult to discuss how to share the gap occasions by different CSI-RS resources.

Observation 1: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

In summary, to support CSI-RS based L1-RSRP measurement outside active BWP, there would be much more restriction on CSI-RS configurations, and an optional feature concurrent gap should be supported at first. In addition, compared to SSB based L1 measurement, we don’t see much gain to support CSI-RS based L1 measurement. It is sufficient to use SSB based L1 measurement for inter-f. We propose to deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Proposal 9: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

## 2.6 Whether and how to define “frequency layer”?

If RAN4 concludes not to define requirements for CSI-RS based L1-RSRP measurement outside active BWP and also RTD>CP case, maybe it is not necessary to define “frequency layer”.

In legacy, UE only performs L1-RSRP measurement on CSI-RS resources within active BWP in serving cell and RAN4 didn’t define any frequency layer concept for CSI-RS based L1 measurement. If only to consider CSI-RS based L1 measurement within active BWP and within CP, it will be similar as CSI-RS based L1 measurement in serving cell. It is not necessary to define frequency layer.

If RAN4 will define requirements for RTD>CP case or outside active BWP case, frequency layer shall be first defined. It is straight forward for CSI-RS based L1-RSRP measurement outside active BWP. In the next, we would like to discuss why frequency layer shall be first defined if to support RTD>CP case. To support RTD>CP case, another FFT supporting 48 PRBs is needed. As shown in Fig. 1, CSI-RS #1,2,3 are three resources from the same neighbor cell. It is obvious that CSI-RS #1 and #2 should be measured TDM-ly as they can not be covered by a single FFT window. If not defining the concept frequency layer, there will be ambiguity on how to measure CSI-RS#3. Or another much wider FFT is needed.

Fig. 1

Proposal 10: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

# Summary

In this paper, we provide some views on the R19 mobility. We have the following observation and proposals:

Proposal 1: Open to limit to SSB based L3 measurement only in the first condition of known cell definition in CSI-RS based L1-RSRP measurement.

Proposal 2: Keep L1 measurement for FR2-1 in the second condition of known cell definition in CSI-RS based L1-RSRP measurement.

Proposal 3: The third condition of known cell definition in CSI-RS based L1-RSRP measurement is: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB.

Proposal 4: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1.

Proposal 5: Considering that there is no gain compared to SSB based L1 measurement, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition OFF in FR2-1.

Proposal 6: For CSI-RS based L1-RSRP measurement on neighbor cell in FR2-1, only consider the following procedure:

- Step 1: SSB based L3 measurements

- Step 2: SSB-based L1 measurements and reports on the same candidate cell

- Step 3: CSI-RS based L1 measurements on the same candidate cell

  - Note: The CSI-RS in Step 3 should be Type-D QCL with the SSB in Step 2 and configured with repetition OFF.

Proposal 7: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

Proposal 8: Prefer to categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. We can compromise to call “CSI-RS based L1-RSRP measurement within active BWP” intra-frequency if it is the majority view.

Observation 1: If to support CSI-RS based L1-RSRP measurement outside active BWP, all CSI-RS in the same frequency layer shall be confined in the same 5ms window.

Observation 2: If to support CSI-RS based L1-RSRP measurement outside active BWP, concurrent gap shall be used.

Observation 3: If to support CSI-RS based L1-RSRP measurement outside active BWP, the definition of frequency layer should be discussed at first.

Proposal 9: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19.

Proposal 10: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

Proposal 7: CSI-RS based L1-RSRP measurement is performed only on known cells.

Proposal 8: For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 9: For CSI-RS resources with repetition OFF in FR2, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB.

Proposal 10: Not to support CSI-RS resources with repetition ON in FR2 for L1 measurement on neighbor cell.

Proposal 11: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs.

Proposal 12: Not to define CSI-RS based L1-RSRP measurement requirements on neighbor cell for RTD>CP case.

Proposal 13: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion.

# 4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#112bis.