3GPP TSG-RAN WG4 Meeting # 113	                                       R4-2417898

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda Item: 5.23.1

Source: MediaTek Inc.

Title: Discussion on R18 mobility test cases

Document for: Discussion

# Introduction

After further checking R18 mobility test cases, we found some missed points when define the test cases. We will discuss the related issues in this contribution.

# Discussion

## 2.1 Test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2)

We found a missed point in the test case for PDCCH-order RACH to inter-frequency candidate cell in FR1. We will elaborate the issue at first, and then propose a solution.

In the test case, MG is configured for inter-frequency measurement. In the current version, MGRP is set as 80ms, MGL and gapOffset are missing. The default value in RAN5 is MGL=6ms, and gapOffset=39 as specified in 38.508-1 table 4.6.3-70. In the next, we will use these settings for discussion. Based on the Gap configuration, MG starts from subframe 9 (Subframe = gapOffset mod10  Subframe=9) of the frames with “SFN mod 8 = 3” (SFN mod (MGRP/10) = FLOOR (gapOffset/10)  SFN mod 8 = 3). According to the SSB configuration used, SSB is on the first one or two slots in even SFNs.

In the test, RACH configuration 6 is used. After checking table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211, it can be found that RACH occasions are in subframe 7 under FDD, subframe 2&7 under TDD.

For information: From 38.133 A.3.8.2.6 RACH configuration 6: Field; Value; Comment. prach-ConfigurationIndex; 102; 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6].. From TS 38.211 table 6.3.3.2-2 FDD: PRACH Configuration Index; Preamble format; nf mod x=y; Subframe number; Starting symbol; Number of PRACH slots within a subframe; , number of time-domain PRACH occasions within a PRACH slot; , PRACH duration. 102; A1; 1; 0; 7; 0; 2; 6; 2. From TS 38.211 table 6.3.3.2-3 TDD: PRACH Configuration Index; Preamble format; nf mod x=y; Subframe number; Starting symbol; Number of PRACH slots within a subframe; , number of time-domain PRACH occasions within a PRACH slot; , PRACH duration. 102; A2; 1; 0; 2,7; 0; 1; 3; 4.

In test 1, with early TCI state activation, PDCCH order RACH delay is NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, i.e., N2 symbols + 0 + 0.5ms + 0 + 0 + {1ms,3ms,5ms,10ms} ≈ {2ms, 4ms, 6ms, 11ms}.

The interruption due to RF retuning before and after RACH transmission is calculated by ceil (Y/NR Slot length) +1 slots ({2, 2, 2, 3} slots with 15kHz corresponding to Y= {0.25,0.5,1,2}). There will be at most 3 slots interruption at 15kHz before and after RACH transmission.

Based on the configurations, it can be observed that:

- Under TDD, with 15kHz SCS, if PDCCH order is sent at slot#6 in the frame whose SFN mod 8 =3, UE may not have enough time to reschedule the up-coming MG and will fail to transmit RACH on slot#2 in the next frame. That’s because is slot#2 overlapped with MG as shown in Fig.1. In addition, RACH transmission on slot#7 will also fail because the RF retuning time before RACH overlaps with MG.

- Under FDD, with 15kHz, as shown in Fig.2, RACH transmission on slot#7 is also impacted by MG due to the retuning time is overlapped with MG. If PDCCH order is sent at slot#6,7,8 in the frame whose SFN mod 8 =3, UE may not have enough time to reschedule MG and fail to transmit RACH on slot#7 in the next frame.

Fig. 1

Fig.2

Observation 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), UE may fail to transmit RACH to the target cell on the first RACH occasion after PDCCH order RACH delay as the RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG.

Recalling the requirements defined in 38.213 cl.8.1, UE is not mandatory to transmit RACH on the first available RACH occasion after PDCCH order RACH delay which is similar as legacy PDCCH order RACH on serving cell. According to the spec, UE is allowed to transmit RACH on the next RACH occasion in the above mentioned scenarios. But in the test, we are mandatory UE to transmit RACH on the first RACH occasion. The intension to ask UE to transmit on the first occasion is to save NW RACH resources and reach common understanding on the occasion that UE transmits RACH. In the field, we think NW can avoid the above cases through smart scheduling or expect UE to transmit RACH on the next RACH occasion.

38.213 cl.8.1 If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation msec, ……

To avoid the case that the first RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG, it is necessary to add some limitation on the PDCCH order occasions. We propose to add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3 which is simple and can solve the issue.

Proposal 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

## 2.2 Test cases for SSB based L1-RSRP measurement in FR1 – intra-f & inter-f without gap (A.6.6.26.1 & A.6.6.28.1.3)

In the test cases for SSB based L1-RSRP measurement in FR1, intra-f & inter-f without gap, report period is set as 80 slots, SSB period is 20ms and DRX is not used. According to core part requirements, measurement period = max max(TReport, ceil(M*P)*TSSB_NBC) = 80ms with 15kHz SCS and 40ms with 30kHz SCS.

Regarding the test requirements, as shown in Fig. 4, there may be some time gap between the report occasion and the time UE finishes the first round of measurement. Therefore, the test requirements should be measurement delay + report delay considering the worst case. But in the above two test cases, UE is required to report a new value no later than 20 ms plus 80 slots from the beginning of time period T2. It is not aligned with core part requirements.

Fig. 4

Proposal 2: In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case, i.e., 80slots+80slots.

# Summary

In this paper, we have the following observation and proposals for R18 mobility test cases:

Observation 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), UE may fail to transmit RACH to the target cell on the first RACH occasion after PDCCH order RACH delay as the RACH occasion or the retuning time is overlapped by MG and UE has no sufficient time to reschedule MG.

Proposal 1: In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

Proposal 2: In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case, i.e., 80slots+80slots.