3GPP TSG-RAN WG4 Meeting #113			                                                   R4-2417938

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.1.1.1.1

Source:	Xiaomi

Title:	Discussion on General aspects for High power UE

Document for:	Discussion

- Introduction

In general, there are two main issues need to be discussed for introducing high power inter-band UL NR-CA/EN-DC with 2 bands and 2Tx and/or 3Tx. One is how to produce a general rule to ease the workload of defining MSD requirements, the other is how to deal with SAR issue. In previous meeting, there are some discussions on these two issues but no consensus. This contribution continues to provide our considerations.

- Discussion

MSD rules

The latest progress on MSD rules is captured in the following WF [1] in the last meeting.

4.1 MSD rules Agreement: - To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3 - Some examples are provided in the Annex of this WF - Other options such as Option2 are not precluded

As defining MSD requirement is the most tedious and time-consuming part when introducing high power class for inter-band CA or DC. If a consensus could be achieved on simplifying the method of defining MSD, it would heavily decrease the workload. Therefore, defining a relatively simple MSD rule is very helpful to decrease the workload of introduction high power class for inter-band CA or DC.

In the last meeting, the method using some equations or look-up tables or simplified guidelines to derive high power class MSD from respective PC3 MSD was discussed. From our perspective, we are a little pessimistic about this method. The reason is that based on our statistics and analysis mentioned in our contribution [3] as shown in figure 1 on band combinations which have IMD MSD requirements for both PC2 case and PC3 case in current spec. it can be observed that for the same IMD orders, the MSD difference between different band combinations vary greatly. And also, from the MSD requirements defined for specific band combination in the spec, the MSD difference between different IMD orders vary greatly. Consequently, it is hard to get a unify equations or look-up tables to derive PC2/PC1.5 MSD from respective PC3 MSD between different band combinations.

1 (a)                                                                                            1 (b)

1(c)

Figur 1, MSD difference between PC2 case and PC3 case. (a) for IMD2; (b) for IMD4; (c) for IMD5

Observation 1:

    - for the same band combinations, the MSD difference between different IMD orders vary greatly.

    - for the same IMD orders, the MSD difference between different band combinations vary greatly.

Observation 2: it is hard to get a unify equations or look-up tables to derive PC2/PC1.5 MSD from respective PC3 MSD between different band combinations.

And consider the following observations we made in contributions [3], we still think only the MSD requirements for default PC3 is enough.

Observation 1: the main reason why a larger MSD is required for high power inter-band CA is that the PA performance (including PA output power and unwanted emissions outside of assigned channel bandwidth) for each band will vary greatly compared with PC3 case, because other influencing factors, such as the performance of duplexers/diplexer/triplexer, PCB isolation, and antenna isolation, remain unchanged from the PC3 case. Observation 2: The PA performance of the constituted band under inter-band CA case can be guaranteed by meeting requirements for single band for corresponding power class. Observation 3: all the influencing factors on MSD with high power class have already been implicitly included by single band case and PC3 inter-band CA case.

Proposal 1: adopt the following option as the MSD general rules when introduction of PC2 and PC1.5 inter-band CA/DC

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

SAR issue

In RAN4#112bis meeting, the following agreement captured in [1] for SAR issue.

1.1 Duty cycle solution for SAR compliance Agreement:  For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed.  For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

For PC1.5 intra-band CA

In current spec 38.306, the dutycycle capabilities i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1 are reported per band which shows as following, therefore it can expected those capabilities could be reused for PC1.5 intra-band CA case. In other words, the capability is already there and there would no new capability introduced.

| maxUplinkDutyCycle-PC2-FR1 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is applicable for FR1 power class 2 UE and also applicable for FR1 power class 1.5 UE as specified in clause 6.2.1 of TS 38.101-1 [2]. If the field and maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 are both absent, 50% shall be applied as the upper limit of the UL duty cycle for power class 2. Value n60 corresponds to 60%, value n70 corresponds to 70% and so on. This capability is not applicable to IAB-MT. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for FR1 power class 1.5 UE as specified in clause 6.2.1 of TS 38.101-1 [2]. If the field and maxUplinkDutyCycle-PC2-FR1 are both absent, 25% shall be applied as the upper limit of the UL duty cycle for power class 1.5. | Band | No | N/A | FR1 only |

Observation 3: the capabilities, i.e., maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1 could be reused for PC1.5 case. In other words, the capability is already there and there would no new capability introduced for PC1.5 case.

Based on the above observation, we think it is not reasonable to preclude the duty-cycle approach for PC1.5 case. Therefore, it is proposed the following agreements could be achieved for PC1.5 intra-band CA

  - P-MPR is available for the UE for SAR mitigation method

  - Duty cycle based SAR solution:  Reuse the capability for single carrier case, i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1

  - Fallback behavior:  same fallback behavior as single carrier PC1.5. i.e. fallback to the corresponding PC3 or PC2 intra-band non-contiguous CA in case of the dutycycle condition is not met.

The detailed duty-cycle approach for high power class could be illustrated in the following figure 2.

Figure 2, the duty-cycle approach for high power class per band

Proposal 2: To mitigate the SAR issue, the following is proposed for PC1.5 intra-band CA.

  - P-MPR is available for the UE for SAR mitigation method

  - Duty cycle based SAR solution:  Reuse the capability for single carrier case, i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1

  - Fallback behavior:  same fallback behavior as single carrier PC1.5. i.e. fallback to the corresponding PC3 or PC2 intra-band non-contiguous CA in case of the dutycycle condition is not met.

For PC1.5 inter-band CA

As mentioned in our contribution [3], the duty-cycle approach has been adopted for PC2 inter-band CA case and PC1.5 inter-band CA with 3Tx in current spec. Our preference is to reuse the traditional way on dutycycle approach for PC1.5 case. Considering the current dutycycle conditions and the definition of the average percentage of uplink symbols for 3Tx are band combination agnostic and could be applicable for all possible PA configurations between two bands, it is reasonable that the existing approach for PC1.5 with 3Tx could be reused for SAR mitigation PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA.

Proposal 3: the existing approach for PC1.5 with 3Tx could be reused for SAR mitigation PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA.

For PC1.5 inter-band EN-DC

For PC1.5 inter-band EN-DC with 2Tx and 3Tx case, the dutycycle approach could be defined based on the existing dutycycle approach for PC2 case with some small changes as shown in the following table 1 at lease for TDD+TDD and FDD+TDD case. If FDD+FDD case is introduced, dutycycle approach should be studied further.

Table 1, SAR solution proposed for PC1.5 inter-band EN-DC with 2Tx and 3Tx case

| TDD + TDD | FDD + TDD |
| --- | --- |
| 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 or 15% (if capability is absent) as the max uplink dutycycle that PC1.5 inter-band CA case. if DutycycleNR≤0.5*maxUplinkDutyCycle, the requirements for PC1.5 applies if 0.5*maxUplinkDutyCycle＜DutycycleNR≤*maxUplinkDutyCycle, the requirements for PC2 applies otherwise the requirement for PC3 applies | Reuse two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2 are introduced if 40%≤ DutycycleLTE≤ 70% and DutycycleNR≤0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1, or if DutycycleLTE≤ 40% and DutycycleNR≤0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2, the requirements for PC1.5 applies if 40%≤ DutycycleLTE≤ 70% and 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1＜DutycycleNR≤maxUplinkDutyCycle-FDD-TDD-EN-DC1, or if DutycycleLTE≤ 40% and 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2＜DutycycleNR≤maxUplinkDutyCycle-FDD-TDD-EN-DC2, the requirements for PC2 applies otherwise, the requirement for PC3 applies |

However, considering the current dutycycle approach for EN-DC is a bit complicated and inconsistent between different power class for different band combination case. We are also ok to reconsider a unify approach on SAR solution for all possible HP UE scenarios. For example, Duty-cycle solution is not considered for any HPUE scenarios in this WI, or SAR solution relies on per single band manner

Proposal 4: For PC1.5 inter-band EN-DC with 2Tx and 3Tx case, the duty cycle approach could be defined based on the existing dutycycle approach for PC2 case with some small changes as shown in the table 1 for TDD+TDD case and FDD+TDD case.

Proposal 5: For simplicity, only P-MPR is considered (i.e., Duty-cycle solution is not considered) for inter-band EN-DC is also acceptable.

- Conclusion

In this paper, we give the further analysis based on the objective listed in the WID and make the following proposals:

MSD rules

Observation 1:

    - for the same band combinations, the MSD difference between different IMD orders vary greatly.

    - for the same IMD orders, the MSD difference between different band combinations vary greatly.

Observation 2: it is hard to get a unify equations or look-up tables to derive PC2/PC1.5 MSD from respective PC3 MSD between different band combinations.

Proposal 1: adopt the following option as the MSD general rules when introduction of PC2 and PC1.5 inter-band CA/DC

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

SAR issue

For PC1.5 intra-band CA

Observation 3: the capabilities, i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1 could be reused for PC1.5 case. In other words, the capability is already there and there would no new capability introduced for PC1.5 case.

Proposal 2: To mitigate the SAR issue, the following is proposed for PC1.5 intra-band CA.

  - P-MPR is available for the UE for SAR mitigation method

  - Duty cycle based SAR solution:  Reuse the capability for single carrier case, i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1

  - Fallback behavior:  same fallback behavior as single carrier PC1.5. i.e. fallback to the corresponding PC3 or PC2 intra-band non-contiguous CA in case of the dutycycle condition is not met.

For PC1.5 inter-band CA

Proposal 3: the existing approach for PC1.5 with 3Tx could be reused for SAR mitigation PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA.

For PC1.5 inter-band EN-DC

Proposal 4: For PC1.5 inter-band EN-DC with 2Tx and 3Tx case, the duty cycle approach could be defined based on the existing dutycycle approach for PC2 case with some small changes as shown in the table 1 for TDD+TDD case and FDD+TDD case.

Proposal 5: For simplicity, only P-MPR is considered (i.e., Duty-cycle solution is not considered) for inter-band EN-DC is also acceptable.

Reference

- R4-2417098, WF on HPUE UE for UL CA and EN-DC, Samsung

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei)

- R4-2415054, Discussion on general aspects for high power UE, Xiaomi