3GPP TSG-RAN WG4 Meeting # 113	R4-2417951

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for AI mobility

Agenda Item:	7.18.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss RRM impact on AI mobility.

- RSRP prediction accuracy requirement - Measurement error impact - Intra-frequency temporal domain prediction - Inter-frequency inter-cell prediction

- Discussion

In RAN#105 meeting, it’s agreed that RAN4 will start to study the impacts on requirements based on RAN2 assumptions[1].

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In the following, we will analyse the possible impacts on requirements based on RAN2 assumptions.

      - 2.1 RSRP prediction accuracy requirement

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

In TR[1], L3 beam-level prediction is in low priority, therefore, we prefer to deprioritize L3 beam-level(Point E) currently.

3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A

Observation 1: L3 filtered beam level prediction sub-use cases are lower priority.

Proposal 1: RAN4 to deprioritize L3 beam-level(Point E) measurement prediction.

For L1 beam level prediction accuracy, we are open to further discuss. In TR[2], it’s agreed that L3 cell-level measurement prediction accuracy is defined for all RRM sub-use cases.

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

It means that even RSRP is predicted at L1 beam level, there will be post-processing for these predicted results to derive L3 cell level results. Then the prediction accuracy will be evaluated at L3 cell-level RSRP. L1 beam level prediction is an intermediate result of L3 cell-level measurement prediction.

We also understand that in legacy RAN4, RSRP measurement accuracy is defined based on physical layer L1 filtered(Point A1) RSRP. Besides, L1 RSRP prediction will have impact on core requirement for observation window and prediction window. If no accuracy requirement is defined for L1 beam level RSRP prediction, it’s difficult to define observation window and prediction window for L1 beam level RSRP prediction. Therefore, we prefer to define L1 beam level RSRP prediction accuracy.

Proposal 2: Prefer RAN4 to study L1 beam level RSRP prediction accuracy requirement.

      - 2.2 Measurement error impact on RSRP prediction accuracy

Currently, in RAN2 simulation assumption, for both training and inference, ideal measured RSRP without error is used as input and label for AI model, similar as AI beam management. While in reality, the input of AI model will be measurement with error. Then there will be mismatch between training and inference. RAN4 also need to evaluate the impact of measurement error on RSRP prediction accuracy.

For measurement error impact on AI mobility, there is some similarities with that on AI beam management.

In last meeting, for AI based beam management, simulation assumption about how to model measurement error is agreed. AI model is trained without error, while for inference, input data is added with measured error. The performance degradation is evaluated by input data based on SLS for LLS. For error modeling, measurement error for different SNR condition can be simulated by CDL or TDL channel. For different TX beam, different measure error can be added as the SNR of each beam is different. This error modeling method is mainly target for spatial domain prediction, i.e. BM case-1. For BM case-2, it involves both spatial and time domain beam prediction. due to fading channel, SNR for each time instance in time domain is also varying, how to model measurement error for time domain has not been discussed yet.

In summary, for measurement error modeling, some similar issues can be discussed for AI mobility and RAN4 to check whether the method adopted by AI BM can be re-used:

- Evaluation metric

  - AI model is trained without error, while for inference, input data is added with measured error.

- Data set for performance evaluation

  - SLS or LLS

- How to set up measurement error model

  - Gaussian, or

  - Simulated by TDL or CDL or AWGN

- How to add measurement error

  - For spatial domain of FR2, different measurement error for different TX beams

  - For time domain, FFS

Proposal 3: RAN4 to discuss which part of procedure agreed in AI BM can be re-used for measurement error analysis of AI mobility.

      - 2.3 Intra-frequency temporal domain prediction

      - 2.3.1 RRM impact

RAN2 prioritize 3 scenarios:

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Two scenarios are related to intra-frequency intra-cell prediction:

- FR1 to FR1 intra-frequency temporal domain case B

- FR2 to FR2 intra-frequency temporal domain case A

Case A and case B

In TR[1], RAN2 defines observation window and prediction window, which will have impact on RAN4.

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

For legacy L3 RSRP measurement requirement, measurement will be conducted periodically. RAN4 defined measurement period which is required to satisfy the L1 filtered RSRP accuracy requirement. However, when AI is applied, some measurement instance can be skipped. Take case B for example, the target is to reduce measurement in temporal. Therefore, the measured samples will be reduced.

In prediction window, UE can skip measurement. It’s possible that scheduling restriction can be relaxed.

Proposal 4: Observation window and prediction window defined in RAN2 will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

      - 2.3.2 Observation window and prediction window

Observation window and prediction length is dependent on different sub-cases, different pattern and different L1/L3 filtering method.

Impact of L1/L3 sliding filtering

RAN2 defines L1/L3 sliding filtering and non-sliding filtering as below:

Fig.1: non-sliding L1/L3 filtering

While for L1/L3 filtering sliding filtering case, it’s defined as below:

Fig.2 Sliding L1/L3 filtering

The key difference between non-sliding filtering and sliding filtering is that:

  - For L1/L3 filtering non-sliding window, every 5 L1 samples will generate a L3 RSRP.

  - For L1/L3 filtering sliding window, every 1 L1 sample will generate a L3 RSRP.

Therefore, the final observation window length for the two cases is different for L3 RSRP to L3 RSRP prediction case.

Observation 2: Observation window and prediction length is dependent on different L1/L3 filtering method.

Impact of different sub-cases

Currently, RAN2 defined 3 subcases for RSRP prediction.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

- Sub-case 1: RSRP is predicted at L1

For sub-case 1, the prediction happens at L1. In legacy, 5 L1 samples are needed to be measured in one measurement period, which is shown in Fig.3(a). While when AI is applied, for example, only 3 samples are needed to be measured during observation window. Then UE can predict other samples with grey colour, as shown in Fig.3(b). Then measurement period can be reduced. Here observation window is similar as legacy measurement period with reduced samples. Observation window and prediction window length are based on number of L1 samples.

Fig.3(a) measurement period for non-AI L1 filtered RSRP for FR1

Fig.3(a) observation window and prediction window for AI based L1 filtered RSRP

- Sub-case 2: RSRP is predicted at L3

For sub-case 2, we give examples for non-sliding L1/L3 filtering case.

For sub-case 2, filtered L3 RSRP will be predicted by L3 RSRP. Suppose that every 2 L3 filtered RSRP can predict one L3 filtered RSRP. As shown in Fig.4, UE will measure for 10 samples to get 2 L3 results and skip the following L1 5 samples. For this case, the observation window and prediction window length are based on number of L1 measurement period.

Fig.4 observation window and prediction window for AI based L3 filtered RSRP

The observation window will be different for initial stage and later stage. At initial stage, new L1 filtered RSRP will be obtained very L1 sample.

- Sub-case 3: RSRP is predicted at L3

For sub-case 3, RSRP is also predicted at L3. Then observation window and prediction window are similar as sub-case 2.

Observation 3: Observation window and prediction length is dependent on different sub-cases.

Impact of pattern

RAN2 also defines different patterns for prediction window and observation window as below, it will also have impact on the prediction window length and observation window length

Fig.5 Example 1 of intra-frequency temporal domain case B

Fig.6 Example 2 of intra-frequency temporal domain case B

Observation 4: Observation window and prediction length is dependent on different patterns.

Proposal 4: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window.

      - 2.4 Inter-frequency inter-cell prediction

In TR[1], RAN2 agrees that:

For both Intra-frequency prediction inter-cell prediction and FR1 to FR1 inter-frequency inter-cell prediction, no measurement is reduced in both temporal and spatial domain for cell to be measured. For FR1 to FR1 inter-frequency inter-cell prediction, focus on the case where cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighboring site. If inter-frequency correlation model is assumed, section 7.6.5 in [4] is taken as baseline for inter-frequency correlation model. FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub-use cases.

For inter-frequency inter-cell RSRP prediction, UE may skip the measurement for another cell on other frequency. Then it’s possible to reduce CSSF.

Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 5: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: L3 filtered beam level prediction sub-use cases are lower priority.

Proposal 1: RAN4 to deprioritize L3 beam-level(Point E) measurement prediction.

Proposal 2: Prefer RAN4 to study L1 beam level RSRP prediction accuracy requirement.

Proposal 3: RAN4 to discuss which part of procedure agreed in AI BM can be re-used for measurement error analysis of AI mobility.

Proposal 4: Observation window and prediction window defined in RAN2 will have RRM impact:

  - Observation window will have impact on measurement duration

  - Prediction window will have impact on scheduling restriction

Observation 2: Observation window and prediction length is dependent on different L1/L3 filtering method.

Observation 3: Observation window and prediction length is dependent on different sub-cases.

Observation 4: Observation window and prediction length is dependent on different patterns.

Proposal 4: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window requirement.

Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period.

Proposal 5: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

- Reference

[1] RP-242393 Revised SID: Study on Artificial Intelligence (AI)/ Machine Learning (ML) for mobility in NR, OPPO

[2] R2-2406309, text proposal on TR 38.744 v0.03

[3] R2-2407849, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions.