3GPP TSG-RAN WG4 Meeting # 113	R4-2417954

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on UE-initiated/event-driven beam management on Rel-19 MIMO phase 5

Agenda Item:	7.19.3.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the RRM requirement of UE-initiated/event-driven beam management for Rel-19 MIMO phase 5.

- Discussion

2.1 Measurement result in L1 report

Issue 1-5: How to consider UL resource in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of UL resource waiting time for L1 event triggered reporting. - Option 2: - The beam reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. - No need to differentiate mode A and mode B - Option 3: - Report delay is defined as Tfirst-PUCCH or TPUCCH + TDCI + T1. Where TDCI = 0 for Mode B, Time to receive DCI with UL grant in Mode A; T1 = K2 for Mode A; Tfirst-PUSCH or TPUSCH for Mode-B - Option 4: - To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not (for Mode-A). - Other options are not precluded. Issue 1-6: Whether to define requirements if end of measurement period is before the UL resource? <Way forward > - Option 1: - RAN4 to discuss whether and how to define requirement when measurement period is shorter than the UL resource waiting time

For Issue 1-5 and issue 1-6, the major impact is which L1 measurement result is put into the L1 report.

In legacy L3 event triggered reporting, according to 38.331, if L3 measurement result satisfies the event triggering condition, UE will initiate the message reporting procedure and submit the MeasurementReport message to lower layers for transmission. It means that the L3 measurement result which triggers reporting will be included in the L3 report.

### 5.5.4 Measurement report triggering #### 5.5.4.1 General 2> if the reportType is set to eventTriggered, and if the corresponding reportConfig does not include numberOfTriggeringCells, and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event): 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> if useT312 is set to true in reportConfig for this event: 4> if T310 for the corresponding SpCell is running; and 4> if T312 is not running for corresponding SpCell: 5> start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR; 3> initiate the measurement reporting procedure, as specified in 5.5.5; ### 5.5.5 Measurement reporting #### 5.5.5.1 General ……. 1> else if the UE is in NR-DC: 2> if the measurement configuration that triggered this measurement report is associated with the SCG: 3> if SRB3 is configured and the SCG is not deactivated: 4> submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends; 3> else: 4> submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3; 2> else: 3> submit the MeasurementReport message via SRB1 to lower layers for transmission, upon which the procedure ends; 1> else: 2> submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends.

Observation 1: For legacy L3 event triggered reporting, it is specified in 38.331 that the measurement result which triggers L3 reporting will be included in the L3 report.

For L1 event triggered reporting, different from L3, L1 measurement is much faster, it’s possible that UE can obtain several new L1 results between the time UE trigger L1 reporting and the time when UL resource arrives.

Fig.1 New L1 results

Similar as L3 event triggered reporting, it needs to specify which measurement result will be included in L1 reporting. Following the same logic of L3, L1 result which triggers L1 reporting will be included in L1 reporting. It means that if L1 reporting procedure starts, the following L1 result will not be evaluated anymore.

Proposal 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

2.2 Needed measurement time

Until now, the supported triggering event agreed in RAN1 include:

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signaling - UE may only indicate a single candidate value or not support Event-7.

Take event 2 as example, suppose that current beam is from serving cell and new beam is from cell with different PCI(CDP). Measurement periods for two beams are different, e.g. TSSB_serving and TSSB_CDP. As shown in Fig.1, if follow the legacy L3 requirement, total measurement time is defined based one measurement period of TSSB_CDP.  However, since TSSB_CDP < TSSB_serving, when UE obtain L1-RSRP of new beam, UE doesn’t obtain L1-RSRP from current beam. UE can’t compare with criteria and trigger L1 report.

Fig.2 Different measurement period for different beams in inter-cell mTRP

While for event 1, UE only to measure current beams. Measurement time only needs to consider one beam.

For event 7, UE needs to measure for beams with activated TCI states. Total measurement time also needs to consider the measurement period of multiple beams.

Therefore, if multiple beams will be measured with different measurement period, total measurement time will be maximum measurement period between multiple beams.

Proposal 2: If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

2.3 Impact of Triggering window

From RAN1, for event-2, a window including M instance(s) for new beam will be defined for triggering L1 reporting. Here, one instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam.

[117] Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In legacy L3 reporting, only one measurement period is defined in reporting delay to get rid of impact from high levels. While for event triggered L1 reporting, the impact of triggering window in physical layer needs to be considered. The event will be triggered if L1 results during whole window satisfied the condition. However, from RAN1 agreement, since the minimum number of M can be 1, we are also fine that triggering window length can be 1. From test aspect, it can verify UE behavior to triggers the L1 report as soon as possible.

Fig.3 Triggering window

Proposal 3: For measurement time, prefer to consider the impact of triggering window defined in RAN1.

Issue 1-11: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward > - Option 1: - Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place - Option 2: - Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. No enhancement. - Option 3: - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. - Other options are not precluded.

We suggest to reuse the current assumption of beam sweeping and don’t introduce enhancement. Therefore, we support option 2.

Proposal 4: Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting.

2.4 L1-RSRP Accuracy requirement

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: - The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements. - Option 2: - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements. - Option 3: - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement. - Other options are not precluded.

For legacy L3 event triggered reporting, there is no new requirement for measurement accuracy. Similar, for L1 event triggered reporting, there is also no new requirement for measurement accuracy.

Proposal 5: No new accuracy requirements for L1 measurements are required.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: For legacy L3 event triggered reporting, it is specified in 38.331 that the measurement result which triggers L3 reporting will be included in the L3 report.

Proposal 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting.

Proposal 2: If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams.

Proposal 3: For measurement time, prefer to consider the impact of triggering window defined in RAN1.

Proposal 4: Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting.

Proposal 5: No new accuracy requirements for L1 measurements are required.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung