3GPP TSG-RAN WG4 Meeting # 113	R4-2417955

# Orlando, US, 18th – 22nd November, 2024

Source: 	Xiaomi

Title: 	Discussion on other RRM requirement on Rel-19 MIMO phase 5

Agenda Item:	7.19.3.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 MIMO phase 5.

- UL TCI state activation in Asymmetric DL sTRP/UL mTRP - Two TAs in Asymmetric DL sTRP/UL mTRP

- Discussion

2.1 Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/UL mTRP deployment scenarios, there will be one anchor DL-TRP and multiple UL-only TRPs and they are not co-located. In legacy, each TRP has both downlink and uplink transmission. While for UL-only TRP, there is no downlink. From our understanding, in legacy RAN4 has defined following requirement for uplink transmission for mTRP:

UL TCI state activation, including PL-RS calculation

- UL timing requirement

UL TCI state activation

Source RS in UL TCI state activation

For UL-only TRP, DL-RS is not available. There are only two options for source RS in UL TCI state of UL-only TRP.

- SRS

- DL-RS from the anchor DL-TRP

Since anchor DL-TRP and UL-only TRP are not co-located, the UL beam directions to the two TRPs are different. It’s difficult to apply the DL-RS from anchor DL-TRP as source RS in UL TCI state of UL-only TRP. Therefore, SRS will be configured as source RS for UL-only TRP.

Observation 1: For UL-only TRP, SRS will be configured as source RS in UL TCI state activation.

In legacy RAN4, no requirement is defined when SRS will be configured as source RS for UL TCI state activation. However, for UL-only TRP, RAN4 may need to define requirement for SRS based UL TCI state activation, otherwise, no TX beam can be correctly transmitted to UL-only TRP. While for anchor DL-TRP, DL-RS can still be used as source RS in UL TCI state activation. It’s FFS whether SRS can be configured as source RS for DL-TRP as well.

Proposal 1: For UL-only TRP, RAN4 to define UL TCI state activation requirement based on SRS.

Proposal 2: For anchor DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. RAN4 to discuss whether to also define UL TCI state activation requirement based on SRS for anchor DL-TRP.

Known condition for UL TCI state

For UL TCI state activation requirement, known condition for UL TCI state is defined as pre-requisite. The legacy UL TCI activation procedure is tied to DL beam management in advance, specifically through L1-RSRP measurement and reporting. Different delay requirements are specified for known and unknown TCI states. In the case of a known TCI, the measurement report is based on DL-RS. However, for UL-only TRP, if DL-RS is not available, known and unknown definition for UL TCI state can’t be re-used.

Observation 2: For UL-only TRP, if DL-RS is not available, known and unknown definition for UL TCI state activation can’t be re-used.

When SRS is configured as source RS in UL TCI state activation, SRS resource ID will be provided. UE can apply the TX beam associated with the SRS resource ID as the target TX beam. The SRS resource ID needs to be configured for UE to transmit before the UL TCI state activation. In other words, NW needs to configure UE to transmit multiple SRS first and then configure one SRS as source for the following UL TCI state activation. Similar as known TCI state based on DL-RS, a known UL TCI state based on transmission of SRS can also be defined, e.g. the source SRS in UL TCI state activation has been transmitted for UL beam management within [x]ms before UL received UL TCI state activation command.

Proposal 3: RAN4 to discuss how to define known UL TCI state based on SRS.

UL TCI state activation delay for UL-only TRP

In legacy, two types of UL state activation delay requirement are defined:

- MAC CE based UL TCI state activation

- DCI based UL TCI state activation

For DCI based UL TCI state activation, the procedure and delay length are defined in RAN1. Activated TCI state in DCI can include SRS or DL-RS. Therefore, the delay requirement can be re-used for UL-only TRP.

For MAC CE based UL TCI state activation delay, we suggest to only define UL TCI activation for only known case. For known case, which will also include pathloss calculation based on measurement from another TRP and configured or updated pathloss offset associated with this TCI state. The legacy MAC CE based UL TCI state activation delay of known case can be used as baseline.

Proposal 4: Legacy DCI based UL TCI state activation requirement can be re-used for UL-only TRP.

Proposal 5: RAN4 only to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

Two TA

In RAN1#118bis meeting, the agreement is as below.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Observation 3: Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

Observation 4: One downlink reference timing is supported and applied to both TAGs.

Observation 5: One single n-TimingAdvanceoffset is configured and applied to both TAGs.

In Rel-18, for each UL TCI state, an associated TAG ID will be provided. Similar, for UL TCI state for asymmetric DL sTRP/UL mTRP in Rel-19, an associated TAG ID will also be configured.

The UL timing requirement defined in 38.133 is as below:

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAG as the uplink signal. For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

In Rel-18 PUCCH/PUSCH/SRS transmission with two TAs, two DL reference timing will be applied for each TAGs respectively. As agreed in RAN1, since one DL reference timing will be applied for two TAGs, the requirement defined in Rel-18 can’t be re-used directly. It needs some modification. The DL reference timing will be based DL TRP. For asymmetric DL sTRP/UL mTRP deployment with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL sTRP.

For Rel-18 intra-cell mDCI PRACH transmission with two TAs, coresetPoolIndex is used. While coresetPoolIndex will be removed as agreed in RAN1. The reference timing for PRACH needs to be updated either.

Proposal 6: For asymmetric DL sTRP/UL mTRP with two TAs, the reference point for PUCCH/PUSCH/SRS/PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL sTRP.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: For UL-only TRP, SRS will be configured as source RS in UL TCI state activation.

Proposal 1: For UL-only TRP, RAN4 to define UL TCI state activation requirement based on SRS.

Proposal 2: For anchor DL-TRP, legacy UL TCI state activation requirement based on DL-RS can be re-used. RAN4 to discuss whether to also define UL TCI state activation requirement based on SRS for anchor DL-TRP.

Observation 2: For UL-only TRP, if DL-RS is not available, known and unknown definition for UL TCI state activation can’t be re-used.

Proposal 3: RAN4 to discuss how to define known UL TCI state based on SRS.

Proposal 4: Legacy DCI based UL TCI state activation requirement can be re-used for UL-only TRP.

Proposal 5: RAN4 to define MAC CE based UL TCI state activation requirement based on known TCI. The legacy MAC CE based delay requirement can be used as baseline.

Observation 3: Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

Observation 4: One downlink reference timing is supported and applied to both TAGs.

Observation 5: One single n-TimingAdvanceoffset is configured and applied to both TAGs.

Proposal 6: For asymmetric DL sTRP/UL mTRP with two TAs, the reference point for PUCCH/PUSCH/SRS/PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL sTRP.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung