3GPP TSG-RAN WG4 Meeting #113																				 R4-2417959

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.2

Title: 	Discussion on RRM requirements for adaptation of common signal/channel in NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and one of the objectives below was discussed in the last meeting [4, R4-2416860].

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In the las RAN4 meeting, the high level agreements on RRM spec impacts was achieved [4].

R4#112bis discussion Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

Thus, in this meeting, we provide our further considers on these issues.

- Discussion

The agreements made in RAN1 on adaptation mechanism of SSB and scenarios was summarized.

R1#116 Agreement Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration R1#116bis Agreement Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized R1#117 Agreement Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB R1#118bis Agreement Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

  - Transition period

Issue 1-2-2: Whether and how to define requirements for “transition period” For L1 measurement requirements, RAN4 to define RRM requirements for “transition period” of SSB adaptation, considering following approach: - Option 1: Reuse exiting transition requirements - Option 2: longer delay before or after SSB adaption - Option 3: Combine SSB measurement before and after SSB adaptation - Other options are not precluded

From RAN4 requirement perspective, whether and how to define requirements for “SSB burst transition period”  is under hot debating. However, it shall firstly be clarified that the transition period can be defined separately or included in the L1/L3 measurement delay jointly. In our view, the transition period discussed in RAN4#103bis is intend to guarantee UE can perform the consequential L1/L3 measurement based on the new adaptation SSB correctly.

As illustrated in the figure below, the main assumption to include such transition period in the measurement delay is UE shall continue the measurements during a single measurement reporting period. For an example in Figure 1, during a successful measurement reporting period if 4 samples needed and only 3 SSBs with the same period are available before the adaptation SSB is triggered, the measurement delay can be defined as:

3* Tcycle0+1* Tcycle1 + Ttransition

Wherein,  Ttransition is the extra time needed for SSB type transition processing.

Figure 1.

Observation 1: the adaptation SSB configuration happened within a successful measurement report period may result in the measurement requirements updates.

However, the assumption on UE behavior to define this transition period or the corresponding L1/L3 measurement period is questionable. Generally, if the measurement SSB was changed in either period or types UE can

- Alt1: continue the measurements with the new SMTC/SSB period or

- Alt2: discard the previous measurement samples with the obsolete SMTC period but restart a new measurement reporting procedure.

In RAN4 usually took the Alt2 as the baseline assumption to define the measurement requirement to avoid the too complicated requirements and specification works. For an example, as defined in 9.1.7 when the measurements happened during the PreMG status changed, there is no any explicit requirements define for these transition time.

9.1.7.2 Requirements applicability ….. When a pre-configured measurement gap is used to perform PRS measurements, the UE will inform the network that it is going to start/stop PRS measurements with the configured pre-configured measurement gap by initiating the existing LocationMeasurementIndication procedure. If the Pre-MG status changes during a measurement period of a measurement that can be performed without and within measurement gaps, the UE is allowed to restart the measurement. If the Pre-MG status changes from activated to deactivated during a measurement period of a measurement that can only be performed within measurement gaps, the measurement requirements do not apply.

In other words, in order to simplify RAN4’s work, as the method we used before, we can assume the requirements applicability is no measurement period (SMTC) was not updated within a measurement reporting interval as illustrated in Figure 2.

Figure 2.

Therefore, we can propose that:

Proposal 1: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval.

Consequently, we also prefer not define the transition period together with the measurement period.

Proposal 2: The requirements for transition period can be decoupled with L1/L3 measurement. FFS on how to defined them separately.

  - L3 measurement

As interpretated in sub section 2.1. above, the L1/L3 measurement can be decoupled with other transition period. That is if we assumed the adaptation SSBs with the same measurement period will be used within a measurement report period, the corresponding requirements in RAN4 RRM are also expected to be reused as the baseline.

Moreover, from RAN4 RRM perspective, in legacy RRM requirement, both intra and inter-frequency measurements requirement in TS38.133 section 9.2 and 9.3 can be summarized as:

Intra-frequency measurements: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Inter-frequency measurements: Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Observation 2: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement

Thus, UE can follow the indicated SSB adaptation was indicated by NW RRC message or other SIB message as well.

Observation 3: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Especially as agreed in the last meeting, RAN1 agreed the adaptation SSB could not used for PCell sync raster. In our view, that is the corresponding SSB based measurement requirements in clause 9 of TS38.133 shall be clarified because they can applicable for both PCell and SCell without any explicit indication (e.g. Table 9.2.5.2-3: for measurement on deactivated SCell).

R1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Proposal 3: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: the adaptation SSB configuration happened within a successful measurement report period may result in the measurement requirements updates.

Proposal 1: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval.

Proposal 2: The requirements for transition period can be decoupled with L1/L3 measurement. FFS on how to defined them separately.

Proposal 3: The current SSB based measurement requirements in 9.2, 9.3 and 9.4 [2,TS38.133] shall update the applicability for the adaptation SSB for PCell.

- References

- RP-240830

- TS38.133 v18.5.0

- TS38.331 v17.1.0

- R4-2413885