3GPP TSG-RAN WG4 Meeting #113	R4-2417979

# Orlando, USA, 18 – 22, November 2024

Source: 	Huawei, HiSilicon

Title: 	Simulation analysis for Ambient IoT

Agenda Item:	7.21.2.2

Document for:	Approval

Introduction

In RAN4#112bis, the simulation results for Ambient-IoT co-existence study were collected in [1], and plenty of co-existence cases with D1T1/D2T2- A2/-B were concluded in [2]. The contribution is to discuss and analyse the cases without conclusions in deployment scenario and topology D1T1.

Discussion

Evaluation analysis for some cases

For the cases without co-existence conclusions, scenario 1-1 and 2-1 with NR UE located only outdoors, evaluation results provided by most companies show that SINR degradation is less than 1dB in the cases of AIoT system as victim, and negligible NR DL throughput degradation in the cases of NR system as victim. As approved in WF [3] in RAN4#112, for the cases that AIoT system as victim, outage percentage for inter-system interference should be considered if SINR degradation is larger than 1 dB, and outage percentage for intra-system interference should be considered. It should be applied to the case 1-1b, 1-1d, 2-1f and 2-1d.

Observation 1: The conclusion of SINR degradation is less than 1dB should be applied to case 1-1b and case 2-1f. And outage percentage for intra-system interference should be provided for case 1-1b, 1-1d, 2-1f and 2-1d.

For scenario 1-2 and 2-2 with part of NR UEs located indoors, very few companies show that SINR degradation more less than 1dB in the case 1-2b with 10% NR UE indoors, which is quite different from others. In this case, we can try to conclude that SINR degradation is larger than 1 dB. According to the metric of AIoT system, outage percentage for intra-system interference should be considered to the case 1-2b, 1-2d, and 2-2d.

Observation 2: The conclusion of SINR degradation is less than 1dB should be applied to case 1-2b. And outage percentage for intra-system interference should be provided for case 1-2b, 1-2d, and 2-2d.

Case 1-2c: Reader->NR UE

In Case 1-2c, NR downlink throughput degradation was observed in [1]. Among five simulations with 10% indoor NR UEs, most results fell within the acceptable range, that is 5% throughput degradation; but two showed more than 10% average degradation and over 50% at the cell edge. When NR UEs are 100% indoor, degradation worsens, but it is considered an extreme scenario optionally. Realistically, a 10% indoor presence is more practical. The distance from the NR UE to the Reader affects degradation, especially when the UE is directly beneath the Reader. Since Readers are not always active, with inventory times as low as 1/10, the chances of significant degradation occurring are reduced. Shifting NR UEs to another band can also help mitigate issues.

For case c, Reader interfering NR UE, considering no CW may participate in the communication procedure, in our view, there is no difference from case 1-2c with inside topology to case 2-2c with outside topology. Same conclusion can be applied to case 1-2c and case 2-2c. We provide the updated evaluation results and analysis for case 1-2c as follows.

| CDF | Baseline SINR | SINR w. inter-system interference | Baseline T-put | T-put w. inter-system interference | T-put degradation |
| --- | --- | --- | --- | --- | --- |
| 5% | -3.5 | -4.66 | 2.99 | 2.38 | 25.63% |
| 50% | 4.57 | 4.46 | 10.95 | 10.8 | 1.39% |

Observation 3: For case 1-2c and case 2-2c, NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%.

Case 2-2f: NR UL->reader

In Case 2-2f, A-IoT Reader D2R SNR degradation was observed for both 5% and 50% CDF. With 10% indoor NR UEs, one simulation showed SINR degradation within 1 dB, while others indicated more than 1 dB at 5% CDF and at 50% CDF. In practical scenarios, better channel conditions allow NR UEs to transmit at lower power, reducing degradation. The simulation assumed only two active Readers, but if each device had its own nearby Reader, SNR degradation would be less significant. Deployment solutions, such as shifting NR UEs to another band, can alleviate these issues. We provide the updated evaluation results and analysis for case 2-2c as follows with the device cell selection of 18 reader in one factory of D1T1.

| CDF | Baseline SINR | SINR | SINR degradation |
| --- | --- | --- | --- |
| 5% | 11.02 | -13.05 | 24.07 |
| 50% | 23.27 | 19.91 | 3.36 |

Observation 4: SINR degradation is more than 1dB. Inter-system interference is Observed. Outage percentage for inter-system interference should be provided.

Simulation conclusions

| deployment scenario and topology |  | spectrum | Evaluation case No. | (Aggressor ->Victim) | Tentative Co-existence conclusion |
| --- | --- | --- | --- | --- | --- |
| 1-1 | D1T1-A2- legacy UE only outdoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | b | NR DL->reader | Proposal 1: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.64% |  |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | d | NR DL->device | Proposal 2: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2% |  |
| 1-2 | D1T1-A2-legacy UE indoor | R2D: DL CW2D and D2R: DL | a | Device->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed |  |
|  |  |  | b | NR DL->reader | Proposal 3: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.7% |  |
|  |  |  | c | Reader->NR DL | Proposal 4: NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%. Interference coordination/mitigation solution can be considered. |  |
|  |  |  | d | NR DL->device | Proposal 5: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2% |  |
| 2-1 | D1T1-B-legacy UE only outdoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | f | NR UL->reader | Proposal 6: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.26% |  |
|  |  |  | c | Reader->NR DL | Negligible NR DL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | d | NR DL->device | Proposal 7: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2% |  |
| 2-2 | D1T1-B-legacy UE indoor | R2D: DL CW2D and D2R: UL | e | Device->NR UL | Negligible NR UL throughput degradation for both average throughput and cell edge throughput is observed. |  |
|  |  |  | f | NR UL->reader | Proposal 8: SINR degradation is more than 1dB. Inter-system interference is Observed. Outage percentage for inter-system interference is 14.1% |  |
|  |  |  | c | Reader->NR DL | Proposal 9: NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%. Interference coordination/mitigation solution can be considered. |  |
|  |  |  | d | NR DL->device | Proposal 10: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2% |  |

Conclusion

Based on the analysis in this contribution, we have the following observations and proposals:

Observation 1: The conclusion of SINR degradation is less than 1dB should be applied to case 1-1b and case 2-1f. And outage percentage for intra-system interference should be provided for case 1-1b, 1-1d, 2-1f and 2-1d.

Observation 2: The conclusion of SINR degradation is less than 1dB should be applied to case 1-2b. And outage percentage for intra-system interference should be provided for case 1-2b, 1-2d, and 2-2d.

Observation 3: For case 1-2c and case 2-2c, NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%.

Observation 4: SINR degradation is more than 1dB. Inter-system interference is Observed. Outage percentage for inter-system interference should be provided.

For case 1-1b:

Proposal 1: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.64%.

For case 1-1d:

Proposal 2: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2%

For case 1-2b:

Proposal 3: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.7%

For case 1-2c:

Proposal 4: NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%. Interference coordination/mitigation solution can be considered.

For case 1-2d:

Proposal 5: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2%

For case 2-1f:

Proposal 6: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.26%

For case 2-1d:

Proposal 7: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2%

For case 2-2f:

Proposal 8: SINR degradation is more than 1dB. Inter-system interference is Observed. Outage percentage for inter-system interference is 14.1%

For case 2-2c:

Proposal 9: NR DL throughput degradation for cell edge throughput is observed, and NR DL throughput degradation for average throughput is less than 5%. Interference coordination/mitigation solution can be considered.

For case 2-2d:

Proposal 10: SINR degradation is less than 1dB. Inter-system interference is negligible. Outage percentage for intra-system interference is 0.2%

Reference

[1] R4-2416355, Collection of simulation results for Ambient-IoT co-existence study, RAN4#112bis, October 2024.

[2] R4-2417196, WF on co-existence study for A-IOT, RAN4#112bis, October 2024.

[3] R4-2414304, WF on Co-existence study for ambient IoT, RAN4#112, August 2024.