3GPP TSG- WG4 Meeting # 113	R4-2417982

# Orlando, USA, November 18 – 22, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 EESS

Agenda Item:	6.21.2

Document for:	Approval

Introduction

In RAN#105, the new WID on mmWave UE spurious emission was approved with the following objectives.

The objective of this WI includes the following aspects: - For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes are considered in the above work - For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

This paper focus on the UE part for Rel-19 EESS, and provide analysis and proposals.

Regulation background for EESS

According to the justification in [1], in the latest regional regulation on mmWave usage, the UE spurious requirements of -5dBm/200MHz has been required at an earlier stage. Thus, the requirements of 1dBm/200MHz are not sufficient for the commercial deployment in some regions or countries.

- The -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission for commercial deployment needs to be satisfied in China.

- EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

Dating back to Rel-16, EESS was also discussed based on the outcome of WRC-19.

In RAN4#94-e, it was agreed in [2]

The followings are package agreements. - For n257, - 1dBm/200MHz @23.6-24GHz is defined for NS_200 - Note: No A-MPR is required regardless of Power classes: See R4-2000216. - For n258, - 1dBm/200MHz @23.6-24GHz is defined for a new NS as additional requirement - Note: Necessary amount of A-MPR will be captured accordingly. See R4-2000216. - For both n257 and n258 - -5dBm/200MHz @23.6-24GHz is captured as additional requirement if requested - -8dBm/200MHz @23.6-24GHz stays in the spec until EU decides the limit. - The current requirements, however, shall be fixed as they shall have been. - NS_201 and NS_202 must be merged and A-MPR should be generated to meet them simultaneously. Also, -8dBm/200MHz protection for n257 is removed. - A feature for “UE” to report supporting NS is introduced based on WF of R4-2002737.

In RAN4#94-e, it was agreed in [3]

The followings are package agreements. - How to resolve the issues identified in the background slides - NS mapping and associated A-MPR - After the process on how to resolve the newly introduced NS issues and NS mapping is agreed, each band will be handled independently without waiting for the completion of the other bands. A feature for a UE to report newly supported NS value(s) for a legacy band to a network is introduced. Four Alts are listed. - Alt 1-1: Explicit signaling for a UE to report newly supported NS value(s) for a legacy band to the network (introduce a new signaling feature) - Alt 1-2: Explicit signaling for a UE to report newly supported NS value(s) for a legacy band to the network (reuse modifiedMPR bits) - Alt 1-3: Explicit signaling for a UE to report newly supported NS value(s) for a legacy band to the network, such that supporting of NS values is optional - Alt 2: Delay introduction of new NS until beginning of Rel-X, such that it is mandatory for a UE designed against Rel-X to support all NS values defined in the Rel-X version of the specification and which correspond to the bands which the UE supports. For NS mapping and A-MPR, - Adopt one of the two (Option 1 or 2) based on necessity of -8dBm/200MHz. - +1dBm/200MHz for n257 & 7dBm/1GHz for n259 are introduced into NS_200, respectively, at least if no A-MPR is needed. - The same mapping for single CC is applied to that for CA. - A note is added to NS values for changed emissions limits (example shown: -5dBm/200MHz after 1st Sep 2027) - Option 1: -8dBm/200MHz stays Note: n261 is omitted since the band has no connection with lower/upper EESS protection. - Option 2: -8dBm/200MHz isn’t necessary Note: n261 is omitted since the band has no connection with lower/upper EESS protection.

In RAN4#95-e, in [4] Option 2 and Alt.1-2 in [3] was agreed as a package.

In [5], A-MPR needed to comply with the regulatory requirements and NS values aligned with the WRC-19 outcome are listed.

Table 7.5.4.1-2: Summary of A-MPR needed to comply with the regulatory requirements in Table 7.5.4.1-1

| Band | Requirement | A-MPR for PC1 | A-MPR for PC2 | A-MPR for PC3 | A-MPR for PC4 |
| --- | --- | --- | --- | --- | --- |
| n258 | (Case A) Protected range: 23.6 - 24.0 GHz Limit: +1 dBm/200 MHz | 3.0 if Offset frequency < BWchannel, 0.0 otherwise | 0 | 0 | 0 |
| n257 |  | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) |
| n258 | (Case B) Protected range: 23.6 - 24.0 GHz Limit: -5 dBm/200 MHz | 7.0 if Offset frequency < BWchannel, 6.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise |
| n257 |  |  |  |  |  |
| n259 | (Case C) Limit: +7 dBm/1000 MHz and -13 dBm/1 MHz | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) |

Table 7.5.4.1-3: Summary of NS values aligned with the WRC-19 outcome

|  | n257 | n258 | n259 | n260 | Status of each NSs |
| --- | --- | --- | --- | --- | --- |
| NS_200 | 1dBm/200MHz into general | No change | 7dBm/1GHz into general | No change | Completed |
| NS_2011 |  |  |  |  | Completed |
| NS_2022 | -10dBm/100MHz (Harmonic) and +1dBm/200MHz | -10dBm/100MHz (Harmonic) and +1dBm/200MHz |  |  | Completed |
| NS_2033 |  | 1dBm/200MHz |  |  | Completed |
| NS_20X4 | -5 dBm/200 MHz | -5 dBm/200 MHz |  |  | To be introduced in TS 38.101-2 |
| NS_20Y5 | -10dBm/100MHz (Harmonic) and -5dBm/200MHz | -10dBm/100MHz (Harmonic) and -5dBm/200MHz |  |  | To be introduced in TS 38.101-2 |
| NOTE 1: NS_201 is Obsolate NOTE 2: Applicable before 01 January 2024 NOTE 3: Applicable before 01 September 2027 NOTE 4: Applicable after 01 September 2027 NOTE 5: Applicable after 01 January 2024 |  |  |  |  |  |

The main discussion conclusions in Rel-16 EESS are roughly summarized as below:

- Introduce new NS value for the legacy bands: n258 and n257, with modifiedMPR method, extending the applicable scope of modified MPR[4]

- Obsolete the requirement of -8dBm/200MHz in NS_201.

- RAN4 agreed to apply -5dBm/200MHz after Sep, 2027. But it is not reflected in RAN4 specification that revision is needed before the requirement taking effect, given that companies didn’t come to consensus.

- Whether it is mandatory of all NS values that one band supports, there was no conclusion. NS_203 is mandatory for EESS.

In Rel-19 EESS, to facilitate the mmWave industry maturity, the requirements that meet the latest UE emission demand from regulation, -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission from n258 and n257 need to be specified. We need to analyse the A-MPR requirements based on evaluations, and introduce new NS values considering all the power classes, given that NS_202 and NS_203 with +1dBm/200MHz do not meet the latest demand of regulation.

Observation 1: NS_202 and NS_203 with +1dBm/200MHz do not meet the latest demand of regulation

UE RF Requirements

initial evaluation is provided based on the simulation assumptions in [7] after the calibration referring to the PA model in [8]:

      - PA calibration point should follow current definition in Spec 38.101-2:

  - The waveform defined by BW = 100 MHz, SCS = 120 kHz, DFT-S-OFDM QPSK, 20RB23 is the reference waveform with 0 dB MPR and is used for the power class definition.

  - Calculate MPR as total backoff needed for 256QAM from this calibration point.

For band n258, to satisfy the RF requirements of EESS with -5dBm/200MHz additional spurious emission, SEM, ACLR, etc, the maximum power back-off is approximately 4.5dB for PC1, which is not higher than MPR or A-MPR corresponded to NS_202, even though the margin is taken into consideration.

Figure 1 Power back-off for band n258 to meet RF requirements including EESS

In the WF [9] approved in RAN4#112bis

<Way forward for NS signalling > Further analyse whether NS_202 can be updated by just replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection in the 23.6GHz-24GHz for band n257 and n258.

For NS_202, the requirement follows EU regulation. As shown in Table 6.5.3.2.3-1 of TS38.101-2, the current spurious emission requirement of EESS with +1dBm/200MHz is less stringent than -10dBm/100MHz.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

However, the frequency range of EESS may not be fully covered by 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band, given that the latter one doesn’t include the frequency ranges that are less than FOOB as illustrated in Figure 2.

Figure 2 The frequency range illustration for additional spurious emission (NS_202)

In Rel-16 spec 38.101-2 v16.2.0, before the NS_201 with additional spurious emission -8dBm/200MHz was obsoleted, the A-MPR specified for NS_201 was 9.0dB [6]. The A-MPR defined for NS_202 is higher than that for NS_201. In this case, the A-MPR requirement can be applied to Rel-19 EESS for band n258, if additional spurious emission is updated to -5dBm/200MHz for NS_202. If no A-MPR is introduced for NS_202, no need to specify new bit for modifiedMPRbehavior indication for NS_202.

Proposal 1: The A-MPR requirement can be applied to Rel-19 EESS for band n258, if additional spurious emission is updated to -5dBm/200MHz for NS_202.

Proposal 2: The updated requirement with -5dBm/200MHz additional spurious emission for NS_202 is needed for Rel-19. ModifiedMPR is not required for NS_202.

Release independence

In the WID [1], RAN4 may need to specify necessary performance requirements such as release independence in TS 38.307. If RAN4 have conclusions on introducing new NS values and/or new A-MPR for EESS spurious emission, or modifiedMPRbehavior indication for the possible newly introduced NS value, release independent is required. And we can consider release independence from Rel-16.

Observation 2: Release independence from Rel-16 can be considered when new requirements, including NS value and the corresponding A-MPR, modifiedMPRbehavior indication is introduced

For NS_202, if no A-MPR is introduced, no need to specify new bit for modifiedMPRbehavior indication for the NS value. Therefore, release independence for NS_202 is not necessary.

Proposal 3: Release independence for NS_202 is not necessary.

For band n257, if RAN4 specifies the frequency range that A-MPR is not applied, release independence from Rel-16 can be considered.

Proposal 4: For band n257, if RAN4 specifies the frequency range that A-MPR is not applied, release independence from Rel-16 can be considered.

Conclusion

In this contribution we discussed on FR2 EESS protection emission requirement, according to the analysis, we have the following observations and proposals:

Observation 1: NS_202 and NS_203 with +1dBm/200MHz do not meet the latest demand of regulation.

Proposal 1: The A-MPR requirement can be applied to Rel-19 EESS for band n258, if additional spurious emission is updated to -5dBm/200MHz for NS_202.

Proposal 2: The updated requirement with -5dBm/200MHz additional spurious emission for NS_202 is needed for Rel-19. ModifiedMPR is not required for NS_202.

For release independence:

Proposal 3: Release independence for NS_202 is not necessary.

Proposal 4: For band n257, if RAN4 specifies the frequency range that A-MPR is not applied, release independence from Rel-16 can be considered.

- References

[1] RP-242371, New WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE, RAN#105, September, 2024

[2] R4-2002729, WF on WRC-19 resolutions, NTT DOCOMO INC, RAN4#94e, Feb-Mar, 2020.

[3] R4-2005738, WF on WRC-19 outcome and impact on RAN4 specifications, NTT DOCOMO INC, RAN4#94e-bis, April, 2020.

[4] R4-2009141, WF on WRC-19 outcome and impact on RAN4 specifications, NTT DOCOMO INC, RAN4#95e, May, 2020.

[5] 38.817-01 v16.4.0.

[6] R4-1908150, On FR2 NS_201 AMPR, Qualcomm, RAN4#92

[7] R4-2416353, Discussion on Rel-19 EESS, Huawei, Hisilicon, RAN4#112bis

[8] R4-163314, Realistic power amplifier model for the New Radio evaluation, Nokia, RAN4#79

[9] R4-2417083, WF on UE RF requirements for EESS protection, Huawei, China Unicom, RAN4#112bis