3GPP TSG-RAN WG4 Meeting #113	R4-2417999

Orlando, US, 18 – 22 November, 2024

Source:	Nokia

Title:	HPUE MSD requirements

Agenda item:	6.1.1.1.1

Document for:	Approval

- 1		Introduction

In RAN4#111 Nokia presented aspects of captured requirements for MSD in PC3 [1,2] and showed in Annes A how many of the MSD values are carried forward from existing band combinations including those of errors. We tried in RAN4#112 to re-emphasize that some values are erroneous, but again there was little feedback. In this contribution we will base our proposals on the fact that no effort is made to re-discuss the MSD requirements errors, for which we now discuss how to base the HPUE requirements upon.

Furthermore, in previous RAN4 meeting a WF [3] was approved which had templates for how to define deltaMSD over PC3 to HPUE. In this contribution we continue discussion and propose the delta MSD values..

- 2	Discussion

## 2.1	1UL  ∆MSD for HPUE

We have focused on the existing HPUE MSD requirements for 1 uplink. Looking at the PC2 cases of uplink harmonic and harmonic mixing we identify the requirements as shown in the tables below. The test parameters are for PC2 and the PC3 test parameters are not shown. Only the lowest channel BW is used.

Table 1: MSD for uplink harmonic of PC2, PC3 and the ∆MSD

| UL | DL | BW | SCS | UL RB | DL BW | MSD PC2 | MSD PC3 | ∆MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n3 | n78 | 5 | 15 | 12 | 10 | 27.1 | 23.9 | 3.2 |
| n8 | n41 | 5 | 15 | 8 | 10 | 15.9 | 13 | 2.9 |
| n8 | n79 | 5 | 15 | 5 | 10 | 14.9 | 12 | 2.9 |
| n25 | n77 | 5 | 15 | 12 | 10 | 26.9 | 23.9 | 3 |
| n66 | n77 | 5 | 15 | 12 | 10 | 26.9 | 23.9 | 3 |
| n71 | n2510,11 | 5 | 15 | 8 | 5 | 9.4 | 6.9 | 2.5 |
| n71 | n41 | 5 | 15 | 6 | 10 | 13.6 | 10.8 | 2.8 |
| n71 | n77 | 5 | 15 | 5 | 10 | 13.2 | 10.4 | 2.8 |
| NOTE 10: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 11: Void. |  |  |  |  |  |  |  |  |

The ∆MSD for the cases shows in average a value of 2.9dB, and a maximum of 3.2dB. We propose PC2 MSD requirements for uplink harmonic are 3dB above the PC3 requirement.

The ∆MSD for the cases shows in average a value of 2.9dB, and a maximum of 3.2dB

PC2 MSD requirements for uplink harmonic are 3dB above the PC3 requirement.

Similar table for harmonic mixing is presented below where the PC2 cases are shown for UEs not supporting Tx diversity. Here the downlink bandwidths of 5MHz and 10MHz are used.

Table 2: MSD for harmonic mixing of UEs not supporting Tx diversity of PC2, PC3 and the ∆MSD

| UL | DL | BW | SCS | UL RB | DL BW | MSD | MSD PC3 | ∆MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 | n41 | 5 | 15 | 25 | 10 | 2.2 | 1.3 | 0.9 |
| n25 | n713 | 5 | 15 | 25 | 5 | 29.5 | 26.8 | 2.7 |
| n39 | n41 | 5 | 15 | 6 | 5 | 10.6 | 9.3 | 1.3 |
| n40 | n28 | 5 | 15 | 6 | 5 | 40.8 | 37.8 | 3 |
| n41 | n186 | 10 | 15 | 25 | 5 | 29.3 | 26.3 | 3 |
| n41 | n39 | 10 | 15 | 8 | 5 | 6.1 | 4.3 | 1.8 |
| n41 | n77 | 10 | 15 | 6 | 10 | 11.6 | 8.9 | 2.7 |
| n41 | n78 | 10 | 15 | 6 | 10 | 12.1 | 9.3 | 2.8 |
| n77 | n2 | 10 | 15 | 25 | 5 | 9.2 | 6.7 | 2.5 |
| n77 | n3 | 10 | 15 | 25 | 5 | 8.1 | 5.7 | 2.4 |
| n77 | n5 | 10 | 15 | 25 | 5 | 8.1 | 5.7 | 2.4 |
| n77 | n7 | 10 | 15 | 12 | 5 | 17.7 | 14.7 | 3 |
| n77 | n8 | 10 | 15 | 25 | 5 | 8.1 | 5.7 | 2.4 |
| n77 | n12 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n77 | n13 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n77 | n13 | 10 | 15 | 25 | 10 | 30.8 | 27.8 | 3 |
| n77 | n14 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n77 | n14 | 10 | 15 | 25 | 10 | 30.8 | 27.8 | 3 |
| n77 | n25 | 10 | 15 | 25 | 5 | 7.9 | 5.6 | 2.3 |
| n77 | n28 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n772 | n29 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n772 | n29 | 10 | 15 | 25 | 10 | 30.8 | 27.8 | 3 |
| n77 | n30 | 10 | 15 | 12 | 5 | 13.2 | 10.4 | 2.8 |
| n77 | n30 | 10 | 15 | 12 | 10 | 10.2 | 7.6 | 2.6 |
| n77 | n40 | 10 | 15 | 12 | 10 | 14.5 | 11.7 | 2.8 |
| n77 | n41 | 10 | 15 | 12 | 10 | 14.5 | 11.7 | 2.8 |
| n77 | n85 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n78 | n3 | 10 | 15 | 25 | 5 | 8.1 | 5.7 | 2.4 |
| n78 | n5 | 10 | 15 | 25 | 5 | 8.1 | 5.7 | 2.4 |
| n78 | n8 | 10 | 15 | 25 | 5 | 8.1 | 5.7 | 2.4 |
| n78 | n28 | 10 | 15 | 25 | 5 | 34 | 31 | 3 |
| n78 | n40 | 10 | 15 | 12 | 10 | 13.2 |  |  |
| n79 | n8 | 10 | 15 | 25 | 5 | 28 | 25 | 3 |
| NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 3: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWChannelHB/2≤fULHB≤FUL_highHB-BWChannelHB/2 with fULHB the UL carrier frequency and BWChannelHB the channel bandwidth configured in the higher band, both in MHz. NOTE 6: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. |  |  |  |  |  |  |  |  |

The ∆MSD for the cases shows in average a value of 2.6dB, and a maximum of 3dB. We propose PC2 MSD requirements for harmonic mixing for UEs not supporting Tx diversity are 3dB above the PC3 requirement.

The ∆MSD for the cases shows in average a value of 2.6dB, and a maximum of 3dB

PC2 MSD requirements for harmonic mixing for UEs not supporting Tx diversity are 3dB above the PC3 requirement.

For simplicity we see it advantageous to keep both requirements at 3dB, finding only one case of UH CA_n3A-n78A to be challenged from such simplification.

For PC2 MSD requirements for UEs supporting Tx diversity we find only two comparable cases CA_n25A-n41A and CA_n25A-n71A. These hint deltas of 1dB and 5dB, which is quite curious, so we would like to discuss this uplink configuration further and for now, not conclude on PC2 MSD requirements for harmonic mixing if the uplink is Tx Diversity.

For now, not conclude on PC2 MSD requirements for harmonic mixing when the uplink is Tx Diversity.

Looking at the PC2 cases of cross band isolation we identify the requirements as shown in the tables below:

Table 3: MSD for cross band isolation of PC2, PC3 and the ∆MSD

| UL | DL | BW | SCS | UL RB | DL BW | MSD PC2 | MSD PC3 | ∆MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n2 | n66 | 20 | 15 | 50 (RBstart=56) | 5 | 0.7 | 0 | 0.7 |
| n3 | n1 | 50 | 15 | 50 (RBstart=220) | 5 | 0.8 | 0 | 0.8 |
| n3 | n7 | 50 | 15 | 50 (RBstart=220) | 5 | 0.5 | 0 | 0.5 |
| n7 | n1 | 50 | 15 | 45 (RBstart=0) | 5 | 0.8 | 0 | 0.8 |
| n7 | n3 | 50 | 15 | 45 (RBstart=0) | 5 | 0.4 | 0.6 | -0.2 |
| n25 | n41 | 40 | 15 | 40 (RBstart=176) | 10 | 0.8 | 0 | 0.8 |
| n25 | n66 | 40 | 15 | 40 (RBstart=176) | 5 | 0.7 | 0 | 0.7 |
| n39 | n41 | 40 | 15 | 216 (RBstart=0) | 10 | 4.7 | 3.3 | 1.4 |
| n40 | n3 | 100 | 30 | 270 (RBstart=0) | 5 | 0.6 | 0 | 0.6 |
| n40 | n41 | 100 | 30 | 270 (RBstart=3) | 10 | 31.1 | 28.1 | 3 |
| n41 | n1 | 100 | 30 | 270 (RBstart=0) | 5 | 20.8 | 18.1 | 2.7 |
| n41 | n3 | 100 | 30 | 270 (RBstart=0) | 5 | 2.3 | 0.6 | 1.7 |
| n41 | n25 | 100 | 30 | 270 (RBstart=0) | 5 | 1.6 | 0.6 | 1 |
| n41 | n39 | 100 | 30 | 270 (RBstart=3) | 5 | 2.7 | 1.6 | 1.1 |
| n41 | n40 | 100 | 30 | 270 (RBstart=0) | 5 | 34.4 | 31.4 | 3 |
| n41 | n66 | 50 | 30 | 128 (RBstart=0) | 5 | 5.4 | 10.5 | -5.1 |
| n41 | n77 | 100 | 30 | 270 (RBstart=3) | 10 | 10.5 | 8.3 | 2.2 |
| n41 | n79 | 100 | 30 | 270 (RBstart=3) | 10 | 3.1 | 0 | 3.1 |
| n66 | n2 | 40 | 15 | 216 (RBstart=0) | 5 | 1.9 | 1.2 | 0.7 |
| n66 | n25 | 45 | 15 | 240 (RBstart=2) | 5 | 2.2 | 1.4 | 0.8 |
| n71 | n85 | 35 | 15 | 20 (RBstart=168) | 5 | 26 | 23 | 3 |
| n77 | n2 | 100 | 30 | 270 (RBstart=0) | 5 | 1 | 0 | 1 |
| n77 | n7 | 100 | 30 | 270 (RBstart=0) | 5 | 6.5 | 4.5 | 2 |
| n77 | n25 | 100 | 30 | 270 (RBstart=0) | 5 | 1 | 0 | 1 |
| n77 | n30 | 100 | 30 | 270 (RBstart=0) | 5 | 1 | 0 | 1 |
| n77 | n401 | 100 | 30 | 270 (RBstart=0) | 10 | 6.5 | 4.5 | 2 |
| n77 | n411 | 100 | 30 | 270 (RBstart=0) | 10 | 6.5 | 4.5 | 2 |
| n78 | n7 | 100 | 30 | 270 (RBstart=0) | 5 | 6.5 | 4.5 | 2 |
| n783 | n79 | 100 | 30 | 270 (RBstart=3) | 10 | 5 | 2 | 3 |
| n79 | n41 | 100 | 30 | 270 (RBstart=0) | 10 | 3.5 | 0 | 3.5 |
| n79 | n783 | 100 | 30 | 270 (RBstart=0) | 10 | 5.6 | 2.6 | 3 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |

Except for two combinations the tendency is that if the MSD is not defined in PC3, but the impact is present in PC2 the MSD value of 1 will serve. For the cases where the PC3 MSD is defined we see large variations in MSD, even negative (CA_n41A-n66A). Since Nokia has tried to get focus on issues of existing MSD values with no luck, we see little effect in making an attempt to fix cross band isolation either and therefore present the HPUE simplification for MSD for cross band isolation. If the band combination has PC3 MSD cross band isolation MSD add 3dB for PC2 and if the band combination is subject to cross band isolation only in PC2 allow 1dB MSD.

If the band combination has PC3 MSD cross band isolation MSD add 3dB for PC2 and if the band combination is subject to cross band isolation only in PC2 allow 1dB MSD.

## 2.2	2UL ∆MSD for HPUE

For the 2UL cases we re-produce our proposal from [4]

Table 4: deltaIMD requirements for PC2.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 8dB | IMD4 | 10dB | IMD2 | 6.5dB |
| IMD3 same b.gr | 0dB | IMD5 same b.gr | 10dB | IMD3 same b.gr | 2dB |
| IMD3 diff. b.gr | 0dB | IMD5 diff. b.gr | 5dB | IMD3 diff b.gr. | 5dB |
| IMD4 | 10dB | IMD7 same b.gr | 10dB | IMD4 | 10dB |
| IMD5 | 8dB | IMD7 diff. b.gr | 9dB | IMD5 | 15dB |
|  |  | IMD9 | 0dB | IMD7 |  |

Table 5: deltaIMD requirements for PC1.5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 7.5dB | IMD4 | N/A | IMD2 | N/A |
| IMD3 same b.gr | N/A | IMD5 same b.gr | N/A | IMD3 same b.gr | N/A |
| IMD3 diff. b.gr | N/A | IMD5 diff. b.gr | N/A | IMD3 diff b.gr. | N/A |
| IMD4 | 18dB | IMD7 same b.gr | N/A | IMD4 | N/A |
| IMD5 | 25dB | IMD7 diff. b.gr | N/A | IMD5 | N/A |
|  |  | IMD9 | N/A | IMD7 | N/A |

For 2UL ∆MSD for HPUE we proposed values from table 4 and table 5

- 3		Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: The ∆MSD for the cases shows in average a value of 2.9dB, and a maximum of 3.2dB

Proposal 1: PC2 MSD requirements for uplink harmonic are 3dB above the PC3 requirement.

Observation 2: The ∆MSD for the cases shows in average a value of 2.6dB, and a maximum of 3dB

Proposal 2: PC2 MSD requirements for harmonic mixing for UEs not supporting Tx diversity are 3dB above the PC3 requirement.

Proposal 3: For now, not conclude on PC2 MSD requirements for harmonic mixing when the uplink is Tx Diversity.

Proposal 4: If the band combination has PC3 MSD cross band isolation MSD add 3dB for PC2 and if the band combination is subject to cross band isolation only in PC2 allow 1dB MSD.

Proposal 5: For 2UL ∆MSD for HPUE we proposed values from table 4 and table 5

Table 4: IMD requirements for PC2.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 8dB | IMD4 | 10dB | IMD2 | 6.5dB |
| IMD3 same b.gr | 0dB | IMD5 same b.gr | 10dB | IMD3 same b.gr | 2dB |
| IMD3 diff. b.gr | 0dB | IMD5 diff. b.gr | 5dB | IMD3 diff b.gr. | 5dB |
| IMD4 | 10dB | IMD7 same b.gr | 10dB | IMD4 | 10dB |
| IMD5 | 8dB | IMD7 diff. b.gr | 9dB | IMD5 | 15dB |
|  |  | IMD9 | 0dB | IMD7 |  |

Table 5: IMD requirements for PC1.5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 7.5dB | IMD4 | N/A | IMD2 | N/A |
| IMD3 same b.gr | N/A | IMD5 same b.gr | N/A | IMD3 same b.gr | N/A |
| IMD3 diff. b.gr | N/A | IMD5 diff. b.gr | N/A | IMD3 diff b.gr. | N/A |
| IMD4 | 18dB | IMD7 same b.gr | N/A | IMD4 | N/A |
| IMD5 | 25dB | IMD7 diff. b.gr | N/A | IMD5 | N/A |
|  |  | IMD9 | N/A | IMD7 | N/A |

- 4		References

- R4-2409190, On UE RF specifications table improvement, Nokia

- R4-2413338, On UE RF specifications table improvement, Nokia

- R4-2417098, WF on HPUE RF requirements, RAN4#112bis, Samsung, Skyworks, Nokia

- R4-2415392, On UE RF specifications table improvement, RAN4#112bis, Nokia