3GPP TSG-RAN WG4 Meeting #112bis	R4-2418002

Orlando, USA, 18th - 2nd , November, 2024

Title: 	UE RF requirements for Type 4a/4b capable FWA UE for EN-DC/NR-CA

Agenda Item:	7.5.2.1

Source: 	Nokia

Document for:	Approval

# 1		Introduction

In previous RAN4 meetings issue on how to capture the power imbalance and REFSENS requirements has been suspended to wait progress on other issues and will be discuss further in big CR.

In this contribution we present our preference how the coming CR should be crafted, and proposal is based on [1].

# 2		Discussion

There are CRs in this RAN4 meeting to add UE capability type information into a dedicated table. If those CRs are agreed then thi UE type information should be used also in power imbalance requirement table.

Proposal 1: Use table 1 a as basis for Rel-19 Power imbalance parameters for intra-band non-contiguous CA CR, similar approach for EN-DC would be needed.

Table 1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void. NOTE 6: ΔRIB,4R is the reference sensitivity allowance as specified in Table 7.3.2-2. |  |  |  |  |

Another issue which is not yet discussed is that as there are also CR for earlier releases to introduce the UE capability info table then above table should have the type 2 reference in pre-Rel19 specifications.

Proposal 2: For pre Rel-19 specifications term “type 2” is introduced into Power imbalance parameters table both in NR and EN-DC specifications.

# 3		Conclusion

In this contribution we have discussed how to introduce Rel-19 Power imbalance parameters for intra-band non-contiguous CA into coming CR.

Proposal 1: Use table 1 a as basis for Rel-19 Power imbalance parameters for intra-band non-contiguous CA CR, similar approach for EN-DC would be needed.

Table 1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void. NOTE 6: ΔRIB,4R is the reference sensitivity allowance as specified in Table 7.3.2-2. |  |  |  |  |

Proposal 2: For pre Rel-19 specifications term “type 2” is introduced into Power imbalance parameters table both in NR and EN-DC specifications.

# 4		References

[1] R4-2415372, UE RF requirements for Type 4a/4b capable FWA UE for EN-DC/NR-CA, RAN4#112bis