3GPP TSG RAN WG4 #113	R4-2418016

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.11.4.2

Source:	Nokia

Title:	On BS OTA test scope reduction

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#110bis regarding the performance part of OTA test enhancements. Listed in the WF is the work plan for the work item. According to the schedule, for RAN4#113, discussion is to be started on the BS OTA test scope reduction. This contribution further elaborates Nokia’s views on the topic.

# Discussion

## BS OTA test scope reduction

It was agreed in RAN4#110bis:

| RAN4#113 (11/2024) | Core: - Continue to discuss the applicability of existing co-location reference antenna and identify the problems if any. Perf: - Target to finalize conformance testing requirement for EIRP mask for U6GHz and both TPs to TR and CRs to TS 38.141-2. - start to identify BS excessive OTA test scope cases and try to reduce it with the aim to shorten the test duration time for selected RF requirements - start to discuss how to simplify BS TRP test methods for FR1 and FR2 by improving the applicability. - Start to discuss the conformance testing requirement for co-location reference antenna if any |
| --- | --- |

There is a clear need to reduce the test scope that is required by OTA related testing. It has been agreed in the work plan, that this could be done by identifying BS excessive OTA test scope cases and try to reduce it with the aim to shorten the test duration time for selected RF requirements. The following test cases have been mentioned during discussion:

- TX IM

- RX out of band blocking

- Other reduction possibilities are not precluded

We agree that there is need for reducing the test scope for these requirements and one way to start to progress here would be to discuss the measurement steps and the used bandwidths. One thing to be noted here is that both of the listed requirements are co-location requirements and there should be effort put in the whole definition of co-location requirements for higher frequencies together with the discussion of excessive test cases.

### OTA Transmitter Intermodulation (TX IM)

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement applies during the transmitter ON period and the transmitter transient period.

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station to below specified levels.

The transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission in clause 6.7.5 (except co-location with other base stations), OTA out-of-band emissions in clause 6.7.4 and OTA ACLR 6.7.3. Naturally due to TRP test method and CLTA usage, execution of these cases requires quite a much of test effort.

Currently there are 6 interferer frequency offsets in test case, 3 on each side of the carrier defined as the following:

, for n=1, 2 and 3

Number of the offsets should be reconsidered which would effectively impact the test efforts while still keeping the requirement coverage.

Currently, for TX IM there are 6 interferer offset, 3 on each side of the carrier.

To reconsider some of the interferer offsets while still keeping the requirement coverage.

### OTA out-of-band blocking

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

The OTA general blocking interferer is injected via test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz for FR1. Whilst for FR2, 1 MHz step size applies for frequency range 30 MHz to 6000 MHz, and above 6000MHz bigger step sizes is defined.

The OTA co-location blocking interferer for FR1 is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands related to co-located systems

Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

Out of all test requirements of 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts. This is largely due to 1MHz step size used and, therefore, the number of measurements points is significantly large. Increasement of step size from 1 MHz to X MHz effects test efforts proportionally.

Increasement of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

To increase interferer step size of 1 MHz to X MHz, where X is FFS.

# Conclusion

This document shares Nokia’s views on simplification of BS TRP test methods. Following observations and proposals were introduced:

- Currently, for TX IM there are 6 interferer offset, 3 on each side of the carrier.

- Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

- Increasement of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

- To reconsider some of the interferer offsets while still keeping the requirement coverage.

To increase interferer step size of 1 MHz to X MHz, where X is FFS.

# References

- R4-2406088 “Way Forward for [110bis][311] NR_BS_RF”, RAN4#110bis, Changsha, China 15th – 19th April, 2024