3GPP TSG RAN WG4 #113	R4-2418020

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.20.2.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. The requirement with the biggest impact is the RX in-band blocking. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation and emphasizes the importance of setting correctly the RX in-band blocking requirement to enable functionality in real-life deployment.

# Discussion

## In-band blocking simulations

RAN4#112bis concluded with the following agreement and way forward related to the in-band blocking requirements:

Agreement: - For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. WF: - FFS on the followings: - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined? - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel. - Option 2: separate requirements are defined, - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - For FR1 WA BS co-existence study, FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - In-band blocking requirement for FR1 MR SBFD BS - In-band blocking requirement for FR2-1 WA and MR SBFD BS - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

### In-band blocking requirements definition

After RAN4#112-bis, companies are asked to decide over 2 alternatives for the in-band blocking requirements definition. In Option 1, a single in-band blocking requirement is defined for scenarios where SBFD coexists with a TDD deployment in the adjacent channel and scenarios where SBFD coexist with another SBFD deployment in the adjacent channel. On the other hand, Option 2 proposes to have different requirements for the 2 scenarios mentioned above. This discussion originates from the open issue on existing BS RF Tx requirements for SBFD operation. From the Tx side, companies are still to agree on whether to support tighter ACLR and OBUE requirements for the case of SBFD BS coexisting with SBFD BS. As also pointed out in our companion Tdoc [2], there is no need to specify additional/tighter requirements for such scenario and, if needed, those enhanced BS capabilities could be met on a customer demands basis. Moreover, this would require additional discussions on the feasibility of the mechanisms to achieve such enhanced performance, e.g. the implementation of sub-band RF filtering in the transmitter side. Therefore, we don’t support having tighter BS requirements and, thus, we should not expect any differences on the in-band blocking requirements depending on whether the SBFD BS coexists with TDD BS or SBFD BS. In conclusion, our preference is defining a unified in-band blocking requirement that covers all possible deployments cases.

RAN4 to define a unified in-band blocking requirement to cover both the cases where SBFD BS coexists in the adjacent channel with TDD and SBFD base stations

### Clarification on how the in-band blocking requirement derivation

In the previous RAN4#112 meeting, companies agreed to not account for the SBFD specific co-channel interferences to derive the in-band blocking requirement. In other words, BS self-interference and inter-sector interferences are not considered. However, these interferences also impact the receiver performance since they are also received by the BS LNA. This is an important assumption that should be clearly stated as part of the in-band blocking requirement definition for SBFD-capable BS in order to avoid possible misunderstandings.

RAN4 specifications should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

### ACLR and ACS effects

One important aspect for the in-band blocking power derivation is whether effects such as ACLR and ACS effects should be included. As described in TR 38.858, Clause E.2.3.1 the DL adjacent channel interference for the in-band blocking requirement should be measured following the definition of IACI, and it is explicitly mentioned that it does not include the “frequency isolation” aspects. i.e., the ACLR and ACS effects.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

### Grid shift assumptions

Another open FFS topic is the grid shift assumptions for urban macro FR1 simulations, which are used to derive the FR1 WA BS in-band blocking requirements. In [3], we include a comparison of the in-band blocking power at the SBFD BS for different grid shift assumptions, particularly 10%, 50% and 100%. It can be noted that the grid shift assumption is quite important for the derivation of the in-band blocking requirements. It can also be observed that, among the considered grid-shift values, 10% grid-shift results in the worst-case scenario from an in-band blocking perspective since it results in the highest adjacent channel interference. For instance, the relative in-band blocking power difference between 10% grid shift and 100% grid shift exceeds 10 dB. Thus, a SBFD BS that fulfils the in-band blocking requirement based on the 10% grid-shift would be automatically compliant for any of the other considered grid-shift values such as 20%, 50% or 100%. However, if the in-band blocking requirement is derived from, e.g., a grid shift of 50%, the SBFD operation for 10% grid-shift would be compromised since the interfering signal power would be higher than the used to determine the in-band blocking requirement. Note that site-sharing is commonly used when deploying base stations from different operators, and 10% grid shift is what closest resembles the site-sharing scenario.

Results show that the smaller is the grid-shift, the larger is the impact of the adjacent channel interference to the in-band blocking power.

Only consider 10% grid-shift to derive the interfering signal power for the in-band blocking requirement.

### Analysis of the in-band blocking simulation results

A summary of the results shown in [3] can be found in the following tables. Table 1 shows the 99th percentile of the received in-band blocking power at the FR1 WA and MR SBFD BS. Note that after our analysis of grid shift above, we only focus on scenarios with grid shift of 10%.

Table 1. In-band blocking power [dBm] for FR1 scenarios

|  | In-band blocking power at 99th percentile | Current in-band blocking power requirement in TS 38.104 |
| --- | --- | --- |
| Scenario 1 (Urban macro FR1) | -2.5 dBm | FR1 WA BS: -43 dBm |
| Scenario 3 (Urban micro FR1) | -4.3 dBm | FR1 MR BS: -38 dBm |

Based on our results, we can observe that the current in-band blocking power requirements for FR1 WA and FR1 MR BS can’t be re-used for FR1 SBFD BS.

For FR1 WA SBFD BS, the in-band blocking power at the 99th percentile is -2.5 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

For FR1 MR SBFD BS, the in-band blocking power at the 99th percentile is -4.3 dBm, which is approximately 34 dB higher than the current in-band blocking requirement.

Table 2 collects the in-band blocking simulation results from all companies that submitted results to RAN4#112-bis for the FR1 WA BS. A total of 7 companies provided results, which we believe is enough to draw solid observations. The table additionally includes the average across the different companies. It can be noted that the resulting average in-band blocking requirement is much higher than the existing FR1 WA BS in-band blocking requirements (-43 dBm). Therefore, we propose to introduce a new requirement for FR1 WA SBFD BS. This new in-band blocking requirement should be around -2 dBm.

Table 2. Collection of in-band blocking results for FR1 WA BS at 10% grid shift

| Company | Blocking power at 99th percentile (10% grid shift, FR1 WA BS) |
| --- | --- |
| CATT | 0.33 dBm |
| Ericsson | -4.7 dBm |
| Huawei | -5.1 dBm |
| Samsung | -4.39 dBm |
| Tejas Network | -0.82 dBm |
| ZTE | -1.5067 dBm |
| Nokia | -2.5 dBm |
| Average | -2.67 dBm |

Based on companies’ simulation results, the existing FR1 WA BS in-band blocking requirement defined in TS 38.104 can’t be re-used for FR1 WA SBFD BS.

Introduce a new in-band blocking requirement for FR1 WA SBFD BS of -2 dBm.

Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm.

Table 3 includes absolute in-band blocking power and the ratio of in-band blocking power versus wanted signal power at 99th percentile for the FR2-1 WA SBFD BS at 10% grid shift.

Table 3. Ratio of in-band blocking power and wanted signal [dBm] for FR2-1 scenarios

|  | In-band blocking power at 99th percentile | Ratio of in-band blocking power versus wanted signal at 99th percentile | Current ratio of in-band blocking power versus wanted signal in TS 38.104 |
| --- | --- | --- | --- |
| Scenario 6 (Urban macro FR2) | -37.4 dBm | 30 dB | FR2-1 WA BS: [27 - 30] dB |

Based on the results above the following is observed:

For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

We think that there is no need to define new in-band blocking requirements for FR2-1 WA SBFD BS given that the obtained ratio of in-band blocking power versus wanted signal is within the current FR2-1 WA BS requirements.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

## OTA sensitivity degradation

The following was agreed in RAN4#112bis:

WF: - FFS on the followings: - Option 1: 1.0 dB degradation - Option 2: 0.5 dB degradation

Even though SBFD operates in non-overlapping mode in frequency, it is still transmitting and receiving simultaneously in full duplex mode in time domain. This makes it fundamentally different from TDD deployment and the differences in the actual system on the field should be taken into careful consideration. It can be argued that the self-interference plays a major role in sensitivity degradation but nevertheless the effect of co-site inter-sector and inter-site interference cannot be neglected. They are much more difficult to cancel than self-interference, which might be easier to cancel up to some extent. As the SBFD technique is all about improving the UL performance, it is of paramount importance to not sacrifice too much sensitivity when deploying SBFD systems. As sectorized setup for co-site inter-sector interference and inter-site interference from other gNB’s is impossible to realize in the OTA testing, the agreement was made to measure OTA sensitivity only with self-interference. But as noted before, the impact of other interferences is not reflected in the test, and it might lead to unacceptable level of desensitization of the receiver when deployed on the field. Therefore, we propose to limit the OTA sensitivity degradation to maximum of 0.5dB.

OTA sensitivity test only accounts for self-interference.

Other interferences still exist in real-life deployments.

SBFD benefits lie in UL improvements and sensitivity should not be overly degraded.

OTA sensitivity degradation should be limited to maximum of 0.5dB.

## Dynamic Range

The following was agreed in RAN4#112bis:

WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: Dynamic range of FR1/FR2-1 indoor scenario can reuse the existing requirement.

For SBFD, the BS IoT level will be influenced by the RSI capability of the base station as well level of interference from other sectors and neighbouring base stations. Nevertheless, based on our analysis we think 20 dB (Option 2) provides sufficient margin to cover these different sources of interference.

For receiver dynamic range requirement, adopt Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

## ACS Requirement

The following was agreed in RAN4#112bis:

Agreement: - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. WF: - FFS on the followings: - If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed. - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

We don’t see the need for co-location ACS requirement. The same reasons apply as for the co-location ACLR and OBUE requirement. There is no need for 3GPP to define any additional requirements for co-location ACS. If one wants to deploy SBFD system, they should have proprietary customer requirements to satisfy the co-location requirements. We also disagree to add the text in the note proposed in option 1. Rather than stating that ACS requirement is not applied to co-location scenario, it should be stated that current requirements do not enable co-location scenario.

No need for RAN4 to specify additional co-location ACS requirement.

## Necessity of New RX intermodulation requirement with 1 interfering signal

The following was agreed in RAN4#112bis:

WF: - FFS on the followings: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to study further RX intermodulation scenario with 1 interfering signal.

We think that the proposed new RX intermodulation scenario with 1 interfering signal might be a possible scenario, and it should be studied further. We must be very careful when defining RF requirements and especially reusing existing ones. The differences with legacy TDD and SBFD systems are imminent and might end up causing disastrous effects in the field in the worst case. As the power of the own TX sub-band signal (self-interference) could be significantly higher than what is currently assumed in the general RX IMD requirement (-52dBm for WA BS), this could lead to situation where the current RX IMD requirement is inadequate and doesn’t guarantee the linearity of RX is enough. If one would take the values companies calculated for the self-interference signal in gNB TX subband, measured at the input of LNA, the range varied from -23dBm to -56dBm. Additional robustness to RX would be highly welcomed to ensure functionality in this situation. The SBFD RX blocking requirement might not capture this implicitly, unless the own TX is on during the test. Therefore, we think additional RX IMD requirement with self-interference (TX on) and one interfering signal is needed. If self-interference can be cancelled to reasonable level, this should not be an issue for SBFD-capable BS.

Careful consideration is needed to evaluate requirements.

The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

## Necessity of New RX intermodulation requirement with 2 interfering signals

The following was agreed in RAN4#112bis:

WF: - FFS on the following: - Option 1: - For the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - For receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: New IMD requirement with 2 interfering signal scenario is not needed. - Option 3: RAN4 to study further

We feel that the scenario with one interfering signal together with self-interference (TX on) is enough to cover the above mentioned scenario and there is no need to define both for SBFD-capable BS. Self-interference is anyhow the strongest interfering signal and co-existence of SBFD-capable BS with other BS is anyhow not feasible without significant grid shift.

No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. There were observations and proposals as listed below.

- Results show that the smaller is the grid-shift, the larger is the impact of the adjacent channel interference to the in-band blocking power.

- For FR1 WA SBFD BS, the in-band blocking power at the 99th percentile is -2.5 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

- For FR1 MR SBFD BS, the in-band blocking power at the 99th percentile is -4.3 dBm, which is approximately 34 dB higher than the current in-band blocking requirement.

- For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

- OTA sensitivity test only accounts for self-interference.

- Other interferences still exist in real-life deployments.

- SBFD benefits lie in UL improvements and sensitivity should not be overly degraded.

- Careful consideration is needed to evaluate requirements.

- The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

- RAN4 to define a unified in-band blocking requirement to cover both the cases where SBFD BS coexists in the adjacent channel with TDD and SBFD base stations.

RAN4 specifications should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

Only consider 10% grid-shift to derive the interfering signal power for the in-band blocking requirement.

Based on companies’ simulation results, the existing FR1 WA BS in-band blocking requirement defined in TS 38.104 can’t be re-used for FR1 WA SBFD BS.

Introduce a new in-band blocking requirement for FR1 WA SBFD BS of -2 dBm.

Introduce a new in-band blocking requirement for FR1 MR SBFD BS of -4 dBm.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

OTA sensitivity degradation should be limited to maximum of 0.5dB.

For receiver dynamic range requirement, adopt Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

No need for RAN4 to specify additional co-location ACS requirement.

RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined.

# References

- R4-2416585 “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, RAN4#112bis, Hefei, China, 14th-18th October, 2024

- R4-2418022 “On existing BS RF TX requirements for SBFD operation” RAN4#113

- R4-2418021, “SBFD RX in-band blocking simulation results”, Nokia, RAN4#113