3GPP TSG RAN WG4 #113	R4-2418021

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.20.2.3

Source:	Nokia

Title:	SBFD RX in-band blocking simulation results

Document for:		Discussion

# Introduction

A Way Forward was agreed in RAN4#112bis [1] regarding evolution of NR duplex operation. This contribution includes the system-level simulation results for the derivation of the in-band blocking requirements for FR1 WA, FR1 MR, FR2-1 WA and FR2-1 MR SBFD base stations. The analysis and conclusions derived from the results shown below can be found in our Tdoc on existing BS RF Rx requirements for SBFD operation [2].

# In-band blocking simulations

RAN4#112bis agreed on the following agreements and way forward:

Agreement: - For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. WF: - FFS on the followings: - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined? - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel. - Option 2: separate requirements are defined, - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - For FR1 WA BS co-existence study, FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - In-band blocking requirement for FR1 MR SBFD BS - In-band blocking requirement for FR2-1 WA and MR SBFD BS - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

## General assumptions and simulated scenarios

As indicated in the agreements and WF from RAN4#112-bis, the in-band blocking requirements for FR1 WA and FR1 MR SBFD BS as well as FR2-1 WA and FR2-1 MR SBFD BS are still to be defined. The simulated scenarios for deriving the in-band blocking requirements for each of the aforementioned BS types are Scenario 1, Scenario 5, Scenario 6 and Scenario 8, respectively. The coexistence case used to derive the in-band blocking is Case 3, where a SBFD network coexists in the adjacent channel with a TDD DL network.

Table E.1-1: Adjacent channel co-existence scenarios (Annex E, TR 38.858) Scenario; FR; Aggressor; Victim; SBFD BS type. 1; FR1; Urban Macro; Urban Macro; FR1 WA. 2; FR1; Urban Hotspot; Urban Hotspot; FR1 WA. 3; FR1; Indoor; Indoor; FR1 LA. 4; FR1; Urban Macro; Micro; FR1 MR. 5; FR1; Micro; Micro; FR1 MR. 6; FR2-1; Urban Macro; Urban Macro; FR2-1 WA. 7; FR2-1; Urban Hotspot; Urban Hotspot; FR2-1 WA. 8; FR2-1; Urban Dense; Urban Dense; FR2-1 MR. 9; FR2-1; Indoor; Indoor; FR2-1 LA. Table E.1-2: Adjacent channel co-existence cases (Annex E, TR 38.858) Case; Aggressor; Aggressor baseline; Victim; Slot allocation Aggressor Victim. 1; SBFD; TDD DL; TDD DL. 2; SBFD; TDD UL; TDD UL. 3; TDD DL; No aggressor network; SBFD. 4; TDD UL; No aggressor network; SBFD.

In the following sections we present our system-level simulation results for Scenario 1, Scenario 5 and Scenario 6. Unless explicitly mentioned, we assume the “baseline” or “1st priority” assumptions captured in Annex E assumptions, e.g., we selected BS transmit power of 49 dBm and SBFD antenna configuration 2 for the Urban Macro FR1 simulations.

## Scenario 1: FR1, Urban Macro -> Urban Macro

Given the FFS on the grid shift assumptions for this scenario, in Figure 1 we present the CDF of the in-band blocking power measured at the SBFD BS for 3 different grid shifts assumptions, namely, 10%, 50% and 100%. Table 1 below shows the corresponding 99th percentile for each of the simulated cases.

Figure 1. In-band blocking power at SBFD BS for different grid shift assumptions (Scenario 1)

Table 1. In-band blocking power at 99th percentile for Scenario 1

|  | 10% grid shift | 50% grid shift | 100% grid shift |
| --- | --- | --- | --- |
| In-band blocking power at 99th percentile | -2.2 dBm | -10.4 dBm | -16.7 dBm |

## Scenario 5: FR1, Micro -> Micro

We include in the following the results for Scenario 5 which can be used to derive the FR1 MR BS in-band blocking requirements. As also done in the FR1 WA BS, we simulated 3 different grid shift assumptions (10%, 50%, 100%).

Figure 2. In-band blocking power at SBFD BS for different grid shift assumptions (Scenario 5)

Table 2. In-band blocking power at 99th percentile for Scenario 5

|  | 10% grid shift | 50% grid shift | 100% grid shift |
| --- | --- | --- | --- |
| In-band blocking power at 99th percentile | -4.2 dBm | -19.6 dBm | -24.6 dBm |

## Scenario 6: FR2-1, Urban Macro -> Urban Macro

For the FR2-1 BS in-band blocking requirements, we provide both the absolute power of the received in-band blocking (Figure 3) as well as the ratio of the in-band blocking versus the wanted signal powers (Figure 4). Additionally, the values of both metrics at the 99th percentile are captured in Table 3.

Figure 3. In-band blocking power at SBFD BS for different grid shift assumptions (Scenario 6)

Figure 4. Ratio of in-band blocking power versus wanted signal power at SBFD BS for different grid shift assumptions (Scenario 6)

Table 3. In-band blocking power and ratio of in-band blocking power versus wanted signal power at 99th percentile for Scenario 6

|  | 10% grid shift | 50% grid shift | 100% grid shift |
| --- | --- | --- | --- |
| In-band blocking power at 99th percentile | -37.4 dBm | -49.2 dBm | -49.6 dBm |
| Ratio of in-band blocking power versus wanted signal at 99th percentile | 30 dB | 18 dB | 18 dB |

# References

- R4-2416585 “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, RAN4#113, Hefei, China, 14th-18th October, 2024

- R4-2418020, “On existing BS RF RX requirements for SBFD operation”, Nokia, RAN#113