3GPP TSG-RAN WG4 Meeting #113							R4-2418029

Orlando, US, 18th – 22nd November, 2024

Agenda Item:  	7.9.1
Source: 			Intelsat
Title: 			Ku Band Link Budget Analysis    
Document for: 	Discussion and Decision

# Introduction

At RAN #104, the WID in [1] was approved to commence work to introduce the Ku Band to the 3GPP specifications for NTN. As described in the WID, the Ku Band frequency ranges from 10.7 GHz to 14.5 GHz which is between FR1 and FR2. Consequently, RAN4 working group was tasked to select the appropriate Numerology between FR1 or FR2.

From Satellite operator’s perspective, it is important to be able to provide reliable, stable, and high data rate service to customers. This in turn translate to efficient use of the scares radio resources, bandwidth, and the appropriate numerology for various services. Furthermore, with few simple tweaks legacy transparent satellite architecture, operating proprietary systems, can migrate to 5G services, which makes the selection of the appropriate numerology, FR1 or FR2, even more prominent.

A few key technical factors that will influence the decision on whether to select FR1 or FR2 for Ku Band have been identified, namely:

- Uplink timing synchronization,

- Phase Noise,

- Efficient channel utilization,

- Beam hopping granularity,

- Link budget UL/DL as a function of Subcarrier Spacing (SCS) and channel bandwidth.

The goal of this document is to address the link budget and the associated data rate (throughput) of UL/DL as a function of SCS and channel bandwidth.

For 5G, the key parameter for the UL/DL link budget is the resource block which is a function of the SCS and channel bandwidth.  The link budget produces the required Carrier to Noise Ratio (CNR) at the receiver which subsequently maps to data rate for that communication link.

In essence, the link budget and the data rate are inter-related. The approach in this document is to begin with the resource block for each SCS and channel bandwidth and analytically calculate the data rate for each scenario.  Furthermore, we examine the data rate for each SCS and channel bandwidth and make observation regarding the trends.

# Resource Block and Payload Bandwidth

Following Table 1 and Table 2 provide the FR1 and FR2 resource blocks for different subcarrier spacing and channel bandwidth as defined in 3GPP spec.

Table 1: FR1 Resource Block.

|  |  | FR1 Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| 0 | 15 kHz | 52 | 106 | 133 | 188 | 270 |  |  |
| 1 | 30 kHz | 24 | 51 | 65 | 92 | 133 | 189 | 273 |
| 2 | 60 kHz | 11 | 24 | 31 | 44 | 65 | 93 | 135 |

Table 2: FR2 Resource Block.

|  |  | FR2 Channel Bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Numerology (µ) | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 2 | 60 kHz | 66 | 132 | 264 |  |
| 3 | 120 kHz | 32 | 66 | 132 | 264 |

Table 3 and Table 4 provide the FR1 and FR2 payload bandwidth for different subcarrier spacing and channel bandwidth as defined in 3GPP specification.

It is evident that at the payload bandwidth decreases with increasing SCS for the same channel bandwidth.

Table 3: FR1 Payload Bandwidth.

|  |  | FR1 Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| 0 | 15 kHz | 9.375 MHz | 19.095 MHz | 23.955 MHz | 33.855 MHz | 48.615 MHz |  |  |
| 1 | 30 kHz | 8.67 MHz | 18.39 MHz | 23.43 MHz | 33.15 MHz | 47.91 MHz | 68.07 MHz | 98.31 MHz |
| 2 | 60 kHz | 7.98 MHz | 17.34 MHz | 22.38 MHz | 31.74 MHz | 46.86 MHz | 67.02 MHz | 97.26 MHz |

Table 4: FR2 Payload Bandwidth.

|  |  | FR2 Channel Bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 2 | 60 kHz | 47.58 MHz | 95.1 MHz | 190.14 MHz |  |
| 3 | 120 kHz | 46.2 MHz | 95.16 MHz | 190.2 MHz | 380.28 MHz |

In the proceeding section the FR1 and FR2 Numerologies are used for the calculations.

# Data Rate Formula

For 5G, the approximate data rate when only one component carrier is in use is computed as follows as defined in 3GPP TS 38.306 [2]:

Data rate (in Mbps) =vLayers.Qm.f.R NPRBBW,u*12Tsu(1-OH)    Equation 1

- R is the coding rate.

- vLayers is the maximum number of layers.

- Qmis the maximum modulation order.

- f is the scaling factor and can take the values 1, 0.8, 0.75, and 0.4.

- μ is the numerology (as defined in TS 38.211).

- Tsμis the average OFDM symbol during in a subframe for numerology μ, i.e. Tsμ=10-314*2μ, assuming normal cyclic prefix.

- NPRBBW,μthe maximum Resource Block (RB) allocation in UE supported maximum bandwidth BW in a given band with numerology μ.

- OH is the overhead which is different for UL versus DL

For our exercise, assume:

- R =553/1024

- vLayers=1

- Qm=4

- f=1

- μ depends on the SCS as given in  REF _Ref181181386 \h Table 1 and Table 2

- NPRBBW,μ is given in  REF _Ref181181386 \h Table 1 and  REF _Ref181181394 \h Table 2 for FR1 and FR2

- OH the overhead is 0.18 for DL and 0.14 for UL  (assume FR1 like)

# Data Rate Calculation

Using the data rate formula given in previous Section, Equation , the DL data rate for FR1 and FR2 are calculated and presented in following Table 5 and Table 6, respectively.

Table 5: FR1 Downlink Data Rate (Mbps).

|  |  | FR1 Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz |
| 0 | 15 kHz | 15.5 Mbps | 31.5 Mbps | 39.6 Mbps | 55.9 Mbps | 80.3 Mbps |  |  |
| 1 | 30 kHz | 14.3 Mbps | 30.4 Mbps | 38.7 Mbps | 54.8 Mbps | 79.2 Mbps | 112.5 Mbps | 162.5 Mbps |
| 2 | 60 kHz | 13.1 Mbps | 28.6 Mbps | 36.9 Mbps | 52.4 Mbps | 77.4 Mbps | 110.7 Mbps | 160.7 Mbps |

Table 6: FR2 Downlink Data Rate (Mbps).

|  |  | FR2 Channel Bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Numerology (µ ) | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 2 | 60 kHz | 78.6 Mbps | 157.1 Mbps | 314.2 Mbps |  |
| 3 | 120 kHz | 76.2 Mbps | 157.1 Mbps | 314.2 Mbps | 628.5 Mbps |

Following Table 7 illustrates the combined FR1 and FR2 data rates.  Carrier Aggregation (CA) is assumed for 200 MHz and 400 MHz channel bandwidth for FR1 (30kHz and 60kHz).

Table 7: Combined FR1 and FR2 Downlink Data Rate (Mbps) with Carrier Aggregation.

|  |  |  | Channel Bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) |  | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz | 200 MHz | 400 MHz |
| 0 | FR1 | 15 kHz | 15.5 Mbps | 31.5 Mbps | 39.6 Mbps | 55.9 Mbps | 80.3 Mbps |  |  |  |  |
| 1 | FR1 | 30 kHz | 14.3 Mbps | 30.4 Mbps | 38.7 Mbps | 54.8 Mbps | 79.2 Mbps | 112.5 Mbps | 162.5 Mbps | 325.0 Mbps | 649.9 Mbps |
| 2 | FR1 | 60 kHz | 13.1 Mbps | 28.6 Mbps | 36.9 Mbps | 52.4 Mbps | 77.4 Mbps | 110.7 Mbps | 160.7 Mbps | 321.4 Mbps | 642.8 Mbps |
| 2 | FR2 | 60 kHz |  |  |  |  | 78.6 Mbps |  | 157.1 Mbps | 314.2 Mbps |  |
| 3 | FR2 | 120 kHz |  |  |  |  | 76.2 Mbps |  | 157.1 Mbps | 314.2 Mbps | 628.5 Mbps |

From combined Table 7 results we can observe the following:

Observation 1: For a given channel bandwidth, lower subcarrier spacing (e.g., 15kHz) are more bandwidth efficient compared to higher subcarrier spacing (e.g., 120kHz).

Observation 2: Data rates achieved by aggregating FR1 channels at higher bandwidths, such as 200 and 400 MHz are more efficient compared to FR2 for similar bandwidths.

Following Table 8 illustrates the percent data rate decrease for 30kHz, 60kHz, and 120kHz SCS relative to 15kHz SCS.  Moreover, Table 9 illustrates the percent data rate decrease for 60kHz and 120kHz SCS relative to 30kHz SCS.  As an example, the percent data rate decrease formula for 30kHz SCS relative to 15kHz SCS is defined as follows:

Percent Data Rate Decrease=100xData Rate15kHz SCS-Data Rate30kHz SCSData Rate15kHz SCS

Table 8: Percent Downlink Data Rate Decrease for 30kHz, 60kHz, and 120kHz SCS relative to 15kHz SCS.

|  |  |  | Channel Bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) |  | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz | 200 MHz | 400 MHz |
| 0 | FR1 | 15 kHz |  |  |  |  |  |  |  |  |  |
| 1 | FR1 | 30 kHz | 8% | 4% | 2% | 2% | 1% |  |  |  |  |
| 2 | FR1 | 60 kHz | 15% | 9% | 7% | 6% | 4% |  |  |  |  |
| 2 | FR2 | 60 kHz |  |  |  |  | 2% |  |  |  |  |
| 3 | FR2 | 120 kHz |  |  |  |  | 5% |  |  |  |  |

Table 9: Percent Downlink Data Rate Decrease for 60kHz and 120kHz SCS relative to 30kHz SCS.

|  |  |  | Channel Bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Numerology (µ) |  | SCS (kHz) | 10 MHz | 20 MHz | 25 MHz | 35 MHz | 50 MHz | 70 MHz | 100 MHz | 200 MHz | 400 MHz |
| 0 | FR1 | 15 kHz |  |  |  |  |  |  |  |  |  |
| 1 | FR1 | 30 kHz |  |  |  |  |  |  |  |  |  |
| 2 | FR1 | 60 kHz | 8% | 6% | 5% | 4% | 2% | 2% | 1% | 1% | 1% |
| 2 | FR2 | 60 kHz |  |  |  |  | 1% |  | 3% | 3% |  |
| 3 | FR2 | 120 kHz |  |  |  |  | 4% |  | 3% | 3% | 3% |

From results illustrated in Table 9 we can observe the following:

Observation 3: For a given subcarrier spacing (SCS), lower channel bandwidth (e.g., 10 MHz) are less efficient compared to higher channel bandwidth (e.g., 100 MHz).

Following Figure 1 illustrates the data rate versus numerology for 10MHz, 50MHz, 100MHz, and 200MHz channel bandwidths.

Figure 1: Downlink data rate versus Numerology for 10MHz, 50MHz, 100MHz, and 200MHz.

From Figure 1 we can observer that for each channel bandwidth, the data rate (throughput) decreases with increasing SCS value. In Figure 1 the values shown in red represents the percent decrease in data rate relative to the lowest SCS.

From the results illustrated in Figure 1 we can observe that:

Observation 4: Relative to 15kHz SCS, the worst-case and average percent data rate decrease are 15% and 6%, respectively.

Observation 5: Relative to 30kHz SCS, the worst-case and average percent data rate decrease are 8% and 3%, respectively.

The data rate and calculation provided above were all for the downlink.  The downlink data rate assumed a constant 18% overhead as mentioned in Section 3.  The uplink data rate calculation is similar but instead of 18% overhead, it assumes 14% overhead (see Section 3). The data rates for uplink are slightly higher by a constant factor for all SCS and channel bandwidth. As a result, the observations provided above apply equally for the uplink as well.

# Signal to Noise Ratio (SNR)

Table 3 and Table 4 illustrate the FR1 and FR2 payload bandwidth for different subcarrier spacing and channel bandwidth as defined in 3GPP specifications.

For the same channel bandwidth, it is evident that the payload bandwidth decreases with increasing SCS.  For example, for 50MHz channel bandwidth the payload bandwidth is 48.615 MHz, 47.91 MHz, 46.86 MHz, and 46.2 MHz for 15kHz (FR1), 30kHz (FR1), 60kHz (FR1), and 120kHz (FR2), respectively.

As a result, in 5G, the payload does not use all FFT bins because of the guard bands. Moreover, the payload bandwidth decreases with increasing SCS.  However, the noise is added in the time domain and occupies all FFT bins.  As a result, to achieve the same Block Error Rate (BLER) performance, lower SCS (e.g., 15kHz) requires slightly higher SNR compared to higher SCS (e.g., 120 kHz).  For example, Figure 2 illustrates the BLER performance at 50 MHz channel, 16QAM modulation, and coding rate of 0.64 for SCS ranging from 15kHz to 120kHz.

From Figure 2 It is evident that 120kHz SCS requires lower SNR compared to 15kHz SCS to achieve 0.1% BLER.  The exact amount of additional SNR required depends on the coding rate and modulation.

Figure 2: BLER versus SNR at 50 MHz BW for varying SCS.

Given a channel bandwidth, maximizing the bandwidth efficiency is generally more important than minimizing the SNR to achieve specific BLER performance.

Observation 6: For a given channel bandwidth, lower subcarrier spacing (e.g., 15kHz) require slightly higher SNR compared to higher subcarrier spacing (e.g., 120kHz) to achieve the same BLER performance.

# Summary and Conclusion

In this contribution we have demonstrated the impact of the SCS and the channel bandwidth on the UL and DL Link Budget of Ku Band satellite and associated data rates. The analysis was based on the resource blocks and the analytical data rate calculations.

From our detailed analysis we have the following observations:

Observation 1: For a given channel bandwidth, lower subcarrier spacing (e.g., 15kHz) are more bandwidth efficient compared to higher subcarrier spacing (e.g., 120kHz).

Observation 2: Data rates achieved by aggregating FR1 channels at higher bandwidths, such as 200 and 400 MHz are more efficient compared to FR2 for similar bandwidths.

Observation 3: For a given subcarrier spacing (SCS), lower channel bandwidth (e.g., 10 MHz) are less efficient compared to higher channel bandwidth (e.g., 100 MHz).

Observation 4: Relative to 15kHz SCS, the worst-case and average percent data rate decrease are 15% and 6%, respectively.

Observation 5: Relative to 30kHz SCS, the worst-case and average percent data rate decrease are 8% and 3%, respectively.

Observation 6: For a given channel bandwidth, lower subcarrier spacing (e.g., 15kHz) require slightly higher SNR compared to higher subcarrier spacing (e.g., 120kHz) to achieve the same BLER performance.

The conclusions that can be drawn from our analysis are as follows:

- For a given channel bandwidth size, smaller subcarrier spacing, provides higher data rate (throughput).

- Data rates achieved with aggregating FR1 channels for higher bandwidths are more efficient compared with FR2 channels.

Proposal:

Based on our analysis we propose to proceed with FR1 Numerology due to slightly more efficient utilisation of the channel bandwidth.

# References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

[2] 3GPP TS 38.306