3GPP TSG-RAN WG4 Meeting #113							R4-2418030

Orlando, US, 18th – 22nd November, 2024
 
Agenda Item:  	7.9.3
Source: 			Intelsat 
Title: 			Ku Band: Analysis of the Timing Alignment and Cyclic Prefix 
Document for: 	Discussion and Decision

# Introduction

At RAN #104, the WID in [1] was approved to commence work to introduce the Ku Band to the 3GPP specifications for NTN. As described in the WID, the Ku band frequency ranges from 10.7 GHz to 14.5 GHz which is between FR1 and FR2.  As a result, it is not obvious whether to apply numerology for Ku band as FR1 or FR2.  A few key technical factors that will influence the decision on whether to choose FR1 or FR2 for Ku band have been identified, namely: uplink timing synchronization, phase noise, efficient channel utilization, beam hopping granularity, and link budget UL/DL as a function of subcarrier spacing (SCS) and channel bandwidth.  The goal of this document is to address the uplink timing synchronization on numerology.

For 5G integrated GSO and Non-Geostationary Orbit (NGSO) Satellite systems such as Medium Earth Orbit (MEO) or Low Earth Orbit (LEO) satellite communication systems, the Timing Advance (TA) is required to be estimated in the initial random-access procedure to facilitate the uplink frame alignment amongst different users.

However, due to the inherent characteristics of GSO and NGSO satellite communication systems, e.g., wide beam coverage, Doppler shift, and long and time varying propagation delays, the Timing Alignment becomes very challenging.

Uplink time synchronization is challenging because the transmission from all users inside the beam must arrive within the cyclic prefix (CP) of the OFDM waveform.

In this contribution we present detailed analysis of the effect of the Cyclic Prefix of various Sub-carrier Spacing (SCS) on the overall timing alignment for a Ku Band satellite. The values of the SCSs used in the analysis, spans both FR1 and FR2 numerologies.

# Discussion

The Timing Alignment is dependent on the following factors:

- Propagation Delay between the Satellite and the ground terminal

- Doppler Shift of the received signal from the Satellite [impact NGSO satellites]

- Beam Footprint [the beam size and beam’s footprint and its elongation have an impact on the TA

- Cyclic Prefix and Timing margin associated with the selected Numerology which governs the SCS.

In the following sections we provide analysis of each factor and how they affect the overall Timing Alignment.

## Propagation Delay

First, the NTN propagation delay is large due to the satellite altitude. Following Figure 1 illustrates a LEO satellite’s one-way propagation delay for altitude of 600km and 1200km.  The one-way propagation delay depends on the elevation angle and ranges between 2msec and 8msec.  The challenge for LEO satellite system is that the terminal must continuously update the propagation delay and adjust the timing alignment.  Figure 1 also illustrates the one-way propagation delay for GEO which is two-orders-of-magnitude larger as compared to LEO.

Figure 1 LEO and GEO one-way propagation delay versus elevation angle

Observation 1: For LEO satellites at 600km altitude, the propagation delay varies between 2msec and 4.8msec (at 20° elevation).

Observation 2: For LEO satellites at 1200km altitude, the propagation delay varies between 4msec and 8.2msec (at 20° elevation).

Observation 3: For GEO satellite, the propagation delay varies between 119msec and 132msec (at 20° elevation).

## Doppler Shift

The high-speed moving LEO satellites cause a time-varying propagation delay and Doppler shift which must be tracked by the terminal and network. For a LEO satellite at 600km and 1200km altitude, the speed of the satellite is 7562 m/s and 7256 m/s, respectively. The fast-moving LEO satellite also results in Doppler shift at the terminal. For GEO satellite, the propagation delay is large but fixed and the Doppler shift is zero.

Following Figure 2 illustrates the Doppler shift for 600km and 1200km altitude LEO satellite versus elevation angle.  At low elevation angle of 10°, Doppler shift can be as much as 320 kHz at Ku Band uplink.  The Doppler shift results in timing varying frequency offset which must be estimated and compensated at the receiver.

Figure 2: Doppler shift at 600km and 1200km altitude versus elevation angle.

The large propagation delay and/or time-varying propagation delay and Doppler shift makes NTN uplink time synchronization difficult.  To track propagation delay changes, NTN 3GPP has adapted a dual-loop approach with open-loop control and closed-loop control [2,3].

Observation 4: For LEO satellites at 600km altitude, the Doppler shift observed at 10° elevation angle is 330kHz at Ku-band uplink.

Observation 5: For LEO satellites at 1200km altitude, the Doppler shift observed at 10° elevation angle is 280kHz at Ku-band uplink.

Following Figure 3 illustrates the Doppler shift heat map for 600-km LEO satellite, Ku band uplink and downlink, and for two different satellite locations and directions with beam pointed straight down.  The Doppler shift directly below the satellite is zero, as expected.

The Doppler shift is positive for locations opposite to the direction of movement and negative for locations in front of the direction of movement.  The Doppler shift heat map also shows the relationship between the angle of the satellite direction and user terminal.

Figure 3: Doppler shift at Ku-band uplink and downlink and for two sat location (Nadir, 0-deg inclination) and (120 km away, 45-deg inclination)

The following Figure 4 illustrates in one dimension conceptually the results of Figure 3. Again, the Doppler shift is positive for locations opposite to the direction of movement and negative for locations in front of the direction of movement.

Figure 4 Doppler Shift Simulation of Approaching and Separating LEO Satellites

Observation 6: Doppler shift is positive for locations opposite to the direction of movement, and negative for locations in front of the direction of movement.

## Beam Footprint

The beam footprint and the location of the UE within the beam’s footprint has a marked affect on the Doppler Shift and its overall link performance.

Figure 5 illustrates an example beam layout for LEO satellite.  A beam footprint may move along with the satellite or may be fixed at a location on Earth via beam steering until the satellite elevation angle exceeds a threshold (i.e., 20 degrees).

Figure 5: LEO satellite beam footprint.

One popular architecture for LEO constellation is to fix the beam footprint to specific location on Earth as shown in Figure 5.  Moreover, the satellite on the downlink applies the Doppler compensation in the center of the beam footprint.  If the terminal is exactly in the center of the beam footprint, the Doppler shift received will be zero.  But if the terminal is in the edge of the beam footprint, the Doppler shift will be much higher.

Figure 6 illustrates the residual absolute value Doppler shift heat map assuming the satellite applies the Doppler compensation in the center of the beam footprint.  The satellite beam center is at (0,0).

It is well known that the Doppler shift estimators show good performance if the Doppler shift is less than ½ SCS [5].  For example, if the SCS is 15kHz, the Doppler shift algorithm can compensate for residual Doppler below 7.5kHz.  If the Doppler shift is more than 7.5kHz there is Inter-carrier frequency interference, and the orthogonality is destroyed [5].  It is apparent that the satellite beam footprint radius becomes limited by the SCS numerology to not exceed ½ SCS residual Doppler shift.

Figure 6: Differential Doppler shift for two different satellite positions.

Figure 7 illustrates the worst-case beam footprint radius as a function of satellite elevation angle for different numerologies.  For example, 15kHz SCS beam radius should be below 14km to avoid data loss due to Doppler frequency estimation error.  The LEO system design must choose the beam radius below these limits to avoid loss of lock.

Figure 7: Worst case beam radius versus elevation angle for 600km and 1200km altitude.

Observation 7: For LEO satellite with Earth fixed cells, the SCS numerology limits the maximum beam footprint radius.

## Cyclic Prefix and Timing Error

As mentioned earlier, the uplink time and frequency synchronization are essential for NTN because the transmission from all users must arrive within the same cyclic prefix (CP) of the OFDM waveform.  The duration of the CP also varies based on the SCS numerology. Figure 8 illustrates the long and short CP for 15kHz, 30kHz, 60kHz, and 120kHz.

From Figure 8 it is evident that the 120kHz SCS is challenging due to small CP duration period.

Figure 8: Cyclic Prefix and OFDM Structure for 15 kHz, 30 kHz, 60 kHz, and 120 kHz SCS.

Table 1 provides the cyclic prefix duration for the various SCS (15kHz, 30kHz, 60kHz and 120kHz).  It is evident that the short CP duration of 120kHz SCS is eight times smaller compared to 15kHz.  Moreover, Table 2 illustrates the CP samples for the various SCS including the number of long and short CP in half slot.   Both Tables were calculated based on 3GPP spec.

Table 1: Cyclic Prefix and OFDM durations for 15 kHz, 30 kHz, 60 kHz, and 120 kHz SCS.

| Numerology (m) | SCS (kHz) | OFDM Symbol Duration (nsec) | Long CP Duration (nsec) | Short CP Duration (nsec) |
| --- | --- | --- | --- | --- |
| 0 | 15 kHz | 66666.7 | 5208.3 | 4687.5 |
| 1 | 30 kHz | 33333.3 | 2864.6 | 2343.8 |
| 2 | 60 kHz | 16666.7 | 1692.7 | 1171.9 |

Table 2: Cyclic Prefix and OFDM sample for 0.5 msec half slot assuming 50 MHz channel.

| Numerology (m ) | SCS (kHz) | OFDM Symbol Sample | Long CP Sample | Short CP Sample | Num of OFDM Symbols | Num of long CP Symbols | Num of short CP Symbols | Total number of Samples |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 kHz | 4096 | 320 | 288 | 7 | 1 | 6 | 30720 |
| 1 | 30 kHz | 4096 | 352 | 288 | 14 | 1 | 13 | 61440 |
| 2 | 60 kHz | 4096 | 416 | 288 | 28 | 1 | 27 | 122880 |
| 3 | 120 kHz | 4096 | 544 | 288 | 56 | 1 | 55 | 245760 |

Table 3 address the timing requirements for FR1-NTN and FR2-NTN.  The NTN timing error limits were derived from [4].  For FR2-NTN, the 120kHz SCS for Mobile VSAT served by GSO is problematic with a place holder of [10] for timing error which results in negative margin because it is more than ½ CP duration.  Even for fixed VSAT the timing margin is 48.8nsec which is not sufficient for a highly dynamic NTN system.

For FR1-NTN, for all cases the timing error is well within the ½ CP duration.  We expect the timing error for FR1-NTN to improve with higher accuracy VSAT devices and larger antenna.

Table 3: Timing Error Limit for FR1-NTN and FR2-NTN.

| Frequency Range | Scenario | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | NTN Timing Error Limit Te_NTN (nsec) | Uplink CP 0.5 Duration (nsec) | Margin (nsec) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-NTN |  | 15 | 15 | 29*64*Tc | 944.0 | 2343.8 | 1399.7 |
|  |  |  | 30 | 24*64*Tc | 781.3 | 1171.9 | 390.6 |
|  |  |  | 60 | N/A | N/A | 585.9 | N/A |
|  |  | 30 | 15 | 24*64*Tc | 781.3 | 2343.8 | 1562.5 |
|  |  |  | 30 | 22*64*Tc | 716.1 | 1171.9 | 455.7 |
|  |  |  | 60 | N/A | N/A | 585.9 | N/A |
| FR2-NTN | Fixed VSAT served by GSO Fixed VSAT served by NGSO | 120 | 60 | 13*64*Tc | 423.2 | 585.9 | 162.8 |
|  |  |  | 120 | 7.5*64*Tc | 244.1 | 293.0 | 48.8 |
|  | Mobile VSAT served by GSO | 120 | 60 | 13*64*Tc | 423.2 | 585.9 | 162.8 |
|  |  |  | 120 | [10]*64*Tc | 325.5 | 293.0 | -32.6 |

Observation 8: For FR2-NTN, the 120kHz SCS timing for mobile VSAT error margin is negative.

Observation 9: For FR1-NTN, the timing error is significantly less than half the cyclic prefix duration.

Following Figure 9 illustrates graphically the cyclic prefix threshold, timing error and margin duration for various numerologies.  It is evident that the cyclic prefix varies significantly between 15kHz and 120kHz, and the timing margin for fixed and mobile VSAT is small.

Figure 9: Cyclic prefix duration and Timing margin.

Observation 10: Timing alignment with 120 kHz SCS is impractical to achieve.

# Summary and Conclusion

From our detailed analysis we have the following observations:

Observation 1: For LEO satellites at 600km altitude, the propagation delay varies between 2msec and 4.8msec (at 20° elevation).

Observation 2: For LEO satellites at 1200km altitude, the propagation delay varies between 4msec and 8.2msec (at 20° elevation).

Observation 3: For GEO satellite, the propagation delay varies between 119msec and 132msec (at 20° elevation).

Observation 4: For LEO satellites at 600km altitude, the Doppler shift observed at 10° elevation angle is 330kHz at Ku-band uplink.

Observation 5: For LEO satellites at 1200km altitude, the Doppler shift observed at 10° elevation angle is 280kHz at Ku-band uplink.

Observation 6: Doppler shift is positive for locations opposite to the direction of movement, and negative for locations in front of the direction of movement.

Observation 7: For LEO satellite with Earth fixed cells, the SCS numerology limits the maximum beam footprint radius.

Observation 8: For FR2-NTN, the 120kHz SCS timing for mobile VSAT error margin is negative.

Observation 9: For FR1-NTN, the timing error is significantly less than half the cyclic prefix duration.

Observation 10: Timing alignment with 120 kHz SCS is impractical to achieve.

From the detailed analysis presented in this contribution we can draw the following conclusions:

- Propagation delay, Doppler Shift, and time varying propagation delay are phenomena that needs to be managed in the Ku Band NTN satellite design.

- Timing alignment with 15kHz and 30 kHz SCS is manageable and can be implemented practically.

- Timing alignment for 120 kHz SCS is impractical as it only provides a little or negative margin.

- The uplink timing synchronisation for the NTN is dynamic and extremely challenging due to large propagation delay, Doppler Shift, and time-varying propagation delay and it’s imperative to allocate a large timing margin to accommodate practical systems which is possible with FR1 numerology.

# References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

[2] L. Ma, C. Park, X. Wang, P. Gaal and A. R. Alvarino, "Uplink Time Synchronization for Non-Terrestrial Networks," in IEEE Communications Magazine, vol. 61, no. 7, pp. 114-118, July 2023, https://ieeexplore.ieee.org/document/10192287

[3] https://www.researchgate.net/publication/361163791_Uplink_Time_Synchronization_Method_and_Procedure_in_Release-17_NR_NTN

[4] TS 38.133 v18.7.0. Requirements for support of radio resource management (Release 18)

[5]https://www.semanticscholar.org/paper/Doppler-shift-compensation-strategies-for-LEO-Yll/35c62b58c2e9e61d7618de01b1f4c6910a6363a1

Constant beam footprint on the ground is achieved by using beam steering with electronically steered antennas.