# 3GPP TSG-RAN WG4 Meeting #113    	 R4-2418032 
Orlando, US, 18th – 22nd November 2024

Agenda Item:  	7.9.1
Source: 			Intelsat
Title: 			Phase Noise and its Impact on Ku Band Satellites Performance 
Document for: 	Discussion and Decision

# Introduction

At RAN #104, the WID in [1] was approved to commence work to introduce the Ku band to the 3GPP specifications for NTN.  As described in the WID, the Ku band frequency ranges from 10.7 GHz to 14.5 GHz which is between FR1 and FR2.  As a result, it is not obvious whether to apply numerology for Ku band as FR1 or FR2.  A few key technical factors that will influence the decision on whether to choose FR1 or FR2 for Ku band have been identified, namely: uplink timing synchronization, phase noise, efficient channel utilization, beam hopping granularity, and link budget UL/DL as a function of subcarrier spacing (SCS) and channel bandwidth.  The goal of this document is to address the impact of phase noise on numerology.

Intelsat operates the largest integrated geo-stationary space and terrestrial network in the world, including satellites from all major satellite manufactures.  For satellite communication, phase noise are injected into the system at various points in the system including at the gateway, satellite, inter-satellite links, and terminal. The aggregate phase noise experienced by the receiver is the sum of the individual phase noise components at different stages of the communication link.  For space communication, the largest percentage of the aggregate phase noise occurs at the terminal because the phase noise at the gateway, satellite, and inter-satellite links are typically designed with low phase noise specification.

Phase noise is a significant contributor to Signal-to-Noise Ratio (SNR) and as such is an important cause of channel impairment in any communication system. Phase noise, originating from various sources such as satellite transponders, inter-satellite links, and ground-based equipment at the gateway or terminal can significantly degrade the performance of satellite links, leading to reduced throughput and increased error rates:

The channel capacity and the quality of the Satellite link is dependent on various factors such as

- Allocated bandwidth

- Signal to noise ratio (SNR)

- Modulation and coding technique

- Phase Noise

A critical consideration in satellite communication is the impact of phase noise on channel capacity and the link quality. Higher levels of phase noise introduce distortions and errors into the signal, potentially degrading the quality of UL and DL links.  This is particularly true for modern high order modulation schemes with numerous constellation points including 64-QAM and 256-QAM. While the purpose of these high modulation orders is to increase the throughput, they are very sensitive to phase noise levels.

This degradation can result in a lower effective channel capacity and poorer quality. As the SNR decreases, more errors are observed on the links leading to retransmission or packet loss.

Thus understanding, mitigating, and managing phase noise is vital for optimizing satellite communication networks and ensuring efficient data transmission. (See also Technical Annex 1 in section 7.1).

For high mobility scenarios, it has been demonstrated in scientific journals that SCS values could affect the Error Vector Magnitude (EVM) and Block Error Rate (BLER) due to the phase noise and high demand on the local oscillator.

The satellite UL and DL links system can be made robust to phase noise by a proper choice of subcarrier spacing. Phase noise robustness is crucial for all link types where a device transmitter or receiver is involved.

In this contribution we provide a detailed study of the effect of the Phase Noise in a typical Ku Band Satellite link with a varying Sub-Carrier Spacing (SCS) from 15-120 kHz. The aim of this analysis is to ascertain whether various SCS values, and associated Cyclic Prefix values, has significant impact on the satellite link level performance. The SCSs selected for this study spans FR1 and FR2 numerologies.

# Phase Noise Model

To analyse the phase noise, we have considered five phase noise models for comparison:

- Samsung Phase noise Model [2]

- Ericsson Phase noise Model [3]

- DVB Phase noise model 2012-Ku [4]

- DVB Phase noise model 2012-Ka [4]

- Industry standard phase noise model [5]

As a way of comparison, the Power Spectral Density (PSD) of the five models were plotted versus  frequency with varying SCS of 15, 30, 60 and 120 kHz.  Figure 1 shows the Samsung and Ericsson phase noise model [6].  For these models, the phase noise was extrapolated at Ku-band by shifting the PSD as described in [2].  Figure 2 also shows the DVB (2012-Ku and 2012-Ka) and Industry standard phase noise model.  Figure 3 illustrates all the phase noise models in the same Figure.

Figure 1 Samsung and Ericsson phase noise model.

Figure 2 DVB and Industry standard phase noise model.

Figure 3 Comparison of Four Phase Noise Models.

It is worth noting here that the Industry standard phase noise model [5] is meet or exceed by vast majority of satellite component vendors, including [9] and [10].

It important to recognize again that the phase noise model for satellite communication is the sum of the individual phase noise components at different stages of the communication link.

Following  Table 1, for example, illustrates the individual and aggregate phase noise components for the uplink, satellite, downlink, and total for the 2012-Ku phase noise model [4].  For satellite communication, the largest percentage of the aggregate phase noise occurs at the terminal because the phase noise at the gateway, satellite, and inter-satellite links are typically designed with low phase noise specification.

Table 1: 2012-Ku- Non DTH Phase Noise Model [4]

| Offset (Hz) | PROFILE “2012-Ku- Non DTH” SSB (dBc/Hz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | uplink | satellite | downlink | total-equivalent |
| 10 | -42 | -33 | -42 | -32.93 |
| 100 | -72 | -62 | -72 | -61.96 |
| 1 K | -82 | -81 | -82 | -79.23 |
| 10 K | -92 | -91 | -92 | -89.23 |
| 100 K | -102 | -100 | -102 | -98.73 |
| 1 M | -112 | -110 | -112 | -108.73 |
| 10 M | -122 | -120 | -122 | -118.73 |
| 50 M | -124 | -121 | -124 | -120.12 |

Furthermore, the Samsung model is flat at the low frequency offset, and the Ericsson model has poor performance compared to DVB, thus we opted to use the 2012-Ka and 2012-Ku DVB Standard, and Industry standard for our analysis.

Following Table 2 which is an extract of the Figure 3 illustrates the phase noise for 15 and 120 kHz SCS using the DVB standard and Industry standard. From Table 2 we can observe that with all models, the phase noise decreases with increasing SCS size.

Table 2 Phase Noise Power Spectral Density (dBc/Hz) @15kHz and 120kHz.

|  | 2012-ku [4] | 2012-Ka [4] | Industry Standard [5] |
| --- | --- | --- | --- |
| 15kHz | -91 | -90 | -84 |
| 120kHz | -100 | -96 | -93 |

Observation 1: The Phase noise decreases with increasing SCS.

# Simulation Setup

To progress this analysis MATLAB models and libraries were used, and the following illustrates the MATLAB simulation blocks [7]:

Figure 4 MATLAB end-to-end 5G OFDM Phase Noise System Model

The MATLAB software applies the phase noise masks provided in the previous section and determines the performance degradation on the 5G waveform consisting of physical downlink shared channel (PDSCH). The error vector magnitude (EVM) and block error rate (BLER) are used as a metric to determine the performance degradation.  Results are provided for different SCS (15 kHz, 30 kHz, 60 kHz, and 120kHz), 50 MHz bandwidth, and 16QAM modulation.

Again, the phase noise model applied is the sum of the individual phase noise components at different stages of the communication link as shown for example in Table 1 for 2012-Ku phase noise model.  The MATLAB model emulates the receiver on the ground with the aggregate phase noise model collected at the input of the receiver.

The setup shown in Figure 4 has been tested/validated and has been used for analysis by various companies. The test setup has become the industry’s de-facto setup for EVM and BLER analysis with 5G compliant waveform.

In our analysis the following assumptions were made, see Table 3:

Table 3 End-to-End 5G NR Simulation Assumptions

| Parameters | Value |
| --- | --- |
| Modulation | 16-QAM |
| LDPC coding rate | 658/1024 (16-QAM), ~ 0.65 |
| LDPC maximum iteration | 10 |
| 5G bandwidth | 50 MHz |
| Number of block errors | 50 |
| Noise | AWGN |
| Subcarrier Spacing (SCS) | 15kHz, 30kHz, 60kHz, and 120kHz |
| Receiver implementation loss | 0 dB |
| Phase Noise Model | 2012-ku, 2012-ka and Industry standard |
| 5G Payload Bandwidth (MHz) @ 50 MHz | 48.62 MHz(15kHz), 47.91 MHz(30kHz), 47.58 MHz(60kHz), 46.20 MHz(120kHz) |

# Simulation Results and Discussion

## EVM Results

Using the MATLAB simulation and modelling tool, the EVM performance degradation was simulated for various phase noise profiles, namely: 2012-Ka, 2012-Ku, and Industry standard.  The simulation parameters were set to 50 MHz bandwidth and four different SCS settings: 15kHz, 30kHz, 60kHz, and 120kHz. The EVM constellation is shown in Annex 7.3 for 16-QAM symbol constellation.  It’s important to recognize that the EVM values shown here are without noise. Moreover, for satellite communication the expected SNR range is expected to range between 10 dB and 15 dB due to long slant range between source and destination, limited power, and interference.

Figure 5 illustrates the variation in EVM as a function of SCS based on the EVM constellation in the Annex 7.3. We can observe that the phase noise of the Ku Band is much lower than the values shown for Industry standard.

Figure 5 EVM performance 2012-Ka, 2012-Ku and Industry standard profile with varying SCS.

Table 4 illustrates the EVM increase relative to 120 kHz SCS.  It indicates that as the SCS is decreased from 120 kHz to 15 kHz, the EVM increase. Observe that the decrease in EVM is not significant.  For example, the EVM at 15 kHz SCS is approximately 17 % greater that at 120 kHz.

Table 4 EVM increase relative to 120 kHz SCS

| SCS | 2012-Ka | 2012-Ku | Industry standard |
| --- | --- | --- | --- |
| 15 kHz | 7.2% | 17.1% | 7.5% |
| 30 kHz | 4.3% | 5.9% | 4.4% |
| 60 kHz | 4.3% | 4.8% | 2.0% |
| 120 kHz | 0.0% | 0.0% | 0.0% |

## BLER Results

Using the MATLAB simulation and modelling tool, the BLER performance was also simulated for various phase noise profiles.  The simulation parameters were set to 50 MHz bandwidth and four different SCS settings: 15kHz, 30kHz, 60kHz, and 120kHz. The BLER performance are illustrated in Annex 7.4 with 16-QAM modulation.  The maximum DVB phase noise model SNR for 16QAM is 10.5 dB.  Assuming 1.5 dB implementation loss, the SNR become 12 dB for 16QAM.

The following Figure 6 is an extract of results from the BLER plots.  The SNR loss is defined as the difference in SNR between no phase noise and with phase noise at 0.1% BLER for the same SCS.  Figure 6 illustrate that the SNR degradation with 16 QAM modulation is negligible for all phase noise models considered.

Figure 6 SNR degradation @ 0.1% BLER for 2012-Ku and 2012-Ka phase noise model.

Observation 2: Phase noise does not play a significant role on numerology when employing 16-QAM modulation.

Observation 3: 16-QAM/QPSK modulation is expected to be employed in vast majority of satellite links.

# Conclusion

From our detailed analysis we have the following observations:

Observation 1: The Phase noise decreases with increasing SCS.

Observation 2: Phase noise does not play a significant role on numerology when employing 16-QAM modulation.

Observation 3: 16-QAM/QPSK modulation is expected to be employed in vast majority of satellite links.

Thus, from our detailed analysis we can deduce that the effect of SCS numerology, and associated CP, has minor impact on system performance based on the overall phase noise of the satellite system.

# References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

[2] R1-163984, Discussion on phase noise modelling, Samsung, RAN1#85

[3] R4-1906185, Further elaboration on 7-24 GHz phase noise for different example frequencies, Ericsson, TSG-RAN WG4 #91

[4] DVB Document A171-2, Digital Video Broadcasting (DVB) Implementation guidelines for the second-generation system for Broadcasting, Interactive Services, News Gathering and other broadband satellite applications; Part 2 - S2 Extensions (DVB-S2X), March 2015.

[5] Industry standard phase noise model (http://everyspec.com/MIL-STD/MIL-STD-0100-0299/MIL-STD-188-164C_55855/ , MIL-STD-188/164 Rev. C)

[6] R4-2412265, Discussion on NR NTN Ku band plan & system parameters, CHTTL, RAN4#112

[7] https://www.mathworks.com/help/5g/ug/nr-phase-noise-modeling-and-compensation.html

[8] Phase Noise Impact on Satellite Uplink and Downlink Channel Capacity, on Satellite Uplink and Downlink Channel Capacity by Mohsen Zolghadri, William Walsh Microwave Journal

[9] https://www.norsat.com/collections/buc-sspa?page=1&pf_t_band=Ku-Band

[10] https://www.digisat.org/ku-band-block-upconverters-bucs

# Appendix

## Technical Annex 1: What is Phase Noise

Phase noise represents short-term variations in the timing or phase of a signal. In the context of oscillators, essential for frequency translation in RF chains during satellite communication, the instantaneous frequency fluctuates randomly around the nominal frequency. The characterization of this variation is crucial, and phase noise is often depicted in the frequency domain through a spectral density plot [8].

To quantify phase noise, the spectral plot displays phase noise measurements in decibels relative to the carrier (dBc) at specified frequency offsets. See following Figure 7

Figure 7 Phase Noise Measurements with respect to the main carrier frequency.

By integrating the phase plot over these frequency offsets, an RMS value of phase error in radians can be obtained. This value is instrumental in quantifying the contribution of phase noise to the SNR, particularly relevant for estimating its impact on throughput in specific modulation schemes like QAM.

The following figure illustrates an example of excessive Phase Noise on a typical 16 QAM constellation, see Figure 8:

Figure 8 Effect of excessive Phase Noise on a 16 QM constellation.

While long-term frequency deviations, often related to temperature, can contribute to phase and frequency errors, this white paper focuses specifically on short-term, random phase variations. Advanced modem techniques are typically employed to track and correct long-term deviations.

Regarding satellite transponders, local oscillators in both the receiver and transmitter RF chains are central to frequency translation in satellite systems. Factors contributing to phase noise in a transponder's local oscillator include:

• Crystal Reference Oscillator- The stability of the crystal reference oscillator impacts the precision of the local oscillator.

• Temperature Variations- Fluctuations in temperature can affect the performance of the local oscillator

• Frequency Multipliers and Dividers- Components involved in frequency multiplication and division contribute to phase noise.

• Nonlinearities in RF Components- Amplifiers and other RF components introduce nonlinearities affecting phase noise.

• Power Supply Noise- Variations in the power supply can impact the stability of the local oscillator.

• External Interference- Unwanted external signals can introduce interference and affect phase noise.

• Radiation Effects- Exposure to radiation in space can influence the performance of the local oscillator.

• Micro-vibrations- Due to satellite navigation and guidance, attitude control, could affect the phase noise in a lesser degree.

## Numerology Design Principles

A given carrier frequency, phase noise, and Doppler set requirements on the minimum sub carrier

spacing. Use of smaller subcarrier spacings would result in either high error vector magnitude (EVM) due to phase noise or undesirable high requirements on the local oscillator. Too narrow subcarrier spacings also lead to performance degradations in high Doppler scenarios.

Required CP overhead (and thus anticipated delay spread) sets an upper limit for the subcarrier spacing; selecting too large subcarriers would result in undesirable high CP overhead.

The maximum fast Fourier transform (FFT) size of the OFDM modulator together with subcarrier spacing determines the channel bandwidth.

Based on these relations the subcarrier spacing should be as small as possible, while still being robust against phase noise and Doppler and providing the desired channel bandwidth.

## EVM results

The following Figures illustrate the EVM outputs for 16 QAM Modulation for 2012-Ku, 2012-Ka and Industry standard phase noise models with 15kHz, 30kHz, 60kHz, and 120kHz SCS.

Figure 9 Equalized Symbol Constellation with 2012-Ku profile with 16-QAM, and 50 MHz BW.

Figure 10 Equalized Symbol Constellation with 2012-Ka profile with 16-QAM, and 50 MHz BW.

Figure 11 Equalized Symbol Constellation with Industry standard profile with 16-QAM, and 50 MHz BW.

## BLER results

The following Figures illustrate BLER performance with 16 QAM Modulation for 2012-Ku, 2012-Ka and Industry standard phase noise models with 15kHz, 30kHz, 60kHz, and 120kHz SCS.

Figure 12 Block Error Rate versus SNR for 16QAM, LDPC code rate 658/1024 (0.64).

The SNR loss definition at 1% BLER could have been chosen.