# 3GPP TSG-RAN WG4 Meeting #113	R4-2418036

# Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	views on requirements of the low-power wake-up receiver

Agenda item:	7.23.2

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were discussed in RAN4 #112 [1], and a WF was agreed [2]. According to the WF, the link level simulation assumption for ACS/ASCS evaluation has been agreed upon, but some prominent issues for REFSENS, which includes reference architecture, NF, and IM, still need to be further discussed.

In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR based on our LLS results.

- REFSENS for LP-WUR

## One or two sets of requirements

The number of requirements for OFDM and ED-based LP-WUR has been discussed over the past few meetings without a conclusion. In general, both types of WUS signal and the corresponding receivers need to support sufficient coverage of LP-WUS (as PUSCH of Msg 3). Since RAN4 only defines minimum requirements, and the requirements themselves are usually implementation agnostic, it should be sufficient to define only one set of requirements covering all receivers.

Observation 1 	All types of WUS signals and the corresponding receivers shall meet the same coverage target, and thus, one set of minimum requirements is required to ensure this coverage is sufficient.

Meanwhile, we would also like to highlight that there is no clear benefit in specifying multiple sets of requirements based on receiver type. As explained above, both types of receivers will be designed under the same target coverage; thus, there should be no difference from the network perspective. On the other hand, multiple sets of requirements may lead to market and implementation fragmentation and jeopardize the commercial deployment of LP-WUS.

Observation 2	With the same coverage target, there is no clear benefit to specifying multiple sets of requirements for LP-WUR.

Proposal 1	RAN4 should define one set of minimum requirements covering all types of LP-WUS receivers.

Per the outcome of the study item phase in TR38.869, the NF of zero-IF based ED receiver is in the range of 12-16 dB, which is higher than the NF of OFDM receiver that was typically assumed for UE in FR1 (in the range of 9-12 dB). Therefore, it can be expected that the REFSENS of the ED-based receiver will be worse than the OFDM-based receiver, and RAN4 may consider focusing on the ED-based receiver to derive the minimum requirement.

Proposal 2	 RAN4 adopts ED-based receiver to derive a single set of minimum requirements on REFSENS for ED and OFDM receivers.

In our view, what could be more relevant to the number of requirements is not based on the receiver architecture but rather the operation mode of LP-WUS, e.g., whether it is in the Idle mode or the connected mode. Since the usage scenarios are different in idle mode and connected mode for LP-WUS, RAN4 may further discuss whether the same requirement or different requirements for different modes. Meanwhile, at least in FR1, the idle mode can be taken as a baseline to define the minimum requirement, as this is a major usage scenario in our view.

Observation 3 	The operation modes (e.g., Idle or connected) might be more relevant when discussing the number of requirements than the receiver architecture.

Proposal 3 	At least in FR1, RAN4 can define the minimum requirement based on idle mode and further check if the same requirement can be used in connected mode.

## Side condition for REFSENS

Although multiple contributions have been submitted in previous RAN4 meetings on the target SNR for LP-WUR, there is lack of detail on how the sequence has been detected; thus, it is hard to compare the results. For the sake of clarity, we will explain how our detection algorithm works in our simulation. In our simulation, we assume a preamble is prior to the data information, and this is to allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity. The LP-WUR first needs to look for the preamble and only continues to detect the data if the preamble is detected. Fig. 1(a) shows the general signal structure used in our simulation, which contains sequences of length K and L bits, representing the preamble and data, respectively. The data part indicates the UE's subsequent action after LP-WUS detection and/or identity of (group) UE. Fig. 1(b) illustrates a general block diagram of the digital baseband processing used to detect the LP-WUS in this contribution.

- (b)

Fig. 1 (a) LP-WUS structure, consisting of two fields, preamble used for synchronization and possibly cell identity purposes, and data part to indicate the potential subsequent action or UE (group) identity. (b) Digital baseband processing.

Per our understanding, the usage of the preamble can significantly reduce the FAR, and it will further impact the MDR when we design the detection threshold for the sequence. Meanwhile, the receiver could also use LP-SS to compensate for the frequency drift in the oscillator further. Although the preamble and LP-SS design are still under RAN1 discussion, it would be beneficial if RAN4 conclude whether any preamble or LP-SS would be considered in the RF testing before determining the actual SNR threshold.

Proposal 4	RAN4 shall decide if the preamble or LP-SS would be configured in the LP-WUS RF testing.

## REFSENS requirement

In this section, we present our proposed threshold SNR, NF, and REFSENS values based on the zero-IF architecture of the ED-based receiver.

Based on our simulation results, the target SNR based on our sequence and receiver design above can reach about -7 dB for 1% MDR and -3 dB for 1% BER under AWGN channel (see appendix), there are about 4dB processing gain (1% MDR vs. 1% BER) based on our design. Considering that the different receiver implementations may present different processing gains, an SNR value between -7 and -2 dB can be considered.

Observation 4	 Based on our sequence and receiver design, the target SNR is about -7 dB for 1% MDR and -3 dB for 1% BER.

Proposal 5	RAN4 adopts a target SNR value in the range between -7 dB to -2 dB for LP-WUS.

The NF of a zero IF receiver is estimated as below, an antenna switching is included to take into account the antenna sharing mechanism. It can be seen in total 10 dB NF is sufficient to accommodate the receiver performance. In addition, a relative high NF for LNA is assumed with consideration to lower the power consumption on it.

| Component | Gain (dB) | NF (dB) |
| --- | --- | --- |
| Matching network /Antenna switching/RF BPF | -4 | 3 |
| LNA | 15 | 5 |
| Mixer | -1 | 10 |
| BB amplifier | 30 | 3 |
| BB LPF | -1 | 2 |
| 4-bit ADC | -1 | 4 |
| Total | 48 | 9.15 |

Proposal 6	RAN4 adopts the NF value of 10 dB to derive the REFSENS.

The implementation margin (IM) of 2.5 dB has been used to derive the REFSENS for UE in both LTE and NR time., it appears that detailed rationales are not explicitly documented, which is likely emerged as an agreement through cumulative contributions, real-world testing, and agreements within the 3GPP RAN4 working group discussions on RF performance for LTE. In our understanding, this IM value should at least cover both RF and BB margins, and 2.5 dB can be used as a starting point to derive the REFSENS requirement for LP-WUR.

Proposal 7	RAN4 adopts 2.5 dB IM as a starting point to derive the REFSENS.

With all the parameters above, the REFSENS of LP-WUS for 11 RB with 30 kHz can be calculated as below:

REFSENS=-174 dBm+10*log3.96MHz+10 NF+-2SNR+2.5 dB (IM)=-97.5 dBm

Observation 5	 With the proposed SNR, NF, and IM, the REFSENS for LP-WUR with 11 RB and 30 kHz SCS will be about -97.5 dBm.

With such an estimation of REFSENS, RAN4 can further check if the coverage target can be fulfilled.

Proposal 8 	RAN4 further checks if the coverage target can be fulfilled once some baseline REFSENS value is agreed.

- ACS and ASCS Requirement

## Requirement for ACS and ACSC

From the network co-existence aspect, the LP-WUS is expected to co-exist with other signals and suffer the same level of interference as a normal NR receiver. Thus, setting the same interference signal level for the LP-WUR as MR would be reasonable for the ACS requirement.

Observation 6	Since the LP-WUS needs to co-exist with other NR signals, it is reasonable to set the same ACS level for the LP-WUS receiver as the main receiver.

Proposal 9	Define the same interference signal level as MR for LP-WUR for ACS requirements.

Meanwhile, ACSC's requirement is to tolerate the in-band interference that the network can control, so it could potentially be more relaxed than ACS. Therefore, RAN4 should further investigate if ACSC requirements should be set like ACS requirements.

One proposal on ACSC from RAN4#112 is to define only the test case, but there is no ACSC requirement value. For legacy ACS requirements, although a requirement on ACS value is defined (33 dB) since it is difficult to test the ACS value directly, the core requirement is verified by two test cases with low and high interference levels but without testing the ACS level itself.

Therefore, for ASCS, it is a question if RAN4 still needs to put effort on defining an ASCS value itself or just focus on defining one or two test cases to ensure the LP-WUR receiver performance.

Observation  7	Considering the ACS is not measurable, but its performance is verified indirectly with a few test cases on MR, it is a question if RAN4 needs to define a value of ACSC or rather focus on defining a test case.

Based on our simulation results (see appendix), the number of guard RBs does not significantly affect ACSC performance. Therefore, it is proposed that 0 RB be adopted for ACSC test cases.

Proposal 10 	RAN4 adopts 0 guard RB for ACSC test case

- OBB/IBB and other receiver requirements

Similar logic to ACS, it is expected that the blocking signal level in IBB and OBB should be the same as the MR. The exact location of the blocking signal can be further discussed. Assuming the LP-WUS receiver is capable of meeting the MR ACS requirement with configured guard RBs, it is proposed that the configured guard RB should also be configured in the OBB and IBB test., e.g., the offset of blocking signal for LP-WUS OBB/IBB test is shifted further away from the LP-wanted signal with the number of guard RB than in the MR case.

Proposal 11	Define the blocking signal level the same as the MR for LP-WUR, and further study if the offset of the blocking signal to the wanted signal should be shifted with guard RB.

- Testability of LP-WUS

From the testability aspect, different from the legacy REFSENS or ACS test for the MR, there will be no ACK/NACK for the LP-WUS receiver, and therefore, how the test equipment (TE) shall be able to count the missed detection rate is unclear. One possible solution is to implement a dedicated test mode to allow the UE to record whenever it detects an intended wake-up sequence and report this value to TE once the test is done.

Observation 8	It is possible to implement a test mode in the UE so that the number of wake-ups can be counted and recorded.

As an alternative, 3GPP may also investigate whether succeeding transmissions (e.g., PRACH) on the main radio can be detected by the TE once the UE has been woken up. For the sake of test time, it would be preferred that no connection mode needs to be created. However, it would be a question of how the UE LP-WUR would behave if there is no response from the TE on the MR after a few attempts.

Observation 9	As an alternative to the test mode, 3GPP may also investigate whether succeeding transmissions from the main radio can be detected by the TE to measure the missed detection rate once the UE has been woken up.

# Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

- References

R4-2415235		Topic summary for [112bis][131] NR_LPWUS_UERF	Moderator(Vivo)

R4-2417112		WF on UE RF requirements for LP-WUS	Vivo

TR38.869		Study on low-power Wake-up Signal and Receiver for NR

- Appendix LLS

In this section, we present initial simulation results for ASCS analysis with OOK-4 waveform. Our simulations are based on the simulation assumption for link level simulation (LLS) agreed during RAN4#111, shown below:

| Attributes | Assumptions |
| --- | --- |
| Case name (waveform) | OOK-4 waveform |
| Center frequency | 2.6GHz |
| Chip rate | M=4 |
| WUS duration | 126 bits ~ 1mSec |
| Coding | 1/2 rate Manchester coding |
| Time error | 0 |
| Residual Frequency error | 0 |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 |
| WUS RB | - Fixed 11RB ~ 3.96MHz |
| Position within channel | - Center of the channel |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth |
| Filter | - 5th Order Butterworth matching fixed 3.96MHz RF bandwidth |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same average PSD with WUS signal |
| Sampling rate | 7.68MHz |
| ADC bit width | Ideal ADC |
| Phase noise | Not modelled |
| Non-linearities | Not modelled |
| Power boosting | No power boosting |
| Channel Model | TDL-C 300 and AWGN |
| Performance metric | 1% MDR with false alarm rate below 1% |

In our simulation, LP-WUS design has the following characteristics:

- Kasami sequences of equal length of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit LP-WUS, which has a total time duration of 1 msec.

- Overlaid OFDM sequence with length 16 consisting of an m-sequence of length 15 and a random single bit. The overlaid OFDM sequence is generated based on option 1-1 agreed in RAN1, i.e., before DFT/LS block.

- The LP-WUS occupies 16 OFDM-symbols, 11RBs (1 RB GB at each side of the LP-WUS bandwidth) with subcarrier spacing of 30 kHz.

We have used ideal ADC but it is expected that the ADC performance would not degrade our MDR performance significantly. The order of lowpass Butterworth is set to 5 and we assume no frequency and timing error.

We study the results, BER and MDR, for AWGN and TDL-C 300 channel for two guard band settings of 0 RB and 1 RB on each side of the LP-WUS bandwidth. The achieved false-alarm rate for the studied scenarios is below 0.1%, thanks to LP-WUS structure, sequence design, and DBB processing.

Fig. A1  Bit error rate (BER) for AWGN (left) and TDLC (right) channel models.

Fig. A2    Miss detection rate (MDR) for AWGN (left) and TDLC (right) channel models.

Based on this initial simulation results, it can be observed that the number of guard RB does not affect the MDR performance significantly. Additionally, the processing gain from the sequence design can compensate for performance loss of the LP-WUR due to its ultra-low power design, allowing to reach the target coverage level even at relatively high BER.