3GPP TSG-RAN WG4 Meeting #113				                              R4-2418038

Orlando, US, 18th – 22nd November, 2024

Source:	Sony

Title:	OCC scheme on the RF requirements for IoT NTN

Agenda item:	7.27.2

Document for:	Discussion

# Background

In Rel-19, uplink capacity enhancement is one of the objects for IoT NTN enhancement [1], with the detail as below:

- Support of Capacity enhancements for uplink

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4]

    - Multi-tone support for 15 kHz SCS should also be considered

    - Specify necessary signalling, if needed

    - Update RF requirements accordingly, if needed

Note: Impact of impairment shall be taken into account

In addition, the design of OCC scheme for IoT NTN has been discussed over a few meetings in RAN1, with the latest agreements from RAN1#118[1][2] as below:

Agreement

At least the following schemes are supported for single-tone:

- For 3.75kHz SCS OCC for NPUSCH format 1:

  - OCC length 2, Symbol-level

    - FFS: DMRS pattern(s)

- For 15kHz SCS OCC for NPUSCH format 1:

  - OCC length 2, Slot-level, CDM DMRS with legacy pattern

    - FFS: CDM details, e.g. with or without spreading

In this contribution, we provide our futher view on the RF requirement impact for the two agreed scheme from RAN1.

# OCC length 2, Symbol-level for 3.75kHz

For symbol level with OCC length 2, it is our understanding that no RF requirement impact would be expected due to the short time span of symbol-level OCC. In this case, it can be expected that the UE is capable of maintaining a sufficiently stable phase over the repetitions with the existing RF requirement.

Observation 1: No RF requirement impact on the symbol level OCC2 scheme with 3.75kHz.

# OCC length 2, Slot-level for 15kHz

On the other hand, when it comes to OCC2 slot level OCC for 15 kHz, this may not be the case due to the increased time span of the OCC code. Due to the impact of CFO of UE and the stability of active components like PA over the time span of the OCC code, the phase continuity may not be maintained, or there may be excessive phase rotation, and thus RF requirements may need to be specified to limit those effects. Therefore, RAN4 shall further study this aspect when RAN1 has made decisions.

Observation 2: Due to the impact of the CFO of the UE and the stability of active component like PA over the time span of the OCC code The UE may not maintain sufficient phase continuity within the time span of the OCC2 scheme with slot level.

Proposal 1: Pending on RAN1 design, RAN4 may consider the potential RF requirement impact for OCC2 slot level.

Moreover, it is unclear if there could be any gap in the middle of the OCC scheme. As already discussed in RAN1, the following gap may happen For IoT-NTN NPUSCH format 1 transmissions:

- UL gaps for synchronization (from Rel-13)

- Gaps around NPRACH occasions

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

In general, it can be expected that the phase continuity would not be able to be maintained over a gap unless the UE is maintaining its Tx chain during the UL gap, e.g., not switching off or re-configuring the Tx chain. If the OCC scheme needs to be maintained over a gap, new UE RF requirement may need to be specified to support such a feature. Such new RF UE requirements would only apply to certain types of UL gap. For example, during an UL gap for synchronisation the UE is meant to re-synchronise its clock to the DL synchronisation signals and it would hence be impossible to maintain phase continuity across such a gap.

Proposal 2: Pending on RAN1 design, if the OCC scheme would cross certain types of gap in the UL transmission, new UE RF requirement may need to be specified.

Given that the errors in OCC transmission are most likely when paired UEs have high and opposite frequency inaccuracy (e.g. UE1’s frequency inaccuracy is +0.1ppm and UE2’s frequency inaccuracy is -0.1ppm), it would be beneficial for a UE to resynchronise its transmission frequency periodically during an ongoing transmission as this will randomise the relative error in the frequency of the UEs’ transmissions. For example, if a transmission consists of 16 OCC codewords and resynchronisation occurs between the first set of 8 codewords and the second set of 8 codewords, it is unlikely that both the first set and second set of OCC codewords would both observe high and opposite frequency inaccuracy (e.g. the first set of OCC codewords might have inaccuracy of +/- 0.1ppm and the second set of OCC codewords might have an inaccuracy of 0/0ppm). The FEC and interleaving across the OCC codewords would then reduce the probability of block error.

Observation 3: From RF perspective, errors in OCC transmission are most likely when paired UEs have high and opposite frequency inaccuracy (e.g. UE1’s frequency inaccuracy is +0.1ppm and UE2’s frequency inaccuracy is -0.1ppm).

Proposal 3: Based on the RF behaviour of UE, it is beneficial if the UE can resynchronise its transmission frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: No RF requirement impact on the symbol level OCC2 scheme with 3.75kHz.

Observation 2: Due to the impact of the CFO of the UE and the stability of active component like PA over the time span of the OCC code The UE may not maintain sufficient phase continuity within the time span of the OCC2 scheme with slot level.

Observation 3: From RF perspective, errors in OCC transmission are most likely when paired UEs have high and opposite frequency inaccuracy (e.g. UE1’s frequency inaccuracy is +0.1ppm and UE2’s frequency inaccuracy is -0.1ppm).

Proposal 1: Pending on RAN1 design, RAN4 may consider the potential RF requirement impact for OCC2 slot level.

Proposal 2: Pending on RAN1 design, if the OCC scheme would cross certain types of gap in the UL transmission, new UE RF requirement may need to be specified.

Proposal 3: Based on the RF behaviour of UE, it is beneficial if the UE can resynchronise its transmission frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

# References

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R1-2407296	FL Summary #1 for IoT-NTN	Moderator (Sony)

- R1-2407297	FL Summary #2 for IoT-NTN	Moderator (Sony)