3GPP TSG-RAN4 Meeting #113	R4-2418042

# Orlando, United States, 18th - 22nd November, 2024

Agenda item:		7.14.2

Source:				Keysight Technologies UK Ltd

Title:					On Rel-19 Spatial channel model CDL Based Methodologies aspects

Document for:		Discussion and endorsement

# Introduction

In RAN#104 a new study item on spatial channel model for demodulation performance requirements was approved ([1]).

The main goal of this study item is to study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements for both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1:

o	Including the limitations of the channel models used up to Rel-18 regarding UE MIMO performance

o	Considering both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches (using the tuned repeatable spatial channel model of TR38.827 as the starting point for CDL approach).

o	Evaluating test methodology feasibility and achievable results uncertainty while test complexity is not significantly increased.

In RAN4#112bis, the way forward document agreed for this study item [2] included the following statement regarding the subcluster correlation for CDL based methodologies, based on the inputs provided in clause 2.6 in [3]:

This document provides company views on that topic.

# Discussion

This document provides a response to the observation #10 "There is a risk that the fading processes between the sub-clusters become correlated, depending on their random initialization." and the proposal #3 "RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it." in [3]. The same issue is a proposal under the "Subcluster correlation" in [2].

3GPP TR 38.827 V16.8.0 [4] Section 7.2 writes: "Each CDL parameter table of contains two sets of three rows, i.e., three clusters, with exactly same angular parameters. This is harmful for the statistical properties of the models as they become non-WSS across the ensemble of model realizations. Instead of making the angular parameters non-equal by introducing small offsets to angles of the three rows, the problematic clusters are treated as midpaths as intended when the CDLs where drawn from statistical distributions which works across all frequency ranges. For the clusters that look like midpaths, e.g., Cluster 2-4 and 5-7 for CDL-A and Cluster 2-4 and 6-8 for CDL-C, the powers for each of the three clusters are added and using the regular midpath power distribution of 0.5, 0.3, and 0.2 specified in Table 7.5-5 of TR38.901, the powers for the each of the midpaths are calculated. Notice that the intra cluster delay spread in Table 7.5-5 of TR38.901 is not followed, and the same delays as the original CDL are followed for the midpaths (aka Sub-Cluster). This helps keeping the rms DS of the modified CDL to 1s." One of the mentioned CDL parameter tables is exemplified in Figure 1.

In other words, TR [4] says with a few typographic errors and unclarities, that the midpath concept is applied with clusters that “look like” midpaths, i.e., paths that have identical angular parameters but different delays. It means, following Table 7.5-5 of [5], that 10 of 20 rays allocated to the first “sub-cluster”, 6 of 20 to the second, and 4 of 20 to the third. More specifically, as an example concerning rows 2-4 of the CDL table in Figure 1, rays {1,2,3,4,5,6,7,8,19,20} are allocated to the cluster #2, rays {9,10,11,12,17,18} are allocated to the cluster #3, and rays {13,14,15,16} are allocated to the cluster #4. Since each ray has a different offset angle any two clusters will have neither identical ray angles nor identical Doppler frequency components. Consequently, having different initial phases for all rays as specified in [4] and resulting from principles of sum-of-sinusoids theory, generated fading sequencies will be outcomes of independent fading processes.

Figure 1. Example of CDL parameter table highlighting discussed rows and same cluster angles. The figure is copied from [3].

Observation 1: There will be no correlation between subclusters for CDL based methodologies. Hence, there is no need to define mechanisms to avoid it.

Proposal 1: Discussion on potential correlation between subclusters and the definition of mechanisms to avoid it for Rel-19 Demodulation spatial Channel model work item can be closed now. No action is required.

# Conclusion

This document provides company views on whether the potential correlation between subclusters is an issue and whether there should be mechanisms to avoid it.

The following observations and conclusions were made in this contribution:

Observation 1: There will be no correlation between subclusters for CDL based methodologies. Hence, there is no need to define mechanisms to avoid it.

Proposal 1: Discussion on potential correlation between subclusters and the definition of mechanisms to avoid it for Rel-19 Demodulation spatial Channel model work item can be closed now. No action is required.

# References

- RP-241610, Study on spatial channel model for demodulation performance requirements, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, 3GPP TSG RAN Meeting #104, June 2024

- R4-2416591, Way Forward Minutes for [112bis][318] NR_SCM, Moderator (Nokia), 3GPP TSG-RAN WG4 Meeting #112-bis, October 2024

- R4- 2415382, Discussion on Spatial Channel Modelling, MediaTek Inc., 3GPP TSG-RAN WG4 # 112-bis, October 2024

- 3GPP TR 38.827, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”, V16.8.0, September 2022.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, V18.0.0, March 2024.