3GPP TSG-RAN4 Meeting #113	R4-2418047

# Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1309 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR Bugfix 38101-3-i70_s00-05 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_2BLTE_1BNR_3DL2UL, DC_R18_xBLTE_2BNR_yDL2UL, DC_R18_xBLTE_yBNR_zDL2UL, NR_CADC_R18_2BDL_xBUL, NR_CADC_R18_3BDL_xBUL |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Band combinations bug fixing and adding missing fallbacks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting notation bugs, removing duplicate configs and adding some fallbacks of combinations that have been introduced without the required fallbacks |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Bugs are still in the band combinations tables and combinations cannot be supported as they are not correctly specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1.1, 5.5A.1.2, 5.5B.4, 5.5B.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Foreword

## 5.5	Configuration

## 5.5A	Configuration for CA

### 5.5A.1	Inter-band CA configurations between FR1 and FR2

Table 5.5A.1-1a: Void

Table 5.5A.1-1b: Void

Table 5.5A.1-1c: Void

Table 5.5A.1-1d: Void

Table 5.5A.1-1e: Void

Table 5.5A.1-1f: Void

Table 5.5A.1-1g: Void

Table 5.5A.1-1h: Void

Table 5.5A.1-1i: Void

Table 5.5A.1-1j: Void

Table 5.5A.1-1k: Void

Table 5.5A.1-1l: Void

Table 5.5A.1-1m: Void

Table 5.5A.1-1n: Void

Table 5.5A.1-1o: Void

Table 5.5A.1-1p: Void

Table 5.5A.1-2: Void

Table 5.5A.1-3: Void

Table 5.5A.1-4: Void

#### 5.5A.1.0	General

The configurations for operating bands for CA including Band n41 also apply for the corresponding operating bands for CA with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for CA with Band n90 are not listed in the tables below but are covered by this specification.

The configuration tables for CA describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each FR1 band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each FR1 band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 of TS 38.101-1 [2] for a given release are still optional for UEs that support BCS4 or BCS5, where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. The minimum bandwidth per CC and maximum aggregated FDD, TDD and total bandwidth per band combination may be indicated only for BCS5 and BCS5 as described in 38.306 [11] shall not be indicated together with BCS4 for a CA configuration. For inter-band CA combinations including intra-band CA and with BCS4 or BCS5 in the following configuration tables, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5, respectively, and the Bandwidth Combination Sets for the FR2 intra-band CA are BCS0.

In the CA configuration tables of clause 5.5A.1:

- Uplink CA configuration entries with "-" mean that any valid constituent band of the downlink inter-band CA combination can be configured as a single uplink carrier,

- Unless otherwise noted, all of the valid downlink constituent bands can be configured as a single uplink carrier,

- If an uplink CA configuration is supported, its fallback single uplink is also supported.

#### 5.5A.1.1	Inter-band CA configurations between FR1 and FR2 (two bands)

##### Table 5.5A.1.1-1a ~ Table 5.5A.1.1-1g

Table 5.5A.1.1-1a: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n257A | CA_n1A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n257D | CA_n257D CA_n1A-n257A/D | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n1A-n257E | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257E |  |
| CA_n1A-n257F | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257F |  |
| CA_n1A-n257G | CA_n257G CA_n1A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n257H | CA_n257G/H CA_n1A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n257I | CA_n257G/H/I CA_n1A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n1A-n257J | CA_n257G/H/I/J CA_n1A-n257A/G/H/I/J | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n1A-n257K | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n1A-n257L | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n1A-n257M | CA_n257G/H/I/J/K CA_n1A-n257A/G/H/I/J/K | n1 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n1A-n257(2A) | CA_n1A-n257A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n257 | CA_n257(2A) |  |
| CA_n1A-n257(2G) | CA_n1A-n257A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n257 | CA_n257(2G) |  |
| CA_n1A-n257(A-G) | CA_n1A-n257A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n257 | CA_n257(A-G) |  |
| CA_n1A-n258A | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n1A-n258B | CA_n1A-n258A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n1A-n258C | CA_n1A-n258A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n1A-n258D | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258D |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258D |  |
| CA_n1A-n258E | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258E |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258E |  |
| CA_n1A-n258F | CA_n1A-n258A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258F |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258F |  |
| CA_n1A-n258G | CA_n1A-n258A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258G |  |
| CA_n1A-n258H | CA_n1A-n258A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258H |  |
| CA_n1A-n258I | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258I |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258I |  |
| CA_n1A-n258J | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258J |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258J |  |
| CA_n1A-n258K | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258K |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258K |  |
| CA_n1A-n258L | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258L |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258L |  |
| CA_n1A-n258M | CA_n1A-n258A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258M |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n258 | CA_n258M |  |
| CA_n1A-n258R2 | CA_n1A-n258A/R2 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n1A-n258R3 | CA_n1A-n258A/R2/R3 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n1A-n258R4 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n1A-n258R5 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n1A-n258R6 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n1A-n258R7 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n1A-n258R8 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n1A-n258R9 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n1A-n258R10 | CA_n1A-n258A/R2/R3/R4 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R10 |  |
| CA_n1A-n258(2A) | CA_n1A-n258A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n1A-n258(2G) | CA_n1A-n258A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n1A-n258(A-G) | CA_n1A-n258A/G | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n258 | CA_n258(A-G) |  |

Table 5.5A.1.1-1b: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n257A | CA_n2A-n257A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n2A-n257G | CA_n2A-n257A/G | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n2A-n257H | CA_n2A-n257A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n2A-n257I | CA_n2A-n257A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n2A-n257J | CA_n2A-n257A/G/H/I/J | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n2A-n257K | CA_n2A-n257A/G/H/I/J/K | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n2A-n257L | CA_n2A-n257A/G/H/I/J/K/L | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n2A-n257M | CA_n2A-n257A/G/H/I/J/K/L/M | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n2A-n257O | CA_n2A-n257A/O | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257O |  |
| CA_n2A-n257P | CA_n2A-n257A/O/P | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257P |  |
| CA_n2A-n257Q | CA_n2A-n257A/O/P/Q | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n257 | CA_n257Q |  |
| CA_n2A-n258A | CA_n2A-n258A | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n2A-n258G | CA_n2A-n258A/G | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n2A-n258H | CA_n2A-n258A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n2A-n258I | CA_n2A-n258A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n2A-n258J | CA_n2A-n258A/G/H/I/J | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n2A-n258K | CA_n2A-n258A/G/H/I/J/K | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n2A-n258L | CA_n2A-n258A/G/H/I/J/K/L | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n2A-n258M | CA_n2A-n258A/G/H/I/J/K/L/M | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n2A-n258O | CA_n2A-n258A/O | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258O |  |
| CA_n2A-n258P | CA_n2A-n258A/O/P | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258P |  |
| CA_n2A-n258Q | CA_n2A-n258A/O/P/Q | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n258 | CA_n258Q |  |
| CA_n2A-n260A | CA_n2A-n260A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n260G | CA_n2A-n260A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n2A-n260H | CA_n2A-n260A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n2A-n260I | CA_n2A-n260A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n2A-n260J | CA_n2A-n260A/G/H/I/J | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n2A-n260K | CA_n2A-n260A/G/H/I/J/K | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n2A-n260L | CA_n2A-n260A/G/H/I/J/K/L | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n2A-n260M | CA_n2A-n260A/G/H/I/J/K/L/M | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n2A-n260O | CA_n2A-n260A/O | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n260 | CA_n260O |  |
| CA_n2A-n260P | CA_n2A-n260A/O/P | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n260 | CA_n260P |  |
| CA_n2A-n260Q | CA_n2A-n260A/O/P/Q | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n260 | CA_n260Q |  |
| CA_n2A-n260R2 | CA_n2A-n260A/R2 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R2 |  |
| CA_n2A-n260R3 | CA_n2A-n260A/R2/R3 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R3 |  |
| CA_n2A-n260R4 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R4 |  |
| CA_n2A-n260R5 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R5 |  |
| CA_n2A-n260R6 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R6 |  |
| CA_n2A-n260R7 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R7 |  |
| CA_n2A-n260R8 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R8 |  |
| CA_n2A-n260R9 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R9 |  |
| CA_n2A-n260R10 | CA_n2A-n260A/R2/R3/R4 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260R10 |  |
| CA_n2(2A)-n260A | CA_n2A-n260A | n2 | CA_n2(2A) | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2(2A)-n260G | CA_n2A-n260A/G | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n2(2A)-n260H | CA_n2A-n260A/G/H | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n2(2A)-n260I | CA_n2A-n260A/G/H/I | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n2(2A)-n260J | CA_n2A-n260A/G/H/I/J | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n2(2A)-n260K | CA_n2A-n260A/G/H/I/J/K | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n2(2A)-n260L | CA_n2A-n260A/G/H/I/J/K/L | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n2(2A)-n260M | CA_n2A-n260A/G/H/I/J/K/L/M | n2 | CA_n2(2A) | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n2A-n261A | CA_n2A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n261G | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n2A-n261H | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n2A-n261I | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n2A-n261J | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n2A-n261K | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n2A-n261L | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n2A-n261M | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n2A-n261O | CA_n2A-n261A/O | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n261 | CA_n261O |  |
| CA_n2A-n261P | CA_n2A-n261A/O/P | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n261 | CA_n261P |  |
| CA_n2A-n261Q | CA_n2A-n261A/O/P/Q | n2 | 5, 10, 15, 20, 25, 30, 35, 40 | 0 |
|  |  | n261 | CA_n261Q |  |
| CA_n2A-n261(2A) | CA_n2A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n261(2G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n261(2H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n261(2I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2I) |  |
| CA_n2A-n261(3A) | CA_n2A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n261(4A) | CA_n2A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(4A) |  |
| CA_n2A-n261(A-G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n261(A-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n261(A-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n261(A-J) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-J) |  |
| CA_n2A-n261(A-K) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-K) |  |
| CA_n2A-n261(A-L) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-L) |  |
| CA_n2A-n261(G-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n261(H-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n261(G-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n261(A-G-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n261(A-G-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n261(2A-H) | CA_n2A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n261(2A-G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n261(2A-I) | CA_n2A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n261(A-2G) | CA_n2A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(A-2G) |  |

Table 5.5A.1.1-1c: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n257A | CA_n3A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n257D | CA_n3A-n257A/D | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n3A-n257G | CA_n3A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n257H | CA_n3A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n257I | CA_n3A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n257J | CA_n3A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n3A-n257K | CA_n3A-n257A/G/H/I/J | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n3A-n257L | CA_n3A-n257A/G/H/I/J/K | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n3A-n257M | CA_n3A-n257A/G/H/I/J/K/L | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n3A-n257(2A) | CA_n3A-n257A/(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n257 | CA_n257(2A) |  |
| CA_n3A-n257(2G) | CA_n3A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257(2G) |  |
|  | CA_n3A-n257A/G/(2G)/(A-G) | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n257 | CA_n257(2G) |  |
| CA_n3A-n257(A-G) | CA_n3A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257(A-G) |  |
|  | CA_n3A-n257A/G/(A-G) | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n257 | CA_n257(A-G) |  |
| CA_n3(2A)-n257A | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3(2A)-n257G | CA_n3A-n257A/G | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3(2A)-n257H | CA_n3A-n257A/G/H | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3(2A)-n257I | CA_n3A-n257A/G/H/I | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3(2A)-n257J | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n3(2A)-n257K | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n3(2A)-n257L | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n3(2A)-n257M | CA_n3A-n257A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n3B-n257A | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3B-n257G | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n3B-n257H | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n3B-n257I | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n3B-n257J | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n3B-n257K | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n3B-n257L | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n3B-n257M | CA_n3A-n257A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n3A-n258A | CA_n3A-n258A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3A-n258B | CA_n3A-n258A/B | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n3A-n258C | CA_n3A-n258A/B/C | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n3A-n258D | CA_n3A-n258A/D | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n3A-n258E | CA_n3A-n258A/D/E | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n3A-n258F | CA_n3A-n258A/D/E/F | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n3A-n258G | CA_n3A-n258A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n3A-n258H | CA_n3A-n258A/G/H | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n3A-n258I | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n3A-n258J | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n3A-n258K | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n3A-n258L | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n3A-n258M | CA_n3A-n258A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n3A-n258R2 | CA_n3A-n258A/R2 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n3A-n258R3 | CA_n3A-n258A/R2/R3 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n3A-n258R4 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n3A-n258R5 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n3A-n258R6 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n3A-n258R7 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n3A-n258R8 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n3A-n258R9 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n3A-n258R10 | CA_n3A-n258A/R2/R3/R4 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258R10 |  |
| CA_n3A-n258(2A) | CA_n3A-n258A/(2A) | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n3A-n258(2G) | CA_n3A-n258A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  | CA_n3A-n258A/G/(2G)/(A-G) | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n3A-n258(A-G) | CA_n3A-n258A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  | CA_n3A-n258A/G/(A-G) | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n3(2A)-n258A | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3(2A)-n258G | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n3(2A)-n258H | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n3(2A)-n258I | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n3(2A)-n258J | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n3(2A)-n258K | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n3(2A)-n258L | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n3(2A)-n258M | CA_n3A-n258A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n3B-n258A | CA_n3A-n258A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3B-n258B | CA_n3A-n258A/B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n3B-n258C | CA_n3A-n258A/B/C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n3B-n258D | CA_n3A-n258A/D | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n3B-n258E | CA_n3A-n258A/D/E | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n3B-n258F | CA_n3A-n258A/D/E/F | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n3B-n258G | CA_n3A-n258A/G | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n3B-n258H | CA_n3A-n258A/G/H | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n3B-n258I | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n3B-n258J | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n3B-n258K | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n3B-n258L | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n3B-n258M | CA_n3A-n258A/G/H/I | n3 | CA_n3B_BCS0 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n3B-n258R2 | CA_n3A-n258A/R2 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n3B-n258R3 | CA_n3A-n258A/R2/R3 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n3B-n258R4 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n3B-n258R5 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n3B-n258R6 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n3B-n258R7 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n3B-n258R8 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n3B-n258R9 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n3B-n258R10 | CA_n3A-n258A/R2/R3/R4 | n3 | CA_n3B | 0 |
|  | CA_n3A-n258A/R2 | n258 | CA_n258R10 |  |

Table 5.5A.1.1-1d: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration |  | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n257A |  | CA_n5A-n257A | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n5A-n257G |  | CA_n5A-n257A/G | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257G |  |
| CA_n5A-n257H |  | CA_n5A-n257A/G/H | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257H |  |
| CA_n5A-n257I |  | CA_n5A-n257A/G/H/I | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257I |  |
| CA_n5A-n257J |  | CA_n5A-n257A/G/H/I/J | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257J |  |
| CA_n5A-n257K |  | CA_n5A-n257A/G/H/I/J/K | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257K |  |
| CA_n5A-n257L |  | CA_n5A-n257A/G/H/I/J/K/L | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257L |  |
| CA_n5A-n257M |  | CA_n5A-n257A/G/H/I/J/K/L/M | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257M |  |
| CA_n5A-n257O |  | CA_n5A-n257A/O | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257O |  |
| CA_n5A-n257P |  | CA_n5A-n257A/O/P | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257P |  |
| CA_n5A-n257Q |  | CA_n5A-n257A/O/P/Q | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n257 |  | CA_n257Q |  |
| CA_n5A-n258A |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n5A-n258B |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258B |  |
| CA_n5A-n258C |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258C |  |
| CA_n5A-n258D |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258D |  |
| CA_n5A-n258E |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258E |  |
| CA_n5A-n258F |  | CA_n5A-n258A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258F |  |
| CA_n5A-n258G |  | CA_n5A-n258A/G | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258G |  |
| CA_n5A-n258H |  | CA_n5A-n258A/G/H | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258H |  |
| CA_n5A-n258I |  | CA_n5A-n258A/G/H/I | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258I |  |
| CA_n5A-n258J |  | CA_n5A-n258A/G/H/I | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258J |  |
| CA_n5A-n258K |  | CA_n5A-n258A/G/H/I | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258K |  |
| CA_n5A-n258L |  | CA_n5A-n258A/G/H/I | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258L |  |
| CA_n5A-n258M |  | CA_n5A-n258A/G/H/I | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n258 |  | CA_n258M |  |
| CA_n5A-n258O |  | CA_n5A-n258A/O | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n258 |  | CA_n258O |  |
| CA_n5A-n258P |  | CA_n5A-n258A/O/P | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n258 |  | CA_n258P |  |
| CA_n5A-n258Q |  | CA_n5A-n258A/O/P/Q | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n258 |  | CA_n258Q |  |
| CA_n5A-n260A |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n5A-n260G |  | CA_n5A-n260A/G | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260G |  |
| CA_n5A-n260H |  | CA_n5A-n260A/G/H | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260H |  |
| CA_n5A-n260I |  | CA_n5A-n260A/G/H/I | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260I |  |
| CA_n5A-n260J |  | CA_n5A-n260A/G/H/I/J | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260J |  |
| CA_n5A-n260K |  | CA_n5A-n260A/G/H/I/J/K | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260K |  |
| CA_n5A-n260L |  | CA_n5A-n260A/G/H/I/J/K/L | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260L |  |
| CA_n5A-n260M |  | CA_n5A-n260A/G/H/I/J/K/L/M | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260M |  |
| CA_n5A-n260O |  | CA_n5A-n260A/O | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n260 |  | CA_n260O |  |
| CA_n5A-n260P |  | CA_n5A-n260A/O/P | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n260 |  | CA_n260P |  |
| CA_n5A-n260Q |  | CA_n5A-n260A/O/P/Q | n5 |  | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n260 |  | CA_n260Q |  |
| CA_n5A-n260R2 |  | CA_n5A-n260A/R2 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R2 |  |
| CA_n5A-n260R3 |  | CA_n5A-n260A/R2/R3 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R3 |  |
| CA_n5A-n260R4 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R4 |  |
| CA_n5A-n260R5 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R5 |  |
| CA_n5A-n260R6 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R6 |  |
| CA_n5A-n260R7 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R7 |  |
| CA_n5A-n260R8 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R8 |  |
| CA_n5A-n260R9 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R9 |  |
| CA_n5A-n260R10 |  | CA_n5A-n260A/R2/R3/R4 | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260R10 |  |
| CA_n5A-n260(2A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(2A) |  |
| CA_n5A-n260(3A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(3A) |  |
| CA_n5A-n260(4A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(4A) |  |
| CA_n5A-n260(5A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(5A) |  |
| CA_n5A-n260(6A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(6A) |  |
| CA_n5A-n260(7A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(7A) |  |
| CA_n5A-n260(8A) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(8A) |  |
| CA_n5A-n260(2G) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(2G) |  |
| CA_n5A-n260(2H) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(2H) |  |
| CA_n5A-n260(A-G) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(A-G) |  |
| CA_n5A-n260(2A-G) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(2A-G) |  |
| CA_n5A-n260(A-H) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(A-H) |  |
| CA_n5A-n260(2A-2G) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(2A-2G) |  |
| CA_n5A-n260(3A-G) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(3A-G) |  |
| CA_n5A-n260(A-2G) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(A-2G) |  |
| CA_n5A-n260(G-H) |  | CA_n5A-n260A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n260 |  | CA_n260(G-H) |  |
| CA_n5A-n261A |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n261 |  | 50, 100, 200, 400 |  |
| CA_n5A-n261(2A) |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n261 |  | CA_n261(2A) |  |
| CA_n5A-n261(3A) |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n261 |  | CA_n261(3A) |  |
| CA_n5A-n261(4A) |  | CA_n5A-n261A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  |  | n261 |  | CA_n261(4A) |  |
| CA_n5A-n261G | CA_n5A-n261A/G |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261G |  |
| CA_n5A-n261H | CA_n5A-n261A/G/H |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261H |  |
| CA_n5A-n261I | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261I |  |
| CA_n5A-n261J | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261J |  |
| CA_n5A-n261K | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261K |  |
| CA_n5A-n261L | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261L |  |
| CA_n5A-n261M | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261M |  |
| CA_n5A-n261O | CA_n5A-n261A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261O |  |
| CA_n5A-n261P | CA_n5A-n261A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261P |  |
| CA_n5A-n261Q | CA_n5A-n261A |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261Q |  |
| CA_n5A-n261(2G) | CA_n5A-n261A/G |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n5A-n261(2H) | CA_n5A-n261A/G/H |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n5A-n261(2I) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(2I) |  |
| CA_n5A-n261(A-G) | CA_n5A-n261A/G |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n5A-n261(A-H) | CA_n5A-n261A/G/H |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n5A-n261(A-I) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n5A-n261(A-J) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-J) |  |
| CA_n5A-n261(A-K) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-K) |  |
| CA_n5A-n261(A-L) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-L) |  |
| CA_n5A-n261(G-H) | CA_n5A-n261A/G/H |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n5A-n261(G-J) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(G-J) |  |
| CA_n5A-n261(H-I) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n5A-n261(G-I) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n5A-n261(A-G-H) | CA_n5A-n261A/G/H |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n5A-n261(A-G-I) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n5A-n261(2A-H) | CA_n5A-n261A/G/H |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n5A-n261(2A-G) | CA_n5A-n261A/G |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n5A-n261(3A-G) | CA_n5A-n261A/G |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(3A-G) |  |
| CA_n5A-n261(2A-I) | CA_n5A-n261A/G/H/I |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n5A-n261(A-2G) | CA_n5A-n261A/G |  | n5 | 5, 10, 15, 20 | 0 |
|  |  |  | n261 | CA_n261(A-2G) |  |

Table 5.5A.1.1-1e: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n257A | CA_n7A-n257A | n7 |  | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n7 |  | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n257 |  | See n257 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n7A-n257G | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257G |  |  |  |
|  | CA_n7A-n257A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n7A-n257H | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257H |  |  |  |
|  | CA_n7A-n257A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n7A-n257I | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257I |  |  |  |
|  | CA_n7A-n257A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n7A-n257J | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257J |  |  |  |
|  | CA_n7A-n257A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257J |  |  |  |
| CA_n7A-n257K | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257K |  |  |  |
|  | CA_n7A-n257A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257K |  |  |  |
| CA_n7A-n257L | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257L |  |  |  |
|  | CA_n7A-n257A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257L |  |  |  |
| CA_n7A-n257M | CA_n7A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257M |  |  |  |
|  | CA_n7A-n257A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n257 | CA_n257M |  |  |  |
| CA_n7A-n257O | CA_n7A-n257A/O | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257O |  |  |  |
| CA_n7A-n257P | CA_n7A-n257A/O/P | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257P |  |  |  |
| CA_n7A-n257Q | CA_n7A-n257A/O/P/Q | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n257 | CA_n257Q |  |  |  |
| CA_n7A-n258A | CA_n7A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n7A-n258B | CA_n7A-n258A/B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258B |  |  |  |
| CA_n7A-n258C | CA_n7A-n258A/B/C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258C |  |  |  |
| CA_n7A-n258D | CA_n7A-n258A/D | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258D |  |  |  |
| CA_n7A-n258E | CA_n7A-n258A/D/E | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258E |  |  |  |
| CA_n7A-n258F | CA_n7A-n258A/D/E/F | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258F |  |  |  |
| CA_n7A-n258G | CA_n7A-n258A/G | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258G |  |  |  |
| CA_n7A-n258H | CA_n7A-n258A/G/H | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258H |  |  |  |
| CA_n7A-n258I | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258I |  |  |  |
| CA_n7A-n258J | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258J |  |  |  |
| CA_n7A-n258K | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258K |  |  |  |
| CA_n7A-n258L | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258L |  |  |  |
| CA_n7A-n258M | CA_n7A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258M |  |  |  |
| CA_n7A-n258O | CA_n7A-n258A/O | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258O |  |  |  |
| CA_n7A-n258P | CA_n7A-n258A/O/P | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258P |  |  |  |
| CA_n7A-n258Q | CA_n7A-n258A/O/P/Q | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258Q |  |  |  |
| CA_n7A-n258R2 | CA_n7A-n258A/R2 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R2 |  |  |  |
| CA_n7A-n258R3 | CA_n7A-n258A/R2/R3 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R3 |  |  |  |
| CA_n7A-n258R4 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R4 |  |  |  |
| CA_n7A-n258R5 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R5 |  |  |  |
| CA_n7A-n258R6 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R6 |  |  |  |
| CA_n7A-n258R7 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R7 |  |  |  |
| CA_n7A-n258R8 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R8 |  |  |  |
| CA_n7A-n258R9 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R9 |  |  |  |
| CA_n7A-n258R10 | CA_n7A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n258 | CA_n258R10 |  |  |  |
| CA_n7B-n258A | CA_n7A-n258A | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n7B-n258B | CA_n7A-n258A/B | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258B |  |  |  |
| CA_n7B-n258C | CA_n7A-n258A/B/C | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258C |  |  |  |
| CA_n7B-n258D | CA_n7A-n258A/D | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258D |  |  |  |
| CA_n7B-n258E | CA_n7A-n258A/D/E | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258E |  |  |  |
| CA_n7B-n258F | CA_n7A-n258A/D/E/F | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258F |  |  |  |
| CA_n7B-n258G | CA_n7A-n258A/G | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258G |  |  |  |
| CA_n7B-n258H | CA_n7A-n258A/G/H | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258H |  |  |  |
| CA_n7B-n258I | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258I |  |  |  |
| CA_n7B-n258J | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258J |  |  |  |
| CA_n7B-n258K | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258K |  |  |  |
| CA_n7B-n258L | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258L |  |  |  |
| CA_n7B-n258M | CA_n7A-n258A/G/H/I | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258M |  |  |  |
| CA_n7B-n258R2 | CA_n7A-n258A/R2 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R2 |  |  |  |
| CA_n7B-n258R3 | CA_n7A-n258A/R2/R3 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R3 |  |  |  |
| CA_n7B-n258R4 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R4 |  |  |  |
| CA_n7B-n258R5 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R5 |  |  |  |
| CA_n7B-n258R6 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R6 |  |  |  |
| CA_n7B-n258R7 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R7 |  |  |  |
| CA_n7B-n258R8 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R8 |  |  |  |
| CA_n7B-n258R9 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R9 |  |  |  |
| CA_n7B-n258R10 | CA_n7A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |  |
|  |  | n258 | CA_n258R10 |  |  |  |
| CA_n7A-n260A | CA_n7A-n260A | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |  |
| CA_n7A-n260G | CA_n7A-n260A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260G |  |  |  |
| CA_n7A-n260H | CA_n7A-n260A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260H |  |  |  |
| CA_n7A-n260I | CA_n7A-n260A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260I |  |  |  |
| CA_n7A-n260J | CA_n7A-n260A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260J |  |  |  |
| CA_n7A-n260K | CA_n7A-n260A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260K |  |  |  |
| CA_n7A-n260L | CA_n7A-n260A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260L |  |  |  |
| CA_n7A-n260M | CA_n7A-n260A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |  |
|  |  | n260 | CA_n260M |  |  |  |
| CA_n7A-n260O | CA_n7A-n260A/O | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n260 | CA_n260O |  |  |  |
| CA_n7A-n260P | CA_n7A-n260A/O/P | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n260 | CA_n260P |  |  |  |
| CA_n7A-n260Q | CA_n7A-n260A/O/P/Q | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n260 | CA_n260Q |  |  |  |
| CA_n7A-n261A | CA_n7A-n261A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n7A-n261G | CA_n7A-n261A/G | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261G |  |  |  |
| CA_n7A-n261H | CA_n7A-n261A/G/H | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261H |  |  |  |
| CA_n7A-n261I | CA_n7A-n261A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261I |  |  |  |
| CA_n7A-n261J | CA_n7A-n261A/G/H/I/J | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261J |  |  |  |
| CA_n7A-n261K | CA_n7A-n261A/G/H/I/J/K | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261K |  |  |  |
| CA_n7A-n261L | CA_n7A-n261A/G/H/I/J/K/L | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261L |  |  |  |
| CA_n7A-n261M | CA_n7A-n261A/G/H/I/J/K/L/M | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261M |  |  |  |
| CA_n7A-n261O | CA_n7A-n261A/O | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261O |  |  |  |
| CA_n7A-n261P | CA_n7A-n261A/O/P | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261P |  |  |  |
| CA_n7A-n261Q | CA_n7A-n261A/O/P/Q | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  | 0 |  |
|  |  | n261 | CA_n261Q |  |  |  |
| CA_n8A-n257A | CA_n8A-n257A | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |  |
| CA_n8A-n257D | - | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257D |  |  |  |
| CA_n8A-n257E | - | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257E |  |  |  |
| CA_n8A-n257F | - | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257F |  |  |  |
| CA_n8A-n257G | CA_n257G CA_n8A-n257A/G | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257G |  |  |  |
| CA_n8A-n257H | CA_n257G/H CA_n8A-n257A/G/H | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257H |  |  |  |
| CA_n8A-n257I | CA_n257G/H/I CA_n8A-n257A/G/H/I | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257I |  |  |  |
| CA_n8A-n257J | CA_n257G/H/I/J CA_n8A-n257A/G/H/I/J | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257J |  |  |  |
| CA_n8A-n257K | CA_n257G/H/I/J/K CA_n8A-n257A/G/H/I/J/K | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257K |  |  |  |
| CA_n8A-n257L | CA_n8A-n257A/G/H/I/J/K | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257L |  |  |  |
| CA_n8A-n257M | CA_n8A-n257A/G/H/I/J/K | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n257 | CA_n257M |  |  |  |
| CA_n8A-n258A | CA_n8A-n258A | n8 | 5, 10, 15, 20 |  | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |  |
| CA_n8A-n258B | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258B |  |  |
| CA_n8A-n258C | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258C |  |  |
| CA_n8A-n258D | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258D |  |  |
| CA_n8A-n258E | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258E |  |  |
| CA_n8A-n258F | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258F |  |  |
| CA_n8A-n258G | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n8A-n258H | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n8A-n258I | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n8A-n258J | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n8A-n258K | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258K |  |  |
| CA_n8A-n258L | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258L |  |  |
| CA_n8A-n258M | CA_n8A-n258A | n8 |  | 5, 10, 15, 20 |  | 0 |
|  |  | n258 |  | CA_n258M |  |  |

Table 5.5A.1.1-1f: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n12A-n257A | CA_n12A-n257A | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n12A-n257G | CA_n12A-n257A/G | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n12A-n257H | CA_n12A-n257A/G/H | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n12A-n257I | CA_n12A-n257A/G/H/I | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n12A-n257J | CA_n12A-n257A/G/H/I/J | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n12A-n257K | CA_n12A-n257A/G/H/I/J/K | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n12A-n257L | CA_n12A-n257A/G/H/I/J/K/L | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n12A-n257M | CA_n12A-n257A/G/H/I/J/K/L/M | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n12A-n257O | CA_n12A-n257A/O | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257O |  |
| CA_n12A-n257P | CA_n12A-n257A/O/P | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257P |  |
| CA_n12A-n257Q | CA_n12A-n257A/O/P/Q | n12 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257Q |  |
| CA_n12A-n258A | CA_n12A-n258A | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n12A-n258G | CA_n12A-n258A/G | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n12A-n258H | CA_n12A-n258A/G/H | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n12A-n258I | CA_n12A-n258A/G/H/I | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n12A-n258J | CA_n12A-n258A/G/H/I/J | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n12A-n258K | CA_n12A-n258A/G/H/I/J/K | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258K |  |
| CA_n12A-n258L | CA_n12A-n258A/G/H/I/J/K/L | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258L |  |
| CA_n12A-n258M | CA_n12A-n258A/G/H/I/J/K/L/M | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258M |  |
| CA_n12A-n258O | CA_n12A-n258A/O | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258O |  |
| CA_n12A-n258P | CA_n12A-n258A/O/P | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258P |  |
| CA_n12A-n258Q | CA_n12A-n258A/O/P/Q | n12 |  | 5, 10, 15 | 0 |
|  |  | n258 |  | CA_n258Q |  |
| CA_n12A-n260A | CA_n12A-n260A | n12 |  | 5, 10, 15 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n12A-n260G | CA_n12A-n260A/G | n12 |  | 5, 10, 15 | 0 |
|  |  | n260 |  | CA_n260G |  |
| CA_n12A-n260H | CA_n12A-n260A/G/H | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260H |  |  |
| CA_n12A-n260I | CA_n12A-n260A/G/H/I | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260I |  |  |
| CA_n12A-n260J | CA_n12A-n260A/G/H/I/J | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260J |  |  |
| CA_n12A-n260K | CA_n12A-n260A/G/H/I/J/K | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260K |  |  |
| CA_n12A-n260L | CA_n12A-n260A/G/H/I/J/K/L | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260L |  |  |
| CA_n12A-n260M | CA_n12A-n260A/G/H/I/J/K/L/M | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260M |  |  |
| CA_n12A-n260O | CA_n12A-n260A/O | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260O |  |  |
| CA_n12A-n260P | CA_n12A-n260A/O/P | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260P |  |  |
| CA_n12A-n260Q | CA_n12A-n260A/O/P/Q | n12 | 5, 10, 15 |  | 0 |
|  |  | n260 | CA_n260Q |  |  |
| CA_n12A-n261A | CA_n12A-n261A | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n12A-n261G | CA_n12A-n261A/G | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261G |  |  |
| CA_n12A-n261H | CA_n12A-n261A/G/H | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261H |  |  |
| CA_n12A-n261I | CA_n12A-n261A/G/H/I | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261I |  |  |
| CA_n12A-n261J | CA_n12A-n261A/G/H/I/J | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261J |  |  |
| CA_n12A-n261K | CA_n12A-n261A/G/H/I/J/K | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261K |  |  |
| CA_n12A-n261L | CA_n12A-n261A/G/H/I/J/K/L | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261L |  |  |
| CA_n12A-n261M | CA_n12A-n261A/G/H/I/J/K/L/M | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261M |  |  |
| CA_n12A-n261O | CA_n12A-n261A/O | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261O |  |  |
| CA_n12A-n261P | CA_n12A-n261A/O/P | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261P |  |  |
| CA_n12A-n261Q | CA_n12A-n261A/O/P/Q | n12 | 5, 10, 15 |  | 0 |
|  |  | n261 | CA_n261Q |  |  |
| CA_n14A-n260A | CA_n14A-n260A | n14 | 5, 10 |  | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n14A-n260G | CA_n14A-n260A/G | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260G |  |  |
| CA_n14A-n260H | CA_n14A-n260A/G/H | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260H |  |  |
| CA_n14A-n260I | CA_n14A-n260A/G/H/I | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260I |  |  |
| CA_n14A-n260J | CA_n14A-n260A/G/H/I/J | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260J |  |  |
| CA_n14A-n260K | CA_n14A-n260A/G/H/I/J/K | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260K |  |  |
| CA_n14A-n260L | CA_n14A-n260A/G/H/I/J/K/L | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260L |  |  |
| CA_n14A-n260M | CA_n14A-n260A/G/H/I/J/K/L/M | n14 | 5, 10 |  | 0 |
|  |  | n260 | CA_n260M |  |  |
| CA_n18A-n257A | CA_n18A-n257A | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n18A-n257G | CA_n18A-n257A/G | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n18A-n257H | CA_n18A-n257A/G/H | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n18A-n257I | CA_n18A-n257A/G/H/I | n18 |  | 5, 10, 15 | 0 |
|  |  | n257 |  | CA_n257I |  |

Table 5.5A.1.1-1g: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n257A | CA_n25A-n257A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | See n257 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n257G | CA_n25A-n257A/G | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n25A-n257H | CA_n25A-n257A/G/H | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n25A-n257I | CA_n25A-n257A/G/H/I | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n25A-n257J | CA_n25A-n257A/G/H/I/J | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257J |  |
| CA_n25A-n257K | CA_n25A-n257A/G/H/I/J/K | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257K |  |
| CA_n25A-n257L | CA_n25A-n257A/G/H/I/J/K/L | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257L |  |
| CA_n25A-n257M | CA_n25A-n257A/G/H/I/J/K/L/M | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257M |  |
| CA_n25A-n257O | CA_n25A-n257A/O | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n257 | CA_n257O |  |
| CA_n25A-n257P | CA_n25A-n257A/O/P | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n257 | CA_n257P |  |
| CA_n25A-n257Q | CA_n25A-n257A/O/P/Q | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n257 | CA_n257Q |  |
| CA_n25A-n258A | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n258(2A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n25A-n258(3A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n25A-n258(4A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n25A-n258(5A) | CA_n25A-n258A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n25A-n258G | CA_n25A-n258A/G | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n25A-n258(2G) | CA_n25A-n258A/G | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n25A-n258H | CA_n25A-n258A/G/H | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n25A-n258I | CA_n25A-n258A/G/H/I | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258I |  |
| CA_n25A-n258J | CA_n25A-n258A/G/H/I/J | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258J |  |
| CA_n25A-n258(A-G) | CA_n25A-n258A/G | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n25A-n258(A-H) | CA_n25A-n258A/G/H | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n25A-n258(G-H) | CA_n25A-n258A/G/H | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n25A-n260A | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n260(2A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n25A-n260(3A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n25A-n260(4A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n25A-n260(5A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n25A-n260(6A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n25A-n260(7A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n25A-n260(8A) | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n25A-n260G | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n25A-n260A/G | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n25A-n260H | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n25A-n260A/G/H | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n25A-n260I | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n25A-n260A/G/H/I | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n25A-n260J | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n25A-n260A/G/H/I/J | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n25A-n260K | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n25A-n260A/G/H/I/J/K | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n25A-n260L | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n25A-n260A/G/H/I/J/K/L | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n25A-n260M | CA_n25A-n260A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n25A-n260A/G/H/I/J/K/L/M | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n25A-n260O | CA_n25A-n260A/O | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n260 | CA_n260O |  |
| CA_n25A-n260P | CA_n25A-n260A/O/P | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n260 | CA_n260P |  |
| CA_n25A-n260Q | CA_n25A-n260A/O/P/Q | n25 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n260 | CA_n260Q |  |
| CA_n25A-n261A | CA_n25A-n261A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n261(2A) | CA_n25A-n261A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n26A-n258A | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n26A-n258B | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n26A-n258C | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n26A-n258D | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n26A-n258E | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n26A-n258F | CA_n26A-n258A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n26A-n258G | CA_n26A-n258A/G | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n26A-n258H | CA_n26A-n258A/G/H | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n26A-n258I | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n26A-n258J | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n26A-n258K | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n26A-n258L | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n26A-n258M | CA_n26A-n258A/G/H/I | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n26(2A)-n258A | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n26(2A)-n258B | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n26(2A)-n258C | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n26(2A)-n258D | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n26(2A)-n258E | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n26(2A)-n258F | CA_n26A-n258A | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n26(2A)-n258G | CA_n26A-n258A/G | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n26(2A)-n258H | CA_n26A-n258A/G/H | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n26(2A)-n258I | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n26(2A)-n258J | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n26(2A)-n258K | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n26(2A)-n258L | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n26(2A)-n258M | CA_n26A-n258A/G/H/I | n26 | CA_n26(2A) | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n26A-n258R2 | CA_n26A-n258A/R2 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n26A-n258R3 | CA_n26A-n258A/R2/R3 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n26A-n258R4 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n26A-n258R5 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n26A-n258R6 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n26A-n258R7 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n26A-n258R8 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n26A-n258R9 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n26A-n258R10 | CA_n26A-n258A/R2/R3/R4 | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R10 |  |

##### Table 5.5A.1.1-1h ~ Table 5.5A.1.1-1k

Table 5.5A.1.1-1h: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n257A | CA_n28A-n257A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n257D | CA_n28A-n257A/D | n28 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n28A-n257G | CA_n257G CA_n28A-n257A/G | n28 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n28A-n257H | CA_n257G/H CA_n28A-n257A/G/H | n28 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n28A-n257I | CA_n257G/H/I CA_n28A-n257A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n28A-n258A | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n28A-n258B | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n28A-n258C | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n28A-n258D | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n28A-n258E | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n28A-n258F | CA_n28A-n258A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n28A-n258G | CA_n28A-n258A/G | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n28A-n258H | CA_n28A-n258A/G/H | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n28A-n258I | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n28A-n258J | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n28A-n258K | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n28A-n258L | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n28A-n258M | CA_n28A-n258A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n28A-n258R2 | CA_n28A-n258A/R2 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R2 |  |
| CA_n28A-n258R3 | CA_n28A-n258A/R2/R3 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R3 |  |
| CA_n28A-n258R4 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R4 |  |
| CA_n28A-n258R5 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R5 |  |
| CA_n28A-n258R6 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R6 |  |
| CA_n28A-n258R7 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R7 |  |
| CA_n28A-n258R8 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R8 |  |
| CA_n28A-n258R9 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R9 |  |
| CA_n28A-n258R10 | CA_n28A-n258A/R2/R3/R4 | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n258 | CA_n258R10 |  |
| CA_n30A-n257A | CA_n30A-n257A | n30 | 5, 10 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n30A-n257G | CA_n30A-n257A/G | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n30A-n257H | CA_n30A-n257A/G/H | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n30A-n257I | CA_n30A-n257A/G/H/I | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n30A-n257J | CA_n30A-n257A/G/H/I/J | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n30A-n257K | CA_n30A-n257A/G/H/I/J/K | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n30A-n257L | CA_n30A-n257A/G/H/I/J/K/L | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n30A-n257M | CA_n30A-n257A/G/H/I/J/K/L/M | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n30A-n257O | CA_n30A-n257A/O | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257O |  |
| CA_n30A-n257P | CA_n30A-n257A/O/P | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257P |  |
| CA_n30A-n257Q | CA_n30A-n257A/O/P/Q | n30 | 5, 10 | 0 |
|  |  | n257 | CA_n257Q |  |
| CA_n30A-n258A | CA_n30A-n258A | n30 | 5, 10 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n30A-n258G | CA_n30A-n258A/G | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n30A-n258H | CA_n30A-n258A/G/H | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n30A-n258I | CA_n30A-n258A/G/H/I | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n30A-n258J | CA_n30A-n258A/G/H/I/J | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n30A-n258K | CA_n30A-n258A/G/H/I/J/K | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n30A-n258L | CA_n30A-n258A/G/H/I/J/K/L | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n30A-n258M | CA_n30A-n258A/G/H/I/J/K/L/M | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n30A-n258O | CA_n30A-n258A/O | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258O |  |
| CA_n30A-n258P | CA_n30A-n258A/O/P | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258P |  |
| CA_n30A-n258Q | CA_n30A-n258A/O/P/Q | n30 | 5, 10 | 0 |
|  |  | n258 | CA_n258Q |  |
| CA_n30A-n260A | CA_n30A-n260A | n30 | 5, 10 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n30A-n260G | CA_n30A-n260A/G | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n30A-n260H | CA_n30A-n260A/G/H | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n30A-n260I | CA_n30A-n260A/G/H/I | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n30A-n260J | CA_n30A-n260A/G/H/I/J | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n30A-n260K | CA_n30A-n260A/G/H/I/J/K | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n30A-n260L | CA_n30A-n260A/G/H/I/J/K/L | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n30A-n260M | CA_n30A-n260A/G/H/I/J/K/L/M | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n30A-n260O | CA_n30A-n260A/O | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260O |  |
| CA_n30A-n260P | CA_n30A-n260A/O/P | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260P |  |
| CA_n30A-n260Q | CA_n30A-n260A/O/P/Q | n30 | 5, 10 | 0 |
|  |  | n260 | CA_n260Q |  |
| CA_n30A-n261A | CA_n30A-n261A | n30 | 5, 10 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n30A-n261G | CA_n30A-n261A/G | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n30A-n261H | CA_n30A-n261A/G/H | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n30A-n261I | CA_n30A-n261A/G/H/I | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n30A-n261J | CA_n30A-n261A/G/H/I/J | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n30A-n261K | CA_n30A-n261A/G/H/I/J/K | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n30A-n261L | CA_n30A-n261A/G/H/I/J/K/L | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n30A-n261M | CA_n30A-n261A/G/H/I/J/K/L/M | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n30A-n261O | CA_n30A-n261A/O | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261O |  |
| CA_n30A-n261P | CA_n30A-n261A/O/P | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261P |  |
| CA_n30A-n261Q | CA_n30A-n261A/O/P/Q | n30 | 5, 10 | 0 |
|  |  | n261 | CA_n261Q |  |
| CA_n34A-n258A | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n34A-n258B | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n34A-n258C | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n34A-n258D | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n34A-n258E | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n34A-n258F | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n34A-n258G | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n34A-n258H | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n34A-n258I | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n34A-n258J | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n34A-n258K | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n34A-n258L | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n34A-n258M | CA_n34A-n258A | n34 | 5, 10, 15 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n38A-n257A | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n38A-n257G | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n38A-n257H | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n38A-n257I | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n38A-n257J | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n38A-n257K | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n38A-n257L | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n38A-n257M | CA_n38A-n257A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n38A-n258A | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n38A-n258G | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n38A-n258H | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n38A-n258I | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n38A-n258J | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n38A-n258K | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n38A-n258L | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n38A-n258M | CA_n38A-n258A | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n39A-n258A | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n39A-n258B | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n39A-n258C | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n39A-n258D | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n39A-n258E | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n39A-n258F | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n39A-n258G | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n39A-n258H | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n39A-n258I | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n39A-n258J | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n39A-n258K | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n39A-n258L | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n39A-n258M | CA_n39A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258M |  |

Table 5.5A.1.1-1i: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n257A | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n40A-n257D | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n40A-n257E | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257E |  |
| CA_n40A-n257F | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257F |  |
| CA_n40A-n257G | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n40A-n257H | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n40A-n257I | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n40A-n257J | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n40A-n257K | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n40A-n257L | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n40A-n257M | CA_n40A-n257A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n40B-n257A | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n40B-n257D | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257D |  |
| CA_n40B-n257E | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257E |  |
| CA_n40B-n257F | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257F |  |
| CA_n40B-n257G | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n40B-n257H | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n40B-n257I | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n40B-n257J | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n40B-n257K | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n40B-n257L | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n40B-n257M | CA_n40B CA_n40A-n257A | n40 | CA_n40B | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n40A-n258A | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n40A-n258B | CA_n40A-n258A | n40 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n40A-n258C | CA_n40A-n258A | n40 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n40A-n258D | CA_n40A-n258A | n40 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n40A-n258E | CA_n40A-n258A | n40 | 5, 10, 15, 20 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n40A-n258F | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n40A-n258G | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n40A-n258H | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n40A-n258I | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n40A-n258J | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n40A-n258K | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n40A-n258L | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n40A-n258M | CA_n40A-n258A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n258 | CA_n258M |  |

Table 5.5A.1.1-1j: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n257A | CA_n41A-n257A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41A-n257G | CA_n257G CA_n41A-n257A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n41A-n257H | CA_n257G/H CA_n41A-n257A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n41A-n257I | CA_n257G/H/I CA_n41A-n257A/G/H/I | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n41A-n257J | CA_n41A-n257A/G/H/I/J | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257J |  |
| CA_n41A-n257K | CA_n41A-n257A/G/H/I/J/K | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257K |  |
| CA_n41A-n257L | CA_n41A-n257A/G/H/I/J/K/L | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257L |  |
| CA_n41A-n257M | CA_n41A-n257A/G/H/I/J/K/L/M | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257M |  |
| CA_n41A-n257O | CA_n41A-n257A/O | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257O |  |
| CA_n41A-n257P | CA_n41A-n257A/O/P | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257P |  |
| CA_n41A-n257Q | CA_n41A-n257A/O/P/Q | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n257 | CA_n257Q |  |
| CA_n41(2A)-n257A | CA_n41A-n257A | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41(2A)-n257G | CA_n41A-n257A/G | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | CA_n257G |  |
| CA_n41(2A)-n257H | CA_n41A-n257A/G/H | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | CA_n257H |  |
| CA_n41(2A)-n257I | CA_n41A-n257A/G/H/I | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n257 | CA_n257I |  |
| CA_n41A-n258A | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n258B | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n41A-n258C | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n41A-n258D | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n41A-n258E | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n41A-n258F | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n41A-n258G | CA_n41A-n258A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41A-n258H | CA_n41A-n258A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41A-n258I | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258I |  |
|  | CA_n41A-n258A/G/H/I | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258I |  |
| CA_n41A-n258J | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258J |  |
|  | CA_n41A-n258A/G/H/I/J | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258J |  |
| CA_n41A-n258K | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n41A-n258L | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n41A-n258M | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n41A-n258O | CA_n41A-n258A/O | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258O |  |
| CA_n41A-n258P | CA_n41A-n258A/O/P | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258P |  |
| CA_n41A-n258Q | CA_n41A-n258A/O/P/Q | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n258 | CA_n258Q |  |
| CA_n41A-n258(2A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41A-n258(3A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41A-n258(4A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41A-n258(5A) | CA_n41A-n258A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n41A-n258(2G) | CA_n41A-n258A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41A-n258(A-G) | CA_n41A-n258A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41A-n258(A-H) | CA_n41A-n258A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41A-n258(G-H) | CA_n41A-n258A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41C-n258A | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n258B | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258B |  |
| CA_n41C-n258C | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258C |  |
| CA_n41C-n258D | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258D |  |
| CA_n41C-n258E | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258E |  |
| CA_n41C-n258F | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258F |  |
| CA_n41C-n258(2A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41C-n258(3A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41C-n258(4A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41C-n258(5A) | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n41C-n258G | CA_n41A-n258A/G | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41C-n258(2G) | CA_n41A-n258A/G | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41C-n258H | CA_n41A-n258A/G/H | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41C-n258I | CA_n41A-n258A/G/H/I | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258I |  |
| CA_n41C-n258J | CA_n41A-n258A/G/H/I/J | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258J |  |
| CA_n41C-n258K | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n41C-n258L | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n41C-n258M | CA_n41A-n258A | n41 | CA_n41C | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n41C-n258(A-G) | CA_n41A-n258A/G | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41C-n258(A-H) | CA_n41A-n258A/G/H | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41C-n258(G-H) | CA_n41A-n258A/G/H | n41 | CA_n41C_BCS1 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41(2A)-n258A | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n41(2A)-n258(2A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n41(2A)-n258(3A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n41(2A)-n258(4A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n41(2A)-n258(5A) | CA_n41A-n258A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n41(2A)-n258G | CA_n41A-n258A/G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n41(2A)-n258(2G) | CA_n41A-n258A/G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n41(2A)-n258H | CA_n41A-n258A/G/H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n41(2A)-n258I | CA_n41A-n258A/G/H/I | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258I |  |
| CA_n41(2A)-n258J | CA_n41A-n258A/G/H/I/J | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258J |  |
| CA_n41(2A)-n258(A-G) | CA_n41A-n258A/G | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n41(2A)-n258(A-H) | CA_n41A-n258A/G/H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n41(2A)-n258(G-H) | CA_n41A-n258A/G/H | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n41A-n260A | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n260G | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n41A-n260A/G | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41A-n260H | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n41A-n260A/G/H | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41A-n260I | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n41A-n260A/G/H/I | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41A-n260J | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n41A-n260A/G/H/I/J | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41A-n260K | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n41A-n260A/G/H/I/J/K | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41A-n260L | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n41A-n260A/G/H/I/J/K/L | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41A-n260M | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n41A-n260A/G/H/I/J/K/L/M | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41A-n260O | CA_n41A-n260A/O | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n260 | CA_n260O |  |
| CA_n41A-n260P | CA_n41A-n260A/O/P | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n260 | CA_n260P |  |
| CA_n41A-n260Q | CA_n41A-n260A/O/P/Q | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n260 | CA_n260Q |  |
| CA_n41A-n260(2A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41A-n260(3A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41A-n260(4A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41A-n260(5A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41A-n260(6A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41A-n260(7A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41A-n260(8A) | CA_n41A-n260A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41(2A)-n260A | CA_n41A-n260A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n260(2A) | CA_n41A-n260A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41(2A)-n260(3A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41(2A)-n260(4A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41(2A)-n260(5A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41(2A)-n260(6A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41(2A)-n260(7A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41(2A)-n260(8A) | CA_n41A-n260A | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41(2A)-n260G | CA_n41A-n260A/G | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260G |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41(2A)-n260H | CA_n41A-n260A/G/H | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260H |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41(2A)-n260I | CA_n41A-n260A/G/H/I | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260I |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41(2A)-n260J | CA_n41A-n260A/G/H/I/J | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260J |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41(2A)-n260K | CA_n41A-n260A/G/H/I/J/K | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260K |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41(2A)-n260L | CA_n41A-n260A/G/H/I/J/K/L | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260L |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41(2A)-n260M | CA_n41A-n260A/G/H/I/J/K/L/M | n41 | CA_n41(2A) | 0 |
|  |  | n260 | CA_n260M |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41C-n260A | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n260(2A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n41C-n260(3A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n41C-n260(4A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n41C-n260(5A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n41C-n260(6A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n41C-n260(7A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n41C-n260(8A) | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n41C-n260G | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260G |  |
|  | CA_n41A-n260A/G | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n41C-n260H | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260H |  |
|  | CA_n41A-n260A/G/H | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n41C-n260I | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260I |  |
|  | CA_n41A-n260A/G/H/I | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n41C-n260J | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260J |  |
|  | CA_n41A-n260A/G/H/I/J | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n41C-n260K | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260K |  |
|  | CA_n41A-n260A/G/H/I/J/K | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n41C-n260L | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260L |  |
|  | CA_n41A-n260A/G/H/I/J/K/L | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n41C-n260M | CA_n41A-n260A | n41 | CA_n41C | 0 |
|  |  | n260 | CA_n260M |  |
|  | CA_n41A-n260A/G/H/I/J/K/L/M | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n41A-n261A | CA_n41A-n261A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n261G | CA_n41A-n261A/G | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n41A-n261H | CA_n41A-n261A/G/H | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n41A-n261I | CA_n41A-n261A/G/H/I | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n41A-n261J | CA_n41A-n261A/G/H/I/J | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n41A-n261K | CA_n41A-n261A/G/H/I/J/K | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n41A-n261L | CA_n41A-n261A/G/H/I/J/K/L | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n41A-n261M | CA_n41A-n261A/G/H/I/J/K/L/M | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n41A-n261O | CA_n41A-n261A/O | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261O |  |
| CA_n41A-n261P | CA_n41A-n261A/O/P | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261P |  |
| CA_n41A-n261Q | CA_n41A-n261A/O/P/Q | n41 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n261 | CA_n261Q |  |
| CA_n41A-n261(2A) | CA_n41A-n261A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n41C-n261A | CA_n41A-n261A | n41 | CA_n41C | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n261A | CA_n41A-n261A | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n261(2A) | CA_n41A-n261A | n41 | CA_n41C | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n41(2A)-n261(2A) | CA_n41A-n261A | n41 | CA_n41(2A) BCS1 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |

Table 5.5A.1.1-1k: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n258A | - | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n48A-n260A | CA_n48A-n260A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48A-n260G | CA_n48A-n260A/G | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48A-n260H | CA_n48A-n260A/G/H | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48A-n260I | CA_n48A-n260A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48A-n260J | CA_n48A-n260A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48A-n260K | CA_n48A-n260A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48A-n260L | CA_n48A-n260A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48A-n260M | CA_n48A-n260A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48A-n260R2 | CA_n48A-n260A/R2 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R2 |  |
| CA_n48A-n260R3 | CA_n48A-n260A/R2/R3 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R3 |  |
| CA_n48A-n260R4 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R4 |  |
| CA_n48A-n260R5 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R5 |  |
| CA_n48A-n260R6 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R6 |  |
| CA_n48A-n260R7 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R7 |  |
| CA_n48A-n260R8 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R8 |  |
| CA_n48A-n260R9 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R9 |  |
| CA_n48A-n260R10 | CA_n48A-n260A/R2/R3/R4 | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n260 | CA_n260R10 |  |
| CA_n48(2A)-n260A | CA_n48A-n260A | n48 | CA_n48(2A) | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48(2A)-n260G | CA_n48A-n260A/G | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48(2A)-n260H | CA_n48A-n260A/G/H | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48(2A)-n260I | CA_n48A-n260A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48(2A)-n260J | CA_n48A-n260A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48(2A)-n260K | CA_n48A-n260A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48(2A)-n260L | CA_n48A-n260A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48(2A)-n260M | CA_n48A-n260A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48B-n260A | CA_n48A-n260A | n48 | CA_n48B | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48B-n260G | CA_n48A-n260A/G | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48B-n260H | CA_n48A-n260A/G/H | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48B-n260I | CA_n48A-n260A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48B-n260J | CA_n48A-n260A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48B-n260K | CA_n48A-n260A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48B-n260L | CA_n48A-n260A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48B-n260M | CA_n48A-n260A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48(A-B)-n260A | CA_n48A-n260A | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48(A-B)-n260G | CA_n48A-n260A/G | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48(A-B)-n260H | CA_n48A-n260A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48(A-B)-n260I | CA_n48A-n260A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48(A-B)-n260J | CA_n48A-n260A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48(A-B)-n260K | CA_n48A-n260A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48(A-B)-n260L | CA_n48A-n260A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48(A-B)-n260M | CA_n48A-n260A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48A-n261A | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n48A-n261G | CA_n48A-n261A/G | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n48A-n261H | CA_n48A-n261A/G/H | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n48A-n261I | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n48A-n261J | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n48A-n261K | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n48A-n261L | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n48A-n261M | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n48A-n261(2A) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n48A-n261(2G) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n48A-n261(2I) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2I) |  |
| CA_n48A-n261(2H) | CA_n48A-n261A/G/H | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n48A-n261(3A) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n48A-n261(4A) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(4A) |  |
| CA_n48A-n261(A-G) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n48A-n261(A-G-H) | CA_n48A-n261A/G/H | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n48A-n261(A-G-I) | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n48A-n261(A-H) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n48A-n261(A-I) | CA_n48A-n261A | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n48A-n261(G-H) | CA_n48A-n261A/G/H | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n48A-n261(H-I) | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n48A-n261(G-I) | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n48A-n261(2A-G) | CA_n48A-n261A/G | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n48A-n261(2A-H) | CA_n48A-n261A/G/H | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n48A-n261(2A-I) | CA_n48A-n261A/G/H/I | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n48A-n261(A-2G) | CA_n48A-n261A/G | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n261 | CA_n261(A-2G) |  |
| CA_n48(2A)-n261A | CA_n48A-n261A | n48 | CA_n48(2A) | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n48(2A)-n261G | CA_n48A-n261A/G | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n48(2A)-n261H | CA_n48A-n261A/G/H | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n48(2A)-n261I | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n48(2A)-n261J | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n48(2A)-n261K | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n48(2A)-n261L | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n48(2A)-n261M | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n48(2A)-n261(G-H) | CA_n48A-n261A/G/H | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261(G-H) |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n48(2A)-n261(2H) | CA_n48A-n261A/G/H | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261(2H) |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n48(2A)-n261(G-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261(G-I) |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n48(2A)-n261(A-G-H) | CA_n48A-n261A/G/H | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n48(2A)-n261(H-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261(H-I) |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n48(2A)-n261(2A-G) | CA_n48A-n261A/G | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n48(2A)-n261(2A-H) | CA_n48A-n261A/G/H | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n48(2A)-n261(2A-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n48(2A)-n261(2A) | CA_n48A-n261A | n48 | CA_n48(2A) BCS1 | 0 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n48(2A)-n261(3A) | CA_n48A-n261A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n48(2A)-n261(2G) | CA_n48A-n261A/G | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n48(2A)-n261(A-2G) | CA_n48A-n261A/G | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(A-2G) |  |
| CA_n48(2A)-n261(A-G) | CA_n48A-n261A/G | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n48(2A)-n261(A-H) | CA_n48A-n261A/G/H | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n48(2A)-n261(A-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n48(2A)-n261(A-G-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(2A) | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n48B-n261A | CA_n48A-n261A | n48 | CA_n48B | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n48B-n261G | CA_n48A-n261A/G | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n48B-n261H | CA_n48A-n261A/G/H | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n48B-n261I | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n48B-n261J | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n48B-n261K | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n48B-n261L | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n48B-n261M | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n48B-n261(G-H) | CA_n48A-n261A/G/H | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(G-H) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(G-H) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n48B-n261(2H) | CA_n48A-n261A/G/H | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(2H) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(2H) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n48B-n261(G-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(G-I) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(G-I) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n48B-n261(A-G-H) | CA_n48A-n261A/G/H | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(A-G-H) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n48B-n261(H-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(H-I) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(H-I) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n48B-n261(2A-G) | CA_n48A-n261A/G | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(2A-G) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n48B-n261(2A-H) | CA_n48A-n261A/G/H | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(2A-H) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n48B-n261(2A-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(2A-I) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n48B-n261(2A) | CA_n48A-n261A | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n48B-n261(3A) | CA_n48A-n261A | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(3A) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(3A) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n48B-n261(2G) | CA_n48A-n261A/G | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(2G) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(2G) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n48B-n261(A-2G) | CA_n48A-n261A/G | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(A-2G) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(A-2G) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(A-2G) |  |
| CA_n48B-n261(A-G) | CA_n48A-n261A/G | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(A-G) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(A-G) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n48B-n261(A-H) | CA_n48A-n261A/G/H | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(A-H) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(A-H) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n48B-n261(A-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(A-I) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(A-I) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n48B-n261(A-G-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48B | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n261 | CA_n261(A-G-I) |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n48(A-B)-n261A | CA_n48A-n261A | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n48(A-B)-n261G | CA_n48A-n261A/G | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n48(A-B)-n261H | CA_n48A-n261A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n48(A-B)-n261I | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n48(A-B)-n261J | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n48(A-B)-n261K | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n48(A-B)-n261L | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n48(A-B)-n261M | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n48(A-B)-n261(A-G) | CA_n48A-n261A/G | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n48(A-B)-n261(A-H) | CA_n48A-n261A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n48(A-B)-n261(G-H) | CA_n48A-n261A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n48(A-B)-n261(2A) | CA_n48A-n261A | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n48(A-B)-n261(3A) | CA_n48A-n261A | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n48(A-B)-n261(2G) | CA_n48A-n261A/G | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n48(A-B)-n261(2H) | CA_n48A-n261A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n48(A-B)-n261(A-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n48(A-B)-n261(G-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n48(A-B)-n261(2A-G) | CA_n48A-n261A/G | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n48(A-B)-n261(2A-H) | CA_n48A-n261A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n48(A-B)-n261(A-2G) | CA_n48A-n261A/G | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(A-2G) |  |
| CA_n48(A-B)-n261(A-G-H) | CA_n48A-n261A/G/H | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n48(A-B)-n261(H-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n48(A-B)-n261(2A-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n48(A-B)-n261(A-G-I) | CA_n48A-n261A/G/H/I | n48 | CA_n48(A-B) | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n48(3A)-n260A | CA_n48A-n260A | n48 | CA_n48(3A) | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48(3A)-n260G | CA_n48A-n260A/G | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48(3A)-n260H | CA_n48A-n260A/G/H | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48(3A)-n260I | CA_n48A-n260A/G/H/I | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48(3A)-n260J | CA_n48A-n260A/G/H/I | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48(3A)-n260K | CA_n48A-n260A/G/H/I | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48(3A)-n260L | CA_n48A-n260A/G/H/I | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48(3A)-n260M | CA_n48A-n260A/G/H/I | n48 | CA_n48(3A) | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48(4A)-n260A | CA_n48A-n260A | n48 | CA_n48(4A) | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48(4A)-n260G | CA_n48A-n260A/G | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48(4A)-n260H | CA_n48A-n260A/G/H | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48(4A)-n260I | CA_n48A-n260A/G/H/I | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48(4A)-n260J | CA_n48A-n260A/G/H/I | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48(4A)-n260K | CA_n48A-n260A/G/H/I | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48(4A)-n260L | CA_n48A-n260A/G/H/I | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48(4A)-n260M | CA_n48A-n260A/G/H/I | n48 | CA_n48(4A) | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48C-n260A | CA_n48A-n260A | n48 | CA_n48C | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n48C-n260G | CA_n48A-n260A/G | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n48C-n260H | CA_n48A-n260A/G/H | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n48C-n260I | CA_n48A-n260A/G/H/I | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n48C-n260J | CA_n48A-n260A/G/H/I | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n48C-n260K | CA_n48A-n260A/G/H/I | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n48C-n260L | CA_n48A-n260A/G/H/I | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n48C-n260M | CA_n48A-n260A/G/H/I | n48 | CA_n48C | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n48A-n263A | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48A-n263G | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48A-n263H | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48A-n263I | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48A-n263J | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48A-n263K | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48A-n263L | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48A-n263M | CA_n48A-n263A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(2A)-n263A | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(2A)-n263G | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(2A)-n263H | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(2A)-n263I | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(2A)-n263J | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(2A)-n263K | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(2A)-n263L | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(2A)-n263M | CA_n48A-n263A | n48 | CA_n48(2A) | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48B-n263A | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48B-n263G | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48B-n263H | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48B-n263I | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48B-n263J | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48B-n263K | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48B-n263L | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48B-n263M | CA_n48A-n263A | n48 | CA_n48B | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(A-B)-n263A | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(A-B)-n263G | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(A-B)-n263H | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(A-B)-n263I | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(A-B)-n263J | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(A-B)-n263K | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(A-B)-n263L | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(A-B)-n263M | CA_n48A-n263A | n48 | CA_n48(A-B) | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48C-n263A | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48C-n263G | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48C-n263H | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48C-n263I | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48C-n263J | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48C-n263K | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48C-n263L | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48C-n263M | CA_n48A-n263A | n48 | CA_n48C | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(3A)-n263A | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(3A)-n263G | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(3A)-n263H | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(3A)-n263I | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(3A)-n263J | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(3A)-n263K | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(3A)-n263L | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(3A)-n263M | CA_n48A-n263A | n48 | CA_n48(3A) | 0 |
|  |  | n263 | CA_n263M |  |
| CA_n48(4A)-n263A | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | 400, 800, 1600, 2000 |  |
| CA_n48(4A)-n263G | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263G |  |
| CA_n48(4A)-n263H | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263H |  |
| CA_n48(4A)-n263I | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263I |  |
| CA_n48(4A)-n263J | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263J |  |
| CA_n48(4A)-n263K | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263K |  |
| CA_n48(4A)-n263L | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263L |  |
| CA_n48(4A)-n263M | CA_n48A-n263A | n48 | CA_n48(4A) | 0 |
|  |  | n263 | CA_n263M |  |

##### Table 5.5A.1.1-1l ~ Table 5.5A.1.1-1p

Table 5.5A.1.1-1l: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n257A | CA_n66A-n257A | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n66A-n257G | CA_n66A-n257A/G | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n66A-n257H | CA_n66A-n257A/G/H | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n66A-n257I | CA_n66A-n257A/G/H/I | n66 | 5, 10, 15, 20, 40 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n66A-n257J | CA_n66A-n257A/G/H/I/J | n66 | See n66 channel bandwidths in 1 Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257J |  |
| CA_n66A-n257K | CA_n66A-n257A/G/H/I/J/K | n66 | See n66 channel bandwidths in 1 Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257K |  |
| CA_n66A-n257L | CA_n66A-n257A/G/H/I/J/K/L | n66 | See n66 channel bandwidths in 1 Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257L |  |
| CA_n66A-n257M | CA_n66A-n257A/G/H/I/J/K/L/M | n66 | See n66 channel bandwidths in 1 Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257M |  |
| CA_n66A-n257O | CA_n66A-n257A/O | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n257 | CA_n257O |  |
| CA_n66A-n257P | CA_n66A-n257A/O/P | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n257 | CA_n257P |  |
| CA_n66A-n257Q | CA_n66A-n257A/O/P/Q | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n257 | CA_n257Q |  |
| CA_n66A-n258A | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n258G | CA_n66A-n258A/G | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258G |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258G |  |
| CA_n66A-n258H | CA_n66A-n258A/G/H | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258H |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258H |  |
| CA_n66A-n258I | CA_n66A-n258A/G/H/I | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258I |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258I |  |
| CA_n66A-n258J | CA_n66A-n258A/G/H/I/J | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258J |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258J |  |
| CA_n66A-n258K | CA_n66A-n258A/G/H/I/J/K | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n66A-n258L | CA_n66A-n258A/G/H/I/J/K/L | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n66A-n258M | CA_n66A-n258A/G/H/I/J/K/L/M | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n66A-n258O | CA_n66A-n258A/O | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258O |  |
| CA_n66A-n258P | CA_n66A-n258A/O/P | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258P |  |
| CA_n66A-n258Q | CA_n66A-n258A/O/P/Q | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n258 | CA_n258Q |  |
| CA_n66A-n258(2A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(2A) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2A) |  |
| CA_n66A-n258(3A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(3A) |  |
| CA_n66A-n258(4A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(4A) |  |
| CA_n66A-n258(5A) | CA_n66A-n258A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n258 | CA_n258(5A) |  |
| CA_n66A-n258(2G) | CA_n66A-n258A/G | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(2G) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(2G) |  |
| CA_n66A-n258(A-G) | CA_n66A-n258A/G | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-G) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-G) |  |
| CA_n66A-n258(A-H) | CA_n66A-n258A/G/H | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(A-H) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(A-H) |  |
| CA_n66A-n258(G-H) | CA_n66A-n258A/G/H | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n258 | CA_n258(G-H) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 | CA_n258(G-H) |  |
| CA_n66A-n260A | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n66 | See n66 channel bandwidths in 1 Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in 1 Table 5.3.5-1 |  |
| CA_n66A-n260(2A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n66A-n260(3A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n66A-n260(4A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n66A-n260(5A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(5A) |  |
| CA_n66A-n260(6A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(6A) |  |
| CA_n66A-n260(7A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(7A) |  |
| CA_n66A-n260(8A) | CA_n66A-n260A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260(8A) |  |
| CA_n66A-n260G | CA_n66A-n260A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260G |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260G |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n66A-n260H | CA_n66A-n260A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260H |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260H |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n66A-n260I | CA_n66A-n260A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260I |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260I |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n66A-n260J | CA_n66A-n260A/G/H/I/J | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260J |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260J |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n66A-n260K | CA_n66A-n260A/G/H/I/J/K | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260K |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260K |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n66A-n260L | CA_n66A-n260A/G/H/I/J/K/L | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260L |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260L |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n66A-n260M | CA_n66A-n260A/G/H/I/J/K/L/M | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260M |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260M |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n66A-n260O | CA_n66A-n260A/O | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n260 | CA_n260O |  |
| CA_n66A-n260P | CA_n66A-n260A/O/P | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n260 | CA_n260P |  |
| CA_n66A-n260Q | CA_n66A-n260A/O/P/Q | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 | 0 |
|  |  | n260 | CA_n260Q |  |
| CA_n66A-n260R2 | CA_n66A-n260A/R2 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R2 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R2 |  |
| CA_n66A-n260R3 | CA_n66A-n260A/R2/R3 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R3 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R3 |  |
| CA_n66A-n260R4 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R4 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R4 |  |
| CA_n66A-n260R5 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R5 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R5 |  |
| CA_n66A-n260R6 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R6 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R6 |  |
| CA_n66A-n260R7 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R7 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R7 |  |
| CA_n66A-n260R8 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R8 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R8 |  |
| CA_n66A-n260R9 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R9 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R9 |  |
| CA_n66A-n260R10 | CA_n66A-n260A/R2/R3/R4 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n260 | CA_n260R10 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n260 | CA_n260R10 |  |
| CA_n66(2A)-n260A | CA_n66A-n260A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
| CA_n66(2A)-n260G | CA_n66A-n260A/G | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260G |  |
| CA_n66(2A)-n260H | CA_n66A-n260A/G/H | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260H |  |
| CA_n66(2A)-n260I | CA_n66A-n260A/G/H/I | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260I |  |
| CA_n66(2A)-n260J | CA_n66A-n260A/G/H/I/J | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260J |  |
| CA_n66(2A)-n260K | CA_n66A-n260A/G/H/I/J/K | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260K |  |
| CA_n66(2A)-n260L | CA_n66A-n260A/G/H/I/J/K/L | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260L |  |
| CA_n66(2A)-n260M | CA_n66A-n260A/G/H/I/J/K/L/M | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n260 | CA_n260M |  |
| CA_n66A-n261A | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n261(2A) | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A) |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261(2A) |  |
| CA_n66A-n261(3A) | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(3A) |  |
| CA_n66A-n261(4A) | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(4A) |  |
| CA_n66A-n261G | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261G |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261G |  |
| CA_n66A-n261H | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261H |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261H |  |
| CA_n66A-n261I | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261I |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261I |  |
| CA_n66A-n261J | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261J |  |
|  | CA_n66A-n261A/G/H/I/J | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261J |  |
| CA_n66A-n261K | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261K |  |
|  | CA_n66A-n261A/G/H/I/J/K | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261K |  |
| CA_n66A-n261L | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261L |  |
|  | CA_n66A-n261A/G/H/I/J/K/L | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261L |  |
| CA_n66A-n261M | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261M |  |
|  | CA_n66A-n261A/G/H/I/J/K/L/M | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 | CA_n261M |  |
| CA_n66A-n261O | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261O |  |
| CA_n66A-n261P | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261P |  |
| CA_n66A-n261Q | CA_n66A-n261A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261Q |  |
| CA_n66A-n261(2G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2G) |  |
| CA_n66A-n261(2H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2H) |  |
| CA_n66A-n261(2I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2I) |  |
| CA_n66A-n261(A-G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-G) |  |
| CA_n66A-n261(A-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-H) |  |
| CA_n66A-n261(A-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-I) |  |
| CA_n66A-n261(A-J) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-J) |  |
| CA_n66A-n261(A-K) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-K) |  |
| CA_n66A-n261(A-L) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-L) |  |
| CA_n66A-n261(G-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(G-H) |  |
| CA_n66A-n261(H-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(H-I) |  |
| CA_n66A-n261(G-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(G-I) |  |
| CA_n66A-n261(A-G-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-G-H) |  |
| CA_n66A-n261(A-G-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-G-I) |  |
| CA_n66A-n261(2A-H) | CA_n66A-n261A/G/H | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A-H) |  |
| CA_n66A-n261(2A-G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A-G) |  |
| CA_n66A-n261(2A-I) | CA_n66A-n261A/G/H/I | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(2A-I) |  |
| CA_n66A-n261(A-2G) | CA_n66A-n261A/G | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n261 | CA_n261(A-2G) |  |
| CA_n71A-n257A | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
|  | CA_n71A-n257A | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257A |  |
| CA_n71A-n257G | CA_n71A-n257A/G | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257G |  |
| CA_n71A-n257H | CA_n71A-n257A/G/H | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257H |  |
| CA_n71A-n257I | CA_n71A-n257A/G/H/I | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257I |  |
| CA_n71A-n257J | CA_n71A-n257A/G/H/I/J | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257J |  |
| CA_n71A-n257K | CA_n71A-n257A/G/H/I/J/K | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257K |  |
| CA_n71A-n257L | CA_n71A-n257A/G/H/I/J/K/L | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | N257 | CA_n257L |  |
| CA_n71A-n257M | CA_n71A-n257A/G/H/I/J/K/L/M | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n257 | CA_n257M |  |
| CA_n71A-n257O | CA_n71A-n257A/O | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n257 | CA_n257O |  |
| CA_n71A-n257P | CA_n71A-n257A/O/P | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n257 | CA_n257P |  |
| CA_n71A-n257Q | CA_n71A-n257A/O/P/Q | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n257 | CA_n257Q |  |
| CA_n71A-n258A | CA_n71A-n258A | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |
| CA_n71A-n258G | CA_n71A-n258A/G | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258G |  |
| CA_n71A-n258H | CA_n71A-n258A/G/H | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258H |  |
| CA_n71A-n258I | CA_n71A-n258A/G/H/I | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258I |  |
| CA_n71A-n258J | CA_n71A-n258A/G/H/I/J | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258J |  |
| CA_n71A-n258K | CA_n71A-n258A/G/H/I/J/K | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258K |  |
| CA_n71A-n258L | CA_n71A-n258A/G/H/I/J/K/L | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258L |  |
| CA_n71A-n258M | CA_n71A-n258A/G/H/I/J/K/L/M | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258M |  |
| CA_n71A-n258O | CA_n71A-n258A/O | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258O |  |
| CA_n71A-n258P | CA_n71A-n258A/O/P | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258P |  |
| CA_n71A-n258Q | CA_n71A-n258A/O/P/Q | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n258 | CA_n258Q |  |
| CA_n71A-n260A | -CA_n71A-n260A | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | 50, 100, 200, 400 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | See n260 channel bandwidths in 1 Table 5.3.5-1 |  |
| CA_n71A-n260G | CA_n71A-n260A/G | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260G |  |
| CA_n71A-n260H | CA_n71A-n260A/G/H | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260H |  |
| CA_n71A-n260I | CA_n71A-n260A/G/H/I | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260I |  |
| CA_n71A-n260J | CA_n71A-n260A/G/H/I/J | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260J |  |
| CA_n71A-n260K | CA_n71A-n260A/G/H/I/J/K | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260K |  |
| CA_n71A-n260L | CA_n71A-n260A/G/H/I/J/K/L | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260L |  |
| CA_n71A-n260M | CA_n71A-n260A/G/H/I/J/K/L/M | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 | CA_n260M |  |
| CA_n71A-n260O | CA_n71A-n260A/O | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n260 | CA_n260O |  |
| CA_n71A-n260P | CA_n71A-n260A/O/P | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n260 | CA_n260P |  |
| CA_n71A-n260Q | CA_n71A-n260A/O/P/Q | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n260 | CA_n260Q |  |
| CA_n71A-n260(2A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(2A) |  |
| CA_n71A-n260(3A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(3A) |  |
| CA_n71A-n260(4A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n260 | CA_n260(4A) |  |
| CA_n71A-n261A | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n261 | 50, 100, 200, 400 |  |
| CA_n71A-n261G | CA_n71A-n261A/G | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261G |  |
| CA_n71A-n261H | CA_n71A-n261A/G/H | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261H |  |
| CA_n71A-n261I | CA_n71A-n261A/G/H/I | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261I |  |
| CA_n71A-n261J | CA_n71A-n261A/G/H/I/J | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261J |  |
| CA_n71A-n261K | CA_n71A-n261A/G/H/I/J/K | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261K |  |
| CA_n71A-n261L | CA_n71A-n261A/G/H/I/J/K/L | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261L |  |
| CA_n71A-n261M | CA_n71A-n261A/G/H/I/J/K/L/M | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261M |  |
| CA_n71A-n261O | CA_n71A-n261A/O | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261O |  |
| CA_n71A-n261P | CA_n71A-n261A/O/P | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261P |  |
| CA_n71A-n261Q | CA_n71A-n261A/O/P/Q | n71 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n261 | CA_n261Q |  |
| CA_n71A-n261(2A) | - | n71 | 5, 10, 15, 20 | 0 |
|  |  | n261 | CA_n261(2A) |  |

Table 5.5A.1.1-1m: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n77A-n257A | CA_n77A-n257A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n77A-n257D | CA_n77A-n257A/D | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77A-n257E | CA_n77A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n77A-n257F | CA_n77A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n77A-n257G | CA_n257G CA_n77A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77A-n257H | CA_n257G/H CA_n77A-n257A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77A-n257I | CA_n257G/H/I CA_n77A-n257A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77A-n257J | CA_n257G/H/I/J CA_n77A-n257A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n77A-n257K | CA_n257G/H/I/J/K CA_n77A-n257A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n77A-n257L | CA_n257G/H/I/J/K/L CA_n77A-n257A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n77A-n257M | CA_n257G/H/I/J/K/L/M CA_n77A-n257A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n77A-n257O | CA_n77A-n257A/O | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257O |  |
| CA_n77A-n257P | CA_n77A-n257A/O/P | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257P |  |
| CA_n77A-n257Q | CA_n77A-n257A/O/P/Q | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257Q |  |
| CA_n77A-n257(2A) | CA_n77A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257(2A) |  |
| CA_n77A-n257(2G) | CA_n77A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257(2G) |  |
| CA_n77A-n257(A-G) | CA_n77A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n257 |  | CA_n257(A-G) |  |
| CA_n77C-n257A | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n77C-n257D | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77C-n257E | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n77C-n257F | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n77C-n257G | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77C-n257H | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77C-n257I | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77C-n257J | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n77C-n257K | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n77C-n257L | CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n77C-n257M | CA_n77(2A) CA_n77A-n257A | n77 |  | CA_n77C | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n77(2A)-n257A | CA_n77A-n257A | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n77(2A)-n257D | CA_n77A-n257A/D | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77(2A)-n257G | CA_n77A-n257A/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77(2A)-n257H | CA_n77A-n257A/G/H | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77(2A)-n257I | CA_n77A-n257A/G/H/I | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77(2A)-n257J | CA_n77A-n257A/G/H/I/J | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n77(2A)-n257K | CA_n77A-n257A/G/H/I/J/K | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n77(2A)-n257L | CA_n77A-n257A/G/H/I/J/K/L | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n77(2A)-n257M | CA_n77A-n257A/G/H/I/J/K/L/M | n77 |  | CA_n77(2A) | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n77(3A)-n257A | CA_n77A-n257A | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n77(3A)-n257D | CA_n77A-n257A/D | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n77(3A)-n257G | CA_n77A-n257A/G | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n77(3A)-n257H | CA_n77A-n257A/G/H | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n77(3A)-n257I | CA_n77A-n257A/G/H/I | n77 |  | CA_n77(3A) | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n77A-n258A | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | See n258 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n258D | CA_n77A-n258A/D | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n77A-n258G | CA_n77A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258G |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258G |  |
| CA_n77A-n258H | CA_n77A-n258A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258H |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258H |  |
| CA_n77A-n258I | CA_n77A-n258A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258I |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258I |  |
| CA_n77A-n258J | CA_n77A-n258A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258J |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258J |  |
| CA_n77(2A)-n258A | CA_n77A-n258A | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n77A-n258K | CA_n77A-n258A/G/H/I/J/K | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258K |  |
| CA_n77A-n258L | CA_n77A-n258A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258L |  |
| CA_n77A-n258M | CA_n77A-n258A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258M |  |
| CA_n77A-n258O | CA_n77A-n258A/O | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258O |  |
| CA_n77A-n258P | CA_n77A-n258A/O/P | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258P |  |
| CA_n77A-n258Q | CA_n77A-n258A/O/P/Q | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258Q |  |
| CA_n77(2A)-n258D | CA_n77A-n258A/D | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n77(2A)-n258G | CA_n77A-n258A/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n77(2A)-n258H | CA_n77A-n258A/G/H | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n77(2A)-n258I | CA_n77A-n258A/G/H/I | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n77(2A)-n258J | CA_n77A-n258A/G/H/I/J | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n77(3A)-n258A | CA_n77A-n258A | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n77(3A)-n258D | CA_n77A-n258A/D | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n77(3A)-n258G | CA_n77A-n258A/G | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n77(3A)-n258H | CA_n77A-n258A/G/H | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n77(3A)-n258I | CA_n77A-n258A/G/H/I | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n77(3A)-n258J | CA_n77A-n258A/G/H/I/J | n77 |  | CA_n77(3A) | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n77A-n258(2A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(2A) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258(2A) |  |
| CA_n77A-n258(2G) | CA_n77A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(2G) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258(2G) |  |
| CA_n77A-n258(3A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(3A) |  |
| CA_n77A-n258(4A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(4A) |  |
| CA_n77A-n258(5A) | CA_n77A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258(5A) |  |
| CA_n77A-n258(A-D) | CA_n77A-n258A/D | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(A-D) |  |
| CA_n77A-n258(A-G) | CA_n77A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(A-G) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258(A-G) |  |
| CA_n77A-n258(A-H) | CA_n77A-n258A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(A-H) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258(A-H) |  |
| CA_n77A-n258(D-G) | CA_n77A-n258A/D/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(D-G) |  |
| CA_n77A-n258(G-H) | CA_n77A-n258A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n258 |  | CA_n258(G-H) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n258 |  | CA_n258(G-H) |  |
| CA_n77(2A)-n258(2A) | CA_n77A-n258A | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(2A) |  |
| CA_n77(2A)-n258(2G) | CA_n77A-n258A/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(2G) |  |
| CA_n77(2A)-n258(A-D) | CA_n77A-n258A/D | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(A-D) |  |
| CA_n77(2A)-n258(A-G) | CA_n77A-n258A/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(A-G) |  |
| CA_n77(2A)-n258(A-H) | CA_n77A-n258A/G/H | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(A-H) |  |
| CA_n77(2A)-n258(D-G) | CA_n77A-n258A/D/G | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(D-G) |  |
| CA_n77(2A)-n258(G-H) | CA_n77A-n258A/G/H | n77 |  | CA_n77(2A) | 0 |
|  |  | n258 |  | CA_n258(G-H) |  |
| CA_n77A-n259A | CA_n77A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n259 |  | 50, 100, 200, 400 |  |
| CA_n77A-n259G | CA_n259G CA_n77A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259G |  |
| CA_n77A-n259H | CA_n259G/H CA_n77A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259H |  |
| CA_n77A-n259I | CA_n259G/H/I CA_n77A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259I |  |
| CA_n77A-n259J | CA_n259G/H/I/J CA_n77A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259J |  |
| CA_n77A-n259K | CA_n259G/H/I/J/K CA_n77A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259K |  |
| CA_n77A-n259L | CA_n259G/H/I/J/K/L CA_n77A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259L |  |
| CA_n77A-n259M | CA_n259G/H/I/J/K/L/M CA_n77A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | CA_n259M |  |
| CA_n77A-n260A | CA_n77A-n260A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | See n260 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n260G | CA_n77A-n260A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260G |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260G |  |
| CA_n77A-n260H | CA_n77A-n260A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260H |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260H |  |
| CA_n77A-n260I | CA_n77A-n260A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260I |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260I |  |
| CA_n77A-n260J | CA_n77A-n260A/G/H/I/J | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260J |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260J |  |
| CA_n77A-n260K | CA_n77A-n260A/G/H/I/J/K | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260K |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260K |  |
| CA_n77A-n260L | CA_n77A-n260A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260L |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260L |  |
| CA_n77A-n260M | CA_n77A-n260A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260M |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n260 |  | CA_n260M |  |
| CA_n77A-n260O | CA_n77A-n260A/O | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260O |  |
| CA_n77A-n260P | CA_n77A-n260A/O/P | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260P |  |
| CA_n77A-n260Q | CA_n77A-n260A/O/P/Q | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260Q |  |
| CA_n77A-n260R2 | CA_n77A-n260A/R2 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R2 |  |
| CA_n77A-n260R3 | CA_n77A-n260A/R2/R3 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R3 |  |
| CA_n77A-n260R4 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R4 |  |
| CA_n77A-n260R5 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R5 |  |
| CA_n77A-n260R6 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R6 |  |
| CA_n77A-n260R7 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R7 |  |
| CA_n77A-n260R8 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R8 |  |
| CA_n77A-n260R9 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R9 |  |
| CA_n77A-n260R10 | CA_n77A-n260A/R2/R3/R4 | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n260 |  | CA_n260R10 |  |
| CA_n77C-n260A | CA_n77A-n260A | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n77C-n260G | CA_n77A-n260A/G | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260G |  |
| CA_n77C-n260H | CA_n77A-n260A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260H |  |
| CA_n77C-n260I | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260I |  |
| CA_n77C-n260J | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260J |  |
| CA_n77C-n260K | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260K |  |
| CA_n77C-n260L | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260L |  |
| CA_n77C-n260M | CA_n77A-n260A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n260 |  | CA_n260M |  |
| CA_n77(2A)-n260A | CA_n77(2A) CA_n77A-n260A | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | 50, 100, 200, 400 |  |
| CA_n77(2A)-n260G | CA_n77(2A) CA_n77A-n260A/G | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260G |  |
| CA_n77(2A)-n260H | CA_n77(2A) CA_n77A-n260A/G/H | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260H |  |
| CA_n77(2A)-n260I | CA_n77(2A) CA_n77A-n260A/G/H/I | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260I |  |
| CA_n77(2A)-n260J | CA_n77(2A) CA_n77A-n260A/G/H/I/J | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260J |  |
| CA_n77(2A)-n260K | CA_n77(2A) CA_n77A-n260A/G/H/I/J/K | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260K |  |
| CA_n77(2A)-n260L | CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260L |  |
| CA_n77(2A)-n260M | CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L/M | n77 |  | CA_n77(2A)_BCS1 | 0 |
|  |  | n260 |  | CA_n260M |  |
| CA_n77A-n261A | CA_n77A-n261A | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | 50, 100, 200, 400 |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 |  | See n261 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n261D | CA_n77A-n261A/D | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261D |  |
| CA_n77A-n261G | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261G |  |
| CA_n77A-n261H | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261H |  |
| CA_n77A-n261I | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261I |  |
| CA_n77A-n261J | CA_n77A-n261A/G/H/I/J | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261J |  |
| CA_n77A-n261K | CA_n77A-n261A/G/H/I/J/K | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261K |  |
| CA_n77A-n261L | CA_n77A-n261A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261L |  |
| CA_n77A-n261M | CA_n77A-n261A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261M |  |
| CA_n77A-n261O | CA_n77A-n261A/O | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261O |  |
| CA_n77A-n261P | CA_n77A-n261A/O/P | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261P |  |
| CA_n77A-n261Q | CA_n77A-n261A/O/P/Q | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261Q |  |
| CA_n77A-n261(2A) | CA_n77A-n261A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A) |  |
|  |  | n77 |  | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n261 |  | CA_n261(2A) |  |
| CA_n77A-n261(2G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2G) |  |
| CA_n77A-n261(2H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2H) |  |
| CA_n77A-n261(2I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2I) |  |
| CA_n77A-n261(3A) | CA_n77A-n261A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(3A) |  |
| CA_n77A-n261(4A) | CA_n77A-n261A | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(4A) |  |
| CA_n77A-n261(A-G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-G) |  |
| CA_n77A-n261(A-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-H) |  |
| CA_n77A-n261(A-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-I) |  |
| CA_n77A-n261(G-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(G-H) |  |
| CA_n77A-n261(G-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(G-I) |  |
| CA_n77A-n261(H-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 701, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(H-I) |  |
| CA_n77A-n261(A-J) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-J) |  |
| CA_n77A-n261(A-K) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-K) |  |
| CA_n77A-n261(A-L) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-L) |  |
| CA_n77A-n261(A-G-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-G-H) |  |
| CA_n77A-n261(A-G-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-G-I) |  |
| CA_n77A-n261(2A-H) | CA_n77A-n261A/G/H | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A-H) |  |
| CA_n77A-n261(2A-G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A-G) |  |
| CA_n77A-n261(2A-I) | CA_n77A-n261A/G/H/I | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(2A-I) |  |
| CA_n77A-n261(A-2G) | CA_n77A-n261A/G | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n261 |  | CA_n261(A-2G) |  |
| CA_n77C-n261A | CA_n77A-n261A | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | 50, 100, 200, 400 |  |
| CA_n77C-n261G | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261G |  |
| CA_n77C-n261H | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261H |  |
| CA_n77C-n261I | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261I |  |
| CA_n77C-n261J | CA_n77A-n261A/G/H/I/J | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261J |  |
| CA_n77C-n261K | CA_n77A-n261A/G/H/I/J/K | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261K |  |
| CA_n77C-n261L | CA_n77A-n261A/G/H/I/J/K/L | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261L |  |
| CA_n77C-n261M | CA_n77A-n261A/G/H/I/J/K/L/M | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261M |  |
| CA_n77C-n261(G-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(G-H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(G-H) |  |
| CA_n77C-n261(2H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2H) |  |
| CA_n77C-n261(G-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(G-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(G-I) |  |
| CA_n77C-n261(A-G-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-G-H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-G-H) |  |
| CA_n77C-n261(H-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(H-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(H-I) |  |
| CA_n77C-n261(2A-G) | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A-G) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A-G) |  |
| CA_n77C-n261(2A-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A-H) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A-H) |  |
| CA_n77C-n261(2A-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A-I) |  |
| CA_n77C-n261(2A) | CA_n77A-n261A | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2A) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2A) |  |
| CA_n77C-n261(3A) | CA_n77A-n261A | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(3A) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(3A) |  |
| CA_n77C-n261(2G) | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(2G) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(2G) |  |
| CA_n77C-n261(A-2G) | CA_n77A-n261A/G | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-2G) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-2G) |  |
| CA_n77C-n261(A-G) | CA_n77A-n261A/G | n48 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-G) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-G) |  |
| CA_n77C-n261(A-H) | CA_n77A-n261A/G/H | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-H) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-H) |  |
| CA_n77C-n261(A-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-I) |  |
|  |  | n48 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-I) |  |
| CA_n77C-n261(A-G-I) | CA_n77A-n261A/G/H/I | n77 |  | CA_n77C | 0 |
|  |  | n261 |  | CA_n261(A-G-I) |  |
|  |  | n77 |  | CA_n77C_BCS1 | 1 |
|  |  | n261 |  | CA_n261(A-G-I) |  |
| CA_n77(2A)-n257E | CA_n77A-n257A | n77 | CA_n77(2A) |  | 0 |
|  |  | n257 | CA_n257E |  |  |
| CA_n77(2A)-n257F | CA_n77A-n257A | n77 | CA_n77(2A) |  | 0 |
|  |  | n257 | CA_n257F |  |  |

Table 5.5A.1.1-1n: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n257A | CA_n78A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257D | CA_n78A-n257A/D | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n78A-n257E | CA_n78A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n78A-n257F | CA_n78A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n78C-n257A | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78C-n257D | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n78C-n257E | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n78C-n257F | CA_n78A-n257A | n78 |  | CA_n78C | 0 |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n78C-n257G | CA_n78A-n257A/G | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257G |  |  |
| CA_n78C-n257H | CA_n78A-n257A/G/H | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257H |  |  |
| CA_n78C-n257I | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257I |  |  |
| CA_n78C-n257J | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257J |  |  |
| CA_n78C-n257K | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257K |  |  |
| CA_n78C-n257L | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257L |  |  |
| CA_n78C-n257M | CA_n78A-n257A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n257 | CA_n257M |  |  |
| CA_n78A-n257G | CA_n257G CA_n78A-n257A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257G |  |  |
| CA_n78A-n257H | CA_n257G/H CA_n78A-n257A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257H |  |  |
| CA_n78A-n257I | CA_n257G/H/I CA_n78A-n257A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257I |  |  |
| CA_n78A-n257J | CA_n257G/H/I/J CA_n78A-n257A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257J |  |  |
| CA_n78A-n257K | CA_n257G/H/I/J/K CA_n78A-n257A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257K |  |  |
| CA_n78A-n257L | CA_n257G/H/I CA_n78A-n257A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257L |  |  |
| CA_n78A-n257M | CA_n257G/H/I CA_n78A-n257A/G/H/I | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257M |  |  |
| CA_n78A-n257(2A) | CA_n78A-n257A/(2A) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257(2A) |  |  |
| CA_n78A-n257(A-G) | CA_n78A-n257A/G/(A-G) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257(A-G) |  |  |
| CA_n78A-n257(2G) | CA_n78A-n257A/G/(2G)/(A-G) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 | CA_n257(2G) |  |  |
| CA_n78(2A)-n257A | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n78(2A)-n257D | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257D |  |  |
| CA_n78(2A)-n257E | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257E |  |  |
| CA_n78(2A)-n257F | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257F |  |  |
| CA_n78(2A)-n257G | CA_n78A-n257A/G | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257G |  |  |
| CA_n78(2A)-n257H | CA_n78A-n257A/G/H | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257H |  |  |
| CA_n78(2A)-n257I | CA_n78A-n257A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257I |  |  |
| CA_n78(2A)-n257J | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257J |  |  |
| CA_n78(2A)-n257K | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257K |  |  |
| CA_n78(2A)-n257L | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257L |  |  |
| CA_n78(2A)-n257M | CA_n78A-n257A | n78 | CA_n78(2A) |  | 0 |
|  |  | n257 | CA_n257M |  |  |
| CA_n78A-n258A | CA_n78A-n258A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78A-n258B | CA_n258B CA_n78A-n258A/B | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78A-n258C | CA_n258B CA_n78A-n258A/B | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258C |  |  |
| CA_n78A-n258D | CA_n258D CA_n78A-n258A/D | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258D |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258D |  |  |
| CA_n78A-n258E | CA_n258D/E CA_n78A-n258A/D/E | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258E |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258E |  |  |
| CA_n78A-n258F | CA_n258D/E/F CA_n78A-n258A/D/E/F | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258F |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258F |  |  |
| CA_n78A-n258G | CA_n78A-n258A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258G |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258G |  |  |
| CA_n78A-n258H | CA_n78A-n258A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n258 | CA_n258H |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258H |  |  |
| CA_n78A-n258I | CA_n78A-n258A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258I |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258I |  |  |
| CA_n78A-n258J | CA_n78A-n258A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258J |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258J |  |  |
| CA_n78A-n258K | CA_n78A-n258A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258K |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258K |  |  |
| CA_n78A-n258L | CA_n78A-n258A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258L |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258L |  |  |
| CA_n78A-n258M | CA_n78A-n258A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258M |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 1 |
|  |  | n258 | CA_n258M |  |  |
| CA_n78A-n258R2 | CA_n78A-n258A/R2 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n78A-n258R3 | CA_n78A-n258A/R2/R3 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n78A-n258R4 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n78A-n258R5 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n78A-n258R6 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n78A-n258R7 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n78A-n258R8 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n78A-n258R9 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n78A-n258R10 | CA_n78A-n258A/R2/R3/R4 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n78A-n258(2A) | CA_n78A-n258A/(2A) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258(2A) |  |  |
| CA_n78A-n258(2G) | CA_n78A-n258A/G/(2G)/(A-G) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258(2G) |  |  |
| CA_n78A-n258(A-G) | CA_n78A-n258A/G/(A-G) | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n258 | CA_n258(A-G) |  |  |
| CA_n78B-n258A | CA_n78A-n258A | n78 | CA_n78B |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78B-n258B | CA_n78A-n258A | n78 | CA_n78B |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78C-n258A | CA_n78C CA_n78A-n258A | n78 | CA_n78C |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78C-n258B | CA_n78C CA_n258B CA_n78A-n258A/B | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78C-n258C | CA_n78C CA_n258B CA_n78A-n258A/B | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258C |  |  |
| CA_n78C-n258D | CA_n78C CA_n258D CA_n78A-n258A/D | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258D |  |  |
| CA_n78C-n258E | CA_n78C CA_n258D/E CA_n78A-n258A/D/E | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258E |  |  |
| CA_n78C-n258F | CA_n78C CA_n258D/E/F CA_n78A-n258A/D/E/F | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258F |  |  |
| CA_n78C-n258G | CA_n78C CA_n258G CA_n78A-n258A/G | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258G |  |  |
| CA_n78C-n258H | CA_n78C CA_n258G/H CA_n78A-n258A/G/H | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258H |  |  |
| CA_n78C-n258I | CA_n78C CA_n258G/H/I CA_n78A-n258A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258I |  |  |
| CA_n78C-n258J | CA_n78C CA_n258G/H/I CA_n78A-n258A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258J |  |  |
| CA_n78C-n258K | CA_n78C CA_n258G/H/I CA_n78A-n258A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258K |  |  |
| CA_n78C-n258L | CA_n78C CA_n258G/H/I CA_n78A-n258A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258L |  |  |
| CA_n78C-n258M | CA_n78C CA_n258G/H/I CA_n78A-n258A/G/H/I | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258M |  |  |
| CA_n78(2A)-n258A | CA_n78(2A) CA_n78A-n258A CA_n78(2A)-n258A | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n78C-n258R2 | CA_n78C CA_n258R2 CA_n78A-n258A/R2 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n78C-n258R3 | CA_n78C CA_n258R2/R3 CA_n78A-n258A/R2/R3 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n78C-n258R4 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n78C-n258R5 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n78C-n258R6 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n78C-n258R7 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n78C-n258R8 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n78C-n258R9 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n78C-n258R10 | CA_n78C CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n78 | CA_n78C |  | 0 |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n78(2A)-n258B | CA_n78(2A) CA_n258B CA_n78A-n258A/B CA_n78(2A)-n258A/B | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258B |  |  |
| CA_n78(2A)-n258C | CA_n78(2A) CA_n258B/C CA_n78A-n258A/B/C CA_n78(2A)-n258A/B/C | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258C |  |  |
| CA_n78(2A)-n258D | CA_n78(2A) CA_n258D CA_n78A-n258A/D CA_n78(2A)-n258A/D | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258D |  |  |
| CA_n78(2A)-n258E | CA_n78(2A) CA_n258D/E CA_n78A-n258A/D/E CA_n78(2A)-n258A/D/E | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258E |  |  |
| CA_n78(2A)-n258F | CA_n78(2A) CA_n258D/E/F CA_n78A-n258A/D/E/F CA_n78(2A)-n258A/D/E/F | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258F |  |  |
| CA_n78(2A)-n258G | CA_n78(2A) CA_n258G CA_n78A-n258A/G CA_n78(2A)-n258A/G | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258G |  |  |
| CA_n78(2A)-n258H | CA_n78(2A) CA_n258G/H CA_n78A-n258A/G/H CA_n78(2A)-n258A/G/H | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258H |  |  |
| CA_n78(2A)-n258I | CA_n78(2A) CA_n258G/H/I CA_n78A-n258A/G/H/I CA_n78(2A)-n258A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258I |  |  |
| CA_n78(2A)-n258J | CA_n78(2A) CA_n258G/H/I CA_n78A-n258A/G/H/I CA_n78(2A)-n258A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258J |  |  |
| CA_n78(2A)-n258K | CA_n78(2A) CA_n258G/H/I CA_n78A-n258A/G/H/I CA_n78(2A)-n258A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258K |  |  |
| CA_n78(2A)-n258L | CA_n78(2A) CA_n258G/H/I CA_n78A-n258A/G/H/I CA_n78(2A)-n258A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258L |  |  |
| CA_n78(2A)-n258M | CA_n78(2A) CA_n258G/H/I CA_n78A-n258A/G/H/I CA_n78(2A)-n258A/G/H/I | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258M |  |  |
| CA_n78(2A)-n258R2 | CA_n78(2A) CA_n258R2 CA_n78A-n258A/R2 CA_n78(2A)-n258A/R2 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n78(2A)-n258R3 | CA_n78(2A) CA_n258R2/R3 CA_n78A-n258A/R2/R3 CA_n78(2A)-n258A/R2/R3 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n78(2A)-n258R4 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n78(2A)-n258R5 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n78(2A)-n258R6 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n78(2A)-n258R7 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n78(2A)-n258R8 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n78(2A)-n258R9 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n78(2A)-n258R10 | CA_n78(2A) CA_n258R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n78(2A)-n258A/R2/R3/R4 | n78 | CA_n78(2A) |  | 0 |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n78A-n259A | CA_n78A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n259 | 50, 100, 200, 400 |  |  |
| CA_n78A-n259G | CA_n259G CA_n78A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259G |  |  |
| CA_n78A-n259H | CA_n259G/H CA_n78A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259H |  |  |
| CA_n78A-n259I | CA_n259G/H/I CA_n78A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259I |  |  |
| CA_n78A-n259J | CA_n259G/H/I/J CA_n78A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259J |  |  |
| CA_n78A-n259K | CA_n259G/H/I/J/K CA_n78A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259K |  |  |
| CA_n78A-n259L | CA_n259G/H/I/J/K/L CA_n78A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259L |  |  |
| CA_n78A-n259M | CA_n259G/H/I/J/K/L/M CA_n78A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259M |  |  |

Table 5.5A.1.1-1o: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n79A-n257A | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n79A-n257D | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n79A-n257E | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n79A-n257F | CA_n79A-n257A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n79A-n257G | CA_n257G CA_n79A-n257A/G | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n79A-n257H | CA_n257G/H CA_n79A-n257A/G/H | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n79A-n257I | CA_n257G/H/I CA_n79A-n257A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n79A-n257J | CA_n79A-n257A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n79A-n257K | CA_n79A-n257A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n79A-n257L | CA_n79A-n257A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n79A-n257M | CA_n79A-n257A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n79C-n257A | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |
| CA_n79C-n257D | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257D |  |
| CA_n79C-n257E | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257E |  |
| CA_n79C-n257F | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257F |  |
| CA_n79C-n257G | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257G |  |
| CA_n79C-n257H | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257H |  |
| CA_n79C-n257I | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257I |  |
| CA_n79C-n257J | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257J |  |
| CA_n79C-n257K | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257K |  |
| CA_n79C-n257L | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257L |  |
| CA_n79C-n257M | CA_n79A-n257A | n79 |  | CA_n79C | 0 |
|  |  | n257 |  | CA_n257M |  |
| CA_n79A-n258A | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n79A-n258B | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258B |  |
| CA_n79A-n258C | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258C |  |
| CA_n79A-n258D | CA_n79A-n258A/D | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258D |  |
| CA_n79A-n258E | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258E |  |
| CA_n79A-n258F | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258F |  |
| CA_n79A-n258G | CA_n79A-n258A/G | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n79A-n258H | CA_n79A-n258A/G/H | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n79A-n258I | CA_n79A-n258A/G/H/I | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n79A-n258J | CA_n79A-n258A/G/H/I/J | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n79A-n258K | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258K |  |
| CA_n79A-n258L | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258L |  |
| CA_n79A-n258M | CA_n79A-n258A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n258 |  | CA_n258M |  |
| CA_n79C-n258A | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | 50, 100, 200, 400 |  |
| CA_n79C-n258G | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258G |  |
| CA_n79C-n258H | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258H |  |
| CA_n79C-n258I | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258I |  |
| CA_n79C-n258J | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258J |  |
| CA_n79C-n258K | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258K |  |
| CA_n79C-n258L | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258L |  |
| CA_n79C-n258M | CA_n79A-n258A | n79 |  | CA_n79C | 0 |
|  |  | n258 |  | CA_n258M |  |
| CA_n79A-n259A | CA_n79A-n259A | n79 |  | 40, 50, 60, 80, 100 | 0 |
|  |  | n259 |  | 50, 100, 200, 400 |  |
| CA_n79A-n259G | CA_n259G CA_n79A-n259A/G | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259G |  |  |
| CA_n79A-n259H | CA_n259G/H CA_n79A-n259A/G/H | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259H |  |  |
| CA_n79A-n259I | CA_n259G/H/I CA_n79A-n259A/G/H/I | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259I |  |  |
| CA_n79A-n259J | CA_n259G/H/I/J CA_n79A-n259A/G/H/I/J | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259J |  |  |
| CA_n79A-n259K | CA_n259G/H/I/J/K CA_n79A-n259A/G/H/I/J/K | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259K |  |  |
| CA_n79A-n259L | CA_n259G/H/I/J/K/L CA_n79A-n259A/G/H/I/J/K/L | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259L |  |  |
| CA_n79A-n259M | CA_n259G/H/I/J/K/L/M CA_n79A-n259A/G/H/I/J/K/L/M | n79 | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n259 | CA_n259M |  |  |

Table 5.5A.1.1-1p: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n105A-n257A | CA_n105A-n257A | n105 | 5, 10,15, 20, 25, 30, 35 | 0 |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n105A-n258A | CA_n105A-n258A | n105 | 5, 10,15, 20, 25, 30, 35 | 0 |
|  |  | n258 | 50, 100, 200, 400 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is optional in this release of the specification. (From Table 5.3.5-1 of 38.101-1)

NOTE 2:	The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to.

NOTE 3: 	The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively.

NOTE 4:	This UE channel bandwidth is optional in this release of the specification.

NOTE 5:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a SCell part of DC or CA configuration (In Table 5.3.5-1 in 38.101-1).

NOTE 6: 	The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively.

#### 5.5A.1.2	Inter-band CA configurations between FR1 and FR2 (three bands)

##### Table 5.5A.1.2-1a

Table 5.5A.1.2-1a: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (three bands)

| NR CA configuration | Uplink configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n257A | CA_n1A-n3A CA_n1A-n257A CA_n3A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n257G | CA_n1A-n3A CA_n1A-n257A/G CA_n3A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n257H | CA_n1A-n3A CA_n1A-n257A/G/H CA_n3A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n257I | CA_n1A-n3A CA_n1A-n257A/G/H/I CA_n3A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n1A-n3A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n1A-n3A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n1A-n3A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n1A-n3A-n258A | CA_n1A-n3A CA_n1A-n258A CA_n3A-n258A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n258D | CA_n1A-n3A CA_n1A-n258A CA_n3A-n258A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n1A-n3A-n258G | CA_n1A-n3A CA_n1A-n258A/G CA_n3A-n258A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n1A-n3A-n258H | CA_n1A-n3A CA_n1A-n258A/G/H CA_n3A-n258A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n1A-n3A-n258I | CA_n1A-n3A CA_n1A-n258A/G/H/I CA_n3A-n258A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n1A-n3A-n258J | CA_n1A-n3A CA_n1A-n258A/G/H/I CA_n3A-n258A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n1A-n5A-n258A | CA_n1A-n5A CA_n1A-n258A CA_n5A-n258A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n1A-n8A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n8A-n257D | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n1A-n8A-n257E | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257E |  |  |
| CA_n1A-n8A-n257F | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257F |  |  |
| CA_n1A-n8A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n8A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n8A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n8A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n1A-n8A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n1A-n8A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n1A-n8A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n1A-n18A-n257A | CA_n1A-n18A CA_n1A-n257A CA_n18A-n257A |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n18A-n257G | CA_n1A-n18A CA_n1A-n257A/G CA_n18A-n257A/G |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n18A-n257H | CA_n1A-n18A CA_n1A-n257A/G/H CA_n18A-n257A/G/H |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n18A-n257I | CA_n1A-n18A CA_n1A-n257A/G/H/I CA_n1A-n257I CA_n18A-n257A/G/H/I |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n257A | CA_n1A-n28A CA_n1A-n257A CA_n28A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n257G | CA_n257G CA_n1A-n28A CA_n1A-n257A/G CA_n28A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n257H | CA_n257G/H CA_n1A-n28A CA_n1A-n257A/G/H CA_n28A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n257I | CA_n257G/H/I CA_n1A-n28A CA_n1A-n257A/G/H/I CA_n28A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n258A | CA_n1A-n28A CA_n1A-n258A CA_n28A-n258A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n258D | CA_n1A-n28A CA_n1A-n258A CA_n28A-n258A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n1A-n28A-n258G | CA_n1A-n28A CA_n1A-n258A/G CA_n28A-n258A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n1A-n28A-n258H | CA_n1A-n28A CA_n1A-n258A/G/H CA_n28A-n258A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n1A-n28A-n258I | CA_n1A-n28A CA_n1A-n258A/G/H/I CA_n28A-n258A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n1A-n28A-n258J | CA_n1A-n28A CA_n1A-n258A/G/H/I CA_n28A-n258A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n1A-n40A-n258A | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n1A-n40A-n258D | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n1A-n40A-n258E | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50,60 |  |  |
|  |  |  | n258 | CA_n258E |  |  |
| CA_n1A-n40A-n258F | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258F |  |  |
| CA_n1A-n40A-n258G | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n1A-n40A-n258H | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n1A-n40A-n258I | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n1A-n40A-n258J | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n1A-n40A-n258K | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n1A-n40A-n258L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n1A-n40A-n258M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n1A-n41A-n257A | CA_n1A-n41A CA_n1A-n257A CA_n41A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n41A-n257G | CA_n257G CA_n1A-n41A CA_n1A-n257A/G CA_n41A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n41A-n257H | CA_n257G/H CA_n1A-n41A CA_n1A-n257A/G/H CA_n41A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n41A-n257I | CA_n257G/H/I CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n77A-n257A | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n77A-n257G | CA_n257G CA_n1A-n77A CA_n1A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n77A-n257H | CA_n257G/H CA_n1A-n77A CA_n1A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n77A-n257I | CA_n257G/H/I CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n77A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n1A-n77A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n1A-n77A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n1A-n77A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n1A-n77(2A)-n257A | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n77(2A)-n257G | CA_n1A-n77A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  | CA_n1A-n257A/G |  | n77 | CA_n77(2A) |  |  |
|  | CA_n1A-n257G |  | n257 | CA_n257G |  |  |
| CA_n1A-n77(2A)-n257H | CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n77(2A)-n257I | CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n77(2A)-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n1A-n77(2A)-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n1A-n77(2A)-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n1A-n77(2A)-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n1A-n77(3A)-n257A | CA_n1A-n77A CA_n1A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n77(3A)-n257G | CA_n1A-n77A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  | CA_n1A-n257A/G |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n77(3A)-n257H | CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n77(3A)-n257I |  |  | n1 | 5, 10, 15, 20 | 0 |  |
|  | CA_n1A-n77A |  | n77 | CA_n77(3A) |  |  |
|  | CA_n1A-n257A/G/H |  | n257 | CA_n257I |  |  |
| CA_n1A-n78A-n257A | CA_n1A-n78A CA_n1A-n257A CA_n78A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n78A-n257D | - | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n1A-n78A-n257E | - | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n1A-n78A-n257F | - | n1 |  | 5, 10, 15, 20 | 0 |  |
|  |  | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n1A-n78A-n257G | CA_n257G CA_n1A-n78A CA_n1A-n257A/G CA_n78A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n78A-n257H | CA_n257G/H CA_n1A-n257A/G/H CA_n78A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n78A-n257I | CA_n257G/H/I CA_n1A-n78A CA_n1A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n78A-n257J | CA_n257G/H/I/J CA_n1A-n78A CA_n1A-n257A/G/H/I/J CA_n78A-n257A/G/H/I/J |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n1A-n78A-n257K | CA_n257G/H/I/J/K CA_n1A-n78A CA_n1A-n257A/G/H/I/J/K CA_n78A-n257A/G/H/I/J/K |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n1A-n78A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n1A-n78A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n1A-n78A-n258A | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n1A-n78A-n258D | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n1A-n78A-n258E | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258E |  |  |
| CA_n1A-n78A-n258F | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258F |  |  |
| CA_n1A-n78A-n258G | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n1A-n78A-n258H | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n1A-n78A-n258I | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n1A-n78A-n258J | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n1A-n78A-n258K | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n1A-n78A-n258L | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n1A-n78A-n258M | - |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n1A-n79A-n257A | CA_n1A-n79A CA_n1A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n79A-n257G | CA_n257G CA_n1A-n79A CA_n1A-n257A/G CA_n79A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n1A-n79A-n257H | CA_n257G/H CA_n1A-n79A CA_n1A-n257A/G/H CA_n79A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n1A-n79A-n257I | CA_n257G/H/I CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n1A-n105A-n257A | CA_n1A-n105A CA_n1A-n257A CA_n105A-n257A |  | n1 | 5, 10, 15, 20 |  | 0 |
|  |  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n105A-n258A | CA_n1A-n105A CA_n1A-n258A CA_n105A-n258A |  | n1 | 5, 10, 15, 20 |  | 0 |
|  |  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n2A-n5A-n260A | CA_n2A-n5A CA_n2A-n260A CA_n5A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n5A-n260G | CA_n2A-n5A CA_n2A-n260A/G CA_n5A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n5A-n260H | CA_n2A-n5A CA_n2A-n260A/G/H CA_n5A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n5A-n260I | CA_n2A-n5A CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n5A-n260J | CA_n2A-n5A CA_n2A-n260A/G/H/I/J CA_n5A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n5A-n260K | CA_n2A-n5A CA_n2A-n260A/G/H/I/J/K CA_n5A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n5A-n260L | CA_n2A-n5A CA_n2A-n260A/G/H/I/J/K/L CA_n5A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n5A-n260M | CA_n2A-n5A CA_n2A-n260A/G/H/I/J/K/L/M CA_n5A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n5A-n261A | CA_n2A-n5A CA_n2A-n261A CA_n5A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n5A-n261G | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n5A-n261H | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n5A-n261I | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n5A-n261J | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n5A-n261K | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n5A-n261L | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n5A-n261M | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n5A-n261(2G) | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n5A-n261(G-H) | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n5A-n261(A-G-H) | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n5A-n261(G-I) | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n5A-n261(2H) | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n5A-n261(A-G-I) | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n5A-n261(H-I) | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n5A-n261(2A-G) | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n5A-n261(2A-H) | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n5A-n261(2A-I) | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n5A-n261(2A) | CA_n2A-n5A CA_n2A-n261A CA_n5A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n5A-n261(3A) | CA_n2A-n5A CA_n2A-n261A CA_n5A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n5A-n261(A-G) | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n5A-n261(A-2G) | CA_n2A-n5A CA_n2A-n261A/G CA_n5A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n5A-n261(A-H) | CA_n2A-n5A CA_n2A-n261A/G/H CA_n5A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n5A-n261(A-I) | CA_n2A-n5A CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n5 | 5, 10, 15, 20 |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n12A-n260A | CA_n2A-n12A CA_n2A-n260A CA_n12A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n12A-n260G | CA_n2A-n12A CA_n2A-n260A/G CA_n12A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n12A-n260H | CA_n2A-n12A CA_n2A-n260A/G/H CA_n12A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n12A-n260I | CA_n2A-n12A CA_n2A-n260A/G/H/I CA_n12A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n12A-n260J | CA_n2A-n12A CA_n2A-n260A/G/H/I/J CA_n12A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n12A-n260K | CA_n2A-n12A CA_n2A-n260A/G/H/I/J/K CA_n12A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n12A-n260L |  |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n12A-n260M | CA_n2A-n12A CA_n2A-n260A/G/H/I/J/K/L/M CA_n12A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n12 | 5, 10, 15 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n14A-n260A | CA_n2A-n14A CA_n2A-n260A CA_n14A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n14A-n260G | CA_n2A-n14A CA_n2A-n260A/G CA_n14A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n14A-n260H | CA_n2A-n14A CA_n2A-n260A/G/H CA_n14A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n14A-n260I | CA_n2A-n14A CA_n2A-n260A/G/H/I CA_n14A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n14A-n260J | CA_n2A-n14A CA_n2A-n260A/G/H/I/J CA_n14A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n14A-n260K | CA_n2A-n14A CA_n2A-n260A/G/H/I/J/K CA_n14A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n14A-n260L | CA_n2A-n14A CA_n2A-n260A/G/H/I/J/K/L CA_n14A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n14A-n260M | CA_n2A-n14A CA_n2A-n260A/G/H/I/J/K/L/M CA_n14A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n14 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n30A-n260A | CA_n2A-n30A CA_n2A-n260A CA_n30A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n30A-n260G | CA_n2A-n30A CA_n2A-n260A/G CA_n30A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n30A-n260H | CA_n2A-n30A CA_n2A-n260A/G/H CA_n30A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n30A-n260I | CA_n2A-n30A CA_n2A-n260A/G/H/I CA_n30A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n30A-n260J | CA_n2A-n30A CA_n2A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n30A-n260K | CA_n2A-n30A CA_n2A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n30A-n260L | CA_n2A-n30A CA_n2A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n30A-n260M | CA_n2A-n30A CA_n2A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n30 | 5, 10 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n48A-n260A | CA_n2A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n48A-n260G | CA_n2A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n48A-n260H | CA_n2A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n48A-n260I | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n48A-n260J | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n48A-n260K | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n48A-n260L | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n48A-n260M | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n48(2A)-n260A | CA_n2A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n48(2A)-n260G | CA_n2A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n48(2A)-n260H | CA_n2A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n48(2A)-n260I | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n48(2A)-n260J | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n48(2A)-n260K | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n48(2A)-n260L | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n48(2A)-n260M | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n48B-n260A | CA_n2A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n48B-n260G | CA_n2A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n48B-n260H | CA_n2A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n48B-n260I | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n48B-n260J | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n48B-n260K | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n48B-n260L | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n48B-n260M | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n48A-n261A | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n48A-n261G | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n48A-n261H | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n48A-n261I | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n48A-n261J | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n48A-n261K | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n48A-n261L | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n48A-n261M | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n48A-n261(A-G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n48A-n261(A-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n48A-n261(A-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n48A-n261(G-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n48A-n261(2A-G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n48A-n261(2A-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n48A-n261(A-2G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n48A-n261(A-G-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n48A-n261(2A) | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n48A-n261(3A) | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n48A-n261(2G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n48A-n261(2H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n48A-n261(G-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n48A-n261(H-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n48A-n261(2A-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n48A-n261(A-G-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n48(2A)-n261A | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n48(2A)-n261G | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n48(2A)-n261H | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n48(2A)-n261I | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n48(2A)-n261J | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n48(2A)-n261K | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n48(2A)-n261L | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n48(2A)-n261M | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n48(2A)-n261(A-G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n48(2A)-n261(A-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n48(2A)-n261(A-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n48(2A)-n261(G-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n48(2A)-n261(2A-G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n48(2A)-n261(2A-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n48(2A)-n261(A-2G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n48(2A)-n261(A-G-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n48(2A)-n261(2A) | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n48(2A)-n261(3A) | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n48(2A)-n261(2G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n48(2A)-n261(2H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n48(2A)-n261(G-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n48(2A)-n261(H-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n48(2A)-n261(2A-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n48(2A)-n261(A-G-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n48B-n261A | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n48B-n261G | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n48B-n261H | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n48B-n261I | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n48B-n261J | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n48B-n261K | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n48B-n261L | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n48B-n261M | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n48B-n261(A-G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n48B-n261(A-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n48B-n261(A-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n48B-n261(G-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n48B-n261(2A-G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n48B-n261(2A-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n48B-n261(A-2G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n48B-n261(A-G-H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n48B-n261(2A) | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n48B-n261(3A) | CA_n2A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n48B-n261(2G) | CA_n2A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n48B-n261(2H) | CA_n2A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n48B-n261(G-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n48B-n261(H-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n48B-n261(2A-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n48B-n261(A-G-I) | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n48 | CA_n48B |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n66A-n260A | CA_n2A-n66A CA_n2A-n260A CA_n66A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n66A-n260G | CA_n2A-n66A CA_n2A-n260A/G CA_n66A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n66A-n260H | CA_n2A-n66A CA_n2A-n260A/G/H CA_n66A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n66A-n260I | CA_n2A-n66A CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n66A-n260J | CA_n2A-n66A CA_n2A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n66A-n260K | CA_n2A-n66A CA_n2A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n66A-n260L | CA_n2A-n66A CA_n2A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n66A-n260M | CA_n2A-n66A CA_n2A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n66A-n261A | CA_n2A-n66A CA_n2A-n261A CA_n66A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n66A-n261G | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n66A-n261H | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n66A-n261I | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n66A-n261J | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n66A-n261K | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n66A-n261L | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n66A-n261M | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n66A-n261(2G) | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n66A-n261(G-H) | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n66A-n261(A-G-H) | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n66A-n261(G-I) | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n66A-n261(2H) | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n66A-n261(A-G-I) | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n66A-n261(H-I) | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n66A-n261(2A-G) | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n66A-n261(2A-H) | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n66A-n261(2A-I) | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n66A-n261(2A) | CA_n2A-n66A CA_n2A-n261A CA_n66A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n66A-n261(3A) | CA_n2A-n66A CA_n2A-n261A CA_n66A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n66A-n261(A-2G) | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n66A-n261(A-G) | CA_n2A-n66A CA_n2A-n261A/G CA_n66A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n66A-n261(A-H) | CA_n2A-n66A CA_n2A-n261A/G/H CA_n66A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n66A-n261(A-I) | CA_n2A-n66A CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n77A-n260A | CA_n2A-n77A CA_n77A-n260A CA_n2A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n2A-n77A-n260G | CA_n2A-n77A CA_n2A-n260A/G CA_n77A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n77A-n260H | CA_n2A-n77A CA_n2A-n260A/G/H CA_n77A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n77A-n260I | CA_n2A-n77A CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n77A-n260J | CA_n2A-n77A CA_n2A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n77A-n260K | CA_n2A-n77A CA_n2A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n77A-n260L | CA_n2A-n77A CA_n2A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n77A-n260M | CA_n2A-n77A CA_n2A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n77C-n260A | CA_n2A-n260A CA_n77A-n260A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260A |  |  |
| CA_n2A-n77C-n260G | CA_n2A-n260A/G CA_n77A-n260A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260G |  |  |
| CA_n2A-n77C-n260H | CA_n2A-n260A/G/H CA_n77A-n260A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260H |  |  |
| CA_n2A-n77C-n260I | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260I |  |  |
| CA_n2A-n77C-n260J | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260J |  |  |
| CA_n2A-n77C-n260K | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260K |  |  |
| CA_n2A-n77C-n260L | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260L |  |  |
| CA_n2A-n77C-n260M | CA_n2A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n260 | CA_n260M |  |  |
| CA_n2A-n77A-n261A | CA_n77A-n261A CA_n2A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n2A-n77A-n261G | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n77A-n261H | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n77A-n261I | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n77A-n261J | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n77A-n261K | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n77A-n261L | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n77A-n261M | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n77A-n261(A-G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n77A-n261(A-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n77A-n261(G-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n77A-n261(2A-G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n77A-n261(2A-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n77A-n261(A-2G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n77A-n261(A-G-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n77A-n261(A-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n77A-n261(G-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n77A-n261(2A) | CA_n2A-n261A CA_n77A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n77A-n261(3A) | CA_n2A-n261A CA_n77A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n77A-n261(2G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n77A-n261(2H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n77A-n261(2A-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n77A-n261(A-G-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n2A-n77A-n261(H-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n77C-n261A | CA_n2A-n261A CA_n77A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261A |  |  |
| CA_n2A-n77C-n261G | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261G |  |  |
| CA_n2A-n77C-n261H | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261H |  |  |
| CA_n2A-n77C-n261I | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261I |  |  |
| CA_n2A-n77C-n261J | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261J |  |  |
| CA_n2A-n77C-n261K | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261K |  |  |
| CA_n2A-n77C-n261L | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261L |  |  |
| CA_n2A-n77C-n261M | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261M |  |  |
| CA_n2A-n77C-n261(A-G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |
| CA_n2A-n77C-n261(A-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |
| CA_n2A-n77C-n261(G-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |
| CA_n2A-n77C-n261(2A-G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n2A-n77C-n261(2A-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n2A-n77C-n261(A-2G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n2A-n77C-n261(A-G-H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n2A-n77C-n261(A-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |
| CA_n2A-n77C-n261(G-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |
| CA_n2A-n77C-n261(2A) | CA_n2A-n261A CA_n77A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A) |  |  |
| CA_n2A-n77C-n261(3A) | CA_n2A-n261A CA_n77A-n261A |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(3A) |  |  |
| CA_n2A-n77C-n261(2G) | CA_n2A-n261A/G CA_n77A-n261A/G |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2G) |  |  |
| CA_n2A-n77C-n261(2H) | CA_n2A-n261A/G/H CA_n77A-n261A/G/H |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2H) |  |  |
| CA_n2A-n77C-n261(H-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |
| CA_n2A-n77C-n261(2A-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n2A-n77C-n261(A-G-I) | CA_n2A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n2 | 5, 10, 15, 20 | 0 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n3A-n7A-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n7A-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n7A-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n7A-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n7A-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3A-n7A-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3A-n7A-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3A-n7A-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3B-n7A-n257A | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3B-n7A-n257G | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3B-n7A-n257H | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3B-n7A-n257I | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3B-n7A-n257J | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3B-n7A-n257K | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3B-n7A-n257L | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3B-n7A-n257M | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3(2A)-n7A-n257A | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n3(2A)-n7A-n257G | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n3(2A)-n7A-n257H | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n3(2A)-n7A-n257I | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n3(2A)-n7A-n257J | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n3(2A)-n7A-n257K | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n3(2A)-n7A-n257L | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n3(2A)-n7A-n257M | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n3A-n7A-n258A | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3A-n7A-n258B | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258B |  |  |
| CA_n3A-n7A-n258C | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258C |  |  |
| CA_n3A-n7A-n258D | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n3A-n7A-n258E | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258E |  |  |
| CA_n3A-n7A-n258F | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258F |  |  |
| CA_n3A-n7A-n258G | CA_n3A-n258A/G CA_n7A-n258A/G CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3A-n7A-n258H | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3A-n7A-n258I | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3A-n7A-n258J | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3A-n7A-n258K | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3A-n7A-n258L | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3A-n7A-n258M | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3A-n7B-n258A | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3A-n7B-n258B | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258B |  |  |
| CA_n3A-n7B-n258C | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258C |  |  |
| CA_n3A-n7B-n258D | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n3A-n7B-n258E | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258E |  |  |
| CA_n3A-n7B-n258F | CA_n3A-n258A CA_n7A-n258A CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258F |  |  |
| CA_n3A-n7B-n258G | CA_n3A-n258A/G CA_n7A-n258A/G CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3A-n7B-n258H | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3A-n7B-n258I | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3A-n7B-n258J | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3A-n7B-n258K | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3A-n7B-n258L | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3A-n7B-n258M | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n3A-n7A CA_n7B |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n7 | CA_n7B |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3B-n7A-n258A | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3B-n7A-n258G | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3B-n7A-n258H | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3B-n7A-n258I | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3B-n7A-n258J | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3B-n7A-n258K | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3B-n7A-n258L | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3B-n7A-n258M | - |  | n3 | CA_n3B_BCS0 | 0 |  |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3(2A)-n7A-n258A | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n3(2A)-n7A-n258G | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n3(2A)-n7A-n258H | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n3(2A)-n7A-n258I | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n3(2A)-n7A-n258J | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n3(2A)-n7A-n258K | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258K |  |  |
| CA_n3(2A)-n7A-n258L | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258L |  |  |
| CA_n3(2A)-n7A-n258M | - | n3 |  | CA_n3(2A)_BCS1 | 0 |  |
|  |  | n7 |  | 5, 10, 15, 20, 25, 30, 35, 40, 50 |  |  |
|  |  | n258 |  | CA_n258M |  |  |
| CA_n3A-n8A-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n8A-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n8A-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n8A-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n8A-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3A-n8A-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3A-n8A-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3A-n8A-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3A-n18A-n257A | CA_n3A-n18A CA_n3A-n257A CA_n18A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n18A-n257G | CA_n3A-n18A CA_n3A-n257A/G CA_n18A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n18A-n257H | CA_n3A-n18A CA_n3A-n257A/G/H CA_n18A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n18A-n257I | CA_n3A-n18A CA_n3A-n257A/G/H/I CA_n18A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n18 | 5, 10, 15 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n28A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n28A CA_n3A-n257A CA_n28A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n28A-n257D | CA_n3A-n28A CA_n3A-n257A/D CA_n28A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n3A-n28A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n28A CA_n3A-n257A/G CA_n28A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n28A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n28A CA_n3A-n257A/G/H CA_n28A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n28A-n257I | CA_n3A-n28A CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n28A-n258A | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3A-n28A-n258D | CA_n3A-n28A CA_n3A-n258A CA_n28A-n258A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n3A-n28A-n258G | CA_n3A-n28A CA_n3A-n258A/G CA_n28A-n258A/G |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3A-n28A-n258H | CA_n3A-n28A CA_n3A-n258A/G/H CA_n28A-n258A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3A-n28A-n258I | CA_n3A-n28A CA_n3A-n258A/G/H/I CA_n28A-n258A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3A-n28A-n258J | CA_n3A-n28A CA_n3A-n258A/G/H/I CA_n28A-n258A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3A-n41A-n257A | CA_n3A-n41A CA_n3A-n257A CA_n41A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n41A-n257G | CA_n3A-n41A CA_n3A-n257A/G CA_n41A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n41A-n257H | CA_n3A-n41A CA_n3A-n257A/G/H CA_n41A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n41A-n257I | CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n77A-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n77A-n257D | CA_n3A-n77A CA_n3A-n257A/D CA_n77A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n3A-n77A-n257G | CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n77A-n257H | CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n77A-n257I | CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n77A-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3A-n77A-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3A-n77A-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3A-n77A-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3A-n77(2A)-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n77(2A)-n257D | CA_n3A-n77A CA_n3A-n257A/D CA_n77A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n3A-n77(2A)-n257G | CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n77(2A)-n257H | CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n77(2A)-n257I | CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n77(2A)-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3A-n77(2A)-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3A-n77(2A)-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3A-n77(2A)-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(2A) |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3A-n77(3A)-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n77(3A)-n257D | CA_n3A-n77A CA_n3A-n257A/D CA_n77A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n3A-n77(3A)-n257G | CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n77(3A)-n257H | CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n77(3A)-n257I | CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n77 | CA_n77(3A) |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n78A-n257A | CA_n3A-n78A CA_n3A-n257A CA_n78A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n78A-n257D | CA_n3A-n78A CA_n3A-n257A/D CA_n78A-n257A/D |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n3A-n78A-n257G | CA_n3A-n78A CA_n3A-n257A/G CA_n78A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n78A-n257H | CA_n3A-n78A CA_n3A-n257A/G/H CA_n78A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n78A-n257I | CA_n3A-n78A CA_n3A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n78A-n258A | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3A-n78A-n258B | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258B |  |  |
| CA_n3A-n78A-n258C | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258C |  |  |
| CA_n3A-n78A-n258D | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n3A-n78A-n258E | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258E |  |  |
| CA_n3A-n78A-n258F | CA_n3A-n258A CA_n78A-n258A CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258F |  |  |
| CA_n3A-n78A-n258G | CA_n3A-n258A/G CA_n78A-n258A/G CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3A-n78A-n258H | CA_n3A-n258A/G/H CA_n78A-n258A/G/H CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3A-n78A-n258I | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3A-n78A-n258J | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3A-n78A-n258K | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3A-n78A-n258L | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3A-n78A-n258M | CA_n3A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3A-n79A-n257A | CA_n3A-n79A CA_n3A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n79A-n257G | CA_n257G CA_n3A-n79A CA_n3A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n79A-n257H | CA_n257G/H CA_n3A-n79A CA_n3A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n79A-n257I | CA_n257G/H/I CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n79A-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3A-n79A-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3A-n79A-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3A-n79A-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3A-n79A-n258A | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3A-n79A-n258G | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3A-n79A-n258H | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3A-n79A-n258I | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3A-n79A-n258J | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3A-n79A-n258K | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3A-n79A-n258L | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3A-n79A-n258M | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3A-n79C-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n79C-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3A-n79C-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3A-n79C-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3A-n79C-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3A-n79C-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3A-n79C-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3A-n79C-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3A-n79C-n258A | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3A-n79C-n258G | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3A-n79C-n258H | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3A-n79C-n258I | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3A-n79C-n258J | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3A-n79C-n258K | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3A-n79C-n258L | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3A-n79C-n258M | - |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3(2A)-n79A-n257A | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3(2A)-n79A-n257G | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3(2A)-n79A-n257H | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3(2A)-n79A-n257I | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3(2A)-n79A-n257J | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3(2A)-n79A-n257K | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3(2A)-n79A-n257L | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3(2A)-n79A-n257M | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3(2A)-n79A-n258A | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3(2A)-n79A-n258G | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3(2A)-n79A-n258H | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3(2A)-n79A-n258I | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3(2A)-n79A-n258J | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3(2A)-n79A-n258K | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3(2A)-n79A-n258L | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3(2A)-n79A-n258M | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3(2A)-n79C-n257A | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3(2A)-n79C-n257G | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3(2A)-n79C-n257H | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3(2A)-n79C-n257I | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3(2A)-n79C-n257J | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3(2A)-n79C-n257K | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3(2A)-n79C-n257L | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3(2A)-n79C-n257M | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3(2A)-n79C-n258A | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3(2A)-n79C-n258G | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3(2A)-n79C-n258H | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3(2A)-n79C-n258I | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3(2A)-n79C-n258J | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3(2A)-n79C-n258K | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3(2A)-n79C-n258L | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3(2A)-n79C-n258M | - |  | n3 | CA_n3(2A)_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3B-n79A-n257A | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3B-n79A-n257G | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3B-n79A-n257H | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3B-n79A-n257I | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3B-n79A-n257J | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3B-n79A-n257K | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3B-n79A-n257L | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3B-n79A-n257M | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3B-n79A-n258A | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3B-n79A-n258G | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3B-n79A-n258H | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3B-n79A-n258I | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3B-n79A-n258J | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3B-n79A-n258K | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3B-n79A-n258L | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3B-n79A-n258M | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3B-n79C-n257A | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3B-n79C-n257G | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n3B-n79C-n257H | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n3B-n79C-n257I | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n3B-n79C-n257J | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n3B-n79C-n257K | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n3B-n79C-n257L | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n3B-n79C-n257M | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n3B-n79C-n258A | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n3B-n79C-n258G | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n3B-n79C-n258H | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n3B-n79C-n258I | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n3B-n79C-n258J | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n3B-n79C-n258K | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n3B-n79C-n258L | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n3B-n79C-n258M | - |  | n3 | CA_n3B_BCS0 |  | 0 |
|  |  |  | n79 | CA_n79C_BCS0 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n3A-n105A-n257A | CA_n3A-n105A CA_n3A-n257A CA_n105A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n105A-n258A | CA_n3A-n105A CA_n3A-n258A CA_n105A-n258A |  | n3 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |

##### Table 5.5A.1.2-1b

Table 5.5A.1.2-1b: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (three bands)

| NR CA configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- |
| CA_n5A-n30A-n260A | CA_n5A-n30A CA_n5A-n260A CA_n30A-n260A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n30A-n260G | CA_n5A-n30A CA_n5A-n260A/G CA_n30A-n260A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n30A-n260H | CA_n5A-n30A CA_n5A-n260A/G/H CA_n30A-n260A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n30A-n260I | CA_n5A-n30A CA_n5A-n260A/G/H/I CA_n30A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n30A-n260J | CA_n5A-n30A CA_n5A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n30A-n260K | CA_n5A-n30A CA_n5A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n30A-n260L | CA_n5A-n30A CA_n5A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n30A-n260M | CA_n5A-n30A CA_n5A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n48A-n260A | CA_n5A-n260A CA_n48A-n260A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n48A-n260G | CA_n5A-n260A/G CA_n48A-n260A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n48A-n260H | CA_n5A-n260A/G/H CA_n48A-n260A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n48A-n260I | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n48A-n260J | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n48A-n260K | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n48A-n260L | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n48A-n260M | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n48(2A)-n260A | CA_n5A-n260A CA_n48A-n260A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n48(2A)-n260G | CA_n5A-n260A/G CA_n48A-n260A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n48(2A)-n260H | CA_n5A-n260A/G/H CA_n48A-n260A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n48(2A)-n260I | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n48(2A)-n260J | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n48(2A)-n260K | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n48(2A)-n260L | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n48(2A)-n260M | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n48B-n260A | CA_n5A-n260A CA_n48A-n260A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n48B-n260G | CA_n5A-n260A/G CA_n48A-n260A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n48B-n260H | CA_n5A-n260A/G/H CA_n48A-n260A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n48B-n260I | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n48B-n260J | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n48B-n260K | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n48B-n260L | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n48B-n260M | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n48A-n261A | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n5A-n48A-n261G | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261G |  |  |
| CA_n5A-n48A-n261H | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261H |  |  |
| CA_n5A-n48A-n261I | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261I |  |  |
| CA_n5A-n48A-n261J | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261J |  |  |
| CA_n5A-n48A-n261K | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261K |  |  |
| CA_n5A-n48A-n261L | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261L |  |  |
| CA_n5A-n48A-n261M | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261M |  |  |
| CA_n5A-n48A-n261(A-G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n48A-n261(A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n48A-n261(A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n48A-n261(G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n48A-n261(2A-G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n48A-n261(2A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n48A-n261(A-2G) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n5A-n48A-n261(A-G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n48A-n261(2A) | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n5A-n48A-n261(3A) | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n5A-n48A-n261(2G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n48A-n261(2H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n48A-n261(G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n48A-n261(H-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n48A-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n48A-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n48(2A)-n261A | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n5A-n48(2A)-n261G | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261G |  |  |
| CA_n5A-n48(2A)-n261H | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261H |  |  |
| CA_n5A-n48(2A)-n261I | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261I |  |  |
| CA_n5A-n48(2A)-n261J | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261J |  |  |
| CA_n5A-n48(2A)-n261K | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261K |  |  |
| CA_n5A-n48(2A)-n261L | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261L |  |  |
| CA_n5A-n48(2A)-n261M | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261M |  |  |
| CA_n5A-n48(2A)-n261(A-G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n48(2A)-n261(A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n48(2A)-n261(A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n48(2A)-n261(G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n48(2A)-n261(2A-G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n48(2A)-n261(2A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n48(2A)-n261(A-2G) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n5A-n48(2A)-n261(A-G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n48(2A)-n261(2A) | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n5A-n48(2A)-n261(3A) | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n5A-n48(2A)-n261(2G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n48(2A)-n261(2H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n48(2A)-n261(G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n48(2A)-n261(H-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n48(2A)-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n48(2A)-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |  |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n48B-n261A | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 | 0 |  |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n5A-n48B-n261G | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261G |  |  |
| CA_n5A-n48B-n261H | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261H |  |  |
| CA_n5A-n48B-n261I | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261I |  |  |
| CA_n5A-n48B-n261J | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261J |  |  |
| CA_n5A-n48B-n261K | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261K |  |  |
| CA_n5A-n48B-n261L | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261L |  |  |
| CA_n5A-n48B-n261M | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261M |  |  |
| CA_n5A-n48B-n261(A-G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n48B-n261(A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n48B-n261(A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n48B-n261(G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n48B-n261(2A-G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n48B-n261(2A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n48B-n261(A-2G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n5A-n48B-n261(A-G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n48B-n261(2A) | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n5A-n48B-n261(3A) | CA_n5A-n261A CA_n48A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n5A-n48B-n261(2G) | CA_n5A-n261A/G CA_n48A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n48B-n261(2H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n48B-n261(G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n48B-n261(H-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n48B-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n48B-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n48 | CA_n48B |  |  |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n66A-n260A | CA_n5A-n66A CA_n5A-n260A CA_n66A-n260A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n66A-n260G | CA_n5A-n66A CA_n5A-n260A/G CA_n66A-n260A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n66A-n260H | CA_n5A-n66A CA_n5A-n260A/G/H CA_n66A-n260A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n66A-n260I | CA_n5A-n66A CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n66A-n260J | CA_n5A-n66A CA_n5A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n66A-n260K | CA_n5A-n66A CA_n5A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n66A-n260L | CA_n5A-n66A CA_n5A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n66A-n260M | CA_n5A-n66A CA_n5A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n66A-n261A | CA_n5A-n66A CA_n5A-n261A CA_n66A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n5A-n66A-n261G | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261G |  |  |
| CA_n5A-n66A-n261H | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261H |  |  |
| CA_n5A-n66A-n261I | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261I |  |  |
| CA_n5A-n66A-n261J | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261J |  |  |
| CA_n5A-n66A-n261K | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261K |  |  |
| CA_n5A-n66A-n261L | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261L |  |  |
| CA_n5A-n66A-n261M | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261M |  |  |
| CA_n5A-n66A-n261(2G) | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n66A-n261(G-H) | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n66A-n261(A-G-H) | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n66A-n261(G-I) | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n66A-n261(2H) | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n66A-n261(A-G-I) | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n66A-n261(H-I) | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n66A-n261(2A-G) | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n66A-n261(2A-H) | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n66A-n261(2A-I) | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n66A-n261(2A) | CA_n5A-n66A CA_n5A-n261A CA_n66A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n5A-n66A-n261(3A) | CA_n5A-n66A CA_n5A-n261A CA_n66A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n5A-n66A-n261(A-G) | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n66A-n261(A-2G) | CA_n5A-n66A CA_n5A-n261A/G CA_n66A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n5A-n66A-n261(A-H) | CA_n5A-n66A CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n66A-n261(A-I) | CA_n5A-n66A CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n77A-n260A | CA_n5A-n77A CA_n77A-n260A CA_n5A-n260A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n77A-n260G | CA_n5A-n77A CA_n5A-n260A/G CA_n77A-n260A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n77A-n260H | CA_n5A-n77A CA_n5A-n260A/G/H CA_n77A-n260A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n77A-n260I | CA_n5A-n77A CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n77A-n260J | CA_n5A-n77A CA_n5A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n77A-n260K | CA_n5A-n77A CA_n5A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n77A-n260L | CA_n5A-n77A CA_n5A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n77A-n260M | CA_n5A-n77A CA_n5A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n77C-n260A | CA_n5A-n77A CA_n5A-n260A CA_n77A-n260A | n5 | 5, 10, 15, 20, 25 |  | 0 |
|  |  | n77 | CA_n77C |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
|  |  | n5 | 5 |  | 1 |
|  |  | n77 | CA_n77C |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n5A-n77C-n260G | CA_n5A-n260A/G CA_n77A-n260A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n5A-n77C-n260H | CA_n5A-n260A/G/H CA_n77A-n260A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n5A-n77C-n260I | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n5A-n77C-n260J | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n5A-n77C-n260K | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n5A-n77C-n260L | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n5A-n77C-n260M | CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n5A-n77A-n261A | CA_n77A-n261A CA_n5A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | 50, 100, 200, 400 |  |  |
| CA_n5A-n77A-n261G | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261G |  |  |
| CA_n5A-n77A-n261H | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261H |  |  |
| CA_n5A-n77A-n261I | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261I |  |  |
| CA_n5A-n77A-n261J | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261J |  |  |
| CA_n5A-n77A-n261K | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261K |  |  |
| CA_n5A-n77A-n261L | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261L |  |  |
| CA_n5A-n77A-n261M | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261M |  |  |
| CA_n5A-n77A-n261(A-G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n77A-n261(A-H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n77A-n261(G-H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n77A-n261(2A-G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n77A-n261(2A-H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n77A-n261(A-2G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n5A-n77A-n261(A-G-H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n77A-n261(A-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n77A-n261(G-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n77A-n261(2A) | CA_n5A-n261A CA_n77A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n5A-n77A-n261(3A) | CA_n5A-n261A CA_n77A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n5A-n77A-n261(2G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n77A-n261(2H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n77A-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n77A-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n77A-n261(H-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n77C-n261A | CA_n5A-n261A CA_n77A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261A |  |  |
| CA_n5A-n77C-n261G | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261G |  |  |
| CA_n5A-n77C-n261H | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261H |  |  |
| CA_n5A-n77C-n261I | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261I |  |  |
| CA_n5A-n77C-n261J | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261J |  |  |
| CA_n5A-n77C-n261K | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261K |  |  |
| CA_n5A-n77C-n261L | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261L |  |  |
| CA_n5A-n77C-n261M | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261M |  |  |
| CA_n5A-n77C-n261(A-G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(A-G) |  |  |
| CA_n5A-n77C-n261(A-H) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(A-H) |  |  |
| CA_n5A-n77C-n261(G-H) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(G-H) |  |  |
| CA_n5A-n77C-n261(2A-G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(2A-G) |  |  |
| CA_n5A-n77C-n261(2A-H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(2A-H) |  |  |
| CA_n5A-n77C-n261(A-2G) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(A-2G) |  |  |
| CA_n5A-n77C-n261(A-G-H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(A-G-H) |  |  |
| CA_n5A-n77C-n261(A-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(A-I) |  |  |
| CA_n5A-n77C-n261(G-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(G-I) |  |  |
| CA_n5A-n77C-n261(2A) | CA_n5A-n261A CA_n77A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(2A) |  |  |
| CA_n5A-n77C-n261(3A) | CA_n5A-n261A CA_n77A-n261A | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(3A) |  |  |
| CA_n5A-n77C-n261(2G) | CA_n5A-n261A/G CA_n77A-n261A/G | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(2G) |  |  |
| CA_n5A-n77C-n261(2H) | CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(2H) |  |  |
| CA_n5A-n77C-n261(H-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(H-I) |  |  |
| CA_n5A-n77C-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(2A-I) |  |  |
| CA_n5A-n77C-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n5 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |  |
|  |  | n261 | CA_n261(A-G-I) |  |  |
| CA_n5A-n78A-n258A | CA_n5A-n78A CA_n78A-n258A CA_n5A-n258A | n5 | 5, 10, 15, 20, 25 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7A-n25A-n257A | CA_n7A-n257A CA_n25A-n257A | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |
|  |  | n257 | See n257 channel bandwidths in 38.101-2 Table 5.3.5-1 |  |  |
| CA_n7A-n25A-n257G | CA_n7A-n257A/G CA_n25A-n257A/G | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n7A-n25A-n257H | CA_n7A-n257A/G/H CA_n25A-n257A/G/H | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n7A-n25A-n257I | CA_n7A-n257A/G/H/I CA_n25A-n257A/G/H/I | n7 | See n7 channel bandwidths in 38.101-1 Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in 38.101-1 Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n7A-n25A-n257J | CA_n7A-n257A/G/H/I/J CA_n25A-n257A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n7A-n25A-n257K | CA_n7A-n257A/G/H/I/J/K CA_n25A-n257A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n7A-n25A-n257L | CA_n7A-n257A/G/H/I/J/K/L CA_n25A-n257A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n7A-n25A-n257M | CA_n7A-n257A/G/H/I/J/K/L/M CA_n25A-n257A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n7A-n25A-n260A | CA_n7A-n260A CA_n25A-n260A | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n7A-n25A-n260G | CA_n7A-n260A/G CA_n25A-n260A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n7A-n25A-n260H | CA_n7A-n260A/G/H CA_n25A-n260A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n7A-n25A-n260I | CA_n7A-n260A/G/H/I CA_n25A-n260A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n7A-n25A-n260J | CA_n7A-n260A/G/H/I/J CA_n25A-n260A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n7A-n25A-n260K | CA_n7A-n260A/G/H/I/J/K CA_n25A-n260A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n7A-n25A-n260L | CA_n7A-n260A/G/H/I/J/K/L CA_n25A-n260A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n7A-n25A-n260M | CA_n7A-n260A/G/H/I/J/K/L/M CA_n25A-n260A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n7A-n26A-n258A | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7A-n26A-n258B | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n7A-n26A-n258C | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n7A-n26A-n258D | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n7A-n26A-n258E | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n7A-n26A-n258F | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n7A-n26A-n258G | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n7A-n26A-n258H | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n7A-n26A-n258I | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n7A-n26A-n258J | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n7A-n26A-n258K | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n7A-n26A-n258L | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n7A-n26A-n258M | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n7A-n26A-n258R2 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n7A-n26A-n258R3 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n7A-n26A-n258R4 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n7A-n26A-n258R5 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n7A-n26A-n258R6 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n7A-n26A-n258R7 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n7A-n26A-n258R8 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n7A-n26A-n258R9 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n7A-n26A-n258R10 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n7B-n26A-n258A | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7B-n26A-n258B | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n7B-n26A-n258C | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n7B-n26A-n258D | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n7B-n26A-n258E | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n7B-n26A-n258F | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n7B-n26A-n258G | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n7B-n26A-n258H | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n7B-n26A-n258I | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n7B-n26A-n258J | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n7B-n26A-n258K | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n7B-n26A-n258L | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n7B-n26A-n258M | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n7B-n26A-n258R2 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n7B-n26A-n258R3 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n7B-n26A-n258R4 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n7B-n26A-n258R5 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n7B-n26A-n258R6 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n7B-n26A-n258R7 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n7B-n26A-n258R8 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n7B-n26A-n258R9 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n7B-n26A-n258R10 | CA_n7A-n26A CA_n7A-n258A CA_n26A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n7A-n66A-n257A | CA_n7A-n257A CA_n66A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 4 and 5 |  |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n7A-n66A-n257G | CA_n7A-n257A/G CA_n66A-n257A/G | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n7A-n66A-n257H | CA_n7A-n257A/G/H CA_n66A-n257A/G/H | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n7A-n66A-n257I | CA_n7A-n257A/G/H/I CA_n66A-n257A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n66 | 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n7A-n66A-n257J | CA_n7A-n257A/G/H/I/J CA_n66A-n257A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n7A-n66A-n257K | CA_n7A-n257A/G/H/I/J/K CA_n66A-n257A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n7A-n66A-n257L | CA_n7A-n257A/G/H/I/J/K/L CA_n66A-n257A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n7A-n66A-n257M | CA_n7A-n257A/G/H/I/J/K/L/M CA_n66A-n257A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n7A-n66A-n260A | CA_n7A-n260A CA_n66A-n260A | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n7A-n66A-n260G | CA_n7A-n260A/G CA_n66A-n260A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n7A-n66A-n260H | CA_n7A-n260A/G/H CA_n66A-n260A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n7A-n66A-n260I | CA_n7A-n260A/G/H/I CA_n66A-n260A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n7A-n66A-n260J | CA_n7A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n7A-n66A-n260K | CA_n7A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n7A-n66A-n260L | CA_n7A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n7A-n66A-n260M | CA_n7A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n7A-n71A-n257A | CA_n7A-n257A CA_n71A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n71 | 5, 10, 15, 20 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n7A-n71A-n257G | CA_n7A-n257A/G CA_n71A-n257A/G | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n71 | 5, 10, 15, 20 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n7A-n71A-n257H | CA_n7A-n257A/G/H CA_n71A-n257A/G/H | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n71 | 5, 10, 15, 20 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n7A-n71A-n257I | CA_n7A-n257A/G/H/I CA_n71A-n257G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 4 and 5 |
|  |  | n71 | 5, 10, 15, 20 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n7A-n71A-n257J | CA_n7A-n257A/G/H/I/J CA_n71A-n257A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n7A-n71A-n257K | CA_n7A-n257A/G/H/I/J/K CA_n71A-n257A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n7A-n71A-n257L | CA_n7A-n257A/G/H/I/J/K/L CA_n71A-n257A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n7A-n71A-n257M | CA_n7A-n257A/G/H/I/J/K/L/M CA_n71A-n257A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n7A-n71A-n260A | CA_n7A-n260A CA_n71A-n260A | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | See n260 channel bandwidths in Table 5.3.5-1 |  |  |
| CA_n7A-n71A-n260G | CA_n7A-n260A/G CA_n71A-n260A/G | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n7A-n71A-n260H | CA_n7A-n260A/G/H CA_n71A-n260A/G/H | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n7A-n71A-n260I | CA_n7A-n260A/G/H/I CA_n71A-n260A/G/H/I | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n7A-n71A-n260J | CA_n7A-n260A/G/H/I/J CA_n71A-n260A/G/H/I/J | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n7A-n71A-n260K | CA_n7A-n260A/G/H/I/J/K CA_n71A-n260A/G/H/I/J/K | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n7A-n71A-n260L | CA_n7A-n260A/G/H/I/J/K/L CA_n71A-n260A/G/H/I/J/K/L | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n7A-n71A-n260M | CA_n7A-n260A/G/H/I/J/K/L/M CA_n71A-n260A/G/H/I/J/K/L/M | n7 | See n7 channel bandwidths in Table 5.3.5-1 |  | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n7A-n78A-n258A | CA_n7A-n78A CA_n7A-n258A CA_n78A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7A-n78A-n258B | CA_n7A-n78A CA_n7A-n258A/B CA_n78A-n258A/B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n7A-n78A-n258C | CA_n7A-n78A CA_n7A-n258A/B/C CA_n78A-n258A/B/C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n7A-n78A-n258D | CA_n7A-n78A CA_n7A-n258A/D CA_n78A-n258A/D | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n7A-n78A-n258E | CA_n7A-n78A CA_n7A-n258A/D/E CA_n78A-n258A/D/E | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n7A-n78A-n258F | CA_n7A-n78A CA_n7A-n258A/D/E/F CA_n78A-n258A/D/E/F | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n7A-n78A-n258G | CA_n7A-n78A CA_n7A-n258A/G CA_n78A-n258A/G | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n7A-n78A-n258H | CA_n7A-n78A CA_n7A-n258A/G/H CA_n78A-n258G/H | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n7A-n78A-n258I | CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n7A-n78A-n258J | CA_n7A-n78A CA_n7A-n258A/G/H/I/J CA_n78A-n258A/G/H/I/J | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n7A-n78A-n258K | CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K CA_n78A-n258A/G/H/I/J/K | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n7A-n78A-n258L | CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K/L CA_n78A-n258A/G/H/I/J/K/L | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n7A-n78A-n258M | CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K/L/M CA_n78A-n258A/G/H/I/J/K/L/M | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n7A-n78A-n258R2 | CA_n258R2 CA_n7A-n78A CA_n7A-n258A/R2 CA_n78A-n258A/R2 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n7A-n78A-n258R3 | CA_n258R2/R3 CA_n7A-n78A CA_n7A-n258A/R2/R3 CA_n78A-n258A/R2/R3 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n7A-n78A-n258R4 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n7A-n78A-n258R5 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n7A-n78A-n258R6 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n7A-n78A-n258R7 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n7A-n78A-n258R8 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n7A-n78A-n258R9 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n7A-n78A-n258R10 | CA_n258R2/R3/R4 CA_n7A-n78A CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n7A-n78(2A)-n258A | CA_n78(2A) CA_n7A-n78A CA_n7A-n258A CA_n78A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7A-n78(2A)-n258B | CA_n78(2A) CA_n258B CA_n7A-n78A CA_n7A-n258A/B CA_n78A-n258A/B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n7A-n78(2A)-n258C | CA_n78(2A) CA_n258B/C CA_n7A-n78A CA_n7A-n258A/B/C CA_n78A-n258A/B/C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n7A-n78(2A)-n258D | CA_n78(2A) CA_n258D CA_n7A-n78A CA_n7A-n258A/D CA_n78A-n258A/D | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n7A-n78(2A)-n258E | CA_n78(2A) CA_n258D/E CA_n7A-n78A CA_n7A-n258A/D/E CA_n78A-n258A/D/E | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n7A-n78(2A)-n258F | CA_n78(2A) CA_n258D/E/F CA_n7A-n78A CA_n7A-n258A/D/E/F CA_n78A-n258A/D/E/F | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n7A-n78(2A)-n258G | CA_n78(2A) CA_n258G CA_n7A-n78A CA_n7A-n258A/G CA_n78A-n258A/G | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n7A-n78(2A)-n258H | CA_n78(2A) CA_n258G/H CA_n7A-n78A CA_n7A-n258A/G/H CA_n78A-n258G/H | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n7A-n78(2A)-n258I | CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n7A-n78(2A)-n258J | CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n7A-n78(2A)-n258K | CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n7A-n78(2A)-n258L | CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n7A-n78(2A)-n258M | CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n7A-n78(2A)-n258R2 | CA_n78(2A) CA_n7A-n78A CA_n258R2 CA_n7A-n258A/R2 CA_n78A-n258A/R2 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n7A-n78(2A)-n258R3 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3 CA_n7A-n258A/R2/R3 CA_n78A-n258A/R2/R3 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n7A-n78(2A)-n258R4 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n7A-n78(2A)-n258R5 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n7A-n78(2A)-n258R6 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n7A-n78(2A)-n258R7 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n7A-n78(2A)-n258R8 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n7A-n78(2A)-n258R9 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n7A-n78(2A)-n258R10 | CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n7B-n78A-n258A | CA_n7B CA_n7A-n78A CA_n7A-n258A CA_n78A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7B-n78A-n258B | CA_n7B CA_n7A-n78A CA_n7A-n258A/B CA_n78A-n258A/B | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n7B-n78A-n258C | CA_n7B CA_n7B-n78A CA_n7B-n258A/B/C CA_n78A-n258A/B/C | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n7B-n78A-n258D | CA_n7B CA_n7A-n78A CA_n7A-n258A/D CA_n78A-n258A/D | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n7B-n78A-n258E | CA_n7B CA_n7A-n78A CA_n7A-n258A/D/E CA_n78A-n258A/D/E | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n7B-n78A-n258F | CA_n7B CA_n7A-n78A CA_n7A-n258A/D/E/F CA_n78A-n258A/D/E/F | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n7B-n78A-n258G | CA_n7B CA_n7A-n78A CA_n7A-n258A/G CA_n78A-n258A/G | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n7B-n78A-n258H | CA_n7B CA_n7A-n78A CA_n7A-n258A/G/H CA_n78A-n258G/H | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n7B-n78A-n258I | CA_n7B CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n7B-n78A-n258J | CA_n7B CA_n7A-n78A CA_n7A-n258A/G/H/I/J CA_n78A-n258A/G/H/I/J | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n7B-n78A-n258K | CA_n7B CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K CA_n78A-n258A/G/H/I/J/K | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n7B-n78A-n258L | CA_n7B CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K/L CA_n78A-n258A/G/H/I/J/K/L | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n7B-n78A-n258M | CA_n7B CA_n7A-n78A CA_n7A-n258A/G/H/I/J/K/L/M CA_n78A-n258A/G/H/I/J/K/L/M | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n7B-n78A-n258R2 | CA_n7B CA_n7A-n78A CA_n258R2 CA_n7A-n258A/R2 CA_n78A-n258A/R2 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n7B-n78A-n258R3 | CA_n7B CA_n7A-n78A CA_n258R2/R3 CA_n7A-n258A/R2/R3 CA_n78A-n258A/R2/R3 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n7B-n78A-n258R4 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n7B-n78A-n258R5 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n7B-n78A-n258R6 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n7B-n78A-n258R7 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n7B-n78A-n258R8 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n7B-n78A-n258R9 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n7B-n78A-n258R10 | CA_n7B CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n7B-n78(2A)-n258A | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n7A-n258A CA_n78A-n258A | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n7B-n78(2A)-n258B | CA_n7B CA_n78(2A) CA_n258B CA_n7A-n78A CA_n7A-n258A/B CA_n78A-n258A/B | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n7B-n78(2A)-n258C | CA_n7B CA_n78(2A) CA_n258B/C CA_n7A-n78A CA_n7A-n258A/B/C CA_n78A-n258A/B/C | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n7B-n78(2A)-n258D | CA_n7B CA_n78(2A) CA_n258D CA_n7A-n78A CA_n7A-n258A/D CA_n78A-n258A/D | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n7B-n78(2A)-n258E | CA_n7B CA_n78(2A) CA_n258D/E CA_n7A-n78A CA_n7A-n258A/D/E CA_n78A-n258A/D/E | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n7B-n78(2A)-n258F | CA_n7B CA_n78(2A) CA_n258D/E/F CA_n7A-n78A CA_n7A-n258A/D/E/F CA_n78A-n258A/D/E/F | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n7B-n78(2A)-n258G | CA_n7B CA_n78(2A) CA_n258G CA_n7A-n78A CA_n7A-n258A/G CA_n78A-n258A/G | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n7B-n78(2A)-n258H | CA_n7B CA_n78(2A) CA_n258G/H CA_n7A-n78A CA_n7A-n258A/G/H CA_n78A-n258G/H | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n7B-n78(2A)-n258I | CA_n7B CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n7B-n78(2A)-n258J | CA_n7B CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n7B-n78(2A)-n258K | CA_n7B CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n7B-n78(2A)-n258L | CA_n7B CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n7B-n78(2A)-n258M | CA_n7B CA_n78(2A) CA_n258G/H/I CA_n7A-n78A CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n7B-n78(2A)-n258R2 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2 CA_n7A-n258A/R2 CA_n78A-n258A/R2 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n7B-n78(2A)-n258R3 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3 CA_n7A-n258A/R2/R3 CA_n78A-n258A/R2/R3 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n7B-n78(2A)-n258R4 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n7B-n78(2A)-n258R5 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n7B-n78(2A)-n258R6 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n7B-n78(2A)-n258R7 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n7B-n78(2A)-n258R8 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n7B-n78(2A)-n258R9 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n7B-n78(2A)-n258R10 | CA_n7B CA_n78(2A) CA_n7A-n78A CA_n258R2/R3/R4 CA_n7A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 | n7 | CA_n7B |  | 0 |
|  |  | n78 | CA_n78(2A) |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n7A-n105A-n257A | CA_n7A-n105A CA_n7A-n257A CA_n105A-n257A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n7A-n105A-n258A | CA_n7A-n105A CA_n7A-n258A CA_n105A-n258A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n8A-n77A-n257A | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n8A-n77A-n257G | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n8A-n77A-n257H | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n8A-n77A-n257I | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n8A-n77A-n257J | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n8A-n77A-n257K | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n8A-n77A-n257L | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n8A-n77A-n257M | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n8A-n77(2A)-n257A | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n8A-n77(2A)-n257G | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n8A-n77(2A)-n257H | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n8A-n77(2A)-n257I | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n8A-n77(2A)-n257J | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n8A-n77(2A)-n257K | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n8A-n77(2A)-n257L | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n8A-n77(2A)-n257M | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n8A-n78A-n257A | CA_n8A-n78A CA_n8A-n257A CA_n78A-n257A | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n8A-n78A-n257D | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n8A-n78A-n257E | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257E |  |  |
| CA_n8A-n78A-n257F | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257F |  |  |
| CA_n8A-n78A-n257G | CA_n257G CA_n8A-n78A CA_n8A-n257A/G CA_n78A-n257A/G | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n8A-n78A-n257H | CA_n257G/H CA_n8A-n78A CA_n8A-n257A/G/H CA_n78A-n257A/G/H | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n8A-n78A-n257I | CA_n257G/H/I CA_n8A-n78A CA_n8A-n257A/G/H/I CA_n78A-n257A/G/H/I | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n8A-n78A-n257J | CA_n257G/H/I/J CA_n8A-n78A CA_n8A-n257A/G/H/I/J CA_n78A-n257A/G/H/I/J | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257J |  |  |
| CA_n8A-n78A-n257K | CA_n257G/H/I/J/K CA_n8A-n78A CA_n8A-n257A/G/H/I/J/K CA_n78A-n257A/G/H/I/J/K | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257K |  |  |
| CA_n8A-n78A-n257L | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257L |  |  |
| CA_n8A-n78A-n257M | - | n8 | 5, 10, 15, 20 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257M |  |  |
| CA_n12A-n30A-n260A | CA_n12A-n30A CA_n12A-n260A CA_n30A-n260A | n12 | 5, 10, 15 | 0 |  |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n12A-n30A-n260G | CA_n12A-n30A CA_n12A-n260A/G CA_n30A-n260A/G | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n12A-n30A-n260H | CA_n12A-n30A CA_n12A-n260A/G/H CA_n30A-n260A/G/H | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n12A-n30A-n260I | CA_n12A-n30A CA_n12A-n260A/G/H/I CA_n30A-n260A/G/H/I | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n12A-n30A-n260J | CA_n12A-n30A CA_n12A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n12A-n30A-n260K | CA_n12A-n30A CA_n12A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n12A-n30A-n260L | CA_n12A-n30A CA_n12A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n12A-n30A-n260M | CA_n12A-n30A CA_n12A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M | n12 | 5, 10, 15 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n12A-n66A-n260A | CA_n12A-n66A CA_n12A-n260A CA_n66A-n260A | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n12A-n66A-n260G | CA_n12A-n66A CA_n12A-n260A/G CA_n66A-n260A/G | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n12A-n66A-n260H | CA_n12A-n66A CA_n12A-n260A/G/H CA_n66A-n260A/G/H | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n12A-n66A-n260I | CA_n12A-n66A CA_n12A-n260A/G/H/I CA_n66A-n260A/G/H/I | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n12A-n66A-n260J | CA_n12A-n66A CA_n12A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n12A-n66A-n260K | CA_n12A-n66A CA_n12A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n12A-n66A-n260L | CA_n12A-n66A CA_n12A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n12A-n66A-n260M | CA_n12A-n66A CA_n12A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n12 | 5, 10, 15 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n12A-n77A-n260A | CA_n12A-n77A CA_n12A-n260A CA_n77A-n260A | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n12A-n77A-n260G | CA_n12A-n77A CA_n12A-n260A/G CA_n77A-n260A/G | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n12A-n77A-n260H | CA_n12A-n77A CA_n12A-n260A/G/H CA_n77A-n260A/G/H | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n12A-n77A-n260I | CA_n12A-n77A CA_n12A-n260A/G/H/I CA_n77A-n260A/G/H/I | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n12A-n77A-n260J | CA_n12A-n77A CA_n12A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n12A-n77A-n260K | CA_n12A-n77A CA_n12A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n12A-n77A-n260L | CA_n12A-n77A CA_n12A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n12A-n77A-n260M | CA_n12A-n77A CA_n12A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n12 | 5, 10, 15 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n14A-n30A-n260A | CA_n14A-n30A CA_n14A-n260A CA_n30A-n260A | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n14A-n30A-n260G | CA_n14A-n30A CA_n14A-n260A/G CA_n30A-n260A/G | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n14A-n30A-n260H | CA_n14A-n30A CA_n14A-n260A/G/H CA_n30A-n260A/G/H | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n14A-n30A-n260I | CA_n14A-n30A CA_n14A-n260A/G/H/I CA_n30A-n260A/G/H/I | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n14A-n30A-n260J | CA_n14A-n30A CA_n14A-n260A/G/H/I/J CA_n30A-n260A/G/H/I/J | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n14A-n30A-n260K | CA_n14A-n30A CA_n14A-n260A/G/H/I/J/K CA_n30A-n260A/G/H/I/J/K | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n14A-n30A-n260L | CA_n14A-n30A CA_n14A-n260A/G/H/I/J/K/L CA_n30A-n260A/G/H/I/J/K/L | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n14A-n30A-n260M | CA_n14A-n30A CA_n14A-n260A/G/H/I/J/K/L/M CA_n30A-n260A/G/H/I/J/K/L/M | n14 | 5, 10 |  | 0 |
|  |  | n30 | 5, 10 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n14A-n66A-n260A | CA_n14A-n66A CA_n14A-n260A CA_n66A-n260A | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n14A-n66A-n260G | CA_n14A-n66A CA_n14A-n260A/G CA_n66A-n260A/G | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n14A-n66A-n260H | CA_n14A-n66A CA_n14A-n260A/G/H CA_n66A-n260A/G/H | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n14A-n66A-n260I | CA_n14A-n66A CA_n14A-n260A/G/H/I CA_n66A-n260A/G/H/I | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n14A-n66A-n260J | CA_n14A-n66A CA_n14A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n14A-n66A-n260K | CA_n14A-n66A CA_n14A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n14A-n66A-n260L | CA_n14A-n66A CA_n14A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n14A-n66A-n260M | CA_n14A-n66A CA_n14A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n14 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n14A-n77A-n260A | CA_n14A-n77A CA_n14A-n260A CA_n77A-n260A | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n14A-n77A-n260G | CA_n14A-n77A CA_n14A-n260A/G CA_n77A-n260A/G | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n14A-n77A-n260H | CA_n14A-n77A CA_n14A-n260A/G/H CA_n77A-n260A/G/H | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n14A-n77A-n260I | CA_n14A-n77A CA_n14A-n260A/G/H/I CA_n77A-n260A/G/H/I | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n14A-n77A-n260J | CA_n14A-n77A CA_n14A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n14A-n77A-n260K | CA_n14A-n77A CA_n14A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n14A-n77A-n260L | CA_n14A-n77A CA_n14A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n14A-n77A-n260M | CA_n14A-n77A CA_n14A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n14 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n18A-n28A-n257A | CA_n18A-n28A CA_n18A-n257A CA_n28A-n257A | n18 | 5, 10, 15 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n18A-n28A-n257G | CA_n18A-n28A CA_n18A-n257A/G | n18 | 5, 10, 15 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n18A-n28A-n257H | CA_n18A-n28A CA_n18A-n257A/G/H CA_n28A-n257A/G/H | n18 | 5, 10, 15 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n18A-n28A-n257I | CA_n18A-n28A CA_n18A-n257A/G/H/I CA_n28A-n257A/G/H/I | n18 | 5, 10, 15 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n18A-n41A-n257A | CA_n18A-n41A CA_n18A-n257A CA_n41A-n257A | n18 | 5, 10, 15 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n18A-n41A-n257G | CA_n18A-n41A CA_n18A-n257A/G CA_n41A-n257A/G | n18 | 5, 10, 15 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n18A-n41A-n257H | CA_n18A-n41A CA_n18A-n257A/G/H CA_n41A-n257A/G/H | n18 | 5, 10, 15 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n18A-n41A-n257I | CA_n18A-n41A CA_n18A-n257A/G/H/I CA_n41A-n257A/G/H/I | n18 | 5, 10, 15 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n18A-n77A-n257A | CA_n18A-n77A CA_n18A-n257A CA_n77A-n257A | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n18A-n77A-n257G | CA_n18A-n77A CA_n18A-n257A/G CA_n77A-n257A/G | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n18A-n77A-n257H | CA_n18A-n77A CA_n18A-n257A/G/H CA_n77A-n257A/G/H | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n18A-n77A-n257I | CA_n18A-n77A CA_n18A-n257A/G/H/I CA_n77A-n257A/G/H/I | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n18A-n77(2A)-n257A | CA_n18A-n77A CA_n18A-n257A CA_n77A-n257A | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n18A-n77(2A)-n257G | CA_n18A-n77A CA_n18A-n257A/G CA_n77A-n257A/G | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n18A-n77(2A)-n257H | CA_n18A-n77A CA_n18A-n257A/G/H CA_n77A-n257A/G/H | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n18A-n77(2A)-n257I | CA_n18A-n77A CA_n18A-n257A/G/H/I CA_n77A-n257A/G/H/I | n18 | 5, 10, 15 | 0 |  |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n18A-n78A-n257A | CA_n18A-n78A CA_n18A-n257A CA_n78A-n257A | n18 | 5, 10, 15 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n18A-n78A-n257G | CA_n18A-n78A CA_n18A-n257A/G CA_n78A-n257A/G | n18 | 5, 10, 15 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n18A-n78A-n257H | CA_n18A-n78A CA_n18A-n257A/G/H CA_n78A-n257A/G/H | n18 | 5, 10, 15 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n18A-n78A-n257I | CA_n18A-n78A CA_n18A-n257A/G/H/I CA_n78A-n257A/G/H/I | n18 | 5, 10, 15 | 0 |  |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n25A-n41A-n260A | CA_n25A-n260A CA_n41A-n260A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n25A-n41A-n260G | CA_n25A-n260A CA_n41A-n260A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n25A-n41A-n260H | CA_n25A-n260A CA_n41A-n260A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n25A-n41A-n260I | CA_n25A-n260A CA_n41A-n260A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n25A-n41A-n260(2A) | CA_n25A-n260A CA_n41A-n260A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260(2A) |  |  |
| CA_n26A-n78A-n258A | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n26A-n78A-n258B | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n26A-n78A-n258C | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n26A-n78A-n258D | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n26A-n78A-n258E | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n26A-n78A-n258F | CA_n26A-n258A CA_n78A-n258A CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n26A-n78A-n258G | CA_n26A-n258A/G CA_n78A-n258A/G CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n26A-n78A-n258H | CA_n26A-n258A/G/H CA_n78A-n258A/G/H CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n26A-n78A-n258I | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n26A-n78A-n258J | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n26A-n78A-n258K | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n26A-n78A-n258L | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n26A-n78A-n258M | CA_n26A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n26A-n78A-n258R2 | CA_n26A-n78A CA_n26A-n258A/R2 CA_n78A-n258A/R2 CA_n258R2 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R2 |  |  |
| CA_n26A-n78A-n258R3 | CA_n26A-n78A CA_n26A-n258A/R2/R3 CA_n78A-n258A/R2/R3 CA_n258R2/R3 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R3 |  |  |
| CA_n26A-n78A-n258R4 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R4 |  |  |
| CA_n26A-n78A-n258R5 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R5 |  |  |
| CA_n26A-n78A-n258R6 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R6 |  |  |
| CA_n26A-n78A-n258R7 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R7 |  |  |
| CA_n26A-n78A-n258R8 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R8 |  |  |
| CA_n26A-n78A-n258R9 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R9 |  |  |
| CA_n26A-n78A-n258R10 | CA_n26A-n78A CA_n26A-n258A/R2/R3/R4 CA_n78A-n258A/R2/R3/R4 CA_n258R2/R3/R4 | n26 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258R10 |  |  |
| CA_n28A-n41A-n257A | CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A | n28 | 5, 10 |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n41A-n257G | CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G | n28 | 5, 10 |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n41A-n257H | CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H | n28 | 5, 10 |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n41A-n257I | CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I | n28 | 5, 10 |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n77A-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n77A-n257D | CA_n28A-n77A CA_n28A-n257A/D CA_n77A-n257A/D | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n28A-n77A-n257G | CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n77A-n257H | CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n77A-n257I | CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n77(2A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n77(2A)-n257D | CA_n28A-n77A CA_n28A-n257A/D CA_n77A-n257A/D | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n28A-n77(2A)-n257G | CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n77(2A)-n257H | CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n77(2A)-n257I | CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(2A) |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n77(3A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(3A) |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n77(3A)-n257D | CA_n28A-n77A CA_n28A-n257A/D CA_n77A-n257A/D | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(3A) |  |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n28A-n77(3A)-n257G | CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(3A) |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n77(3A)-n257H | CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(3A) |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n77(3A)-n257I | CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n77 | CA_n77(3A) |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n78A-n257A | CA_n28A-n78A CA_n28A-n257A CA_n78A-n257A | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n78A-n257D | CA_n28A-n78A CA_n28A-n257A/D CA_n78A-n257A/D | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257D |  |  |
| CA_n28A-n78A-n257G | CA_n28A-n78A CA_n28A-n257A/G CA_n78A-n257A/G | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n78A-n257H | CA_n28A-n78A CA_n28A-n257A/G/H CA_n78A-n257A/G/H | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n78A-n257I | CA_n28A-n78A CA_n28A-n257A/G/H/I CA_n78A-n257A/G/H/I | n28 | 5, 10, 15, 20 |  | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n28A-n78A-n258A | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n28A-n78A-n258B | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258B |  |  |
| CA_n28A-n78A-n258C | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258C |  |  |
| CA_n28A-n78A-n258D | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258D |  |  |
| CA_n28A-n78A-n258E | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258E |  |  |
| CA_n28A-n78A-n258F | CA_n28A-n258A CA_n78A-n258A CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 | 0 |  |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258F |  |  |
| CA_n28A-n78A-n258G | CA_n28A-n258A/G CA_n78A-n258A/G CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258G |  |  |
| CA_n28A-n78A-n258H | CA_n28A-n258A/G/H CA_n78A-n258A/G/H CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258H |  |  |
| CA_n28A-n78A-n258I | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258I |  |  |
| CA_n28A-n78A-n258J | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258J |  |  |
| CA_n28A-n78A-n258K | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258K |  |  |
| CA_n28A-n78A-n258L | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258L |  |  |
| CA_n28A-n78A-n258M | CA_n28A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n28A-n78A | n28 | 5, 10, 15, 20, 25, 30 |  | 0 |
|  |  | n78 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n258 | CA_n258M |  |  |
| CA_n28A-n79A-n257A | CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A | n28 | 5, 10, 15, 20, 30 |  | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n28A-n79A-n257G | CA_n257G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A/G | n28 | 5, 10, 15, 20, 30 |  | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n28A-n79A-n257H | CA_n257G/H CA_n28A-n79A CA_n28A-n257A/G/H CA_n79A-n257A/G/H | n28 | 5, 10, 15, 20, 30 |  | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n28A-n79A-n257I | CA_n257G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I | n28 | 5, 10, 15, 20, 30 |  | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n30A-n66A-n260A | CA_n30A-n66A CA_n30A-n260A CA_n66A-n260A | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n30A-n66A-n260G | CA_n30A-n66A CA_n30A-n260A/G CA_n66A-n260A/G | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n30A-n66A-n260H | CA_n30A-n66A CA_n30A-n260A/G/H CA_n66A-n260A/G/H | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n30A-n66A-n260I | CA_n30A-n66A CA_n30A-n260A/G/H/I CA_n66A-n260A/G/H/I | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n30A-n66A-n260J | CA_n30A-n66A CA_n30A-n260A/G/H/I/J CA_n66A-n260A/G/H/I/J | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n30A-n66A-n260K | CA_n30A-n66A CA_n30A-n260A/G/H/I/J/K CA_n66A-n260A/G/H/I/J/K | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n30A-n66A-n260L | CA_n30A-n66A CA_n30A-n260A/G/H/I/J/K/L CA_n66A-n260A/G/H/I/J/K/L | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n30A-n66A-n260M | CA_n30A-n66A CA_n30A-n260A/G/H/I/J/K/L/M CA_n66A-n260A/G/H/I/J/K/L/M | n30 | 5, 10 |  | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n30A-n77A-n260A | CA_n30A-n77A CA_n30A-n260A CA_n77A-n260A | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n30A-n77A-n260G | CA_n30A-n77A CA_n30A-n260A/G CA_n77A-n260A/G | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260G |  |  |
| CA_n30A-n77A-n260H | CA_n30A-n77A CA_n30A-n260A/G/H CA_n77A-n260A/G/H | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260H |  |  |
| CA_n30A-n77A-n260I | CA_n30A-n77A CA_n30A-n260A/G/H/I CA_n77A-n260A/G/H/I | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260I |  |  |
| CA_n30A-n77A-n260J | CA_n30A-n77A CA_n30A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260J |  |  |
| CA_n30A-n77A-n260K | CA_n30A-n77A CA_n30A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260K |  |  |
| CA_n30A-n77A-n260L | CA_n30A-n77A CA_n30A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260L |  |  |
| CA_n30A-n77A-n260M | CA_n30A-n77A CA_n30A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n30 | 5, 10 |  | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 | CA_n260M |  |  |
| CA_n39A-n40A-n258A | CA_n39A-n40A CA_n39A-n258A CA_n40A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n39A-n41A-n258A | CA_n39A-n41A CA_n39A-n258A CA_n41A-n258A | n39 | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n258 | 50, 100, 200, 400 |  |  |

##### Table 5.5A.1.2-1c

Table 5.5A.1.2-1c: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (three bands)

| NR CA configuration | Uplink configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A-n258A | CA_n40A-n41A CA_n40A-n258A CA_n41A-n258A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n40A-n77A-n257A | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n40A-n77A-n257D | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n40A-n77A-n257E | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257E |  |  |
| CA_n40A-n77A-n257F | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257F |  |  |
| CA_n40A-n77A-n257G | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n40A-n77A-n257H | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n40A-n77A-n257I | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n40A-n77A-n257J | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n40A-n77A-n257K | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n40A-n77A-n257L | CA_n40A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n40A-n77A-n257M | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n40A-n77C-n257A | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n40A-n77C-n257D | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n40A-n77C-n257E | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257E |  |  |
| CA_n40A-n77C-n257F | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257F |  |  |
| CA_n40A-n77C-n257G | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n40A-n77C-n257H | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n40A-n77C-n257I | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n40A-n77C-n257J | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n40A-n77C-n257K | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n40A-n77C-n257L | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n40A-n77C-n257M | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n40B-n77A-n257A | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n40B-n77A-n257D | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n40B-n77A-n257E | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257E |  |  |
| CA_n40B-n77A-n257F | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257F |  |  |
| CA_n40B-n77A-n257G | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n40B-n77A-n257H | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n40B-n77A-n257I | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n40B-n77A-n257J | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n40B-n77A-n257K | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n40B-n77A-n257L | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n40B-n77A-n257M | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n40B-n77C-n257A | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n40B-n77C-n257D | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257D |  |  |
| CA_n40B-n77C-n257E | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257E |  |  |
| CA_n40B-n77C-n257F | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257F |  |  |
| CA_n40B-n77C-n257G | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n40B-n77C-n257H | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n40B-n77C-n257I | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n40B-n77C-n257J | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257J |  |  |
| CA_n40B-n77C-n257K | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257K |  |  |
| CA_n40B-n77C-n257L | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257L |  |  |
| CA_n40B-n77C-n257M | CA_n40A-n77A CA_n77A-n257A CA_n40A-n257A |  | n40 | CA_n40B | 0 |  |
|  |  |  | n77 | CA_n77C |  |  |
|  |  |  | n257 | CA_n257M |  |  |
| CA_n40A-n78A-n257A | CA_n40A-n257A CA_n78A-n257A |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257A |  |  |
| CA_n40A-n78A-n257D | CA_n40A-n257A/D CA_n78A-n257A/D | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n40A-n78A-n257E | CA_n78A-n257A/D/E CA_n40A-n257A/D/E | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n40A-n78A-n257F | CA_n78A-n257A/D/E/F CA_n40A-n257A/D/E/F | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n40A-n78A-n257G | CA_n78A-n257A/G CA_n40A-n257A/G | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n40A-n78A-n257H | CA_n78A-n257A/G/H CA_n40A-n257A/G/H | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n40A-n78A-n257I | CA_n78A-n257A/G/H/I CA_n40A-n257A/G/H/I | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n40A-n78A-n257J | CA_n78A-n257A/G/H/I/J CA_n40A-n257A/G/H/I/J | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n40A-n78A-n257K | CA_n78A-n257A/G/H/I/J/K CA_n40A-n257A/G/H/I/J/K | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n40A-n78A-n257L | CA_n78A-n257A/G/H/I/J/K/L CA_n40A-n257A/G/H/I/J/K/L | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n40A-n78A-n257M | CA_n78A-n257A/G/H/I/J/K/L/M CA_n40A-n257A/G/H/I/J/K/L/M | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n40A-n78C-n257A | CA_n40A-n257A CA_n78A-n257A | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257A |  |  |
| CA_n40A-n78C-n257D | CA_n40A-n257A/D CA_n78A-n257A/D | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n40A-n78C-n257E | CA_n78A-n257A/D/E CA_n40A-n257A/D/E | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n40A-n78C-n257F | CA_n78A-n257A/D/E/F CA_n40A-n257A/D/E/F | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n40A-n78C-n257G | CA_n78A-n257A/G CA_n40A-n257A/G | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n40A-n78C-n257H | CA_n78A-n257A/G/H CA_n40A-n257A/G/H | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n40A-n78C-n257I | CA_n78A-n257A/G/H/I CA_n40A-n257A/G/H/I | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n40A-n78C-n257J | CA_n78A-n257A/G/H/I/J CA_n40A-n257A/G/H/I/J | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n40A-n78C-n257K | CA_n78A-n257A/G/H/I/J/K CA_n40A-n257A/G/H/I/J/K | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n40A-n78C-n257L | CA_n78A-n257A/G/H/I/J/K/L CA_n40A-n257A/G/H/I/J/K/L | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n40A-n78C-n257M | CA_n78A-n257A/G/H/I/J/K/L/M CA_n40A-n257A/G/H/I/J/K/L/M | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78C |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n40A-n78(2A)-n257A | CA_n40A-n257A CA_n78A-n257A | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n40A-n78(2A)-n257D | CA_n40A-n257A/D CA_n78A-n257A/D | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n40A-n78(2A)-n257E | CA_n78A-n257A/D/E CA_n40A-n257A/D/E | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n40A-n78(2A)-n257F | CA_n78A-n257A/D/E/F CA_n40A-n257A/D/E/F | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n40A-n78(2A)-n257G | CA_n78A-n257A/G CA_n40A-n257A/G | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n40A-n78(2A)-n257H | CA_n78A-n257A/G/H CA_n40A-n257A/G/H | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n40A-n78(2A)-n257I | CA_n78A-n257A/G/H/I CA_n40A-n257A/G/H/I | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n40A-n78(2A)-n257J | CA_n78A-n257A/G/H/I/J CA_n40A-n257A/G/H/I/J | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n40A-n78(2A)-n257K | CA_n78A-n257A/G/H/I/J/K CA_n40A-n257A/G/H/I/J/K | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n40A-n78(2A)-n257L | CA_n78A-n257A/G/H/I/J/K/L CA_n40A-n257A/G/H/I/J/K/L | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n40A-n78(2A)-n257M | CA_n78A-n257A/G/H/I/J/K/L/M CA_n40A-n257A/G/H/I/J/K/L/M | n40 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n40B-n78A-n257A | CA_n40B CA_n40B-n257A CA_n78A-n257A | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n40B-n78A-n257D | CA_n40B CA_n40B-n257A/D CA_n78A-n257A/D | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n40B-n78A-n257E | CA_n40B CA_n78A-n257A/D/E CA_n40B-n257A/D/E | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n40B-n78A-n257F | CA_n40B CA_n78A-n257A/D/E/F CA_n40B-n257A/D/E/F | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n40B-n78A-n257G | CA_n78A-n257A/G CA_n40B-n257A/G | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n40B-n78A-n257H | CA_n40B CA_n78A-n257A/G/H CA_n40B-n257A/G/H | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n40B-n78A-n257I | CA_n40B CA_n78A-n257A/G/H/I CA_n40B-n257A/G/H/I | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n40B-n78A-n257J | CA_n40B CA_n78A-n257A/G/H/I/J CA_n40B-n257A/G/H/I/J | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n40B-n78A-n257K | CA_n40B CA_n78A-n257A/G/H/I/J/K CA_n40B-n257A/G/H/I/J/K | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n40B-n78A-n257L | CA_n40B CA_n78A-n257A/D/E/F/G/H/I/J/K/L CA_n40B-n257A/D/E/F/G/H/I/J/K/L | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n40B-n78A-n257M | CA_n40B CA_n78A-n257A/D/E/F/G/H/I/J/K/L/M CA_n40B-n257A/D/E/F/G/H/I/J/K/L/M | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n40B-n78(2A)-n257A | CA_n40B CA_n78A CA_n40B-n257A CA_n78A-n257A | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n40B-n78(2A)-n257D | CA_n40B CA_n78A CA_n40B-n257A/D CA_n78A-n257A/D | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n40B-n78(2A)-n257E | CA_n40B CA_n78A CA_n78A-n257A/D/E CA_n40B-n257A/D/E | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n40B-n78(2A)-n257F | CA_n40B CA_n78A CA_n78A-n257A/D/E/F CA_n40B-n257A/D/E/F | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n40B-n78(2A)-n257G | CA_n40B CA_n78A CA_n78A-n257A/G CA_n40B-n257A/G | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n40B-n78(2A)-n257H | CA_n40B CA_n78A CA_n78A-n257A/G/H CA_n40B-n257A/G/H | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n40B-n78(2A)-n257I | CA_n40B CA_n78A CA_n78A-n257A/G/H/I CA_n40B-n257A/G/H/I | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n40B-n78(2A)-n257J | CA_n40B CA_n78A CA_n78A-n257A/G/H/I/J CA_n40B-n257A/G/H/I/J | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n40B-n78(2A)-n257K | CA_n40B CA_n78A CA_n78A-n257A/G/H/I/J/K CA_n40B-n257A/G/H/I/J/K | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n40B-n78(2A)-n257L | CA_n40B CA_n78A CA_n78A-n257A/G/H/I/J/K/L CA_n40B-n257A/G/H/I/J/K/L | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n40B-n78(2A)-n257M | CA_n40B CA_n78A CA_n78A-n257A/G/H/I/J/K/L/M CA_n40B-n257A/G/H/I/J/K/L/M | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78(2A)_BCS2 |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n40B-n78C-n257A | CA_n40B CA_n78C CA_n40B-n257A CA_n78C-n257A | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n40B-n78C-n257D | CA_n40B CA_n78C CA_n40B-n257A/D CA_n78C-n257A/D | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257D |  |  |
| CA_n40B-n78C-n257E | CA_n40B CA_n78C CA_n78C-n257A/D/E CA_n40B-n257A/D/E | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257E |  |  |
| CA_n40B-n78C-n257F | CA_n40B CA_n78C CA_n78C-n257A/D/E/F CA_n40B-n257A/D/E/F | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257F |  |  |
| CA_n40B-n78C-n257G | CA_n40B CA_n78C CA_n78C-n257A/G CA_n40B-n257A/G | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n40B-n78C-n257H | CA_n40B CA_n78C CA_n78C-n257A/G/H CA_n40B-n257A/G/H | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n40B-n78C-n257I | CA_n40B CA_n78C CA_n78C-n257A/G/H/I CA_n40B-n257A/G/H/I | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n40B-n78C-n257J | CA_n40B CA_n78C CA_n78C-n257A/G/H/I/J CA_n40B-n257A/G/H/I/J | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257J |  |  |
| CA_n40B-n78C-n257K | CA_n40B CA_n78C CA_n78C-n257A/G/H/I/J/K CA_n40B-n257A/G/H/I/J/K | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257K |  |  |
| CA_n40B-n78C-n257L | CA_n40B CA_n78C CA_n78C-n257A/G/H/I/J/K/L CA_n40B-n257A/G/H/I/J/K/L | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257L |  |  |
| CA_n40B-n78C-n257M | CA_n40B CA_n78C CA_n78C-n257A/G/H/I/J/K/L/M CA_n40B-n257A/G/H/I/J/K/L/M | n40 |  | CA_n40B_BCS1 |  | 0 |
|  |  | n78 |  | CA_n78C_BCS1 |  |  |
|  |  | n257 |  | CA_n257M |  |  |
| CA_n40A-n78A-n258A | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 100 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n40A-n78A-n258D | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258D |  |  |
| CA_n40A-n78A-n258E | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258E |  |  |
| CA_n40A-n78A-n258F | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258F |  |  |
| CA_n40A-n78A-n258G | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258G |  |  |
| CA_n40A-n78A-n258H | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258H |  |  |
| CA_n40A-n78A-n258I | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258I |  |  |
| CA_n40A-n78A-n258J | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258J |  |  |
| CA_n40A-n78A-n258K | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258K |  |  |
| CA_n40A-n78A-n258L | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258L |  |  |
| CA_n40A-n78A-n258M | - |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 90, 100 |  |  |
|  |  |  | n258 | CA_n258M |  |  |
| CA_n40A-n79A-n258A | CA_n40A-n79A CA_n79A-n258A CA_n40A-n258A |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |  |
| CA_n41A-n66A-n260A | CA_n41A-n260A CA_n66A-n260A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |
| CA_n41A-n66A-n260(2A) | CA_n41A-n260A CA_n66A-n260A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  |  | n260 | CA_n260(2A) |  |  |
| CA_n41A-n66A-n260G | CA_n41A-n260A/G CA_n66A-n260A/G | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n41A-n66A-n260H | CA_n41A-n260A/G/H CA_n66A-n260A/G/H | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n41A-n66A-n260I | CA_n41A-n260A/G/H/I CA_n66A-n260A/G/H/I | n41 |  | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n41A-n77A-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n41A-n77A-n257G | CA_n257G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n41A-n77A-n257H | CA_n257G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n41A-n77A-n257I | CA_n257G/H/I CA_n41A-n77A/G/H/I CA_n77A-n257A/G/H/I | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n41A-n77(2A)-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n41A-n77(2A)-n257G | CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n41A-n77(2A)-n257H | CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n41A-n77(2A)-n257I | CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n41A-n77(3A)-n257A | CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(3A) |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n41A-n77(3A)-n257G | CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(3A) |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n41A-n77(3A)-n257H | CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(3A) |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n41A-n77(3A)-n257I | CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77(3A) |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n41A-n78A-n257A | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n41A-n78A-n257G | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257G |  |  |
| CA_n41A-n78A-n257H | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257H |  |  |
| CA_n41A-n78A-n257I | - |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  |  | n257 | CA_n257I |  |  |
| CA_n41A-n79A-n257A | CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n41A-n79A-n257G | CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n41A-n79A-n257H | CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n41A-n79A-n257I | CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n41A-n79A-n258A | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n41A-n79A-n258B | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258B |  |  |
| CA_n41A-n79A-n258C | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258C |  |  |
| CA_n41A-n79A-n258D | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258D |  |  |
| CA_n41A-n79A-n258E | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258E |  |  |
| CA_n41A-n79A-n258F | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258F |  |  |
| CA_n41A-n79A-n258G | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n41A-n79A-n258H | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n41A-n79A-n258I | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n41A-n79A-n258J | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n41A-n79A-n258K | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258K |  |  |
| CA_n41A-n79A-n258L | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258L |  |  |
| CA_n41A-n79A-n258M | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258M |  |  |
| CA_n41C-n79A-n258A | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n41C-n79A-n258B | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258B |  |  |
| CA_n41C-n79A-n258C | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258C |  |  |
| CA_n41C-n79A-n258D | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258D |  |  |
| CA_n41C-n79A-n258E | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258E |  |  |
| CA_n41C-n79A-n258F | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258F |  |  |
| CA_n41C-n79A-n258G | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n41C-n79A-n258H | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n41C-n79A-n258I | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n41C-n79A-n258J | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n41C-n79A-n258K | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258K |  |  |
| CA_n41C-n79A-n258L | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258L |  |  |
| CA_n41C-n79A-n258M | CA_n41A-n79A CA_n41A-n258A CA_n79A-n258A | n41 |  | CA_n41C |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258M |  |  |
| CA_n48A-n66A-n260A | CA_n48A-n260A CA_n66A-n260A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n66A-n260G | CA_n48A-n260A/G CA_n66A-n260A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n48A-n66A-n260H | CA_n48A-n260A/G/H CA_n66A-n260A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n48A-n66A-n260I | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n48A-n66A-n260J | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n48A-n66A-n260K | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n48A-n66A-n260L | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n48A-n66A-n260M | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n48B-n66A-n260A | CA_n48A-n260A CA_n66A-n260A | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48B-n66A-n260G | CA_n48A-n260A/G CA_n66A-n260A/G | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n48B-n66A-n260H | CA_n48A-n260A/G/H CA_n66A-n260A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n48B-n66A-n260I | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n48B-n66A-n260J | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n48B-n66A-n260K | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n48B-n66A-n260L | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n48B-n66A-n260M | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n48(2A)-n66A-n260A | CA_n48A-n260A CA_n66A-n260A | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48(2A)-n66A-n260G | CA_n48A-n260A/G CA_n66A-n260A/G | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n48(2A)-n66A-n260H | CA_n48A-n260A/G/H CA_n66A-n260A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n48(2A)-n66A-n260I | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n48(2A)-n66A-n260J | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n48(2A)-n66A-n260K | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n48(2A)-n66A-n260L | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n48(2A)-n66A-n260M | CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n48A-n66A-n261A | CA_n48A-n261A CA_n66A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n66A-n261G | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n48A-n66A-n261H | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n48A-n66A-n261I | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n48A-n66A-n261J | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n48A-n66A-n261K | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n48A-n66A-n261L | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n48A-n66A-n261M | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n48A-n66A-n261(2A) | CA_n48A-n261A CA_n66A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n48A-n66A-n261(3A) | CA_n48A-n261A CA_n66A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n48A-n66A-n261(A-G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n48A-n66A-n261(A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n48A-n66A-n261(A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n48A-n66A-n261(G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n48A-n66A-n261(2A-G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n48A-n66A-n261(2A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n48A-n66A-n261(A-2G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n48A-n66A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n48A-n66A-n261(2G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n48A-n66A-n261(2H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n48A-n66A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n48A-n66A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n48A-n66A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n48A-n66A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n48B-n66A-n261A | CA_n48A-n261A CA_n66A-n261A | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n48B-n66A-n261G | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n48B-n66A-n261H | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n48B-n66A-n261I | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n48B-n66A-n261J | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n48B-n66A-n261K | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n48B-n66A-n261L | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n48B-n66A-n261M | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n48B-n66A-n261(A-G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n48B-n66A-n261(A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n48B-n66A-n261(A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n48B-n66A-n261(G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n48B-n66A-n261(2A-G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n48B-n66A-n261(2A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n48B-n66A-n261(A-2G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n48B-n66A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n48B-n66A-n261(2A) | CA_n48A-n261A CA_n66A-n261A | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n48B-n66A-n261(3A) | CA_n48A-n261A CA_n66A-n261A | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n48B-n66A-n261(2G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n48B-n66A-n261(2H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n48B-n66A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n48B-n66A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n48B-n66A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n48B-n66A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48B_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n48(2A)-n66A-n261A | CA_n48A-n261A CA_n66A-n261A | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n48(2A)-n66A-n261G | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n48(2A)-n66A-n261H | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n48(2A)-n66A-n261I | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n48(2A)-n66A-n261J | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n48(2A)-n66A-n261K | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n48(2A)-n66A-n261L | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n48(2A)-n66A-n261M | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n48(2A)-n66A-n261(A-G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n48(2A)-n66A-n261(A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n48(2A)-n66A-n261(A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n48(2A)-n66A-n261(G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n48(2A)-n66A-n261(2A-G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n48(2A)-n66A-n261(2A-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n48(2A)-n66A-n261(A-2G) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n48(2A)-n66A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n48(2A)-n66A-n261(2A) | CA_n48A-n261A CA_n66A-n261A | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n48(2A)-n66A-n261(3A) | CA_n48A-n261A CA_n66A-n261A | n48 |  | CA_n48(2A) |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n48(2A)-n66A-n261(2G) | CA_n48A-n261A/G CA_n66A-n261A/G | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n48(2A)-n66A-n261(2H) | CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n48(2A)-n66A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n48(2A)-n66A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n48(2A)-n66A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n48(2A)-n66A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n48 |  | CA_n48(2A)_BCS1 |  | 0 |
|  |  | n66 |  | 5, 10, 15, 20, 40 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n48A-n77A-n260A | CA_n48A-n260A CA_n77A-n260A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n77A-n260G | CA_n48A-n260A/G CA_n77A-n260A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n48A-n77A-n260H | CA_n48A-n260A/G/H CA_n77A-n260A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n48A-n77A-n260I | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n48A-n77A-n260J | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n48A-n77A-n260K | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n48A-n77A-n260L | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n48A-n77A-n260M | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n48A-n77C-n260A | CA_n48A-n260A CA_n77A-n260A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n77C-n260G | CA_n48A-n260A/G CA_n77A-n260A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n48A-n77C-n260H | CA_n48A-n260A/G/H CA_n77A-n260A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n48A-n77C-n260I | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n48A-n77C-n260J | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n48A-n77C-n260K | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n48A-n77C-n260L | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n48A-n77C-n260M | CA_n48A-n260A/G/H/I CA_n77A-n260A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n48A-n77A-n261A | CA_n48A-n261A CA_n77A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n77A-n261G | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n48A-n77A-n261H | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n48A-n77A-n261I | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n48A-n77A-n261J | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n48A-n77A-n261K | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n48A-n77A-n261L | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n48A-n77A-n261M | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n48A-n77A-n261(A-G) | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n48A-n77A-n261(A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n48A-n77A-n261(A-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n48A-n77A-n261(G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n48A-n77A-n261(2A) | CA_n48A-n261A CA_n77A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n48A-n77A-n261(3A) | CA_n48A-n261A CA_n77A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n48A-n77A-n261(2G) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n48A-n77A-n261(2H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n48A-n77A-n261(2A-G) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n48A-n77A-n261(2A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n48A-n77A-n261(A-2G) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n48A-n77A-n261(A-G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n48A-n77A-n261(G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n48A-n77A-n261(H-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n48A-n77A-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n48A-n77A-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n48A-n77C-n261A | CA_n48A-n261A CA_n77A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n77C-n261G | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n48A-n77C-n261H | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n48A-n77C-n261I | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n48A-n77C-n261J | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n48A-n77C-n261K | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n48A-n77C-n261L | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n48A-n77C-n261M | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n48A-n77C-n261(A-G) | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n48A-n77C-n261(A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n48A-n77C-n261(A-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n48A-n77C-n261(G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n48A-n77C-n261(2A) | CA_n48A-n261A CA_n77A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n48A-n77C-n261(3A) | CA_n48A-n261A CA_n77A-n261A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n48A-n77C-n261(2G) | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n48A-n77C-n261(2H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n48A-n77C-n261(2A-G) | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n48A-n77C-n261(2A-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n48A-n77C-n261(A-2G) | CA_n48A-n261A/G CA_n77A-n261A/G | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n48A-n77C-n261(A-G-H) | CA_n48A-n261A/G/H CA_n77A-n261A/G/H | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n48A-n77C-n261(G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n48A-n77C-n261(H-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n48A-n77C-n261(2A-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n48A-n77C-n261(A-G-I) | CA_n48A-n261A/G/H/I CA_n77A-n261A/G/H/I | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n66A-n77A-n260A | CA_n66A-n77A CA_n77A-n260A CA_n66A-n260A | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n66A-n77A-n260G | CA_n66A-n77A CA_n66A-n260A/G CA_n77A-n260A/G | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n66A-n77A-n260H | CA_n66A-n77A CA_n66A-n260A/G/H CA_n77A-n260A/G/H | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n66A-n77A-n260I | CA_n66A-n77A CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n66A-n77A-n260J | CA_n66A-n77A CA_n66A-n260A/G/H/I/J CA_n77A-n260A/G/H/I/J | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n66A-n77A-n260K | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K CA_n77A-n260A/G/H/I/J/K | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n66A-n77A-n260L | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L CA_n77A-n260A/G/H/I/J/K/L | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n66A-n77A-n260M | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L/M CA_n77A-n260A/G/H/I/J/K/L/M | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n66A-n77(2A)-n260A | CA_n66A-n77A CA_n66A-n260A CA_n77(2A) CA_n77A-n260A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n66A-n77(2A)-n260G | CA_n66A-n77A CA_n66A-n260A/G CA_n77(2A) CA_n77A-n260A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260G |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n66A-n77(2A)-n260H | CA_n66A-n77A CA_n66A-n260A/G/H CA_n77(2A) CA_n77A-n260A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260H |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n66A-n77(2A)-n260I | CA_n66A-n77A CA_n66A-n260A/G/H/I CA_n77(2A) CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260I |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n66A-n77(2A)-n260J | CA_n66A-n77A CA_n66A-n260A/G/H/I/J CA_n77(2A) CA_n77A-n260A/G/H/I/J | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260J |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n66A-n77(2A)-n260K | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K CA_n77(2A) CA_n77A-n260A/G/H/I/J/K | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260K |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n66A-n77(2A)-n260L | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260L |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n66A-n77(2A)-n260M | CA_n66A-n77A CA_n66A-n260A/G/H/I/J/K/L/M CA_n77(2A) CA_n77A-n260A/G/H/I/J/K/L/M | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77(2A) |  |  |
|  |  | n260 |  | CA_n260M |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | CA_n77(2A)_BCS1 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n66A-n77C-n260A | CA_n66A-n260A CA_n77A-n260A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260A |  |  |
| CA_n66A-n77C-n260G | CA_n66A-n260A/G CA_n77A-n260A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260G |  |  |
| CA_n66A-n77C-n260H | CA_n66A-n260A/G/H CA_n77A-n260A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260H |  |  |
| CA_n66A-n77C-n260I | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260I |  |  |
| CA_n66A-n77C-n260J | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260J |  |  |
| CA_n66A-n77C-n260K | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260K |  |  |
| CA_n66A-n77C-n260L | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260L |  |  |
| CA_n66A-n77C-n260M | CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n260 |  | CA_n260M |  |  |
| CA_n66A-n77A-n261A | CA_n77A-n261A CA_n66A-n261A | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | 50, 100, 200, 400 |  |  |
| CA_n66A-n77A-n261G | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n66A-n77A-n261H | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n66A-n77A-n261I | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n66A-n77A-n261J | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n66A-n77A-n261K | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n66A-n77A-n261L | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n66A-n77A-n261M | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
|  |  | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 1 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n66A-n77A-n261(2A) | CA_n66A-n261A CA_n77A-n261A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n66A-n77A-n261(3A) | CA_n66A-n261A CA_n77A-n261A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n66A-n77A-n261(A-G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n66A-n77A-n261(A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n66A-n77A-n261(G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n66A-n77A-n261(2A-G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n66A-n77A-n261(2A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n66A-n77A-n261(A-2G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n66A-n77A-n261(A-G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n66A-n77A-n261(A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n66A-n77A-n261(G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n66A-n77A-n261(2G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n66A-n77A-n261(2H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n66A-n77A-n261(2A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n66A-n77A-n261(A-G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n66A-n77A-n261(H-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n66A-n77C-n261A | CA_n66A-n261A CA_n77A-n261A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261A |  |  |
| CA_n66A-n77C-n261G | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261G |  |  |
| CA_n66A-n77C-n261H | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261H |  |  |
| CA_n66A-n77C-n261I | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261I |  |  |
| CA_n66A-n77C-n261J | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261J |  |  |
| CA_n66A-n77C-n261K | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261K |  |  |
| CA_n66A-n77C-n261L | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261L |  |  |
| CA_n66A-n77C-n261M | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261M |  |  |
| CA_n66A-n77C-n261(A-G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-G) |  |  |
| CA_n66A-n77C-n261(A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-H) |  |  |
| CA_n66A-n77C-n261(G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(G-H) |  |  |
| CA_n66A-n77C-n261(2A-G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A-G) |  |  |
| CA_n66A-n77C-n261(2A-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A-H) |  |  |
| CA_n66A-n77C-n261(A-2G) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-2G) |  |  |
| CA_n66A-n77C-n261(A-G-H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-G-H) |  |  |
| CA_n66A-n77C-n261(A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-I) |  |  |
| CA_n66A-n77C-n261(G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(G-I) |  |  |
| CA_n66A-n77C-n261(2A) | CA_n66A-n261A CA_n77A-n261A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A) |  |  |
| CA_n66A-n77C-n261(3A) | CA_n66A-n261A CA_n77A-n261A | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(3A) |  |  |
| CA_n66A-n77C-n261(2G) | CA_n66A-n261A/G CA_n77A-n261A/G | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2G) |  |  |
| CA_n66A-n77C-n261(2H) | CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2H) |  |  |
| CA_n66A-n77C-n261(H-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(H-I) |  |  |
| CA_n66A-n77C-n261(2A-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(2A-I) |  |  |
| CA_n66A-n77C-n261(A-G-I) | CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n66 |  | 5, 10, 15, 20, 25, 30, 40 |  | 0 |
|  |  | n77 |  | CA_n77C_BCS1 |  |  |
|  |  | n261 |  | CA_n261(A-G-I) |  |  |
| CA_n77A-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n79A-n257G | CA_n257G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n77A-n79A-n257H | CA_n257G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n77A-n79A-n257I | CA_n257G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n77(2A)-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n77(2A)-n79A-n257G | CA_n257G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n77(2A)-n79A-n257H | CA_n257G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n77(2A)-n79A-n257I | CA_n257G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n77(3A)-n79A-n257A | CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n77 |  | CA_n77(3A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n77(3A)-n79A-n257G | CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n77 |  | CA_n77(3A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n77(3A)-n79A-n257H | CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n77 |  | CA_n77(3A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n77(3A)-n79A-n257I | CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n77 |  | CA_n77(3A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n77A-n79A-n258A | CA_n77A-n79A CA_n77A-n258A CA_n79A-n258A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n79A-n258D | CA_n77A-n79A CA_n77A-n258A/D CA_n79A-n258A/D | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258D |  |  |
| CA_n77A-n79A-n258G | CA_n77A-n79A CA_n77A-n258A/G CA_n79A-n258A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n77A-n79A-n258H | CA_n77A-n79A CA_n77A-n258A/G/H CA_n79A-n258A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n77A-n79A-n258I | CA_n77A-n79A CA_n77A-n258A/G/H/I CA_n79A-n258A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n77A-n79A-n258J | CA_n77A-n79A CA_n77A-n258A/G/H/I/J CA_n79A-n258A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n77(2A)-n79A-n258A | CA_n77A-n79A CA_n77A-n258A CA_n79A-n258A | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n77(2A)-n79A-n258D | CA_n77A-n79A CA_n77A-n258A/D CA_n79A-n258A/D | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258D |  |  |
| CA_n77(2A)-n79A-n258G | CA_n77A-n79A CA_n77A-n258A/G CA_n79A-n258A/G | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258G |  |  |
| CA_n77(2A)-n79A-n258H | CA_n77A-n79A CA_n77A-n258A/G/H CA_n79A-n258A/G/H | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258H |  |  |
| CA_n77(2A)-n79A-n258I | CA_n77A-n79A CA_n77A-n258A/G/H/I CA_n79A-n258A/G/H/I | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258I |  |  |
| CA_n77(2A)-n79A-n258J | CA_n77A-n79A CA_n77A-n258A/G/H/I/J CA_n79A-n258A/G/H/I/J | n77 |  | CA_n77(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n258 |  | CA_n258J |  |  |
| CA_n77A-n79A-n259A | CA_n77A-n79A CA_n77A-n259A CA_n79A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n79A-n259G | CA_n259G CA_n77A-n79A CA_n77A-n259A/G CA_n79A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n77A-n79A-n259H | CA_n259G/H CA_n77A-n79A CA_n77A-n259A/G/H CA_n79A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n77A-n79A-n259I | CA_n259G/H/I CA_n77A-n79A CA_n77A-n259A/G/H/I CA_n79A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n77A-n79A-n259J | CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n259A/G/H/I/J CA_n79A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n77A-n79A-n259K | CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n259A/G/H/I/J/K CA_n79A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n77A-n79A-n259L | CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n77A-n79A-n259M | CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n77A-n257A-n259A | CA_n77A-n257A CA_n77A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n257A-n259G | CA_n259G CA_n77A-n257A CA_n77A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n77A-n257A-n259H | CA_n259G/H CA_n77A-n257A CA_n77A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n77A-n257A-n259I | CA_n259G/H/I CA_n77A-n257A CA_n77A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n77A-n257A-n259J | CA_n259G/H/I/J CA_n77A-n257A CA_n77A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n77A-n257A-n259K | CA_n259G/H/I/J/K CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n77A-n257A-n259L | CA_n259G/H/I/J/K/L CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n77A-n257A-n259M | CA_n259G/H/I/J/K/L/M CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n77A-n257G-n259A | CA_n257G CA_n77A-n257A/G CA_n77A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n257G-n259G | CA_n257G CA_n259G CA_n77A-n257A/G CA_n77A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n77A-n257G-n259H | CA_n257G CA_n259G/H CA_n77A-n257A/G CA_n77A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n77A-n257G-n259I | CA_n257G CA_n259G/H/I CA_n77A-n257A/G CA_n77A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n77A-n257G-n259J | CA_n257G CA_n259G/H/I/J CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n77A-n257G-n259K | CA_n257G CA_n259G/H/I/J/K CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n77A-n257G-n259L | CA_n257G CA_n259G/H/I/J/K/L CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n77A-n257G-n259M | CA_n257G CA_n259G/H/I/J/K/L/M CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n77A-n257H-n259A | CA_n257G/H CA_n77A-n257A/G/H CA_n77A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n257H-n259G | CA_n257G/H CA_n259G CA_n77A-n257A/G/H CA_n77A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n77A-n257H-n259H | CA_n257G/H CA_n259G/H CA_n77A-n257A/G/H CA_n77A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n77A-n257H-n259I | CA_n257G/H CA_n259G/H/I CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n77A-n257H-n259J | CA_n257G/H CA_n259G/H/I/J CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n77A-n257H-n259K | CA_n257G/H CA_n259G/H/I/J/K CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n77A-n257H-n259L | CA_n257G/H CA_n259G/H/I/J/K/L CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n77A-n257H-n259M | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n77A-n257I-n259A | CA_n257G/H/I CA_n77A-n257A/G/H/I CA_n77A-n259A | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n77A-n257I-n259G | CA_n257G/H/I CA_n259G CA_n77A-n257A/G/H/I CA_n77A-n259A/G | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n77A-n257I-n259H | CA_n257G/H/I CA_n259G/H CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n77A-n257I-n259I | CA_n257G/H/I CA_n259G/H/I CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n77A-n257I-n259J | CA_n257G/H/I CA_n259G/H/I/J CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n77A-n257I-n259K | CA_n257G/H/I CA_n259G/H/I/J/K CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n77A-n257I-n259L | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n77A-n257I-n259M | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L/M | n77 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n78A-n79A-n257A | CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n79A-n257G | CA_n257G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n78A-n79A-n257H | CA_n257G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n78A-n79A-n257I | CA_n257G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n78(2A)-n79A-n257A | CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A | n78 |  | CA_n78(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78(2A)-n79A-n257G | CA_n257G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G | n78 |  | CA_n78(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257G |  |  |
| CA_n78(2A)-n79A-n257H | CA_n257G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H | n78 |  | CA_n78(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257H |  |  |
| CA_n78(2A)-n79A-n257I | CA_n257G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I | n78 |  | CA_n78(2A) |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 |  | CA_n257I |  |  |
| CA_n78A-n79A-n259A | CA_n78A-n79A CA_n78A-n259A CA_n79A-n259A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n79A-n259G | CA_n259G CA_n78A-n79A CA_n78A-n259A/G CA_n79A-n259A/G | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n78A-n79A-n259H | CA_n259G/H CA_n78A-n79A CA_n78A-n259A/G/H CA_n79A-n259A/G/H | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n78A-n79A-n259I | CA_n259G/H/I CA_n78A-n79A CA_n78A-n259A/G/H/I CA_n79A-n259A/G/H/I | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n78A-n79A-n259J | CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n259A/G/H/I/J CA_n79A-n259A/G/H/I/J | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n78A-n79A-n259K | CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n259A/G/H/I/J/K CA_n79A-n259A/G/H/I/J/K | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n78A-n79A-n259L | CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n259A/G/H/I/J/K/L | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n78A-n79A-n259M | CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n259A/G/H/I/J/K/L/M | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n79 |  | 40, 50, 60, 80, 100 |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n78A-n105A-n257A | CA_n78A-n105A CA_n78A-n257A CA_n105A-n257A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n105A-n258A | CA_n78A-n105A CA_n78A-n258A CA_n105A-n258A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n105 |  | 5, 10, 15, 20, 25, 30, 35 |  |  |
|  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257A-n259A | CA_n78A-n257A CA_n78A-n259A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257A-n259G | CA_n259G CA_n78A-n257A CA_n78A-n259A/G | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n78A-n257A-n259H | CA_n259G/H CA_n78A-n257A CA_n78A-n259A/G/H | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n78A-n257A-n259I | CA_n259G/H/I CA_n78A-n257A CA_n78A-n259A/G/H/I | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n78A-n257A-n259J | CA_n259G/H/I/J CA_n78A-n257A CA_n78A-n259A/G/H/I/J | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n78A-n257A-n259K | CA_n259G/H/I/J/K CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n78A-n257A-n259L | CA_n259G/H/I/J/K/L CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n78A-n257A-n259M | CA_n259G/H/I/J/K/L/M CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L/M | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n78A-n257G-n259A | CA_n257G CA_n78A-n257A/G CA_n78A-n259A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257G-n259G | CA_n257G CA_n259G CA_n78A-n257A/G CA_n78A-n259A/G | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n78A-n257G-n259H | CA_n257G CA_n259G/H CA_n78A-n257A/G CA_n78A-n259A/G/H | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n78A-n257G-n259I | CA_n257G CA_n259G/H/I CA_n78A-n257A/G CA_n78A-n259A/G/H/I | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n78A-n257G-n259J | CA_n257G CA_n259G/H/I/J CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n78A-n257G-n259K | CA_n257G CA_n259G/H/I/J/K CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n78A-n257G-n259L | CA_n257G CA_n259G/H/I/J/K/L CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n78A-n257G-n259M | CA_n257G CA_n259G/H/I/J/K/L/M CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L/M | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n78A-n257H-n259A | CA_n257G/H CA_n78A-n257A/G/H CA_n78A-n259A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257H-n259G | CA_n257G/H CA_n259G CA_n78A-n257A/G/H CA_n78A-n259A/G | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n78A-n257H-n259H | CA_n257G/H CA_n259G/H CA_n78A-n257A/G/H CA_n78A-n259A/G/H | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n78A-n257H-n259I | CA_n257G/H CA_n259G/H/I CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n78A-n257H-n259J | CA_n257G/H CA_n259G/H/I/J CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n78A-n257H-n259K | CA_n257G/H CA_n259G/H/I/J/K CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n78A-n257H-n259L | CA_n257G/H CA_n259G/H/I/J/K/L CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n78A-n257H-n259M | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L/M | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n78A-n257I-n259A | CA_n257G/H/I CA_n78A-n257A/G/H/I CA_n78A-n259A | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n78A-n257I-n259G | CA_n257G/H/I CA_n259G CA_n78A-n257A/G/H/I CA_n78A-n259A/G | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n78A-n257I-n259H | CA_n257G/H/I CA_n259G/H CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n78A-n257I-n259I | CA_n257G/H/I CA_n259G/H/I CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n78A-n257I-n259J | CA_n257G/H/I CA_n259G/H/I/J CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n78A-n257I-n259K | CA_n257G/H/I CA_n259G/H/I/J/K CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n78A-n257I-n259L | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n78A-n257I-n259M | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L/M | n78 |  | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n79A-n257A-n259A | CA_n79A-n257A CA_n79A-n259A | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n79A-n257A-n259G | CA_n259G CA_n79A-n257A CA_n79A-n259A/G | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n79A-n257A-n259H | CA_n259G/H CA_n79A-n257A CA_n79A-n259A/G/H | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n79A-n257A-n259I | CA_n259G/H/I CA_n79A-n257A CA_n79A-n259A/G/H/I | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n79A-n257A-n259J | CA_n259G/H/I/J CA_n79A-n257A CA_n79A-n259A/G/H/I/J | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n79A-n257A-n259K | CA_n259G/H/I/J/K CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n79A-n257A-n259L | CA_n259G/H/I/J/K/L CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n79A-n257A-n259M | CA_n259G/H/I/J/K/L/M CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L/M | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | 50, 100, 200, 400 |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n79A-n257G-n259A | CA_n257G CA_n79A-n257A/G CA_n79A-n259A | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n79A-n257G-n259G | CA_n257G CA_n259G CA_n79A-n257A/G CA_n79A-n259A/G | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n79A-n257G-n259H | CA_n257G CA_n259G/H CA_n79A-n257A/G CA_n79A-n259A/G/H | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n79A-n257G-n259I | CA_n257G CA_n259G/H/I CA_n79A-n257A/G CA_n79A-n259A/G/H/I | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n79A-n257G-n259J | CA_n257G CA_n259G/H/I/J CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n79A-n257G-n259K | CA_n257G CA_n259G/H/I/J/K CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n79A-n257G-n259L | CA_n257G CA_n259G/H/I/J/K/L CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n79A-n257G-n259M | CA_n257G CA_n259G/H/I/J/K/L/M CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L/M | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257G |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n79A-n257H-n259A | CA_n257G/H CA_n79A-n257A/G/H CA_n79A-n259A | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n79A-n257H-n259G | CA_n257G/H CA_n259G CA_n79A-n257A/G/H CA_n79A-n259A/G | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n79A-n257H-n259H | CA_n257G/H CA_n259G/H CA_n79A-n257A/G/H CA_n79A-n259A/G/H | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n79A-n257H-n259I | CA_n257G/H CA_n259G/H/I CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n79A-n257H-n259J | CA_n257G/H CA_n259G/H/I/J CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n79A-n257H-n259K | CA_n257G/H CA_n259G/H/I/J/K CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n79A-n257H-n259L | CA_n257G/H CA_n259G/H/I/J/K/L CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n79A-n257H-n259M | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L/M | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257H |  |  |
|  |  | n259 |  | CA_n259M |  |  |
| CA_n79A-n257I-n259A | CA_n257G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | 50, 100, 200, 400 |  |  |
| CA_n79A-n257I-n259G | CA_n257G/H/I CA_n259G CA_n79A-n257A/G/H/I CA_n79A-n259A/G | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259G |  |  |
| CA_n79A-n257I-n259H | CA_n257G/H/I CA_n259G/H CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259H |  |  |
| CA_n79A-n257I-n259I | CA_n257G/H/I CA_n259G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259I |  |  |
| CA_n79A-n257I-n259J | CA_n257G/H/I CA_n259G/H/I/J CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259J |  |  |
| CA_n79A-n257I-n259K | CA_n257G/H/I CA_n259G/H/I/J/K CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259K |  |  |
| CA_n79A-n257I-n259L | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259L |  |  |
| CA_n79A-n257I-n259M | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L/M | n79 |  | 40, 50, 60, 80, 100 |  | 0 |
|  |  | n257 |  | CA_n257I |  |  |
|  |  | n259 |  | CA_n259M |  |  |

The following notes are applied to the above tables.

NOTE 1:	The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively.

NOTE 2:	The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to.

NOTE 3: 	The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively.

< Unchanged sections omitted>

### 5.5B.4	Inter-band EN-DC within FR1

5.5B.4.0 General

By default, power class 3 is applicable for the EN-DC configurations listed in the following sub-clauses. The applicability of higher power class(es) is described in the EN-DC configuration tables. For higher order EN-DC band combinations, the applicability of higher power class(es) is extended based on the following conditions:

- For an inter-band EN-DC combination with intra-band CA, UE may support the same higher power class(es) that are defined for the inter-band EN-DC combination composed of the same band without intra-band CA.

- For an inter-band EN-DC combination with 4 or more DL bands without intra-band CA, the higher power class(es) may be supported when the same higher power class(es) are specified for all its fallback combinations.

#### 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A DC_1A_n7B | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n26A | DC_1A_n26A | No |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A)7,21 DC_1A_n77(3A)7 | DC_1A_n77A21 | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7, 21 | DC_1A_n78A21 | No | No |
| DC_1A_n78(2A)7,21 DC_1A_n78(A-C)7 | DC_1A_n78A21 | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_1A_n105A | DC_1A_n105A | No |  |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n7A DC_2C_n7A | DC_2A_n7A | No |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |
| DC_2A-2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n12A | DC_2A_n12A | No |  |
| DC_2A-2A_n12A | DC_2A_n12A | No |  |
| DC_2A_n25A11, 13, 20 | N/A | N/A |  |
| DC_2A_n28A DC_2C_n28A | DC_2A_n28A | No |  |
| DC_2A_n30A | DC_2A_n30A | No |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |
| DC_2A_n38A | DC_2A_n38A | No |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |
| DC_2A_n46A | DC_2A_n46A | No |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A | No |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A DC_2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n77(2A)21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77A21 DC_2A-2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77(2A)21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A_n78(2A)21 | DC_2A_n78A21 | DC_2_n78 |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |
| DC_3A_n8A | DC_3A_n8A | No |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |
| DC_3A_n26A DC_3C_n26A | DC_3A_n26A DC_3C_n26A | Yes |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |  |
| DC_3A_n34A | DC_3A_n34A | No |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |
| DC_3A_n40A DC_3A_n40B DC_3C_n40A | DC_3A_n40A | No |  |
| DC_3A_n41A7 DC_3A_n41C DC_3C_n41A7 | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |
| DC_3A_n50A | DC_3A_n50A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7,21 | DC_3A_n77A21 DC_3C_n77A | DC_3_n77 | No |
| DC_3A_n77(2A)7,21 DC_3A_n77(3A)7 DC_3C_n77(2A)7,21 | DC_3A_n77A,21 DC_3C_n77A | DC_3_n77 | No |
| DC_3A-3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7,23 DC_3A_n78C7 DC_3C_n78A7,21 | DC_3A_n78A,21,23 DC_3C_n78A | DC_3_n78 | No |
| DC_3A_n78(2A)7,21 DC_3A_n78(A-C)7 DC_3C_n78(2A)7,21 | DC_3A_n78A,21 DC_3C_n78A | DC_3_n78 | No |
| DC_3A-3A_n78A7, 21 | DC_3A_n78A21 | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No | No |
| DC_3A-3A_n79A7 | DC_3A_n79A | No |  |
| DC_3A_n105A | DC_3A_n105A | No |  |
| DC_4A_n2A | DC_4A_n2A | No |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |
| DC_4A_n7A | DC_4A_n7A | No |  |
| DC_4A_n28A | DC_4A_n28A | No |  |
| DC_4A_n38A | DC_4A_n38A | No |  |
| DC_4A_n41A | DC_4A_n41A | No |  |
| DC_4A_n78A | DC_4A_n78A | No |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |
| DC_5A_n1A | DC_5A_n1A | No |  |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |  |
| DC_5A_n2(2A) | DC_5A_n2A | No |  |
| DC_5A-5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n3A | DC_5A_n3A | DC_5_n3 |  |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n12A | DC_5A_n12A | No |  |
| DC_5A_n25A | DC_5A_n25A | No |  |
| DC_5A_n28A | DC_5A_n28A | No |  |
| DC_5A_n30A | DC_5A_n30A | No |  |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |  |
| DC_5A_n40A | DC_5A_n40A | No |  |
| DC_5A_n41A | DC_5A_n41A | No |  |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |  |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A DC_5A_n77C21 | DC_5A_n77A21 | No |  |
| DC_5A_n77(2A)21 DC_5A_n77(3A) | DC_5A_n77A21 | No |  |
| DC_5A_n71A | DC_5A_n71A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_5A_n78(2A)7,21 DC_5A_n78(A-C)7 | DC_5A_n78A21 | No | No |
| DC_5A_n79A | DC_5A_n79A | No | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |  |
| DC_7A-7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |  |
| DC_7A_n2(2A) | DC_7A_n2A | No |  |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |  |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |  |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n8A | DC_7A_n8A | No |  |
| DC_7A-7A_n8A | DC_7A_n8A | No |  |
| DC_7A_n12A | DC_7A_n12A | No |  |
| DC_7A-7A_n78(2A)7,21 | DC_7A_n78A21 | No |  |
| DC_7A_n20A | DC_7A_n20A | No |  |
| DC_7A_n25A DC_7C_n25A | DC_7A_n25A | No |  |
| DC_7A_n26A DC_7C_n26A | DC_7A_n26A DC_7C_n26A | Yes |  |
| DC_7A-7A_n25A | DC_7A_n25A | No |  |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |  |
| DC_7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n51A | DC_7A_n51A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A-7A_n66A | DC_7A_n66A | No |  |
| DC_7A_n71A | DC_7A_n71A | No |  |
| DC_7A_n77A7 DC_7C_n77A | DC_7A_n77A | No |  |
| DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77(2A) | DC_7A_n77A | No |  |
| DC_7A-7A_n77A7 | DC_7A_n77A | No |  |
| DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) | DC_7A_n77A | No |  |
| DC_7A_n78A7,23 DC_7C_n78A7,21 DC_7A_n78C7 | DC_7A_n78A21,23 DC_7C_n78A | No |  |
| DC_7A_n78(2A)7,21 DC_7A_n78(A-C)7 DC_7C_n78(2A)7, 21 | DC_7A_n78A21 DC_7C_n78A | No |  |
| DC_7A-7A_n78A7, 21 DC_7A-7A_n78C7 | DC_7A_n78A21 | No |  |
| DC_7A-7A_n78(A-C)7 | DC_7A_n78A | No |  |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A | No |  |
| DC_7A-7A_n79A | DC_7A_n79A | No |  |
| DC_7A_n105A | DC_7A_n105A | No |  |
| DC_8A_n1A DC_8B_n1A | DC_8A_n1A DC_8B_n1A | No |  |
| DC_8A_n2A | DC_8A_n2A | DC_8_n2 |  |
| DC_8A_n3A DC_8B_n3A | DC_8A_n3A | No |  |
| DC_8A_n7A | DC_8A_n7A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n28A | DC_8A_n28A | No |  |
| DC_8A_n34A | DC_8A_n34A | No |  |
| DC_8A_n38A | DC_8A_n38A | No |  |
| DC_8A_n39A | DC_8A_n39A | No |  |
| DC_8A_n40A7 | DC_8A_n40A | No |  |
| DC_8A_n41A7 DC_8A_n41C | DC_8A_n41A | No | No |
| DC_8A_n41(2A) | DC_8A_n41A | No | No |
| DC_8A_n77A7 DC_8B_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n77(2A)7,21 DC_8B_n77(2A)7 DC_8A_n77(3A)7 | DC_8A_n77A21 | No | No |
| DC_8A_n78A7,23 DC_8B_n78A7, 21 | DC_8A_n78A21,23 DC_8B_n78A | No | No |
| DC_8A_n78(2A)7, 21 | DC_8A_n78A21 | No | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |  |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |
| DC_11A_n1A | DC_11A_n1A | No |  |
| DC_11A_n3A | DC_11A_n3A | No |  |
| DC_11A_n28A | DC_11A_n28A | No |  |
| DC_11A_n41A7 | DC_11A_n41A | No |  |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n77(2A)7 DC_11A_n77(3A)7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n78(2A) | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n2A | DC_12A_n2A | No |  |
| DC_12A_n2(2A) | DC_12A_n2A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n7A | DC_12A_n7A | No |  |
| DC_12A_n7(2A) | DC_12A_n7A | No |  |
| DC_12A_n25A | DC_12A_n25A | No |  |
| DC_12A_n30A | DC_12A_n30A | No |  |
| DC_12A_n38A | DC_12A_n38A | No |  |
| DC_12A_n41A | DC_12A_n41A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n66(2A) | DC_12A_n66A | No |  |
| DC_12A_n71A | DC_12A_n71A18,19 | DC_12_n71 |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n77(2A)21 | DC_12A_n77A21 | DC_12_n77 |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |  |
| DC_13A_n7A | DC_13A_n7A | No |  |
| DC_13A_n7(2A) | DC_13A_n7A | No |  |
| DC_13A_n25A | DC_13A_n25A | No |  |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n71A | DC_13A_n71A | No |  |
| DC_13A_n77A DC_13A_n77C21 | DC_13A_n77A21 | No |  |
| DC_13A_n78A | DC_13A_n78A | No |  |
| DC_13A_n78(2A)21 | DC_13A_n78A21 | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n5A | DC_14A_n5A | DC_14_n5 |  |
| DC_14A_n30A | DC_14A_n30A | No |  |
| DC_14A_n41A | DC_14A_n41A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_14A_n77A | DC_14A_n77A | No |  |
| DC_14A_n77(2A)21 | DC_14A_n77A21 | No |  |
| DC_18A_n3A | DC_18A_n3A | No |  |
| DC_18A_n28A | DC_18A_n28A | No |  |
| DC_18A_n41A16 | DC_18A_n41A | No |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n78(2A)7 | DC_18A_n78A | No | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n77(2A)7, 21 | DC_19A_n77A21 | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n78(2A)7, 21 | DC_19A_n78A21 | No | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n38A | DC_20A_n38A | No |  |
| DC_20A_n40A | DC_20A_n40A | No |  |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |
| DC_20A_n50A | DC_20A_n50A | No |  |
| DC_20A_n51A | DC_20A_n51A | No |  |
| DC_20A_n77A7 | DC_20A_n77A | No |  |
| DC_20A_n78A7,23 DC_20A_n78C7 | DC_20A_n78A23 | No |  |
| DC_20A_n78(2A)7 | DC_20A_n78A | No |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |  |
| DC_21A_n77(2A)7,21 | DC_21A_n77A21 | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n78(2A)7,21 | DC_21A_n78A21 | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_25A-25A_n41A | DC_25A_n41A | No |  |
| DC_25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A-25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_25A-25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_26A_n25A | DC_26A_n25A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n78(2A) | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n1A | DC_28A_n1A | No |  |
| DC_28A_n2A | DC_28A_n2A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A | DC_28A_n5A | No |  |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |  |
| DC_28A_n51A | DC_28A_n51A | No |  |
| DC_28A_n8A | DC_28A_n8A | No |  |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| DC_28A_n38A | DC_28A_n38A | No |  |
| DC_28A_n40A DC_28C_n40A | DC_28A_n40A | No |  |
| DC_28A_n41A7 | DC_28A_n41A | No |  |
| DC_28A_n50A | DC_28A_n50A | No |  |
| DC_28A_n66A | DC_28A_n66A | No |  |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No | No |
| DC_28A_n78A7,23 DC_28A_n78C7 | DC_28A_n78A23 | No | No |
| DC_28A_n78(2A)7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |  |
| DC_28A_n105A | DC_28A_n105A18 | DC_28_n105 |  |
| DC_30A_n2A | DC_30A_n2A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_30A_n77A | DC_30A_n77A | No |  |
| DC_30A_n77(2A)21 | DC_30A_n77A21 | No |  |
| DC_38A_n1A | DC_38A_n1A | No |  |
| DC_38A_n3A | DC_38A_n3A | No |  |
| DC_38A_n8A | DC_38A_n8A | No |  |
| DC_38A_n28A | DC_38A_n28A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A | No |  |
| DC_39A_n40A3 | DC_39A_n40A | No |  |
| DC_39A_n41A DC_39C_n41A DC_39A_n41C | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |  |
| DC_39A_n79A7 DC_39A_n79C7 | DC_39A_n79A | No | No |
| DC_40A_n1A DC_40C_n1A | DC_40A_n1A | No |  |
| DC_40A_n3A | DC_40A_n3A | No |  |
| DC_40A_n7A | DC_40A_n7A | No |  |
| DC_40A_n41A DC_40A_n41C DC_40C_n41A | DC_40A_n41A | No |  |
| DC_40A_n41(2A) | DC_40A_n41A | No |  |
| DC_40A_n77A DC_40A_n77C DC_40C_n77A21 DC_40C_n77C DC_40D_n77A21 | DC_40A_n77A21 | No |  |
| DC_40A_n78A DC_40A_n78C DC_40C_n78A21 DC_40C_n78C DC_40D_n78A21 | DC_40A_n78A21, 23 DC_40C_n78A | No |  |
| DC_40A_n78(2A) DC_40C_n78(2A) | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No | DC_41A_n1A DC_41C_n1A |
| DC_41A_n3A7 DC_41C_n3A7 | DC_41A_n3A DC_41C_n3A | No |  |
| DC_41A_n28A7 DC_41C_n28A7 | DC_41A_n28A DC_41C_n28A | No |  |
| DC_41A_n77A DC_41C_n77A21 | DC_41A_n77A21 DC_41C_n77A | No |  |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n78A23 DC_41C_n78A DC_41D_n78A | DC_41A_n78A23 DC_41C_n78A | No |  |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41A_n79C DC_41C_n79A | No | No |
| DC_42A_n1A7 DC_42C_n1A7 | DC_42A_n1A DC_42C_n1A | No |  |
| DC_42A_n3A7 DC_42C_n3A7 | DC_42A_n3A DC_42C_n3A | DC_42_n3 |  |
| DC_42A_n28A7 DC_42C_n28A7 | DC_42A_n28A DC_42C_n28A | No |  |
| DC_42A_n51A | DC_42A_n51A | No |  |
| DC_42A_n77A3,4,9,11,13 DC_42A_n77C3,4,9 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |
| DC_42A_n77(2A)3,4,9 DC_42C_n77(2A)3,4,9 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11,13 DC_42A_n78C3,4,9 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 | N/A | N/A |  |
| DC_46A_n77A2 | N/A | N/A |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | DC_48A_n2A | No |  |
| DC_48A_n5A DC_48B_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A | DC_48A_n5A | No |  |
| DC_48A_n12A | DC_48A_n12A | No |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D | DC_48A_n46A DC_48B_n46A | No |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A | DC_48A_n66A | No |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |
| DC_48A-48A_n77A | N/A | N/A |  |
| DC_48A-48A-48A_n77A | N/A | N/A |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n2(2A) | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n7A | DC_66A_n7A | No |  |
| DC_66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A-66A_n7A | DC_66A_n7A | No |  |
| DC_66A-66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A_n12A | DC_66A_n12A | No |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |
| DC_66A_n28A | DC_66A_n28A | No |  |
| DC_66A_n30A | DC_66A_n30A | No |  |
| DC_66A-66A_n30A | DC_66A_n30A | No |  |
| DC_66A_n38A | DC_66A_n38A | No |  |
| DC_66A-66A_n38A | DC_66A_n38A | No |  |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |  |
| DC_66A_n41(2A) | DC_66A_n41A | No |  |
| DC_66A_n46A | DC_66A_n46A | No |  |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |  |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |  |
| DC_66A-66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n77A DC_66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77A21 DC_66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77A21 DC_66A-66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77(2A)21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_66A_n78(2A) 21 | DC_66A_n78A21 | No |  |
| DC_66A-66A_n78A21 | DC_66A_n78A21 | No |  |
| DC_66A-66A_n78(2A)21 | DC_66A_n78A21 | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| DC_71A_n2(2A) | DC_71A_n2A | No |  |
| DC_71A_n5A | DC_71A_n5A | No |  |
| DC_71A_n12A | DC_71A_n12A18,19 | Yes |  |
| DC_71A_n38A | DC_71A_n38A | No |  |
| DC_71A_n7A | DC_71A_n7A | No |  |
| DC_71A_n25A | DC_71A_n7A | No |  |
| DC_71A_n41A | DC_71A_n41A | No |  |
| DC_71A_n48A | DC_71A_n48A | No |  |
| DC_71A_n66A | DC_71A_n66A | No |  |
| DC_71A_n77A DC_71A_n77C | DC_71A_n77A | No |  |
| DC_71A_n77(2A) | DC_71A_n77A | No |  |
| DC_71A_n78A | DC_71A_n78A | No |  |
| DC_71A_n78(2A)21 | DC_71A_n78A21 | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: If a UE does not indicate interBandMRDC-WithOverlapDL-Bands-r16 or a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and IE nonCollocatedTypeMRDC-r18 is provided, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. If a UE does not indicate interBandMRDC-WithOverlapDL-Bands-r16 or a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and IE nonCollocatedTypeMRDC-r18 is provided and UE indicates interBandContiguousMRDC, the minimum requirements for intra-band contiguous EN-DC also apply in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 /28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also applies for any band combinations when DC_20_n28/DC_28_n20/CA_20-28/CA_n20-n28 is a subset of a higher order band combination. NOTE 9: The combination is not used alone as fall-back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: If a UE does not indicate interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. If the UE indicates interBandMRDC-WithOverlapDL-Bands-r16 but does not indicate requirementTypeIndication-r18 or a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and IE nonCollocatedTypeMRDC-r18 is not provided when maxMIMO-Layers with value less than or equal to 2, the power imbalance requirement defined in clause 7.10B.3 apply. If a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and IE nonCollocatedTypeMRDC-r18 is provided, the minimum requirements apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: If a UE does not indicated interBandMRDC-WithOverlapDL-Bands-r16 or a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 and IE nonCollocatedTypeMRDC-r18 is provided, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 21: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration with 1Tx antenna connector in each band. NOTE 22: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration without additional indication of NOTE 21. NOTE 23: Minimum requirements for Power Class 2 are applicable for this EN-DC configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n1A | DC_1A_n1A2 DC_3A_n1A |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |
| DC_1A-(n)3AA | DC_1A_n3A |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-1A-3A-3A_n7A DC_1A-1A-3A-3A_n7B | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |
| DC_1A-3A_n26A DC_1A-3C_n26A | DC_1A_n26A DC_3A_n26A |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-1A-3A_n28A DC_1A-1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |
| DC_1A_n3A-n38A | DC_1A_n3A DC_1A_n38A |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |
| DC_1A-3A_n41A5, 14 DC_1A-3C_n41A14 | DC_1A_n41A14 DC_3A_n41A14 DC_3C_n41A14 |
| DC_1A_n3A-n41A5 | DC_1A_n3A DC_1A_n41A |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |
| DC_1A-3A_n77A5, 14 DC_1A-3A_n77C5 DC_1A-3C_n77A5,14 | DC_1A_n77A14 DC_3A_n77A14 DC_3C_n77A |
| DC_1A-3A_n77(2A)5,14 DC_1A-3C_n77(2A)5,14 | DC_1A_n77A14 DC_3A_n77A14 DC_3C_n77A |
| DC_1A-3A_n77(3A)5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5,14 DC_1A-3A_n78C5 DC_1A-3C_n78A5,14 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-3A_n78(2A)5, 14 DC_1A-3C_n78(2A)5, 14 | DC_1A_n78A14 DC_3A_n78A14 DC_3C_n78A |
| DC_1A-3A_n78(A-C)5 | DC_1A_n78A DC_3A_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-1A-3A-3A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-3A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A |
| DC_1A_n3A-n75A | DC_1A_n3A |
| DC_1A_n3A-n77A5, 14 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n77(2A) 5 | DC_1A_n3A DC_1A_n77A |
| DC_1A_n3A-n78A5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n78(2A)5 | DC_1A_n3A DC_1A_n78A |
| DC_1A_n3A-n79A14 | DC_1A_n3A DC_1A_n79A14 |
| DC_1A-3A_n79A5,14 DC_1A-3A_n79C5 | DC_1A_n79A14 DC_3A_n79A14 |
| DC_1A-3A_n105A | DC_1A_n105A DC_3A_n105A |
| DC_1A-5A_n28A | DC_1A_n28A DC_5A_n28A |
| DC_1A-5A_n40A | DC_1A_n40A DC_5A_n40A |
| DC_1A_n5A-n40A | DC_1A_n5A DC_1A_n40A |
| DC_1A-5A_n77A | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n77(2A) DC_1A-5A_n77(3A) | DC_1A_n77A DC_5A_n77A |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(2A)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n78(A-C)5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |
| DC_1A_n5A-n78A5 | DC_1A_n5A DC_1A_n78A |
| DC_1A-7A_n1A | DC_1A_n1A DC_7A_n1A |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-(n)7AA | DC_1A_n7A |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A_n20A | DC_1A_n20A DC_7A_n20A |
| DC_1A-7A_n26A | DC_1A_n26A DC_7A_n26A |
| DC_1A-7C_n26A | DC_1A_n26A DC_7A_n26A DC_7C_n26A |
| DC_1A-7A_n28A5 DC_1A-7C_n28A5 | DC_1A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-1A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-7A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n77(2A) DC_1A-7A_n77(3A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77A | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A-7A_n77(2A) DC_1A-7A-7A_n77(3A) | DC_1A_n77A DC_7A_n77A |
| DC_1A-7A_n78A5 DC_1A-7C_n78A5 DC_1A-7A_n78C5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(2A)5 DC_1A-7C_n78(2A)5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(A-C)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-1A-7A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78A5 DC_1A-7A-7A_n78C5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(2A)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-7A_n78(A-C)5 | DC_1A_n78A DC_7A_n78A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |
| DC_1A_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A |
| DC_1A-7A_n105A | DC_1A_n105A DC_7A_n105A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8B_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n7A | DC_8A_n7A DC_1A_n7A |
| DC_1A-8A_n20A | DC_1A_n20A DC_8A_n20A |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |
| DC_1A-8A_n40A | DC_1A_n40A DC_8A_n40A |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |
| DC_1A-8A_n77A5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| DC_1A-8B_n77A5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77(2A)5,14 | DC_1A_n77A14 DC_8A_n77A14 |
| DC_1A-8B_n77(2A)5 | DC_1A_n77A DC_8A_n77A |
| DC_1A_n8A-n77A | DC_1A_n8A DC_1A_n77A |
| DC_1A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A |
| DC_1A-8A_n77(3A)5 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5,14 DC_1A-8B_n78A5,14 | DC_1A_n78A14 DC_8A_n78A14 |
| DC_1A-8A_n78(2A)5,14 | DC_1A_n78A14 DC_8A_n78A14 |
| DC_1A_n8A-n78A5 | DC_1A_n8A DC_1A_n78A |
| DC_1A-8A_n79A5,14 | DC_1A_n79A14 DC_8A_n79A14 |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |
| DC_1A-11A_n28A | DC_1A_n28A DC_11A_n28A |
| DC_1A-11A_n41A5 | DC_1A_n41A DC_11A_n41A |
| DC_1A-11A_n77A5,14 | DC_1A_n77A14 DC_11A_n77A |
| DC_1A-11A_n77(2A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(3A)5 | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n78A5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n78(2A)5 | DC_1A_n78A DC_11A_n78A |
| DC_1A-11A_n79A5,14 | DC_1A_n79A14 DC_11A_n79A14 |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |
| DC_1A-18A_n28A | DC_1A_n28A DC_18A_n28A |
| DC_1A-18A_n41A | DC_1A_n41A DC_18A_n41A |
| DC_1A-18A_n77A5,14 | DC_1A_n77A14 DC_18A_n77A |
| DC_1A-18A_n77(2A)5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n78(2A)5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A_n77A5,14 DC_1A-19A_n77C5 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n77(2A)5,14 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n78A5,14 DC_1A-19A_n78C5 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n78(2A)5,14 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n79A5, 14 DC_1A-19A_n79C5 | DC_1A_n79A14 DC_19A_n79A14 |
| DC_1A-20A_n1A | DC_1A_n1A2 DC_20A_n1A |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n7A | DC_1A_n7A DC_20A_n7A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n41A | DC_1A_n41A DC_20A_n41A |
| DC_1A-20A_n78A5 DC_1A-20A_n78C5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n78(2A)5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n28A13 | DC_1A_n28A DC_21A_n28A |
| DC_1A-21A_n77A5, 14 DC_1A-21A_n77C5, 14 | DC_1A_n77A14 DC_21A_n77A14 |
| DC_1A-21A_n77(2A)5,14 | DC_1A_n77A14 DC_21A_n77A14 |
| DC_1A-21A_n78A5,14 DC_1A-21A_n78C5 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n78(2A)5,14 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n79A5,14 DC_1A-21A_n79C5 | DC_1A_n79A14 DC_21A_n79A14 |
| DC_1A-26A_n78A | DC_1A_n78A DC_26A_n78A |
| DC_1A-26A_n78(2A) | DC_1A_n78A DC_26A_n78A |
| DC_1A_n26A-n78A | DC_1A_n26A DC_1A_n78A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-28A_n20A22 | DC_1A_n20A DC_28A_n20A22 |
| DC_1A-28A_n38A | DC_1A_n38A DC_28A_n38A |
| DC_1A_n28A-n38A | DC_1A_n28A DC_1A_n38A |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |
| DC_1A_n28A-n41A5 | DC_1A_n28A DC_1A_n41A |
| DC_1A_n28A-n75A | DC_1A_n28A |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 | DC_1A_n78A DC_28A_n78A |
| DC_1A-1A-28A_n78A | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n78(2A) | DC_1A_n78A DC_28A_n78A |
| DC_1A_n28A-n77A5, 14 | DC_1A_n28A DC_1A_n77A14 |
| DC_1A_n28A-n77(2A)5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A_n28A-n78(2A)5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A5 DC_1A-28A_n79C5 | DC_1A_n79A DC_28A_n79A |
| DC_1A_n28A-n79A5, 14 | DC_1A_n28A DC_1A_n79A14 |
| DC_1A-32A_n3A | DC_1A_n3A |
| DC_1A-32A_n8A | DC_1A_n8A |
| DC_1A-32A_n28A | DC_1A_n28A |
| DC_1A-32A_n78A DC_1A-32A_n78C | DC_1A_n78A |
| DC_1A-32A_n78(2A) | DC_1A_n78A |
| DC_1A-38A_n3A | DC_1A_n3A DC_38A_n3A |
| DC_1A-38A_n8A | DC_1A_n8A DC_38A_n8A |
| DC_1A-38A_n28A | DC_1A_n28A DC_38A_n28A |
| DC_1A-(n)38AA | DC_1A_n38A |
| DC_1A_n38A-n78A | DC_1A_n38A DC_1A_n78A |
| DC_1A-38A_n78A | DC_1A_n78A DC_38A_n78A |
| DC_1A-38A_n78(2A) | DC_1A_n78A |
| DC_1A_n40A-n77A | DC_1A_n40A DC_1A_n77A |
| DC_1A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A |
| DC_1A-40A_n78A DC_1A-40C_n78A | DC_1A_n78A DC_40A_n78A |
| DC_1A-40A_n78(2A) DC_1A-40C_n78(2A) | DC_1A_n78A DC_40A_n78A |
| DC_1A_n40A-n78A DC_1A_n40A-n78C | DC_1A_n40A DC_1A_n78A |
| DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A |
| DC_1A_n40A-n105A | DC_1A_n40A DC_1A_n105A |
| DC_1A-41A_n3A5 DC_1A-41C_n3A5 | DC_1A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |
| DC_1A-41A_n77A14 DC_1A-41C_n77A14 | DC_1A_n77A14 DC_41A_n77A DC_41C_n77A |
| DC_1A-41A_n77(2A) 14 DC_1A-41C_n77(2A) 14 | DC_1A_n77A14 DC_41A_n77A DC_41C_n77A |
| DC_1A_n41A-n77A14 | DC_1A_n41A14 DC_1A_n77A14 |
| DC_1A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A_n41A-n78A | DC_1A_n41A DC_1A_n78A |
| DC_1A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-41A_n79A5 DC_1A-41C_n79A5 | DC_1A_n79A |
| DC_1A-42A_n3A5 | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A5 | DC_1A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A5 | DC_1A_n28A DC_42A_n28A |
| DC_1A-42C_n28A5 | DC_1A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-42A_n77A14, 15,16 DC_1A-42A_n77C15,16 DC_1A-42C_n77A14, 15,16 DC_1A-42C_n77C15,16 DC_1A-42D_n77A14, 15,16 DC_1A-42D_n77C15,16 DC_1A-42E_n77A14, 15,16 DC_1A-42E_n77C15,16 | DC_1A_n77A14, |
| DC_1A-42A_n77(2A)15,16 DC_1A-42C_n77(2A)15,16 | DC_1A_n77A |
| DC_1A-42A_n78A14,15,16 DC_1A-42A_n78C15,16 DC_1A-42C_n78A14,15,16 DC_1A-42C_n78C15,16 DC_1A-42D_n78A14,15,16 DC_1A-42D_n78C15,16 DC_1A-42E_n78A14,15,16 DC_1A-42E_n78C15,16 | DC_1A_n78A14 |
| DC_1A-42A_n79A14 DC_1A-42A_n79C DC_1A-42C_n79A14 DC_1A-42C_n79C DC_1A-42D_n79A14 DC_1A-42D_n79C DC_1A-42E_n79A14 DC_1A-42E_n79C | DC_1A_n79A14 |
| DC_1A_n75A-n78A | DC_1A_n78A |
| DC_1A_n75A-n78(2A) | DC_1A_n78A |
| DC_1A_n77A-n79A14, 23 | DC_1A_n77A14 DC_1A_n79A14 |
| DC_1A_n77(2A)-n79A23 | DC_1A_n77A DC_1A_n79A |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |
| DC_1A_n78A-n79A14, 24 | DC_1A_n78A14 DC_1A_n79A14 |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |
| DC_1A_n78A-n105A | DC_1A_n78A DC_1A_n105A |
| DC_2A_n2A-n38A | DC_2A_n38A |
| DC_2A_n2A-n41A | DC_2A_n41A |
| DC_2A_n2A-n66A | DC_2A_n66A |
| DC_2A_n2A-n71A | DC_2A_n71A |
| DC_2A_n2A-n77A14 DC_2A_n2A-n77C14 | DC_2A_n77A14 |
| DC_2A_n2A-n78A | DC_2A_n78A |
| DC_2A-4A_n28A | DC_2A_n28A DC_4A_n28A |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |
| DC_2A-4A_n78A | DC_2A_n78A DC_4A_n78A |
| DC_2A-5A_n2A | DC_5A_n2A DC_2A_n2A2 |
| DC_2A-5B_n2A | DC_5A_n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A |
| DC_2A-5A_n5A | DC_2A_n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A |
| DC_2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-2A-(n)5AA | DC_2A_n5A DC_(n)5AA2 |
| DC_2A-5A_n7A | DC_2A_n7A DC_5A_n7A |
| DC_2A-2A-5A_n7A | DC_2A_n7A DC_5A_n7A |
| DC_2A-5A_n12A | DC_2A_n12A DC_5A_n12A |
| DC_2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-2A-5A_n30A | DC_2A_n30A DC_5A_n30A |
| DC_2A-5A_n41A | DC_2A_n41A DC_5A_n41A |
| DC_2A-2A-5A_n41A | DC_2A_n41A DC_5A_n41A |
| DC_2A-5A_n48A DC_2A-5A_n48B | DC_2A_n48A DC_5A_n48A |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A_n66A DC_2A-2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |
| DC_2A-5A_n77A14 DC_2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n77(2A)14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-2A-5A_n77A14 DC_2A-2A-5A_n77C14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-2A-5A_n77(2A)14 | DC_2A_n77A14 DC_5A_n77A14 |
| DC_2A-5A_n78A | DC_2A_n78A DC_5A_n78A |
| DC_2A-2A-5A_n78A | DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n78(2A) | DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A | DC_7A_n2A |
| DC_2A-7A_n5A DC_2A-7C_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A-7A_n5A | DC_2A_n5A DC_7A_n5A |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |
| DC_2A-7A_n12A | DC_2A_n12A DC_7A_n12A |
| DC_2A-2A-7A_n12A | DC_2A_n12A DC_7A_n12A |
| DC_2A-7A_n25A15, 16 DC_2A-7A-7A_n25A15, 16 DC_2A-7C_n25A15, 16 | DC_7A_n25A |
| DC_2A-7A_n28A DC_2C-7A_n28A DC_2A-7C_n28A | DC_2A_n28A DC_7A_n28A |
| DC_2A_n5A-n77A14 DC_2A-2A_n5A-n77A14 DC_2A_n5A-n77C14 DC_2A-2A_n5A-n77C14 | DC_2A_n5A DC_2A_n77A14 |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-2A-7A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A_n7A-n66A | DC_2A_n7A DC_2A_n66A |
| DC_2A_n7(2A)-n66A | DC_2A_n7A DC_2A_n66A |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-7A_n77A DC_2A-7C_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-2A-7A_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77A | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n77(2A) DC_2A-7C_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-7A_n77(2A) | DC_2A_n77A DC_7A_n77A |
| DC_2A-7A_n78A5,14 DC_2A-7C_n78A5,14 | DC_2A_n78A14 DC_7A_n78A14 DC_7C_n78A |
| DC_2A-7A_n78(2A) 5,14 DC_2A-7C_n78(2A) 5,14 | DC_2A_n78A14 DC_7A_n78A14 DC_7C_n78A |
| DC_2A-2A-7A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7A-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A-7A-7A_n78A5,14 | DC_2A_n78A14 DC_7A_n78A14 |
| DC_2A-7A-7A_n78(2A) 5,14 | DC_2A_n78A14 DC_7A_n78A14 |
| DC_2A-8A_n2A | DC_2A_n2A2 DC_8A_n2A |
| DC_2A-12A_n2A | DC_12A_n2A |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-2A-12A_n5A | DC_2A_n5A DC_12A_n5A |
| DC_2A-12A_n7A | DC_2A_n7A DC_12A_n7A |
| DC_2A-2A-12A_n7A | DC_2A_n7A DC_12A_n7A |
| DC_2A-12A_n7(2A) | DC_2A_n7A DC_12A_n7A |
| DC_2A-(n)12AA | DC_2A_n12A DC_(n)12AA2 |
| DC_2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-2A-12A_n30A | DC_2A_n30A DC_12A_n30A |
| DC_2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-2A-12A_n41A | DC_2A_n41A DC_12A_n41A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-12A_n77A14 DC_2A-2A-12A_n77A14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A-12A_n77(2A)14 DC_2A-2A-12A_n77(2A)14 | DC_2A_n77A14 DC_12A_n77A14 |
| DC_2A_n12A-n77A | DC_2A_n77A DC_2A_n12A |
| DC_2A-2A_n12A-n77A | DC_2A_n12A DC_2A_n77A |
| DC_2A_n12A-n78A | DC_2A_n12A DC_2A_n78A |
| DC_2A-13A_n2A | DC_13A_n2A |
| DC_2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-12A_n78(2A) | DC_2A_n78A DC_12A_n78A |
| DC_2A-2A-12A_n78A | DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n5A | DC_2A_n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A |
| DC_2A-13A_n25A16,20 | DC_13A_n25A |
| DC_2A-13A_n48A DC_2A-13A_n48B | DC_2A_n48A DC_13A_n48A |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A_n77A14 DC_2A-13A_n77C14 DC_2A-2A-13A_n77C14 | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-2A-13A_n77A14 | DC_2A_n77A14 DC_13A_n77A14 |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-2A-14A_n5A | DC_2A_n5A DC_14A_n5A |
| DC_2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-2A-14A_n30A | DC_2A_n30A DC_14A_n30A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-14A_n77A14 DC_2A-2A-14A_n77A14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A-14A_n77(2A) 14 DC_2A-2A-14A_n77(2A) 14 | DC_2A_n77A14 DC_14A_n77A14 |
| DC_2A_n25A-n66A | DC_2A_n66A |
| DC_2A-28A_n7A DC_2C-28A_n7A | DC_2A_n7A DC_28A_n7A |
| DC_2A-28A_n66A | DC_2A_n66A DC_28A_n66A |
| DC_2A-28A_n78A | DC_2A_n78A DC_28A_n78A |
| DC_2A-28A_n78(2A) | DC_2A_n78A DC_28A_n78A |
| DC_2A-29A_n30A | DC_2A_n30A |
| DC_2A-2A-29A_n30A | DC_2A_n30A |
| DC_2A-29A_n66A | DC_2A_n66A |
| DC_2A-2A-29A_n66A | DC_2A_n66A |
| DC_2A-29A_n77A14 DC_2A-2A-29A_n77A14 | DC_2A_n77A14 |
| DC_2A-29A_n78A | DC_2A_n78A |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n2A | DC_2A_n2A2 DC_30A_n2A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-30A_n77A14 DC_2A-2A-30A_n77A14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A-30A_n77(2A) 14 DC_2A-2A-30A_n77(2A) 14 | DC_2A_n77A14 DC_30A_n77A14 |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_n71A |
| DC_2A-38A_n78A | DC_2A_n78A DC_38A_n78A |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-2A_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-2A_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n2A3 DC_2A-46C_n2A3 DC_2A-46D_n2A3 DC_2A-46E_n2A3 | DC_2A_n2A2 |
| DC_2A-46A_n5A3 DC_2A-46C_n5A3 DC_2A-46D_n5A3 DC_2A-46E_n5A3 DC_2A-2A-46A_n5A3 DC_2A-2A-46C_n5A3 DC_2A-2A-46D_n5A3 | DC_2A_n5A |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A DC_2A-46E_n66A | DC_2A_n66A |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |
| DC_2A-46A_n77A | DC_2A_n77A |
| DC_2A-46A-46A_n77A | DC_2A_n77A |
| DC_2A-48A_n2A DC_2A-48C_n2A DC_2A-48D_n2A DC_2A-48E_n2A | DC_2A_n2A2 DC_48A_n2A21 |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-48C_n5A DC_2A-48D_n5A DC_2A-48E_n5A | DC_2A_n5A |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |
| DC_2A-48A_n48A | DC_2A_n48A |
| DC_2A-48A_n66A DC_2A-48C_n66A DC_2A-48D_n66A DC_2A-48E_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-48A_n77A14,15,16 | DC_2A_n77A14 |
| DC_2A-48A-48A_n77A14,15,16 | DC_2A_n77A14 |
| DC_2A-48A-48A-48A_n77A14,15,16 | DC_2A_n77A14 |
| DC_2A-48C_n77A14,15,16 DC_2A-48D_n77A14,15,16 DC_2A-48E_n77A14,15,16 DC_2A-48A_n77C14,15,16 DC_2A-48C_n77C14,15,16 DC_2A-48D_n77C14,15,16 | DC_2A_n77A14 |
| DC_2A-66A_n2A | DC_2A_n2A2 DC_66A_n2A |
| DC_2A-66A-66A_n2A | DC_66A_n2A |
| DC_2A-66A_n5A DC_2A-66B_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-2A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A-66A_n7A | DC_2A_n7A DC_66A_n7A |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |
| DC_2A-66A_n25A16,20 | DC_66A_n25A |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |
| DC_2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A_n41A14 DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A14 |
| DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-(n)66AA | DC_2A_n66A DC_(n)66AA2 |
| DC_2A-2A-(n)66AA | DC_2A_n66A DC_(n)66AA2 |
| DC_2A-66A-66A_n66A DC_2A-66B_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A-(n)66AA | DC_2A_n66A DC_(n)66AA2 DC_66A_n66A2 |
| DC_2A-2A-66A-(n)66AA | DC_2A_n66A DC_(n)66AA2 DC_66A_n66A2 |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-2A-66A-66A_n66A | DC_2A_n66A |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-66A_n77A14 DC_2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A_n77(2A)14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A_n77A14 DC_2A-2A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A_n77(2A) 14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77A14 DC_2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-66A-66A_n77(2A) 14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A-2A-66A-66A_n77A14 DC_2A-2A-66A-66A_n77C14 | DC_2A_n77A14 DC_66A_n77A14 |
| DC_2A_n66A-n77A14 DC_2A_n66A-n77C14 DC_2A-2A_n66A-n77A14 DC_2A-2A_n66A-n77C14 | DC_2A_n77A14 DC_2A_n66A |
| DC_2A-66A_n78A5,14 DC_2A-2A-66A_n78A | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A-66A_n78(2A) 5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A_n66A-n78A DC_2A-2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66A-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A |
| DC_2A-66A-66A_n78A5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A-66A-66A_n78(2A) 5,14 | DC_2A_n78A14 DC_66A_n78A14 |
| DC_2A-71A_n2A | DC_71A_n2A |
| DC_2A-71A_n7A | DC_2A_n7A DC_71A_n7A |
| DC_2A-2A-71A_n7A | DC_2A_n7A DC_71A_n7A |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-2A-71A_n41A | DC_2A_n41A DC_71A_n41A |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-71A_n71A | DC_2A_n71A |
| DC_2A-71A_n77A | DC_2A_n77A DC_71A_n77A |
| DC_2A-2A-71A_n77A | DC_2A_n77A DC_71A_n77A |
| DC_2A-71A_n77(2A) | DC_2A_n77A DC_71A_n77A |
| DC_2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |
| DC_2A_n71A-n77(2A) | DC_2A_n71A DC_2A_n77A |
| DC_2A-2A_n71A-n77A | DC_2A_n71A DC_2A_n77A |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-71A_n78(2A) | DC_71A_n78A DC_2A_n78A |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |
| DC_2A-2A_n71A-n78A | DC_2A_n71A DC_2A_n78A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n5A | DC_3A_n1A DC_3A_n5A |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |
| DC_3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n28A DC_3C_n1A |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A |
| DC_3A_n1A-n75A | DC_3A_n1A |
| DC_3C_n1A-n75A | DC_3C_n1A |
| DC_3A_n1A-n77A5, 14 | DC_3A_n1A DC_3A_n77A14 |
| DC_3A_n1A-n78A5, 14 DC_3C_n1A-n78A5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A14 DC_3C_n78A |
| DC_3A_n1A-n78(2A)5 DC_3C_n1A-n78(2A)5 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n1A-n78A5,14 | DC_3A_n1A DC_3A_n78A14 |
| DC_3A_n1A-n79A5,14 | DC_3A_n1A DC_3A_n79A14 |
| DC_3A_n1A-n105A | DC_3A_n1A DC_3A_n105A |
| DC_(n)3AA-n7A | DC_(n)3AA2 DC_3A_n7A |
| DC_3A_n3A-n7A | DC_3A_n3A2 DC_3A_n7A |
| DC_(n)3AA-n8A | DC_(n)3AA2 DC_3A_n8A |
| DC_(n)3AA-n28A | DC_(n)3AA2 DC_3A_n28A |
| DC_3A_n3A-n28A | DC_3A_n3A2 DC_3A_n28A |
| DC_3A_n3A-n41A | DC_3A_n41A DC_3A_n3A2 |
| DC_(n)3AA-n67A | DC_(n)3AA2 |
| DC_3A_n3A-n67A | DC_3A_n3A2 |
| DC_3A_n3A-n77A5 | DC_3A_n77A DC_3A_n3A2 |
| DC_(n)3AA-n77A | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n77(2A) | DC_(n)3AA2 DC_3A_n77A |
| DC_(n)3AA-n78A | DC_(n)3AA1 DC_3A_n78A |
| DC_(n)3AA-n78(2A) | DC_(n)3AA1 DC_3A_n78A |
| DC_3A_n3A-n78A5 | DC_3A_n78A DC_3A_n3A2 |
| DC_3A-5A_n28A | DC_3A_n28A DC_5A_n28A |
| DC_3A-5A_n40A | DC_3A_n40A DC_5A_n40A |
| DC_3A_n5A-n40A | DC_3A_n5A DC_3A_n40A |
| DC_3A-5A_n77A | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n77(2A) DC_3A-5A_n77(3A) | DC_3A_n77A DC_5A_n77A |
| DC_3A-5A_n78A5 DC_3C-5A_n78A DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(2A)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-5A_n78(A-C)5 | DC_3A_n78A DC_5A_n78A |
| DC_3A_n5A-n78A5, 14 DC_3C_n5A-n78A5, 14 | DC_3A_n5A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A-5A_n79A5 | DC_3A_n79A DC_5A_n79A |
| DC_3A_n5A-n105A | DC_3A_n5A DC_3A_n105A |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |
| DC_3A-3A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A DC_3A-7C_n3A | DC_3A_n3A2 DC_7A_n3A DC_7C_n3A |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |
| DC_3A-(n)7AA DC_3C-(n)7AA | DC_3A_n7A |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-3A-7A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A_n26A DC_3A-7C_n26A DC_3C-7A_n26A DC_3C-7C_n26A | DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_3A-7A-7A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A-7A_n77A5 | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n77(2A) DC_3A-7A_n77(3A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A-7A_n77(2A) DC_3A-7A-7A_n77(3A) | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n78A5,14 DC_3C-7A_n78A5,14 DC_3A-7C_n78A5,14 DC_3C-7C_n78A5,14 DC_3A-7A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_7A_n78A14 DC_7C_n78A14 |
| DC_3A_n7A-n28A DC_3C_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n28A DC_3C_n7A |
| DC_3A-7A_n78(2A)5 DC_3C-7A_n78(2A)5 DC_3A-7C_n78(2A)5 DC_3C-7C_n78(2A)5 | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A |
| DC_3A-7A_n78(A-C)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78A5, 14 DC_3A-7A-7A_n78C5 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A-7A-7A_n78(2A)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-7A-7A_n78(A-C)5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A-7A_n78A5, 14 | DC_3A_n78A14 DC_7A_n78A14 |
| DC_3A_n7A-n78A5 DC_3A_n7B-n78A5 DC_3C_n7A-n78A5 DC_3C_n7B-n78A5 | DC_3A_n7A DC_3C_n7A DC_3A_n78A DC_3C_n78A |
| DC_3A-3A_n7A-n78A5 DC_3A-3A_n7B-n78A5 | DC_3A_n7A DC_3A_n7B DC_3A_n78A |
| DC_3A_n7A-n78(2A)5 DC_3C_n7A-n78(2A)5 | DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_3A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-3A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-7A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-3A-7A-7A_n79A5 | DC_3A_n79A DC_7A_n79A |
| DC_3A-7A_n105A | DC_3A_n105A DC_7A_n105A |
| DC_3A-8A_n1A DC_3A-8B_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A DC_3A_n1A |
| DC_3A-3A-8A_n1A DC_3A-3A-8B_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n7A | DC_3A_n7A DC_8A_n7A |
| DC_3A-3A_n8A-n78A5,14 | DC_3A_n8A DC_3A_n78A14 |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A |
| DC_3A-8A_n41A | DC_3A_n41A DC_8A_n41A |
| DC_3A_n8A-n41A | DC_3A_n41A DC_3A_n8A |
| DC_3A-8A_n28A DC_3C-8A_n28A | DC_3A_n28A DC_3C_n28A DC_8A_n28A |
| DC_3A-8A_n40A | DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n77A5,14 DC_3C-8A_n77A5,14 | DC_3A_n77A14 DC_3C_n77A DC_8A_n77A14 |
| DC_3A-8B_n77A5 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77(2A) 5, 14 DC_3C-8A_n77(2A) 5,14 | DC_3A_n77A14 DC_3C_n77A DC_8A_n77A14 |
| DC_3A-8A_n77(3A) 5 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A5, 14 DC_3C-8A_n78A5,14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8A_n78(2A) 5,14DC_3C-8A_n78(2A)5,14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-3A-8A_n78A5, 14 | DC_3A_n78A14 DC_8A_n78A14 |
| DC_3A-8B_n78A5,14 | DC_3A_n78A14 DC_8A_n78A14 DC_8B_n78A14 |
| DC_3A-3A-8B_n78A5,14 | DC_3A_n78A14 DC_8A_n78A14 DC_8B_n78A14 |
| DC_3A-8A_n79A5,14 DC_3A-8A_n79C5 | DC_3A_n79A14 DC_8A_n79A14 |
| DC_3A_n8A-n77A5 | DC_3A_n8A DC_3A_n77A |
| DC_3A_n8A-n77(2A)5 DC_3A_n8A-n77A | DC_3A_n8A DC_3A_n77A |
| DC_3A_n8A-n78A5,14 | DC_3A_n8A DC_3A_n78A14 |
| DC_3A-11A_n28A | DC_3A_n28A DC_11A_n28A |
| DC_3A-11A_n77A5,14 | DC_3A_n77A14 DC_11A_n77A |
| DC_3A-11A_n77(2A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n77(3A) 5 | DC_3A_n77A DC_11A_n77A |
| DC_3A-11A_n79A | DC_3A_n79A |
| DC_3A-18A_n3A | DC_3A_n3A2 DC_18A_n3A |
| DC_3A-18A_n28A | DC_3A_n28A DC_18A_n28A |
| DC_3A-18A_n41A | DC_3A_n41A DC_18A_n41A |
| DC_3A-18A_n77A14 | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n77(2A) | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n78(2A) | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A | DC_3A_n1A DC_19A_n1A |
| DC_3A-19A_n77A5,14 DC_3A-19A_n77C5 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n77(2A)5,14 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n78A5,14 DC_3A-19A_n78C5 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n78(2A)5,14 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n79A5,14 DC_3A-19A_n79C5 | DC_3A_n79A14 DC_19A_n79A14 |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-3A-20A_n1A | DC_3A_n1A DC_20A_n1A |
| DC_3A-20A_n3A | DC_3A_n3A2 DC_20A_n3A |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |
| DC_3A-20A_n28A5,6,16,20 DC_3C-20A_n28A5,6,16,20 | DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_3A_n20A-n67A DC_3C_n20A-n67A | DC_3A_n20A |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 DC_3A-20A_n78C5 | DC_3A_n78A DC_3C_n78A DC_20A_n78A |
| DC_3A-3A-20A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n78(2A)5 | DC_3A_n78A DC_20A_n78A |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A |
| DC_3A-21A_n1A10,11 | DC_3A_n1A DC_21A_n1A |
| DC_3A-21A_n28A13 | DC_3A_n28A DC_21A_n28A |
| DC_3A-21A_n77A5, 14 DC_3A-21A_n77C5, 14 | DC_3A_n77A14 DC_21A_n77A14 |
| DC_3A-21A_n77(2A)5,14 | DC_3A_n77A14 DC_21A_n77A14 |
| DC_3A-21A_n78A5,14 DC_3A-21A_n78C5 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n78(2A)5,14 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n79A5,14 DC_3A-21A_n79C5 | DC_3A_n79A14 DC_21A_n79A14 |
| DC_3A-26A_n78A DC_3C-26A_n78A | DC_3A_n78A DC_26A_n78A |
| DC_3A-26A_n78(2A) DC_3C-26A_n78(2A) | DC_3A_n78A DC_26A_n78A |
| DC_3A_n26A-n78A | DC_3A_n26A DC_3A_n78A |
| DC_3C_n26A-n78A | DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n1A DC_3C-28A_n1A | DC_3A_n1A DC_3C_n1A DC_28A_n1A |
| DC_3A-28A_n3A | DC_3A_n3A2 DC_28A_n3A |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_28A_n5A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A-28A_n38A | DC_3A_n38A DC_28A_n38A |
| DC_3A_n28A-n38A | DC_3A_n28A DC_3A_n38A |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A |
| DC_3A_n28A-n41A5 | DC_3A_n28A DC_3A_n41A |
| DC_3A-28A_n41A5,14 | DC_3A_n41A14 DC_28A_n41A14 |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | DC_3A_n28A DC_3C_n28A |
| DC_3A-28A_n77A5, 14 DC_3A-28A_n77C5 | DC_3A_n77A14 DC_28A_n77A14 |
| DC_3A-28A_n77(2A)5 | DC_3A_n77A DC_28A_n77A |
| DC_3A_n28A-n77A5,14 | DC_3A_n28A DC_3A_n77A14 |
| DC_3A_n28A-n77(2A)5 | DC_3A_n28A DC_3A_n77A |
| DC_3A-28A_n78A5,14 DC_3C-28A_n78A5,14 DC_3A-28A_n78(2A)5,14 DC_3A-28A_n78C5 | DC_3A_n78A14 DC_3C_n78A14 DC_28A_n78A14 |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n78(2A)5 | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5, 14 DC_3C_n28A-n78A5, 14 | DC_3A_n28A DC_3C_n28A DC_3A_n78A14 DC_3C_n78A14 |
| DC_3A_n28A-n78(2A)5 DC_3C_n28A-n78(2A)5 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A |
| DC_3A-28A_n79A5 DC_3A-28A_n79C5 | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n79A5, 14 | DC_3A_n28A DC_3A_n79A14 |
| DC_3A-32A_n1A DC_3C-32A_n1A | DC_3A_n1A DC_3C_n1A |
| DC_3A-32A_n7A | DC_3A_n7A |
| DC_3A-32A_n28A DC_3C-32A_n28A | DC_3A_n28A DC_3C_n28A |
| DC_3A-32A_n78A DC_3C-32A_n78A DC_3A-32A_n78C | DC_3A_n78A DC_3C_n78A |
| DC_3A-32A_n78(2A) | DC_3A_n78A |
| DC_3A-38A_n28A DC_3C-38A_n28A | DC_3A_n28A DC_3C_n28A DC_38A_n28A |
| DC_3A_n38A-n40A25 | DC_3A_n38A DC_3A_n40A |
| DC_3A-38A_n78A | DC_3A_n78A DC_38A_n78A |
| DC_3A-38A_n78(2A) | DC_3A_n78A |
| DC_3A_n38A-n78A | DC_3A_n38A DC_3A_n78A |
| DC_3C-38A_n78A DC_3C-38A_n78(2A) | DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_3A-40A_n1A DC_3A-40C_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A_n40A-n41A DC_3A_n40A-n41C | DC_3A_n40A DC_3A_n41A |
| DC_3A-40A_n77A DC_3A-40C_n77A | DC_3A_n77A DC_40A_n77A |
| DC_3A_n40A-n77A | DC_3A_n40A DC_3A_n77A |
| DC_3A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A |
| DC_3A-40A_n78A DC_3A-40C_n78A | DC_3A_n78A DC_40A_n78A |
| DC_3A-40A_n78(2A) DC_3A-40C_n78(2A) | DC_3A_n78A DC_40A_n78A |
| DC_3A_n40A-n78A DC_3A_n40A-n78C | DC_3A_n40A DC_3A_n78A |
| DC_3A_n40A-n79A DC_3A_n40A-n79C | DC_3A_n40A DC_3A_n79A |
| DC_3A_n40A-n105A | DC_3A_n40A DC_3A_n105A |
| DC_3A-41A_n1A | DC_3A_n1A DC_41A_n1A |
| DC_3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-41A_n1A | DC_3A_n1A DC_41A_n1A |
| DC_3A-3A-41C_n1A | DC_3A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-41A_n3A DC_3A-41C_n3A | DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| DC_3A-41A_n28A5 | DC_3A_n28A DC_41A_n28A |
| DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A DC_41A_n41A |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A DC_(n)41AA |
| DC_3A-41A_n77A14 DC_3A-41C_n77A14 | DC_3A_n77A14 DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n77(2A) 14 DC_3A-41C_n77(2A) 14 | DC_3A_n77A14 DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-41A_n78A DC_3A-3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A_n41A-n78A | DC_3A_n41A DC_3A_n78A |
| DC_3A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-42A_n1A5 DC_3A-42C_n1A5 | DC_3A_n1A DC_42A_n1A |
| DC_3A-42A_n28A5 | DC_3A_n28A DC_42A_n28A |
| DC_3A-42C_n28A5 | DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_3A-41A_n79A5 DC_3A-41C_n79A5 | DC_3A_n79A DC_41A_n79A |
| DC_3A_n41A-n77A14 | DC_3A_n41A14 DC_3A_n77A14 |
| DC_3A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A |
| DC_3A_n41A-n79A5 DC_3A_n41C-n79A5 DC_3A_n41A-n79C5 DC_3A_n41C-n79C5 | DC_3A_n41A DC_3A_n79A |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |
| DC_3A-42A_n77A14, 15,16 DC_3A-42A_n77C15,16 DC_3A-42C_n77A14, 15,16 DC_3A-42C_n77C15,16 DC_3A-42D_n77A14, 15,16 DC_3A-42D_n77C15,16 DC_3A-42E_n77A14, 15,16 DC_3A-42E_n77C15,16 | DC_3A_n77A14, |
| DC_3A-42A_n77(2A)15,16 DC_3A-42C_n77(2A)15,16 | DC_3A_n77A |
| DC_3A-42A_n78A14,15,16 DC_3A-42A_n78C15,16 DC_3A-42C_n78A14,15,16 DC_3A-42C_n78C15,16 DC_3A-42D_n78A14,15,16 DC_3A-42D_n78C15,16 DC_3A-42E_n78A14,15,16 DC_3A-42E_n78C15,16 | DC_3A_n78A14 |
| DC_3A-42A_n79A14 DC_3A-42A_n79C DC_3A-42C_n79A14 DC_3A-42C_n79C DC_3A-42D_n79A14 DC_3A-42D_n79C DC_3A-42E_n79A14 DC_3A-42E_n79C | DC_3A_n79A14 |
| DC_3A-67A_n3A | DC_3A_n3A2 |
| DC_3A_n75A-n78A DC_3C_n75A-n78A | DC_3A_n78A DC_3C_n78A |
| DC_3A_n75A-n78(2A) | DC_3A_n78A |
| DC_3A_n77A-n79A14, 23 | DC_3A_n77A14 DC_3A_n79A14 |
| DC_3A_n78A-n79A14, 24 DC_3A_n78A-n79C24 | DC_3A_n78A14 DC_3A_n79A14 |
| DC_3A-3A_n78A-n79A24 | DC_3A_n78A DC_3A_n79A |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |
| DC_3A_SUL_n78A-n80A5 DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |
| DC_3A_n78A-n105A | DC_3A_n78A DC_3A_n105A |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A |
| DC_4A-5A_n78A | DC_4A_n78A DC_5A_n78A |
| DC_4A-7A_n28A | DC_4A_n28A DC_7A_n28A |
| DC_4A-7A_n78A DC_4A-7C_n78A | DC_4A_n78A DC_7A_n78A DC_7C_n78A |
| DC_5A_n1A-n28A | DC_5A_n1A DC_5A_n28A |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A |
| DC_5A_n2A-n41A | DC_5A_n2A DC_5A_n41A |
| DC_5A_n2A-n66A | DC_5A_n2A DC_5A_n66A |
| DC_5A_n2A-n77A14 DC_5A_n2A-n77C14 | DC_5A_n77A14 DC_5A_n2A |
| DC_5A_n2A-n78A | DC_5A_n2A DC_5A_n78A |
| DC_5A_n3A-n28A | DC_5A_n3A DC_5A_n28A |
| DC_5A_n3A-n78A | DC_5A_n3A DC_5A_n78A |
| DC_5A_n5A-n77A14 DC_5A_n5A-n77C14 | DC_5A_n77A14 |
| DC_5A-7A_n1A | DC_5A_n1A DC_7A_n1A |
| DC_5A-7A_n2A | DC_7A_n2A |
| DC_5A-7A_n2(2A) | DC_7A_n2A |
| DC_5A-7A_n7A | DC_5A_n7A DC_7A_n7A2 |
| DC_5A-7A_n25A | DC_5A_n25A DC_7A_n25A |
| DC_5A-7A_n28A | DC_5A_n28A DC_7A_n28A |
| DC_5A-7A_n40A | DC_5A_n40A DC_7A_n40A |
| DC_5A-7A-7A_n40A | DC_5A_n40A DC_7A_n40A |
| DC_5A-7A_n66A DC_5A-7C_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A-7A_n66A | DC_5A_n66A DC_7A_n66A |
| DC_5A-7A_n71A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77A | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n77(2A) DC_5A-7A_n77(3A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A-7A_n77(2A) DC_5A-7A-7A_n77(3A) | DC_5A_n77A DC_7A_n77A |
| DC_5A-7A_n78A DC_5A-7A_n78C DC_5A-7C_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A |
| DC_5A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n78(A-C) | DC_5A_n78A DC_7A_n78A |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7A-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A-7A-7A_n78A DC_5A-7A-7A_n78C | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(2A) | DC_5A_n78A DC_7A_n78A |
| DC_5A-7A-7A_n78(A-C) | DC_5A_n78A DC_7A_n78A |
| DC_5A-(n)12AA | DC_5A_n12A DC_(n)12AA2 |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |
| DC_5A-13A_n66A | DC_5A_n66A DC_13A_n66A |
| DC_5A-13A_n77A DC_5A-13A_n77C | DC_5A_n77A DC_13A_n77A |
| DC_5A_n28A-n77A DC_5A_n28A-n77C | DC_5A_n77A |
| DC_5A_n28A-n78A DC_5A_n28A-n78C | DC_5A_n78A DC_5A_n28A |
| DC_5A_n28A-n79A | DC_5A_n28A DC_5A_n79A |
| DC_5A-30A_n2A | DC_5A_n2A DC_30A_n2A |
| DC_5A-30A_n5A | DC_30A_n5A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_5A-30A_n77A14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A-30A_n77(2A) 14 | DC_5A_n77A14 DC_30A_n77A14 |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_n66A |
| DC_5A-40A_n77A DC_5A-40C_n77A DC_5A-40A_n77C DC_5A-40C_n77C | DC_5A_n77A DC_40A_n77A |
| DC_5A_n40A-n77A | DC_5A_n40A DC_5A_n77A |
| DC_5A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A |
| DC_5A-40A_n78A DC_5A-40C_n78A DC_5A-40A_n78C DC_5A-40C_n78C | DC_5A_n78A DC_40A_n78A |
| DC_5A_n40A-n78A DC_5A_n40A-n78C | DC_5A_n40A DC_5A_n78A |
| DC_5A_n41A-n66A | DC_5A_n41A DC_5A_n66A |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |
| DC_5A-46A_n66A | DC_5A_n66A DC_46A_n66A |
| DC_5A-48A_n5A | DC_48A_n5A |
| DC_5A-48A_n12A | DC_5A_n12A DC_48A_n12A |
| DC_5A-48A_n71A | DC_5A_n71A DC_48A_n71A |
| DC_5A-48A_n77A14,15,16 DC_5A-48C_n77A14,15,16 DC_5A-48D_n77A14,15,16 DC_5A-48A_n77C14,15,16 DC_5A-48C_n77C14,15,16 DC_5A-48D_n77C14,15,16 | DC_5A_n77A14 |
| DC_5A-66A_n2A DC_5B-66A_n2A DC_5A-66B_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-5A-66A-66A_n2A | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A_n2(2A) | DC_5A_n2A DC_66A_n2A |
| DC_5A-66A_n5A | DC_66A_n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A |
| DC_5A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A-66A_n7A | DC_5A_n7A DC_66A_n7A |
| DC_5A-66A_n12A | DC_5A_n12A DC_66A_n12A |
| DC_5A-66A_n25A | DC_5A_n25A DC_66A_n25A |
| DC_5A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A-66A_n30A | DC_5A_n30A DC_66A_n30A |
| DC_5A-66A_n41A | DC_5A_n41A DC_66A_n41A |
| DC_5A-66A_n48A DC_5A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A-66A_n48A DC_5A-66A-66A_n48B | DC_5A_n48A DC_66A_n48A |
| DC_5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |
| DC_5A-(n)66AA | DC_5A_n66A DC_(n)66AA2 |
| DC_5A-5A-66A_n66A | DC_5A_n66A |
| DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A-(n)66AA | DC_5A_n66A DC_(n)66AA2 DC_66A_n66A2 |
| DC_5A-5A-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |
| DC_5A-66A_n77A14 DC_5A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A_n77(2A) 14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A-66A_n77A14 DC_5A-66A-66A_n77C14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A-66A-66A_n77(2A) 14 | DC_5A_n77A14 DC_66A_n77A14 |
| DC_5A_n66A-n77A14 DC_5A_n66A-n77C14 | DC_5A_n66A DC_5A_n77A14 |
| DC_5A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_n78A |
| DC_5A-66A-66A_n78A | DC_5A_n78A DC_66A_n78A |
| DC_7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A |
| DC_7A_n1A-n28A | DC_7A_n1A DC_7A_n28A |
| DC_7C_n1A-n28A | DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A |
| DC_7A_n1A-n75A | DC_7A_n1A |
| DC_7A_n1A-n78A5, 14 DC_7C_n1A-n78A5 | DC_7A_n1A DC_7A_n78A14 DC_7C_n1A DC_7C_n78A |
| DC_7A_n1A-n78(2A)5 DC_7C_n1A-n78(2A)5 | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_7A-7A_n1A-n78A5, 14 | DC_7A_n1A DC_7A_n78A14 |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_n66A |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_n71A |
| DC_7A_n2A-n77A | DC_7A_n2A DC_7A_n77A |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_n78A |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n3A-n78(2A) DC_7C_n3A-n78(2A) | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n5A-n40A | DC_7A_n5A DC_7A_n40A |
| DC_7A_n5A-n78A14 DC_7C_n5A-n78A14 | DC_7A_n5A DC_7C_n5A DC_7A_n78A14 DC_7C_n78A14 |
| DC_7A_n7A-n78A5 | DC_7A_n78A DC_7A_n7A2 |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |
| DC_7A-8A_n1A DC_7A-8B_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-7A-8A_n1A DC_7A-7A-8B_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-8A_n7A | DC_7A_n7A DC_8A_n7A |
| DC_7A-8A_n20A | DC_7A_n20A DC_8A_n20A |
| DC_7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-7A-8A_n28A | DC_7A_n28A DC_8A_n28A |
| DC_7A-8A_n40A | DC_7A_n40A DC_8A_n40A |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A |
| DC_7A-8A_n77A5 | DC_7A_n77A DC_8A_n77A |
| DC_7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-8A_n78(2A) | DC_7A_n78A DC_8A_n78A |
| DC_7A-7A-8A_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-7A-8A_n78(2A)5, 14 | DC_7A_n78A14 DC_8A_n78A14 |
| DC_7A-7A_n8A-n78A5, 14 | DC_7A_n8A DC_7A_n78A14 |
| DC_7A-8B_n78A5, 14 DC_7A-7A-8B_n78A5, 14 | DC_7A_n78A14 DC_8A_n78A14 DC_8B_n78A |
| DC_7A_n8A-n78A5, 14 | DC_7A_n8A DC_7A_n78A14 |
| DC_7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n2(2A) | DC_7A_n2A DC_12A_n2A |
| DC_7A-12A_n25A | DC_7A_n25A DC_12A_n25A |
| DC_7A-12A_n66A | DC_7A_n66A DC_12A_n66A |
| DC_7A-12A_n77A | DC_7A_n77A DC_12A_n77A |
| DC_7A-12A_n77(2A) | DC_7A_n77A DC_12A_n77A |
| DC_7A_n12A-n77A | DC_7A_n12A DC_7A_n77A |
| DC_7A-12A_n78A | DC_7A_n78A DC_12A_n78A |
| DC_7A-12A_n78(2A) | DC_7A_n78A DC_12A_n78A |
| DC_7A_n12A-n78A | DC_7A_n12A DC_7A_n78A |
| DC_7A-13A_n25A DC_7C-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-7A-13A_n25A | DC_7A_n25A DC_13A_n25A |
| DC_7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-7A-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,16,20 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 DC_7A-20A_n78C5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n78(2A)5 | DC_7A_n78A DC_20A_n78A |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A |
| DC_7A_n25A-n71A | DC_7A_n25A DC_7A_n71A |
| DC_7A-25A_n77A DC_7C-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A-25A_n77A DC_7C-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-7A-25A-25A_n77A | DC_7A_n77A DC_25A_n77A |
| DC_7A-25A_n78A DC_7C-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-25A-25A_n78A DC_7C-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-7A-25A-25A_n78A | DC_7A_n78A DC_25A_n78A |
| DC_7A-26A_n78A DC_7C-26A_n78A | DC_7A_n78A DC_26A_n78A |
| DC_7A-26A_n78(2A) DC_7C-26A_n78(2A) | DC_7A_n78A DC_26A_n78A |
| DC_7A_n26A-n78A DC_7A_n26A-n78(2A) | DC_7A_n26A DC_7A_n78A |
| DC_7C_n26A-n78A DC_7C_n26A-n78(2A) | DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-7A-28A_n1A | DC_28A_n1A DC_7A_n1A |
| DC_7A-28A_n2A | DC_7A_n2A DC_28A_n2A |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |
| DC_7A-28A_n20A | DC_7A_n20A DC_28A_n20A |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |
| DC_7A-28A_n66A DC_7C-28A_n66A | DC_7A_n66A DC_28A_n66A |
| DC_7A-28A_n78A5,14 DC_7C-28A_n78A5,14 DC_7A-28A_n78(2A)5,14 DC_7C-28A_n78(2A)5,14 | DC_7A_n78A14 DC_7C_n78A14 DC_28A_n78A14 |
| DC_7A_n28A-n78A5,14 DC_7C_n28A-n78A14 | DC_7A_n28A DC_7A_n78A14 DC_7C_n28A DC_7C_n78A14 |
| DC_7A-29A_n78A DC_7C-29A_n78A | DC_7A_n78A |
| DC_7A-7A-29A_n78A | DC_7A_n78A |
| DC_7A-32A_n1A | DC_7A_n1A |
| DC_7A-32A_n3A DC_7C-32A_n3A | DC_7A_n3A |
| DC_7A-32A_n8A | DC_7A_n8A |
| DC_7A-32A_n28A | DC_7A_n28A |
| DC_7A-32A_n78A | DC_7A_n78A |
| DC_7A-40A_n1A DC_7A-40C_n1A | DC_7A_n1A DC_40A_n1A |
| DC_7A_n40A-n77A | DC_7A_n40A DC_7A_n77A |
| DC_7A_n40A-n77(2A) | DC_7A_n40A DC_7A_n77A |
| DC_7A-7A_n40A-n77A | DC_7A_n40A DC_7A_n77A |
| DC_7A-7A_n40A-n77(2A) | DC_7A_n40A DC_7A_n77A |
| DC_7A-40A_n78A DC_7A-40C_n78A | DC_7A_n78A DC_40A_n78A |
| DC_7A-40A_n78(2A) DC_7A-40C_n78(2A) | DC_7A_n78A DC_40A_n78A |
| DC_7A_n40A-n78A DC_7A_n40A-n78C | DC_7A_n40A DC_7A_n78A |
| DC_7A-7A_n40A-n78A DC_7A-7A_n40A-n78C | DC_7A_n40A DC_7A_n78A |
| DC_7A_n40A-n105A | DC_7A_n40A DC_7A_n105A |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |
| DC_7A-66A_n2A DC_7A-66A_n2(2A) | DC_7A_n2A DC_66A_n2A |
| DC_7A-66A_n5A DC_7C-66A_n5A DC_7A-66A-66A_n5A DC_7C-66A-66A_n5A DC_7A-7A-66A_n5A DC_7A-7A-66A-66A_n5A | DC_7A_n5A DC_66A_n5A |
| DC_7A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A-66A_n7A | DC_7A_n7A2 DC_66A_n7A |
| DC_7A-66A_n12A | DC_7A_n12A DC_66A_n12A |
| DC_7A-66A_n25A DC_7C-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-7A-66A_n25A | DC_7A_n25A DC_66A_n25A |
| DC_7A-66A_n28A | DC_7A_n28A DC_66A_n28A |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-(n)66AA DC_7C-(n)66AA | DC_7A_n66A DC_(n)66AA2 |
| DC_7A-7A-(n)66AA | DC_7A_n66A DC_(n)66AA2 |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A-(n)66AA | DC_7A_n66A DC_(n)66AA2 DC_66A_n66A2 |
| DC_7A-7A-66A-(n)66AA | DC_7A_n66A DC_(n)66AA2 DC_66A_n66A2 |
| DC_7A-7A-66A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_n71A |
| DC_7A-66A_n77A DC_7C-66A_n77A | DC_7A_n77A DC_66A_n77A |
| DC_7A-7A-66A_n77A | DC_7A_n66A DC_66A_n77A |
| DC_7A-7A-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A-66A_n77(2A) DC_7C-66A_n77(2A) | DC_7A_n66A DC_66A_n77A |
| DC_7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A-7A_n66A-n77A | DC_7A_n66A DC_7A_n77A |
| DC_7A_n66A-n78A DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-7A_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-66A_n78A5,14 DC_7C-66A_n78A5,14 | DC_7A_n78A14 DC_7C_n78A DC_66A_n78A14 |
| DC_7A-66A_n78(2A) 5,14 DC_7C-66A_n78(2A) 5,14 | DC_7A_n78A14 DC_7C_n78A DC_66A_n78A14 |
| DC_7A-7A-66A_n78A5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-7A-66A_n78(2A)5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-7A-66A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78A5,14 DC_7C-66A-66A_n78A5,14 | DC_7A_n78A14 DC_7C_n78A DC_66A_n78A14 |
| DC_7A-66A-66A_n78(2A) 5,14 DC_7C-66A-66A_n78(2A) 5,14 | DC_7A_n78A14 DC_66A_n78A14 |
| DC_7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n2(2A) | DC_7A_n2A DC_71A_n2A |
| DC_7A-71A_n12A | DC_7A_n12A |
| DC_7A-71A_n25A | DC_7A_n25A DC_71A_n25A |
| DC_7A-71A_n66A | DC_7A_n66A DC_71A_n66A |
| DC_7A-71A_n77A | DC_7A_n77A DC_71A_n77A |
| DC_7A-71A_n77(2A) | DC_7A_n77A DC_71A_n77A |
| DC_7A_n71A-n77A | DC_7A_n71A DC_7A_n77A |
| DC_7A-71A_n78A | DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n78(2A) | DC_7A_n78A DC_71A_n78A |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_n78A |
| DC_7A_n75A-n78A | DC_7A_n78A |
| DC_7A_n78A-n79A24 DC_7A_n78A-n79C24 | DC_7A_n78A DC_7A_n79A |
| DC_7A-7A_n78A-n79A24 | DC_7A_n78A DC_7A_n79A |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |
| DC_7A_n78A-n105A | DC_7A_n78A DC_7A_n105A |
| DC_8A_n1A-n3A DC_8B_n1A-n3A | DC_8A_n1A DC_8A_n3A |
| DC_8A_n1A-n28A | DC_8A_n1A DC_8A_n28A |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A |
| DC_8A_n1A-n77A5,14 DC_8B_n1A-n77A5 | DC_8A_n1A DC_8A_n77A14 |
| DC_8A_n1A-n77(2A)5 | DC_8A_n1A DC_8A_n77A |
| DC_8A_n1A-n78A5,14 DC_8B_n1A-n78A5 | DC_8A_n1A DC_8B_n1A DC_8A_n78A14 DC_8B_n78A |
| DC_8A_n1A-n79A5,14 | DC_8A_n79A14 |
| DC_8A-(n)3AA | DC_(n)3AA DC_8A_n3A |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |
| DC_8A_n3A-n77A5,14 | DC_8A_n3A DC_8A_n77A14 |
| DC_8B_n3A-n77A5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n77(2A) 5 | DC_8A_n3A DC_8A_n77A |
| DC_8A_n3A-n78A | DC_8A_n3A DC_8A_n78A |
| DC_8A_n3A-n79A5,14 | DC_8A_n3A DC_8A_n79A14 |
| DC_8A_n7A-n78A | DC_8A_n7A DC_8A_n78A |
| DC_8A-11A_n1A DC_8B-11A_n1A | DC_8A_n1A DC_11A_n1A |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8B-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8A-11A_n28A | DC_8A_n28A DC_11A_n28A |
| DC_8A-11A_n77A5 DC_8B-11A_n77A5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(2A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8B-11A_n77(2A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(3A)5 | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n78A5 | DC_8A_n78A DC_11A_n78A |
| DC_8A-11A_n79A5,14 | DC_8A_n79A14 DC_11A_n79A |
| DC_8A-20A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A-20A_n28A6,16,19,20 | DC_8A_n28A DC_20A_n28A |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |
| DC_8A-28A_n3A | DC_8A_n3A DC_28A_n3A |
| DC_8A-28A_n78A | DC_8A_n78A DC_28A_n78A |
| DC_8A_n28A-n77A5,14 | DC_8A_n28A DC_8A_n77A14 |
| DC_8A_n28A-n77(2A)5 | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n78A5,14 | DC_8A_n28A DC_8A_n78A14 |
| DC_8A_n28A-n79A5,14 | DC_8A_n28A DC_8A_n79A14 |
| DC_8A-32A_n1A | DC_8A_n1A |
| DC_8A-32A_n3A | DC_8A_n3A |
| DC_8A-32A_n28A | DC_8A_n28A |
| DC_8A-32A_n78A | DC_8A_n78A |
| DC_8A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |
| DC_8A-39A_n40A | DC_8A_n40A DC_39A_n40A |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A |
| DC_8A-39A_n41A DC_8A-39A_n41C | DC_8A_n41A DC_39A_n41A |
| DC_8A_n39A-n41A | DC_8A_n39A DC_8A_n41A |
| DC_8A-39A_n79A DC_8A-39A_n79C | DC_8A_n79A DC_39A_n79A |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A |
| DC_8A-40A_n1A DC_8A-40C_n1A | DC_8A_n1A DC_40A_n1A |
| DC_8A_n40A-n41A DC_8A_n40A-n41C | DC_8A_n40A DC_8A_n41A |
| DC_8A-40A_n78A DC_8A-40C_n78A | DC_8A_n78A DC_40A_n78A |
| DC_8A-40A_n78(2A) DC_8A-40C_n78(2A) | DC_8A_n78A DC_40A_n78A |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A |
| DC_8A_n40A-n79A DC_8A_n40A-n79C | DC_8A_n40A DC_8A_n79A |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |
| DC_8A-41A_n3A5 DC_8A-41C_n3A5 | DC_8A_n3A DC_41A_n3A DC_41C_n3A |
| DC_8A-41A_n77A DC_8A-41C_n77A | DC_8A_n77A DC_41A_n77A DC_41C_n77A |
| DC_8A-41A_n78A | DC_8A_n78A DC_41A_n78A |
| DC_8A-41C_n78A | DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_8A_n41A-n79A5 DC_8A_n41A-n79C5 DC_8A_n41C-n79A5 DC_8A_n41C-n79C5 | DC_8A_n41A DC_8A_n79A |
| DC_8A-42A_n1A5 DC_8A-42C_n1A5 | DC_8A_n1A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A5 | DC_8A_n3A DC_42A_n3A |
| DC_8A-42C_n3A5 | DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n28A5 | DC_8A_n28A DC_42A_n28A |
| DC_8A-42C_n28A5 | DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n77A14,15,16 DC_8A-42C_n77A15,16 | DC_8A_n77A14 |
| DC_8A-42A_n77(2A) 15,16 DC_8A-42C_n77(2A) 15,16 | DC_8A_n77A |
| DC_8A-42A_n79A | DC_8A_n79A |
| DC_8A_SUL_n41A-n81A | DC_8A_n41A DC_8A_n81A_ULSUP-TDM_n41A |
| DC_8A_n77A-n79A14,23 | DC_8A_n77A14 DC_8A_n79A14 |
| DC_8A_n77(2A)-n79A23 | DC_8A_n77A DC_8A_n79A |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A |
| DC_11A_n1A-n77A5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n1A-n77(2A)5 | DC_11A_n1A DC_11A_n77A |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A |
| DC_11A_n3A-n79A5 | DC_11A_n3A DC_11A_n79A |
| DC_11A-18A_n3A | DC_11A_n3A DC_18A_n3A |
| DC_11A-18A_n28A | DC_11A_n28A |
| DC_11A-18A_n41A | DC_11A_n41A DC_18A_n41A |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n77(2A) | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |
| DC_11A-18A_n78(2A) | DC_11A_n78A DC_18A_n78A |
| DC_11A_n28A-n77A5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n28A-n77(2A) 5 | DC_11A_n28A DC_11A_n77A |
| DC_11A_n77A-n79A23 | DC_11A_n77A DC_11A_n79A |
| DC_11A_n77(2A)-n79A23 | DC_11A_n77A DC_11A_n79A |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_n38A |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_n41A |
| DC_12A_n2A-n66A | DC_12A_n2A DC_12A_n66A |
| DC_12A_n2A-n77A | DC_12A_n2A DC_12A_n77A |
| DC_12A_n2A-n78A | DC_12A_n2A DC_12A_n78A |
| DC_12A-(n)5AA | DC_12A_n5A DC_(n)5AA2 |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7A-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A_n25A-n41A | DC_12A_n25A DC_12A_n41A |
| DC_12A_n25A-n66A | DC_12A_n25A DC_12A_n66A |
| DC_12A_n25A-n77A | DC_12A_n25A DC_12A_n77A |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_12A-30A_n5A | DC_12A_n5A DC_30A_n5A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_12A-30A_n77A14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A-30A_n77(2A) 14 | DC_12A_n77A14 DC_30A_n77A14 |
| DC_12A_n41A-n66A | DC_12A_n41A DC_12A_n66A |
| DC_12A-48A_n5A | DC_12A_n5A DC_48A_n5A |
| DC_12A-48A_n12A | DC_48A_n12A |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n2(2A) | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n7A | DC_12A_n7A DC_66A_n7A |
| DC_12A-66A-66A_n5A | DC_12A_n5A DC_66A_n5A |
| DC_12A-66A_n12A | DC_66A_n12A |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |
| DC_12A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A-66A_n30A | DC_12A_n30A DC_66A_n30A |
| DC_12A-66A_n41A | DC_12A_n41A DC_66A_n41A |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |
| DC_12A-(n)66AA | DC_12A_n66A DC_(n)66AA2 |
| DC_12A-66A_n77A14 DC_12A-66A-66A_n77A14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A-66A_n77(2A) 14 DC_12A-66A-66A_n77(2A) 14 | DC_12A_n77A14 DC_66A_n77A14 |
| DC_12A_n66A-n77A | DC_12A_n66A DC_12A_n77A |
| DC_12A-66A_n78A | DC_12A_n78A DC_66A_n78A |
| DC_12A-66A_n78(2A) | DC_12A_n78A DC_66A_n78A |
| DC_12A_n66A-n78A | DC_12A_n66A DC_12A_n78A |
| DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |
| DC_12A-71A_n2A | DC_12A_n2A DC_71A_n2A |
| DC_12A-71A_n77A | DC_12A_n77A DC_71A_n77A |
| DC_13A_n2A-n77A14 DC_13A_n2A-n77C14 | DC_13A_n2A DC_13A_n77A14 |
| DC_13A_n5A-n48A | DC_13A_n48A |
| DC_13A_n5A-n77A14 DC_13A_n5A-n77C14 | DC_13A_n77A14 |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A |
| DC_13A-46A_n2A3 | DC_13A_n2A |
| DC_13A-46A_n5A | DC_13A_n5A |
| DC_13A-46A_n66A3 | DC_13A_n66A |
| DC_13A-46A_n77A DC_13A-46A-46A_n77A | DC_13A_n77A |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_n66A |
| DC_13A-66A_n2A DC_13A-66B_n2A DC_13A-66C_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n5A DC_13A-66A-66A_n5A | DC_13A_n5A DC_66A_n5A |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A_n66A DC_13A-66B_n66A | DC_13A_n66A |
| DC_13A-(n)66AA | DC_13A_n66A DC_(n)66AA2 |
| DC_13A-66A-66A_n66A | DC_13A_n66A |
| DC_13A-66A-(n)66AA | DC_13A_n66A DC_(n)66AA2 DC_66A_n66A2 |
| DC_13A-66A_n77A14 DC_13A-66A_n77C14 DC_13A-66A-66A_n77C14 | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A-66A-66A_n77A | DC_13A_n77A14 DC_66A_n77A14 |
| DC_13A_n66A-n77A14 DC_13A_n66A-n77C14 | DC_13A_n66A DC_13A_n77A14 |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48C_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48C_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |
| DC_13A-48A_n77A14,15,16 DC_13A-48A_n77C14,15,16 DC_13A-48C_n77A14,15,16 DC_13A-48C_n77C14,15,16 DC_13A-48D_n77A14,15,16 DC_13A-48D_n77C14,15,16 DC_13A-48A-48A_n77A14,15,16 | DC_13A_n77A14 |
| DC_14A-30A_n2A | DC_14A_n2A DC_30A_n2A |
| DC_14A-30A_n5A | DC_14A_n5A DC_30A_n5A |
| DC_14A-30A_n66A | DC_14A_n66A DC_30A_n66A |
| DC_14A-30A_n77A14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-30A_n77(2A) 14 | DC_14A_n77A14 DC_30A_n77A14 |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A-66A_n5A | DC_14A_n5A DC_66A_n5A |
| DC_14A-66A_n30A DC_14A-66A-66A_n30A | DC_14A_n30A DC_66A_n30A |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_14A-66A_n77A14 DC_14A-66A-66A_n77A14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_14A-66A_n77(2A) 14 DC_14A-66A-66A_n77(2A) 14 | DC_14A_n77A14 DC_66A_n77A14 |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |
| DC_18A_n28A-n77A5,14 | DC_18A_n28A DC_18A_n77A14 |
| DC_18A_n28A-n77(2A)5 | DC_18A_n28A DC_18A_n77A |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |
| DC_18A_n28A-n78A5 | DC_18A_n28A DC_18A_n78A |
| DC_18A_n28A-n78(2A)5 | DC_18A_n28A DC_18A_n78A |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A |
| DC_18A_n41A-n77(2A) | DC_18A_n41A DC_18A_n77A |
| DC_18A-42A_n77A14,15,16 DC_18A-42C_n77A14,15,16 | DC_18A_n77A14 |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A |
| DC_18A_n41A-n78(2A) | DC_18A_n41A DC_18A_n78A |
| DC_18A-42A_n78A15,16 DC_18A-42C_n78A15,16 | DC_18A_n78A |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |
| DC_19A-21A_n1A | DC_19A_n1A DC_21A_n1A |
| DC_19A_n1A-n77A5 | DC_19A_n1A DC_19A_n77A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5,14 DC_19A-21A_n77C5 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n77(2A)5,14 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n78A5, 14 DC_19A-21A_n78C5 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n78(2A)514 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n79A5,14 DC_19A-21A_n79C5 | DC_19A_n79A14 DC_21A_n79A14 |
| DC_19A-42A_n1A5,10,12 DC_19A-42C_n1A5,10,12 | DC_19A_n1A DC_42A_n1A |
| DC_19A-42A_n77A14,15,16 DC_19A-42A_n77C15,16 DC_19A-42C_n77A14,15,16 DC_19A-42C_n77C15,16 DC_19A-42D_n77A15,16 DC_19A-42D_n77C15,16 | DC_19A_n77A14 |
| DC_19A-42A_n78A14,15,16 DC_19A-42A_n78C15,16 DC_19A-42C_n78A14,15,16 DC_19A-42C_n78C15,16 DC_19A-42D_n78A15,16 DC_19A-42D_n78C15,16 | DC_19A_n78A14 |
| DC_19A-42A_n79A14 DC_19A-42A_n79C DC_19A-42C_n79A14 DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A14 |
| DC_19A_n77A-n79A14,23 | DC_19A_n77A14 DC_19A_n79A14 |
| DC_19A_n78A-n79A14,24 | DC_19A_n78A14 DC_19A_n79A14 |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A |
| DC_20A_n1A-n28A16,20 | DC_20A_n1A DC_20A_n28A |
| DC_20A_n1A-n67A | DC_20A_n1A |
| DC_20A_n1A-n75A | DC_20A_n1A |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |
| DC_20A-(n)3AA | DC_(n)3AA2 DC_20A_n3A |
| DC_20A_n3A-n38A | DC_20A_n3A DC_20A_n38A |
| DC_20A_n3A-n67A | DC_20A_n3A |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_20A_n7A-n28A, 16, 20 | DC_20A_n7A DC_20A_n28A |
| DC_20A_n7A-n78A | DC_20A_n7A DC_20A_n78A |
| DC_20A_n8A-n75A6 | DC_20A_n8A |
| DC_20A_n8A-n78A | DC_20A_n78A DC_20A_n8A |
| DC_20A-28A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A_n78A | DC_20A_n78A DC_28A_n78A |
| DC_20A_n28A-n75A6,16,20 | DC_20A_n28A |
| DC_20A_n28A-n78A5,6,16,20 | DC_20A_n28A DC_20A_n78A |
| DC_20A-32A_n1A | DC_20A_n1A |
| DC_20A-32A_n3A | DC_20A_n3A |
| DC_20A-32A_n8A | DC_20A_n8A |
| DC_20A-32A_n28A16,20 | DC_20A_n28A |
| DC_20A-32A_n7A | DC_20A_n7A |
| DC_20A-32A_n78A DC_20A-32A_n78C | DC_20A_n78A |
| DC_20A-32A_n78(2A) | DC_20A_n78A |
| DC_20A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A | DC_20A_n3A DC_38A_n3A |
| DC_20A-38A_n8A | DC_38A_n8A |
| DC_20A-(n)38AA | DC_20A_n38A |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |
| DC_20A-38A_n78(2A) | DC_20A_n78A |
| DC_20A_n38A-n78A | DC_20A_n38A DC_20A_n78A |
| DC_20A-40A_n1A DC_20A-40C_n1A | DC_20A_n1A DC_40A_n1A |
| DC_20A-40A_n78A DC_20A-40C_n78A | DC_20A_n78A DC_40A_n78A |
| DC_20A-40A_n78(2A) DC_20A-40C_n78(2A) | DC_20A_n78A DC_40A_n78A |
| DC_20A-41A_n1A | DC_20A_n1A DC_41A_n1A |
| DC_20A-41C_n1A | DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_20A-41A_n41A DC_20A-41C_n41A | DC_41A_n41A |
| DC_20A-41A_n78A | DC_20A_n78A DC_41A_n78A |
| DC_20A-41C_n78A | DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_20A_n41A-n78A | DC_20A_n41A DC_20A_n78A |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |
| DC_20A-67A_n3A | DC_20A_n3A |
| DC_20A_n75A-n78A5 | DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |
| DC_20A_n78A-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_21A_n1A-n77A5 | DC_21A_n1A DC_21A_n77A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-28A_n77A5 DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |
| DC_21A_n28A-n77A5,13 | DC_21A_n28A DC_21A_n77A |
| DC_21A-28A_n78A5 DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |
| DC_21A_n28A-n78A5,13 | DC_21A_n28A DC_21A_n78A |
| DC_21A-28A_n79A5 DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n79A5,13 | DC_21A_n28A DC_21A_n79A |
| DC_21A-42A_n1A510,12 DC_21A-42C_n1A510,12 | DC_21A_n1A DC_42A_n1A |
| DC_21A-42A_n77A14, 15,16 DC_21A-42A_n77C15,16 DC_21A-42C_n77A14, 15,16 DC_21A-42C_n77C15,16 DC_21A-42D_n77A15,16 DC_21A-42D_n77C15,16 DC_21A-42E_n77A15,16 DC_21A-42E_n77C15,16 | DC_21A_n77A14, |
| DC_21A-42A_n78A14,15,16 DC_21A-42A_n78C15,16 DC_21A-42C_n78A14,15,16 DC_21A-42C_n78C15,16 DC_21A-42D_n78A14,15,16 DC_21A-42D_n78C15,16 DC_21A-42E_n78A14,15,16 DC_21A-42E_n78C15,16 | DC_21A_n78A14 |
| DC_21A-42A_n79A14 DC_21A-42A_n79C DC_21A-42C_n79A14 DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A14 |
| DC_28A-(n)7AA | DC_28A_n7A |
| DC_28A-32A_n1A | DC_28A_n1A |
| DC_28A-32A_n3A | DC_28A_n3A |
| DC_28A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-38A_n78A | DC_28A_n78A DC_38A_n78A |
| DC_28A-66A_n7A | DC_28A_n7A DC_66A_n7A |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 |
| DC_21A_n77A-n79A14, 23 | DC_21A_n77A14 DC_21A_n79A14 |
| DC_21A_n78A-n79A14, 24 | DC_21A_n78A14 DC_21A_n79A14 |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-25A-66A_n77A | DC_25A_n77A DC_66A_n77A |
| DC_25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_25A-25A-66A_n78A | DC_25A_n78A DC_66A_n78A |
| DC_28A_n1A-n105A | DC_28A_n1A |
| DC_28A_n5A-n40A | DC_28A_n5A DC_28A_n40A |
| DC_28A_n5A-n105A | DC_28A_n5A |
| DC_28A_n40A-n77A | DC_28A_n40A DC_28A_n77A |
| DC_28A-40A_n78A DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |
| DC_28A-41A_n79A5 DC_28A-41C_n79A5 | DC_28A_n79A DC_41A_n79A |
| DC_28A_n1A-n5A | DC_28A_n1A DC_28A_n5A |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A |
| DC_28A_n1A-n78A5 | DC_28A_n1A DC_28A_n78A |
| DC_28A_n3A-n77A5 | DC_28A_n3A DC_28A_n77A |
| DC_28A_n3A-n78A5 | DC_28A_n3A DC_28A_n78A |
| DC_28A_n5A-n78A5 | DC_28A_n5A DC_28A_n78A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |
| DC_28A_n8A-n78A5 | DC_28A_n8A DC_28A_n78A |
| DC_28A_n78A-n105A | DC_28A_n78A |
| DC_28A_n38A-n78A | DC_28A_n38A DC_28A_n78A |
| DC_28A_n40A-n78A | DC_28A_n40A DC_28A_n78A |
| DC_28A_n41A-n77A | DC_28A_n41A DC_28A_n77A |
| DC_28A_SUL_n41A-n83A5 | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41A |
| DC_28A-42A_n77A15,16 DC_28A-42A_n77C15,16 DC_28A-42C_n77A15,16 DC_28A-42C_n77C15,16 | DC_28A_n77A |
| DC_28A-42A_n78A15,16 DC_28A-42A_n78C15,16 DC_28A-42C_n78A15,16 DC_28A-42C_n78C15,16 | DC_28A_n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A DC_28A-42C_n79C | DC_28A_n79A |
| DC_28A_SUL_n78A-n83A5 | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |
| DC_29A-30A_n2A | DC_30A_n2A |
| DC_29A-30A_n66A | DC_30A_n66A |
| DC_29A-30A_n77A14 | DC_30A_n77A14 |
| DC_29A-66A_n2A | DC_66A_n2A |
| DC_29A-66A-66A_n2A | DC_66A_n2A |
| DC_29A-66A_n30A | DC_66A_n30A |
| DC_29A-(n)66AA | DC_(n)66AA2 |
| DC_29A-66A-66A_n30A | DC_66A_n30A |
| DC_29A-66A_n77A14 DC_29A-66A-66A_n77A14 | DC_66A_n77A14 |
| DC_29A-66A_n78A | DC_66A_n78A |
| DC_30A-(n)5AA | DC_30A_n5A DC_(n)5AA2 |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A_n66A | DC_30A_n66A DC_66A_n66A2 |
| DC_30A-66A_n77A14 DC_30A-66A-66A_n77A14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_30A-66A_n77(2A) 14 DC_30A-66A-66A_n77(2A) 14 | DC_30A_n77A14 DC_66A_n77A14 |
| DC_32A-38A_n1A | DC_38A_n1A |
| DC_32A-38A_n28A | DC_38A_n28A |
| DC_38A_n3A-n78A | DC_38A_n3A DC_38A_n78A |
| DC_38A_n28A-n78A | DC_38A_n28A DC_38A_n78A |
| DC_39A_n40A-n41A DC_39A_n40A-n41C | DC_39A_n40A DC_39A_n41A |
| DC_39A_n40A-n79A DC_39A_n40A-n79C | DC_39A_n40A DC_39A_n79A |
| DC_39A_n41A-n79A DC_39A_n41A-n79C DC_39A_n41C-n79A DC_39A_n41C-n79C | DC_39A_n41A DC_39A_n79A |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | DC_40A_n1A DC_40A_n78A |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |
| DC_40A-42A_n77A DC_40A-42A_n77C | DC_40A_n77A |
| DC_40A-42A_n78A | DC_40A_n78A |
| DC_41A_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41C_n1A-n3A | DC_41A_n1A DC_41A_n3A |
| DC_41A_n1A-n77A | DC_41A_n1A DC_41A_n77A |
| DC_41C_n1A-n77A | DC_41A_n1A DC_41A_n77A DC_41C_n77A |
| DC_41A_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41C_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41A_n3A-n41A | DC_41A_n3A DC_41A_n41A |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_41A_n28A-n41A | DC_41A_n28A |
| DC_41A_n28A-n77A14 | DC_41A_n28A DC_41A_n77A14 |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | DC_41A_n78A |
| DC_41A_n41A-n77A | DC_41A_n77A |
| DC_41A_n41A-n78A DC_41C_n41A-n78A | DC_41A_n78A |
| DC_41A-42A_n77A15,16 DC_41A-42C_n77A15,16 DC_41C-42A_n77A15,16 DC_41C-42C_n77A15,16 | DC_41A_n77A |
| DC_41A-42A_n77(2A)15,16 DC_41A-42C_n77(2A)15,16 | DC_41A_n77A |
| DC_41A-42A_n78A15,16 DC_41A-42C_n78A15,16 DC_41C-42A_n78A15,16 DC_41C-42C_n78A15,16 | DC_41A_n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |
| DC_42A_n1A-n3A5 | DC_42A_n1A DC_42A_n3A |
| DC_42C_n1A-n3A5, | DC_42A_n1A DC_42A_n3A DC_42C_n1A DC_42C_n3A |
| DC_42A_n1A-n77A15,16 | DC_42A_n1A |
| DC_42C_n1A-n77A15,16 | DC_42A_n1A DC_42C_n1A |
| DC_42A_n1A-n78A15,16 DC_42C_n1A-n78A15,16 | N/A |
| DC_42A_n1A-n79A DC_42C_n1A-n79A | N/A |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A |
| DC_42A_n3A-n77A15,16 | DC_42A_n3A |
| DC_42A_n3A-n77(2A)15,16 | DC_42A_n3A |
| DC_42C_n3A-n77A15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42C_n3A-n77(2A)15,16 | DC_42A_n3A DC_42C_n3A |
| DC_42A_n28A-n77A15,16 | DC_42A_n28A |
| DC_42A_n28A-n77(2A)15,16 | DC_42A_n28A |
| DC_42C_n28A-n77A15,16 | DC_42A_n28A DC_42C_n28A |
| DC_42C_n28A-n77(2A)15,16 | DC_42A_n28A DC_42C_n28A |
| DC_46A-48A_n2A3 DC_46C-48A_n2A3 DC_46D-48A_n2A3 DC_46E-48A_n2A3 | DC_48A_n2A |
| DC_46A-48A_n5A3 DC_46C-48A_n5A3 DC_46D-48A_n5A3 DC_46E-48A_n5A3 | DC_48A_n5A |
| DC_46A-48A_n66A3 DC_46C-48A_n66A3 DC_46D-48A_n66A3 DC_46E-48A_n66A3 | DC_48A_n66A |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A DC_46A-66A-66A_n5A DC_46C-66A-66A_n5A DC_46D-66A-66A_n5A | DC_66A_n5A |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |
| DC_46A-66A_n77A DC_46A-46A-66A_n77A | DC_66A_n77A |
| DC_48A-(n)5AA | DC_48A_n5A DC_(n)5AA2 |
| DC_48A-(n)12AA | DC_48A_n12A DC_(n)12AA2 |
| DC_48A_n25A-n48A | DC_48A_n25A |
| DC_48A_n48A-n66A | DC_48A_n66A |
| DC_48A-66A_n2A DC_48C-66A_n2A DC_48D-66A_n2A DC_48E-66A_n2A | DC_66A_n2A DC_48A_n2A |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48C-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A DC_48A_n5A |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |
| DC_48A-66A_n25A DC_48C-66A_n25A DC_48D-66A_n25A | DC_48A_n25A DC_66A_n25A |
| DC_48A-66A_n48A | DC_66A_n48A |
| DC_48A-66A_n66A DC_48C-66A_n66A DC_48D-66A_n66A DC_48E-66A_n66A | DC_66A_n66A2 DC_48A_n66A |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |
| DC_48A-66A_n77A14,15,16 DC_48A-66A_n77C14,15,16 DC_48C-66A_n77A14,15,16 DC_48C-66A_n77C14,15,16 DC_48D-66A_n77A14,15,16 DC_48D-66A_n77C14,15,16 DC_48E-66A_n77A14,15,16 | DC_66A_n77A14 |
| DC_48A-48A-66A_n77A14,15,16 | DC_66A_n77A14 |
| DC_67A-(n)3AA | DC_(n)3AA2 |
| DC_66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A-66A-(n)5AA | DC_66A_n5A DC_(n)5AA2 |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_n38A |
| DC_66A_n2A-n41A | DC_66A_n2A DC_66A_n41A |
| DC_66A_n2A-n66A | DC_66A_n2A |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_n71A |
| DC_66A_n2A-n77A14 DC_66A_n2A-n77C14 DC_66A-66A_n2A-n77C14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A-66A_n2A-n77A14 | DC_66A_n2A DC_66A_n77A14 |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_n48A |
| DC_66A_n5A-n77A14 DC_66A_n5A-n77C14 DC_66A-66A_n5A-n77C14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A-66A_n5A-n77A14 | DC_66A_n5A DC_66A_n77A14 |
| DC_66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A-66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n12A-n77A | DC_66A_n77A DC_66A_n12A |
| DC_66A_n12A-n78A | DC_66A_n12A DC_66A_n78A |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A2 |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_66A_n66A-n77A14 DC_66A_n66A-n77C14 | DC_66A_n77A14 |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |
| DC_66A-(n)12AA | DC_66A_n12A DC_(n)12AA2 |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_n71A |
| DC_66A_n41A-n66A | DC_66A_n41A DC_66A_n66A2 |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_n71A |
| DC_(n)66AA-n71A | DC_66A_n71A DC_(n)66AA2 |
| DC_(n)66AA-n78A | DC_66A_n78A DC_(n)66AA2 |
| DC_66A-71A_n2A | DC_71A_n2A DC_66A_n2A |
| DC_66A-71A_n2(2A) | DC_66A_n2A DC_71A_n2A |
| DC_66A-71A_n7A | DC_66A_n7A DC_71A_n7A |
| DC_66A-71A_n12A | DC_66A_n12A |
| DC_66A-71A_n25A | DC_66A_n25A DC_71A_n25A |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |
| DC_66A-71A_n41A | DC_66A_n41A DC_71A_n41A |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |
| DC_66A-71A_n71A | DC_66A_n71A |
| DC_66A-71A_n77A | DC_66A_n77A DC_71A_n77A |
| DC_66A-71A_n77(2A) | DC_66A_n77A DC_71A_n77A |
| DC_66A_n71A-n77A | DC_66A_n71A DC_66A_n77A |
| DC_66A-71A_n78A | DC_71A_n78A DC_66A_n78A |
| DC_66A-71A_n78(2A) | DC_71A_n78A DC_66A_n78A |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_n78A |
| DC_66A_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_66A_SUL_n78(2A)-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_n41A |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_n66A |
| DC_71A_n2A-n77A | DC_71A_n77A DC_71A_n2A |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_n78A |
| DC_71A_n25A-n41A | DC_71A_n25A DC_71A_n41A |
| DC_71A_n25A-n66A | DC_71A_n25A DC_71A_n66A |
| DC_71A_n25A-n77A | DC_71A_n25A DC_71A_n77A |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_n66A |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_n78A |
| DC_71A_n41A-n66A | DC_71A_n41A DC_71A_n66A |
| DC_71A_n66A-n77A | DC_71A_n66A DC_71A_n77A |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: N/A NOTE 7: Void. NOTE 8: Void NOTE 9: Void NOTE 10: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE 11: The frequency range in band 3 is restricted for this band combination to 1765 - 1785 MHz for the UL and 1860-1880 MHz for the DL. NOTE 12: The frequency range in band 42 is restricted for this band combination to 3440 - 3520 MHz. NOTE 13: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. NOTE 14: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 17: Void. NOTE 18: Void. NOTE 19: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 20: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 21: The downlink DC_2_n2 RESSENS requirements only apply when the band n2 downlink carrier is configured closer to the uplink operating band than the E-UTRA Band 2 downlink carrier. NOTE 22: The frequency range in band 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758 - 788 MHz for the DL. NOTE 23: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. NOTE 24: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. NOTE 25: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-(n)3AA-n8A | DC_1A_n3A DC_1A_n8A DC_(n)3AA1 DC_3A_n8A |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A |
| DC_1A-3A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A |
| DC_1A-3A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A |
| DC_1A-3A-5A_n28A | DC_1A_n28A DC_3A_n28A DC_5A_n28A |
| DC_1A-3A-5A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A |
| DC_1A-3A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(3A) DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |
| DC_1A-3A-5A_n78A2 DC_1A-3A-5A_n78C2 DC_1A-3C-5A_n78A DC_1A-1A-3A-5A_n78A DC_1A-1A-3C-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A-5A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A_n5A-n78A2 DC_1A-3C_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-5A_n79A2 | DC_1A_n79A DC_3A_n79A DC_5A_n79A |
| DC_1A-3A-7A_n1A | DC_1A_n1A DC_3A_n1A DC_7A_n1A |
| DC_1A-3A-7A_n3A DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A |
| DC_1A-3A-7A_n5A DC_1A-3A-7C_n5A DC_1A-3C-7A_n5A DC_1A-3C-7C_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-3A-7A_n7A DC_1A-3C-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |
| DC_1A-1A-3A-7A_n7A DC_1A-1A-3C-7A_n7A DC_1A-3A-3A-7A_n7A DC_1A-1A-3A-3A-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |
| DC_1A-3A-(n)7AA DC_1A-3C-(n)7AA | DC_1A_n7A DC_3A_n7A |
| DC_1A-3A-7A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A |
| DC_1A-3A-7A_n26A DC_1A-3A-7C_n26A DC_1A-3C-7A_n26A DC_1A-3C-7C_n26A | DC_1A_n26A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_1A-3A-7A_n28A DC_1A-3A-7C_n28A DC_1A-3C-7A_n28A DC_1A-3C-7C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-3A-7A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-1A-3A-7A_n28A DC_1A-1A-3C-7A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A |
| DC_1A-3A-7A_n38A12,13 | CA_1A-3A |
| DC_1A-3A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n77(2A) DC_1A-3A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A-7A_n77(2A) DC_1A-3A-7A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |
| DC_1A-3A-7A_n78A2 DC_1A-3A-7C_n78A DC_1A-3C-7A_n78A2 DC_1A-3C-7C_n78A DC_1A-3A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-3A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A_n78(2A) DC_1A-3C-7A_n78(2A) DC_1A-3A-7C_n78(2A) DC_1A-3C-7C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-1A-3A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A_n7A-n78A DC_1A-3A_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |
| DC_1A-3A_n7A-n78(2A) DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3C_n7A-n78A DC_1A-3C_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A DC_3C_n78A |
| DC_1A-3A-7A-7A_n78A2 DC_1A-1A-3C-7A_n78A DC_1A-3A-7A-7A_n78C2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-3A-7A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7A_n105A | DC_1A_n105A DC_3A_n105A DC_7A_n105A |
| DC_1A-3A-8A_n7A | DC_1A_n7A DC_3A_n7A DC_8A_n7A |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A |
| DC_1A-3A-8A_n77A2,9 DC_1A-3C-8A_n77A2,9 | DC_1A_n77A9 DC_3A_n77A9 DC_3C_n77A DC_8A_n77A9 |
| DC_1A-3A-8A_n77(2A)2 DC_1A-3C-8A_n77(2A) | DC_1A_n77A9 DC_3A_n77A9 DC_3C_n77A DC_8A_n77A9 |
| DC_1A_n3A-n8A-n77A | DC_1A_n3A DC_1A_n8A DC_1A_n77A |
| DC_1A_n3A-n8A-n77(2A) | DC_1A_n3A DC_1A_n8A DC_1A_n77A |
| DC_1A-3A-8A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A_n8A-n77A DC_1A-3A_n8A-n77(2A) | DC_1A_n8A DC_1A_n77A DC_3A_n8A DC_3A_n77A |
| DC_1A-3A-8A_n78A2,9 DC_1A-3A-8B_n78A2 DC_1A-3C-8A_n78A2,9 | DC_1A_n78A9 DC_3A_n78A9 DC_8A_n78A9 |
| DC_1A-3A-8A_n78(2A)2 DC_1A-3C-8A_n78(2A)2 | DC_1A_n78A9 DC_3A_n78A9 DC_8A_n78A9 |
| DC_1A-3A-3A-8A_n78A2 DC_1A-3A-3A-8B_n78A2 | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A |
| DC_1A-3A-8A_n79A2 | DC_1A_n79A DC_3A_n79A DC_8A_n79A |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |
| DC_1A-3A-11A_n77A2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-11A_n77(2A) 2 DC_1A-3A-11A_n77(3A)2 | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A4 DC_18A_n3A |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A_n77A2,9 DC_1A-3A-19A_n77C2 | DC_1A_n77A9 DC_3A_n77A9 DC_19A_n77A9 |
| DC_1A-3A-19A_n77(2A)2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2, 9 DC_1A-3A-19A_n78C2 | DC_1A_n78A9 DC_3A_n78A9 DC_19A_n78A9 |
| DC_1A-3A-19A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2,9 DC_1A-3A-19A_n79C2 | DC_1A_n79A9 DC_3A_n79A9 DC_19A_n79A9 |
| DC_1A-3A-20A_n1A | DC_1A_n1A DC_3A_n1A DC_20A_n1A |
| DC_1A-3A-20A_n3A | DC_1A_n3A DC_3A_n3A DC_20A_n3A |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |
| DC_1A-3A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_20A_n8A |
| DC_1A-3A-20A_n28A3,8,14 DC_1A-3C-20A_n28A3,8,14 | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_1A-3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A_n41A DC_1A-3C-20A_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A DC_20A_n41A |
| DC_1A-3A-20A_n78A2 DC_1A-3A-20A_n78C2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-1A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2,9 DC_1A-3A-21A_n77C2 | DC_1A_n77A9 DC_3A_n77A9 DC_21A_n77A9 |
| DC_1A-3A-21A_n77(2A)2,9 | DC_1A_n77A9 DC_3A_n77A9 DC_21A_n77A9 |
| DC_1A-3A-21A_n78A2,9 DC_1A-3A-21A_n78C2 | DC_1A_n78A9 DC_3A_n78A9 DC_21A_n78A9 |
| DC_1A-3A-21A_n78(2A)2,9 | DC_1A_n78A9 DC_3A_n78A9 DC_21A_n78A9 |
| DC_1A-3A-21A_n79A2,9 DC_1A-3A-21A_n79C2 | DC_1A_n79A9 DC_3A_n79A9 DC_21A_n79A9 |
| DC_1A-3A-26A_n78A DC_1A-3C-26A_n78A | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3A-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_26A_n78A |
| DC_1A-3A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A |
| DC_1A-3C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A |
| DC_1A-3A-28A_n5A DC_1A-3C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_28A_n5A |
| DC_1A-3A-28A_n7A DC_1A-3C-28A_n7A DC_1A-3A-28A_n7B DC_1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-3A-28A_n7A DC_1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_28A_n7A |
| DC_1A-1A-3A-28A_n7A DC_1A-1A-3C-28A_n7A DC_1A-1A-3A-28A_n7B DC_1A-1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-1A-3A-3A-28A_n7A DC_1A-1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-28A_n38A | DC_1A_n38A DC_3A_n38A DC_28A_n38A |
| DC_1A-3A_n28A-n38A | DC_1A_n28A DC_3A_n28A DC_1A_n38A DC_3A_n38A |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A |
| DC_1A-3A_n28A-n41A2 | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A |
| DC_1A-3A_n28A-n75A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3C_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-3A-28A_n77A2 DC_1A-3A-28A_n77C2 | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A |
| DC_1A-3A-28A_n78A2 DC_1A-3C-28A_n78A2 DC_1A-3A-28A_n78C2 DC_1A-1A-3A-28A_n78A DC_1A-1A-3C-28A_n78A DC_1A-3A-28A_n78(2A) 2 DC_1A-3C-28A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3A-3A-28A_n78A2 | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n79A2 DC_1A-3A-28A_n79C2 | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A |
| DC_1A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A |
| DC_1A-3A_n28A-n78A2 DC_1A-3C_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n28A-n78(2A)2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-32A_n28A DC_1A-3C-32A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-3A-32A_n78A DC_1A-3A-32A_n78C | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-32A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3C-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_38A_n28A |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-38A_n78(2A) DC_1A-3C-38A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_3A_n38A DC_3A_n78A |
| DC_1A-3C-38A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_38A_n78A |
| DC_1A-3A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A |
| DC_1A-3A_n40A-n78A DC_1A-3A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A_n40A-n105A | DC_1A_n40A DC_1A_n105A DC_3A_n40A DC_3A_n105A |
| DC_1A-3A-40A_n78(2A) DC_1A-3A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_40A_n78A |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A4 DC_41A_n3A DC_41C_n3A |
| DC_1A-3A-41A_n28A2 DC_1A-3A-41C_n28A2 | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-(n)41AA | DC_1A_n41A DC_3A_n41A |
| DC_1A-3A-41A_n77A DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A-41A_n77(2A) DC_1A-3A-41C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A |
| DC_1A-3A-41A_n78A DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A-41A_n78(2A) DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A-41A_n79A2 DC_1A-3A-41C_n79A2 | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A2 DC_1A-3A-42C_n28A2 | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42A_n77A7,8,9 DC_1A-3A-42A_n77C7,8 DC_1A-3A-42C_n77A7,8,9 DC_1A-3A-42C_n77C7,8 DC_1A-3A-42D_n77A7,8,9 | DC_1A_n77A9 DC_3A_n77A9 |
| DC_1A-3A-42A_n77(2A) 7,8 DC_1A-3A-42C_n77(2A) 7,8 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n78A7,8,9 DC_1A-3A-42A_n78C7,8 DC_1A-3A-42C_n78A7,8,9 DC_1A-3A-42C_n78C7,8 DC_1A-3A-42D_n78A7,8,9 | DC_1A_n78A9 DC_3A_n78A9 |
| DC_1A-3A-42A_n79A9 DC_1A-3A-42A_n79C DC_1A-3A-42C_n79A9 DC_1A-3A-42C_n79C DC_1A-3A-42D_n79A9 | DC_1A_n79A9 DC_3A_n79A9 |
| DC_1A-3A_n75A-n78A DC_1A-3C_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n77A-n79A9 | DC_1A_n77A9 DC_1A_n79A9 DC_3A_n77A9 DC_3A_n79A9 |
| DC_1A-(n)3AA-n77A DC_1A-(n)3AA-n77(2A) | DC_1A_n3A DC_1A_n77A DC_3A_n77A |
| DC_1A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n78A-n79A9 | DC_1A_n78A9 DC_1A_n79A9 DC_3A_n78A9 DC_3A_n79A9 |
| DC_1A-3A_n78A-n105A | DC_1A_n78A DC_1A_n105A DC_3A_n78A DC_3A_n105A |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_1A-5A-7A_n28A | DC_1A_n28A DC_5A_n28A DC_7A_n28A |
| DC_1A-5A-7A_n40A | DC_1A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-5A-7A-7A_n40A | DC_1A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-5A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n77(2A) DC_1A-5A-7A_n77(3A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77A | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A-7A_n77(2A) DC_1A-5A-7A-7A_n77(3A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-5A-7A_n78A DC_1A-5A-7A_n78C DC_1A-1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A_n78(A-C) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78A DC_1A-5A-7A-7A_n78C | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78(A-C) | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_5A_n28A DC_5A_n78A |
| DC_1A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-5A_n40A-n78A DC_1A-5A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-7A_n3A-n38A | DC_1A_n3A |
| DC_1A-7A_n3A-n78A DC_1A-7C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n3A-n78(2A) DC_1A-7C_n3A-n78(2A) | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7C_n3A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_7A_n5A DC_7A_n40A |
| DC_1A-7A_n5A-n78A DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A |
| DC_1A-7A_n38A-n78A | DC_1A_n78A |
| DC_1A-7A-8A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A |
| DC_1A-7A-8A_n7A | DC_1A_n7A DC_7A_n7A DC_8A_n7A |
| DC_1A-7A-8A_n20A | DC_1A_n20A DC_7A_n20A DC_8A_n20A |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-7A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-8A_n78A2 DC_1A-7A-8B_n78A2 | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-7A-8A_n78A2 DC_1A-7A-7A-8B_n78A2 | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-7A-8A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-7A-20A_n3A DC_1A-7C-20A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A_n28A3,8,14 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n38A12,13 | CA_1A-20A |
| DC_1A-7A-20A_n78A2 DC_1A-7A-20A_n78C2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-1A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-26A_n78A DC_1A-7C-26A_n78A | DC_1A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-7A-26A_n78(2A) DC_1A-7C-26A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A-28A_n3A DC_1A-7C-28A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-7A-28A_n5A DC_1A-7C-28A_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-7A-28A_n20A | DC_1A_n20A DC_7A_n20A DC_28A_n20A |
| DC_1A-7A-28A_n38A | 1A16 28A16 |
| DC_1A-7A_n28A-n38A | DC_1A_n28A16 |
| DC_1A-7A-28A_n40A | DC_1A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-7A-28A_n78(2A) DC_1A-7C-28A_n78(2A) | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-1A-7A-28A_n78A | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-7A-32A_n3A DC_1A-7C-32A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-38A_n3A | DC_1A_n3A |
| DC_1A-7A-38A_n8A | DC_1A_n8A |
| DC_1A-7A-38A_n28A10 | DC_1A_n28A |
| DC_1A-7A-38A_n78A10 | DC_1A_n78A |
| DC_1A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A-40A_n78(2A) DC_1A-7A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-7A_n40A-n78A DC_1A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A-7A_n40A-n78A DC_1A-7A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A_n40A-n105A | DC_1A_n40A DC_1A_n105A DC_7A_n40A DC_7A_n105A |
| DC_1A-7A_n75A-n78A | DC_1A_n78A DC_7A_n78A |
| DC_1A-7A_n78A-n105A | DC_1A_n78A DC_1A_n105A DC_7A_n78A DC_7A_n105A |
| DC_1A-8A-(n)3AA | DC_1A_n3A DC_(n)3AA4 DC_8A_n3A |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A |
| DC_1A-8A_n3A-n77A2 DC_1A-8B_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n77(2A)2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A |
| DC_1A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_8A_n7A DC_8A_n78A |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-8A-11A_n77A2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n77(2A)2 DC_1A-8A-11A_n77(3A)2 | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n78A2 | DC_1A_n78A DC_8A_n78A DC_11A_n78A |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| DC_1A-8A-20A_n3A | DC_1A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-8A-20A_n28A3,8,11,14 | DC_1A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-8A-28A_n3A | DC_1A_n3A DC_8A_n3A DC_28A_n3A |
| DC_1A-8A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n28A-n77(2A)2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-8A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A-32A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A-32A_n78A | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-40A_n78(2A) DC_1A-8A-40C_n78(2A) | DC_1A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-8A-42A_n3A2 DC_1A-8A-42C_n3A2 | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A2 DC_1A-8A-42C_n28A2 | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n77A7,8 DC_1A-8A-42C_n77A7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A-42A_n77(2A) 7,8 DC_1A-8A-42C_n77(2A) 7,8 | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77A-n79A DC_1A-8A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |
| DC_1A-11A-18A_n77A | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n77(2A) | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A-18A_n78(2A) | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-18A-28A_n79A2 | DC_1A_n79A DC_18A_n79A DC_28A_n79A |
| DC_1A-18A-41A_n3A DC_1A-18A-41C_n3A | DC_1A_n3A DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-18A-41A_n77A DC_1A-18A-41C_n77A | DC_1A_n77A DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-18A-41A_n78A DC_1A-18A-41C_n78A | DC_1A_n78A DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-18A_n41A-n78A | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n41A-n78(2A) | DC_1A_n41A DC_18A_n41A DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n77A7,8 DC_1A-18A-42C_n77A7,8 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A-42A_n78A7,8 DC_1A-18A-42C_n78A7,8 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n79A DC_1A-18A-42C_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A-21A_n77A2,9 DC_1A-19A-21A_n77C2 | DC_1A_n77A9 DC_19A_n77A9 DC_21A_n77A9 |
| DC_1A-19A-21A_n77(2A) 2 | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A2, 9 DC_1A-19A-21A_n78C2 | DC_1A_n78A9 DC_19A_n78A9 DC_21A_n78A9 |
| DC_1A-19A-21A_n78(2A) 2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A2,9 DC_1A-19A-21A_n79C2 | DC_1A_n79A9 DC_19A_n79A9 DC_21A_n79A9 |
| DC_1A-19A-42A_n77A7,8,9 DC_1A-19A-42A_n77C7,8 DC_1A-19A-42C_n77A7,8,9 DC_1A-19A-42C_n77C7,8 | DC_1A_n77A9 DC_19A_n77A9 |
| DC_1A-19A-42A_n78A7,8,9 DC_1A-19A-42A_n78C7,8 DC_1A-19A-42C_n78A7,8,9 DC_1A-19A-42C_n78C7,8 | DC_1A_n78A9 DC_19A_n78A9 |
| DC_1A-19A-42A_n79A9 DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A9 DC_1A-19A-42C_n79C | DC_1A_n79A9 DC_19A_n79A9 |
| DC_1A-19A_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_1A-19A_n78A-n79A9 | DC_1A_n78A DC_1A_n79A DC_19A_n78A9 DC_19A_n79A9 |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_20A_n7A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A_n28A-n75A | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-28A_n78A | DC_1A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-20A_n28A-n78A2,3,8,14 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A-32A_n28A8,14 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A DC_38A_n3A |
| DC_1A-20A-(n)38AA | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A DC_38A_n8A |
| DC_1A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A DC_38A_n78A |
| DC_1A-20A-38A_n78(2A) | DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n38A-n78A | DC_1A_n38A DC_20A_n38A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A-40A_n78A DC_1A-20A-40C_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-21A-28A_n77A2 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A |
| DC_1A-21A-28A_n78A2 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A |
| DC_1A-21A-28A_n79A2 | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A |
| DC_1A-21A-42A_n77A7,8,9 DC_1A-21A-42A_n77C7,8 DC_1A-21A-42C_n77A7,8,9 DC_1A-21A-42C_n77C7,8 DC_1A-21A-42D_n77A7,8 DC_1A-21A-42D_n77C7,8 | DC_1A_n77A9 DC_21A_n77A9 |
| DC_1A-21A-42A_n78A7,8,9 DC_1A-21A-42A_n78C7,8 DC_1A-21A-42C_n78A7,8,9 DC_1A-21A-42C_n78C7,8 DC_1A-21A-42D_n78A7,8 DC_1A-21A-42D_n78C7,8 | DC_1A_n78A9 DC_21A_n78A9 |
| DC_1A-21A-42A_n79A9 DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A9 DC_1A-21A-42C_n79C DC_1A-21A-42D_n79A DC_1A-21A-42D_n79C | DC_1A_n79A9 DC_21A_n79A9 |
| DC_1A-21A_n77A-n79A9 | DC_1A_n77A9 DC_1A_n79A9 |
| DC_1A-21A_n78A-n79A9 | DC_1A_n78A9 DC_1A_n79A9 DC_21A_n78A9 DC_21A_n79A9 |
| DC_1A-28A_n3A-n77A2 | DC_28A_n3A DC_28A_n77A |
| DC_1A-28A_n3A-n78A2 | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A |
| DC_1A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-28A-(n)7AA | DC_1A_n7A DC_28A_n7A |
| DC_1A-28A_n7A-n78A | DC_1A_n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n7B-n78A | DC_1A_n7A DC_1A_n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A-40A_n78A DC_1A-28A-40C_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-28A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_28A_n38A DC_28A_n78A |
| DC_1A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-28A-42A_n77A7,8 DC_1A-28A-42A_n77C7,8 DC_1A-28A-42C_n77A7,8 DC_1A-28A-42C_n77C7,8 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A-42A_n78A7,8 DC_1A-28A-42A_n78C7,8 DC_1A-28A-42C_n78A7,8 DC_1A-28A-42C_n78C7,8 | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-42A_n79A DC_1A-28A-42A_n79C DC_1A-28A-42C_n79A DC_1A-28A-42C_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A |
| DC_1A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A |
| DC_1A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_38A_n3A DC_38A_n78A |
| DC_1A-38A_n7A-n78A | DC_1A_n78A |
| DC_1A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A_n40A-n78A-n105A | DC_1A_n40A DC_1A_n78A DC_1A_n105A |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |
| DC_1A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A |
| DC_1A-42A_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42A_n3A-n77(2A) 7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A |
| DC_1A-42C_n3A-n77A7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42C_n3A-n77(2A)7,8 | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-42A_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42A_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A |
| DC_1A-42C_n28A-n77A7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n28A-n77(2A)7,8 | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-41A-42A_n77A7,8 DC_1A-41A-42C_n77A7,8 DC_1A-41C-42A_n77A7,8 DC_1A-41C-42C_n77A7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n77(2A)7,8 DC_1A-41A-42C_n77(2A)7,8 | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n78A7,8 DC_1A-41A-42C_n78A7,8 DC_1A-41C-42A_n78A7,8 DC_1A-41C-42C_n78A7,8 | DC_1A_n78A DC_41A_n78A |
| DC_1A-41A-42A_n79A DC_1A-41A-42C_n79A DC_1A-41C-42A_n79A DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A |
| DC_1A-42A_n77A-n79A7,8,9 DC_1A-42C_n77A-n79A7,8,9 | DC_1A_n77A9 DC_1A_n79A9 |
| DC_1A-42A_n78A-n79A7,8,9 DC_1A-42C_n78A-n79A7,8,9 | DC_1A_n78A9 DC_1A_n79A9 |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |
| DC_2A-4A-7A_n78A DC_2A-4A-7C_n78A | DC_2A_n78A DC_4A_n78A |
| DC_2A-5A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_5A_n2A DC_5A_n41A |
| DC_2A-5A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_5A_n2A DC_5A_n66A |
| DC_2A-5A_n2A-n77A DC_2A-5A_n2A-n77C | DC_2A_n77A DC_5A_n2A DC_5A_n77A |
| DC_2A-5A_n2A-n78A | DC_5A_n2A DC_2A_n78A DC_5A_n78A |
| DC_2A-5A_n5A-n77A DC_2A-5A_n5A-n77C | DC_2A_n5A DC_2A_n77A DC_5A_n77A |
| DC_2A-5A-7A_n2A | DC_5A_n2A DC_7A_n2A |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-7A_n77A | DC_2A_n77A DC_5A_n77A DC_7A_n77A |
| DC_2A-5A-7A_n77(2A) | DC_2A_n77A DC_5A_n77A DC_7A_n77A |
| DC_2A-5A-7A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A |
| DC_2A-2A-5A-7A_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |
| DC_2A-5A-7A-(n)66AA DC_2A-5A-7C-(n)66AA | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-5A-7A-7A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-5A-7A_n78(2A) | DC_2A_n78A DC_5A_n78A DC_7A_n78A |
| DC_2A-5A-(n)12AA | DC_5A_n12A DC_2A_n12A DC_(n)12AA4 |
| DC_2A-5A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_5A_n41A DC_5A_n66A |
| DC_2A-12A-(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |
| DC_2A-5A-30A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A |
| DC_2A-5A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A |
| DC_2A-5A-30A_n77A9 DC_2A-2A-5A-30A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-30A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_30A_n77A9 |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |
| DC_2A-5A-48A_n77A7,8,9 DC_2A-5A-48A_n77C7,8,9 DC_2A-5A-48C_n77A7,8,9 DC_2A-5A-48C_n77C7,8,9 | DC_2A_n77A9 DC_5A_n77A9 |
| DC_2A-5A-66A_n2A DC_2A-5B-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A-66A_n2A DC_2A-5B-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |
| DC_2A-5A-66A_n12A | DC_2A_n12A DC_5A_n12A DC_66A_n12A |
| DC_2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-2A-5A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A-66A_n30A | DC_2A_n30A DC_5A_n30A DC_66A_n30A |
| DC_2A-5A-66A_n41A DC_2A-2A-5A-66A_n41A | DC_2A_n41A DC_5A_n41A DC_66A_n41A |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |
| DC_2A-5A-66A_n66A DC_2A-5B-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 |
| DC_2A-5A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_(n)66AA4 |
| DC_2A-2A-5A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_(n)66AA4 |
| DC_2A-5A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A-66A_n66A DC_2A-5B-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-2A-5A-66A-(n)66AA | DC_2A_n66A DC_5A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-2A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A-66A-66A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-66A_n71A | DC_2A_n71A DC_5A_n71A DC_66A_n71A |
| DC_2A-5A-66A_n77A9 DC_2A-5A-66A_n77C9 DC_2A-2A-5A-66A_n77C9 DC_2A-5A-66A-66A_n77C9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n77(2A) 9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-5A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A-66A_n77A9 | DC_2A_n77A9 DC_5A_n77A9 DC_66A_n77A9 |
| DC_2A-5A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_66A_n78A |
| DC_2A-5A-66A_n78(2A) | DC_2A_n78A DC_5A_n78A DC_66A_n78A |
| DC_2A-5A_n66A-n77A DC_2A-5A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A |
| DC_2A-5A_n66A-n78A | DC_2A_n66A DC_5A_n66A DC_2A_n78A DC_5A_n78A |
| DC_2A-7A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_7A_n2A DC_7A_n66A |
| DC_2A-7A_n2A-n71A | DC_2A_n71A DC_7A_n2A DC_7A_n71A |
| DC_2A-7A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_7A_n2A DC_7A_n77A |
| DC_2A-7A_n2A-n78A | DC_7A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-12A_n2A | DC_7A_n2A DC_12A_n2A |
| DC_2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A |
| DC_2A-7A-12A_n77A | DC_2A_n77A DC_7A_n77A DC_12A_n77A |
| DC_2A-7A_n12A-n77A | DC_2A_n12A DC_2A_n77A DC_7A_n12A DC_7A_n77A |
| DC_2A-7A-12A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_12A_n77A |
| DC_2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-7A-12A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_12A_n78A |
| DC_2A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7C-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7C-13A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-7A_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7C_n25A-n66A8,14 | DC_2A_n66A DC_7A_n25A DC_7A_n66A |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A |
| DC_2A-7A-28A_n78A DC_2A-7C-28A_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_2A-7A-28A_n78(2A) DC_2A-7C-28A_n78(2A) | DC_2A_n78A DC_28A_n78A |
| DC_2A-7A_n38A-n66A DC_2A-7C_n38A-n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-29A_n78A DC_2A-7C-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-38A_n78A DC_2A-7C-38A_n78A | DC_2A_n78A |
| DC_2A-7A_n38A-n78A DC_2A-7C_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-7A_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-66A_n2A | DC_7A_n2A DC_66A_n2A |
| DC_2A-7A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-7A-66A_n12A | DC_2A_n12A DC_7A_n12A DC_66A_n12A |
| DC_2A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7C-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |
| DC_2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-(n)66AA DC_2A-7C-(n)66AA | DC_2A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-7A-7A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-7A-7A-66A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-7A-7A-66A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-7A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_7A_n66A DC_7A_n71A |
| DC_2A-7A-66A_n77A DC_2A-7C-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n77(2A) DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-7A-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |
| DC_2A-7A-66A_n78A9 DC_2A-7C-66A_n78A9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-2A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A_n66A-n78A DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-66A_n78(2A)9 DC_2A-7C-66A_n78(2A)9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-7A-66A_n78A9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-66A-66A_n78A9 DC_2A-7C-66A-66A_n78A9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-66A-66A_n78(2A) 9 DC_2A-7C-66A-66A_n78(2A) 9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-7A-66A_n78(2A) 9 | DC_2A_n78A9 DC_7A_n78A9 DC_66A_n78A9 |
| DC_2A-7A-7A-66A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-71A_n2A | DC_7A_n2A DC_71A_n2A |
| DC_2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_71A_n66A |
| DC_2A-7A-71A_n77A | DC_2A_n77A DC_7A_n77A DC_71A_n77A |
| DC_2A-7A-71A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_71A_n77A |
| DC_2A-7A_n71A-n77A | DC_2A_n71A DC_2A_n77A DC_7A_n71A DC_7A_n77A |
| DC_2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-2A-7A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-7A-71A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_71A_n78A |
| DC_2A-7A_n71A-n78A | DC_2A_n71A DC_7A_n71A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A_n2A-n41A | DC_2A_n41A DC_12A_n2A DC_12A_n41A |
| DC_2A-12A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_12A_n2A DC_12A_n66A |
| DC_2A-12A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_12A_n2A DC_12A_n77A |
| DC_2A-12A_n2A-n78A | DC_12A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_2A-12A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_12A_n41A DC_12A_n66A |
| DC_2A-12A-48A_n5A | DC_2A_n5A DC_12A_n5A DC_48A_n5A |
| DC_2A-12A-66A_n5A | DC_2A_n5A DC_12A_n5A DC_66A_n5A |
| DC_2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-12A-30A_n77A9 DC_2A-2A-12A-30A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-30A_n77(2A)9 | DC_2A_n77A9 DC_12A_n77A9 DC_30A_n77A9 |
| DC_2A-12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A_n7A | DC_2A_n7A DC_12A_n7A DC_66A_n7A |
| DC_2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-2A-12A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A-66A_n30A | DC_2A_n30A DC_12A_n30A DC_66A_n30A |
| DC_2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A DC_12A_n41A DC_66A_n41A |
| DC_2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-12A-(n)66AA | DC_2A_n66A DC_12A_n66A DC_(n)66AA4 |
| DC_2A-2A-12A-(n)66AA | DC_2A_n66A DC_12A_n66A DC_(n)66AA4 |
| DC_2A-2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-12A-66A_n77A9 DC_2A-2A-12A-66A_n77A9 DC_2A-12A-66A-66A_n77A9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A-66A_n77(2A) 9 | DC_2A_n77A9 DC_12A_n77A9 DC_66A_n77A9 |
| DC_2A-12A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_12A_n66A DC_12A_n77A |
| DC_2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-12A-66A_n78(2A) | DC_2A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-12A_n66A-n78A | DC_2A_n66A DC_12A_n66A DC_2A_n78A DC_12A_n78A |
| DC_2A-13A_n2A-n77A DC_2A-13A_n2A-n77C | DC_2A_n77A DC_13A_n2A DC_13A_n77A |
| DC_2A-13A_n5A-n77A9 DC_2A-2A-13A_n5A-n77A9 DC_2A-13A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A9 DC_13A_n77A9 |
| DC_2A-13A_n25A-n66A8,14 | DC_2A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-13A-48A_n77A7,8,9 DC_2A-13A-48A_n77C7,8,9 DC_2A-13A-48C_n77A7,8,9 DC_2A-13A-48C_n77C7,8,9 | DC_2A_n77A9 DC_13A_n77A |
| DC_2A-13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A_n48A DC_2A-13A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A-66A_n48A DC_2A-13A-66A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A_n66A DC_2A-2A-13A-66A_n66A DC_2A-13A-66A-66A_n66A DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-13A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-2A-13A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-13A-66A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-2A-13A-66A-(n)66AA | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 DC_(n)66AA4 |
| DC_2A-13A-66B_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-13A-66A_n77A9 DC_2A-13A-66A_n77C9 DC_2A-2A-13A-66A_n77C9 DC_2A-2A-13A-66A-66A_n77A DC_2A-13A-66A-66A_n77C9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-2A-13A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A-66A-66A_n77A9 | DC_2A_n77A9 DC_13A_n77A9 DC_66A_n77A9 |
| DC_2A-13A_n66A-n77A9 DC_2A-13A_n66A-n77C9 DC_2A-2A-13A_n66A-n77A9 | DC_2A_n66A DC_2A_n77A9 DC_13A_n66A DC_13A_n77A9 |
| DC_2A-14A-30A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A_n77A9 DC_2A-2A-14A-30A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-30A_n77(2A) 9 | DC_2A_n77A9 DC_14A_n77A9 DC_30A_n77A9 |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-2A-14A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A-66A_n30A | DC_2A_n30A DC_14A_n30A DC_66A_n30A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-14A-66A_n77A9 DC_2A-2A-14A-66A_n77A9 DC_2A-14A-66A-66A_n77A9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-14A-66A_n77(2A) 9 | DC_2A_n77A9 DC_14A_n77A9 DC_66A_n77A9 |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A_n2A | DC_2A_n2A4 DC_30A_n2A |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-29A-30A_n77A9 DC_2A-2A-29A-30A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 |
| DC_2A-29A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-2A-29A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A-66A_n30A | DC_2A_n30A DC_66A_n30A |
| DC_2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-29A-(n)66AA | DC_2A_n66A DC_(n)66AA4 |
| DC_2A-2A-29A-(n)66AA | DC_2A_n66A DC_(n)66AA4 |
| DC_2A-2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-29A-66A_n77A9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |
| DC_2A-30A-(n)5AA DC_2A-2A-30A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_(n)5AA4 |
| DC_2A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-30A-66A_n77A9 DC_2A-2A-30A-66A_n77A9 DC_2A-30A-66A-66A_n77A9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-30A-66A_n77(2A) 9 | DC_2A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_2A-46A_n41A-n66A DC_2A-46C_n41A-n66A DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-46A_n41A-n71A DC_2A-46C_n41A-n71A DC_2A-46D_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41(2A)-n71A DC_2A-46C_n41(2A)-n71A DC_2A-46D_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |
| DC_2A-46A-48A_n5A DC_2A-46C-48A_n5A DC_2A-46D-48A_n5A DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-46A-48A_n66A DC_2A-46C-48A_n66A DC_2A-46D-48A_n66A DC_2A-46E-48A_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-46A-66A_n5A DC_2A-46C-66A_n5A DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-46A-66A_n41A DC_2A-46C-66A_n41A DC_2A-46D-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n41(2A) DC_2A-46C-66A_n41(2A) DC_2A-46D-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n71A DC_2A-46C-66A_n71A DC_2A-46D-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-48A-(n)5AA | DC_2A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_2A-46A_n66A-n71A DC_2A-46C_n66A-n71A DC_2A-46D_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |
| DC_2A-48A-66A_n5A | DC_2A_n5A DC_48A_n5A DC_66A_n5A |
| DC_2A-48C-66A_n5A DC_2A-48D-66A_n5A DC_2A-48E-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-48A-66A_n12A | DC_2A_n12A DC_48A_n12A DC_66A_n12A |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |
| DC_2A-48A-66A_n71A | DC_2A_n71A DC_48A_n71A DC_66A_n71A |
| DC_2A-48A-66A_n77A7,8,9 DC_2A-48C-66A_n77A7,8,9 DC_2A-48A-66A_n77C7,8,9 DC_2A-48C-66A_n77C7,8,9 DC_2A-48D-66A_n77A7,8,9 DC_2A-48E-66A_n77A7,8,9 | DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_66A_n2A DC_66A_n41A |
| DC_2A-66A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_2A-66A_n2A-n71A | DC_2A_n2A4 DC_2A_n71A DC_66A_n2A DC_66A_n71A |
| DC_2A-66A_n2A-n77A DC_2A-66A_n2A-n77C | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-66A-(n)5AA DC_2A-2A-66A-(n)5AA DC_2A-66A-66A-(n)5AA | DC_2A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-66A_n2A-n78A | DC_66A_n2A DC_2A_n78A DC_66A_n78A |
| DC_2A-66A_n5A-n77A9 DC_2A-2A-66A_n5A-n77A9 DC_2A-66A-66A_n5A-n77A9 DC_2A-66A_n5A-n77C9 | DC_2A_n5A DC_2A_n77A9 DC_5A_n77A DC_66A_n5A DC_66A_n77A9 |
| DC_2A-66A_n12A-n77A | DC_2A_n12A DC_2A_n77A DC_66A_n12A DC_66A_n77A |
| DC_2A-66A_n12A-n78A | DC_2A_n12A DC_2A_n78A DC_66A_n12A DC_66A_n78A |
| DC_2A-66A_n25A-n66A7,8 | DC_2A_n66A DC_66A_n25A |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |
| DC_2A-66A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_2A-(n)66AA-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n78A DC_(n)66AA4 |
| DC_2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A DC_66A_n41A DC_71A_n41A |
| DC_2A-66A-71A_n66A | DC_2A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A-71A_n77A | DC_2A_n77A DC_66A_n77A DC_71A_n77A |
| DC_2A-66A-71A_n77(2A) | DC_2A_n77A DC_66A_n77A DC_71A_n77A |
| DC_2A-66A_n71A-n77A | DC_2A_n71A DC_2A_n77A DC_66A_n71A DC_66A_n77A |
| DC_2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-71A_n78(2A) | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-(n)71AA DC_2A-66C-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_2A-66A_n41A-n71A DC_2A-66A_n41C-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n66A-n77A9 DC_2A-2A-66A_n66A-n77A9 DC_2A-66A_n66A-n77C9 | DC_2A_n66A DC_2A_n77A9 DC_66A_n77A9 |
| DC_2A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-66A-71A_n2A | DC_66A_n2A DC_71A_n2A |
| DC_2A-66A_n71A-n78A | DC_2A_n71A DC_66A_n71A DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n2A-n41A | DC_2A_n41A DC_71A_n2A DC_71A_n41A |
| DC_2A-71A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_71A_n2A DC_71A_n66A |
| DC_2A-71A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_71A_n2A DC_71A_n77A |
| DC_2A-71A_n2A-n78A | DC_71A_n2A DC_2A_n78A DC_7A_n78A |
| DC_2A-71A_n41A-n66A | DC_2A_n41A DC_2A_n66A DC_71A_n41A DC_71A_n66A |
| DC_2A-71A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_71A_n66A DC_71A_n77A |
| DC_2A-71A_n66A-n78A | DC_2A_n66A DC_71A_n66A DC_2A_n78A DC_71A_n78A |
| DC_3A_n1A-n5A-n78A | DC_3A_n1A DC_3A_n5A DC_3A_n78A |
| DC_3A_n1A-n5A-n105A | DC_3A_n1A DC_3A_n5A DC_3A_n105A |
| DC_3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A-3A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A |
| DC_3A_n1A-n28A-n75A DC_3C_n1A-n28A-n75A | DC_3A_n1A DC_3C_n1A DC_3A_n28A DC_3C_n28A |
| DC_3A_n1A-n40A-n78A | DC_3A_n1A DC_3A_n40A DC_3A_n78A |
| DC_3A_n1A-n75A-n78A DC_3C_n1A-n75A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A |
| DC_3A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A |
| DC_3A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A |
| DC_3A_n1A-n78A-n105A | DC_3A_n1A DC_3A_n78A DC_3A_n105A |
| DC_3A-5A-7A_n28A | DC_3A_n28A DC_5A_n28A DC_7A_n28A |
| DC_3A-5A-7A_n40A | DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_3A-5A-7A-7A_n40A | DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_3A-5A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n77(2A) DC_3A-5A-7A_n77(3A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77A | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A-7A_n77(2A) DC_3A-5A-7A-7A_n77(3A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_3A-5A-7A_n78A DC_3C-5A-7A_n78A DC_3A-5A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A_n78(A-C) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78A DC_3A-5A-7A-7A_n78C | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(2A) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n78(A-C) | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-5A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_5A_n28A DC_5A_n78A |
| DC_3A-5A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_3A-5A_n40A-n78A DC_3A-5A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A |
| DC_3A_n5A-n40A-n78A | DC_3A_n5A DC_3A_n40A DC_3A_n78A |
| DC_3A_n5A-n78A-n105A | DC_3A_n5A DC_3A_n78A DC_3A_n105A |
| DC_3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A |
| DC_3A-7A_n1A-n28A DC_3C-7A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_7A_n1A DC_7A_n28A |
| DC_3A-7C_n1A-n28A DC_3C-7C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_7A_n1A DC_7A_n28A DC_7C_n1A DC_7C_n28A |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A |
| DC_3A-7A_n1A-n78A2, 9 DC_3C-7A_n1A-n78A2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A9 DC_3C_n78A DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7A_n1A-n78(2A)2 DC_3C-7A_n1A-n78(2A)2 | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-3A-7A-7A_n1A-n78A2, 9 | DC_3A_n1A DC_3A_n78A9 DC_7A_n1A DC_7A_n78A9 |
| DC_3A-7C_n1A-n78A DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-7C_n1A-n78(2A)2 DC_3C-7C_n1A-n78(2A)2 | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_3A-7A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_3C-7A_n1A-n75A | DC_3C_n1A DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A |
| DC_3A-7A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A |
| DC_3A-7A_n5A-n78A9 DC_3A-7C_n5A-n78A9 DC_3C-7A_n5A-n78A9 DC_3C-7C_n5A-n78A9 | DC_3A_n5A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n5A DC_7C_n5A DC_7A_n78A9 DC_7C_n78A9 |
| DC_3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3A-3A-7A_n7A-n78A2 | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-8A_n1A DC_3A-7A-8B_n1A DC_3C-7A-8A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-8A_n1A DC_3A-3A-7A-8B_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-7A-8A_n1A DC_3A-7A-7A-8B_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-7A-8A_n1A DC_3A-3A-7A-7A-8B_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A_n7A | DC_3A_n7A DC_7A_n7A4 DC_8A_n7A |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_3A-7A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n77A2 | DC_3A_n77A DC_7A_n77A DC_8A_n77A |
| DC_3A-7A-8A_n78A2, 9 DC_3A-7A-8B_n78A2, 9 DC_3C-7A-8A_n78A | DC_3A_n78A9 DC_3C_n78A DC_7A_n78A 9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-8A_n78(2A) | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |
| DC_3A-3A-7A-8A_n78A2, 9 DC_3A-3A-7A-8B_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-7A-8A_n78A2, 9 DC_3A-7A-7A-8B_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A-7A-8A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_3A-3A-7A-7A-8A_n78A2, 9 DC_3A-3A-7A-7A-8B_n78A2, 9 | DC_3A_n78A9 DC_7A_n78A9 DC_8A_n78A9 DC_8B_n78A |
| DC_3A-7A_n8A-n78A2, 9 | DC_3A_n8A DC_3A_n78A9 DC_7A_n8A DC_7A_n78A9 |
| DC_3A-3A-7A_n8A-n78A2, 9 | DC_3A_n8A DC_3A_n78A9 DC_7A_n8A DC_7A_n78A9 |
| DC_3A-7A-7A_n8A-n78A2, 9 | DC_3A_n8A DC_3A_n78A9 DC_7A_n8A DC_7A_n78A9 |
| DC_3A-3A-7A-7A_n8A-n78A2, 9 | DC_3A_n8A DC_3A_n78A9 DC_7A_n8A DC_7A_n78A9 |
| DC_3A-7A-20A_n1A DC_3C-7A-20A_n1A DC_3A-7C-20A_n1A DC_3C-7C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_3A-7A-20A_n3A | DC_3A_n3A DC_7A_n3A DC_20A_n3A |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3,8,14 DC_3C-7A-20A_n28A3 | DC_3A_n28A DC_7A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-7A-20A_n38A12,13 | CA_3A-20A |
| DC_3A-7A-20A_n78A2 DC_3C-7A-20A_n78A2 DC_3A-7A-20A_n78C2 | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_7A_n78A |
| DC_3A-3A-7A-20A_n78A2 | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-7A-20A_n78A2 | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-20A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-26A_n78A DC_3C-7A-26A_n78A DC_3A-7C-26A_n78A DC_3C-7C-26A_n78A | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7A-26A_n78(2A) DC_3A-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| DC_3C-7A-26A_n78(2A) DC_3C-7C-26A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |
| DC_3A-7A-28A_n1A DC_3C-7A-28A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |
| DC_3A-7A-28A_n3A DC_3A-7C-28A_n3A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_3A-7A-28A_n5A DC_3A-7C-28A_n5A DC_3C-7A-28A_n5A DC_3C-7C-28A_n5A | DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_3A-7A-28A_n7A DC_3C-7A-28A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-3A-7A-28A_n7A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-7A-28A_n38A | 3A17 28A17 |
| DC_3A-7A_n28A-n38A | DC_3A_n28A17 |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_3A-7A-28A_n78A2, 9 DC_3A-7C-28A_n78A2, 9 DC_3C-7A-28A_n78A9 DC_3C-7C-28A_n78A9 | DC_3A_n78A9 DC_3C_n78A9 DC_7A_n78A9 DC_7C_n78A9 DC_28A_n78A9 |
| DC_3A-7A-28A_n78(2A) DC_3A-7C-28A_n78(2A) DC_3C-7A-28A_n78(2A)2 DC_3C-7C-28A_n78(2A)2 | DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A_n28A-n78A2, 9 DC_3A-7C_n28A-n78A9 DC_3C-7A_n28A-n78A9 DC_3C-7C_n28A-n78A9 | DC_3A_n28A DC_3C_n28A DC_3A_n78A9 DC_3C_n78A9 DC_7A_n28A DC_7A_n78A9 DC_7C_n28A DC_7C_n78A9 |
| DC_3A-7A-32A_n1A DC_3C-7A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A |
| DC_3A-7A-32A_n28A DC_3C-7A-32A_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-38A_n28A10 DC_3C-7A-38A_n28A10 | DC_3A_n28A DC_3C_n28A |
| DC_3A-7A-38A_n78A10 DC_3C-7A-38A_n78A10 | DC_3A_n78A DC_3C_n78A |
| DC_3A-7A_n38A-n78A | DC_3A_n78A |
| DC_3A-7A-40A_n1A DC_3A-7A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_40A_n1A |
| DC_3A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A-40A_n78(2A) DC_3A-7A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_3A-7A_n40A-n78A DC_3A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A-7A_n40A-n78A DC_3A-7A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A_n40A-n105A | DC_3A_n40A DC_3A_n105A DC_7A_n40A DC_7A_n105A |
| DC_3A-7A_n75A-n78A DC_3C-7A_n75A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-3A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-7A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-3A-7A-7A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_7A_n78A DC_7A_n79A |
| DC_3A-7A_n78A-n105A | DC_3A_n78A DC_3A_n105A DC_7A_n78A DC_7A_n105A |
| DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A |
| DC_3A-8A_n1A-n28A | DC_3A_n1A DC_8A_n1A DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A |
| DC_3A-8A_n1A-n77A DC_3A-8B_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_8A_n1A DC_8A_n77A |
| DC_3A-8A_n1A-n78A2,9 DC_3A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_3A-3A-8A_n1A-n78A2,9 DC_3A-3A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_3A-8A_n7A-n78A | DC_3A_n7A DC_3A_n78A DC_8A_n7A DC_8A_n78A |
| DC_(n)3AA-n8A-n77A DC_(n)3AA-n8A-n77(2A) | DC_(n)3AA4 DC_3A_n8A DC_3A_n77A |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_3A-8A-11A_n77A2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-11A_n77(2A) 2 DC_3A-8A-11A_n77(3A)2 | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-8A-20A_n28A DC_3C-8A-20A_n28A | DC_3A_n28A DC_3C_n28A DC_8A_n28A DC_20A_n28A |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_3A-8A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A_n28A-n77(2A)2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_3A-8A_n28A-n78A2 | DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-8A-32A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A-32A_n28A DC_3C-8A-32A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A-32A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-8A_n40A-n41A | DC_3A_n40A DC_3A_n41A DC_8A_n40A DC_8A_n41A |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A-40A_n78(2A) DC_3A-8A-40C_n78(2A) | DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-8A_n40A-n79A DC_3A-8A_n40A-n79C | DC_3A_n40A DC_3A_n79A DC_8A_n40A DC_8A_n79A |
| DC_3A-8A-41A_n1A DC_3A-8A-41C_n1A | DC_3A_n1A DC_8A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-8A-41A_n1A DC_3A-3A-8A-41C_n1A | DC_3A_n1A DC_8A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-8A-41A_n78A DC_3A-8A-41C_n78A | DC_3A_n78A DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-8A-41A_n78A DC_3A-3A-8A-41C_n78A | DC_3A_n78A DC_8A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-8A_n41A-n79A DC_3A-8A_n41A-n79C | DC_3A_n41A DC_3A_n79A DC_8A_n41A DC_8A_n79A |
| DC_3A-8A-42A_n77A7,8 DC_3A-8A-42C_n77A7,8 | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A |
| DC_3A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-18A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_3A-18A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_3A-18A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n28A-n78(2A) | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n77(2A) | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A_n41A-n78(2A) | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_3A-18A-42A_n77A7,8 DC_3A-18A-42C_n77A7,8 | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A-42A_n78A7,8 DC_3A-18A-42C_n78A7,8 | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A-42A_n79A DC_3A-18A-42C_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-19A-21A_n77A2 DC_3A-19A-21A_n77C2 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A2 DC_3A-19A-42C_n1A2 | DC_3A_n1A DC_19A_n1A DC_42A_n1A |
| DC_3A-19A-42A_n77A7,8,9 DC_3A-19A-42A_n77C7,8 DC_3A-19A-42C_n77A7,8,9 DC_3A-19A-42C_n77C7,8 DC_3A-19A-42D_n77A7,8 DC_3A-19A-42D_n77C7,8 | DC_3A_n77A9 DC_19A_n77A9 |
| DC_3A-19A-42A_n78A7,8,9 DC_3A-19A-42A_n78C7,8 DC_3A-19A-42C_n78A7,8,9 DC_3A-19A-42C_n78C7,8 DC_3A-19A-42D_n78A7,8 DC_3A-19A-42D_n78C7,8 | DC_3A_n78A9 DC_19A_n78A9 |
| DC_3A-19A-42A_n79A9 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A9 DC_3A-19A-42C_n79C DC_3A-19A-42D_n79A DC_3A-19A-42D_n79C | DC_3A_n79A9 DC_19A_n79A9 |
| DC_3A-19A_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_3A-19A_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A |
| DC_3A-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |
| DC_3C-20A_n1A-n28A8,14 | DC_3A_n1A DC_3A_n28A DC_3C_n28A DC_20A_n1A DC_3C_n1A DC_20A_n28A |
| DC_3A-20A_n1A-n75A | DC_3A_n1A DC_20A_n1A |
| DC_3C-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n1A-n78A DC_3A-3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-20A_n3A-n67A | DC_3A_n3A4 DC_20A_n3A |
| DC_3A-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A |
| DC_3C-20A_n7A-n28A8,14 | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_20A_n7A DC_20A_n28A |
| DC_3A-20A_n7A-n78A | DC_3A_n7A DC_3A_n78A DC_20A_n7A DC_20A_n78A |
| DC_3A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-20A_n28A-n75A | DC_3A_n28A DC_20A_n28A |
| DC_3C-20A_n28A-n75A | DC_20A_n28A DC_3A_n28A DC_3C_n28A |
| DC_3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-20A_n28A-n78A2,3,8,14 DC_3C-20A_n28A-n78A2,3,8,14 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-20A-32A_n1A DC_3C-20A-32A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A-32A_n7A | DC_3A_n7A DC_20A_n7A |
| DC_3A-20A-32A_n28A8,14 DC_3C-20A-32A_n28A8,14 | DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A-38A_n78A DC_3C-20A-38A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A DC_38A_n78A |
| DC_3A-20A-38A_n78(2A) | DC_3A_n78A DC_20A_n78A |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A |
| DC_3A-20A-40A_n78A DC_3A-20A-40C_n78A | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-40A_n78(2A) DC_3A-20A-40C_n78(2A) | DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_3A-20A-41A_n1A DC_3A-20A-41C_n1A | DC_3A_n1A DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-3A-20A-41A_n1A DC_3A-3A-20A-41C_n1A | DC_3A_n1A DC_20A_n1A DC_41A_n1A DC_41C_n1A |
| DC_3A-20A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_3A-20A-41A_n78A DC_3A-20A-41C_n78A | DC_3A_n78A DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-3A-20A-41A_n78A DC_3A-3A-20A-41C_n78A | DC_3A_n78A DC_20A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-20A-67A_n3A | DC_3A_n3A4 DC_20A_n3A |
| DC_3A-20A_SUL_n78A-n80A DC_3C-20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A |
| DC_3A-21A_n28A-n79A2 | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A |
| DC_3A-21A-42A_n1A2 DC_3A-21A-42C_n1A2 | DC_3A_n1A DC_21A_n1A DC_42A_n1A |
| DC_3A-21A_n1A-n77A2 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A_n1A-n78A2 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A_n1A-n79A2 | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-21A-42A_n77A7,8,9 DC_3A-21A-42A_n77C7,8 DC_3A-21A-42C_n77A7,8,9 DC_3A-21A-42C_n77C7,8 DC_3A-21A-42D_n77A7,8 DC_3A-21A-42D_n77C7,8 | DC_3A_n77A9 DC_21A_n77A9 |
| DC_3A-21A-42A_n78A7,8,9 DC_3A-21A-42A_n78C7,8 DC_3A-21A-42C_n78A7,8,9 DC_3A-21A-42C_n78C7,8 DC_3A-21A-42D_n78A7,8 DC_3A-21A-42D_n78C7,8 | DC_3A_n78A9 DC_21A_n78A9 |
| DC_3A-21A-42A_n79A9 DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A9 DC_3A-21A-42C_n79C DC_3A-21A-42D_n79A DC_3A-21A-42D_n79C | DC_3A_n79A9 DC_21A_n79A9 |
| DC_3A-21A_n77A-n79A9 | DC_3A_n77A9 DC_3A_n79A9 DC_21A_n77A9 DC_21A_n79A9 |
| DC_3A-21A_n78A-n79A9 | DC_3A_n78A9 DC_3A_n79A9 DC_21A_n78A9 DC_21A_n79A9 |
| DC_3A-28A_n1A-n5A | DC_3A_n1A DC_3A_n5A DC_28A_n1A DC_28A_n5A |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-28A_n1A-n78A2 | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n1A-n105A | DC_3A_n1A DC_3A_n105A DC_28A_n1A |
| DC_3A-28A_n3A-n78A2 | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_28A_n5A DC_28A_n40A |
| DC_3A-28A_n5A-n78A2 DC_3C-28A_n5A-n78A2 | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A_n5A-n105A | DC_3A_n5A DC_3A_n105A DC_28A_n5A |
| DC_3A-28A-(n)7AA DC_3C-28A-(n)7AA | DC_3A_n7A DC_28A_n7A |
| DC_3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7A-n78A | DC_3A_n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3C-28A_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3A-28A-32A_n1A | DC_3A_n1A DC_28A_n1A |
| DC_3A-28A-40A_n78A DC_3A-28A-40C_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_3A-28A_n38A-n78A | DC_3A_n38A DC_3A_n78A DC_28A_n38A DC_28A_n78A |
| DC_3A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-28A_n41A-n77A | DC_3A_n41A DC_28A_n41A DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-41A_n78A DC_3A-28A-41C_n78A | DC_3A_n78A DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-28A-42A_n77A7,8 DC_3A-28A-42A_n77C7,8 DC_3A-28A-42C_n77A7,8 DC_3A-28A-42C_n77C7,8 | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-42A_n78A7,8 DC_3A-28A-42A_n78C7,8 DC_3A-28A-42C_n78A7,8 DC_3A-28A-42C_n78C7,8 | DC_3A_n78A DC_28A_n78A |
| DC_3A-28A-42A_n79A DC_3A-28A-42A_n79C DC_3A-28A-42C_n79A DC_3A-28A-42C_n79C | DC_3A_n79A DC_28A_n79A |
| DC_3A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_3A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_3A-28A_n78A-n105A | DC_3A_n78A DC_3A_n105A DC_28A_n78A |
| DC_3A-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n28A DC_3C_n1A |
| DC_3A-32A_n1A-n78A DC_3C-32A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A |
| DC_3A-38A_n7A-n78A | DC_3A_n78A |
| DC_3A-32A-38A_n28A DC_3C-32A-38A_n28A | DC_3A_n28A DC_38A_n28A |
| DC_3A-38A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3C-38A_n28A-n78A | DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A_n40A-n41A-n79A | DC_3A_n40A DC_3A_n41A DC_3A_n79A |
| DC_3A_n40A-n78A-n105A | DC_3A_n40A DC_3A_n78A DC_3A_n105A |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_3A-41A_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_3A-41C_n3A-n77A | DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_3A-41A_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_3A-41C_n3A-n78A | DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_3A-41A_n28A-n77A9 | DC_3A_n28A DC_3A_n77A9 DC_41A_n28A DC_41A_n77A9 |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n77A7,8 DC_3A-41A-42C_n77A7,8 DC_3A-41C-42A_n77A7,8 DC_3A-41C-42C_n77A7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n77(2A)7,8 DC_3A-41A-42C_n77(2A)7,8 | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n78A7,8 DC_3A-41A-42C_n78A7,8 DC_3A-41C-42A_n78A7,8 DC_3A-41C-42C_n78A7,8 | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n79A DC_3A-41A-42C_n79A DC_3A-41C-42A_n79A DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A-42A_n1A-n77A7,8 DC_3A-42C_n1A-n77A7,8 | DC_3A_n1A DC_3A_n77A |
| DC_3A-42A_n1A-n78A7,8 DC_3A-42C_n1A-n78A7,8 | DC_3A_n1A DC_3A_n78A |
| DC_3A-42A_n1A-n79A DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A |
| DC_3A-42A_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42A_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_3A-42C_n28A-n77A7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42C_n28A-n77(2A)7,8 | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_3A-42A_n77A-n79A7,8,9 DC_3A-42C_n77A-n79A7,8,9 | DC_3A_n77A9 DC_3A_n79A9 |
| DC_3A-42A_n78A-n79A7,8,9 DC_3A-42C_n78A-n79A7,8,9 | DC_3A_n78A9 DC_3A_n79A9 |
| DC_5A-7A_n1A-n78A | DC_5A_n1A DC_5A_n78A DC_7A_n1A DC_7A_n78A |
| DC_5A-7A_n2A-n66A | DC_5A_n2A DC_5A_n66A DC_7A_n2A DC_7A_n66A |
| DC_5A-7A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_7A_n2A DC_7A_n77A |
| DC_5A-7A_n2A-n78A | DC_5A_n2A DC_7A_n2A DC_5A_n78A DC_7A_n78A |
| DC_5A-7A_n28A-n78A | DC_5A_n28A DC_5A_n78A DC_7A_n28A DC_7A_n78A |
| DC_5A-7A_n40A-n77A | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A-7A_n40A-n77A | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A-7A_n40A-n77(2A) | DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_5A-7A_n40A-n78A | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A_n40A-n78C | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A-7A_n40A-n78A DC_5A-7A-7A_n40A-n78C | DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_5A-7A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A DC_5A-7A-7A-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_5A-7A-(n)66AA DC_5A-7C-(n)66AA | DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_5A-7A-7A-(n)66AA | DC_5A_n66A DC_7A_n66A DC_(n)66AA4 |
| DC_5A-7A-66A_n77A | DC_5A_n77A DC_7A_n77A DC_66A_n77A |
| DC_5A-7A-66A_n77(2A) | DC_5A_n77A DC_7A_n77A DC_66A_n77A |
| DC_5A-7A_n66A-n77A | DC_5A_n66A DC_5A_n77A DC_7A_n66A DC_7A_n77A |
| DC_5A-7A-66A_n78A DC_5A-7C-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A-66A_n78A DC_5A-7C-66A-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A_n78(2A) | DC_5A_n78A DC_7A_n78A DC_66A_n78A |
| DC_5A-7A_n66A-n78A | DC_5A_n66A DC_7A_n66A DC_5A_n78A DC_7A_n78A |
| DC_5A-30A-66A_n2A DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_5A-30A-66A_n5A DC_5A-30A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_5A-30A-66A_n77A9 DC_5A-30A-66A-66A_n77A9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-30A-66A_n77(2A) 9 | DC_5A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_5A-48A-(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA4 |
| DC_5A-48A-66A_n12A | DC_5A_n12A DC_48A_n12A DC_66A_n12A |
| DC_5A-48A-66A_n71A | DC_5A_n71A DC_48A_n71A DC_66A_n71A |
| DC_5A-66A_n2A-n41A | DC_5A_n2A DC_5A_n41A DC_66A_n2A DC_66A_n41A |
| DC_5A-66A_n2A-n66A | DC_5A_n2A DC_5A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_5A-66A_n2A-n77A DC_5A-66A_n2A-n77C | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-66A_n5A-n77A DC_5A-66A_n5A-n77C | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_5A-48A-66A_n77A7,8,9 DC_5A-48A-66A_n77C7,8,9 DC_5A-48C-66A_n77A7,8,9 DC_5A-48C-66A_n77C7,8,9 | DC_5A_n77A9 DC_66A_n77A9 |
| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A |
| DC_5A-66A-(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |
| DC_5A-66A_n66A-n77A9 DC_5A-66A_n66A-n77C9 | DC_5A_n66A DC_5A_n77A9 DC_66A_n77A9 |
| DC_7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A_n1A-n40A-n78A | DC_7A_n1A DC_7A_n40A DC_7A_n78A |
| DC_7A_n1A-n75A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A-7A_n1A-n8A-n78A2 | DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A |
| DC_7A-8A_n1A-n78A2,9 DC_7A-8B_n1A-n78A2 | DC_7A_n1A DC_7A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_7A-7A-8A_n1A-n78A2,9 DC_7A-7A-8B_n1A-n78A2 | DC_7A_n1A DC_7A_n78A9 DC_8A_n1A DC_8A_n78A9 |
| DC_7A-8A_n7A-n78A | DC_7A_n7A DC_7A_n78A DC_8A_n7A DC_8A_n78A |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_7A-8A-20A_n28A | DC_7A_n28A |
| DC_7A-8A-20A_n78A | DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A-32A_n78A | DC_7A_n78A DC_8A_n78A |
| DC_7A-8A-38A_n1A | DC_8A_n1A |
| DC_7A-8A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A-40A_n78(2A) DC_7A-8A-40C_n78(2A) | DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_7A-12A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_12A_n2A DC_12A_n66A |
| DC_7A-12A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_12A_n2A DC_12A_n77A |
| DC_7A-12A_n2A-n78A | DC_7A_n2A DC_12A_n2A DC_7A_n78A DC_12A_n78A |
| DC_7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_7A-12A-66A_n25A | DC_7A_n25A DC_12A_n25A DC_66A_n25A |
| DC_7A-12A-66A_n66A | DC_7A_n66A DC_12A_n66A DC_66A_n66A |
| DC_7A-12A-66A_n77A | DC_7A_n77A DC_12A_n77A DC_66A_n77A |
| DC_7A-12A-66A_n77(2A) | DC_7A_n77A DC_12A_n77A DC_66A_n77A |
| DC_7A-12A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_12A_n66A DC_12A_n77A |
| DC_7A-12A-66A_n78A | DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_7A-12A-66A_n78(2A) | DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_7A-12A_n66A-n78A | DC_7A_n66A DC_12A_n66A DC_7A_n78A DC_12A_n78A |
| DC_7A-12A-71A_n77A | DC_7A_n77A DC_12A_n77A DC_71A_n77A |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_7A-13A-66A_n66A DC_7C-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-13A-(n)66AA DC_7C-13A-(n)66AA | DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_7A-7A-13A-(n)66AA | DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_7A-7A-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_7A-20A_n3A-n38A | DC_20A_n3A |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n8A-n78A | DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A_n3A DC_7C-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A-28A_n78A2 | DC_7A_n78A DC_20A_n78A DC_28A_n78A |
| DC_7A-20A_n28A-n78A2,3 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A-32A_n3A DC_7C-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |
| DC_7A-20A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A | DC_20A_n3A |
| DC_7A-20A-38A_n8A | DC_20A_n8A |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A |
| DC_7A-20A_n38A-n78A15 | DC_20A_n78A |
| DC_7A-25A-66A_n77A DC_7C-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-25A-66A_n77A DC_7C-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-7A-25A-25A-66A_n77A | DC_7A_n77A DC_25A_n77A DC_66A_n77A |
| DC_7A-25A-66A_n78A DC_7C-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-25A-25A-66A_n78A DC_7C-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-7A-25A-25A-66A_n78A | DC_7A_n78A DC_25A_n78A DC_66A_n78A |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A_n3A-n78A | DC_7A_n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A |
| DC_7C-28A_n3A-n78A | DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A-28A_n5A-n40A | DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_7A-28A_n5A-n78A DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A |
| DC_7A-28A-32A_n1A | DC_7A_n1A DC_28A_n1A |
| DC_7A-28A-32A_n3A DC_7C-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A-38A_n1A | DC_28A_n1A |
| DC_7A-28A-38A_n78A DC_7C-28A-38A_n78A | DC_28A_n78A |
| DC_7A-28A_n38A-n78A15 | DC_28A_n78A |
| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_7A-66A_n38A-n78A DC_7C-66A_n38A-n78A | DC_66A_n78A |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_7A-7A-(n)66AA-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n78A DC_(n)66AA2 |
| DC_7A-28A-66A_n7A | DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_7A-29A-66A_n78A DC_7C-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |
| DC_7A-32A_n1A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A_n40A-n78A-n105A | DC_7A_n40A DC_7A_n78A DC_7A_n105A |
| DC_7A-66A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_7A-66A_n2A-n71A | DC_7A_n2A DC_7A_n71A DC_66A_n2A DC_66A_n71A |
| DC_7A-66A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_66A_n2A DC_66A_n77A |
| DC_7A-66A_n2A-n78A | DC_7A_n2A DC_66A_n2A DC_7A_n78A DC_66A_n78A |
| DC_7A-66A_n12A-n77A | DC_7A_n12A DC_7A_n77A DC_66A_n12A DC_66A_n77A |
| DC_7A-66A_n12A-n78A | DC_7A_n12A DC_7A_n78A DC_66A_n12A DC_66A_n78A |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_7A-66A_n66A-n71A | DC_7A_n66A DC_7A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |
| DC_7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-(n)66AA-n78A DC_7C-(n)66AA-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n78A DC_(n)66AA2 |
| DC_7A-7A-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_7A-66A-71A_n25A | DC_7A_n25A DC_66A_n25A DC_71A_n25A |
| DC_7A-66A-71A_n66A | DC_7A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_7A-66A-71A_n77A | DC_7A_n77A DC_66A_n77A DC_71A_n77A |
| DC_7A-66A-71A_n77(2A) | DC_7A_n77A DC_66A_n77A DC_71A_n77A |
| DC_7A-66A_n71A-n77A | DC_7A_n71A DC_7A_n77A DC_66A_n71A DC_66A_n77A |
| DC_7A-66A-71A_n78A | DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_7A-66A-71A_n78(2A) | DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_7A-66A_n71A-n78A | DC_7A_n71A DC_66A_n71A DC_7A_n78A DC_66A_n78A |
| DC_7A-71A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_71A_n2A DC_71A_n66A |
| DC_7A-71A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_71A_n2A DC_71A_n77A |
| DC_7A-71A_n2A-n78A | DC_7A_n2A DC_71A_n2A DC_7A_n78A DC_71A_n78A |
| DC_7A-71A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_71A_n66A DC_71A_n77A |
| DC_7A-71A_n66A-n78A | DC_7A_n66A DC_71A_n66A DC_7A_n78A DC_71A_n78A |
| DC_8A_n1A-n3A-n77A | DC_8A_n1A DC_8A_n3A DC_8A_n77A |
| DC_8A-(n)3AA-n77A | DC_8A_n3A DC_8A_n77A DC_(n)3AA4 DC_3A_n77A |
| DC_8A-(n)3AA-n77(2A) | DC_8A_n3A DC_8A_n77A DC_(n)3AA4 DC_3A_n77A |
| DC_8A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n77(2A)2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_8A_n3A-n28A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_8A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A_n3A-n77(2A)-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n1A-n3A DC_8B-11A_n1A-n3A | DC_8A_n1A DC_8A_n3A |
| DC_8A-11A_n1A-n77A DC_8B-11A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_8A-11A_n3A-n77A2 DC_8B-11A_n3A-n77A2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n77(2A) 2 | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_8A-11A_n3A-n79A | DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_8A-11A_n28A-n77A2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n28A-n77(2A) 2 | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n77A-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-11A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-28A_n3A | DC_8A_n3A DC_20A_n3A DC_28A_n3A |
| DC_8A-20A-28A_n78A | DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_8A-20A-32A_n3A | DC_8A_n3A DC_20A_n3A |
| DC_8A_n28A-n77A-n79A | DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-32A-38A_n1A | DC_8A_n1A DC_38A_n1A |
| DC_8A_n39A-n40A-n41A | DC_8A_n39A DC_8A_n40A DC_8A_n41A |
| DC_8A_n39A-n40A-n79A | DC_8A_n39A DC_8A_n40A DC_8A_n79A |
| DC_8A_n39A-n41A-n79A | DC_8A_n39A DC_8A_n41A DC_8A_n79A |
| DC_8A_n40A-n41A-n79A | DC_8A_n40A DC_8A_n41A DC_8A_n79A |
| DC_8A-39A_n40A-n41A DC_8A-39A_n40A-n41C | DC_39A_n40A DC_39A_n41A DC_8A_n40A DC_8A_n41A |
| DC_8A-39A_n40A-n79A DC_8A-39A_n40A-n79C | DC_39A_n40A DC_39A_n79A DC_8A_n40A DC_8A_n79A |
| DC_8A-39A_n41A-n79A DC_8A-39A_n41C-n79A DC_8A-39A_n41A-n79C DC_8A-39A_n41C-n79C | DC_39A_n41A DC_39A_n79A DC_8A_n41A DC_8A_n79A |
| DC_8A-41A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A |
| DC_8A-41A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-41C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_8A-41A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41A_n77A |
| DC_8A-41C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A |
| DC_8A-42A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42A_n3A |
| DC_8A-42C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A |
| DC_8A-42A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A |
| DC_8A-42C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A DC_42C_n1A |
| DC_8A-42A_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A2 | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_11A_n3A-n28A-n77A2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n28A-n77(2A) 2 | DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_11A_n3A-n77A-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_11A_n3A-n77(2A)-n79A | DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A_n66A | DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_12A-30A-66A_n77A9 DC_12A-30A-66A-66A_n77A9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-30A-66A_n77(2A) 9 | DC_12A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_12A-48A-(n)5AA | DC_12A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_12A-48A-66A_n5A | DC_12A_n5A DC_48A_n5A DC_66A_n5A |
| DC_12A-66A-(n)5AA | DC_12A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_12A-66A_n2A-n41A | DC_12A_n2A DC_12A_n41A DC_66A_n2A DC_66A_n41A |
| DC_12A-66A_n2A-n66A | DC_12A_n2A DC_12A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_12A-66A_n2A-n77A | DC_12A_n2A DC_12A_n77A DC_66A_n2A DC_66A_n77A |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A |
| DC_12A-66A_n66A-n77A | DC_12A_n66A DC_12A_n77A DC_66A_n66A4 DC_66A_n77A |
| DC_13A-48A-66A_n77A9 DC_13A-48C-66A_n77A9 DC_13A-48A-66A_n77C9 DC_13A-48C-66A_n77C9 | DC_13A_n77A9 DC_66A_n77A9 |
| DC_13A-66A_n2A-n77A9 DC_13A-66A-66A_n2A-n77A9 DC_13A-66A_n2A-n77C9 | DC_13A_n2A DC_13A_n77A9 DC_66A_n2A DC_66A_n77A9 |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A |
| DC_13A-66A_n5A-n77A9 DC_13A-66A-66A_n5A-n77A9 DC_13A-66A_n5A-n77C9 DC_13A-66A-66A_n5A-n77C9 | DC_66A_n5A DC_13A_n77A9 DC_66A_n77A9 |
| DC_13A-66A_n66A-n77A9 DC_13A-66A_n66A-n77C | DC_13A_n66A DC_13A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_14A-30A-66A_n77A9 DC_14A-30A-66A-66A_n77A9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_14A-30A-66A_n77(2A) 9 | DC_14A_n77A9 DC_30A_n77A9 DC_66A_n77A9 |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_19A_n1A-n77A-n79A | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A_n1A-n78A-n79A | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_19A-21A_n1A-n77A2 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A_n1A-n78A2 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A_n1A-n79A2 | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n1A2 DC_19A-21A-42C_n1A2 | DC_19A_n1A DC_21A_n1A DC_42A_n1A |
| DC_19A-21A-42A_n77A9 DC_19A-21A-42A_n77C DC_19A-21A-42C_n77A9 DC_19A-21A-42C_n77C | DC_19A_n77A9 DC_21A_n77A9 |
| DC_19A-21A-42A_n78A9 DC_19A-21A-42A_n78C DC_19A-21A-42C_n78A9 DC_19A-21A-42C_n78C | DC_19A_n78A9 DC_21A_n78A9 |
| DC_19A-21A-42A_n79A9 DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A9 DC_19A-21A-42C_n79C | DC_19A_n79A9 DC_21A_n79A9 |
| DC_19A-21A_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_19A-21A_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_19A-42A_n1A-n77A DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A |
| DC_19A-42A_n1A-n78A DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A |
| DC_19A-42A_n1A-n79A DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A |
| DC_19A-42A_n77A-n79A9 DC_19A-42C_n77A-n79A9 | DC_19A_n77A9 DC_19A_n79A9 |
| DC_19A-42A_n78A-n79A9 DC_19A-42C_n78A-n79A9 | DC_19A_n78A9 DC_19A_n79A9 |
| DC_20A_n1A-n28A-n75A | DC_20A_n1A DC_20A_n28A |
| DC_20A-(n)3AA-n67A | DC_(n)3AA4 DC_20A_n3A |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |
| DC_20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| DC_20A-32A_n1A-n28A | DC_20A_n1A DC_20A_n28A |
| DC_20A-32A-38A_n1A | DC_20A_n1A DC_38A_n1A |
| DC_20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A DC_38A_n3A DC_38A_n78A |
| DC_20A-41A_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-41C_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-67A-(n)3AA | DC_(n)3AA4 DC_20A_n3A |
| DC_21A_n1A-n77A-n79A | DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_21A_n1A-n78A-n79A | DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_21A-28A-42A_n77A DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A-42A_n78A DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A-42A_n79A DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A |
| DC_21A_n28A-n77A-n79A | DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_21A_n28A-n78A-n79A | DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_21A-42A_n1A-n77A DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A |
| DC_21A-42A_n1A-n78A DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A |
| DC_21A-42A_n1A-n79A DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A-n79A9 DC_21A-42C_n77A-n79A9 | DC_21A_n77A9 DC_21A_n79A9 |
| DC_21A-42A_n78A-n79A9 DC_21A-42C_n78A-n79A9 | DC_21A_n78A9 DC_21A_n79A9 |
| DC_28A_n1A-n5A-n78A | DC_28A_n1A DC_28A_n5A DC_28A_n78A |
| DC_28A_n1A-n5A-n105A | DC_28A_n1A DC_28A_n5A |
| DC_28A_n1A-n40A-n78A | DC_28A_n1A DC_28A_n40A DC_28A_n78A |
| DC_28A_n1A-n78A-n105A | DC_28A_n1A DC_28A_n78A |
| DC_28A_n5A-n40A-n78A | DC_28A_n5A DC_28A_n40A DC_28A_n78A |
| DC_28A_n5A-n78A-n105A | DC_28A_n5A DC_28A_n78A |
| DC_28A-32A-38A_n1A | DC_28A_n1A DC_38A_n1A |
| DC_28A-41A-42A_n78A DC_28A-41C-42A_n78A DC_28A-41A-42C_n78A DC_28A-41C-42C_n78A | DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_29A-30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A_n66A | DC_30A_n66A DC_66A_n66A4 |
| DC_29A-30A-66A_n77A9 | DC_30A_n77A9 DC_66A_n77A9 |
| DC_30A-66A-(n)5AA | DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_42A_n1A-n77A-n79A7,8 | N/A |
| DC_42A_n1A-n78A-n79A7,8 | N/A |
| DC_42A_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42A_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42A_n28A |
| DC_42C_n3A-n28A-n77A7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_42C_n3A-n28A-n77(2A)7,8 | DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_46A-66A_n25A-n41A DC_46C-66A_n25A-n41A DC_46D-66A_n25A-n41A | DC_66A_n25A DC_66A_n41A |
| DC_46A-66A_n25A-n71A DC_46C-66A_n25A-n71A DC_46D-66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_46A-66A_n41A-n71A DC_46C-66A_n41A-n71A DC_46D-66A_n41A-n71A | DC_66A_n41A DC_66A_n71A |
| DC_46A-66A_n41(2A)-n71A DC_46C-66A_n41(2A)-n71A DC_46D-66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A |
| DC_66A-71A_n2A-n41A | DC_66A_n2A DC_66A_n41A DC_71A_n2A DC_71A_n41A |
| DC_66A-71A_n2A-n66A | DC_66A_n2A DC_66A_n66A4 DC_71A_n2A DC_71A_n66A |
| DC_66A-71A_n2A-n77A | DC_66A_n2A DC_66A_n77A DC_71A_n2A DC_71A_n77A |
| DC_66A-71A_n2A-n78A | DC_66A_n2A DC_71A_n2A DC_66A_n78A DC_71A_n78A |
| DC_66A-71A_n66A-n77A | DC_66A_n66A4 DC_66A_n77A DC_71A_n66A DC_71A_n77A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 9: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configuration. NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 11: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 12: Void. NOTE 13: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. NOTE 15: Band 7 and Band n38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 16: UL carrier shall be supported in Band 1 or band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 17: UL carrier shall be supported in Band 3 or band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

#### 5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_5A_n28A DC_7A_n28A |
| DC_1A-(n)3AA-n8A-n77A | DC_1A_n3A DC_1A_n8A DC_1A_n77A DC_(n)3AA3 DC_3A_n8A DC_3A_n77A |
| DC_1A-3A-5A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-3A-5A-7A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_5A_n40A DC_7A_n40A |
| DC_1A-3A-5A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A_n77(2A) DC_1A-3A-5A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n77A | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A-7A_n77(2A) DC_1A-3A-5A-7A-7A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A DC_7A_n77A |
| DC_1A-3A-5A-7A_n78A DC_1A-3C-5A-7A_n78A DC_1A-3A-5A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78A DC_1A-3A-5A-7A-7A_n78C | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78(A-C) | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_5A_n28A DC_5A_n78A |
| DC_1A-3A-5A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-3A-5A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A |
| DC_1A-3A-5A_n40A-n78A DC_1A-3A-5A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-3A-7A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-7C_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A |
| DC_1A-3A-7A_n5A-n78A DC_1A-3C-7A_n5A-n78A DC_1A-3A-7C_n5A-n78A DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3C-7A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-8A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_8A_n7A |
| DC_1A-3A-7A-8A_n28A DC_1A-3A-7A-8B_n78A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_8A_n28A |
| DC_1A-3A-7A-8A_n78A DC_1A-3C-7A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-3A-7A-8A_n78A2 DC_1A-3A-3A-7A-8B_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-7A-8A_n78A2 DC_1A-3A-7A-7A-8B_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-3A-7A-7A-8A_n78A2 DC_1A-3A-3A-7A-7A-8B_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-8A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A |
| DC_1A-3A-7A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-3A-7A-20A_n28A3 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-3A-7A-20A_n78A2 DC_1A-3A-7A-20A_n78C2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-1A-3A-7A-20A_n78A2 DC_1A-3A-3A-7A-20A_n78A2 DC_1A-3A-7A-7A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-20A_n78(2A)2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-26A_n78A DC_1A-3C-7A-26A_n78A DC_1A-3A-7C-26A_n78A DC_1A-3C-7C-26A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-3A-7A-26A_n78(2A) DC_1A-3A-7C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-3C-7A-26A_n78(2A) DC_1A-3C-7C-26A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_26A_n78A |
| DC_1A-3A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3C-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3C-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A-28A_n3A DC_1A-3A-7C-28A_n3A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_1A-3A-7A-28A_n5A DC_1A-3C-7A-28A_n5A DC_1A-3A-7C-28A_n5A DC_1A-3C-7C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-3A-7A-28A_n7A DC_1A-3C-7A-28A_n7A DC_1A-3A-3A-7A-28A_n7A DC_1A-1A-3C-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-3A-3A-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-3A-7A-28A_n38A | 1A7 3A7 28A7 |
| DC_1A-3A-7A_n28A-n38A | DC_1A_n28A7 DC_3A_n28A7 |
| DC_1A-3A-7A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-3A-7A-28A_n78(2A) DC_1A-3A-7C-28A_n78(2A) DC_1A-3C-7A-28A_n78(2A) DC_1A-3C-7C-28A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-1A-3A-7A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-7A_n28A-n78A2 DC_1A-3A-7C_n28A-n78A DC_1A-3C-7A_n28A-n78A DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-3A-7A-32A_n28A DC_1A-3C-7A-32A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A |
| DC_1A-3A-7A-32A_n78A DC_1A-3C-7A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-38A_n28A7 DC_1A-3C-7A-38A_n28A7 | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A-3A-7A-38A_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-7A_n38A-n78A7 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-7A-40A_n78A DC_1A-3A-7A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-3A-7A-40A_n78(2A) DC_1A-3A-7A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_40A_n78A |
| DC_1A-3A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_3A_n40A DC_3A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-3A-7A_n40A-n105A | DC_1A_n40A DC_1A_n105A DC_3A_n40A DC_3A_n105A DC_7A_n40A DC_7A_n105A |
| DC_1A-3A-7A_n75A-n78A DC_1A-3C-7A_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A_n78A-n105A | DC_1A_n78A DC_1A_n105A DC_3A_n78A DC_3A_n105A DC_7A_n78A DC_7A_n105A |
| DC_1A-3A-8A-40A_n78A DC_1A-3A-8A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-8A-40A_n78(2A) DC_1A-3A-8A-40C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-3A-7A_n40A-n78A DC_1A-3A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-3A-7A-7A_n40A-n78A DC_1A-3A-7A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-3A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_8A_n7A DC_8A_n78A |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |
| DC_1A-3A-8A-11A_n77A2 DC_1A-3A-8A-11A_n77(2A) 2 DC_1A-3A-8A-11A_n77(3A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-3A-8A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-3A-8A_n28A-n77A2 DC_1A-3A-8A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-3A-8A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A DC_28A_n78A |
| DC_1A-3A-8A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-3A-8A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-8A-42A_n77A DC_1A-3A-8A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A |
| DC_1A-3A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-3A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_18A_n3A DC_18A_n41A |
| DC_1A-3A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_18A_n3A DC_18A_n77A |
| DC_1A-3A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-3A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |
| DC_1A-3A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-3A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A1 DC_3A_n77A DC_18A_n28A DC_18A_n77A |
| DC_1A-3A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-3A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_3A_n28A1 DC_3A_n78A DC_18A_n28A DC_18A_n78A |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |
| DC_1A-3A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_1A-3A-18A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |
| DC_1A-3A-18A-42A_n77A DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A-42A_n78A DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A-42A_n79A DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A-21A_n77A2 DC_1A-3A-19A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-3A-19A-21A_n78A2 DC_1A-3A-19A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-3A-19A-21A_n79A2 DC_1A-3A-19A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-3A-19A-42A_n77A5,6,8 DC_1A-3A-19A-42A_n77C5,6 DC_1A-3A-19A-42C_n77A5,6,8 DC_1A-3A-19A-42C_n77C5,6 | DC_1A_n77A8 DC_3A_n77A8 DC_19A_n77A8 |
| DC_1A-3A-19A-42A_n78A5,6,8 DC_1A-3A-19A-42A_n78C5,6 DC_1A-3A-19A-42C_n78A5,6,8 DC_1A-3A-19A-42C_n78C5,6 | DC_1A_n78A8 DC_3A_n78A8 DC_19A_n78A8 |
| DC_1A-3A-19A-42A_n79A8 DC_1A-3A-19A-42A_n79C DC_1A-3A-19A-42C_n79A8 DC_1A-3A-19A-42C_n79C | DC_1A_n79A8 DC_3A_n79A8 DC_19A_n79A8 |
| DC_1A-3A-20A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_20A_n7A DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_3A_n8A DC_3A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-3A-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3C-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_1A-3A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-3A-3A-20A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-3A-20A_n28A-n78A2,3,6,11 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-20A-32A_n28A6,11 DC_1A-3C-20A-32A_n28A,6,11 | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_1A-3A-20A-32A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A-38A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_1A_n38A DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A-38A_n78(2A) | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A-40A_n78A DC_1A-3A-20A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_40A_n78A |
| DC_1A-3A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-3A-21A-42A_n77A5,6,8 DC_1A-3A-21A-42A_n77C5,6 DC_1A-3A-21A-42C_n77A5,6,8 DC_1A-3A-21A-42C_n77C5,6 | DC_1A_n77A8 DC_3A_n77A8 DC_21A_n77A8 |
| DC_1A-3A-21A-42A_n78A5,6,8 DC_1A-3A-21A-42A_n78C5,6 DC_1A-3A-21A-42C_n78A5,6,8 DC_1A-3A-21A-42C_n78C5,6 | DC_1A_n78A8 DC_3A_n78A8 DC_21A_n78A8 |
| DC_1A-3A-21A-42A_n79A8 DC_1A-3A-21A-42A_n79C DC_1A-3A-21A-42C_n79A8 DC_1A-3A-21A-42C_n79C | DC_1A_n79A8 DC_3A_n79A8 DC_21A_n79A8 |
| DC_1A-3A-21A_n77A-n79A8 | DC_3A_n77A8 DC_3A_n79A8 |
| DC_1A-3A-21A_n78A-n79A8 | DC_3A_n78A8 DC_3A_n79A8 |
| DC_1A-3A-28A_n3A-n78A2 | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_3A_n5A DC_3A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-3A-28A_n5A-n78A2 DC_1A-3C-28A_n5A-n78A2 | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-28A-(n)7AA DC_1A-3C-28A-(n)7AA | DC_1A_n7A DC_3A_n7A DC_28A_n7A |
| DC_1A-3A-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7B-n78A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_1A_n7B DC_3A_n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3A-28A-40A_n78A DC_1A-3A-28A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A DC_40A_n78A |
| DC_1A-3A-28A_n38A-n78A | DC_1A_n38A DC_1A_n78A DC_3A_n38A DC_3A_n78A DC_28A_n38A DC_28A_n78A |
| DC_1A-3A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-28A-42A_n77A DC_1A-3A-28A-42A_n77C DC_1A-3A-28A-42C_n77A DC_1A-3A-28A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A-28A-42A_n78A DC_1A-3A-28A-42A_n78C DC_1A-3A-28A-42C_n78A DC_1A-3A-28A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A-42A_n79A DC_1A-3A-28A-42A_n79C DC_1A-3A-28A-42C_n79A DC_1A-3A-28A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_3A_n28A DC_3A_n77A DC_3A_n79A |
| DC_1A_n3A-n28A-n77A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_1A_n79A |
| DC_1A-3A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_3A_n28A DC_3A_n78A DC_3A_n79A |
| DC_1A-3A-38A_n7A-n78A7 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3A_n40A-n78A-n105A | DC_1A_n40A DC_1A_n78A DC_1A_n105A DC_3A_n40A DC_3A_n78A DC_3A_n105A |
| DC_1A-3C-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-3A-41C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A4 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-3A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-3A-41C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A4 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-3A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_41A_n28A |
| DC_1A-3A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-3A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-3A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-3A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-3A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n77A5,6 DC_1A-3A-41A-42C_n77A5,6 DC_1A-3A-41C-42A_n77A5,6 DC_1A-3A-41C-42C_n77A5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n77(2A)5,6 DC_1A-3A-41A-42C_n77(2A)5,6 | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n78A5,6 DC_1A-3A-41A-42C_n78A5,6 DC_1A-3A-41C-42A_n78A5,6 DC_1A-3A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n79A DC_1A-3A-41A-42C_n79A DC_1A-3A-41C-42A_n79A DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_1A-3A-42A_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A |
| DC_1A-3A-42C_n28A-n77A5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-3A-42C_n28A-n77(2A)5,6 | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-5A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_5A_n28A DC_5A_n78A DC_7A_n28A DC_7A_n78A |
| DC_1A-5A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-5A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-5A-7A-7A_n40A-n77A | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-5A-7A-7A_n40A-n77(2A) | DC_1A_n40A DC_1A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_1A-5A-7A_n40A-n78A DC_1A-5A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-5A-7A-7A_n40A-n78A DC_1A-5A-7A-7A_n40A-n78C | DC_1A_n40A DC_1A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_1A-7A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_7A_n7A DC_7A_n78A DC_8A_n7A DC_8A_n78A |
| DC_1A-7A-8A-20A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A DC_20A_n3A |
| DC_1A-7A-8A-20A_n28A3,9 | DC_1A_n28A DC_7A_n28A DC_8A_n28A DC_20A_n28A |
| DC_1A-7A-8A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-7A-8A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_1A-7A-8A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-8A-40A_n78A DC_1A-7A-8A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-7A-8A-40A_n78(2A) DC_1A-7A-8A-40C_n78(2A) | DC_1A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_20A_n3A DC_20A_n78A |
| DC_1A-7A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A-n78A | DC_1A_n8A DC_1A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_1A-7A-20A-28A_n3A DC_1A-7C-20A-28A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-7A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-7A-20A-32A_n3A DC_1A-7C-20A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_20A_n3A |
| DC_1A-7A-20A-32A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A-32A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-20A-32A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-7A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-7A-20A-38A_n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-7A-20A_n38A-n78A | DC_1A_n78A DC_20A_n78A |
| DC_1A-7A-28A_n3A-n78A DC_1A-7C-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-7A-28A_n5A-n40A | DC_1A_n5A DC_1A_n40A DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_1A-7A-28A_n5A-n78A DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-7A-28A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A-28A-32A_n3A DC_1A-7C-28A-32A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |
| DC_1A-7A-28A_n38A-n78A | DC_1A_n78A DC_28A_n78A |
| DC_1A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-7A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A |
| DC_1A-7A_n40A-n78A-n105A | DC_1A_n40A DC_1A_n78A DC_1A_n105A DC_7A_n40A DC_7A_n78A DC_7A_n105A |
| DC_1A-8A-(n)3AA-n77A DC_1A-8A-(n)3AA-n77(2A) | DC_1A_n3A DC_1A_n77A DC_(n)3AA4 DC_3A_n77A DC_8A_n3A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_8A_n3A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n3A-n28A-n79A | DC_1A_n3A DC_1A_n28A DC_1A_n79A DC_8A_n3A DC_8A_n28A DC_8A_n79A |
| DC_1A-8A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_8A_n3A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A |
| DC_1A-8A-11A_n3A-n77A2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n77(2A) 2 | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A |
| DC_1A-8A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A |
| DC_1A-8A-11A_n28A-n77A2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n28A-n77(2A) 2 | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_1A-8A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-8A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A |
| DC_1A-8A-20A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_1A-8A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_1A-8A-42A_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A |
| DC_1A-8A-42C_n3A-n28A2 | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |
| DC_1A-8A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A |
| DC_1A-8A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_1A-8A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A |
| DC_1A-11A_n3A-n28A-n77A2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n28A-n77(2A) 2 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_1A-11A_n3A-n77A-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-11A_n3A-n77(2A)-n79A | DC_1A_n3A DC_1A_n77A DC_1A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_1A-19A-21A-42A_n77A5,6,8 DC_1A-19A-21A-42A_n77C5,6 DC_1A-19A-21A-42C_n77A5,6,8 DC_1A-19A-21A-42C_n77C5,6 | DC_1A_n77A8 DC_19A_n77A8 DC_21A_n77A8 |
| DC_1A-19A-21A-42A_n78A5,6,8 DC_1A-19A-21A-42A_n78C5,6 DC_1A-19A-21A-42C_n78A5,6,8 DC_1A-19A-21A-42C_n78C5,6 | DC_1A_n78A8 DC_19A_n78A8 DC_21A_n78A8 |
| DC_1A-19A-21A-42A_n79A8 DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A8 DC_1A-19A-21A-42C_n79C | DC_1A_n79A8 DC_19A_n79A8 DC_21A_n79A8 |
| DC_1A-19A-42A_n77A-n79A5,6,8 DC_1A-19A-42C_n77A-n79A5,6,8 | DC_19A_n77A8 DC_19A_n79A8 |
| DC_1A-19A-42A_n78A-n79A5,6,8 DC_1A-19A-42C_n78A-n79A5,6,8 | DC_1A_n78A DC_1A_n79A DC_19A_n78A8 DC_19A_n79A8 |
| DC_1A-20A-28A-32A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |
| DC_1A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_38A_n3A DC_1A_n78A DC_20A_n78A DC_38A_n78A |
| DC_1A-21A-28A-42A_n77A5,6 DC_1A-21A-28A-42C_n77A5,6 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A-28A-42A_n78A5,6 DC_1A-21A-28A-42C_n78A5,6 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A-28A-42A_n79A DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A_n28A-n77A-n79A | DC_1A_n28A DC_1A_n77A DC_1A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_1A-21A_n28A-n78A-n79A | DC_1A_n28A DC_1A_n78A DC_1A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_1A-21A-42A_n77A-n79A5,6,8 DC_1A-21A-42C_n77A-n79A5,6,8 | DC_1A_n77A8 DC_1A_n79A8 |
| DC_1A-21A-42A_n78A-n79A5,6,8 DC_1A-21A-42C_n78A-n79A5,6,8 | DC_1A_n78A8 DC_1A_n79A8 DC_21A_n78A DC_21A_n79A |
| DC_1A-42A_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42A_n3A-n28A-n77(2A)5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42A_n28A |
| DC_1A-42C_n3A-n28A-n77A5,6 | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_1A-42C_n3A-n28A-n77(2A) | DC_1A_n3A DC_1A_n28A DC_1A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_2A-5A-7A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_5A_n2A DC_5A_n66A DC_7A_n2A DC_7A_n66A |
| DC_2A-5A-7A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_5A_n2A DC_5A_n77A DC_7A_n2A DC_7A_n77A |
| DC_2A-5A-7A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_7A_n2A DC_7A_n78A |
| DC_2A-5A-7A-66A_n2A | DC_5A_n2A DC_7A_n2A DC_66A_n2A |
| DC_2A-5A-7A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-5A-7A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A4 DC_66A_n7A |
| DC_2A-5A-7A-66A_n66A DC_2A-5A-7C-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-5A-7A-7A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-5A-7A-66A_n77A | DC_2A_n77A DC_5A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-5A-7A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A DC_7A_n66A DC_7A_n77A |
| DC_2A-5A-7A-66A_n77(2A) | DC_2A_n77A DC_5A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-5A-7A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-5A-7A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_5A_n66A DC_5A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-5A-30A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-5A-30A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-5A-30A-66A_n77A8 | DC_2A_n77A8 DC_5A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-5A-66A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_5A_n2A DC_5A_n41A DC_66A_n2A DC_66A_n41A |
| DC_2A-5A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-5A-66A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_5A_n2A DC_5A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_2A-5A-66A_n2A-n77A8 DC_2A-5A-66A-66A_n2A-n77A8 | DC_5A_n2A DC_2A_n77A8 DC_5A_n77A8 DC_66A_n2A DC_66A_n77A8 |
| DC_2A-5A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_5A_n2A DC_5A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-5A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_5A_n66A DC_5A_n77A8 DC_66A_n77A8 |
| DC_2A-7A-12A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_7A_n2A DC_7A_n66A DC_12A_n2A DC_12A_n66A |
| DC_2A-7A-12A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_7A_n2A DC_7A_n77A DC_12A_n2A DC_12A_n77A |
| DC_2A-7A-12A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_12A_n2A DC_12A_n78A |
| DC_2A-7A-12A-66A_n2A | DC_7A_n2A DC_12A_n2A DC_66A_n2A |
| DC_2A-7A-12A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-7A-12A-66A_n77A | DC_2A_n77A DC_7A_n77A DC_12A_n77A DC_66A_n77A |
| DC_2A-7A-12A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_7A_n66A DC_7A_n77A DC_12A_n66A DC_12A_n77A |
| DC_2A-7A-12A-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_12A_n77A DC_66A_n77A |
| DC_2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-2A-7A-12A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_12A_n78A DC_66A_n78A |
| DC_2A-7A-12A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_12A_n66A DC_12A_n78A |
| DC_2A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7C-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A |
| DC_2A-7A-13A-66A_n66A DC_2A-7C-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-13A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-7A-7A-13A-(n)66AA | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_(n)66AA4 |
| DC_2A-7A-7A-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-28A-66A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A DC_66A_n7A |
| DC_2A-7A-28A-66A_n66A DC_2A-7C-28A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A DC_66A_n66A4 |
| DC_2A-7A-29A-66A_n78A DC_2A-7C-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-29A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-66A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_2A-7A-66A_n2A-n71A | DC_2A_n2A4 DC_2A_n71A DC_7A_n2A DC_7A_n71A DC_66A_n2A DC_66A_n71A |
| DC_2A-7A-66A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_7A_n2A DC_7A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-7A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7A-7A-66A_n25A-n66A6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7C-66A_n25A-n66A,6,11 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A |
| DC_2A-7A-66A_n66A-n71A | DC_2A_n66A DC_2A_n71A DC_7A_n66A DC_7A_n71A DC_66A_n66A4 DC_66A_n71A |
| DC_2A-7A-66A_n66A-n77A DC_2A-7A-7A-66A_n66A-n77A DC_2A-7C-66A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A DC_66A_n77A |
| DC_2A-7A-66A_n66A-n78A DC_2A-7C-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-7A-(n)66AA-n78A DC_2A-7C-(n)66AA-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n78A DC_(n)66AA4 |
| DC_2A-7A-7A-(n)66AA-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n78A DC_(n)66AA4 |
| DC_2A-7A-7A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |
| DC_2A-7A-66A-71A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_2A-7A-66A-71A_n77A | DC_2A_n77A DC_7A_n77A DC_66A_n77A DC_71A_n77A |
| DC_2A-7A-66A_n71A-n77A | DC_2A_n71A DC_2A_n77A DC_7A_n71A DC_7A_n77A DC_66A_n71A DC_66A_n77A |
| DC_2A-7A-66A-71A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A DC_71A_n77A |
| DC_2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-7A-66A-71A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-2A-7A-66A-71A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-7A-66A_n71A-n78A | DC_2A_n71A DC_2A_n78A DC_7A_n71A DC_7A_n78A DC_66A_n71A DC_66A_n78A |
| DC_2A-7A-71A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_7A_n2A DC_7A_n66A DC_71A_n2A DC_71A_n66A |
| DC_2A-7A-71A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_7A_n2A DC_7A_n77A DC_71A_n2A DC_71A_n77A |
| DC_2A-7A-71A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_7A_n2A DC_7A_n78A DC_71A_n2A DC_71A_n78A |
| DC_2A-7A-71A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_7A_n66A DC_7A_n77A DC_71A_n66A DC_71A_n77A |
| DC_2A-7A-71A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_71A_n66A DC_71A_n78A |
| DC_2A-12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-12A-30A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-12A-30A-66A_n77A8 | DC_2A_n77A8 DC_12A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-12A-66A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_12A_n2A DC_12A_n41A DC_66A_n2A DC_66A_n41A |
| DC_2A-12A-66A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_12A_n2A DC_12A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_2A-12A-66A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_12A_n2A DC_12A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-12A-66A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_12A_n2A DC_12A_n78A DC_66A_n2A DC_66A_n78A |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-12A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_12A_n66A DC_12A_n77A DC_66A_n66A4 DC_66A_n77A |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-13A-66A_n66A-n77A8 DC_2A-2A-13A-66A_n66A-n77A8 | DC_2A_n66A DC_2A_n77A8 DC_13A_n66A DC_13A_n77A8 DC_66A_n77A8 |
| DC_2A-14A-30A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-14A-30A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-14A-30A-66A_n77A8 | DC_2A_n77A8 DC_14A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-29A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-29A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A-66A_n77A8 | DC_2A_n77A8 DC_30A_n77A8 DC_66A_n77A8 |
| DC_2A-30A-66A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-46A-66A_n41A-n71A DC_2A-46C-66A_n41A-n71A DC_2A-46D-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A-71A_n2A-n41A | DC_2A_n2A4 DC_2A_n41A DC_66A_n2A DC_66A_n41A DC_71A_n2A DC_71A_n41A |
| DC_2A-66A-71A_n2A-n66A | DC_2A_n2A4 DC_2A_n66A DC_66A_n2A DC_66A_n66A4 DC_71A_n2A DC_71A_n66A |
| DC_2A-66A-71A_n2A-n77A | DC_2A_n2A4 DC_2A_n77A DC_66A_n2A DC_66A_n77A DC_71A_n2A DC_71A_n77A |
| DC_2A-66A-71A_n2A-n78A | DC_2A_n2A4 DC_2A_n78A DC_66A_n2A DC_66A_n78A DC_71A_n2A DC_71A_n78A |
| DC_2A-66A-71A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_66A_n66A4 DC_66A_n77A DC_71A_n66A DC_71A_n77A |
| DC_3A_n1A-n5A-n78A-n105A | DC_3A_n1A DC_3A_n5A DC_3A_n78A DC_3A_n105A |
| DC_3A-5A-7A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_5A_n28A DC_5A_n78A DC_7A_n28A DC_7A_n78A |
| DC_3A-5A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-5A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-5A-7A-7A_n40A-n77A | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-5A-7A-7A_n40A-n77(2A) | DC_3A_n40A DC_3A_n77A DC_5A_n40A DC_5A_n77A DC_7A_n40A DC_7A_n77A |
| DC_3A-5A-7A_n40A-n78A DC_3A-5A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A_n1A-n75A-n78A DC_3C-7A_n1A-n75A-n78A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-5A-7A-7A_n40A-n78A DC_3A-5A-7A-7A_n40A-n78C | DC_3A_n40A DC_3A_n78A DC_5A_n40A DC_5A_n78A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A_n1A-n40A-n78A | DC_3A_n1A DC_3A_n40A DC_3A_n78A DC_7A_n1A DC_7A_n40A DC_7A_n78A |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A |
| DC_3A-7A-8A_n1A-n78A2,8 DC_3A-7A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A8 DC_7A_n1A DC_7A_n78A8 DC_8A_n1A DC_8A_n78A8 |
| DC_3A-3A-7A-8A_n1A-n78A2,8 DC_3A-7A-7A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A8 DC_7A_n1A DC_7A_n78A8 DC_8A_n1A DC_8A_n78A8 |
| DC_3A-7A-7A-8A_n1A-n78A2,8 DC_3A-3A-7A-8B_n1A-n78A2 DC_3A-3A-7A-7A-8B_n1A-n78A2 | DC_3A_n1A DC_3A_n78A8 DC_7A_n1A DC_7A_n78A8 DC_8A_n1A DC_8A_n78A8 |
| DC_3A-3A-7A-7A-8A_n1A-n78A2,8 | DC_3A_n1A DC_3A_n78A8 DC_7A_n1A DC_7A_n78A8 DC_8A_n1A DC_8A_n78A8 |
| DC_3A-7A-8A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_7A_n7A DC_7A_n78A DC_8A_n7A DC_8A_n78A |
| DC_3A-7A-8A-20A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_3A-7A-8A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_8A_n28A DC_8A_n78A |
| DC_3A-7A-8A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_3A-7A-8A-40A_n1A DC_3A-7A-8A-40C_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_40A_n1A |
| DC_3A-7A-8A-40A_n78A DC_3A-7A-8A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A-40A_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A-40C_n78(2A) | DC_3A_n78A DC_7A_n78A DC_8A_n78A DC_40A_n78A |
| DC_3A-7A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A |
| DC_3A-7A-20A_n1A-n75A DC_3C-7A-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_20A_n1A |
| DC_3A-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3C-7A-20A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_3A_n78A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A |
| DC_3A-7A-20A_n8A-n78A | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A DC_20A_n8A DC_20A_n78A |
| DC_3A-7A-20A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_3A-7A-20A-28A_n78A | DC_3A_n78A DC_7A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-7A-20A_n28A-n78A2,3,6,11 DC_3C-7A-20A_n28A-n78A2,3,6,11 | DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_3C_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-7A-20A-32A_n1A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |
| DC_3A-7A-20A-32A_n78A | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-20A_n38A-n78A DC_3A-7A-20A-38A_n78A DC_3C-7A-20A-38A_n78A | DC_3A_n78A DC_3C_n78A DC_20A_n78A |
| DC_3A-7A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A |
| DC_3A-7A-28A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_28A_n1A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7C-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_3A-7A-28A_n5A-n40A | DC_3A_n5A DC_3A_n40A DC_7A_n5A DC_7A_n40A DC_28A_n5A DC_28A_n40A |
| DC_3A-7A-28A_n5A-n78A DC_3C-7A-28A_n5A-n78A DC_3A-7C-28A_n5A-n78A DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-7A-28A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3C-7A-28A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n38A-n78A | DC_3A_n78A DC_28A_n78A |
| DC_3A-7A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-7A-32A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_3A_n78A DC_7A_n78A |
| DC_3C-7A-32A_n1A-n78A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-7A_n40A-n78A-n105A | DC_3A_n40A DC_3A_n78A DC_3A_n105A DC_7A_n40A DC_7A_n78A DC_7A_n105A |
| DC_3A-8A-11A_n28A-n77A2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-8A-11A_n28A-n77(2A) 2 | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A |
| DC_3A-8A-20A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A DC_28A_n78A |
| DC_3A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-19A-21A-42A_n77A5,6 DC_3A-19A-21A-42A_n77C5,6 DC_3A-19A-21A-42C_n77A5,6 DC_3A-19A-21A-42C_n77C5,6 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42A_n78A5,6 DC_3A-19A-21A-42A_n78C5,6 DC_3A-19A-21A-42C_n78A5,6 DC_3A-19A-21A-42C_n78C5,6 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A DC_3A-19A-21A-42A_n79C DC_3A-19A-21A-42C_n79A DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n1A-n77A5,6 DC_3A-19A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A |
| DC_3A-19A-42A_n1A-n78A5,6 DC_3A-19A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A |
| DC_3A-19A-42A_n1A-n79A DC_3A-19A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A |
| DC_3A-20A_n1A-n28A-n75A DC_3C-20A_n1A-n28A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A-32A_n1A-n28A | DC_3A_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A |
| DC_3C-20A-32A_n1A-n28A | DC_3A_n1A DC_3C_n1A DC_20A_n1A DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-21A_n1A-n77A-n79A | DC_3A_n1A DC_3A_n77A DC_3A_n79A DC_21A_n1A DC_21A_n77A DC_21A_n79A |
| DC_3A-21A_n1A-n78A-n79A | DC_3A_n1A DC_3A_n78A DC_3A_n79A DC_21A_n1A DC_21A_n78A DC_21A_n79A |
| DC_3A-21A_n28A-n77A-n79A | DC_3A_n28A DC_3A_n77A DC_3A_n79A DC_21A_n28A DC_21A_n77A DC_21A_n79A |
| DC_3A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_3A-3A-7A_n1A-n8A-n78A2 DC_3A-7A-7A_n1A-n8A-n78A2 DC_3A-3A-7A-7A_n1A-n8A-n78A2 | DC_3A_n1A DC_3A_n8A DC_3A_n78A DC_7A_n1A DC_7A_n8A DC_7A_n78A |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-21A_n28A-n78A-n79A | DC_3A_n28A DC_3A_n78A DC_3A_n79A DC_21A_n28A DC_21A_n78A DC_21A_n79A |
| DC_3A-21A-42A_n1A-n77A5,6 DC_3A-21A-42C_n1A-n77A5,6 | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A |
| DC_3A-21A-42A_n1A-n78A5,6 DC_3A-21A-42C_n1A-n78A5,6 | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A |
| DC_3A-21A-42A_n1A-n79A DC_3A-21A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A |
| DC_3A-28A_n1A-n5A-n78A | DC_3A_n1A DC_3A_n5A DC_3A_n78A DC_28A_n1A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A_n1A-n5A-n105A | DC_3A_n1A DC_3A_n5A DC_3A_n105A DC_28A_n1A DC_28A_n5A |
| DC_3A-28A_n1A-n40A-n78A | DC_3A_n1A DC_3A_n40A DC_3A_n78A DC_28A_n1A DC_28A_n40A DC_28A_n78A |
| DC_3A-28A_n1A-n78A-n105A | DC_3A_n1A DC_3A_n78A DC_3A_n105A DC_28A_n1A DC_28A_n78A |
| DC_3A-28A_n5A-n78A-n105A | DC_3A_n5A DC_3A_n78A DC_3A_n105A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A-41A-42A_n78A5,6 DC_3A-28A-41A-42C_n78A5,6 DC_3A-28A-41C-42A_n78A5,6 DC_3A-28A-41C-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_5A-7A-66A_n2A-n66A | DC_5A_n2A DC_5A_n66A DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_5A-7A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_7A_n2A DC_7A_n77A DC_66A_n2A DC_66A_n77A |
| DC_5A-7A-66A_n2A-n78A | DC_5A_n2A DC_5A_n78A DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A |
| DC_5A-7A-66A_n66A-n77A | DC_5A_n66A DC_5A_n77A DC_7A_n66A DC_7A_n77A DC_66A_n66A4 DC_66A_n77A |
| DC_7A-8A-20A-32A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A |
| DC_7A-8A-32A-38A_n1A | DC_8A_n1A |
| DC_7A-8A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-8A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A |
| DC_7A-12A-66A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_12A_n2A DC_12A_n66A DC_66A_n2A DC_66A_n66A4 |
| DC_7A-12A-66A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_12A_n2A DC_12A_n77A DC_66A_n2A DC_66A_n77A |
| DC_7A-12A-66A_n2A-n78A | DC_7A_n2A DC_7A_n78A DC_12A_n2A DC_12A_n78A DC_66A_n2A DC_66A_n78A |
| DC_7A-12A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_12A_n66A DC_12A_n77A DC_66A_n66A4 DC_66A_n77A |
| DC_7A-20A-28A-32A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-28A-32A_n3A DC_7C-20A-28A-32A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |
| DC_7A-20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A |
| DC_7A-20A-32A-38A_n1A | DC_20A_n1A |
| DC_7A-20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_7A-28A_n1A-n40A-n78A | DC_7A_n1A DC_7A_n40A DC_7A_n78A DC_28A_n1A DC_28A_n40A DC_28A_n78A |
| DC_7A-28A-32A-38A_n1A | DC_28A_n1A |
| DC_7A-66A-71A_n2A-n66A | DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A4 DC_71A_n2A DC_71A_n66A |
| DC_7A-66A-71A_n2A-n77A | DC_7A_n2A DC_7A_n77A DC_66A_n2A DC_66A_n77A DC_71A_n2A DC_71A_n77A |
| DC_7A-66A-71A_n2A-n78A | DC_7A_n2A DC_7A_n78A DC_66A_n2A DC_66A_n78A DC_71A_n2A DC_71A_n78A |
| DC_7A-66A-71A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n66A4 DC_66A_n77A DC_71A_n66A DC_71A_n77A |
| DC_8A_n3A-n28A-n77A-n79A | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_8A_n79A |
| DC_8A-11A_n3A-n28A-n77A2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n28A-n77(2A) 2 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_11A_n3A DC_11A_n28A DC_11A_n77A |
| DC_8A-11A_n3A-n77A-n79A | DC_8A_n3A DC_8A_n77A DC_8A_n79A DC_11A_n3A DC_11A_n77A DC_11A_n79A |
| DC_8A-20A-32A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |
| DC_8A-42A_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42A_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42A_n28A |
| DC_8A-42C_n3A-n28A-n77A5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_8A-42C_n3A-n28A-n77(2A)5,6 | DC_8A_n3A DC_8A_n28A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A |
| DC_19A-21A-42A_n1A-n77A5,6 DC_19A-21A-42C_n1A-n77A5,6 | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A |
| DC_19A-21A-42A_n1A-n78A5,6 DC_19A-21A-42C_n1A-n78A5,6 | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A |
| DC_19A-21A-42A_n1A-n79A DC_19A-21A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A |
| DC_19A-21A-42A_n77A-n79A5,6,8 DC_19A-21A-42C_n77A-n79A5,6,8 | DC_19A_n77A8 DC_19A_n79A8 |
| DC_19A-21A-42A_n78A-n79A5,6,8 DC_19A-21A-42C_n78A-n79A5,6,8 | DC_19A_n78A8 DC_19A_n79A8 |
| DC_19A-42A_n1A-n77A-n79A5,6 | DC_19A_n1A DC_19A_n77A DC_19A_n79A |
| DC_19A-42A_n1A-n78A-n79A5,6 | DC_19A_n1A DC_19A_n78A DC_19A_n79A |
| DC_20A-28A-32A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |
| DC_28A_n1A-n5A-n78A-n105A | DC_28A_n1A DC_28A_n5A DC_28A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Only single switched UL is supported NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 8: Minimum requirements for PC2 are applicable for this uplink EN-DC configuration in this downlink/uplink EN-DC configurations. NOTE 9: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

< Unchanged sections omitted>

#### 5.5B.7.2	Inter-band NR-DC configurations between FR1 and FR2 (three bands)

Table 5.5B.7-2: Inter-band NR-DC configurations between FR1 and FR2 (three bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n3A-n257A DC_n1A-n3A-n257G DC_n1A-n3A-n257H DC_n1A-n3A-n257I | DC_n1A-n3A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I |
| DC_n1A-n3A-n258A DC_n1A-n3A-n258D DC_n1A-n3A-n258G DC_n1A-n3A-n258H DC_n1A-n3A-n258I DC_n1A-n3A-n258J | DC_n1A-n3A DC_n1A-n258A DC_n1A-n258D DC_n1A-n258G DC_n1A-n258H DC_n1A-n258I DC_n1A-n258J DC_n3A-n258A DC_n3A-n258D DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n3A-n258J |
| DC_n1A-n18A-n257A DC_n1A-n18A-n257G DC_n1A-n18A-n257H DC_n1A-n18A-n257I | DC_n1A-n18A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I |
| DC_n1A-n28A-n257A DC_n1A-n28A-n257G DC_n1A-n28A-n257H DC_n1A-n28A-n257I | DC_n1A-n28A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n1A-n28A-n258A DC_n1A-n28A-n258D DC_n1A-n28A-n258G DC_n1A-n28A-n258H DC_n1A-n28A-n258I DC_n1A-n28A-n258J | DC_n1A-n28A DC_n1A-n258A DC_n1A-n258D DC_n1A-n258G DC_n1A-n258H DC_n1A-n258I DC_n1A-n258J DC_n28A-n258A DC_n28A-n258D DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n28A-n258J |
| DC_n1A-n41A-n257A DC_n1A-n41A-n257G DC_n1A-n41A-n257H DC_n1A-n41A-n257I | DC_n1A-n41A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n1A-n77A-n257A DC_n1A-n77A-n257G DC_n1A-n77A-n257H DC_n1A-n77A-n257I | DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n1A-n77(2A)-n257A DC_n1A-n77(2A)-n257G DC_n1A-n77(2A)-n257H DC_n1A-n77(2A)-n257I | DC_n1A-n77A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n1A-n78A-n257A1 DC_n1A-n78A-n257G1 DC_n1A-n78A-n257H1 DC_n1A-n78A-n257I1 DC_n1A-n78A-n257J1 DC_n1A-n78A-n257K1 DC_n1A-n78A-n257L1 DC_n1A-n78A-n257M1 | DC_n1A-n78A DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n1A-n257J DC_n1A-n257K DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K |
| DC_n1A-n79A-n257A DC_n1A-n79A-n257G DC_n1A-n79A-n257H DC_n1A-n79A-n257I | DC_n1A-n257A DC_n1A-n257G DC_n1A-n257H DC_n1A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n2A-n5A-n260A DC_n2A-n5A-n260G DC_n2A-n5A-n260H DC_n2A-n5A-n260I DC_n2A-n5A-n260J DC_n2A-n5A-n260K DC_n2A-n5A-n260L DC_n2A-n5A-n260M | DC_n2A-n5A DC_n2A-n260A DC_n5A-n260A DC_n2A-n260G DC_n5A-n260G DC_n2A-n260H DC_n5A-n260H DC_n2A-n260I DC_n5A-n260I DC_n2A-n260J DC_n5A-n260J DC_n2A-n260K DC_n5A-n260K DC_n2A-n260L DC_n5A-n260L DC_n2A-n260M DC_n5A-n260M |
| DC_n2A-n5A-n261A DC_n2A-n5A-n261G DC_n2A-n5A-n261H DC_n2A-n5A-n261I DC_n2A-n5A-n261J DC_n2A-n5A-n261K DC_n2A-n5A-n261L DC_n2A-n5A-n261M | DC_n2A-n5A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I |
| DC_n2A-n5A-n261(2G) DC_n2A-n5A-n261(G-H) DC_n2A-n5A-n261(A-G-H) DC_n2A-n5A-n261(G-I) DC_n2A-n5A-n261(2H) DC_n2A-n5A-n261(A-G-I) DC_n2A-n5A-n261(H-I) DC_n2A-n5A-n261(A-G) DC_n2A-n5A-n261(A-H) DC_n2A-n5A-n261(2A-H) DC_n2A-n5A-n261(A-2G) DC_n2A-n5A-n261(A-I) DC_n2A-n5A-n261(2A-I) DC_n2A-n5A-n261(2A) DC_n2A-n5A-n261(3A) DC_n2A-n5A-n261(2A-G) | DC_n2A-n5A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I |
| DC_n2A-n12A-n260A DC_n2A-n12A-n260G DC_n2A-n12A-n260H DC_n2A-n12A-n260I DC_n2A-n12A-n260J DC_n2A-n12A-n260K DC_n2A-n12A-n260L DC_n2A-n12A-n260M | DC_n2A-n12A DC_n2A-n260A DC_n12A-n260A DC_n2A-n260G DC_n12A-n260G DC_n2A-n260H DC_n12A-n260H DC_n2A-n260I DC_n12A-n260I DC_n2A-n260J DC_n12A-n260J DC_n2A-n260K DC_n12A-n260K DC_n2A-n260L DC_n12A-n260L DC_n2A-n260M DC_n12A-n260M |
| DC_n2A-n14A-n260A DC_n2A-n14A-n260G DC_n2A-n14A-n260H DC_n2A-n14A-n260I DC_n2A-n14A-n260J DC_n2A-n14A-n260K DC_n2A-n14A-n260L DC_n2A-n14A-n260M | DC_n2A-n14A DC_n2A-n260A DC_n14A-n260A DC_n2A-n260G DC_n14A-n260G DC_n2A-n260H DC_n14A-n260H DC_n2A-n260I DC_n14A-n260I DC_n2A-n260J DC_n14A-n260J DC_n2A-n260K DC_n14A-n260K DC_n2A-n260L DC_n14A-n260L DC_n2A-n260M DC_n14A-n260M |
| DC_n2A-n30A-n260A DC_n2A-n30A-n260G DC_n2A-n30A-n260H DC_n2A-n30A-n260I DC_n2A-n30A-n260J DC_n2A-n30A-n260K DC_n2A-n30A-n260L DC_n2A-n30A-n260M | DC_n2A-n30A DC_n2A-n260A DC_n30A-n260A DC_n2A-n260G DC_n30A-n260G DC_n2A-n260H DC_n30A-n260H DC_n2A-n260I DC_n30A-n260I DC_n2A-n260J DC_n30A-n260J DC_n2A-n260K DC_n30A-n260K DC_n2A-n260L DC_n30A-n260L DC_n2A-n260M DC_n30A-n260M |
| DC_n2A-n48A-n260A DC_n2A-n48A-n260G DC_n2A-n48A-n260H DC_n2A-n48A-n260I DC_n2A-n48A-n260J DC_n2A-n48A-n260K DC_n2A-n48A-n260L DC_n2A-n48A-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n2A-n48(2A)-n260A DC_n2A-n48(2A)-n260G DC_n2A-n48(2A)-n260H DC_n2A-n48(2A)-n260I DC_n2A-n48(2A)-n260J DC_n2A-n48(2A)-n260K DC_n2A-n48(2A)-n260L DC_n2A-n48(2A)-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n2A-n48B-n260A DC_n2A-n48B-n260G DC_n2A-n48B-n260H DC_n2A-n48B-n260I DC_n2A-n48B-n260J DC_n2A-n48B-n260K DC_n2A-n48B-n260L DC_n2A-n48B-n260M | DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n2A-n48A-n261A DC_n2A-n48A-n261G DC_n2A-n48A-n261H DC_n2A-n48A-n261I DC_n2A-n48A-n261J DC_n2A-n48A-n261K DC_n2A-n48A-n261L DC_n2A-n48A-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48A-n261(G-H) DC_n2A-n48A-n261(A-G-H) DC_n2A-n48A-n261(2H) DC_n2A-n48A-n261(H-I) DC_n2A-n48A-n261(A-G-I) DC_n2A-n48A-n261(A-H) DC_n2A-n48A-n261(2G) DC_n2A-n48A-n261(2A-H) DC_n2A-n48A-n261(A-2G) DC_n2A-n48A-n261(G-I) DC_n2A-n48A-n261(2A-I) DC_n2A-n48A-n261(A-G) DC_n2A-n48A-n261(2A-G) DC_n2A-n48A-n261(A-I) DC_n2A-n48A-n261(2A) DC_n2A-n48A-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48(2A)-n261A DC_n2A-n48(2A)-n261G DC_n2A-n48(2A)-n261H DC_n2A-n48(2A)-n261I DC_n2A-n48(2A)-n261J DC_n2A-n48(2A)-n261K DC_n2A-n48(2A)-n261L DC_n2A-n48(2A)-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48(2A)-n261(G-H) DC_n2A-n48(2A)-n261(A-G-H) DC_n2A-n48(2A)-n261(2H) DC_n2A-n48(2A)-n261(H-I) DC_n2A-n48(2A)-n261(A-G-I) DC_n2A-n48(2A)-n261(A-H) DC_n2A-n48(2A)-n261(2G) DC_n2A-n48(2A)-n261(2A-H) DC_n2A-n48(2A)-n261(A-2G) DC_n2A-n48(2A)-n261(G-I) DC_n2A-n48(2A)-n261(2A-I) DC_n2A-n48(2A)-n261(A-G) DC_n2A-n48(2A)-n261(2A-G) DC_n2A-n48(2A)-n261(A-I) DC_n2A-n48(2A)-n261(2A) DC_n2A-n48(2A)-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48B-n261A DC_n2A-n48B-n261G DC_n2A-n48B-n261H DC_n2A-n48B-n261I DC_n2A-n48B-n261J DC_n2A-n48B-n261K DC_n2A-n48B-n261L DC_n2A-n48B-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n48B-n261(G-H) DC_n2A-n48B-n261(A-G-H) DC_n2A-n48B-n261(2H) DC_n2A-n48B-n261(H-I) DC_n2A-n48B-n261(A-G-I) DC_n2A-n48B-n261(A-H) DC_n2A-n48B-n261(2G) DC_n2A-n48B-n261(2A-H) DC_n2A-n48B-n261(A-2G) DC_n2A-n48B-n261(G-I) DC_n2A-n48B-n261(2A-I) DC_n2A-n48B-n261(A-G) DC_n2A-n48B-n261(2A-G) DC_n2A-n48B-n261(A-I) DC_n2A-n48B-n261(2A) DC_n2A-n48B-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n2A-n66A-n260A DC_n2A-n66A-n260G DC_n2A-n66A-n260H DC_n2A-n66A-n260I DC_n2A-n66A-n260J DC_n2A-n66A-n260K DC_n2A-n66A-n260L DC_n2A-n66A-n260M | DC_n2A-n66A DC_n2A-n260A DC_n66A-n260A DC_n2A-n260G DC_n66A-n260G DC_n2A-n260H DC_n66A-n260H DC_n2A-n260I DC_n66A-n260I DC_n2A-n260J DC_n66A-n260J DC_n2A-n260K DC_n66A-n260K DC_n2A-n260L DC_n66A-n260L DC_n2A-n260M DC_n66A-n260M |
| DC_n2A-n66A-n261A DC_n2A-n66A-n261G DC_n2A-n66A-n261H DC_n2A-n66A-n261I DC_n2A-n66A-n261J DC_n2A-n66A-n261K DC_n2A-n66A-n261L DC_n2A-n66A-n261M | DC_n2A-n66A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n2A-n66A-n261(2G) DC_n2A-n66A-n261(G-H) DC_n2A-n66A-n261(A-G-H) DC_n2A-n66A-n261(G-I) DC_n2A-n66A-n261(2H) DC_n2A-n66A-n261(A-G-I) DC_n2A-n66A-n261(H-I) DC_n2A-n66A-n261(A-G) DC_n2A-n66A-n261(A-H) DC_n2A-n66A-n261(2A-H) DC_n2A-n66A-n261(A-2G) DC_n2A-n66A-n261(A-I) DC_n2A-n66A-n261(2A-I) DC_n2A-n66A-n261(2A) DC_n2A-n66A-n261(3A) DC_n2A-n66A-n261(2A-G) | DC_n2A-n66A DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n2A-n77A-n260A DC_n2A-n77A-n260G DC_n2A-n77A-n260H DC_n2A-n77A-n260I DC_n2A-n77A-n260J DC_n2A-n77A-n260K DC_n2A-n77A-n260L DC_n2A-n77A-n260M DC_n2A-n77C-n260A DC_n2A-n77C-n260G DC_n2A-n77C-n260H DC_n2A-n77C-n260I DC_n2A-n77C-n260J DC_n2A-n77C-n260K DC_n2A-n77C-n260L DC_n2A-n77C-n260M | DC_n2A-n77A DC_n2A-n260A DC_n2A-n260G DC_n2A-n260H DC_n2A-n260I DC_n2A-n260J DC_n2A-n260K DC_n2A-n260L DC_n2A-n260M DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n2A-n77A-n261A DC_n2A-n77A-n261G DC_n2A-n77A-n261H DC_n2A-n77A-n261I DC_n2A-n77A-n261J DC_n2A-n77A-n261K DC_n2A-n77A-n261L DC_n2A-n77A-n261M DC_n2A-n77C-n261A DC_n2A-n77C-n261G DC_n2A-n77C-n261H DC_n2A-n77C-n261I DC_n2A-n77C-n261J DC_n2A-n77C-n261K DC_n2A-n77C-n261L DC_n2A-n77C-n261M | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n2A-n77A-n261(G-H) DC_n2A-n77A-n261(A-G-H) DC_n2A-n77A-n261(G-I) DC_n2A-n77A-n261(2H) DC_n2A-n77A-n261(A-G-I) DC_n2A-n77A-n261(H-I) DC_n2A-n77A-n261(A-H) DC_n2A-n77A-n261(2G) DC_n2A-n77A-n261(2A-H) DC_n2A-n77A-n261(A-2G) DC_n2A-n77A-n261(A-I) DC_n2A-n77A-n261(2A-I) DC_n2A-n77A-n261(A-G) DC_n2A-n77A-n261(2A-G) DC_n2A-n77A-n261(2A) DC_n2A-n77A-n261(3A) DC_n2A-n77C-n261(G-H) DC_n2A-n77C-n261(A-G-H) DC_n2A-n77C-n261(G-I) DC_n2A-n77C-n261(2H) DC_n2A-n77C-n261(A-G-I) DC_n2A-n77C-n261(H-I) DC_n2A-n77C-n261(A-H) DC_n2A-n77C-n261(2G) DC_n2A-n77C-n261(2A-H) DC_n2A-n77C-n261(A-2G) DC_n2A-n77C-n261(A-I) DC_n2A-n77C-n261(2A-I) DC_n2A-n77C-n261(A-G) DC_n2A-n77C-n261(2A-G) DC_n2A-n77C-n261(2A) DC_n2A-n77C-n261(3A) | DC_n2A-n261A DC_n2A-n261G DC_n2A-n261H DC_n2A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n3A-n7A-n258A DC_n3A-n7A-n258B DC_n3A-n7A-n258C DC_n3A-n7A-n258D DC_n3A-n7A-n258E DC_n3A-n7A-n258F DC_n3A-n7A-n258G DC_n3A-n7A-n258H DC_n3A-n7A-n258I DC_n3A-n7A-n258J DC_n3A-n7A-n258K DC_n3A-n7A-n258L DC_n3A-n7A-n258M | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I |
| DC_n3A-n7B-n258A DC_n3A-n7B-n258B DC_n3A-n7B-n258C DC_n3A-n7B-n258D DC_n3A-n7B-n258E DC_n3A-n7B-n258F DC_n3A-n7B-n258G DC_n3A-n7B-n258H DC_n3A-n7B-n258I DC_n3A-n7B-n258J DC_n3A-n7B-n258K DC_n3A-n7B-n258L DC_n3A-n7B-n258M | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I |
| DC_n3A-n18A-n257A DC_n3A-n18A-n257G DC_n3A-n18A-n257H DC_n3A-n18A-n257I | DC_n3A-n18A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I |
| DC_n3A-n28A-n257A1 DC_n3A-n28A-n257G1 DC_n3A-n28A-n257H1 DC_n3A-n28A-n257I1 | DC_n3A-n28A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n3A-n28A-n258A DC_n3A-n28A-n258D DC_n3A-n28A-n258G DC_n3A-n28A-n258H DC_n3A-n28A-n258I DC_n3A-n28A-n258J | DC_n3A-n28A DC_n3A-n258A DC_n3A-n258D DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n3A-n258J DC_n28A-n258A DC_n28A-n258D DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n28A-n258J |
| DC_n3A-n41A-n257A DC_n3A-n41A-n257G DC_n3A-n41A-n257H DC_n3A-n41A-n257I | DC_n3A-n41A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n3A-n77A-n257A1 DC_n3A-n77A-n257G1 DC_n3A-n77A-n257H1 DC_n3A-n77A-n257I1 | DC_n3A-n77A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n3A-n77(2A)-n257A1 DC_n3A-n77(2A)-n257G1 DC_n3A-n77(2A)-n257H1 DC_n3A-n77(2A)-n257I1 | DC_n3A-n77A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n3A-n78A-n257A1 DC_n3A-n78A-n257G1 DC_n3A-n78A-n257H1 DC_n3A-n78A-n257I1 | DC_n3A-n78A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n3A-n78A-n258A DC_n3A-n78A-n258B DC_n3A-n78A-n258C DC_n3A-n78A-n258D DC_n3A-n78A-n258E DC_n3A-n78A-n258F DC_n3A-n78A-n258G DC_n3A-n78A-n258H DC_n3A-n78A-n258I DC_n3A-n78A-n258J DC_n3A-n78A-n258K DC_n3A-n78A-n258L DC_n3A-n78A-n258M | DC_n3A-n258A DC_n3A-n258G DC_n3A-n258H DC_n3A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n3A-n78A |
| DC_n3A-n79A-n257A DC_n3A-n79A-n257G DC_n3A-n79A-n257H DC_n3A-n79A-n257I | DC_n3A-n79A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n5A-n30A-n260A DC_n5A-n30A-n260G DC_n5A-n30A-n260H DC_n5A-n30A-n260I DC_n5A-n30A-n260J DC_n5A-n30A-n260K DC_n5A-n30A-n260L DC_n5A-n30A-n260M | DC_n5A-n30A DC_n5A-n260A DC_n30A-n260A DC_n5A-n260G DC_n30A-n260G DC_n5A-n260H DC_n30A-n260H DC_n5A-n260I DC_n30A-n260I DC_n5A-n260J DC_n30A-n260J DC_n5A-n260K DC_n30A-n260K DC_n5A-n260L DC_n30A-n260L DC_n5A-n260M DC_n30A-n260M |
| DC_n5A-n48A-n260A DC_n5A-n48A-n260G DC_n5A-n48A-n260H DC_n5A-n48A-n260I DC_n5A-n48A-n260J DC_n5A-n48A-n260K DC_n5A-n48A-n260L DC_n5A-n48A-n260M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48(2A)-n260A DC_n5A-n48(2A)-n260G DC_n5A-n48(2A)-n260H DC_n5A-n48(2A)-n260I DC_n5A-n48(2A)-n260J DC_n5A-n48(2A)-n260K DC_n5A-n48(2A)-n260L DC_n5A-n48(2A)-n260M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48B-n260A DC_n5A-n48B-n260G DC_n5A-n48B-n260H DC_n5A-n48B-n260I DC_n5A-n48B-n260J DC_n5A-n48B-n260K DC_n5A-n48B-n260L DC_n5A-n48B-n260M | DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I |
| DC_n5A-n48A-n261A DC_n5A-n48A-n261G DC_n5A-n48A-n261H DC_n5A-n48A-n261I DC_n5A-n48A-n261J DC_n5A-n48A-n261K DC_n5A-n48A-n261L DC_n5A-n48A-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48A-n261(G-H) DC_n5A-n48A-n261(A-G-H) DC_n5A-n48A-n261(2H) DC_n5A-n48A-n261(H-I) DC_n5A-n48A-n261(A-G-I) DC_n5A-n48A-n261(A-H) DC_n5A-n48A-n261(2G) DC_n5A-n48A-n261(2A-H) DC_n5A-n48A-n261(A-2G) DC_n5A-n48A-n261(G-I) DC_n5A-n48A-n261(2A-I) DC_n5A-n48A-n261(A-G) DC_n5A-n48A-n261(2A-G) DC_n5A-n48A-n261(A-I) DC_n5A-n48A-n261(2A) DC_n5A-n48A-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48(2A)-n261A DC_n5A-n48(2A)-n261G DC_n5A-n48(2A)-n261H DC_n5A-n48(2A)-n261I DC_n5A-n48(2A)-n261J DC_n5A-n48(2A)-n261K DC_n5A-n48(2A)-n261L DC_n5A-n48(2A)-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48(2A)-n261(G-H) DC_n5A-n48(2A)-n261(A-G-H) DC_n5A-n48(2A)-n261(2H) DC_n5A-n48(2A)-n261(H-I) DC_n5A-n48(2A)-n261(A-G-I) DC_n5A-n48(2A)-n261(A-H) DC_n5A-n48(2A)-n261(2G) DC_n5A-n48(2A)-n261(2A-H) DC_n5A-n48(2A)-n261(A-2G) DC_n5A-n48(2A)-n261(G-I) DC_n5A-n48(2A)-n261(2A-I) DC_n5A-n48(2A)-n261(A-G) DC_n5A-n48(2A)-n261(2A-G) DC_n5A-n48(2A)-n261(A-I) DC_n5A-n48(2A)-n261(2A) DC_n5A-n48(2A)-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48B-n261A DC_n5A-n48B-n261G DC_n5A-n48B-n261H DC_n5A-n48B-n261I DC_n5A-n48B-n261J DC_n5A-n48B-n261K DC_n5A-n48B-n261L DC_n5A-n48B-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n48B-n261(G-H) DC_n5A-n48B-n261(A-G-H) DC_n5A-n48B-n261(2H) DC_n5A-n48B-n261(H-I) DC_n5A-n48B-n261(A-G-I) DC_n5A-n48B-n261(A-H) DC_n5A-n48B-n261(2G) DC_n5A-n48B-n261(2A-H) DC_n5A-n48B-n261(A-2G) DC_n5A-n48B-n261(G-I) DC_n5A-n48B-n261(2A-I) DC_n5A-n48B-n261(A-G) DC_n5A-n48B-n261(2A-G) DC_n5A-n48B-n261(A-I) DC_n5A-n48B-n261(2A) DC_n5A-n48B-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I |
| DC_n5A-n66A-n260A DC_n5A-n66A-n260G DC_n5A-n66A-n260H DC_n5A-n66A-n260I DC_n5A-n66A-n260J DC_n5A-n66A-n260K DC_n5A-n66A-n260L DC_n5A-n66A-n260M | DC_n5A-n66A DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n5A-n260J DC_n5A-n260K DC_n5A-n260L DC_n5A-n260M DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M |
| DC_n5A-n66A-n261A DC_n5A-n66A-n261G DC_n5A-n66A-n261H DC_n5A-n66A-n261I DC_n5A-n66A-n261J DC_n5A-n66A-n261K DC_n5A-n66A-n261L DC_n5A-n66A-n261M | DC_n5A-n66A DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n5A-n66A-n261(2G) DC_n5A-n66A-n261(G-H) DC_n5A-n66A-n261(A-G-H) DC_n5A-n66A-n261(G-I) DC_n5A-n66A-n261(2H) DC_n5A-n66A-n261(A-G-I) DC_n5A-n66A-n261(H-I) DC_n5A-n66A-n261(2A-G) DC_n5A-n66A-n261(2A-H) DC_n5A-n66A-n261(2A-I) DC_n5A-n66A-n261(2A) DC_n5A-n66A-n261(3A) DC_n5A-n66A-n261(A-2G) DC_n5A-n66A-n261(A-G) DC_n5A-n66A-n261(A-H) DC_n5A-n66A-n261(A-I) | DC_n5A-n66A DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n5A-n77A-n260A DC_n5A-n77A-n260G DC_n5A-n77A-n260H DC_n5A-n77A-n260I DC_n5A-n77A-n260J DC_n5A-n77A-n260K DC_n5A-n77A-n260L DC_n5A-n77A-n260M DC_n5A-n77C-n260A DC_n5A-n77C-n260G DC_n5A-n77C-n260H DC_n5A-n77C-n260I DC_n5A-n77C-n260J DC_n5A-n77C-n260K DC_n5A-n77C-n260L DC_n5A-n77C-n260M | DC_n5A-n77A DC_n5A-n260A DC_n5A-n260G DC_n5A-n260H DC_n5A-n260I DC_n5A-n260J DC_n5A-n260K DC_n5A-n260L DC_n5A-n260M DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n5A-n77A-n261A DC_n5A-n77A-n261G DC_n5A-n77A-n261H DC_n5A-n77A-n261I DC_n5A-n77A-n261J DC_n5A-n77A-n261K DC_n5A-n77A-n261L DC_n5A-n77A-n261M DC_n5A-n77C-n261A DC_n5A-n77C-n261G DC_n5A-n77C-n261H DC_n5A-n77C-n261I DC_n5A-n77C-n261J DC_n5A-n77C-n261K DC_n5A-n77C-n261L DC_n5A-n77C-n261M | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n5A-n77A-n261(G-H) DC_n5A-n77A-n261(A-G-H) DC_n5A-n77A-n261(G-I) DC_n5A-n77A-n261(2H) DC_n5A-n77A-n261(A-G-I) DC_n5A-n77A-n261(H-I) DC_n5A-n77A-n261(A-H) DC_n5A-n77A-n261(2G) DC_n5A-n77A-n261(2A-H) DC_n5A-n77A-n261(A-2G) DC_n5A-n77A-n261(A-I) DC_n5A-n77A-n261(2A-I) DC_n5A-n77A-n261(A-G) DC_n5A-n77A-n261(2A-G) DC_n5A-n77A-n261(2A) DC_n5A-n77A-n261(3A) DC_n5A-n77C-n261(G-H) DC_n5A-n77C-n261(A-G-H) DC_n5A-n77C-n261(G-I) DC_n5A-n77C-n261(2H) DC_n5A-n77C-n261(A-G-I) DC_n5A-n77C-n261(H-I) DC_n5A-n77C-n261(A-H) DC_n5A-n77C-n261(2G) DC_n5A-n77C-n261(2A-H) DC_n5A-n77C-n261(A-2G) DC_n5A-n77C-n261(A-I) DC_n5A-n77C-n261(2A-I) DC_n5A-n77C-n261(A-G) DC_n5A-n77C-n261(2A-G) DC_n5A-n77C-n261(2A) DC_n5A-n77C-n261(3A) | DC_n5A-n261A DC_n5A-n261G DC_n5A-n261H DC_n5A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n7A-n25A-n257A DC_n7A-n25A-n257G DC_n7A-n25A-n257H DC_n7A-n25A-n257I DC_n7A-n25A-n257J DC_n7A-n25A-n257K DC_n7A-n25A-n257L DC_n7A-n25A-n257M | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n7A-n257J DC_n7A-n257K DC_n7A-n257L DC_n7A-n257M DC_n25A-n257A DC_n25A-n257G DC_n25A-n257H DC_n25A-n257I DC_n25A-n257J DC_n25A-n257K DC_n25A-n257L DC_n25A-n257M |
| DC_n7A-n25A-n260A DC_n7A-n25A-n260G DC_n7A-n25A-n260H DC_n7A-n25A-n260I DC_n7A-n25A-n260J DC_n7A-n25A-n260K DC_n7A-n25A-n260L DC_n7A-n25A-n260M | DC_n7A-n260A DC_n7A-n260G DC_n7A-n260H DC_n7A-n260I DC_n7A-n260J DC_n7A-n260K DC_n7A-n260L DC_n7A-n260M DC_n25A-n260A DC_n25A-n260G DC_n25A-n260H DC_n25A-n260I DC_n25A-n260J DC_n25A-n260K DC_n25A-n260L DC_n25A-n260M |
| DC_n7A-n26A-n258A DC_n7A-n26A-n258B DC_n7A-n26A-n258C DC_n7A-n26A-n258D DC_n7A-n26A-n258E DC_n7A-n26A-n258F DC_n7A-n26A-n258G DC_n7A-n26A-n258H DC_n7A-n26A-n258I DC_n7A-n26A-n258J DC_n7A-n26A-n258K DC_n7A-n26A-n258L DC_n7A-n26A-n258M DC_n7A-n26A-n258R2 DC_n7A-n26A-n258R3 DC_n7A-n26A-n258R4 DC_n7A-n26A-n258R5 DC_n7A-n26A-n258R6 DC_n7A-n26A-n258R7 DC_n7A-n26A-n258R8 DC_n7A-n26A-n258R9 DC_n7A-n26A-n258R10 | DC_n7A-n26A DC_n7A-n78A DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n26A-n78A DC_n26A-n258A DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I DC_n26A-n258R2 DC_n26A-n258R3 DC_n26A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n7B-n26A-n258A DC_n7B-n26A-n258B DC_n7B-n26A-n258C DC_n7B-n26A-n258D DC_n7B-n26A-n258E DC_n7B-n26A-n258F DC_n7B-n26A-n258G DC_n7B-n26A-n258H DC_n7B-n26A-n258I DC_n7B-n26A-n258J DC_n7B-n26A-n258K DC_n7B-n26A-n258L DC_n7B-n26A-n258M DC_n7B-n26A-n258R2 DC_n7B-n26A-n258R3 DC_n7B-n26A-n258R4 DC_n7B-n26A-n258R5 DC_n7B-n26A-n258R6 DC_n7B-n26A-n258R7 DC_n7B-n26A-n258R8 DC_n7B-n26A-n258R9 DC_n7B-n26A-n258R10 | DC_n7A-n26A DC_n7A-n78A DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n26A-n78A DC_n26A-n258A DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I DC_n26A-n258R2 DC_n26A-n258R3 DC_n26A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n7A-n66A-n257A DC_n7A-n66A-n257G DC_n7A-n66A-n257H DC_n7A-n66A-n257I DC_n7A-n66A-n257J DC_n7A-n66A-n257K DC_n7A-n66A-n257L DC_n7A-n66A-n257M | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n7A-n257J DC_n7A-n257K DC_n7A-n257L DC_n7A-n257M DC_n66A-n257A DC_n66A-n257G DC_n66A-n257H DC_n66A-n257I DC_n66A-n257J DC_n66A-n257K DC_n66A-n257L DC_n66A-n257M |
| DC_n7A-n66A-n260A DC_n7A-n66A-n260G DC_n7A-n66A-n260H DC_n7A-n66A-n260I DC_n7A-n66A-n260J DC_n7A-n66A-n260K DC_n7A-n66A-n260L DC_n7A-n66A-n260M | DC_n7A-n260A DC_n7A-n260G DC_n7A-n260H DC_n7A-n260I DC_n7A-n260J DC_n7A-n260K DC_n7A-n260L DC_n7A-n260M DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M |
| DC_n7A-n71A-n257A DC_n7A-n71A-n257G DC_n7A-n71A-n257H DC_n7A-n71A-n257I DC_n7A-n71A-n257J DC_n7A-n71A-n257K DC_n7A-n71A-n257L DC_n7A-n71A-n257M | DC_n7A-n257A DC_n7A-n257G DC_n7A-n257H DC_n7A-n257I DC_n7A-n257J DC_n7A-n257K DC_n7A-n257L DC_n7A-n257M DC_n71A-n257A DC_n71A-n257G DC_n71A-n257H DC_n71A-n257I DC_n71A-n257J DC_n71A-n257K DC_n71A-n257L DC_n71A-n257M |
| DC_n7A-n71A-n260A DC_n7A-n71A-n260G DC_n7A-n71A-n260H DC_n7A-n71A-n260I DC_n7A-n71A-n260J DC_n7A-n71A-n260K DC_n7A-n71A-n260L DC_n7A-n71A-n260M | DC_n7A-n260A DC_n7A-n260G DC_n7A-n260H DC_n7A-n260I DC_n7A-n260J DC_n7A-n260K DC_n7A-n260L DC_n7A-n260M DC_n71A-n260A DC_n71A-n260G DC_n71A-n260H DC_n71A-n260I DC_n71A-n260J DC_n71A-n260K DC_n71A-n260L DC_n71A-n260M |
| DC_n7A-n78A-n258A DC_n7A-n78A-n258B DC_n7A-n78A-n258C DC_n7A-n78A-n258D DC_n7A-n78A-n258E DC_n7A-n78A-n258F DC_n7A-n78A-n258G DC_n7A-n78A-n258H DC_n7A-n78A-n258I DC_n7A-n78A-n258J DC_n7A-n78A-n258K DC_n7A-n78A-n258L DC_n7A-n78A-n258M DC_n7A-n78A-n258R2 DC_n7A-n78A-n258R3 DC_n7A-n78A-n258R4 DC_n7A-n78A-n258R5 DC_n7A-n78A-n258R6 DC_n7A-n78A-n258R7 DC_n7A-n78A-n258R8 DC_n7A-n78A-n258R9 DC_n7A-n78A-n258R10 | DC_n7A-n78A DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n7A-n78(2A)-n258A DC_n7A-n78(2A)-n258B DC_n7A-n78(2A)-n258C DC_n7A-n78(2A)-n258D DC_n7A-n78(2A)-n258E DC_n7A-n78(2A)-n258F DC_n7A-n78(2A)-n258G DC_n7A-n78(2A)-n258H DC_n7A-n78(2A)-n258I DC_n7A-n78(2A)-n258J DC_n7A-n78(2A)-n258K DC_n7A-n78(2A)-n258L DC_n7A-n78(2A)-n258M DC_n7A-n78(2A)-n258R2 DC_n7A-n78(2A)-n258R3 DC_n7A-n78(2A)-n258R4 DC_n7A-n78(2A)-n258R5 DC_n7A-n78(2A)-n258R6 DC_n7A-n78(2A)-n258R7 DC_n7A-n78(2A)-n258R8 DC_n7A-n78(2A)-n258R9 DC_n7A-n78(2A)-n258R10 | DC_n7A-n78A DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n7B-n78A-n258A DC_n7B-n78A-n258B DC_n7B-n78A-n258C DC_n7B-n78A-n258D DC_n7B-n78A-n258E DC_n7B-n78A-n258F DC_n7B-n78A-n258G DC_n7B-n78A-n258H DC_n7B-n78A-n258I DC_n7B-n78A-n258J DC_n7B-n78A-n258K DC_n7B-n78A-n258L DC_n7B-n78A-n258M DC_n7B-n78A-n258R2 DC_n7B-n78A-n258R3 DC_n7B-n78A-n258R4 DC_n7B-n78A-n258R5 DC_n7B-n78A-n258R6 DC_n7B-n78A-n258R7 DC_n7B-n78A-n258R8 DC_n7B-n78A-n258R9 DC_n7B-n78A-n258R10 | DC_n7A-n78A DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n7B-n78(2A)-n258A DC_n7B-n78(2A)-n258B DC_n7B-n78(2A)-n258C DC_n7B-n78(2A)-n258D DC_n7B-n78(2A)-n258E DC_n7B-n78(2A)-n258F DC_n7B-n78(2A)-n258G DC_n7B-n78(2A)-n258H DC_n7B-n78(2A)-n258I DC_n7B-n78(2A)-n258J DC_n7B-n78(2A)-n258K DC_n7B-n78(2A)-n258L DC_n7B-n78(2A)-n258M DC_n7B-n78(2A)-n258R2 DC_n7B-n78(2A)-n258R3 DC_n7B-n78(2A)-n258R4 DC_n7B-n78(2A)-n258R5 DC_n7B-n78(2A)-n258R6 DC_n7B-n78(2A)-n258R7 DC_n7B-n78(2A)-n258R8 DC_n7B-n78(2A)-n258R9 DC_n7B-n78(2A)-n258R10 | DC_n7A-n78A DC_n7A-n258A DC_n7A-n258G DC_n7A-n258H DC_n7A-n258I DC_n7A-n258R2 DC_n7A-n258R3 DC_n7A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n8A-n78A-n257A1 DC_n8A-n78A-n257G1 DC_n8A-n78A-n257H1 DC_n8A-n78A-n257I1 DC_n8A-n78A-n257J1 DC_n8A-n78A-n257K1 | DC_n8A-n78A DC_n8A-n257A DC_n8A-n257G DC_n8A-n257H DC_n8A-n257I DC_n8A-n257J DC_n8A-n257K DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K |
| DC_n12A-n30A-n260A DC_n12A-n30A-n260G DC_n12A-n30A-n260H DC_n12A-n30A-n260I DC_n12A-n30A-n260J DC_n12A-n30A-n260K DC_n12A-n30A-n260L DC_n12A-n30A-n260M | DC_n12A-n30A DC_n12A-n260A DC_n30A-n260A DC_n12A-n260G DC_n30A-n260G DC_n12A-n260H DC_n30A-n260H DC_n12A-n260I DC_n30A-n260I DC_n12A-n260J DC_n30A-n260J DC_n12A-n260K DC_n30A-n260K DC_n12A-n260L DC_n30A-n260L DC_n12A-n260M DC_n30A-n260M |
| DC_n12A-n66A-n260A DC_n12A-n66A-n260G DC_n12A-n66A-n260H DC_n12A-n66A-n260I DC_n12A-n66A-n260J DC_n12A-n66A-n260K DC_n12A-n66A-n260L DC_n12A-n66A-n260M | DC_n12A-n66A DC_n12A-n260A DC_n66A-n260A DC_n12A-n260G DC_n66A-n260G DC_n12A-n260H DC_n66A-n260H DC_n12A-n260I DC_n66A-n260I DC_n12A-n260J DC_n66A-n260J DC_n12A-n260K DC_n66A-n260K DC_n12A-n260L DC_n66A-n260L DC_n12A-n260M DC_n66A-n260M |
| DC_n12A-n77A-n260A DC_n12A-n77A-n260G DC_n12A-n77A-n260H DC_n12A-n77A-n260I DC_n12A-n77A-n260J DC_n12A-n77A-n260K DC_n12A-n77A-n260L DC_n12A-n77A-n260M | DC_n12A-n77A DC_n12A-n260A DC_n77A-n260A DC_n12A-n260G DC_n77A-n260G DC_n12A-n260H DC_n77A-n260H DC_n12A-n260I DC_n77A-n260I DC_n12A-n260J DC_n77A-n260J DC_n12A-n260K DC_n77A-n260K DC_n12A-n260L DC_n77A-n260L DC_n12A-n260M DC_n77A-n260M |
| DC_n14A-n30A-n260A DC_n14A-n30A-n260G DC_n14A-n30A-n260H DC_n14A-n30A-n260I DC_n14A-n30A-n260J DC_n14A-n30A-n260K DC_n14A-n30A-n260L DC_n14A-n30A-n260M | DC_n14A-n30A DC_n14A-n260A DC_n30A-n260A DC_n14A-n260G DC_n30A-n260G DC_n14A-n260H DC_n30A-n260H DC_n14A-n260I DC_n30A-n260I DC_n14A-n260J DC_n30A-n260J DC_n14A-n260K DC_n30A-n260K DC_n14A-n260L DC_n30A-n260L DC_n14A-n260M DC_n30A-n260M |
| DC_n14A-n66A-n260A DC_n14A-n66A-n260G DC_n14A-n66A-n260H DC_n14A-n66A-n260I DC_n14A-n66A-n260J DC_n14A-n66A-n260K DC_n14A-n66A-n260L DC_n14A-n66A-n260M | DC_n14A-n66A DC_n14A-n260A DC_n66A-n260A DC_n14A-n260G DC_n66A-n260G DC_n14A-n260H DC_n66A-n260H DC_n14A-n260I DC_n66A-n260I DC_n14A-n260J DC_n66A-n260J DC_n14A-n260K DC_n66A-n260K DC_n14A-n260L DC_n66A-n260L DC_n14A-n260M DC_n66A-n260M |
| DC_n14A-n77A-n260A DC_n14A-n77A-n260G DC_n14A-n77A-n260H DC_n14A-n77A-n260I DC_n14A-n77A-n260J DC_n14A-n77A-n260K DC_n14A-n77A-n260L DC_n14A-n77A-n260M | DC_n14A-n77A DC_n14A-n260A DC_n77A-n260A DC_n14A-n260G DC_n77A-n260G DC_n14A-n260H DC_n77A-n260H DC_n14A-n260I DC_n77A-n260I DC_n14A-n260J DC_n77A-n260J DC_n14A-n260K DC_n77A-n260K DC_n14A-n260L DC_n77A-n260L DC_n14A-n260M DC_n77A-n260M |
| DC_n18A-n28A-n257A DC_n18A-n28A-n257G DC_n18A-n28A-n257H DC_n18A-n28A-n257I | DC_n18A-n28A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n18A-n41A-n257A DC_n18A-n41A-n257G DC_n18A-n41A-n257H DC_n18A-n41A-n257I | DC_n18A-n41A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n18A-n77A-n257A DC_n18A-n77A-n257G DC_n18A-n77A-n257H DC_n18A-n77A-n257I | DC_n18A-n77A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n18A-n77(2A)-n257A DC_n18A-n77(2A)-n257G DC_n18A-n77(2A)-n257H DC_n18A-n77(2A)-n257I | DC_n18A-n77A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n18A-n78A-n257A DC_n18A-n78A-n257G DC_n18A-n78A-n257H DC_n18A-n78A-n257I | DC_n18A-n78A DC_n18A-n257A DC_n18A-n257G DC_n18A-n257H DC_n18A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n25A-n41A-n260A DC_n25A-n41A-n260G DC_n25A-n41A-n260H DC_n25A-n41A-n260I DC_n25A-n41A-n260(2A) | DC_n25A-n260A DC_n41A-n260A |
| DC_n26A-n78A-n258A DC_n26A-n78A-n258B DC_n26A-n78A-n258C DC_n26A-n78A-n258D DC_n26A-n78A-n258E DC_n26A-n78A-n258F DC_n26A-n78A-n258G DC_n26A-n78A-n258H DC_n26A-n78A-n258I DC_n26A-n78A-n258J DC_n26A-n78A-n258K DC_n26A-n78A-n258L DC_n26A-n78A-n258M DC_n26A-n78A-n258R2 DC_n26A-n78A-n258R3 DC_n26A-n78A-n258R4 DC_n26A-n78A-n258R5 DC_n26A-n78A-n258R6 DC_n26A-n78A-n258R7 DC_n26A-n78A-n258R8 DC_n26A-n78A-n258R9 DC_n26A-n78A-n258R10 | DC_n26A-n78A DC_n26A-n258A DC_n26A-n258G DC_n26A-n258H DC_n26A-n258I DC_n26A-n258R2 DC_n26A-n258R3 DC_n26A-n258R4 DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I DC_n78A-n258R2 DC_n78A-n258R3 DC_n78A-n258R4 |
| DC_n28A-n41A-n257A DC_n28A-n41A-n257G DC_n28A-n41A-n257H DC_n28A-n41A-n257I | DC_n28A-n41A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |
| DC_n28A-n77A-n257A1 DC_n28A-n77A-n257G1 DC_n28A-n77A-n257H1 DC_n28A-n77A-n257I1 | DC_n28A-n77A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n28A-n77(2A)-n257A DC_n28A-n77(2A)-n257G DC_n28A-n77(2A)-n257H DC_n28A-n77(2A)-n257I | DC_n28A-n77A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n28A-n78A-n257A1 DC_n28A-n78A-n257G1 DC_n28A-n78A-n257H1 DC_n28A-n78A-n257I1 | DC_n28A-n78A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n28A-n78A-n258A DC_n28A-n78A-n258B DC_n28A-n78A-n258C DC_n28A-n78A-n258D DC_n28A-n78A-n258E DC_n28A-n78A-n258F DC_n28A-n78A-n258G DC_n28A-n78A-n258H DC_n28A-n78A-n258I DC_n28A-n78A-n258J DC_n28A-n78A-n258K DC_n28A-n78A-n258L DC_n28A-n78A-n258M | DC_n28A-n258A DC_n28A-n258G DC_n28A-n258H DC_n28A-n258I DC_n78A-n258A DC_n78A-n258G DC_n78A-n258H DC_n78A-n258I |
| DC_n28A-n79A-n257A1 DC_n28A-n79A-n257G1 DC_n28A-n79A-n257H1 DC_n28A-n79A-n257I1 | DC_n28A-n79A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n30A-n66A-n260A DC_n30A-n66A-n260G DC_n30A-n66A-n260H DC_n30A-n66A-n260I DC_n30A-n66A-n260J DC_n30A-n66A-n260K DC_n30A-n66A-n260L DC_n30A-n66A-n260M | DC_n30A-n66A DC_n30A-n260A DC_n30A-n260G DC_n30A-n260H DC_n30A-n260I DC_n30A-n260J DC_n30A-n260K DC_n30A-n260L DC_n30A-n260M DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M |
| DC_n30A-n77A-n260A DC_n30A-n77A-n260G DC_n30A-n77A-n260H DC_n30A-n77A-n260I DC_n30A-n77A-n260J DC_n30A-n77A-n260K DC_n30A-n77A-n260L DC_n30A-n77A-n260M | DC_n30A-n77A DC_n30A-n260A DC_n77A-n260A DC_n30A-n260G DC_n77A-n260G DC_n30A-n260H DC_n77A-n260H DC_n30A-n260I DC_n77A-n260I DC_n30A-n260J DC_n77A-n260J DC_n30A-n260K DC_n77A-n260K DC_n30A-n260L DC_n77A-n260L DC_n30A-n260M DC_n77A-n260M |
| DC_n40A-n77A-n257A DC_n40A-n77A-n257D DC_n40A-n77A-n257E DC_n40A-n77A-n257F DC_n40A-n77A-n257G DC_n40A-n77A-n257H DC_n40A-n77A-n257I DC_n40A-n77A-n257J DC_n40A-n77A-n257K DC_n40A-n77A-n257L DC_n40A-n77A-n257M DC_n40A-n77C-n257A DC_n40A-n77C-n257D DC_n40A-n77C-n257E DC_n40A-n77C-n257F | DC_n40A-n77A DC_n40A-n257A DC_n40A-n257D DC_n40A-n257E DC_n40A-n257F DC_n40A-n257G DC_n40A-n257H DC_n40A-n257I DC_n40A-n257J DC_n40A-n257K DC_n40A-n257L DC_n40A-n257M DC_n77A-n257A DC_n77A-n257E DC_n77A-n257F DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M |
| DC_n40A-n78A-n257A DC_n40A-n78A-n257D DC_n40A-n78A-n257E DC_n40A-n78A-n257F DC_n40A-n78A-n257G DC_n40A-n78A-n257H DC_n40A-n78A-n257I DC_n40A-n78A-n257J DC_n40A-n78A-n257K DC_n40A-n78A-n257L DC_n40A-n78A-n257M DC_n40A-n78C-n257A DC_n40A-n78C-n257D DC_n40A-n78C-n257E DC_n40A-n78C-n257F DC_n40A-n78C-n257G DC_n40A-n78C-n257H DC_n40A-n78C-n257I DC_n40A-n78C-n257J DC_n40A-n78C-n257K DC_n40A-n78C-n257L DC_n40A-n78C-n257M | DC_n40A-n78A DC_n40A-n257A DC_n40A-n257D DC_n40A-n257E DC_n40A-n257F DC_n40A-n257G DC_n40A-n257H DC_n40A-n257I DC_n40A-n257J DC_n40A-n257K DC_n40A-n257L DC_n40A-n257M DC_n78A-n257A DC_n78A-n257E DC_n78A-n257F DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K DC_n78A-n257L DC_n78A-n257M |
| DC_n41A-n66A-n260A DC_n41A-n66A-n260G DC_n41A-n66A-n260H DC_n41A-n66A-n260I | DC_n41A-n260A DC_n41A-n260G DC_n41A-n260H DC_n41A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n41A-n66A-n260(2A) | DC_n41A-n260A DC_n66A-n260A |
| DC_n41A-n77A-n257A DC_n41A-n77A-n257G DC_n41A-n77A-n257H DC_n41A-n77A-n257I | DC_n41A-n77A DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n41A-n77(2A)-n257A DC_n41A-n77(2A)-n257G DC_n41A-n77(2A)-n257H DC_n41A-n77(2A)-n257I | DC_n41A-n77A DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n41A-n78A-n257A DC_n41A-n78A-n257G DC_n41A-n78A-n257H DC_n41A-n78A-n257I | DC_n41A-n78A DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n48A-n66A-n260A DC_n48A-n66A-n260G DC_n48A-n66A-n260H DC_n48A-n66A-n260I DC_n48A-n66A-n260J DC_n48A-n66A-n260K DC_n48A-n66A-n260L DC_n48A-n66A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n48(2A)-n66A-n260A DC_n48(2A)-n66A-n260G DC_n48(2A)-n66A-n260H DC_n48(2A)-n66A-n260I DC_n48(2A)-n66A-n260J DC_n48(2A)-n66A-n260K DC_n48(2A)-n66A-n260L DC_n48(2A)-n66A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n48B-n66A-n260A DC_n48B-n66A-n260G DC_n48B-n66A-n260H DC_n48B-n66A-n260I DC_n48B-n66A-n260J DC_n48B-n66A-n260K DC_n48B-n66A-n260L DC_n48B-n66A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I |
| DC_n48A-n77A-n260A DC_n48A-n77A-n260G DC_n48A-n77A-n260H DC_n48A-n77A-n260I DC_n48A-n77A-n260J DC_n48A-n77A-n260K DC_n48A-n77A-n260L DC_n48A-n77A-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n48A-n77C-n260A DC_n48A-n77C-n260G DC_n48A-n77C-n260H DC_n48A-n77C-n260I DC_n48A-n77C-n260J DC_n48A-n77C-n260K DC_n48A-n77C-n260L DC_n48A-n77C-n260M | DC_n48A-n260A DC_n48A-n260G DC_n48A-n260H DC_n48A-n260I DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n48A-n66A-n261A DC_n48A-n66A-n261G DC_n48A-n66A-n261H DC_n48A-n66A-n261I DC_n48A-n66A-n261J DC_n48A-n66A-n261K DC_n48A-n66A-n261L DC_n48A-n66A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48A-n66A-n261(G-H) DC_n48A-n66A-n261(A-G-H) DC_n48A-n66A-n261(2H) DC_n48A-n66A-n261(H-I) DC_n48A-n66A-n261(A-G-I) DC_n48A-n66A-n261(A-H) DC_n48A-n66A-n261(2G) DC_n48A-n66A-n261(2A-H) DC_n48A-n66A-n261(A-2G) DC_n48A-n66A-n261(G-I) DC_n48A-n66A-n261(2A-I) DC_n48A-n66A-n261(A-G) DC_n48A-n66A-n261(2A-G) DC_n48A-n66A-n261(A-I) DC_n48A-n66A-n261(2A) DC_n48A-n66A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48(2A)-n66A-n261A DC_n48(2A)-n66A-n261G DC_n48(2A)-n66A-n261H DC_n48(2A)-n66A-n261I DC_n48(2A)-n66A-n261J DC_n48(2A)-n66A-n261K DC_n48(2A)-n66A-n261L DC_n48(2A)-n66A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48(2A)-n66A-n261(G-H) DC_n48(2A)-n66A-n261(A-G-H) DC_n48(2A)-n66A-n261(2H) DC_n48(2A)-n66A-n261(H-I) DC_n48(2A)-n66A-n261(A-G-I) DC_n48(2A)-n66A-n261(A-H) DC_n48(2A)-n66A-n261(2G) DC_n48(2A)-n66A-n261(2A-H) DC_n48(2A)-n66A-n261(A-2G) DC_n48(2A)-n66A-n261(G-I) DC_n48(2A)-n66A-n261(2A-I) DC_n48(2A)-n66A-n261(A-G) DC_n48(2A)-n66A-n261(2A-G) DC_n48(2A)-n66A-n261(A-I) DC_n48(2A)-n66A-n261(2A) DC_n48(2A)-n66A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48A-n77A-n261A DC_n48A-n77A-n261G DC_n48A-n77A-n261H DC_n48A-n77A-n261I DC_n48A-n77A-n261J DC_n48A-n77A-n261K DC_n48A-n77A-n261L DC_n48A-n77A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48A-n77A-n261(G-H) DC_n48A-n77A-n261(2H) DC_n48A-n77A-n261(A-G-H) DC_n48A-n77A-n261(H-I) DC_n48A-n77A-n261(A-G-I) DC_n48A-n77A-n261(A-H) DC_n48A-n77A-n261(2G) DC_n48A-n77A-n261(2A-H) DC_n48A-n77A-n261(A-2G) DC_n48A-n77A-n261(G-I) DC_n48A-n77A-n261(2A-I) DC_n48A-n77A-n261(A-G) DC_n48A-n77A-n261(2A-G) DC_n48A-n77A-n261(A-I) DC_n48A-n77A-n261(2A) DC_n48A-n77A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48A-n77C-n261A DC_n48A-n77C-n261G DC_n48A-n77C-n261H DC_n48A-n77C-n261I DC_n48A-n77C-n261J DC_n48A-n77C-n261K DC_n48A-n77C-n261L DC_n48A-n77C-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48A-n77C-n261(G-H) DC_n48A-n77C-n261(2H) DC_n48A-n77C-n261(A-G-H) DC_n48A-n77C-n261(H-I) DC_n48A-n77C-n261(A-G-I) DC_n48A-n77C-n261(A-H) DC_n48A-n77C-n261(2G) DC_n48A-n77C-n261(2A-H) DC_n48A-n77C-n261(A-2G) DC_n48A-n77C-n261(G-I) DC_n48A-n77C-n261(2A-I) DC_n48A-n77C-n261(A-G) DC_n48A-n77C-n261(2A-G) DC_n48A-n77C-n261(A-I) DC_n48A-n77C-n261(2A) DC_n48A-n77C-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n48B-n66A-n261A DC_n48B-n66A-n261G DC_n48B-n66A-n261H DC_n48B-n66A-n261I DC_n48B-n66A-n261J DC_n48B-n66A-n261K DC_n48B-n66A-n261L DC_n48B-n66A-n261M | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n48B-n66A-n261(G-H) DC_n48B-n66A-n261(A-G-H) DC_n48B-n66A-n261(2H) DC_n48B-n66A-n261(H-I) DC_n48B-n66A-n261(A-G-I) DC_n48B-n66A-n261(A-H) DC_n48B-n66A-n261(2G) DC_n48B-n66A-n261(2A-H) DC_n48B-n66A-n261(A-2G) DC_n48B-n66A-n261(G-I) DC_n48B-n66A-n261(2A-I) DC_n48B-n66A-n261(A-G) DC_n48B-n66A-n261(2A-G) DC_n48B-n66A-n261(A-I) DC_n48B-n66A-n261(2A) DC_n48B-n66A-n261(3A) | DC_n48A-n261A DC_n48A-n261G DC_n48A-n261H DC_n48A-n261I DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I |
| DC_n66A-n77A-n260A DC_n66A-n77A-n260G DC_n66A-n77A-n260H DC_n66A-n77A-n260I DC_n66A-n77A-n260J DC_n66A-n77A-n260K DC_n66A-n77A-n260L DC_n66A-n77A-n260M DC_n66A-n77C-n260A DC_n66A-n77C-n260G DC_n66A-n77C-n260H DC_n66A-n77C-n260I DC_n66A-n77C-n260J DC_n66A-n77C-n260K DC_n66A-n77C-n260L DC_n66A-n77C-n260M | DC_n66A-n77A DC_n66A-n260A DC_n66A-n260G DC_n66A-n260H DC_n66A-n260I DC_n66A-n260J DC_n66A-n260K DC_n66A-n260L DC_n66A-n260M DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I DC_n77A-n260J DC_n77A-n260K DC_n77A-n260L DC_n77A-n260M |
| DC_n66A-n77A-n261A DC_n66A-n77A-n261G DC_n66A-n77A-n261H DC_n66A-n77A-n261I DC_n66A-n77A-n261J DC_n66A-n77A-n261K DC_n66A-n77A-n261L DC_n66A-n77A-n261M DC_n66A-n77C-n261A DC_n66A-n77C-n261G DC_n66A-n77C-n261H DC_n66A-n77C-n261I DC_n66A-n77C-n261J DC_n66A-n77C-n261K DC_n66A-n77C-n261L DC_n66A-n77C-n261M | DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n66A-n77A-n261(G-H) DC_n66A-n77A-n261(A-G-H) DC_n66A-n77A-n261(G-I) DC_n66A-n77A-n261(2H) DC_n66A-n77A-n261(A-G-I) DC_n66A-n77A-n261(H-I) DC_n66A-n77A-n261(A-H) DC_n66A-n77A-n261(2G) DC_n66A-n77A-n261(2A-H) DC_n66A-n77A-n261(A-2G) DC_n66A-n77A-n261(A-I) DC_n66A-n77A-n261(2A-I) DC_n66A-n77A-n261(A-G) DC_n66A-n77A-n261(2A-G) DC_n66A-n77A-n261(2A) DC_n66A-n77A-n261(3A) DC_n66A-n77C-n261(G-H) DC_n66A-n77C-n261(A-G-H) DC_n66A-n77C-n261(G-I) DC_n66A-n77C-n261(2H) DC_n66A-n77C-n261(A-G-I) DC_n66A-n77C-n261(H-I) DC_n66A-n77C-n261(A-H) DC_n66A-n77C-n261(2G) DC_n66A-n77C-n261(2A-H) DC_n66A-n77C-n261(A-2G) DC_n66A-n77C-n261(A-I) DC_n66A-n77C-n261(2A-I) DC_n66A-n77C-n261(A-G) DC_n66A-n77C-n261(2A-G) DC_n66A-n77C-n261(2A) DC_n66A-n77C-n261(3A) | DC_n66A-n261A DC_n66A-n261G DC_n66A-n261H DC_n66A-n261I DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I |
| DC_n77A-n79A-n257A DC_n77A-n79A-n257G DC_n77A-n79A-n257H DC_n77A-n79A-n257I | DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n77(2A)-n79A-n257A DC_n77(2A)-n79A-n257G DC_n77(2A)-n79A-n257H DC_n77(2A)-n79A-n257I | DC_n77A-n79A DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n77A-n79A-n259A DC_n77A-n79A-n259G DC_n77A-n79A-n259H DC_n77A-n79A-n259I DC_n77A-n79A-n259J DC_n77A-n79A-n259K DC_n77A-n79A-n259L DC_n77A-n79A-n259M | DC_n77A-n79A DC_n77A-n259A DC_n77A-n259G DC_n77A-n259H DC_n77A-n259I DC_n77A-n259J DC_n77A-n259K DC_n77A-n259L DC_n77A-n259M DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |
| DC_n77A-n257A-n259A1 DC_n77A-n257A-n259G1 DC_n77A-n257A-n259H1 DC_n77A-n257A-n259I1 DC_n77A-n257A-n259J1 DC_n77A-n257A-n259K1 DC_n77A-n257A-n259L1 DC_n77A-n257A-n259M1 DC_n77A-n257G-n259A1 DC_n77A-n257G-n259G1 DC_n77A-n257G-n259H1 DC_n77A-n257G-n259I1 DC_n77A-n257G-n259J1 DC_n77A-n257G-n259K1 DC_n77A-n257G-n259L1 DC_n77A-n257G-n259M1 DC_n77A-n257H-n259A1 DC_n77A-n257H-n259G1 DC_n77A-n257H-n259H1 DC_n77A-n257H-n259I1 DC_n77A-n257H-n259J1 DC_n77A-n257H-n259K1 DC_n77A-n257H-n259L1 DC_n77A-n257H-n259M1 DC_n77A-n257I-n259A1 DC_n77A-n257I-n259G1 DC_n77A-n257I-n259H1 DC_n77A-n257I-n259I1 DC_n77A-n257I-n259J1 DC_n77A-n257I-n259K1 DC_n77A-n257I-n259L1 DC_n77A-n257I-n259M1 | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n259A DC_n77A-n259G DC_n77A-n259H DC_n77A-n259I DC_n77A-n259J DC_n77A-n259K DC_n77A-n259L DC_n77A-n259M |
| DC_n77A-n79A-n258A DC_n77A-n79A-n258D DC_n77A-n79A-n258G DC_n77A-n79A-n258H DC_n77A-n79A-n258I DC_n77A-n79A-n258J | DC_n77A-n79A DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J DC_n79A-n258A DC_n79A-n258D DC_n79A-n258G DC_n79A-n258H DC_n79A-n258I DC_n79A-n258J |
| DC_n77(2A)-n79A-n258A DC_n77(2A)-n79A-n258D DC_n77(2A)-n79A-n258G DC_n77(2A)-n79A-n258H DC_n77(2A)-n79A-n258I DC_n77(2A)-n79A-n258J | DC_n77A-n79A DC_n77A-n258A DC_n77A-n258D DC_n77A-n258G DC_n77A-n258H DC_n77A-n258I DC_n77A-n258J DC_n79A-n258A DC_n79A-n258D DC_n79A-n258G DC_n79A-n258H DC_n79A-n258I DC_n79A-n258J |
| DC_n78A-n79A-n257A DC_n78A-n79A-n257G DC_n78A-n79A-n257H DC_n78A-n79A-n257I | DC_n78A-n79A DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n78(2A)-n79A-n257A DC_n78(2A)-n79A-n257G DC_n78(2A)-n79A-n257H DC_n78(2A)-n79A-n257I | DC_n78A-n79A DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I |
| DC_n78A-n79A-n259A DC_n78A-n79A-n259G DC_n78A-n79A-n259H DC_n78A-n79A-n259I DC_n78A-n79A-n259J DC_n78A-n79A-n259K DC_n78A-n79A-n259L DC_n78A-n79A-n259M | DC_n78A-n79A DC_n78A-n259A DC_n78A-n259G DC_n78A-n259H DC_n78A-n259I DC_n78A-n259J DC_n78A-n259K DC_n78A-n259L DC_n78A-n259M DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |
| DC_n79A-n257A-n259A DC_n79A-n257A-n259G DC_n79A-n257A-n259H DC_n79A-n257A-n259I DC_n79A-n257A-n259J DC_n79A-n257A-n259K DC_n79A-n257A-n259L DC_n79A-n257A-n259M DC_n79A-n257G-n259A DC_n79A-n257G-n259G DC_n79A-n257G-n259H DC_n79A-n257G-n259I DC_n79A-n257G-n259J DC_n79A-n257G-n259K DC_n79A-n257G-n259L DC_n79A-n257G-n259M DC_n79A-n257H-n259A DC_n79A-n257H-n259G DC_n79A-n257H-n259H DC_n79A-n257H-n259I DC_n79A-n257H-n259J DC_n79A-n257H-n259K DC_n79A-n257H-n259L DC_n79A-n257H-n259M DC_n79A-n257I-n259A DC_n79A-n257I-n259G DC_n79A-n257I-n259H DC_n79A-n257I-n259I DC_n79A-n257I-n259J DC_n79A-n257I-n259K DC_n79A-n257I-n259L DC_n79A-n257I-n259M | DC_n79A-n257A DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I DC_n79A-n259A DC_n79A-n259G DC_n79A-n259H DC_n79A-n259I DC_n79A-n259J DC_n79A-n259K DC_n79A-n259L DC_n79A-n259M |
| NOTE 1: Applicable for UE supporting inter-band NR DC with mandatory simultaneous Rx/Tx capability. |  |

#### 5.5B.7.3	Inter-band NR-DC configurations between FR1 and FR2 (four bands)

Table 5.5B.7-3: Inter-band NR-DC configurations between FR1 and FR2 (four bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n77A-n79A-n257A DC_n1A-n77A-n79A-n257G DC_n1A-n77A-n79A-n257H DC_n1A-n77A-n79A-n257I | DC_n1A-n257A/G/H/I DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n1A-n78A-n79A-n257A DC_n1A-n78A-n79A-n257G DC_n1A-n78A-n79A-n257H DC_n1A-n78A-n79A-n257I | DC_n1A-n257A/G/H/I DC_n78A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n2A-n5A-n48A-n260A DC_n2A-n5A-n48A-n260G DC_n2A-n5A-n48A-n260H DC_n2A-n5A-n48A-n260I DC_n2A-n5A-n48A-n260J DC_n2A-n5A-n48A-n260K DC_n2A-n5A-n48A-n260L DC_n2A-n5A-n48A-n260M | DC_n2A-n260A/G/H/I DC_n5A-n260A/G/H/I DC_n48A-n260A/G/H/I |
| DC_n2A-n5A-n48A-n261A DC_n2A-n5A-n48A-n261G DC_n2A-n5A-n48A-n261H DC_n2A-n5A-n48A-n261I DC_n2A-n5A-n48A-n261J DC_n2A-n5A-n48A-n261K DC_n2A-n5A-n48A-n261L DC_n2A-n5A-n48A-n261M DC_n2A-n5A-n48A-n261(A-G) DC_n2A-n5A-n48A-n261(A-H) DC_n2A-n5A-n48A-n261(A-I) DC_n2A-n5A-n48A-n261(A-2G) DC_n2A-n5A-n48A-n261(2A-G) DC_n2A-n5A-n48A-n261(2A-H) DC_n2A-n5A-n48A-n261(2A-I) DC_n2A-n5A-n48A-n261(G-H) DC_n2A-n5A-n48A-n261(G-I) DC_n2A-n5A-n48A-n261(2A) DC_n2A-n5A-n48A-n261(3A) DC_n2A-n5A-n48A-n261(2G) DC_n2A-n5A-n48A-n261(2H) DC_n2A-n5A-n48A-n261(A-G-H) DC_n2A-n5A-n48A-n261(H-I) DC_n2A-n5A-n48A-n261(A-G-I) | DC_n2A-n261A/G/H/I DC_n5A-n261A/G/H/I DC_n48A-n261A/G/H/I |
| DC_n2A-n5A-n66A-n260A DC_n2A-n5A-n66A-n260G DC_n2A-n5A-n66A-n260H DC_n2A-n5A-n66A-n260I DC_n2A-n5A-n66A-n260J DC_n2A-n5A-n66A-n260K DC_n2A-n5A-n66A-n260L DC_n2A-n5A-n66A-n260M | DC_n2A-n260A/G/H/I DC_n5A-n260A/G/H/I DC_n77A-n260A/G/H/I |
| DC_n2A-n5A-n66A-n261A DC_n2A-n5A-n66A-n261G DC_n2A-n5A-n66A-n261H DC_n2A-n5A-n66A-n261I DC_n2A-n5A-n66A-n261J DC_n2A-n5A-n66A-n261K DC_n2A-n5A-n66A-n261L DC_n2A-n5A-n66A-n261M DC_n2A-n5A-n66A-n261(2A) DC_n2A-n5A-n66A-n261(3A) DC_n2A-n5A-n66A-n261(2G) DC_n2A-n5A-n66A-n261(G-H) DC_n2A-n5A-n66A-n261(A-G-H) DC_n2A-n5A-n66A-n261(G-I) DC_n2A-n5A-n66A-n261(2H) DC_n2A-n5A-n66A-n261(A-G-I) DC_n2A-n5A-n66A-n261(H-I) DC_n2A-n5A-n66A-n261(A-G) DC_n2A-n5A-n66A-n261(A-H) DC_n2A-n5A-n66A-n261(2A-G) DC_n2A-n5A-n66A-n261(2A-H) DC_n2A-n5A-n66A-n261(A-2G) DC_n2A-n5A-n66A-n261(A-I) DC_n2A-n5A-n66A-n261(2A-I) | DC_n2A-n261A/G/H/I DC_n5A-n261A/G/H/I DC_n66A-n261A/G/H/I |
| DC_n2A-n5A-n77A-n260A DC_n2A-n5A-n77A-n260G DC_n2A-n5A-n77A-n260H DC_n2A-n5A-n77A-n260I DC_n2A-n5A-n77A-n260J DC_n2A-n5A-n77A-n260K DC_n2A-n5A-n77A-n260L DC_n2A-n5A-n77A-n260M | DC_n2A-n260A/G/H/I DC_n5A-n260A/G/H/I DC_n77A-n260A/G/H/I |
| DC_n2A-n5A-n77A-n261A DC_n2A-n5A-n77A-n261G DC_n2A-n5A-n77A-n261H DC_n2A-n5A-n77A-n261I DC_n2A-n5A-n77A-n261J DC_n2A-n5A-n77A-n261K DC_n2A-n5A-n77A-n261L DC_n2A-n5A-n77A-n261M DC_n2A-n5A-n77A-n261(A-G) DC_n2A-n5A-n77A-n261(A-H) DC_n2A-n5A-n77A-n261(A-I) DC_n2A-n5A-n77A-n261(A-2G) DC_n2A-n5A-n77A-n261(2A-G) DC_n2A-n5A-n77A-n261(2A-H) DC_n2A-n5A-n77A-n261(2A-I) DC_n2A-n5A-n77A-n261(G-H) DC_n2A-n5A-n77A-n261(2A) DC_n2A-n5A-n77A-n261(3A) DC_n2A-n5A-n77A-n261(2G) DC_n2A-n5A-n77A-n261(2H) DC_n2A-n5A-n77A-n261(A-G-H) DC_n2A-n5A-n77A-n261(G-I) DC_n2A-n5A-n77A-n261(H-I) DC_n2A-n5A-n77A-n261(A-G-I) | DC_n2A-n261A/G/H/I DC_n5A-n261A/G/H/I DC_n77A-n261A/G/H/I |
| DC_n2A-n48A-n66A-n260A DC_n2A-n48A-n66A-n260G DC_n2A-n48A-n66A-n260H DC_n2A-n48A-n66A-n260I DC_n2A-n48A-n66A-n260J DC_n2A-n48A-n66A-n260K DC_n2A-n48A-n66A-n260L DC_n2A-n48A-n66A-n260M | DC_n2A-n260A/G/H/I DC_n66A-n260A/G/H/I DC_n48A-n260A/G/H/I |
| DC_n2A-n48A-n66A-n261A DC_n2A-n48A-n66A-n261G DC_n2A-n48A-n66A-n261H DC_n2A-n48A-n66A-n261I DC_n2A-n48A-n66A-n261J DC_n2A-n48A-n66A-n261K DC_n2A-n48A-n66A-n261L DC_n2A-n48A-n66A-n261M DC_n2A-n48A-n66A-n261(A-G) DC_n2A-n48A-n66A-n261(A-H) DC_n2A-n48A-n66A-n261(A-I) DC_n2A-n48A-n66A-n261(A-2G) DC_n2A-n48A-n66A-n261(2A-G) DC_n2A-n48A-n66A-n261(2A-H) DC_n2A-n48A-n66A-n261(2A-I) DC_n2A-n48A-n66A-n261(G-H) DC_n2A-n48A-n66A-n261(2A) DC_n2A-n48A-n66A-n261(3A) DC_n2A-n48A-n66A-n261(2G) DC_n2A-n48A-n66A-n261(2H) DC_n2A-n48A-n66A-n261(A-G-H) DC_n2A-n48A-n66A-n261(G-I) DC_n2A-n48A-n66A-n261(H-I) DC_n2A-n48A-n66A-n261(A-G-I) | DC_n2A-n261A/G/H/I DC_n66A-n261A/G/H/I DC_n48A-n261A/G/H/I |
| DC_n2A-n66A-n77A-n260A DC_n2A-n66A-n77A-n260G DC_n2A-n66A-n77A-n260H DC_n2A-n66A-n77A-n260I DC_n2A-n66A-n77A-n260J DC_n2A-n66A-n77A-n260K DC_n2A-n66A-n77A-n260L DC_n2A-n66A-n77A-n260M | DC_n2A-n260A/G/H/I DC_n66A-n260A/G/H/I DC_n77A-n260A/G/H/I |
| DC_n2A-n66A-n77A-n261A DC_n2A-n66A-n77A-n261G DC_n2A-n66A-n77A-n261H DC_n2A-n66A-n77A-n261I DC_n2A-n66A-n77A-n261J DC_n2A-n66A-n77A-n261K DC_n2A-n66A-n77A-n261L DC_n2A-n66A-n77A-n261M DC_n2A-n66A-n77A-n261(A-G) DC_n2A-n66A-n77A-n261(A-H) DC_n2A-n66A-n77A-n261(A-I) DC_n2A-n66A-n77A-n261(A-2G) DC_n2A-n66A-n77A-n261(2A-G) DC_n2A-n66A-n77A-n261(2A-H) DC_n2A-n66A-n77A-n261(2A-I) DC_n2A-n66A-n77A-n261(G-H) DC_n2A-n66A-n77A-n261(2A) DC_n2A-n66A-n77A-n261(3A) DC_n2A-n66A-n77A-n261(2G) DC_n2A-n66A-n77A-n261(2H) DC_n2A-n66A-n77A-n261(A-G-H) DC_n2A-n66A-n77A-n261(G-I) DC_n2A-n66A-n77A-n261(H-I) DC_n2A-n66A-n77A-n261(A-G-I) | DC_n2A-n261A/G/H/I DC_n66A-n261A/G/H/I DC_n77A-n261A/G/H/I |
| DC_n3A-n7A-n78A-n258A DC_n3A-n7A-n78A-n258B DC_n3A-n7A-n78A-n258C DC_n3A-n7A-n78A-n258D DC_n3A-n7A-n78A-n258E DC_n3A-n7A-n78A-n258F DC_n3A-n7A-n78A-n258G DC_n3A-n7A-n78A-n258H DC_n3A-n7A-n78A-n258I DC_n3A-n7A-n78A-n258J DC_n3A-n7A-n78A-n258K DC_n3A-n7A-n78A-n258L DC_n3A-n7A-n78A-n258M | DC_n3A-n7A DC_n3A-n78A DC_n7A-n78A DC_n3A-n258A/G/H/I DC_n7A-n258A/G/H/I DC_n78A-n258A/G/H/I |
| DC_n3A-n7B-n78A-n258A DC_n3A-n7B-n78A-n258B DC_n3A-n7B-n78A-n258C DC_n3A-n7B-n78A-n258D DC_n3A-n7B-n78A-n258E DC_n3A-n7B-n78A-n258F DC_n3A-n7B-n78A-n258G DC_n3A-n7B-n78A-n258H DC_n3A-n7B-n78A-n258I DC_n3A-n7B-n78A-n258J DC_n3A-n7B-n78A-n258K DC_n3A-n7B-n78A-n258L DC_n3A-n7B-n78A-n258M | DC_n3A-n7A DC_n3A-n78A DC_n7A-n78A DC_n3A-n258A/G/H/I DC_n7A-n258A/G/H/I DC_n78A-n258A/G/H/I |
| DC_n3A-n28A-n77A-n257A DC_n3A-n28A-n77A-n257G DC_n3A-n28A-n77A-n257H DC_n3A-n28A-n77A-n257I | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A/G/H/I DC_n28A-n77A DC_n28A-n257A/G/H/I DC_n77A-n257A/G/H/I DC_n3A-n257G/G/H/I |
| DC_n3A-n28A-n77(2A)-n257A DC_n3A-n28A-n77(2A)-n257G DC_n3A-n28A-n77(2A)-n257H DC_n3A-n28A-n77(2A)-n257I | DC_n3A-n28A DC_n3A-n77A DC_n3A-n257A/G/H/I DC_n28A-n77A DC_n28A-n257A/G/H/I DC_n77A-n257A/G/H/I |
| DC_n3A-n28A-n78A-n257A DC_n3A-n28A-n78A-n257G DC_n3A-n28A-n78A-n257H DC_n3A-n28A-n78A-n257I | DC_n3A-n257A/G/H/I DC_n28A-n257A/G/H/I DC_n78A-n257A/G/H/I |
| DC_n3A-n28A-n79A-n257A DC_n3A-n28A-n79A-n257G DC_n3A-n28A-n79A-n257H DC_n3A-n28A-n79A-n257I | DC_n3A-n28A DC_n3A-n79A DC_n3A-n257A/G/H/I DC_n28A-n79A DC_n28A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n3A-n77A-n79A-n257A DC_n3A-n77A-n79A-n257G DC_n3A-n77A-n79A-n257H DC_n3A-n77A-n79A-n257I | DC_n3A-n257A/G/H/I DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n3A-n77(2A)-n79A-n257A DC_n3A-n77(2A)-n79A-n257G DC_n3A-n77(2A)-n79A-n257H DC_n3A-n77(2A)-n79A-n257I | DC_n3A-n257A/G/H/I DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n5A-n48A-n66A-n260A DC_n5A-n48A-n66A-n260G DC_n5A-n48A-n66A-n260H DC_n5A-n48A-n66A-n260I DC_n5A-n48A-n66A-n260J DC_n5A-n48A-n66A-n260K DC_n5A-n48A-n66A-n260L DC_n5A-n48A-n66A-n260M | DC_n5A-n260A/G/H/I DC_n48A-n260A/G/H/I DC_n66A-n260A/G/H/I |
| DC_n5A-n48A-n66A-n261A DC_n5A-n48A-n66A-n261G DC_n5A-n48A-n66A-n261H DC_n5A-n48A-n66A-n261I DC_n5A-n48A-n66A-n261J DC_n5A-n48A-n66A-n261K DC_n5A-n48A-n66A-n261L DC_n5A-n48A-n66A-n261M DC_n5A-n48A-n66A-n261(A-G) DC_n5A-n48A-n66A-n261(A-H) DC_n5A-n48A-n66A-n261(A-I) DC_n5A-n48A-n66A-n261(A-2G) DC_n5A-n48A-n66A-n261(2A-G) DC_n5A-n48A-n66A-n261(2A-H) DC_n5A-n48A-n66A-n261(2A-I) DC_n5A-n48A-n66A-n261(G-H) DC_n5A-n48A-n66A-n261(2A) DC_n5A-n48A-n66A-n261(3A) DC_n5A-n48A-n66A-n261(2G) DC_n5A-n48A-n66A-n261(2H) DC_n5A-n48A-n66A-n261(A-G-H) DC_n5A-n48A-n66A-n261(G-I) DC_n5A-n48A-n66A-n261(H-I) DC_n5A-n48A-n66A-n261(A-G-I) | DC_n5A-n261A/G/H/I DC_n66A-n261A/G/H/I DC_n48A-n261A/G/H/I |
| DC_n5A-n66A-n77A-n260A DC_n5A-n66A-n77A-n260G DC_n5A-n66A-n77A-n260H DC_n5A-n66A-n77A-n260I DC_n5A-n66A-n77A-n260J DC_n5A-n66A-n77A-n260K DC_n5A-n66A-n77A-n260L DC_n5A-n66A-n77A-n260M | DC_n5A-n260A/G/H/I DC_n66A-n260A/G/H/I DC_n77A-n260A/G/H/I |
| DC_n5A-n66A-n77A-n261A DC_n5A-n66A-n77A-n261G DC_n5A-n66A-n77A-n261H DC_n5A-n66A-n77A-n261I DC_n5A-n66A-n77A-n261J DC_n5A-n66A-n77A-n261K DC_n5A-n66A-n77A-n261L DC_n5A-n66A-n77A-n261M DC_n5A-n66A-n77A-n261(A-G) DC_n5A-n66A-n77A-n261(A-H) DC_n5A-n66A-n77A-n261(A-I) DC_n5A-n66A-n77A-n261(A-2G) DC_n5A-n66A-n77A-n261(2A-G) DC_n5A-n66A-n77A-n261(2A-H) DC_n5A-n66A-n77A-n261(2A-I) DC_n5A-n66A-n77A-n261(G-H) DC_n5A-n66A-n77A-n261(2A) DC_n5A-n66A-n77A-n261(3A) DC_n5A-n66A-n77A-n261(2G) DC_n5A-n66A-n77A-n261(2H) DC_n5A-n66A-n77A-n261(A-G-H) DC_n5A-n66A-n77A-n261(G-I) DC_n5A-n66A-n77A-n261(H-I) DC_n5A-n66A-n77A-n261(A-G-I) | DC_n5A-n261A/G/H/I DC_n66A-n261A/G/H/I DC_n77A-n261A/G/H/I |
| DC_n7A-n26A-n78A-n258A DC_n7A-n26A-n78A-n258B DC_n7A-n26A-n78A-n258C DC_n7A-n26A-n78A-n258D DC_n7A-n26A-n78A-n258E DC_n7A-n26A-n78A-n258F DC_n7A-n26A-n78A-n258G DC_n7A-n26A-n78A-n258H DC_n7A-n26A-n78A-n258I DC_n7A-n26A-n78A-n258J DC_n7A-n26A-n78A-n258K DC_n7A-n26A-n78A-n258L DC_n7A-n26A-n78A-n258M DC_n7A-n26A-n78A-n258R2 DC_n7A-n26A-n78A-n258R3 DC_n7A-n26A-n78A-n258R4 DC_n7A-n26A-n78A-n258R5 DC_n7A-n26A-n78A-n258R6 DC_n7A-n26A-n78A-n258R7 DC_n7A-n26A-n78A-n258R8 DC_n7A-n26A-n78A-n258R9 DC_n7A-n26A-n78A-n258R10 | DC_n7A-n26A DC_n7A-n78A DC_n7A-n258A/G/H/I/R2/R3/R4 DC_n26A-n78A DC_n26A-n258A/G/H/I/R2/R3/R4 DC_n78A-n258A/G/H/I/R2/R3/R4 |
| DC_n7B-n26A-n78A-n258A DC_n7B-n26A-n78A-n258B DC_n7B-n26A-n78A-n258C DC_n7B-n26A-n78A-n258D DC_n7B-n26A-n78A-n258E DC_n7B-n26A-n78A-n258F DC_n7B-n26A-n78A-n258G DC_n7B-n26A-n78A-n258H DC_n7B-n26A-n78A-n258I DC_n7B-n26A-n78A-n258J DC_n7B-n26A-n78A-n258K DC_n7B-n26A-n78A-n258L DC_n7B-n26A-n78A-n258M DC_n7B-n26A-n78A-n258R2 DC_n7B-n26A-n78A-n258R3 DC_n7B-n26A-n78A-n258R4 DC_n7B-n26A-n78A-n258R5 DC_n7B-n26A-n78A-n258R6 DC_n7B-n26A-n78A-n258R7 DC_n7B-n26A-n78A-n258R8 DC_n7B-n26A-n78A-n258R9 DC_n7B-n26A-n78A-n258R10 | DC_n7A-n26A DC_n7A-n78A DC_n7A-n258A/G/H/I/R2/R3/R4 DC_n26A-n78A DC_n26A-n258A/G/H/I/R2/R3/R4 DC_n78A-n258A/G/H/I/R2/R3/R4 |
| DC_n28A-n77A-n79A-n257A DC_n28A-n77A-n79A-n257G DC_n28A-n77A-n79A-n257H DC_n28A-n77A-n79A-n257I | DC_n28A-n77A DC_n28A-n79A DC_n28A-n257A/G/H/I DC_n77A-n79A DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n28A-n77(2A)-n79A-n257A DC_n28A-n77(2A)-n79A-n257G DC_n28A-n77(2A)-n79A-n257H DC_n28A-n77(2A)-n79A-n257I | DC_n28A-n77A DC_n28A-n79A DC_n28A-n257A/G/H/I DC_n77A-n79A DC_n77A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n28A-n78A-n79A-n257A DC_n28A-n78A-n79A-n257G DC_n28A-n78A-n79A-n257H DC_n28A-n78A-n79A-n257I | DC_n28A-n257A/G/H/I DC_n78A-n257A/G/H/I DC_n79A-n257A/G/H/I |
| DC_n77A-n79A-n257A-n259A DC_n77A-n79A-n257A-n259G DC_n77A-n79A-n257A-n259H DC_n77A-n79A-n257A-n259I DC_n77A-n79A-n257A-n259J DC_n77A-n79A-n257A-n259K DC_n77A-n79A-n257A-n259L DC_n77A-n79A-n257A-n259M DC_n77A-n79A-n257G-n259A DC_n77A-n79A-n257G-n259G DC_n77A-n79A-n257G-n259H DC_n77A-n79A-n257G-n259I DC_n77A-n79A-n257G-n259J DC_n77A-n79A-n257G-n259K DC_n77A-n79A-n257G-n259L DC_n77A-n79A-n257G-n259M DC_n77A-n79A-n257H-n259A DC_n77A-n79A-n257H-n259G DC_n77A-n79A-n257H-n259H DC_n77A-n79A-n257H-n259I DC_n77A-n79A-n257H-n259J DC_n77A-n79A-n257H-n259K DC_n77A-n79A-n257H-n259L DC_n77A-n79A-n257H-n259M DC_n77A-n79A-n257I-n259A DC_n77A-n79A-n257I-n259G DC_n77A-n79A-n257I-n259H DC_n77A-n79A-n257I-n259I DC_n77A-n79A-n257I-n259J DC_n77A-n79A-n257I-n259K DC_n77A-n79A-n257I-n259L DC_n77A-n79A-n257I-n259M | DC_n77A-n79A DC_n77A-n257A/G/H/I DC_n77A-n259A/G/H/I/J/K/L/M DC_n79A-n257A/G/H/I DC_n79A-n259A/G/H/I/J/K/L/M |