3GPP RAN WG4 Meeting #113																	R4-2418048

Orlando, USA, 18th – 22nd November 2024

Agenda item:	6.16.1

Source:	Apple

Title:	n68: Introduction of the new band and NS_26 / NS_36

WI/SI:	NR_band_n68-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

The WF [1] captures agreements for further simulations  on A-MPR for band n68. It was agreed to re-run simulations with a certain duplex filter characteristic. This contribution provides simulations results including the agreed duplex characteristic.

# 2  Discussion

##### 2.1 Simulation assumptions

The following assumptions and requirements for the simulations are applied:

- Power Class 3

- NR Waveform

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- CIM5: 70dBc

- Requirements from NS_26 and NS_36

##### 2.2 Simulation results for NS_26

This section presents the QPSK simulation results for 5MHz, 10MHz and 15MHz channels.

Fig 1: Simulation results for NS_26 at lower band edge with QPSK modulation

To check whether A-MPR is needed for channels placed with a gap to the lower band edge we conducted simulations with a gap of CBW/2. The 5MHz channel and the 10MHz channel do not require additional power back-off. The 15MHz channel required a larger gap of  9.7MHz.

Observation: No A-MPR is needed for a 5MHz and a 10MHz channel if the gap to the lower band edge is CBW/2 or larger. For a 15MHz channel the gap needs to be 9.7MHz or larger.

##### 2.3 Simulation results for NS_36

Due to the short time between RAN4#113 and the previous meeting we couldn’t complete all simulations in time. We plan to provide the simulation results in the February next meeting.

# 4	Conclusions

This contribution provides simulation results for NS_26. Due to the short time between RAN4#113 and the previous meeting the results for NS_36 will be provided next meeting. This paper makes following observation:

Observation: No A-MPR is needed for a 5MHz and a 10MHz channel if the gap to the lower band edge is CBW/2 or larger. For a 15MHz channel the gap needs to be 9.7MHz or larger.

# 4	References

- R4-2417080, “WF on A-MPR for NR band n68”, Ericsson, 3GPP TSG-RAN WG4 Meeting # 112bis