3GPP TSG-RAN WG4 Meeting #113 	R4-2418063

Orlando, Florida, USA, 18th – 22nd November, 2024

Source:	T-Mobile USA

Title:	TP for TR38.746 for DL CA_n41-n66-n77 with 3Tx 2BUL

Agenda item:	6.8.3

Document for:	Approval

- Introduction

This contribution is a text proposal to introduce PC2 and PC1.5 3Tx 2BDL/2BUL for DL CA_n41-n66-n77.

The power class 1.5 MSD is calculated by calculating the interference power for PC2 IMD MSD, increasing the interference by 3 dB * IMD order, and then calculating the resulting MSD for PC1.5. The equations are included.

- Reference

[1] RP-242003, Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

- Text Proposal

<Beginning of Text Proposal>

## 6.x	CA_n41-n66-n77 3Tx

### 6.x.1	Operating bands for CA and Configurations

Table 6.x.1-1:  CA band combination of band n25+n41+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n25-n41-n77 | n25, n41, n77 |

Table 6.x.1-1:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A |  | n417,9 n777,9 CA_n41A-n66A7,13,14 CA_n41A-n77A7 CA_n66A-n77A7,13,14 |  | n41 |  | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 6.x.2	Maximum output power for inter-band CA

### No changes needed to the 3Tx tables in 38.101-1.

Table 6.x.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 6.x.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 6.x.3	MSD scenario studies

3BDL/2BUL configurations only have MSD requirements for intermodulation.

Table 6.x.3-1 and 6.x.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for DL CA_n41-n66-n77.

Table 6.x.3-1: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 2

| Uplink combination |  | IMD |  | Aggressor Tx |  | Victim Rx |  |  | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A |  | IMD3 |  | 2*f n41 – f n66 |  | n77 |  |  | Yes |
| CA_n66A-n77A |  | IMD5 |  | 4*f n66 – f n41 |  | n41 |  |  | Yes (But not testable for US n77 configurations) |

Table 6.x.3-2: Intermodulation MSD scenarios for DL CA_n25A-n41A-n77A with total power class 1.5

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | No |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n77 | No |

### 6.x.4	REFSENS requirements

#### 6.x.4.1	REFSENS requirements for total power class 2

- The MSD due to intermodulation PC2 for UL CA_n41A-n66A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n66A-n77A is already specified in 38.101-1.

#### 6.x.4.2	REFSENS requirements for total power class 1.5

For UL CA_n41A-n66A PC1.5, since the n66 power remains PC3, the n41 power increases by 4.7 dB compared to UL CA_n41A-n66A PC2.

For the UL CA_n41A-n66A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=2*4.7 = 9.4 dB, so the MSD increases from 25.0 dB to 34.4 dB, using the following equation:

New_MSD

For the UL CA_n66A-n77A IMD5, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 18.7 dB to 23.4 dB, using the equation above.

The MSD due to IMDs for PC1.5 3Tx are specified in the following table. At the time of this TP, the 3DL/2UL PC1.5 table does not yet exist in 38.101.1.

Table 6.x.4.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  | Source of IMD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  |  | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |  |
| CA_n41-n66-n77 |  |  | n41 |  | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |  |  |
|  |  |  | n66 | 1730 |  | 5 | 25 | 2130 | N/A | FDD | N/A |  |  |
|  |  |  | n77 | N/A |  | 10 | N/A | 3470 | 34.4 | TDD | IMD31,2 |  |  |
|  |  |  | n41 | N/A |  | 5 | N/A | 2670 | 23.4 | TDD | IMD55 |  |  |
|  |  |  | n66 | 1715 |  | 5 | 25 | 2115 | N/A | FDD | N/A |  |  |
|  |  |  | n77 | 4190 |  | 10 | 50 | 4190 | N/A | TDD | N/A |  |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Text Proposal>