3GPP TSG-RAN WG4 Meeting #113	R4-2418087

Orlando, US, 18th – 22nd November, 2024

Agenda item:	4.5.2

Source: 	Qualcomm Incorporated

Title: 		Low SNR in RedCap test-cases

Document for:	Discussion

# Introduction

For release 17 RedCap performance requirements, RAN4 specified the test-cases for 1Rx and 2Rx RedCap UEs.

Most of the test-cases follow the same configuration and same SNR for both 1Rx and 2Rx UEs. However, the demod requirements as well as the REFSENS requirements provide relaxation of about 2.5-3db for 1Rx UEs as compared to 2Rx UEs.

In the test-cases for SS-SINR and SS-RSRQ accuracy requirements for 1Rx, UEs are being tested at the same SNR level as 2Rx UEs, which is quite low (i.e., Es/Iot = -5.46db).

|  | dB | -1.76 |  | -4.7 |  | -5..46 | -5.46 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |

At such SNR level, while 2Rx UEs can successfully receive the data, 1Rx UEs cannot, leading to UE declaring RLF and failing the test. Since the purpose of these test is not to test the UE for demod performance, but rather to test the SS-SINR and SS-RSRQ accuracy requirements, the test conditions should ensure that the demod is clean. Currently, MCS4 is being used for all RRM test-cases and demod requirements show that a higher SNR is needed to decode MCS4 for 1Rx UEs.

There are two ways to address this problem:

- Specify sufficiently high SNR in the test case

- Use lower MCS in the test case.

We can raise the test-case SNR by about 3db or use a lower MCS such as MCS0 in the test cases. Since, we currently don’t have any RMCs corresponding to MCS0, we may need to introduce a new RMC if we go with solution 2. For solution 1, we need to evaluate the correct SNR for the test-cases.

Observation 1: RAN4 specifies the same SNR and same MCS for both 1Rx and 2Rx UEs in RRM test-cases.

Observation 2: UE cannot decode MCS4 at very low SNR in SS-SINR and SS-RSRQ measurement accuracy test-cases for 1Rx RedCap UEs and may declare RLF.

Observation 3: The purpose of these test is not to test UEs demod performance.

Proposal 1: RAN4 to discuss the solutions to address UE encountering RLF during the SS-SINR and SS-RSRQ measurement accuracy test-cases for 1Rx RedCap UEs. Two potential solutions are:

- Use a lower MCS (e.g., MCS0) in the test-cases

- Use a higher SNR (e.g., 3db higher) in the test-cases

Note: We have reserved a tdoc number (R4-2418085) for a CR to submit the changes needed based on the discussion.

# Conclusion

Observation 1: RAN4 specifies the same SNR and same MCS for both 1Rx and 2Rx UEs in RRM test-cases.

Observation 2: UE cannot decode MCS4 at very low SNR in SS-SINR and SS-RSRQ measurement accuracy test-cases for 1Rx RedCap UEs and may declare RLF.

Observation 3: The purpose of these test is not to test UEs demod performance.

Proposal 1: RAN4 to discuss the solutions to address UE encountering RLF during the SS-SINR and SS-RSRQ measurement accuracy test-cases for 1Rx RedCap UEs. Two potential solutions are:

- Use a lower MCS (e.g., MCS0) in the test-cases

- Use a higher SNR (e.g., 3db higher) in the test-cases