3GPP TSG-RAN WG4 Meeting #113                                                               R4-2418099

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	On the modification of existing TX requirements for SBFD-capable BS

Agenda item:			7.20.2.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111, #112 and #112bis, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8] [10]. In this contribution, accordingly, we would continue to provide our analysis and viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing TX requirement for SBFD

## 2.1  Base Station output power and radiated transmit power

For the requirement of BS output power and radiated transmit power, the following agreements is achieved in the study phase [2]:

Since configuration (e.g. antenna, power configuration etc) between SBFD and non-SBFD symbols/slots might be different, RAN4 reached the following consensus for the BS RF requirement of BS output power for both conducted and OTA output power: - It is allowed to have the different conducted declaration for normal DL symbols/slots and SBFD DL symbols/slots. - It is allowed to have different EIRP/TRP declaration (for level and direction) for normal DL symbols/slots and SBFD DL symbols/slots. - Accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

And the following agreement is achieved in RAN4#111 [6]:

Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols - Agreement: - PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols - Vendors can declare different TX power values for normal and SBFD slots/symbols

Based on the above agreements achieved in study item and RAN4#111, we expect the manufacturer declaration-based method will be used for base station output power, i.e., radiated transmit power and OTA base station output power, and further discussion could be expected on the specification implementation by allowing different declarations of output power for normal DL symbols and SBFD DL symbols. Based on our accompanying contribution on general aspects for SBFD BS RF requirement, we suggest that RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B. Accordingly, the following proposal is given.

Proposal 1: For conducted output power and radiated transmit power (radiated transmit power and OTA base station output power), the following new sub-clauses with suffix-B shall be adopted to capture the corresponding declaration for SBFD symbol:

- New clause 6.2B, titled as “Base station output power for SBFD”

- New clause 9.2B, titled as “Radiated transmit power for SBFD”

- New clause 9.3B, titled as “OTA base station output power for SBFD”

Taking the conducted requirement 6.2B “Base station output power for SBFD” as example, we see the following text proposal can be given for the corresponding requirement for SBFD operation.

Proposal 2: The following text proposal is adopted for BS output power requirement (taking the new clause 6.2B, titled as “Base station output power for SBFD” as example):

=================== Start of Text Proposal ===================

## 6.2B	Base station output power for SBFD

### 6.2.1B	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

### 6.2.2B	Minimum requirement for BS type 1-C

The same requirement as clause 6.2.2 is applicable to BS output power requirement for SBFD operation.

### 6.2.3B	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

## 2.2  Output power dynamics

In the Rel-18 study item, the following conclusion has been reached for output power dynamics:

#### 10.1.2.2 Output power dynamics Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus: - To reuse the existing RE power control dynamic range requirement for SBFD BS; - The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, it is agreed to define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Furthermore, in RAN4#112, the following agreement is achieved [8]:

Agreement: - Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement.

The major discussion in RAN4#112bis focus on how to have a formula to specify the total power dynamic range requirement for SBFD. Based on our accompanying discussion paper on the general aspects [R4-2418097], we propose to use NRB,SBFD,DL,1 and NRB,SBFD,DL,2 to represent the DL subband configuration for DUD case:

<Proposed symbol explanations in our accompanying Tdoc> - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks. - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU or UD case, expressed in resource blocks. - NRB,SBFD,GB,1: The size of guardband between SBFD UL subband and the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks. - NRB,SBFD,GB,2: The size of guardband between SBFD UL subband and the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

Based on the above symbols, the requirements can be specified accordingly.

Proposal 3: The requirement of total power dynamic range for SBFD shall be specified based on transmission bandwidth configuration for SBFD DL subband(2), i.e.,

- The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

Based on the above discussion, we further propose to setting up the new sub-clauses 6.3.3B (Total power dynamic range for SBFD) and 9.4.3B (OTA total power dynamic range for SBFD) by defining the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Proposal 4: The following text proposal is adopted for BS output power dynamics requirement (taking the new clause 6.3.3B, titled as “Total power dynamic range for SBFD” as example):

=================== Start of Text Proposal ===================

### 6.3.3B	Total power dynamic range for SBFD

#### 6.3.3.1B	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 6.3.3.2	Minimum requirement for BS type 1-C and BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

## 2.3 Transmit ON/OFF power

For the requirement of transmit ON/OFF power, the following agreement is provided in the study item:

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

For the study item conclusion that “transmit ON/OFF power requirement is not applicable within SBFD time slot”, we expect there are two options for specification implementation: (1) adding Notes in both clause 6.4.1.1 and 9.5.2.1; (2) adding new sub-clauses 6.4.1B and 9.5.2B for SBFD operation. Among these two options, “adding Notes in both clause 6.4.1.1 and 9.5.2.1” is preferred by us for its simplicity.

Proposal 5: To implement agreements, RAN4 can add Notes in both clause 6.4.1.1 and 9.5.2.1 to avoid OFF power requirement to be applied for SBFD symbols, i.e.,

- “Transmitter OFF power requirements shall not be applied in SBFD symbols.”

## 2.4 Transmitted signal quality

For the requirement transmitted signal quality requirement, particularly for EVM measurement, the following bullet is agreed for separate measurement over SBFD and non-SBFD symbols:

Agreement: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

As mentioned in some companies’ contributions, the detailed averaging method could be updated for SBFD, based on the following procedure for TDD:

For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×NslotNdlTDD and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2

Because the SBFD time domain configuration is based on the symbol-level granularity, so we see the necessity to use the number of SBFD symbols and non-SBFD DL symbols within a 10 ms measurement interval, and define the minimum SBFD symbols and non-SBFD DL symbols for averaging in a 10 ms measurement interval.

Proposal 6: To update the EVM measurement procedure for SBFD operation, the procedure of averaging EVM shall be revised by considering the number of SBFD symbols and non-SBFD DL symbols within a 10 ms measurement interval.

## 2.5 Unwanted emissions

For the ACLR requirement of unwanted emissions, the following agreements is achieved in the study phase [2]:

- For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

In RAN4#111, the following agreement is achieved:

- Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement. - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

By separating the discussion on ACLR requirement by (1) coexisting with legacy TDD system in adjacent channel and (2) coexisting with new SBFD system in adjacent channel, the following agreement and way forward is achieved in RAN4#112:

## 1.3. ACLR and OBUE Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

Especially for the scenario in which SBFD BS coexisting with new SBFD BS in the adjacent channel, RAN4 has not yet achieved further progress in RAN4#112bis.

Based on our understanding, for the scenario of SBFD BS coexisting with new SBFD BS in the adjacent channel, although it is not explicitly analyzed in the Rel-18 study phase, the situation could be similar as Case 3 (the aggressor as NR TDD DL and victim as SBFD DU). The Case 3 results can be applied for the SBFD-SBFD coexistence scenario due to the assumption of flat adjacent channel interference modelling.

For the unwanted emission requirement for BS coexisting with new SBFD BS in the adjacent channel, it is FFS that RAN4 shall define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. Based on the technical analysis in [R4-2413283], it is proposed that the new OBUE requirement can be defined with tightened emission mask. Similarly, the ACLR requirement when coexisting with new SBFD BS in the adjacent channel is another option. Introducing either of new tightened requirements can be regarded as the requirement of implementing TX filtering before band filter. By investigating the victim BS DESENS, we propose to reuse the 1dB DESENS assumption used in Rel-18.

Proposal 7: RAN4 only discuss the necessity of the additional ACLR and/or OBUE requirements for FR1 macro BS.

Proposal 8: The support of additional ACLR and/or OBUE requirements for FR1 macro BS is manufacture declaration based, for certain gNB.

If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

Proposal 9: If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

To ensure feasible coexistence with SBFD-capable gNB, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

Proposal 10: If the method of new OBUE requirement is considered, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

## 2.6 Transmitter intermodulation

Based on last meeting’s agreement, there will be no receiver requirement specified for transmitter intermodulation requirement applicable in SBFD symbols, i.e.,

Issue 2-2-1: TX intermodulation requirement - Agreement: - The transmitter intermodulation requirement is applicable in SBFD slots/symbols: - Follow the existing requirement - No receiver requirement is specified.

To implement the above agreement, we have the following simple proposal:

Proposal 11: For transmitter intermodulation requirement, RAN4 can specify a new sub-clause 6.7B (Transmitter intermodulation for SBFD) and a new sub-clause 9.8B (OTA transmitter intermodulation for SBFD), by simply explain that the existing requirement shall be applied while no receiver requirement is specified for SBFD symbols.

Proposal 12: The following text proposal is adopted for transmitter intermodulation requirement (taking the new clause 6.7B, titled as “Transmitter intermodulation for SBFD” as example):

=================== Start of Text Proposal ===================

## 6.7B	Transmitter intermodulation for SBFD

The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied.

=================== End of Text Proposal ===================

# 3. Conclusion

In this contribution, we provided the viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Base Station output power and radiated transmit power

Proposal 1: For conducted output power and radiated transmit power (radiated transmit power and OTA base station output power), the following new sub-clauses with suffix-B shall be adopted to capture the corresponding declaration for SBFD symbol:

- New clause 6.2B, titled as “Base station output power for SBFD”

- New clause 9.2B, titled as “Radiated transmit power for SBFD”

- New clause 9.3B, titled as “OTA base station output power for SBFD”

Proposal 2: The following text proposal is adopted for BS output power requirement (taking the new clause 6.2B, titled as “Base station output power for SBFD” as example):

=================== Start of Text Proposal ===================

## 6.2B	Base station output power for SBFD

### 6.2.1B	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

### 6.2.2B	Minimum requirement for BS type 1-C

The same requirement as clause 6.2.2 is applicable to BS output power requirement for SBFD operation.

### 6.2.3B	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

Output power dynamics

Proposal 3: The requirement of total power dynamic range for SBFD shall be specified based on transmission bandwidth configuration for SBFD DL subband(2), i.e.,

- The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

Proposal 4: The following text proposal is adopted for BS output power dynamics requirement (taking the new clause 6.3.3B, titled as “Total power dynamic range for SBFD” as example):

=================== Start of Text Proposal ===================

### 6.3.3B	Total power dynamic range for SBFD

#### 6.3.3.1B	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 6.3.3.2	Minimum requirement for BS type 1-C and BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

Transmit ON/OFF power

Proposal 5: To implement agreements, RAN4 can add Notes in both clause 6.4.1.1 and 9.5.2.1 to avoid OFF power requirement to be applied for SBFD symbols, i.e.,

- “Transmitter OFF power requirements shall not be applied in SBFD symbols.”

Transmitted signal quality

Proposal 6: To update the EVM measurement procedure for SBFD operation, the procedure of averaging EVM shall be revised by considering the number of SBFD symbols and non-SBFD DL symbols within a 10 ms measurement interval.

Unwanted emissions

Proposal 7: RAN4 only discuss the necessity of the additional ACLR and/or OBUE requirements for FR1 macro BS.

Proposal 8: The support of additional ACLR and/or OBUE requirements for FR1 macro BS is manufacture declaration based, for certain gNB.

Proposal 9: If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

Proposal 10: If the method of new OBUE requirement is considered, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

Transmitter intermodulation

Proposal 11: For transmitter intermodulation requirement, RAN4 can specify a new sub-clause 6.7B (Transmitter intermodulation for SBFD) and a new sub-clause 9.8B (OTA transmitter intermodulation for SBFD), by simply explain that the existing requirement shall be applied while no receiver requirement is specified for SBFD symbols.

Proposal 12: The following text proposal is adopted for transmitter intermodulation requirement (taking the new clause 6.7B, titled as “Transmitter intermodulation for SBFD” as example):

=================== Start of Text Proposal ===================

## 6.7B	Transmitter intermodulation for SBFD

The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied.

=================== End of Text Proposal ===================

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411638, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.

[9] R4-2416499, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112bis.

[10] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#112bis.