3GPP TSG-RAN WG4 Meeting # 113	R4-2418106

# Orlando, USA, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM impacts of SBFD operation

Agenda item:	7.20.3.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remain open issues on RRM impacts of SBFD operation described in WF [1].

Discussion

Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

We support option 1. Because in case of option 2, in our understanding, the UE for inter-frequency measurement without gap can operate concurrently DL receive and SSB measurement. But if the UE transmit with measure SSB concurrently, even though the RF structure can support, the SSB measurement performance can be impacted by the UL transmission. So, we think that for inter-frequency measurement without gap UL transmission should not be prioritized.

Proposal 1: RAN4 should not support prioritized UL transmission for inter-frequency measurement without gap. Support option 1.

Issue 1-2-4: Requirements for RACH requirements Recommended WF - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

During last RAN2#127bis meeting, RAN2 made an agreement regarding RACH resource selection as below:

RACH resource selection - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

According to the agreement, the UE is allowed to switch from SBFD RACH occasions to legacy RACH occasions after certain (configured) number of times of RACH attempt. And the opposite direction also can be discussed further from RAN2. RAN4 need to check whether the RACH occasion switching can be impact RRM.

Proposal 2: RAN4 need to update the correct behavior for the RACH occasion switching operation. And other RRM impact by RACH occasion switching operation also need to check.

Issue 1-2-7: Requirements for UL resource muting Recommended WF - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

Based on the feature of UL resource muting for PUSCH, the UE can measure CLI at muted RE. Currently RAN1 assuming comb-2 muting and up to 2 OFDM symbols in time domain. For keeping the time domain power level, the transmit power of PUSCH REs in the same OFDM symbols are considered 3dB power boosting.

RF part need to check the impact on transmit signal quality/MPR requirement. But RRM perspective, there is no spec. impact.

Proposal 3: RAN4 RRM does not need to consider the impact by UL resource muting for PUSCH feature.

Summary

Proposal 1: RAN4 should not support prioritized UL transmission for inter-frequency measurement without gap. Support option 1.

Proposal 2: RAN4 need to update the correct behavior for the RACH occasion switching operation. And other RRM impact by RACH occasion switching operation also need to check.

Proposal 3: RAN4 RRM does not need to consider the impact by UL resource muting for PUSCH feature.

Reference

[1] R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei