3GPP TSG-RAN WG4 Meeting #113		R4-2418115

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.26.4.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

The objectives of Rel-19 “Non-Terrestrial Networks (NTN) for NR Phase 3” described in latest WID [1]:

The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In last RAN4#112bis meeting, we have already discussed RRM impacts for objective #1; #2; #3 and #4 in NTN and achieved some agreements in [2]. There are still some open issues in [2]. In this contribution, we continue to discuss the open issues of RRM requirements for DL coverage enhancement.

- Discussion

In RAN4#112bis meeting, the RRM impacts of objective #1 DL coverage enhancement are still open in [2] as below:

### Issue 1-1: SSB periodicity enhancement Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc. ### Issue 1-3: RRM impacts due to Satellite Tx Beam Hopping Agreement: RAN4 to wait for more progress from RAN1/2 on the detailed solutions to support a satellite T/Rx beam sweeping pattern, e.g. to adopt the concept of Rel-18 cell DTX/DRX, before discussing whether and how to modify the relevant RRM requirements.

In last RAN1#118bis meeting, the agreement for SSB periodicity extension has been approved as:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms.

In RAN1 discussion, only SSB periodicity is changed to 160ms for initial access and FFS on 320ms by now. In RAN4 RRM requirements, there is no requirements for initial cell search.

However, in RRC_Idle mode/Inactive modes, the cell reselection requirements are based on the DRX and SMTC. The SMTC period can be the same or different from SSB period. However, in SMTC duration time, if there is always no SSB, it doesn’t make sense for the measurement.

Observation 1: In RAN1, only SSB periodicity is changed to 160ms for initial access and FFS on 320ms. No RAN4 requirements for initial cell search.

Observation 2: RAN4 cell reselection requirements are based on SMTC and DRX.

In the RAN1 discussion of hopping beams, RAN1’s agreement is only X simultaneously active beams of 1058 total beams in FR1. X can be 16 or other values for different configurations. It implied that only some beams are active in the total Y (for example: 160ms) periodicity. Each beam can only exist Y/(4SSB) period. Current SMTC for measurement may not work well for the hopping beam scenario.

In RAN1 LS to RAN2 [3]:

Answer to Q4: Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

By our understanding of the LS, this “off” is in RAN1 discussion. How/Whether to specify it is still pending.

Observation 3: Current SMTC for measurement nay not work well for the hopping beam scenario. SMTC might be changed in hopping beam scenario. How/Whether to specify for hopping beams are still pending.

In the WID, the bullet shows: Rel-18 network energy saving techniques should be considered as baseline in the system level study. In Rel-18, cell level DTX are introduced. However, cell level DTX doesn’t affect DL measurement RSs. But in this NTN hopping beam scenarios, most beams may be in off states which means no any DL signals including DL measurement RSs.

Observation 4: In Rel-18 NES, no DL measurement impacts due to cell level DTX.

Observation 5: Different from Rel-18 NES cell level DTX, NTN beams may be in “off” with no DL measurement RS.

In RAN4 RRC_Idle mode and RRC_Connected mode, the measurements are based on SMTC/MG/SSB(CSI-RS) period/DTX pattern.

In RRC_Connected mode, until now, RAN1 haven’t extend the SSB periodicity which means the legacy maximum SSB periodicity are still considered (160ms).

Proposal 1: No RAN4 requirements for initial cell search. RRM requirements such as cell (re)-selection, L3 measurement/L1 measurement, RLM/BFD, etc. are based on the SMTC/MG/SSB(CSI-RS) period/DTX pattern. SMTC may need update and DTX pattern should be considered of hopping beams. RAN4 to discuss the measurement in NTN hopping beams scenario.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for DL coverage enhancement:

Observation 1: In RAN1, only SSB periodicity is changed to 160ms for initial access and FFS on 320ms. No RAN4 requirements for initial cell search.

Observation 2: RAN4 cell reselection requirements are based on SMTC and DRX.

Observation 3: Current SMTC for measurement may not work well for the hopping beam scenario. SMTC might be changed in hopping beam scenario. How/Whether to specify for hopping beams are still pending.

Observation 4: In Rel-18 NES, no DL measurement impacts due to cell level DTX.

Observation 5: Different from Rel-18 NES cell level DTX, NTN beams may be in “off” with no DL measurement RS.

Proposal 1: No RAN4 requirements for initial cell search. RRM requirements such as cell (re)-selection, L3 measurement/L1 measurement, RLM/BFD, etc. are based on the SMTC/MG/SSB(CSI-RS) period/DTX pattern. SMTC may need update and DTX pattern should be considered of hopping beams. RAN4 to discuss the measurement in NTN hopping beams scenario.

- Reference

[1] RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

[2] R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm

[3] R1-2407538, Draft Reply LS on DL coverage enhancements, RAN1