3GPP TSG-RAN WG4 Meeting # 113	R4- 2418163

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Agenda item:	7.22.2.2

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of adaptation of SSB for Rel-19 NES.

Discussion

General

In the previous meeting, following agreement was captured in WF[2].

Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement(RAN4#112bis): - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement(RAN4#112bis): - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

In RAN4#112bis meeting, scenarios and impacted requirements has been agreed, and sent LS to RAN2 regarding SMTC configuration . In addition, requirements of transition period between SSB adapatation indication has been discussed, but not captured in WF.

Until now, basic mechanisms of SSB adapatation are still open. However, there was not much progress in the previous meeting. Only precluding IDLE mode was concluded. Given the situation, we think RAN4 needs to focus on L1 measurement and transition period first. Once RAN2 decides whether to introduce new SMTC configuration, L3 measurement can be revisited.

Proposal 1: RAN4 to focus on L1 measurement requirement, and defer L3 measurement requirement

L1 measurement requirements

In general, L1 measurement can be happened before SSB adaptation and after SSB adaptation. As shown in Table 1, in the legacy requirement of L1 measurement, the period of SSB is used for the requirements. For L1-RSRP, L1-SINR, L1 measurement during SCell activation, TCI state swiching delay, ssb-periodicityServingCell which is the periodicity of the SSB-Index configured for L1-RSRP measurement are used. On the other hand, BFD and CBD, the periodicity of SSB in the set q0 and q1 are used.

Since the period of SSB used for the legacy requirements are configured in RRC or static way, it may not be applied to SSB adaptation, which changes periodicity of SSB dynamically or semi-persistently. Details can be decided based on the other WG progress, but we think the indicated period of SSB should be used for L1 measurement in principle.

Table 1 L1 measurement requirements for SSB adaptation

| Requirements | As is | To be |
| --- | --- | --- |
| L1-RSRP, L1-SINR, L1 measurement during SCell activation, TCI state swiching delay | ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement | the periodicity of the SSB-Index indicated for SSB adaptation configured for L1-RSRP measurement |
| BFD ,CBD | the periodicity of SSB in the set q0, q1 | the periodicity of SSB indicated for SSB adaptation in the set q0,q1 |

Proposal 2: Apply indicated period of SSB adaptation for L1 measurement requirement

Whether and how to define requirements for “transition period”

When the period of SSB is changed due to SSB adaptation transition period can be occurred. From UE perspective, SSB adaptation is just changing configurations, which is already specified in legacy requirements in 8.5.10, 9.1.6, and 9.5.7 as highlighted below. In our understanding, transition period due to SSB adaptation falls into these legacy transition requirement. We fail to see motivation to introduce new transition requiremens. In conclusion, we think reusing legacy transition requirements is reasonable.

Proposal 3: Reusing legacy transition requirements for SSB adapation

8.5.10 Minimum requirement at transitions for beam failure detection When the UE transitions from a first configuration of BFD resources to a second configuration of BFD resources that is different from the first configuration, for each BFD resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each BFD resource present in the second configuration. 9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. 9.5.7 Minimum requirement at transitions For UE supporting ncd-SSB-BWP-Wor-r18, when the L1-RSRP measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching during one measurement period, the UE shall use a L1-RSRP measurement period that is the maximum of the L1-RSRP measurement period corresponding to the first SSB type and the second SSB type after the BWP switching.

Conclusion

In this contribution, we provide views on RRM requireiments for SSB adaptation, and we propose

Proposal 1: RAN4 to focus on L1 measurement requirement, and defer L3 measurement requirement

Proposal 2: Apply indicated period of SSB adaptation for L1 measurement requirement

Proposal 3: Reusing legacy transition requirements for SSB adapation

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2416860, “WF on RRM requirements for Netw_Energy_NR_enh_Part2,” Huawei