3GPP TSG-RAN4 Meeting #113	R4-2418172

# Orlando, US, Nov 18 – 22, 2024

Agenda item:			7.1.1.2.2

Source:	vivo

Title:	Discussion on MPR applicability for FR1 intra-band UL CA

Document for:	Discussion

- Introduction

In RAN#112bis meeting, the WF[1] has been agreed, and the remaining issue is as follows:

### Sub-topic 2-1: Intra-band contiguous UL CA #### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - WF - For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are indicated - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD and/or UL-MIMO capability are absent. - The Rel-18 power boosting feature can be supported depending on UE capability - Feature group description shall clarify the interaction between Rel-18 power boosting capabilities and Rel-19 MPR applicability enhancement capability. - FFS PC1.5 MPR enhancement as intra-band CA MPR requirements for PC1.5 are not available yet #### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: activated CC CBW or aggregated CBW adopted for integral region and boundary of spurious emissions/ACLR/SEM depending on dualPA-Architecture IE indication. - Proposal 2: Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated. - WF - FFS in next meeting #### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: CA requirements of ACLR/SEM/SE should be applied for intra-band contiguous CA with single activated cell. - Proposal 2: - For UE supporting contiguous ULCA with the dualPA IE: - The applicable SEM, ACLR and spurious emissions are the single CC emissions defined in Table 6.5.2.2-1, Table 6.5.2.4.1-1 and Table 6.5.3.1-2 respectively. - For UE supporting contiguous ULCA without signalling the dualPA IE or signalling PC1.5 or TxD or UL MIMO support: - The applicable SEM, ACLR and spurious emissions are the configured contiguous ULCA - Proposal 1: After the initial configuration of contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the aggregated CA BW SEM as stated in sub-clause 6.5A.2.2.1 in TS38.101-1. Further discuss SEM mask for single CC fall back only if dualPA-architecture is supported - Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. - WF - FFS in next meeting #### Issue 2-1-4: Whether UL interruption is allowed for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: No interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intra-band contiguous UL CA. - Agreement in AH - Proposal 1 ### Sub-topic 2-2: Intra-band non-contiguous CA #### Issue 2-2-1: Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals Option 1: No spec impact or minimum spec impact with some clarification but no changes of requirements - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. - Proposal 2: adding 1CC activation as a supplementary scenario in the spec but no changes of the requirements - Proposal 3: single CC MPR applied with single CC emission requirements: Option 2: Single CC MPR/SEM applied on indication of dualPA IE - Proposal 4: To apply non-CA (single CC) MPR based on indication of dualPA IE - Proposal 5: After the initial configuration of non-contiguous ULCA, upon SCELL deactivation with only 1CC active, the applied SEM mask should be the composite SEM of the 2CCs as stated in sub-clause 6.5A.2.2.2 in TS38.101-1. Further discuss to support the single CC SEM only if dualPA-architecture is supported. Option 3: Possible spec impact on LO retuning - Proposal 6: Discuss impact on requirements if LO needs to be re-configured to support single CC MPR. This discussion would be required for intra-band contiguous and non-contiguous carrier aggregation - WF (to be discussed in main session) - For PC2 and PC3, no MPR requirements to be changed. - FFS PC1.5 when NC CA requirements are available - FFS clarification for the applicable emission requirements whether they are single CC based or composite based - Further check whether LO retuning is out of WI scope, if not included, drop the corresponding discussion in Rel-19 Agreement in main session - Drop the discussion for LO retuning since it is out of WI scope - FFS for FR2

- Discussion

## 2.1. Sub-topic 2-1: Intra-band contiguous UL CA

According to the testing data and analysis of [2], when there is only one CC activated but the LO does not switch to the middle of the CC accordingly, the image would fall into the frequency range of the deactivated CC. And if the activated CBW is too narrow the image would be higher than the SEM mask of the activated single carrier, therefore a bigger MPR is needed to guarantee the output performance.

Based on the issue above, there are a couple of different schemes in the non-dualPA architecture:

Option 1. For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, the MPR and out-of-band emission requirements of the single carrier would be applied. At the same time, specifying a new MPR value which is larger than original single carrier in the case the that bandwidth of the activated CC is too narrow.

Option 2. For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, the MPR and out-of-band emission requirements of the single carrier would be applied with the premise that the LO could switch with the CC activating.

Option 3. For PC3/PC2 intra-band contiguous carrier aggregation with single CC activated, the MPR of the single carrier would be applied, and the out-of-band emission requirements should refer to the CA CBW.

For option 1, it is similar to the “exception case” with narrow RB allocation bandwidth in non-contiguous CA configuration, setting a higher MPR to leave some margin for the IMD product exceeding the spurious limit. But specifying a new MPR is not in the WID scope so option 1 should not be prioritized. As to option 2, it would limit the UE implementation and transmission interruption problems may occur, and according to the WF[1] in #112bis, no interruptions are allowed for enabling single carrier MPR. So option3 is preferred.

Observation 1: For PC3/PC2 intra-band contiguous CA with single CC with activated cell, when dualPA-Architecture IE is not indicated, the premise of using single carrier’s MPR is applying CA’s SE/ACLR/SEM.

In order to simplify the evaluation method and test procedure, and also considering the dynamic switching of activating 1CC or multi CCs, when dualPA-Architecture IE is indicated the integral region and boundary of SE/ACLR/SEM could also be based on the aggregated CBW.

Proposal 1: For PC3/PC2 intra-band contiguous CA with single CC with activated cell, the value, integral region and boundary of SE/ACLR/SEM should be based on CA and the aggregated bandwidth.

As for PC1.5, at least at this stage the evaluation of 1CC activation should be delayed as the corresponding discussion in the other thread has not completed yet.

Proposal 2: PC1.5 MPR enhancement should be excluded from the discussion, both for C CA and NC CA.

## 2.2. Sub-topic 2-2: Intra-band UL non-contiguous CA

For NC CA with 1CC activated, if each CC has a separate LO then the MPR could fall back to the single carrier level without any problem, and no spec changes are needed expect for adding 1CC activated as a supplementary scenario after configuring 1CC. However, if there is only one LO which could not switch with the activating state changed, and the RB numbers of the two CCs are allocated asymmetrical then the image of the activated CC would fall into the gap and exceed the SEM sometimes.

In the current spec, the MPR of 1CC configured with B < 9/15 MHz is specified to be higher than the single carrier as the IMD product of the configured narrow CC would fail the spurious limit requirement. If considering the image issue together the MPR value and the width of B should be re-evaluated to provide enough room for the worst transmission cases. But re-evaluation should be triggered in other items as it is out of scope. At the same time, LO retuning is also dropped in the WF of #112bis meeting.

From the analysis of [2], it seems a good choice to rely on UE implementation to suppress its image to a lower level and guarantee the utilization of single carrier’s MPR. Moreover, it is necessary to apply the integral region and boundary of SE/ACLR/SEM based on the aggregated CBW when dualPA-Architecture IE is not indicated. Follow the same mechanism of intra-band C CA, unifying the application method of dualPA-Architecture and non-dualPA-Architecture is simpler.

Proposal 3: For PC3/PC2 intra-band non-contiguous CA with single CC with activated cell, the value, integral region and boundary of SE/ACLR/SEM should be based on CA and the aggregated bandwidth.

Proposal 4: For PC3/PC2 intra-band non-contiguous CA with single CC with activated cell, no spec changes are needed except for adding 1CC activation as a supplementary scenario.

For UE indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively. For UE without indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used; For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used; For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

- Conclusion

Observation 1: For PC3/PC2 intra-band contiguous CA with single CC with activated cell, when dualPA-Architecture IE is not indicated, the premise of using single carrier’s MPR is applying CA’s SE/ACLR/SEM.

Proposal 1: For PC3/PC2 intra-band contiguous CA with single CC with activated cell, the value, integral region and boundary of SE/ACLR/SEM should be based on CA and the aggregated bandwidth.

Proposal 2: PC1.5 MPR enhancement should be excluded from the discussion, both for C CA and NC CA.

Proposal 3: For PC3/PC2 intra-band non-contiguous CA with single CC with activated cell, the value, integral region and boundary of SE/ACLR/SEM should be based on CA and the aggregated bandwidth.

Proposal 4: For PC3/ PC2 intra-band non-contiguous CA with single CC with activated cell, no spec changes are needed except for adding 1CC activation as a supplementary scenario.

For UE indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively Else If AND( FIM3,low_block,low > SEM-13,low , FIM3,high_block,high < SEM-13,high ) MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively. Else MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively. For UE without indicating dualPA-Architecture supported If OR( LCRB1 = 0, LCRB2 = 0, only CC1 is activated, only CC2 is activated) For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used; For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used; For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

- References

- R4-2417204, WF on power domain enhancements, Huawei, HiSilicon, RAN4 #112bis.

- R4-2415273, Using single CC MPR and ULCA related emissions when only one CC has RBs allocated, Skyworks Solutions, Inc. RAN4 #112bis.