3GPP TSG-RAN WG4 Meeting # 113 	R4-2418176

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on the UE parameter for 14800 to 15350 MHz

Agenda Item:	7.2.4.2

Document for:	Approval

- Introduction

In the last meeting, the parameters for 15GHz were discussed but there are many parameters still pending. In this contribution, we provide our views on these UE parameters.

- Discussion

- Channel bandwidth

Since this frequency range is target to next generation, how to achieve higher system performance compared to 5G need to be considered, and using larger bandwidth is the easiest way to do, so our preference is to use 200MHz as the typical channel bandwidth. However, companies have concern that we can not finished 200MHz related requirement on time since we need to send LS in this meeting. We can go with 100MHz in LS reply but larger BW should be recorded in TR.

Proposal 1: 100MHz is used in the LS reply and larger BWs are recorded in TR.

- Signal bandwidth

For the 4GHz and 8GHz LS, the following agreement is used in the LS:

- Quote formula of RBs * SCS without number of RBs

we can follow the same rule in the 15GHz.

Proposal 2: For the signal bandwidth in the 15GHz, it is suggested to quote formula of RBs * SCS without number of RBs.

- Power dynamic

There are two options in the last meeting：

- Option 1: 56dB for 100MHz assuming 23dBm

- Option 2: 56dB for 200MHz assuming 23dBm

It seems both option use 56dB as the value and the difference is channel bandwidth, which can be discussed separately.

Proposal 3： The power dynamic for 14800 to 15350MHz frequency range is 56 dB

- Emissions mask

For emission mask, there are also two options:

- Option 1: UE SEM given in Section 7.1.2, TR 38.921.

- Option 2: Other options.

Compared to the SEM for the frequency lower than 6GHz, the SEM in 38.921 is relaxed 3dB for the nearest 5MHz, which can help on the higher power UE design. Since we need send the LS reply in this meeting and don’t have enough time to agree on the other options, we can agree option 1 and the value can be revisited during WI phase.

Proposal 4: For the UE SEM, use the same mask in TR38.921

- Spurious emission

The spurious emission is from regulation, so it is not needed to make changes on it.

Proposal 5: For the spurious emission, the current requirement in TS 38.101-1 is reused.

- Conclusion

Proposal 1: 100MHz is used in the LS reply and larger BWs are recorded in TR.

Proposal 2: For the signal bandwidth in the 15GHz, it is suggested to quote formula of RBs * SCS without number of RBs.

Proposal 3：The power dynamic for 14800 to 15350MHz frequency range is 56 dB

Proposal 4: For the UE SEM, use the same mask in TR38.921.

Proposal 5: For the spurious emission, the current requirement in TS 38.101-1 is reused.

- References

- R4-2417108   WF for 15GHz UE parameters, RAN4#112bis, Qualcomm