3GPP TSG-RAN WG4 Meeting # 103 	R4-2418177

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Further co-existence evaluation on NTN HPUE

Agenda Item:	7.8.2.1

Document for:	Approval

- Introduction

- Consider following observations can be used as a starting point for RF requirements analysis/studies:

  - Candidate ACLR value for NR-NTN PC2/PC1.5/PC1, based on initial co-ex study results received in #112bis meeting, is 30 dBc

  - The current TN requirements are more stringent than the observations from co-existence results. Overlapping geographic coverage between NTN and TN is discussion in RAN4 in other WIs.

- FFS for final ACLR requirements

In the last meeting, based on the companies’ input, the following observation was achieved [1]:

Considering companies still have concern on reusing the PC3 ACLR for both NR NTN and IoT NTN, we provide more evaluations in this contribution.

- Discussion

  - Wrap-around assumption

Whether the wrap-around is enabled was discussed in several meetings. It is important to fully understand the influence of such technique. The details of wrap-around are record in [2], as listed below:

The UE may access to the image cluster if the coupling loss is smaller, and with such operation, we can mimic infinite cellular network and eliminate the boundary effect. However, when we evaluate the co-existence between TN and NTN, enabling wrap around actually means that the TN BS is still located in the isolation area and the interference from NTN will be higher than we assumed initially, which will lead to larger throughput loss for the UE at cell edge.

Table I Comparison between wrap-around is ON and OFF for NR NTN HPUE under Scenario 4

|  |  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| With Wrap- around | PC3 | 2.03 | 1.44 | 0.68 | 0.46 | 0.32 | 7.97 | 4.88 | 2.42 | 1.17 | 1.01 |
|  | PC2 | 2.90 | 1.67 | 1.01 | 0.69 | 0.57 | 12.04 | 6.81 | 3.97 | 2.47 | 1.78 |
|  | PC1.5 | 4.08 | 2.40 | 1.48 | 1.04 | 0.86 | 17.82 | 9.59 | 6.25 | 4.09 | 3.23 |
|  | PC1 | 5.07 | 3.04 | 1.90 | 1.35 | 1.12 | 22.64 | 12.67 | 7.46 | 5.43 | 4.59 |
| Without Wrap- around | PC3 | 1.91 | 1.10 | 0.67 | 0.47 | 0.39 | 7.89 | 3.86 | 2.02 | 1.60 | 1.37 |
|  | PC2 | 2.71 | 1.58 | 0.98 | 0.69 | 0.58 | 11.72 | 6.13 | 3.45 | 2.07 | 1.86 |
|  | PC1.5 | 3.79 | 2.15 | 1.41 | 1.01 | 0.85 | 16.00 | 9.33 | 5.45 | 3.62 | 2.80 |
|  | PC1 | 4.72 | 2.83 | 1.79 | 1.30 | 1.08 | 20.89 | 12.54 | 7.16 | 4.79 | 3.80 |

Observation 1: When the wrap-around is turned on, the UEs at TN edge may access to the image cluster which make the isolation distance is smaller than the assumption, leading to higher throughput loss at cell edge.

  - Isolation distance

During the offline discussion, some operator has concern on the 1.5km isolation distance, the following results shows the results when isolation distance is 0km. As we analyzed above, we disable the warp-around otherwise the TN and NTN area will overlap.

Table II NR/IoT NTN GEO and TN Uma with AAS antenna (Isolation distance=0km)

|  |  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| NR NTN | PC3 | 1.91 | 1.10 | 0.67 | 0.47 | 0.39 | 7.89 | 3.86 | 2.02 | 1.60 | 1.37 |
|  | PC2 | 2.71 | 1.58 | 0.98 | 0.69 | 0.58 | 11.72 | 6.13 | 3.45 | 2.07 | 1.86 |
|  | PC1.5 | 3.79 | 2.15 | 1.41 | 1.01 | 0.85 | 16.00 | 9.33 | 5.45 | 3.62 | 2.80 |
|  | PC1 | 4.72 | 2.83 | 1.79 | 1.30 | 1.08 | 20.89 | 12.54 | 7.16 | 4.79 | 3.80 |
| IoT NTN (15kHz) | PC3 | 0.16 | 0.08 | 0.03 | 0.01 | 0.00 | 0.71 | 0.10 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.25 | 0.12 | 0.06 | 0.02 | 0.01 | 1.03 | 0.34 | 0.02 | 0.00 | 0.00 |
|  | PC1 | 0.47 | 0.25 | 0.12 | 0.06 | 0.03 | 2.16 | 1.03 | 0.35 | 0.02 | 0.00 |
| IoT NTN (3.75kHz) | PC3 | 0.07 | 0.03 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.12 | 0.04 | 0.01 | 0.00 | 0.00 | 0.07 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC1 | 0.28 | 0.12 | 0.05 | 0.01 | 0.00 | 0.50 | 0.07 | 0.00 | 0.00 | 0.00 |

Even under such extreme assumption, the interference from IoT NTN is ignorable when ACLR is larger than 20 dB. For NR NTN, the required ACLR is about 31dB for PC2 and 34 dB for PC1.

Observation 2: With 0km isolation assumption, the required ACLR for PC1/PC2 IoT NTN is less than 20 dB, and for NR NTN, the required ACLR is 27dB for PC2, 31 dB for PC1.5 and 34 dB for PC1.

However, if we take current PC3 ACLR to observe the system degradation, the NR NTN throughput degradation for cell edge is 5.45% for PC1.5 and 7.16% for PC1, which is not significantly increased compared to 5% throughput loss metric.

Observation 3: With 0km isolation assumption and 30 dB ACLR, the throughput loss does not significantly exceed the 5% threshold for PC1.5 and PC1 NR NTN.

  - Evaluation for LEO-1200

In [1], the LEO-1200 was agreed as optional case to be evaluated. The following results compare the results for LEO-1200 and GEO.

Table III NR NTN GEO/LEO-1200 and TN Uma with AAS antenna (Isolation distance=1.5km)

|  |  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| NR NTN GEO | PC3 | 0.35 | 0.18 | 0.10 | 0.06 | 0.05 | 1.49 | 0.62 | 0.16 | 0.11 | 0.09 |
|  | PC2 | 0.52 | 0.28 | 0.18 | 0.10 | 0.08 | 2.07 | 1.25 | 0.59 | 0.22 | 0.16 |
|  | PC1.5 | 0.76 | 0.42 | 0.24 | 0.16 | 0.13 | 3.62 | 1.76 | 1.04 | 0.57 | 0.46 |
|  | PC1 | 0.97 | 0.62 | 0.32 | 0.22 | 0.18 | 5.21 | 2.92 | 1.41 | 0.94 | 0.62 |
| NR NTN LEO1200 | PC3 | 0.31 | 0.17 | 0.09 | 0.06 | 0.05 | 1.50 | 0.73 | 0.30 | 0.20 | 0.20 |
|  | PC2 | 0.47 | 0.25 | 0.14 | 0.09 | 0.07 | 2.19 | 1.17 | 0.72 | 0.36 | 0.23 |
|  | PC1.5 | 0.69 | 0.37 | 0.22 | 0.14 | 0.12 | 3.23 | 1.78 | 1.11 | 0.72 | 0.55 |
|  | PC1 | 0.82 | 0.49 | 0.29 | 0.19 | 0.16 | 3.75 | 2.31 | 1.41 | 0.93 | 0.73 |

Table IV NR NTN GEO/LEO-1200 and TN Uma with AAS antenna (Isolation distance=0km)

|  |  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| NR NTN GEO | PC3 | 1.91 | 1.10 | 0.67 | 0.47 | 0.39 | 7.89 | 3.86 | 2.02 | 1.60 | 1.37 |
|  | PC2 | 2.71 | 1.58 | 0.98 | 0.69 | 0.58 | 11.72 | 6.13 | 3.45 | 2.07 | 1.86 |
|  | PC1.5 | 3.79 | 2.15 | 1.41 | 1.01 | 0.85 | 16.00 | 9.33 | 5.45 | 3.62 | 2.80 |
|  | PC1 | 4.72 | 2.83 | 1.79 | 1.30 | 1.08 | 20.89 | 12.54 | 7.16 | 4.79 | 3.80 |
| NR NTN LEO1200 | PC3 | 1.68 | 0.95 | 0.57 | 0.39 | 0.32 | 6.59 | 3.51 | 1.89 | 1.39 | 1.29 |
|  | PC2 | 2.40 | 1.38 | 0.84 | 0.58 | 0.48 | 10.14 | 5.29 | 3.04 | 1.91 | 1.63 |
|  | PC1.5 | 3.40 | 1.99 | 1.23 | 0.86 | 0.72 | 14.50 | 8.09 | 4.88 | 3.14 | 2.43 |
|  | PC1 | 4.26 | 2.51 | 1.57 | 1.11 | 0.93 | 18.49 | 10.52 | 6.08 | 4.17 | 3.38 |

The impact of LEO is lower than GEO since under same SNR target, the NTN UE output power can be lower for LEO.

Observation 4: For NR NTN HPUE, the impact from LEO is lower than GEO under same isolation assumption.

Table V IoT NTN GEO/LEO-1200 and TN Uma with AAS antenna (Isolation distance=1.5km)

|  |  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| IoT NTN GEO (3.75kHz) | PC3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC1 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| IoT NTN LEO1200 (3.75kHz) | PC3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.02 | 0.02 | 0.03 | 0.04 | 0.02 |
|  | PC2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.06 | 0.02 | 0.06 | 0.02 |
|  | PC1 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.06 | 0.02 | 0.08 |
| IoT NTN GEO (15kHz) | PC3 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | PC2 | 0.03 | 0.12 | 0.00 | 0.00 | 0.00 | 0.03 | 0.01 | 0.0 | 0.0 | 0.0 |
|  | PC1 | 0.07 | 0.23 | 0.01 | 0.00 | 0.00 | 0.11 | 0.03 | 0.0 | 0.0 | 0.0 |
| IoT NTN LEO1200 (15kHz) | PC3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC1 | 0.03 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table VI IoT NTN GEO/LEO-1200 and TN Uma with AAS antenna (Isolation distance=0km)

|  |  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| IoT NTN GEO (3.75kHz) | PC3 | 0.07 | 0.03 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.12 | 0.04 | 0.01 | 0.00 | 0.00 | 0.07 | 0.00 | 0.00 | 0.00 | 0.00 |
|  | PC1 | 0.28 | 0.12 | 0.05 | 0.01 | 0.00 | 0.50 | 0.07 | 0.00 | 0.00 | 0.00 |
| IoT NTN LEO1200 (3.75kHz) | PC3 | 0.16 | 0.07 | 0.03 | 0.01 | 0.00 | 0.21 | 0.10 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.25 | 0.12 | 0.05 | 0.02 | 0.00 | 1.05 | 0.15 | 0.10 | 0.00 | 0.00 |
|  | PC1 | 0.48 | 0.25 | 0.12 | 0.05 | 0.02 | 1.50 | 1.05 | 0.15 | 0.10 | 0.00 |
| IoT NTN GEO (15kHz) | PC3 | 0.16 | 0.08 | 0.03 | 0.01 | 0.00 | 0.71 | 0.10 | 0.00 | 0.00 | 0.00 |
|  | PC2 | 0.25 | 0.12 | 0.06 | 0.02 | 0.01 | 1.03 | 0.34 | 0.02 | 0.00 | 0.00 |
|  | PC1 | 0.47 | 0.25 | 0.12 | 0.06 | 0.03 | 2.16 | 1.03 | 0.35 | 0.02 | 0.00 |
| IoT NTN LEO1200 (15kHz) | PC3 | 0.24 | 0.13 | 0.07 | 0.03 | 0.01 | 1.08 | 0.63 | 0.50 | 0.17 | 0.04 |
|  | PC2 | 0.34 | 0.19 | 0.10 | 0.05 | 0.03 | 1.64 | 0.96 | 0.53 | 0.41 | 0.13 |
|  | PC1 | 0.59 | 0.34 | 0.19 | 0.11 | 0.06 | 2.95 | 1.64 | 0.97 | 0.54 | 0.45 |

The results show that the ACLR for IoT NTN is less than 20 dB no matter for GEO and LEO.

Observation 5: For IoT NTN HPUE, the required ACLR is less than 20 dB no matter for GEO or LEO.

  - SNR Target

Traditionally, 15 dB target SNR is used for TN co-existence study since LTE, but for NTN, due to the lower data rate, it is not necessary to force NTN UE to achieve such high SNR and 3 dB was agreed in the last meeting as an optional assumption. With lower SNR target, the UE power can be reduced and the interference will be also lower.

Observation 6: With lower SNR target, the interference from NTN HPUE can be further reduced due to the lower power of UE.

Based on the analysis in previous section, we believe that reuse PC3 ACLR is safe enough to guarantee the co-existence between NTN HPUE and TN.

Proposal: The PC3 ACLR can be reused for HPUE for both NR NTN and IoT NTN.

- Conclusion

In this contribution, we provide more evaluation for NTN HPUE co-existence.

Observation 1: When the wrap-around is turned on, the UEs at TN edge may access to the image cluster which make the isolation distance is smaller than the assumption, leading to higher throughput loss at cell edge.

Observation 2: With 0km isolation assumption, the required ACLR for PC1/PC2 IoT NTN is less than 20 dB, and for NR NTN, the required ACLR is 27dB for PC2, 31 dB for PC1.5 and 34 dB for PC1.

Observation 3: With 0km isolation assumption and 30 dB ACLR, the throughput loss does not significantly exceed the 5% threshold for PC1.5 and PC1 NR NTN.

Observation 4: For NR NTN HPUE, the impact from LEO is lower than GEO under same isolation assumption.

Observation 5: For IoT NTN HPUE, the required ACLR is less than 20 dB no matter for GEO or LEO.

Observation 6: With lower SNR target, the interference from NTN HPUE can be further reduced due to the lower power of UE.

Proposal: The PC3 ACLR is reused for HPUE for both NR NTN and IoT NTN.

- References

- R4-2417122 	WF on co-existence study and HPUE for NR-NTN	Samsung

- https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2101-0-201702-I!!PDF-E.pdf