3GPP TSG-RAN WG4 Meeting # 103 	R4-2418182

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Further evaluation on the D2T2 co-existence

Agenda Item:	7.21.2.2

Document for:	Approval

- Introduction

In the last meeting, the simulation results are collected from companies. The performance degradation observed in several cases and the outage probability need to be decided to show the real impact for the system. In this contribution, we provide more analysis on these cases.

- Discussion

  - SINR threshold and outage probability

- Consider 3 SINR values for D2R and R2D respectively and derive the outage percentage value/range.

- The 3 values can be selected according to following steps:

- Select one set SNR/CNR values @ 10% BLER from companies based on RAN1 LLS (refer to latest RAN1 LLS summary)

- Set the SNR/CNR range by removing the highest and lowest value from the set in step 1

- Pick 3 values from the range in step 2, e.g. highest, lowest, middle

Note: R2D CNR needs to be transformed to SINR

In [1], the principle to decide the SINR threshold was agreed:

The final SINR thresholds based on the RAN1 latest simulation results are provided as:

Table 1 SINR threshold @ 10% BLER for AIoT co-existence

|  | D1T1 D2R | D2T2 D2R | R2D SNR for device 1 (coherent) | R2D SNR for device 2a (coherent) |
| --- | --- | --- | --- | --- |
| Min/dB | -7.8 | -7.6 | -14.8 | -14.1 |
| Max/dB | 0.23 | -5 | 9.3 | 5.1 |
| Average/dB | -4.3 | -6 | -6.5 | -7.3 |

In the following part, we summarize the outage probability when AIoT system as victim for D2T2 based on the threshold above. One general issue here is that we have two assumptions for indoor ratio, 10% and 100%. The intention for 100% UE indoor is to show the impact from NR UE under the worst case, but for the conclusion of the co-existence study, 10% indoor ratio is more reasonable.

Observation 1: The 100% UE indoor is an extremely impractical assumption, which is not suitable as the baseline for co-existence conclusion.

Proposal 1: For the cases where legacy UE is indoor, the co-existence conclusion is based on 10% UE indoor assumption.

- Case 5-f CW in UL spectrum NR UL->Reader

| Device type | UE distribution | Outage probability |  |  |
| --- | --- | --- | --- | --- |
|  |  | Min | Max | Average |
| Device 1 | 100% outdoor | 0% | 0% | 0% |
|  | 10% indoor | 3.3% | 5.2% | 4.4% |
|  | 100% indoor | 35.5% | 40% | 37.3% |
| Device 2a | 100% outdoor | 0% | 0% | 0% |
|  | 10% indoor | 1.2% | 3.5% | 2.3% |
|  | 100% indoor | 10.2% | 14.5% | 13.2% |

- Case 5-h CW in UL spectrum NR UL->Device

| Device type | UE distribution | Outage probability |  |  |
| --- | --- | --- | --- | --- |
|  |  | Min | Max | Average |
| Device 1 | 100% outdoor | 0% | 3.5% | 0% |
|  | 10% indoor | 0% | 9.5% | 0% |
|  | 100% indoor | 0% | 60.5% | 4.4% |
| Device 2a | 100% outdoor | 0% | 3.3% | 0% |
|  | 10% indoor | 0% | 4.7% | 0% |
|  | 100% indoor | 0% | 35.2% | 2.5% |

- Case 6-b CW in DL spectrum NR DL->Reader

| Device type | UE distribution | Outage probability |  |  |
| --- | --- | --- | --- | --- |
|  |  | Min | Max | Average |
| Device 1 | 100% outdoor | 1.2% | 3.0% | 2.5% |
|  | 10% indoor | 1.2% | 3.0% | 2.5% |
|  | 100% indoor | 1.2% | 3.0% | 2.5% |
| Device 2a | 100% outdoor | 1.1% | 2.8% | 2.4% |
|  | 10% indoor | 1.1% | 2.8% | 2.4% |
|  | 100% indoor | 1.1% | 2.8% | 2.4% |

- Case 6-h CW in DL spectrum NR UL->Device

| Device type | UE distribution | Outage probability |  |  |
| --- | --- | --- | --- | --- |
|  |  | Min | Max | Average |
| Device 1 | 100% outdoor | 0% | 3.3% | 0% |
|  | 10% indoor | 0% | 9.6% | 0% |
|  | 100% indoor | 0% | 60.3% | 4.4% |
| Device 2a | 100% outdoor | 0% | 3.3% | 0% |
|  | 10% indoor | 0% | 4.7% | 0% |
|  | 100% indoor | 0% | 35.2% | 2.5% |

The results show that for D2T2, the outage probability is at quite low level, and in the most cases, the outage probability is less than 5%

Observation 2: Even though the SINR degradation is observed for AIoT system when UE indoor for D2T2, the outage probability of AIoT system is not significantly increased.

Proposal 2: The following table can be considered as the D2T2 co-existence study conclusion when AIoT system as victim.

| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | f | NR UL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. The maximum outage percentage is 0% |
| --- | --- | --- | --- | --- | --- |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. The maximum outage percentage is less than 5% |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | f | NR UL->reader | SINR degradation is larger than 1dB. The maximum outage percentage is less than 5% |
|  |  |  | h | NR UL->device | SINR degradation is larger than 1dB. The maximum outage percentage is less than 10% |
| 6-1(O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | b | NR DL->reader | SINR degradation is less than 1dB. Inter-system interference is negligible. The maximum outage percentage is 0% |
|  |  |  | h | NR UL->device | SINR degradation is less than 1dB. Inter-system interference is negligible. The maximum outage percentage is less than 5% |
| 6-2(O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | b | NR DL->reader | SINR degradation is larger than 1dB. The maximum outage percentage is less than 5% |
|  |  |  | h | NR UL->device | SINR degradation is larger than 1dB. The maximum outage percentage is less than 10% |

  - Co-existence issue in D2T2

For D2T2, the performance degradation is observed when AIoT reader interfere NR UL, as listed below:

| Deployment scenario No. (Case No.) | Topology | Spectrum | Aggressor | Victim |
| --- | --- | --- | --- | --- |
| 4-1-g | D2T2-A2- NR UE only outdoor | R2D: UL CW2D and D2R: DL | Reader | NR UL |
| 4-2-g | D2T2-A2- NR UE indoor | R2D: UL CW2D and D2R: DL |  |  |
| 5-1-g | D2T2-B- NR UE only outdoor | R2D: UL CW2D and D2R: DL |  |  |
| 5-2-g | D2T2-B- NR UE indoor | R2D: UL CW2D and D2R: DL |  |  |

There are mainly two reason behind:

- The readers always transmit with maximum power while the NR UE needs to follow the power control, and the interference increased when the factory is closer to NR base station.

- The legacy NR UE IBE is used for the emission assumption of D2T2 reader, which is a quite pessimistic assumption.

Considering intermediate UE is a new UE type, it is possible to define a better emission requirement to avoid co-existence issue. According to [1], a classic 2-step ACLR model is used for asymmetric bandwidth which can be also applied to intermediate UE, as shown in figure 1:

Figure 1 2-step ACLR model for intermediate UE

Figure 2 Simulation results based on 2-step ACLR model

The simulation results are shown in Figure 2. Based on the 2-step model, the throughput loss can be less than 5% when the ACLR is around 30~35 dB, so there is no co-existence issue if better emission requirement is defined for intermediated UE.

Observation 3: The performance degradation when reader interfere legacy NR can be solved by defining tighten ACLR Tx requirement for intermediate UE.

Figure 3 Measurement result of commercial NR UE when only 1RB is allocated

To validate the feasibility from UE design perspective, the measurement result based on a commercial NR UE is provided. It shows that the current real UE still can behave better than the required ACLR above even though it is not specifically designed for AIoT.

The performance degradation is also observed for NR UL interfere AIoT when NR UEs are inside factory, as listed below:

| Deployment scenario No. (Case No.) | Topology | Spectrum | Aggressor | Victim |
| --- | --- | --- | --- | --- |
| 4-2-f | D2T2-A2- NR UE indoor | R2D: UL CW2D and D2R: UL | NR UL | reader |
| 4-2-h |  | R2D: UL CW2D and D2R: UL | NR UL | device |
| 5-2-h | D2T2-B- NR UE indoor | R2D: UL CW2D and D2R: DL | NR UL | device |

In the simulation, the device sensitivity is assumed as -36 dBm for device1 and -45 dBm for device 2a, and the corresponding maximum distance between reader and device is 12.6m and 21.7m based on the InH NLOS pathloss model at 900MHz. Compared to D1T1, the intermediate UE may not be fixed and can move around the factory while interacting with device, so the distance between intermediate UE and device in the field can be much closer than the assumption and the ability to resist interference from other systems will be better. The Figure 4 shows the SINR distribution when lower down the sensitivity assumption.

Figure 4 SINR distribution under different sensitivity assumption

It is noted that lower down the sensitivity here does not mean that capability of device is reduced, but it is assumed that the coverage of reader become smaller. The results show that when the intermediate UE close to AIoT device, the co-existence issue can be solved.

Observation 4: The performance degradation when legacy NR UE interfere AIoT can be solved by sacrificing the coverage of reader since the intermediate UE can move around the factory.

Proposal 3: Capture the analysis above for mitigating the performance degradation in D2T2 in TR.

A TP also provided in [2]

- Conclusion

In this contribution, we provide our views on D2T2 co-existence study.

Observation 1: The 100% UE indoor is an extremely impractical assumption, which is not suitable as the baseline for co-existence conclusion.

Proposal 1: For the cases where legacy UE is indoor, the co-existence conclusion is based on 10% UE indoor assumption.

Observation 2: Even though the SINR degradation is observed for AIoT system when UE indoor for D2T2, the outage probability of AIoT system is not significantly increased.

Proposal 2: The following table can be considered as the D2T2 co-existence study conclusion when AIoT system as victim.

Observation 3: The performance degradation when reader interfere legacy NR can be solved by defining tighten ACLR Tx requirement for intermediate UE.

Observation 4: The performance degradation when legacy NR UE interfere AIoT can be solved by sacrificing the coverage of reader since the intermediate UE can move around the factory.

Proposal 3: Capture the analysis above for mitigating the performance degradation in D2T2 in TR.

- References

- R4-2417196,	WF on co-existence study for A-IOT, CMCC, RAN4#112bis

- R4-241xxxx,	TP on D2T2 co-existence evaluation and analysis, vivo, RAN4#113