3GPP TSG-RAN WG4 Meeting # 103 	R4-2418184

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT device

Agenda Item:	7.21.3.2

Document for:	Approval

- Introduction

In the previous meeting, the RF requirements for device were discussed, but there are still several requirements are marked as TBD. In this contribution, we provide our views on these pending requirements.

- Discussion

  - Tx

The following table was agreed in the last meeting [1]:

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss | NO for maximum output power YES for backscattering power or power backscattering loss | YES |
| Output power dynamic | Transmit OFF power | No | NO | YES, as RAN4#112 agreed |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
|  | Minimum output power | N/A | N/A | YES |
|  | Power control | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD | TBD | TBD |
|  | In band emissions | NO, as RAN4#112 agreed | TBD | TBD. |
|  | Carrier leakage | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | TBD | TBD | TBD |
|  | ACLR | TBD | TBD | TBD |
|  | Spurious emissions | TBD | TBD | TBD |
| Unwanted emissions |  | TBD | TBD | TBD |
| Transmit intermodulation |  | NO | NO | TBD |

- Modulation quality requirements

Currently, both OOK and BPSK are possible waveform for AIoT, and the modulation quality still need to be guaranteed, to ensure the performance of reception. However, for OOK, it may need a new requirement and the legacy EVM for QAM signal is not suitable.

- In band emissions

In NR, the in-band emission intends to lower down the interference between UE. In RAN1, FDMA operation is still under discussion, so we can discuss this requirement in WI phase.

- Frequency error /Carrier leakage

Whether these requirements are needed or not depends on the device architecture. If a device support converting its transmit signal frequency to other spectrum, e.g., from DL to UL, these requirements are needed since the LO exist in the Tx chain. For device 2b, this should be the baseline but for device 2a, may be not all device support such operation but we can further discuss the details in WI phase.

- SEM/Spurious emissions/Unwanted emissions

In our understanding, all these requirements are related to the regulation, it is better to keep them at this stage.

- ACLR

Based on current co-existence simulation, the interfere from device 1/2a to NR is ignorable no matter for UE outdoor or indoor. To relieve the cost of device, we prefer not define the ACLR for device 1/2a and only define it for device 2b.

- Transmit intermodulation

For the device 2b, the requirement needs to be defined should be similar to NB-IoT and transmit intermodulation is needed.

Proposal 1: For device Tx requirements, take the following table as baseline:

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss | NO for maximum output power YES for backscattering power or power backscattering loss | YES |
| Output power dynamic | Transmit OFF power | No | NO | YES |
|  | Transmit ON/OFF time mask | NO | NO | NO |
|  | Minimum output power | NO | NO | YES |
|  | Power control | NO | NO | YES |
| Transmit signal quality | Frequency error | NO | Discussed in WI phase | YES |
|  | Modulation quality requirements | YES | YES | YES |
|  | In band emissions | NO | Discussed in WI phase | YES |
|  | Carrier leakage | NO | Discussed in WI phase | YES |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | YES | YES | YES |
|  | ACLR | YES | YES | YES |
|  | Spurious emissions | YES | YES | YES |
| Unwanted emissions |  | YES | YES | YES |
| Transmit intermodulation |  | NO | NO | YES |

  - Rx

The following table was agreed in the last meeting [1]:

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Maximum input power | TBD | TBD | TBD |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | NO |
| In-band blocking | NO | NO | TBD |
| Out-of-band blocking | NO | NO | TBD |
| Receiver intermodulation | NO | NO | TBD |
| Rx spurious emission | NO | NO | TBD |
| Spurious response | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
| Interference rejection | TBD | TBD | TBD |

- Maximum input power

In NR, the maximum input power is to guarantee the UE can still work normally when it is close to BS. Similarly, this requirement still needed for AIoT device to ensure the device have reasonable dynamic range.

- In-band blocking/Out-of-band blocking/Receiver intermodulation/Rx spurious emission

For NB IoT, all these requirements are defined except Rx spurious emission, we can follow the same requirement list.

- Interference rejection

This requirement is from RFID, and it is not clear the intention behind this requirement. In co-existence study, we assume the device has no selectivity and the ACS is not defined also, so the device is not capable to do the interference rejection, but we can keep this requirement to WI phase for more discussion.

Proposal 2: For device Rx requirements, take the following table as baseline:

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES | YES | YES |
| Maximum input power | YES | YES | YES |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | YES |
| In-band blocking | NO | NO | YES |
| Out-of-band blocking | NO | NO | YES |
| Receiver intermodulation | NO | NO | YES |
| Rx spurious emission | NO | NO | NO |
| Spurious response | NO | NO | NO |
| Interference rejection | Discussed in WI phase | Discussed in WI phase | Discussed in WI phase |

- Conclusion

In this contribution, we provide our views on TBD requirements.

Proposal 1: For device Tx requirements, take the following table as baseline:

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss | NO for maximum output power YES for backscattering power or power backscattering loss | YES |
| Output power dynamic | Transmit OFF power | No | NO | YES |
|  | Transmit ON/OFF time mask | NO | NO | NO |
|  | Minimum output power | NO | NO | YES |
|  | Power control | NO | NO | YES |
| Transmit signal quality | Frequency error | NO | Discussed in WI phase | YES |
|  | Modulation quality requirements | YES | YES | YES |
|  | In band emissions | NO | Discussed in WI phase | YES |
|  | Carrier leakage | NO | Discussed in WI phase | YES |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | YES | YES | YES |
|  | ACLR | YES | YES | YES |
|  | Spurious emissions | YES | YES | YES |
| Unwanted emissions |  | YES | YES | YES |
| Transmit intermodulation |  | NO | NO | YES |

Proposal 2: For device Rx requirements, take the following table as baseline:

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES | YES | YES |
| Maximum input power | YES | YES | YES |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | YES |
| In-band blocking | NO | NO | YES |
| Out-of-band blocking | NO | NO | YES |
| Receiver intermodulation | NO | NO | YES |
| Rx spurious emission | NO | NO | NO |
| Spurious response | NO | NO | NO |
| Interference rejection | Discussed in WI phase | Discussed in WI phase | Discussed in WI phase |

- References

- R4-2417088	WF on impacts of A-IoT on RF requirements	Huawei