3GPP TSG-RAN WG4 Meeting # 113	R4-2418197

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	MSD rules and duty cycle related Issues for CA HPUE

Agenda Item:	7.1.1.1.1

Document for:	Approval

- Introduction

The discussion of how to extend the MSD requirements to High power combinations has been existed in multiple agendas even before Rel-19 WI. In Rel-19, this was mainly discussed in Rel-19 UE RF and spec improvement. The duty cycle solution related contributions have been presented for a few meetings but not treated before RAN4#112bis.

In RAN4#112bis, a dedicated agenda was provided for the MSD requirements and these two topics is set up. Some major break through were made on these two topics and a WF was approved in [1].

In this contribution, further discussion were made considering these two topics.

- Discussion

  - MSD Issues

Background

In the WF [1], the following WF has been reached:

Option 1:

  - ….

  - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables:

  - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

- Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

- [Agree to capture just single worst case MSD in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 2:

  - Current working procedure of HPUE CA/DC introduction are kept unchanged

  - HPUE MSD tables are kept

  - Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed

- …

NOTE: Freedom for any company to provide PC3 MSD evaluations using companies’ calculation methods and other assumptions fully remains because PC3 is the baseline to derive PC2/PC1.5 MSD. It is not precluded to provide evaluations for PC2/PC1.5.

Option 3:

  - PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission

  - Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS

  - …

NOTE 1: …

NOTE 2: …

…

Agreement:

- To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3

  - Some examples are provided in the Annex of this WF

- Other options such as Option2 are not precluded

It is noted that some details and duplicate part were omitted for simplicity.

Traditionally the requirements are generally normative in the core requirements spec, in that all the requirements should be tested unless otherwise stated. In some cases, some requirements were defined without need testing. E.g. certain requirements are surely to be met according to the system design, but still agreed to be defined for some reason.

Observation 1: There are already existing cases that defined requirements are not tested.

We felt that the current situation of HPUE requirements can readily satisfy the general principles of informative-only requirements. During the discussion, in more and more cases, the HPUE requirements are defined based on certain derivation rules, which is already based on current implementation of transceiver and front end design. In these cases, the requirements themselves are actually “derived ” value based on default power class requirements and existing implementation. In fact, this is an important reason why the topic of MSD recently.

Observation 2: The HPUE MSD requirements are no longer independent.

Proposal 1a: Normative MSD requirements are not needed for higher power class .

However, totally remove HPUE MSD requirements seems not acceptable frome some interested parties. One of the reasons that provided is that the requirements dedicated for HPUE is still needed at least provide a guidance for the network how much degradation is expected, though this guidance maybe seriously degraded by some other factors that previously discussed, e.g. the restriction of conductive test and wide divergence of implementation and real performance.

Still, these purposes are not need to do real testing, since they are basically for information only and do not have much function for verification.

Proposal 1b: Informative requirements for HPUE MSD can be considered, as long as not tested.

In the agreed WF, some examples are already provided in the annex, what kind of LUP and equations could be. However, one problem is that there are many cases (e.g. PC2/PC1.5, Harmonics/IMD/, 1Tx/2Tx…). We also have other examples for IMD in [3] and also incorporated in the Annex.

Currently there is no framework on the detailed scenarios for the HPUE MSD. Without a framework, it could be difficult to arrange the work and keep the progress could be challenging.

Proposal 2a: Setting up a framework for the various scenarios for the HPUE MSD.

Furthermore, with the reference of a framework, it might be also helpful to do some coordination with Rel-19 basket WIs which have some crossovers with the current topic. It may be even possible to do some cross Work item coordination on the work split etc. This may save a lot of efforts to avoid duplicate work based on legacy procedure.

Proposal 2b: Considering some work split with Rel-19 basket WIs.

  - Duty cycle solution

The duty cycle solution was discussed and the WF in [1] is as following:

Agreement:

- For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed.

- For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

Majority companies prefer to further define duty cycle based solutions, though some restriction such as no new IE introduction was also confirmed. Based on this spirit, we proposed to discuss based on the scenarios needed. The details are re-submitted from [2].

Scenarios

PC1.5 of intra-band CA, PC1.5 of inter-band CA, PC1.5 of inter band EN-DC, PC2 of Inter-band EN-DC are proposed to compatible with the legacy duty cycle solution.

PC1.5 of intra-band CA

Considering the duty cycle solution for PC2 of intra-band CA is mandatory and the duty cycle restriction is default, when maxUplinkDutyCycle-PC2-FR1 is absent, the default threshold is 50%. If the duty cycle exceeds the threshold, the power reduction is expected.

Solution for PC1.5 of intra-band CA is extend for the similar way and compatible with PC2, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 3: For PC1.5 of intra-band CA, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of inter-band CA

The duty cycle solution for PC2 of inter-band CA is optional and also specified for 3Tx. When maxUplinkDutyCycle-interBandCA-PC2 is absent, there is no default threshold. The duty cycle solution is only applied when maxUplinkDutyCycle-interBandCA-PC2 is reported. If the capacity maxUplinkDutyCycle-interBandCA-PC2 is reported the duty cycle exceeds it, the power reduction is expected.

The solution for PC1.5 of inter-band CA is proposed to follow the similar way and compatible with PC2. There is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 4a: To compatible with PC2 of inter-band CA, for PC1.5 of inter-band CA, there is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of Inter-band EN-DC(TDD+TDD)

The duty cycle solution for PC2 of Inter-band EN-DC(TDD+TDD) is mandatory and the duty cycle restriction is default, when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, the default threshold is 30%. If the duty cycle exceeds the threshold, the power reduction is expected.

The solution for PC1.5 of Inter-band EN-DC(TDD+TDD) could be extended for the similar way and compatible with PC2, the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

Proposal 4b: To compatible with PC2 of inter-band EN-DC(TDD+TDD), for PC1.5 of Inter-band EN-DC(TDD+TDD) , the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

PC2 of inter-band EN-DC(FDD+FDD)

As PC2 of inter-band EN-DC(FDD+TDD) PC2 is already specified in 38.101-3, the scenario is FDD on LTE and TDD on NR.  On LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2. These two capabilities could be easily reused for FDD+FDD with editorial changes.

The solution for PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

Proposal 4c: For PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

- Conclusion

In this paper, the following observations and proposals are provided:

MSD rules

Observation 1: There are already existing cases that defined requirements are not tested.

Observation 2: The HPUE MSD requirements are no longer independent.

Proposal 1a: Normative MSD requirements are not needed for higher power class .

Proposal 1b: Informative requirements for HPUE MSD can be considered, as long as not tested.

Proposal 2a: Setting up a framework for the various scenarios for the HPUE MSD.

Proposal 2b: Considering some work split with Rel-19 basket WIs.

Duty Cycle solution (Re-submission):

PC1.5 of intra-band CA:

Proposal 3: For PC1.5 of intra-band CA, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of inter-band CA:

Proposal 4a: To compatible with PC2 of inter-band CA, for PC1.5 of inter-band CA, there is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of Inter-band EN-DC(TDD+TDD):

Proposal 4b: To compatible with PC2 of inter-band EN-DC(TDD+TDD), for PC1.5 of Inter-band EN-DC(TDD+TDD) , the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

PC2 of inter-band EN-DC(FDD+FDD):

Proposal 4c: For PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

- References

- R4-2417098, WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis

- R4-2415784, MSD rules and duty cycle related Issues for CA HPUE, vivo, RAN4#112bis

- R4-2408131, Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx, vivo, RAN4#111

- Annex Conclusions in [3] “MSD framework for IMD”

Observation 1: MSD of PC1.5 2TX = MSD of PC2 2TX + x +Δ. When MSD>=10, Δ is approximately equal to 0 and can be ignored.

Observation 2: For 26+26 PC1.5 2TX architecture, x=3*n, where n is the order of IMDn, n<=5.

Proposal 1: If MSD of 2TX PC2 is higher than 10dB, MSD of 2TX PC1.5 (26+26 architecture) = MSD of 2TX PC2 + 3n, where n is the order of IMDn, n<=5.

Observation3: For 23+29 PC 1.5 3TX architecture, x = 6*|y| where y is the coefficient of the 29dBm band.

Proposal 2: For the 23+29 3TX architecture of PC1.5, the MSD of IMD2 for 2TX PC1.5 could be reused.

Proposal 3: If the MSD of 2TX PC2 is higher than 10dB, for the intermodulation product of the 23+29 3TX architecture higher than the second order, MSD = 2TX PC2 MSD + 6|y| where y is the coefficient of the 29dBm band.

Observation 4: If the IIPn of PA increases linearly with the gain, the x for PC1.5 2TX (26+26) = 3, and the x for PC1.2 3TX (23+29) = 6*|y| -3*(n-1).

Proposal 4: If the IIPn of PA increases linearly with the gain and the MSD of PC2 is higher than 10dB, MSD of PC1.5 2TX(26+26) = MSD of PC2 2TX +3. MSD of PC1.5 2TX(23+29) = MSD of PC2 2TX + 6|y| -3(n-1), where y is the coefficient of the 29dBm band and n is the order of IMDn.