3GPP TSG-RAN WG4 Meeting # 113	R4-2418203

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Iot NTN HPUE Rx Requirements

Agenda Item:	7.8.2.3.2

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. In it, the key agreements:

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

…

Issue 4-3: RSD for IoT-NTN

- Agreement:

  - For NB-Iot based Iot-NTN, no RSD needed for HPUE

  - For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE

  - For eMTC based Iot-NTN with full duplex, further discuss RSD impact

A paper [4] was submitted for including an initial study on NTN HPUE RF Rx Requirements for both NR-based and Iot-based NTN HPUE Tx requirements, in which some topics were not really discussed.

In RAN4#112bis, a paper to discuss Iot NTN HPUE RX requirements were submitted in [8]. An approved WF in [9].

Issue 2-1-2: RSD definition

- …

Agreement:

- Follow NR approach for RSD as much as possible.

Issue 2-1-3: PC2 RSD

- ….

- WF

  - Reuse NR agreements:

- For 256: [0dB] RSD for PC2 for 1Tx and 2Tx (if considered)

  - FFS on the duplexer isolation and PA interference

Issue 2-1-4: PC1 RSD

- Proposals

  - Proposal 1: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (for n254/n255/n256).

- Recommended WF

  - FFS

In this paper, a discussion of the impact of HPUE to RSD requirements for Iot NTN are provided.

HPUE Impact for Iot-NTN RSD

General

As already agreed, for all the half duplex mode cases, no RSD is needed. Only for the case of eMTC based Iot NTN with full duplex, RSD is needed to be discussed.

In fact, the general guideline for RSD is somewhat similar to NR which can be copied here from [10]:

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

For Iot NTN, according to the previous agreement, the scenarios to be considered is as following:

Table 1: Status for requirements consideration and feasibility for eMTC based Iot NTN with full duplex

|  | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- |
| PC1 | 1Tx | [Non-handheld] |
|  | [2Tx] | [Non-handheld] |
| PC2 | 1Tx | Both |

Similar to discussion in For the particular case of eMTC based Iot NTN with full duplex, actual the guidelines should be the same to NR-NTN, since the conditions are basically the same apart from the Channel BW.

Proposal 1: For full duplex eMTC based Iot-NTN, the confirmed scenarios (PC2 1Tx and PC1 1Tx) can be discussed with higher priority, while scenarios under discussion (PC1 2Tx) may be postponed.

PC2

For PC2, an analysis has been provided in [10] for NR-NTN RSD analysis, and two proposals are provided:

Proposal 2a: For NR-NTN power class 2,

- For n256 and n254, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

It is believed that the basic principles to derive requirements are the same, and these two proposals can be re-used except for minor difference. E.g. if in the future non-zero RSD is considered, different channel bandwidth should be considered.

In addition, for the differentiation of handheld and non-handheld, it is also based on the agreement in [9]:

    - Iot NTN can be differentiated by handheld and non-handheld

Based on the previous proposals, and the scenarios that need to be considered in the following RSD value for different bands and accompanying information is as following:

Proposal 2a: For PC2 full duplex eMTC based Iot-NTN,

- For 256 and 254, RSD = 0dB, for [both 2Tx and] 1Tx cases.

- For 255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1

It is noted that for full duplex eMTC based Iot-NTN, the PC1 analysis is also similar to NR-NTN. All the main observations and proposals can be reused from [10].

In order to reduce redundancy, only the key observations and proposals were kept here, and more detailed contents can reference from [10].

Observation 3: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 4: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band 254/255/256 are easier compared to legacy PC1 bands.

Proposal 4a: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB.

Proposal 4b: FFS for PC1 with 2 Tx cases.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

Proposal 1: For full duplex eMTC based Iot-NTN, the confirmed scenarios (PC2 1Tx and PC1 1Tx) can be discussed with higher priority, while scenarios under discussion (PC1 2Tx) may be postponed.

PC2

Proposal 2a: For PC2 full duplex eMTC based Iot-NTN,

- For 256 and 254, RSD = 0dB, for [both 2Tx and] 1Tx cases.

- For 255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1

Observation 3: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 4: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band 254/255/256 are easier compared to legacy PC1 bands.

Proposal 4a: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB.

Proposal 4b: FFS for PC1 with 2 Tx cases.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412100, Initial analysis for NTN HPUE RF Rx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415789, Analysis for NR NTN HPUE Rx Requirements, vivo, RAN4#112bis

[7] TR 37.880, High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71

[8] R4-2415791, Analysis for Iot NTN HPUE Rx Requirements , vivo, RAN4#112bis

[9] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis

[10] R4-2418201, NR NTN HPUE Rx Requirements, vivo, RAN4#113

[11] R4-2418202, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113