3GPP TSG-RAN4 Meeting #113	R4-2418216

# Orlando, US, 18th – 22nd November, 2024

Source:	vivo

Title:	TP for TR 38.774 on LP-WUS RF

Agenda item:			7.23.2.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.774 to capture few general agreements for LP-WUS RF related aspects from WF [1] [2] [3].

2. Reference

[1] R4-2418215, TR 38.774 v0.0.1

[2] R4-2417112, WF on UE RF requirements for LP-WUS, vivo, RAN4#112bis

[3] R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

[4] R4-2410569, WF for 111136 NR_LPWUS_UERF, vivo, RAN4#111

# 3. Text Proposal

<Start of Text Proposal>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.869: "Study on low-power Wake-up Signal and Receiver for NR".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[5]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”.

<End of Text Proposal>

<Start of Text Proposal>

5	System parameters

5.1	General

<Editor’s note: general aspects, details TBD. >

5.2	Operating bands

<Editor’s note: LP-WUS is a generic feature can apply to all bands.  >

The LP-WUS is a generic feature could be applied to all bands specified in Table 5.2-1 of TS 38.101-1 [2] and Table 5.2-1 of TS 38.101-2 [3]. A UE supports LP-WUS feature may not support LP-WUS reception on all the bands supported by the UE MR.

5.3	Channel raster

<Editor’s note: FFS whether new considerations for LP-WUS/SS raster. >

<End of Text Proposal>

<Start of Text Proposal>

# 6	Simulation assumptions and evaluation

## 6.1	Performance metric

<Editor’s note: performance metric to specify RF requirements. >

For RF requirements and conformance testing, the performance metric is 1% MDR. Meanwhile, the FAR assumption for MDR evaluation is </=1%.

## 6.2	Simulation assumption

<Editor’s note: aligned simulation assumptions for evaluation. >

### 6.2.1	General

To evaluate the RF performance of LP-WUS/WUR, the link level simulation is performed in this WI. This clause captures the simulation assumption for LP-WUS UE RF and BS RF analysis.

### 6.2.2	Simulation parameters

The simulation parameters for FR1 LP-WUS RF:

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8/16 bits |  |  |
| Chip rate | M=1 | M=1/2/4 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |
| Phase noise | Can be considered, detailed model FFS |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - Less than 1% Providing the information whether the false alarm rate is considered or not |  |  |

<End of Text Proposal>