3GPP TSG-RAN WG4 Meeting #113	R4-2418217

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	 7.23.2.2

Source:	 vivo

Title:	Further discussions on LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

Based on agreements for REFSENS, ASCS and ACS achieved in previous meetings [1] [2] [3], this contribution shares our further simulation results for LP-WUS REFSENS, ASCS and ACS requirements.

- LP-WUS UE RF

  - Link-level simulation for LP-WUS target SNR

In LTE/NR, SNR = -1 dB is widely used to calculate REFSENS and this value is derived from 95% throughput at link-level simulation under AWGN, 1/3QPSK, no HARQ, 12,5 symbol per subframe as shown in [4][5].

Figure 1 Target LTE SNR simulation with 5, 10 and 20 MHz bandwidths (throughput in Mbps); one receiving antenna [4].

Figure 2 Target NR SNR simulation with 5, and 10 MHz bandwidths [5].

RAN4 has agreed that AWGN channel model with 1% MDR is selected to sepecify RF requirements, and the target SNR is the performance without repetition. Using the agreed simulation parameters, our LLS simulation results for LP-WUS SNR with AWGN model at 1% MDR is presented in Table 1.

Table 1 SNR@1% MDR with 4bit ADC under AWGN (no repetition)

| Modulation | Chip rate | Channel structure | Receiver type | SNR@1% MDR |
| --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 8bit | Envelope | -3.0 dB |
|  |  |  | OFDM | -9.3 dB |
|  |  | 16bit | Envelope | -2.9 dB |
|  |  |  | OFDM | -9.2 dB |
| OOK-4 | M = 1 | 8bit | Envelope | -3.1 dB |
|  |  | 16bit |  | -3.0 dB |
|  | M = 2 | 8bit |  | -4.2 dB |
|  |  | 16bit |  | -4.0 dB |
|  | M = 4 | 8bit |  | -2.5 dB |
|  |  | 16bit |  | -2.3 dB |
| OOK-4 | M = 1 | 8bit | OFDM | -9.3 dB |
|  |  | 16bit |  | -9.1 dB |
|  | M = 2 | 8bit |  | -9.5 dB |
|  |  | 16bit |  | -9.2 dB |
|  | M = 4 | 8bit |  | -6.5 dB |
|  |  | 16bit |  | -6.2 dB |

Observation 1: The worst case is OOK-4, M=4, payload = 16bit with envelope-based receiver, and the SNR at 1% MDR is about -2.3 dB.

Observation 2: With the same modulation, the SNR performance of OFDM-based receiver is always 4~6dB better than Envelope-based receiver.

For the SNR evaluation above we use the bitmap scheme, i.e., CRC is adopted. In the last RAN4 meeting, companies commented that few details of waveform generation is not specified in RAN1, then RAN4 should not move forward on SNR evaluations. In our understanding, RAN1 will not specify all the details, and some of the schemes are left to companies’ implementation choice, RAN4 can not wait for RAN1 to fix each detailed aspect for RAN4 requirements simulation. Meanwhile, we also performed some simulations to compare the SNR performance with CRC and non-CRC (codepoint scheme) cases, the results show that there is only less than 0.5dB difference. Therefore, even though some detailed scheme for waveform generation and detection is different among companies, the OOK-4 M=4 non-CRC results as worst case could be selected as SNR target.

Figure 3 OOK-4 M =4 SNR for codepoint scheme (non-CRC)

Observation 3: The SNR performance with CRC and non-CRC case is less than 0.5dB difference. non-CRC results as worst case could be selected as SNR target.

In last RAN4 meeting, there are two different views on potential target SNR value for LP-WUS/WUR, i.e., single SNR value (similar to MR approach), or two different values for OOK-based and OFDM-based receivers. For NR, the baseband SNR performance at different modulations is different, but single target SNR as -1dB is used to evaluate the REFSENS requirements.

Issue 2-1-3: Target FR1 SNR value for LP-WUS/WUR

Agreements:

  - RAN4 can further discuss target SNR for OOK-based and OFDM-based receivers.

From the above simulation results, we could say that if single target SNR value is used to specify REFSENS, then [-2dB] can be adopted. If two different SNR targets for OOK-based and OFDM-based, then additionally, OFDM-based receiver can use [-5dB].

Proposal 1: If single SNR is defined, then select -2dB as worst case for both receiver types. If two sets SNR should be defined, then -2dB for OOK-based and -5dB for OFDM-based.

  - ASCS requirements

Using the agreed simulation parameters in [1], the ASCS link level simulation results are summarized in Table 2.

Table 2 SNR performance degradation for ASCS (without guard RBs inside)

| modulation | Chip rate | payload | receiver | Freq | Freq offset | Filter order | ASCS SNR @ 1% MDR (with in-band NR) | target SNR@1%MDR (no in-band NR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -2.9 dB | -3.0 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.7 dB | -7.7 dB |
|  |  |  |  |  |  | 5th | -8.0 dB | -8.0 dB |
|  |  |  |  |  | 20 ppm | 3rd | -6.0 dB | -6.0 dB |
|  |  |  |  |  |  | 5th | -6.2 dB | -6.2 dB |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -7.7 dB | -7.7 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.1 dB | -5.1 dB |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -7.5 dB | -7.5 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.0 dB | -5.0 dB |
| OOK-4 | M = 4 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -2.3 dB | -2.3 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -6.1 dB | -6.1 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -5.9 dB | -5.9 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -5.5 dB | -5.5 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -3.8 dB | -3.8 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -5.1 dB | -5.1 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -2.2 dB | -2.2 dB |
|  |  |  |  |  |  | 5th |  |  |

In the Table above, we don’t differentiate the performance under different guard RB (0/1/2) and the reason is that the benefit of guard RB is not significant for ASCS.

Observation 4: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

Issue 2-3-1: ASCS requirements value

Agreements:

  - Confirm ASCS RF requirements is needed for LP-WUS/WUR.

Issue 2-3-2: Required number of guard RB for ASCS

Agreements:

  - RAN4 further discuss required guard RBs for ASCS requirements.

Issue 2-3-3: Whether Test case for ASCS should be defined in RAN4

Agreements:

  - Detailed test case for ASCS should be defined.

It is also observed that the frequency offset has no impact on Envelope receiver but significant impact on OFDM receiver, the reason is that the frequency offset impacts a lot the coherence detection of receiver rather than the interference power level.

Based on the above simulation, to ensure the implementation margin, max 1 RB can be considered as guard RB for ASCS requirements.

Proposal 2: Max 1 RB is required for ASCS requirements.

Proposal 3: RAN4 can define the ASCS test parameters as following in Table 3.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1] blank RB at each side |
| MDR |  | Less than 1% |

  - ACS requirements

Issue 2-4-1: Phase noise for simulation assumption

Agreements:

  - Phase noise can be considered in LLS simulation analysis.

    - Phase noise used in R4-2415201, R4-2415780, and R4-2309204 could be considered as a reference. Other profiles are not precluded.

  - Number of guard RBs will be further discussed until Feb meeting. Encourage companies to provide simulation results with phase noise modelling.

In the last RAN4 meeting, there were few update on simulation assumptions in WF [3]:

2.3.1 ACS High-signal and High interference approach (similar to MR)

For ACS, the simulation is performed under a similar condition of MR, i.e., a high LP-WUS SNR condition of 14dB+LP-WUS REFSENS. Based on the discussions, most companies believe that the interference condition of ACS for LR and MR is nearly the same, so in our simulation we use the same interference level as MR for LP-WUS ACS evaluation.

Our simulation results were provided in the previous RAN4 meeting in [6], and we identified that OOK4 M=4 is also the worst case of ACS. We provide the simulation results for the required RB numbers under different increased interference power level on top of LR REFSENS which vary from 20dB to 33dB. Since the MR and LR are implemented in the same UE, the common understanding is that they would suffer from same interference condition, and from this perspective, it is unnecessary for LR to endure same increased interference power ratio, as shown in the figure below:

Figure 4: Illustration for the gap between MR and LR under same interference

Due to the potential worse REFSENS performance of LR, the required interference power only about 17.7~22.7 dB on top of the LR REFSENS, which also depends on the LR SNR and Noise Figure.

Observation 5: under same interference power level of MR, the required ACS of LR will not exceed 23 dB.

Based on the phase noise model options listed in WF [3], we keep using the phase noise model from IEEE [7]. We model this profile in the simulation and mimic the reciprocal mixing by multiplying the phase noise with the received signal in the time domain, as shown in Figure 5.

Figure 5: phase noise profile and LP-WUS signal before/after reciprocal mixing

After adding the phase noise, the simulation results of ACS are shown in Figure 6. In our simulation results shared last meeting [8], the bandwidth of low pass filter was mistakenly set as doubled value, i.e., the full bandwidth of WUS, so the filtering effect is very poor which does not match with the practical baseband procedure and agreed LPF simulation parameter. In this meeting, we correct it in the simulation platform and share the updated simulation results of ACS in Figure 6.

Figure 6: updated ACS results with consideration of phase noise model (LP-WUS level is fixed at REFSENS+14dB)

The results show that the under 23 dB higher interference power level (the interference level is nearly same as MR condition, as illustrated in Figure 4) 1 guard RB is still enough when the phase noise impact of reciprocal mixing is considered.

Observation 6: After taking the impact of reciprocal mixing of phase noise, 1 guard RB is still enough for WUR under similar MR interference level.

With above analysis and observations, due to potential worse sensitivity of LP-WUR and similar interference level, then a different ACS requirement for this new type of receiver is needed, e.g., ACS=20~23dB.

Observation 7: The ACS requirements for LP-WUS should be defined within 20~23dB (signal level is LP-WUS REFSENS+14dB and similar interference power level as MR), with number of guard RB as 0~1.

Proposal 4: LP-WUS ACS requirements can be specified as: ACS 23dB with Guard RB as 1, or 20dB with guard RB as 0.

2.3.2 ACS Low signal level and low interference level approach

Last RAN4 meeting, there are also simulation results proposed to define the ACS requirement by using sensitivity-level signal but lower down the interference power level [9]. Just for comparison, we also perform the simulation with the same condition, and the following Table 4 show our simulation result. The waveform is OOK-4 M=4.

Table 4 SNR vs gurad RB under different interference power level

| Phase Noise | Interferer (dB) | Guard RB and SNR |  |  |
| --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 4 |
| w/ | 31.5 | N/A | N/A | N/A |
|  | 26.5 | N/A | N/A | 2.04 |
|  | 19.5 | 0.67 | -1.35 | -2.13 |
|  | 13.5 | -2.20 | -2.27 | -2.40 |
|  | 7.5 | -2.41 | -2.41 | -2.44 |
|  | -INF | -2.55 | -2.55 | -2.55 |
| w/o | 31.5 | N/A | N/A | N/A |
|  | 26.5 | N/A | N/A | -2.1 |
|  | 19.5 | -1.45 | -2.20 | -2.51 |
|  | 13.5 | -2.48 | -2.52 | -2.54 |
|  | 7.5 | -2.52 | -2.54 | -2.55 |
|  | -INF | -2.55 | -2.55 | -2.55 |

The simulation shows that when phase noise is added, the receiver can achieve -2 dB SNR with 0 guard RB if the interference power is around 15 dB, but 4 guard RB is needed if the interference is around 20 dB. By defining the ACS with this approach, more guard RB is needed which would force the WUS signal allocated in the center of channel. Besides, due to the low interference power level, the receiver performance may not be guaranteed in many cases in the real network.

  - REFSENS requirements

The following agreements for REFSENS have been agreed last meeting [3]:

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR

Agreements:

  - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed.

  - Encourage companies to also input the justification for each element.

Given the number of RB for LP-WUS is a fixed value, i.e., 11 RBs with 15kHz/30kHz SCS, then single REFSENS for each band at each SCS is needed. In our understanding, rather than discussing the REFSENS value per-band for LP-WUS, it is better to use 5MHz MR REFSENS as a reference to discuss the offset value, it could be a more efficient way to finalize requirements for all the bands.

Observation 8: Given fixed number of RBs for LP-WUS signal, the REFSENS is single value for each band (with each SCS). Instead of discussing the REFSENS value per-band for WUS, it is better to use 5MHz MR REFSENS as a reference to discuss the offset.

Due to the quite different performance of OOK-based and OFDM-based receiver, we would like to discuss them separately.

- For OOK-based receiver, the NF could be around [15]dB, and if SNR is set as -2dB, then we could use 8dB offset compared to MR REFSENS (generally assume MR as 9dB). The IM value could be the same as MR (2.5dB) and there is no diversity gain for LP-WUR.

- For OFDM-based receiver, the NF could be around [11]dB, and if SNR is set as -5dB, then we could use 1dB offset compared to MR REFSENS (generally assume MR as 9dB). The IM value could be the same as MR (2.5dB) and there is no diversity gain for LP-WUR.

Proposal 5: For OOK-based LP-WUS REFSENS, the value for each key element is: SNR=-2dB, IM=2.5dB, diversity =0dB, NF=15dB.

Proposal 6: For OFDM-based LP-WUS REFSENS, the value for each key element is: SNR=-5dB, IM=2.5dB, diversity=0dB, NF=11dB.

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Observation 1: The worst case is OOK-4, M=4, payload = 16bit with envelope-based receiver, and the SNR at 1% MDR is about -2.3 dB.

Observation 2: With the same modulation, the SNR performance of OFDM-based receiver is always 4~6dB better than Envelope-based receiver.

Observation 3: The SNR performance with CRC and non-CRC case is less than 0.5dB difference. non-CRC results as worst case could be selected as SNR target.

Observation 4: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

Observation 5: under same interference power level of MR, the required ACS of LR will not exceed 23 dB.

Observation 6: After taking the impact of reciprocal mixing of phase noise, 1 guard RB is still enough for WUR under similar MR interference level.

Observation 7: The ACS requirements for LP-WUS should be defined within 20~23dB (signal level is LP-WUS REFSENS+14dB and similar interference power level as MR), with number of guard RB as 0~1.

Observation 8: Given fixed number of RBs for LP-WUS signal, the REFSENS is single value for each band (with each SCS). Instead of discussing the REFSENS value per-band for WUS, it is better to use 5MHz MR REFSENS as a reference to discuss the offset.

Proposal 1: If single SNR is defined, then select -2dB as worst case for both receiver types. If two sets SNR should be defined, then -2dB for OOK-based and -5dB for OFDM-based.

Proposal 2: Max 1 RB is required for ASCS requirements.

Proposal 3: RAN4 can define the ASCS test parameters as following in Table 3.

Proposal 4: LP-WUS ACS requirements can be specified as: ACS 23dB with Guard RB as 1, or 20dB with guard RB as 0.

Proposal 5: For OOK-based LP-WUS REFSENS, the value for each key element is: SNR=-2dB, IM=2.5dB, diversity =0dB, NF=15dB.

Proposal 6: For OFDM-based LP-WUS REFSENS, the value for each key element is: SNR=-5dB, IM=2.5dB, diversity=0dB, NF=11dB.

# References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo, RAN4#111

- R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

- R4-2417112, WF on UE RF requirements for LP-WUS, vivo, RAN4#112bis

- R4-080125,  TP Receiver Requirements: REFSENS, ACS and blocking, Ericsson, RAN4#46

- R4-1709564, Preliminary SNR values for NR UE REFSENS, Huawei, Hisilicon, NR ad-hoc #3, 2017

- R4-2412059, Discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112

- IEEE802.11-17/0326r0, WUR Phase Noise Model Follow-Up

- R4-2415780, Further discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112bis

- R4-2415201, Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS, Huawei, HiSilicon, RAN4#112bis