3GPP TSG-RAN WG4 Meeting #113	R4-2418254

Orlando, USA, 18th – 22nd November 2024

Source:	Nokia

Title:	Discussion on NR NTN SAN requirement for Ku band

Agenda item:	7.9.5

Document for:	Approval

Introduction

WI on the introduction of Ku Band for NR NTN has been approved in [1] and one objective is to specify RF requirement for SAN in the following.

- Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

RAN4 discussed SAN RF aspects in RAN4#112 meeting [3][4][5][6][7]. The following WF was agreed for NTN Ku band SAN [8].

Issue 2-1-1: Supported SAN classes for NR NTN Ku band - The supported SAN classes for NR NTN Ku band include GEO, LEO-600, and LEO-1200.

In RAN4#112-bis, the following agreement is made for SAN RF.

Issue 2-1-1: Supported SAN type for NR NTN Ku band - At least the OTA type of requirements will be specified, FFS on whether conducted type of requirements is considered

Furthermore, coexistence scenarios were agreed in RAN4#112 [9] and SAN parameters for the coexistence analysis are agreed in RAN4#112-bis.

Sub-topic 2-1: SAN equivalent antenna aperture - Use 9.1 m DL, 7.14 UL (GEO ), 0.91 m DL, 0.714 UL (LEO DL 600 & 1200) Sub-topic 2-2: SAN antenna pattern - Use section 6.4.1 from 38.811 Sub-topic 2-3: SAN Tx antenna maximum gain - Use 58.5 dBi (GEO), 38.5 dBi (LEO 600 & 1200) Sub-topic 2-4: SAN Rx antenna maximum gain - Use 58.5 dB (GEO), 38.5 dB (LEO 600 & 1200) Sub-topic 2-5: SAN antenna beamwidth - Use 0.177 (GEO), 1.768 (LEO 600 & 1200) Sub-topic 2-7: SAN max EIRP - 43 dBw/MHz (GEO), 13 dBw/MHz (LEO 1200), 7 dBW/MHz (LEO 600) using 100 MHz bandwidth

In this contribution, we discuss issues on SAN requirement.

Discussion

RF requirement for SAN is specified in TS 38.108, where three requirement types 1-H, 1-O and 2-O are currently used. As our preference is to use FR1-NTN numerology for Ku band [11], 1-O and 1-H are our preferred option for Ku band SAN. As

It is proposed to consider 1-O requirement as a baseline of the OTA requirement for Ku band SAN.

The requirement for 1-O type SAN is specified for SCS 15 kHz, 30 kHz and 60 kHz. As the WID [1] mentions the priority 1 option for Ku band FR1-NTN numerology (though not limited to) to support 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. If we agree FR1-NTN with only 30 kHz SCS, other SCS (e.g., 15 kHz and 60 kHz) do not apply to Ku band.

It is for further discussion if only FR1-NTN 30 kHz requirement in TS 38.108 is applicable to Ku band SAN.

The impact of Ku band introduction to SAN specifications was discussed in RAN4#112 [5][6][7]. The main open issue for SAN transmitter and receiver requirement is to derive ACLR and ACS requirement through the coexistence analysis in Ku band. Other than ACLR and ACS, the specifications impact is relatively straightforward by scaling some requirements according to the additional channel bandwidths, since most SAN requirements are already specified in band agnostic way.

Except for ACLR and ACS (and other requirements scaling needed for additional channel bandwidth), the exiting SAN transmitter and receiver specifications in TS 38.108 for FR1-NTN are reused for Ku band.

Conclusion

The following Observations and Proposals were made:

Proposal 1: It is proposed to consider 1-O requirement as a baseline of the OTA requirement for Ku band SAN.

Proposal 2: It is for further discussion if only FR1-NTN 30 kHz requirement in TS 38.108 is applicable to Ku band SAN.

Proposal 3: Except for ACLR and ACS (and other requirements scaling needed for additional channel bandwidth), the exiting SAN transmitter and receiver specifications in TS 38.108 for FR1-NTN are reused for Ku band.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

- R4-2413415 Topic summary for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112

- DRAFT Meeting Report for TSG RAN WG4 meeting: 112

- R4-2411122 Discussion on SAN RF core requirments for Ku-band for NR NTN, CATT, RAN4#112

- R4-2411191 NTN Ku-band – SAN RF requirements overview, Ericsson, RAN4#112

- R4-2411861 Discussion on SAN RF requirements for NR NTN Ku band, ZTE, Sanechips, RAN4#112

- R4-2413525 Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112

- R4-2413519	Way Forward for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2411507 Ku Band Co-existence, Intelsat, RAN4#112

- R4-2418252 Discussion on NR NTN system parameters for Ku band, Nokia, RAN4#113

- R4-2416563 Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112bis

- R4-2416593	Way Forward for [112bis][313] NR_NTN_Ku_Band_General, RAN4#112bis