3GPP TSG-RAN WG4 Meeting #113	R4-2418257

Orlando, USA, 18th – 22nd November 2024

Source:	Nokia

Title:	Channel locations for UE capable of 40 MHz in n28

Agenda item:	6.20.1

Document for:	Approval

Introduction

RAN#104 has agreed the WI to introduce Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 [1]. The objectives of the WI are quoted in the following.

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

Regarding the channel location and channel raster restriction, as well as the release independence, the following issues are for further study as agreed in RAN4#112-bis [7].

Issue 1-2-6 channel location Agreement: - Update the NOTE 7 as follows: - NOTE 7: For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] 703-733MHz or 718-748MHz [for the 20, 25 and 30MHz bandwidth]. - Introduce the new note as follows: - For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] [703-743.04MHz] or 718-748MHz [for the [20, 25, and] 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.] - FFS the content in [] Issue 1-2-7 channel raster Agreement: - Alt1: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz. - Alt2: Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz. Issue 1-2-8 Release independence Agreement: - Option 1: Rel-16 - Option 2: Rel-15

In this contribution we discuss the channel raster limitations for supporting 40 MHz channel bandwidth.

Discussion

BS 40 MHz channel bandwidth has been already introduced as TEI16 feature [2,3]. It is dedicated to the channel raster entry at 723.04 MHz for UL and 778.04 MHz for DL.

This channel raster entry for BS 40 MHz channel bandwidth (signaled in SIB1) supports 30 MHz UE specific channel bandwidth centered at 718 MHz for UL and 763 MHz for DL. This UE channel bandwidth location is aligned with the lower band edge of n28 with the minimum guard band 592.5 kHz. At the same time, 30 MHz UE specific channel bandwidth is PRB-grid aligned with BS 40 MHz channel bandwidth centered at 723.04 MHz for UL and 778.04 MHz for DL. There is a 40 kHz frequency shift for BS from the legacy channel raster of 100 kHz.

For the legacy UEs that can support only 100 kHz raster and up to 30 MHz channel bandwidth, 40 MHz BS channel bandwidth (signaled in SIB1) should be configured at this raster entry (UL 723.04MHz and DL 778.04MHz). This is because the legacy 30 MHz UEs cannot camp on or operate in this cell if 40 MHz is configured at 100 kHz raster and UE channel bandwidth is off the 100 kHz channel raster.

BS 40 MHz channel bandwidth centred at 723.04 MHz for UL and 778.04 MHz for DL has been already introduced in Rel-16 as TEI16 feature to nest UE 30 MHz channel bandwidth centred at 718.0 MHz for UL and 763.0 MHz for DL.

As we discussed in RAN4#112-bis [8], we considered 40 MHz UL channel bandwidth confined in 703-743.04MHz. This means the channel bandwidths centered at 723.0, 723.01, 723.02, 723.03, and 723.04 MHz are covered. Among them, 723.04 MHz is the only option that can nest 30 MHz channel bandwidth at the lowest band edge without violating the minimum guard band requirement.

Regarding the note in Issue 1-2-6, we support the following sentence.

NOTE 7: For UEs supporting 30 MHz max channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-733 MHz or 718-748 MHz. For UEs supporting 40 MHz max bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-743.04 MHz or 718-748 MHz.

NOTE 7 is changed to “For UEs supporting 30 MHz max channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-733 MHz or 718-748 MHz. For UEs supporting 40 MHz max bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-743.04 MHz or 718-748 MHz.”

The above-mentioned channel raster entries, e.g., 723.01, 723.02, 723.03, and 723.04 MHz are covered by Enhanced channel raster in Table 5.4.2.3-5 of TS 38.101-1 in Rel-18. The support of Enhanced channel raster is mandatory from Rel-18 for band n1. The capability signalling is early implementable from Rel-16 [4][9]. For the issue 1-2-7, it is not necessary to specify the specific UE channel raster entry but is sufficient to cover them by Enhanced channel raster. Therefore alternative 2 is proposed.

The capability signalling for Enhanced channel raster is early implementable from Rel-16.

It is not necessary to specify the specific UE channel raster entry in UE specifications but is sufficient to cover by Enhanced channel raster.

As mentioned above, 40 MHz channel bandwidth is specified in Rel-16 BS specifications as a TEI16 feature. The signalling of Enhanced channel raster capability is early implementable from Rel-16. It is not possible to make it release independent from Rel-15. Therefore, we propose that the UE release independence of 40 MHz for n28 is from Rel-16.

The release independence of 40 MHz for n28 is from Rel-16.

Conclusion

The following Observations and Proposals were made:

Observation 1: BS 40 MHz channel bandwidth centred at 723.04 MHz for UL and 778.04 MHz for DL has been already introduced in Rel-16 as TEI16 feature to nest UE 30 MHz channel bandwidth centred at 718.0 MHz for UL and 763.0 MHz for DL.

Proposal 1: NOTE 7 is changed to “For UEs supporting 30 MHz max channel bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-733 MHz or 718-748 MHz. For UEs supporting 40 MHz max bandwidth, the minimum requirements are specified for any NR UL channel bandwidth confined to 703-743.04 MHz or 718-748 MHz.”

Observation 2: The capability signalling for Enhanced channel raster is early implementable from Rel-16.

Proposal 2: It is not necessary to specify the specific UE channel raster entry in UE specifications but is sufficient to cover by Enhanced channel raster.

Proposal 3: The release independence of 40 MHz for n28 is from Rel-16.

- References

- RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN, RAN#104

- RP-223523 CR on R16 TS 38.104 to add channel raster exception for band n28, CMCC, RAN#98e

- RP-223524 CR on R17 TS 38.104 to add channel raster exception for band n28, CMCC, RAN#98e

- R4-2410712 Rel-18 RAN4 UE feature list for NR (version 5), CMCC, RAN4#111

- R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, RAN4#112

- R4-2412817 Topic summary for [112][115] NR_n28_PC2_40MHz, CMCC, RAN4#112

- R4-2417082 WF on PC2 and 40MHz CBW for n28, CMCC, RAN4#112-bis

- R4-2415216 Topic summary for [112bis][112] NR_n28_PC2_40MHz, RAN4#112-bis

- TS 38.331 Annex C: List of CRs Containing Early Implementable Features and Corrections