3GPP TSG-RAN WG4 Meeting #113	R4-2418296

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.18.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RAN4 requirement impacts of AIML for mobility

Document for:	Discussion

- Introduction

In the last meeting, the discussion of AI/ML for mobility was started and agreement was summarized in WF [1]. According to this WF, this paper proposes possible RAN4 requirement impacts.

- Discussion

During previous meeting, RN4 agreed with starting the discussion from RRM measurement prediction of case 2,3, and 4 including all sub-usecases.

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

According to RAN2 assumptions, the prediction is only for obtaining the cell quality. However, RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition.

Observation 1: RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition.

For scenario 2 and 4, since this is intra-frequency prediction, PSS/SSS sync may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results. For Scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases. The feasibility of SSB time index acquisition is need more discussions for all the scenarios.

Proposal 1: RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML.

Proposal 2: For PSS/SSS sync,

- In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results.

- In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases.

Proposal 3: For SSB time index acquisition is need more discussions for all the scenarios.

- Conclusion

In this contribution, we proposed our views on RAN4 requirement impacts of AI/ML for mobility.

Observation 1: RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition.

Proposal 1: RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML.

Proposal 2: For PSS/SSS sync,

- In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results.

- In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases.

Proposal 3: For SSB time index acquisition is need more discussions for all the scenarios.

References

[1] 3GPP, R4-2416855, “WF on RRM requirements for FS_NR_AIML_Mob”, Nokia