3GPP TSG-RAN WG4 Meeting #113	R4-2418298

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.25.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirements for XR

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document proposes RRM core requirements for XR.

- Discussion

During previous meeting, the necessity of consideration for scheduling restrictions on the measurement without gap was discussed.

Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Support skipping of measurements without gaps - Option 2: Measurement skipping does not apply for measurements without gaps Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Measurement skipping does not apply for L1 measurements and procedures Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

In previous meeting, some companies mentioned that the performance gain is small for these scenarios because they only affect few symbols. However, even if one symbol is restricted, throughput degradation is large especially in 100MHz CBW band.

Proposal 1: At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps

Proposal 2: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

Proposal 3: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

- Conclusion

In this contribution, we proposed our views on RRM core requirements for XR.

Proposal 1: At least for 100MHz CBW band, support skipping of measurements for L3 measurements without gaps

Proposal 2: At least for 100MHz CBW band, support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

Proposal 3: At least for 100MHz CBW band, support skipping of L1 measurements and procedures for LTM within and outside gap

References

[1] 3GPP, R4-2416864, “WF on RRM requirements for NR_XR_Ph3”, Nokia