3GPP TSG-RAN WG4 Meeting # 113	R4-2418301

# Orlando, USA, 18th - 22th, Nov 2024

Source: 	Samsung

Title: 	Discussion on Fragmented carriers

Agenda Item:	7.6.2

Document for:	Discussion

- Introduction

In RAN#103, a SID was approved [1] with following objectives on the study of how to process the fragmented carriers with one Rx Chain. RAN4 continues the discussion in RAN4#112bis and reached a lot of agreements on referenced architecture and evaluation assumptions [2].

We further present our consideration in this paper on left over issues.

- 2. Discussion

2.1 UE implementation

Our proposals on the terminology of “Rx RF Chain” and the referenced architecture (fully shared RF Chain architecture) proposed in [4] were agreed in last meeting, excepted as following. As illustrated, 2 NC CCs are supposed to share the Rx RF chain from antenna to the end of ADC.

After ADC, the signals are fed into digital BB which could be either shared digital BB or separated digital BB from implementation perspective, better attenuation capability for in-gap interference is expected with separate digital BB due to separate digital BB filters, the following Fig.1 is drafted for better interpretation. On the other hand, we understand there is no IE to indicate the implementation for digital BB is shared or separated, as well as usually there is only one set of requirements cover all possible implementation as long as the discrepancy is not large, so we are ok to use shared digital BB as starting point for UE performance degradation study purpose.

Proposal 1: Both shared digital BB and separated digital BB are possible from UE implementation perspective, it is fine for us to adopt shared digital BB as starting point for study purpose.

(→ Separate FFT)

(→ Single FFT or separate FFT)

Figure 1: “Fully shared Rx Chain” architecture

→ Separate FFT

One issue has not been touched in last meeting is whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier. Our understanding is that single or separated FFT has no impact on UE RF requirements but we see companies’ interest to share the views on the possible implementation.

It is worth nothing as [5] mentioned “if a single FFT is used to receive two non-contiguous CCs that are SCS aligned, if the CCs are 15KHz SCS that maximum BW is 50MHz and if 30KHz it is 100MHz”. Because maximum BW is related not only RF considerations but also sampling rate which is a function of FFT size and SCS selection.

Therefore, we think both single FFT and separate FFT can be considered for implementation in case digital BB is shared. Single FFT could be used assuming aligned SCS between CCs. Separate FFT can be used for not aligned SCS, or when frequency separation (CC1+gap+CC2) is larger than 50MHz and SCS is 15KHz for both CCs. In case digital BB is not shared, separate FFT should be assumed.

Proposal 2: From implementation perspective, both single FFT and separate FFT could be considered for “shared digital BB” architecture, separate FFT should be assumed for “separate digital BB” architecture.

- Single FFT could be used if SCS between CCs are aligned

- With single FFT, maximum supported frequency separation (CC1+gap+CC2) is 50MHz with SCS 15KHz, and 100MHz with SCS 30KHz.

There is a FFS bullet in WF [3] suggests RAN4 to study the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz (our understanding is this is specific for intra-band CA, rather than for single CC operation). More specific, for example, n7 is mandatory to support 4Rx operation, so n7(2A) shall at least support 4Rx in total, note 4Rx per CC is not requested/mandated by current spec. As a result, if UE is equipped with 4Rx, Rx0 and Rx1 would be for CC1 and Rx2 and Rx3 would be for CC2, as current popular implementation. While, with “fully shared Rx Chain” architecture, Rx0/Rx1 can support both CC1 and CC2 without maximum MIMO layer change, then Rx2 and Rx3 can be freed up and released to other bands/CCs with Lpamid-switch switching, which is beneficial.

Proposal 3: When 2CC NC CA is configured in “fragmented carriers” mode with fully shared Rx Chain, UE is allowed to only support 2Rx in total, in the mandatory 4Rx band, thus the freed-up Rx chains can be used for other bands/carriers, per NW configuration.

Note: It should be clear that for single CC operation, 4Rx shall still be mandated, in the mandatory 4Rx band.

2.2 PSD difference consideration

On NW deployment, as we shared in last two meetings [4], there is no “co-construction and co-site” deployment (meaning sharing infrastructure such as RU/BS antenna) in Canada but only “co-location” (meaning only location is shared but not infrastructure). Other operators also expressed that “co-location” is the most common scenario. Though Canadian regulation for maximum allowed EIPR is in the terms of PSD(XXdBm/MHz) and basically no operator wants to have worse coverage than the other, we still think in the field it is difficult to predict the real-time power/PSD imbalances for any band as the maximum EIRP and the transmitted CBW could be different for two gNBs of two operators in the same location. Thus in last meeting we and other companies proposed to perform the study more from UE perspective which got agreed as well, as following.

On “acceptable” PSD difference, we echo the proposal in [6] that UE fallback behavior is supposed to be taken into account. As [6] pointed out, the determination of the in-gap interferer should consider that the intra-band DL CA performance in fragmented carrier mode not be worse than the performance of single CC operation. Further, we are wondering if we should further say no worse than the Pcell single CC operation.

Proposal 4: The determination of the reasonable/acceptable PSD difference should consider the intra-band DLCA performance in fragmented carrier mode not be worse than the performance of Pcell single CC operation.

2.3 UE fallback behavior

When the significant in-gap interferer precludes the UE to work in fragmented carrier mode, UE shall fallback to either single CC or regular intra-band DL CA implementation, per NW configuration. Vice versa. This is more scheduler perspective and has no impact on RAN4 spec.

Proposal 5：When the significant in-gap interferer precludes the UE to work in fragmented carrier mode, UE shall fallback to either single CC or regular intra-band DL CA implementation, per NW configuration. Vice versa.

- Conclusion

（SID clarification）

Proposal 1: Both shared digital BB and separated digital BB are possible from UE implementation perspective, it is fine for us to adopt shared digital BB as starting point for study purpose.

Proposal 2: From implementation perspective, both single FFT and separate FFT could be considered for “shared digital BB” architecture, separate FFT should be assumed for “separate digital BB” architecture.

- Single FFT could be used if SCS between CCs are aligned

- With single FFT, maximum supported frequency separation (CC1+gap+CC2) is 50MHz with SCS 15KHz, and 100MHz with SCS 30KHz.

Proposal 3: When 2CC NC CA is configured in “fragmented carriers” mode with fully shared Rx Chain, UE is allowed to only support 2Rx in total, in the mandatory 4Rx band, thus the freed-up Rx chains can be used for other bands/carriers, per NW configuration.

Note: It should be clear that for single CC operation, 4Rx shall still be mandated, in the mandatory 4Rx band.

Proposal 4: The determination of the reasonable/acceptable PSD difference should consider the intra-band DLCA performance in fragmented carrier mode not be worse than the performance of Pcell single CC operation.

Proposal 5：When the significant in-gap interferer precludes the UE to work in fragmented carrier mode, UE shall fallback to either single CC or regular intra-band DL CA implementation, per NW configuration. Vice versa.

- 4. Reference

| [1] | RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MTK, RAN#104 |
| --- | --- |
| [2] | R4-2417201, WF on NR FR1 DL fragmented carriers study, MTK, RAN4#112bis |
| [3] | R4-2414311, WF on fragmented DL carriers study, MediaTek Inc., RAN4 #112 |
| [4] | R4-2416287, Discussion on UE architecture and PSD difference for fragmented carriers, Samsung, TELUS, Bell mobility, RAN4#112bis |
| [5] | R4-2413031, On architecture options for fragmented spectrum reception, Skyworks Solutions, Inc., RAN4 #112 |