3GPP TSG-RAN4 Meeting #113	R4-2418318

# Orlando, USA, 18th Nov 2024 – 22th Nov2024

Source:	Samsung, Boost Mobile Network

Title:	TP for TR 38.750 CA_n66-n70 with PC2 FDD single UL

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.750 to include CA_n66-n70 according to the request in [1].

PC3 is already in spec.

2. Reference

[1] RP-2412266 Revised WID: Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	CA_n66-n70

### 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz)(NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n70A | n668 n708 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66B-n70A | n668 n708 | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(2A)-n70A | n668 n708 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(3A)-n70A | n668 n708 | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |

NOTE 8:   Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

### 5.x.2	Reference sensitivity requirements

#### 5.x.2.0	General

Based on calculation, there is no MSD issue for CA_n66-n70.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n66/n70 without TxD

According to clause 5.3.2.0, there is no new MSD needs to be defined.

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n66/n70 with TxD

According to clause 5.3.2.0, there is no new MSD needs to be defined.

### 5.x.3	∆TIB and ∆RIB values (Void)

<End of Text Proposal>