3GPP TSG-RAN WG4 Meeting #113	R4-2418342

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on static MIMO OTA in Rel-19

Source:	Samsung

Agenda item:	7.12.3.2

Document for:	Discussion

1.	Introduction

For static MIMO OTA in Rel-19, the TRMS requirements for several new bands are to be specified. For band n3, it is not determined yet between 4x4 MIMO and 2x2 MIMO [1, R4-2416579]:

Issue 2-1-2: 4x4 requirement or 2x2 requirement for band n3 Agreements: - RAN4 should make the decision on defining which MIMO OTA requirement for band n3 at RAN4#113 (Nov. 2024). - Option 1: 4x4 MIMO OTA requirement - Option 2: 2x2 MIMO OTA requirement - Encourage OEMs to provide the information on the proportion of 4Rx UEs in band n3.

In this contribution we share our view on this open issue, and discuss some other performance part issue as well.

2. 	Discussion

Though the middle band is not mandated to support 4Rx as high band does, in commercial devices 4Rx support in middle band is quite popular, that’s why in Rel-18 n1 MIMO OTA TRMS requirement are specified based on 4x4 MIMO. Band n3 shares the similar situation as n1, so it is reasonable to follow the same approach, i.e., to specify band n3 MIMO OTA TRMS requirement based on 4x4 MIMO.

In order guarantee enough test time for test labs in performance measurement campaign, it would be better to conclude this issue in this meeting, therefore the subsequent 2~3 months could be made use of.

Proposal 1:	RAN4 to conclude this meeting that band n3 MIMO OTA TRMS requirement is to be specified based on 4x4 MIMO.

In last meeting the framework for Rel-19 measurement campaign was agreed. It was later noticed that the power class restriction is not necessary [1]:

- Test cases for Rel-19 FR1 MIMO OTA Measurement Campaign: - Test bands: the bands listed in the WID (n3, n8, n77, n79) is the baseline; FFS whether to further prioritize the bands in the WID, and whether to introduce more bands (keep a reasonable workload with 4 bands) - MIMO layer: - For band n8: 2x2 MIMO (with FR1 UMi CDL-C channel model) - For bands n77, n79: 4x4 MIMO (with FR1 UMa CDL-C channel model) - For band n3: TBD (2x2 or 4x4) - Use scenario: Free space - Operation mode: SA (first stage) - Power class: PC3 (first stage)

According to FR1 common test parameters in Table E1-1 of TS 38.151, the UE uplink transmission power is fixed at 13dBm during MIMO OTA TRMS testing, so the FR1 MIMO OTA requirements are power class agnostic, but more dependent on UE type than power class:

| …… |  |  |
| --- | --- | --- |
| Symbols for all unused REs |  | OCNG Annex A.5 of TS 38.101-4 |
| Minimum Number of Slots per Stream |  | 20000 for 15kHz SCS 40000 for 30kHz SCS (Note 3) |
| Transmit Power Control | dBm | 13 |
| DL power level (RS EPRE of SSS) | dBm / SCS | Set at gNodeB simulator with correction from calibration |
| …… |  |  |

Hence it is suggested to remove the power class restriction to improve the test device availability.

Proposal 2:	Rel-19 MIMO OTA TRMS measurement campaign is targeted on smart phone without power class restriction.

The thresholds for specifying FR1 MIMO OTA performance requirement are relatively loosen compared with that of FR1 TRP TRS. The minimum amount of data per band is 15 to save the test burden, however, it would be beneficial to test as more devices as possible, and the participation of more test labs are better in terms of data diversity.

In order to lower the bar for test labs to provide testing data, maybe it can be considered to relax the minimum number of submitted devices from each lab, e.g., 3  2.

- The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed): - The minimum number of submitted devices from each lab is 3, the maximum number is 15. Meanwhile, labs are encouraged to provide as much data as possible within 15 - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to 3GPP RAN4

Proposal 3:	RAN4 to discuss if the minimum number of submitted devices from each lab can be changed from 3 to 2 in order to lower the bar for test labs to provide testing data, while encouraging test lab to provide as much data as possible within 15.

3. 	Conclusion

Proposal 1:	RAN4 to conclude this meeting that band n3 MIMO OTA TRMS requirement is to be specified based on 4x4 MIMO.

Proposal 2:	Rel-19 MIMO OTA TRMS measurement campaign is targeted on smart phone without power class restriction.

Proposal 3:	RAN4 to discuss if the minimum number of submitted devices from each lab can be changed from 3 to 2 in order to lower the bar for test labs to provide testing data, while encouraging test lab to provide as much data as possible within 15.

4. 	References

- R4-2416579    “WF for [112bis][324] MIMO_OTA_Ph3”, CAICT