# 3GPP TSG-RAN WG4 Meeting #113	R4-2418350

# Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS on carrierBandwidth configuration for less-than-5MHz carriers

Response to:

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:	Li Lu

E-mail Address:	<lu.li@zte.com.cn>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN2 is discussing carrierBandwidth configuration in SIB1 (i.e. in SIB1->servingCellConfigCommon ->downlinkConfigCommon->frequencyInfoDL->scs-SpecificCarrierList->carrierBandwidth) and in RRC dedicated signaling (i.e. in CellGroupConfig-> spCellConfig-> spCellConfigDedicated-> downlinkChannelBW-PerSCS-List-> carrierBandwidth) for the transmission bandwidth configuration of 12 PRB and 20 PRB.

According to TS 38.331, the carrierBandwidth shall be larger or equal to a “UE supported maximum transmission bandwidth”. Otherwise the cell would be considered as barred.

2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which - is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP ….. 2> else: 3> consider the cell as barred in accordance with TS 38.304 [20]; and

Now according to the RAN4 spec, RAN2 understands that the minimum “UE supported maximum transmission bandwidth” would be 15 PRB (for 12 PRB case) or 25 PRB (for 20 PRB case). Therefore, to avoid unnecessary cell barring, from RAN2 perspective, the network needs to set carrierBandwidth to a value equal or larger than 15 PRB for the 12 PRB case and a value equal or larger than 25 PRB for the 20 PRB case.

RAN2 would like to understand from RAN4 perspective whether it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling, and if not what values should be configured.

RAN4 reply: Yes, it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling. It is a common understanding that carrierBandwidth configuration in SIB1 (i.e. in SIB1->servingCellConfigCommon ->downlinkConfigCommon->frequencyInfoDL->scs-SpecificCarrierList->carrierBandwidth) and in RRC dedicated signaling (i.e. in CellGroupConfig-> spCellConfig-> spCellConfigDedicated-> downlinkChannelBW-PerSCS-List-> carrierBandwidth) are corresponding to the transmission bandwidth configuration NRB defined in TS 38.104 clause 5.3.2. Besides, additional GSCNs were designed to distinguish 12 PRB or 20 PRB transmission bandwidth configuration case.

# 2	Actions

To RAN2/RAN1 group:

- ACTION:  RAN4 respectfully requests RAN2/RAN1 to take the provided information into account.

# 3	Dates of Next RAN WG4 Meetings

TSG-RAN2 Meeting #114	Feb 17 - 21, 2025	Athens, GR

TSG-RAN2 Meeting #114bis	Apr 07 - 11, 2024	China, CN