# 3GPP TSG-RAN WG4 Meeting #113	R4-2418364

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on band definition requirements for NR NTN Ku band

# Agenda item:	7.9.1

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from UE RF requirements perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

- RAN4 #112 meeting: - Sub-topic 1-2: Band definitions Agreement: Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S). Additional band definitions are expected once the applicable regulations have been identified and studied.

- RAN4 #112bis meeting: - Sub-topic 1-2: Region 3 band definition Consider the following two bands for the NR NTN Ku band for the studied regulations (i.e. Taiwan, Japan, Australia) in the region 3: - Band nX: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 14 – 14.5 GHz (Earth-to-Space) - Band nY: downlink 10.70 – 12.75 GHz (Space-to-Earth) and uplink 13.75 – 14 GHz (Earth-to-Space) with priority for nX. - Sub-topic 1-4: Inclusion of US in WI scope The US is in scope for the WI using the current allocation of the Ku band. - Sub-topic 1-5: Region 2 band definition Agree the band 10.7-12.7 GHz (S-to-E) and 14.0-14.5 GHz (E-to-S) for North America.

The above are agreements in last two meetings, we reached some agreements on band definitions for region 1 and region 2. For region 3, we tend to consider two bands including Band nX: downlink 10.70–12.75 GHz (Space-to-Earth) and uplink 14–14.5 GHz (Earth-to-Space) and Band nY: downlink 10.70–12.75 GHz (Space-to-Earth) and uplink 13.75–14 GHz (Earth-to-Space) with priority for nX. According to the inputs from companies, in Japan, regulations will be based on ETSI, while Taiwan uses FCC regulation as a reference. Given this difference in regulations across countries within Region 3, we propose that band definitions be aligned with the referred regulations. Hence, it is proposed to define the following two bands based on referred regulations.

Table 2.1: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 2: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Proposal 1: It is proposed to define the following two bands based on referred regulations.

# 3 Conclusion

In this contribution, we shared some views on band definitions for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to define the following two bands based on referred regulations.

Table 2.1: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 2: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.