3GPP TSG-RAN WG4 Meeting #113                 	R4-2418381

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	(LTE_NR_DC_enh2-Core) Discussion on Rel-17 SCG activation (FR2)

Agenda item:	4.5.2

Document for:	Approval

- Introduction

In this paper we discuss the Rel-17 aspects of the Rel-18 SCG activation delay discussed in [23]. Following discussion paper is very much the same as our Rel-18 discussion paper in [23]

Basically, the aspect of UE requirements for SCG activation delay started in Rel-17. While the WI was closed some aspects remained open and unsolved in Rel-17. Rel-17 discussion focused on solution for FR2 while Rel-18 included FR1. Hence, there is clear overlap between the discussions.

However, we want to provide the Rel-17 discussion to ensure that Rel-17 part of the discussion is not left without conclusion.

- Discussion on SCG Activation/deactivation delay

2.1	Introduction

For the discussion related to SCG activation and deactivation 2 main issues have been discussed in RAN4:

- Tsearch in RACH-less based PSCell activation delay.

- Tsearch in RACH-based PSCell activation delay.

Issues related to RACH-less based PSCell activation seems to be having somehow clear requirements in the current version of the specification. Missing some further discussion is the Tsearch part of the PSCell activation delay for RACH-based PSCell activation:

- Tsearch delay for RACH-based PSCell activation for a UE being configured with RLM-and-bfd=true.

2.1.1	Tsearch for RACH-based PSCell activation background

When the UE is configured ‘bdf-and-RLM’ with value ‘true’ for the deactivated PSCell, the UE shall perform BFD (according to section 8.5) and RLM (according to section 8.1) on the deactivated PSCell.

- When ‘bdf-and-RLM’ with value ‘true’ is configured for the deactivated PSCell the UE shall perform BFD and RLM on the deactivated PSCell.

For the UE to perform these measurements and evaluate RLM and BFD, the UE is required to measure the associated RSs according to the requirements defined in sections 8.1 and 8.5. The measurements and evaluation are performed in a regular manner as defined in those sections.

The procedure for performing BFD and RLM on the deactivated PSCell and on the PCell or activated PSCell in DRX is the same. Hence, just like when the PSCell is in activated state, the UE shall measure the deactivated PSCell regularly to evaluate the downlink radio link quality for the cell (RLF) and beam (BFD).

- Evaluation of RLM and BFD on a deactivated PSCell requires the UE to measure the deactivated PSCell regularly to evaluate the downlink radio link quality.

Basic principle is illustrated in the following figure.

While the PSCell is deactivated and UE monitors bfd-and-RLM on the deactivated PSCell, following may occur:

- No BF occur and no RLF occur.

- BF occur but no RLF occur.

- Both BF and RLF occurs.

When the UE is required to perform bfd-and-RLM and has not experienced either beam failure or RLF on the deactivated PSCell, the UE has not experienced link quality problems severe enough to declare radio link or beam failure.

When the UE has not experienced link quality problems severe enough to declare radio link or beam failure, the link is considered good enough for continuing the operation in the PSCell once the PSCell is activated again. Therefore, the same can be expected for the deactivated PSCell when it is being activated:

If the UE is required to perform bfd-and-RLM and has not experienced either beam failure or RLF on the deactivated PSCell prior to activation, there is no need for prolonged activation delay including cell detection, measurements, or beam recovery time upon PSCell activation.

- If no link problems have been detected, the link is currently regarded good enough for continuing the operation in a cell.

- When UE is configured with bfd-and-RLM on the deactivated PSCell, and the UE no link problems (beam failure or RLF) have been detected on the deactivated PSCell, link is regarded good enough for continuing operation.

- When UE is configured with bfd-and-RLM = true and UE has not detected link problems, there is no need for prolonged measurements at PSCell activation.

Figure 1 Illustration of deactivated SCG (FR2) with UE configured with bfd-and-RLM  with value True.

If we consider an illustrative example as in figure 1, where we initially consider the FR2 scenario but the same applies to FR1. If the gNB1 is the deactivated PSCell with bfd-and-RLM  = true configured, the UE is required to measure and evaluate RLM and BFD according to the defined requirements for a deactivated PSCell (defined in Tables 8.1.2.2-5 and 8.5.2.2-5).

If neither RLM nor BFD has been detected while PSCell was deactivated, for example because the UE has not moved, the need for measurements at PSCell activation will be greatly reduced.

If the UE has experienced link quality problems leading to a failure (either beam failure or RLF) on the PSCell while PSCell was in deactivated state, for example due to changed radio conditions, it can be expected that the UE may need some additional search time when PSCell is activated.

In general, a UE which has not detected neither BFD nor RLF on the deactivated PSCell while deactivated, the search time needed at PSCell activation is greatly reduced (Tsearch << 24 Trsms).

A UE which has detected either BFD or RLF on the deactivated PSCell would need some search time at PSCell activation.

These proposals are simply following the same principle which is defined for RACH-less based PSCell activation:

For RACH-less based PSCell activation, if bfd-and-RLM is configured and TCI state is known, Tsearch = 0 ms. There are no requirements if TCI state is unknown.

2.1.2 Tsearch in RACH-based PSCell activation delay

The latest version of 38.133, section 8.17.2 states following concerning RACH-based PSCell activation:

- For RACH based PSCell activation, if the FR2 PSCell is known, Tsearch = 0 ms. If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

Based on the discussion in section 2.2 above, we propose applying the same principle for RACH-based PSCell activation delay as defined for RACH-less based PSCell activation, when UE is configured with bfd-and-RLM=true. Hence, for the UE which is configured with bfd-and-RLM =true for the deactivated PSCell and which has not experienced either beam failure or RLF on the deactivated PSCell while being deactivated, Tsearch can be reduced.

If the UE is configured with bfd-and-RLM on the PSCell and no RLF has been detected, it means that the deactivated PSCell is available, has been measured recently (according to the UE measurement requirements for a deactivated PSCell) and is evaluated having a good enough link quality. Hence, for this scenario, there seems reasonable to discuss reduced Tsearch at PSCell activation.

For RACH based PSCell activation for unknown PSCell, RAN4 to agree on conditions for reduced Tsearch at PSCell activation, when the UE is configured with bfd-and-RLM=true.

We suggest applying same principle as defined for PRACH-less PSCell activation when bfd-and-RLM=true is configured.

For the RACH-based PSCell activation of an unknown PSCell, we suggest that the condition when Tsearch can be reduced from 24*Trs, is when no RLM has occurred on the deactivated PSCell.

The condition for when Tsearch for PSCell activation of an unknown PSCell, can be reduced from 24*Trs, is when no RLM has occurred.

As discussed, because the UE is continuously measuring and evaluating BFD and RLM on the deactivated PSCell being activated, we believe a single sweep should allow UE enough margin during the PSCell activation. Hence, we propose that Tsearch can be reduced to 8 for FR2. Additionally, RAN4 can add additional signal quality conditions applicable already for unknown PSCell.

In this case the UE requirements could, for example, be as follows:

- For RACH based PSCell activation, if the FR2 PSCell is known, Tsearch = 0 ms. If the PSCell is an unknown FR2 PSCell, configured with bfd-and-RLM with value true, provided no RLM has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [8]* Trs ms. Otherwise, if the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

For RACH based PSCell activation, if the FR2 PSCell is known, Tsearch = 0 ms. If the PSCell unknown FR2 PSCell, configured with bfd-and-RLM with value true, provided no RLM has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [8]* Trs ms. Otherwise, if the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

In fact, the proposed TP in the CR is as follows with the new proposed addition in yellow:

For RACH based PSCell activation:

-	if the FR2 PSCell is known:

-	Tsearch = 0 ms.

-	If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB:

-	If UE is configured with bfd-and-RLM with value true, provided no RLM has occurred:

-	Tsearch = [8]* Trs ms.

-	Otherwise

-	Tsearch = 24* Trs ms.

The reasoning and benefit from discussing and defining a more accurate UE delay requirement for PRACH-based PSCell activation has been raised recently. Following we try to bring some further justification why we are proposing clearer UE requirements.

From both UE and network point of view, deactivation of the SCG (PSCell) is a new feature which is supported under capability by the UE. The feature has been introduced to enable improved UE power saving when the SCG/PSCell is not actively in use. For this scenario, the network can deactivate the SCG/PSCell which enables more relaxed UE requirements on the deactivated SCG/PSCell. Hence, the UE have reduced measurement requirements, reduced PDCCH monitoring, reduced CSI measurement to mention some. From network point of view the gain is more limited. Hence, in order to improve overall system performance and ensure the success of the feature, there would need to be some enhancements compared to existing options in the field (here basic PSCell addition and removal).

Currently, one drawback of deactivated SCell/SCG/PSCell is that the network does not have visibility to the PSCell ‘known/unknown’ state unless the network configures the UE with periodic reporting. However, for the deactivated SCG/PSCell and having bfd-and-RLM configured (true) the network can avoid the need for having periodic reporting configured for the PSCell. Difference being that for the deactivated PSCell with bfd-and-RLM=true, the network will be informed and thereby know, if either bfd or RLM has occurred. Hence, network need not configure the UE with periodic reporting and UE need not transmit periodic reports to the network.

2.2 PSCell activation delay and PSCell DRX

Finally, we would like to clarify the UE behavior at PSCell activation regarding monitoring for DL scheduling which is an important aspect from network scheduling point of view (as well as for the UE TP).

RAN4 has introduced PSCell activation delay requirements. One set of requirements are related to RACH-less based delay requirements and another set is for RACH-based requirements. RAN4 has currently defined following:

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH].

However, it is not clear from network perspective when the UE is required to be monitoring DL on the activated PSCell for possible scheduling.

For example, in RACH-less based access, the network would have to know when it is possible to initiate UE scheduling e.g. for allocating the PUSCH. However, as the PSCell have own DRX cycle this becomes ambiguous. The PSCell activation delay may end during a PSCell on-duration or not. If the delay ends during an on-duration the UE behavior is clear. However, if this is not the case the UE behavior is not clear.

- UE PDCCH monitoring requirements in the newly activated PSCell is ambiguous.

One purpose of activating the PSCell is due to immediate need for the PSCell resources. Therefore, it essential for the network to know when UE can be scheduled. It seems reasonable to assume that the UE will start monitoring the PDCCH for scheduling on the activated PSCell immediately after being activated.

UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay.

For the RAN4 requirements this would mean:

TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3].

When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH]. The UE shall apply no DRX immediately after Tactivation_time.

We have earlier suggested sending LS to RAN2 clarifying this with RAN2. We think this is still needed and have provided a draft LS in Appendix.

Send LS to RAN2 clarifying PDCCH monitoring assumption with RAN2.

- Conclusion

In this paper we address the remaining open issues related to SCG activation/deactivation delay in FR2. Based on the discussion in this and former meetings we propose:

Tsearch for RACH-based PSCell activation background:

- When ‘bdf-and-RLM’ with value ‘true’ is configured for the deactivated PSCell the UE shall perform BFD and RLM on the deactivated PSCell.

- Evaluation of RLM and BFD on a deactivated PSCell requires the UE to measure the deactivated PSCell regularly to evaluate the downlink radio link quality.

- If no link problems have been detected, the link is currently regarded good enough for continuing the operation in a cell.

- When UE is configured with bfd-and-RLM on the deactivated PSCell, and the UE no link problems (beam failure or RLF) have been detected on the deactivated PSCell, link is regarded good enough for continuing operation.

- When UE is configured with bfd-and-RLM = true and UE has not detected link problems, there is no need for prolonged measurements at PSCell activation.

- In general, a UE which has not detected neither BFD nor RLF on the deactivated PSCell while deactivated, the search time needed at PSCell activation is greatly reduced (Tsearch << 24 Trsms).

A UE which has detected either BFD or RLF on the deactivated PSCell would need some search time at PSCell activation.

Tsearch in RACH-based PSCell activation delay:

For RACH based PSCell activation for unknown PSCell, RAN4 to agree on conditions for reduced Tsearch at PSCell activation, when the UE is configured with bfd-and-RLM=true.

The condition for when Tsearch for PSCell activation of an unknown PSCell, can be reduced from 24*Trs, is when no RLM has occurred.

For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the PSCell is an unknown FR1 or FR2 PSCell, configured with bfd-and-RLM with value true, provided no RLM has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [8]* Trs ms. Otherwise, if the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms.

PSCell activation delay and PSCell DRX:

- UE PDCCH monitoring requirement in the newly activated PSCell is ambiguous.

UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay.

Send LS to RAN2 clarifying PDCCH monitoring assumption with RAN2.

We have captured the proposed changes in our CR [24] and provided draft LS in Appendix. The CR currently addresses necessary change for Release 17. Note, our Rel-18 discussion paper and CR includes both Rel-17 and Rel-18 aspects.

- References

- RP-221696	Revised WID on Rel-18 even Further RRM enhancement for NR and MRDC, Apple, Oppo, RAN#96

- TS38.133, i40

- R4-2317360	WF on NR eFeRRM (part2), Oppo, RAN4#108bis

- R4-2314337	WF on NR FeRRM (part2), Oppo, RAN4#108

- R4-2314395	WF on LTE_NR_DC_enh2 maintenance - PSCell activation, Nokia, RAN4#108

- R4-2310167 WF on FeRRM part2, Oppo, RAN4#107

- R4-2310080	Big CR for R18 RRM enhancement- FR1+FR1 NR-DC, Oppo, RAN4#107

- R4-2306322 WF on R18 RRM enhancement - FR1+FR1 NR-DC, Oppo, RAN4#106bis-e

- R4-2303304 Way forward on R18 RRM enhancement (part 2), Oppo, RAN4#106

- R4-2220736 WF on FR1-FR1 NR-DC in eFeRRM, Oppo, RAN4#105

- R4-2217250 WF on R18 eFeRRM – RRM requirements for FR1-FR1 NRDC, Oppo, RAN4#104bis-e

- R4-2214345	WF on R18 eFeRRM, Apple, RAN4#104e

- R4-2401733 DraftCR on SCG activation delay for FR1+FR1 NR-DC, Nokia, Nokia Shanghai Bell

- R4-2401729 (LTE_NR_DC_enh2-Core) Discussion concerning SCG Activation and Deactivation Delay, Nokia, Nokia Shanghai Bell

- R4-2403552, (LTE_NR_DC_enh2-Core) CR corrections for SCG Activation and Deactivation Delay, Nokia, Nokia Shanghai Bell

- R4-2401731, (LTE_NR_DC_enh2-Core) CR corrections for SCG Activation and Deactivation Delay, Nokia, Nokia Shanghai Bell

- R4-240xxxx, DraftCR on SCG activation delay for FR1+FR1 NR-DC, Nokia

- R4-2406426, FR1-FR1 SCG activation, Nokia

- R4-2408162, FR1-FR1 SCG activation, Nokia

- R4-24108161, Discussion on FR1-FR1 SCG activation, Nokia

- R4-2411965, CR on FR1-FR1 SCG activation, Nokia

- R4-2411964, Discussion on FR1-FR1 SCG activation, Nokia

- R4-2418384, Discussion on FR1-FR1 SCG activation, Nokia

- R4-2418382, (LTE_NR_DC_enh2-Core) CR on FR2 SCG activation, Nokia

- Appendix

3GPP TSG RAN WG 4 Meeting #113		TDoc R4-24xxxxx

Orlando, USA, Nov 18th – Nov 22nd, 2024

Title:	LS on PSCell activation and PSCell DRX

Response to:

Release:	Rel-18

Work Item:	NR_RRM_enh3-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Lars Dalsgaard

lars.dalsgaard@nokia.com

Attachments:	None

# 1	Overall description

RAN4 is introducing PSCell activation delay requirements. One set of requirements are related to RACH-less based delay requirements and another set is for RACH-based requirements. However, at PSCell activation it is not clear from network perspective when the network can expect the UE monitoring the DL for possible scheduling.

Currently it is unclear whether the UE will follow the DRX monitoring rules immediately after PSCell activation or whether the network can schedule the UE immediately after the PSCell activation.

RAN4 would like to ask RAN2 how the UE will apply the PSCell DRX monitoring rules after PSCell activation?

# 2	Actions

To RAN2

ACTION: 	RAN4 would like to ask RAN2 how the UE will apply the PSCell DRX monitoring rules after PSCell activation?

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #114	17 Feb – 21 Feb, 2025		Athens, Greece

TSG-RAN4 Meeting #114bis	07 Apr – 11 Apr, 2025		China, China