TSG-RAN Working Group 4 (Radio) meeting #113	R4-2418398

Orlando, US, 18th to 22nd November 2024

Source:	Ericsson

Title:	Simulation of BS-to-BS isolation for edge-to-edge co-location scenario

Agenda item:	7.11.3

Document for:	Discussion

# Introduction

At the last RAN4 meeting (RAN4#112-bis in Hefei) improvements of co-location requirement definition and conformance testing were discussed as part of Rel-19 BS RF evolution Work Item (WI) (NR_BS_RF_req_evo). The outcome from the discussion was captured in a Way Forward (WF) in [1]. The WF can be summarized as:

- CLTA definition: Potential outcomes of discussion on co-location reference antenna concept to be considered as alternatives to CLTA, rather than replacing it in Rel-19 OTA test specifications.

- BS-to-BS isolation analysis: BS-to-BS isolation analysis can be further discussed in this WI, with no additional objectives needed.

- Beam steering consideration for co-location scenario: Evaluation of the beam steering impacts is considered as applicable to the co-location scenarios analyses. Applicable frequency range within FR1 is FFS.

- TX IMD co-location: Continue analysis of potential consequences due to TX IMD co-location requirement removal from Rel-19 specification. Applicable frequency boundary for TX IMD requirement removal (within FR1) is FFS.

In this contribution we present more detailed information related to topic (3) in WF from last meeting. The other topics have been or will be handled in separate contributions.

To better understand the impact of beam steering to co-location coupling loss for edge-to-edge scenario, a simulation exercise has been conducted. In this contribution we will present the simulation assumptions adopted for this study and corresponding simulation results together with some general observations.

# Discussion

To better understand the technical background for AAS base station co-location including aspects as larger array antenna structures than what was considered in Rel-15 and beamforming we have conducted a simulation campaign.

To speed up the simulation we have adopted a two-stage approach including full wave simulation on array level with an additional stage of electrical circuit calculation as a second stage.

  - Definition of isolation

By tradition the BS-to-BS isolation have been established as a constant value and applied to several requirements. As pointed out in previous contribution [2], the BS-to-BS isolation to consider for AAS base stations depends on requirement in mind.

- Co-location spurious emissions: For this requirement it is more suitable to consider Array to Array (A-to-A) isolation. This will include effects due to power spread over the whole antenna aperture and beamforming effect on both victim and aggressor side. The A-to-A isolation is defined as the total conducted power fed to the aggressor array antenna to the total conducted received power at the victim array antenna.

- Co-location receiver blocking: For this requirement it is more suitable to consider Array to Sub-Array (A-to-SA) isolation. This is since the receiver front-end will only be exposed by power received by a single sub-array. The composite signal will be created in the digital domain in baseband. The A-to-SA isolation is defined as the total conducted power fed to the aggressor array antenna to the conducted received power per sub-array at the victim array antenna.

For other requirements used the BS-to-BS isolation assumption (e.g., TDD OFF power requirement and Transmitter Intermodulation requirement) different definitions and co-location scenarios may be relevant. Hence, there is not one isolation definition that fits all requirements.

  - Simulation assumptions

In the simulation it is assumed that two AAS base stations are co-located at the same site according to the edge-by-edge scenario defined in Rel-15 for co-location requirements, see TS 38.104, subclause 4.9. In this scenario it is assumed that both the aggressor AAS base station and the victim AAS base station are mounted edge-by-edge separated 0.1 m.

# Figure 2.2-1: Network layout

The victim and aggressor networks are co-sited narrow beams (similar to layer 1 codebook generated beams) are formed from the aggressor AAS to the UEs of the aggressor network and the corresponding power is received on the subarray ports of the co-sited AAS of the victim network. Only one cell is considered in the analysis. Note, that there is no dependence of the UE positions of the victim system.

To better understand the beamforming impact to BS-to-BS isolation we here assume co-location within the same band, hence no frequency separation is assumed between aggressor network and victim network. Details related to deployment and BS assumptions is listed in Table 2.2-1, and Table 2.2-2.

Table 2.2-1: Network deployment assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | Hexagon |
| ISD | 0.5 km |
| Grid shift | 0% |
| Carrier Frequency | 3500 MHz |
| BS height | 25 m |
| UE height | 1.5 m |
| Number of UEs per sector | 1500 |
| Minimum 2D distance BS to UE | 35 m |

Table 2.2-2: BS assumptions

| Parameter | Assumption |
| --- | --- |
| Sub-array configuration | 3x1 |
| Array configuration | 4x8 |
| Pre-set sub-array down-tilt | 3 degrees |
| Mechanical down-tilt | 6 degrees |
| Vertical element separation | 0.7 l |
| Horizontal element separation | 0.5 l |

The placement of the victim base station and aggressor base station and the individual base station array antenna geometry is visualised in Figure 2.2-2.

# Figure 2.2-2: Array and subarray layout. Array 1 (aggressor network) is the left array and array 2 (victim network) is the right array.

Both arrays were designed with a vertical element separation of 0.7 wavelengths and horizontal element separation of 0.5 wavelengths at 3.5 GHz. Elements, see Fig 2.2-3, within each subarray were connected with a 1:3 Radio Distribution Network (RDN), designed for the pre-set subarray tilt at 3.5 GHz. The element-to-element scattering matrix was simulated for three frequencies (3100, 3500 and 3900 MHz) and combined with the subarray models (by cascading scattering matrices) to get the subarray-to-subarray coupling. The detailed layout for the element model and array, is visualised in Figure 2.2-3.

# Figure 2.2-3: CST array (top) and element (bottom) layouts. A simplified antenna element was used to get reasonable simulation time for the element-element S-matrix.

The relation between the aggressor antenna and the victim antenna and coordinate system is visualized in Figure 2.2-4.

# Figure 2.2-4: Spherical coordinates used for the BS antenna coordinate system. The front-right edge (blue dot) of array 1 and front-left edge of antenna 2 (green dot) are positioned symmetrically around the origin and on the yaaxis, and separated 10 cm.

The elements are combined to sub-arrays with a RDN, as visualised in Figure 2.2-5.

Z0, L13

Z0, L12

Z0, L23

Z0, L22

S1

S2

# Figure 2.2-5: A 1:3 RDN is simulated by standard RF circuit techniques using S-matrix cascading.

An RDN, of a 3-element subarray, having 3 output ports and one feed port, is modelled with ideal transmission lines and T-splitters. To achieve equal power on the three elements, one of the T-splitters, S1, has an uneven power split ratio of 2:1, and the other, S2, has an even power split. Impedances on the quarter wave transformers on the output ports are chosen so that the desired power split ratio and impedance matching is achieved. The lengths of the lines L12, L13, L22 and L23 are chosen to achieve the desired fixed subarray tilt.

Downtilt indicator

BSs at 25m height

Figure 2.2-6: Scenario with 1500 random users and 500m ISD. A 6-deg mechanical down-tilt is applied to the entire 2-array block.

  - Coupling loss simulation methodology

In the simulations, the antenna element scattering matrix is calculated using PLM CST full-wave simulation. This data is then used in MathWorks Matlab and the elements are coupled together using S-matrix algebra.

First, the subarray ports are created using the RDN described earlier, and the coupling matrix between all subarray ports is calculated. Next, the deployment parameters, i.e., antenna height, mechanical tilt, pointing direction, etc. are used together with the position of UEs to determine the beamforming weights for the AAS of the aggressor network. These weights are then applied to the subarray ports, in order to obtain the coupling loss between the beam ports on the aggressor antenna and the subarray ports on the victim antenna. The resulting CDF is an aggregate of all UE positions and all subarray ports.

The simulation methodology is visualized in Figure 2.3-1.

Figure 2.3-1: Overview of the simulation methodology

  - Results

Using the simulation assumptions and methodology described in previous sections results for Array to Sub-Array coupling loss have been produced.

A snapshot of the UE distribution and associated coupling loss is visualised in Figure 2.4-1.

Figure 2.4-1: Array to Array-subarray coupling loss level vs beam direction for the aggressor AAS, see Figure 2.2-3, in the antenna spherical coordinate system.

Observation 1: Our results indicate angular regions of beam pointing directions with higher and lower coupling loss reflecting variations in the nearfield antenna pattern.

The Minimum Coupling Loss (MCL) value in this scenario is observed around phi = 20 degs.

The Array to Sub-Array (A-to-SA) coupling loss CDF is plotted for three frequencies in Figure 2.4-2. The CDF includes coupling loss for all sub-arrays in the victim base station.

# Figure 2.4-2: Array to subarray coupling loss levels as an empirical CDF.

Observation 2: The total variation of coupling loss due to beamforming is more than 40 dB and the mean value is around 55 dB.

For AAS BS the beamforming will add a spread to the statistics not considered for non-AAS BS. This is an aspect to further consider to properly establish relevant foundation for requirement derivation. For the requirement level derivation instead of minimum coupling a statistical approach should be considered including aspects related to the larger variation of coupling loss in networks with AAS BSs.

The legacy assumption of 30 dB was derived based on passive antennas and a specific scenario considering aggressor BS and victim BS being located in the same geographical area. Detailed information for the scenario can be found in TR 25.942, subclause 10.2. For AAS BS with beamforming a statical approach should be adopted when coupling loss for requirement derivation is considered.

In current specifications, a fixed value for BS-to-BS isolation have been derived from an in-band assumptions and applied for requirement derivation for cases with large frequency offset between victim and aggressor.

Observation 3: Here we consider a scenario where aggressor and victim are operating within the same band. But in reality, co-location requirements are defined between bands. Therefore, further analysis is required to understand the coupling loss between aggressor and victim with large frequency offset. It is expected that the coupling loss will increase with larger frequency offset.

In this contribution we looked at BS-to-BS coupling loss for a situation where both victim and the aggressor have the same physical size and operating at the same frequency. Identically sized AASs have been studied. In a contribution presented to an earlier meeting [3], simulation results indicate such symmetrical size of antennas would minimize the coupling loss.

Observation 4: In this contribution identically sized AASs have been studied. Previous studies indicate such size matching is the worst-case corresponding to minimum coupling loss.

The co-location scenario analysed in this contribution is based on two array models located side-by-side with separation of 10 cm. This colocation scenario represents a situation rarely seen at real sites for wide area base stations. Often the separation of base stations operating in different networks operating in different bands are larger than 10 cm in real situations. Moreover, the antennas interfere with each other’s reactive nearfield regions. This will complicate standardization of measurement with standard test antennas and simulations.

In reality for a real AAS BS implementation the spacing between enclosures is specified to be 10 cm, this means that the total spacing between the arrays will be larger than 10 cm.

# Conclusion

In this contribution we have studied the impact of beamforming for the co-location scenario defined for AAS BS in Rel-15. In the study, two similar co-located AAS BS used by two different operators was considered. The intention was to evaluate the BS-to-BS array to subarray coupling loss for signals within the operating band.

We can conclude that the Array-to-Subarray coupling loss varies with the selected beam of the aggressor system and also the subarray location.

When analysing the results we observed following:

Observation 1: Our results indicate angular regions of beam pointing directions with higher and lower coupling loss reflecting variations in the nearfield antenna pattern.

Observation 2: The total variation of coupling loss due to beamforming is more than 40 dB and the mean value is around 55 dB.

Observation 3: Here we consider a scenario where aggressor and victim are operating within the same band. But in reality, co-location requirements are defined between bands. Therefore, further analysis is required to understand the coupling loss between aggressor and victim with large frequency offset. It is expected that the coupling loss will increase with larger frequency offset.

Observation 4: In this contribution identically sized AASs have been studied. Previous studies indicate such size matching is the worst-case corresponding to minimum coupling loss.

To understand all aspects of AAS BS near-field coupling effects a more thorough analysis is required considering victim and aggressor array antenna to be of different size, frequency offset between victim and aggressor, vertical offset, and impact of array structures, etc.

The definition of coupling loss needs to be agreed for the array-array and array-subarray cases and should reflect the large variation due to beam steering.

# References

[1]	R4-2416574, “Way Forward for [112bis][303] NR_BS_RF_Part2_CLTA”, Huawei

[2] 	R4-2415971, “On the topic of aspects related to BS-to-BS co-location”, Ericsson

[3]	R4-2009967, “On the criteria for selecting a proper CLTA”, Ericsson