3GPP TSG-RAN4 Meeting #113	R4-2418405

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

Source:	LG Electronics

Title:	Discussion for NTN HPUE MPR

Agenda item:	7.8.2.2.1

Document for:	Discussion

Introduction

In the RAN4#112bis meeting,  the WF on NTN HPUE was approved. In this discussion, we share our point of view for the ΔTRxSRS requirement based on the approved WF[1]

Discussion

Table 1 WF on LP-WUS REFSENS requirements in the RAN4#112bis [1]

Issue 2-6: MPR - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm) - Option 2: Reusing NR-NTN PC3 MPR values (both single Tx and Tx diversity) for PC2 MPR as starting point (Mediatek, Samsung) - Option 3: MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. (ZTE) Way forward: - Further discuss with ACLR candidate values, following agreement and above options. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.. Issue 2-7: A-MPR - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm) - Option 2: To reduce number of power back-off simulations for the FR1 satellite bands, we suggest not considering scenarios/combinations that are not expected for the satellite bands: large RB allocations, CP-OFDM, higher-order modulations. (Apple) - Option 3: Regarding the A-MPR requirements, once HPUE e.g. PC2/PC1.5or PC1 are supported in any specific bands, then the A-MPR requirement should be revisited again to identify the appropriate the A-MPR requirements. (ZTE) Way forward: - Further discuss with following agreement and above options. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors.

The discussion on NTN HPUE is progressing as a Rel-19 Work Item, and it’s likely that PC2, PC1.5, and PC1 will be specified. Therefore, MPR requirements for NTN HPUE need to be defined. FR1 NTN UE is assumed to have an antenna gain of 0 dBi and that the distance between the UE and NTN is very large, there is a significant lack of link margin, particularly on the uplink. Introducing HPUE can help mitigate this shortage in the uplink link margin. However, as power class increases, the MPR requirement tends to increase as well, which could diminish the effectiveness of NTN HPUE. Therefore, to effectively address the uplink link margin deficiency in NTN UEs, it would be beneficial to consider a lower MPR value for NTN HPUE compared to that of legacy TN HPUE.

Observation: To effectively address the uplink link margin deficiency in NTN UEs, it would be beneficial to consider a lower MPR value for NTN HPUE compared to that of legacy TN HPUE.

The MPR values have different limiting factors depending on the modulation order and the location of the RB allocation, with possible limiting factors including ACLR, IBE, EVM, and SEM.. Therefore, to consider a lower MPR value than that of existing HPUE, it is necessary to identify the limiting factors that were used in defining the MPR for  NR HPUE, and Study will be needed to determine how to adjust these limiting factors when defining the MPR for NTN HPUE.

Proposal: It is necessary to identify the limiting factors that were used in defining the MPR for  NR HPUE, and study will be needed to determine how to adjust these limiting factors when defining the MPR for NTN HPUE.

Conclusion

Observation: To effectively address the uplink link margin deficiency in NTN UEs, it would be beneficial to consider a lower MPR value for NTN HPUE compared to that of legacy TN HPUE.

Proposal: It is necessary to identify the limiting factors that were used in defining the MPR for  NR HPUE, and study will be needed to determine how to adjust these limiting factors when defining the MPR for NTN HPUE.

- References

# [1] R4-2417122, WF on co-existence study and HPUE for NR-NTN (samsung)