3GPP TSG-RAN4 Meeting #113	R4-2418417

# Orlando, US, November 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5072 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Perf) CR on correction of test parameters and requirements for TCs for inter-frequency cell reselection on FR1 NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronices |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wrong test parameters and requirements were defined, and higher priority cell and enhancedMeasurementNGSO-r17 were not captured for inter-frequency cell reselection test cases in clause A.14.1.5 and A.14.1.6. And Notes were not captured in test parameter table. Wrong clause number for core requirements number were captured in A.14.1.9 and A.14.1.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The higher priority cell for Cell 2 has been described for inter-frequency cell reselection TCs. “The flag enhancedMeasurementNGSO-r17 should be set.” has been added in clause A.14.1.6. Following parts have been revised and added to align inter-frequency cell reselection TC in clause A.14.1.5 and A.14.1.6. - The values and comments for T1, T2, and T3 in general test parameters - The Es/Iot, Es/Noc, SS-RSRP in cell specific test parameters - Threshx, highP,Threshserving, lowP, and Threshx, lowP - Test requirements when enhancedMeasurementNGSO-r17 is set - Notes in Table A.14.1.6.2-3 Corrected the clause numbers of the core requirements corresponding to the test cases in A.14.1.9 and A.14.1.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will leave wrong test parameters and requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.1.5, A.14.1.6, A.14.1.9, A.14.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of Change <1> --------------

A.14.1.5	Cell reselection to FR1 inter-frequency NR case

A.14.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements for satellite access specified in clause 4.2C.2.4.

A.14.1.5.2	Test Parameters

The test scenario comprises of 2 NR carriers and 2 cells as given in tables A.14.1.5.2-1, A.14.1.5.2-2 and A.14.1.5.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

Table A.14.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.5.2-2: General test parameters for inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
| T2 end condition | Active cell |  | 1 | Cell2 |  |
|  | Neighbour cells |  | 1 | Cell1 |  |
| Final condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
| SMTC configuration#1 |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| SMTC configuration#2 |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| DRX cycle length |  | s | 1 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1 | Not configured |  |
| T1 |  | s | 1 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1 | >7 | During T2, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.5.2-3: Cell specific test parameters for inter frequency NR cell re-selection test case

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | 1 | SSC.1 for GSO test SSC.2 for NGSO test |  |  | SSC.1 for GSO test SSC.2 for NGSO test |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
| Pcompensation | dB | 1 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -84 | -102 | -infinity | 86 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
| Treselection | s | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonIntraSearchP | dB | 1 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.14.1.5.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2C.2.9

Tevaluate, NR_ inter	See Table 4.2C.2.4-1 in clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements

A.14.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements for satellite access specified in clause 4.2C.2.4.

A.14.1.6.2	Test Parameters

The test scenario comprises of 2 NR carriers and 2 cells as given in tables A.14.1.6.2-1, A.14.1.6.2-2 and A.14.1.6.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2. The flag enhancedMeasurementNGSO-r17 should be set.

Table A.14.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.6.2-2: General test parameters for inter frequency NR cell re-selection test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| Final condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| RF Channel Number |  |  | 1, 2 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | SSB.1 FR1 |  |
| SMTC configuration#1 |  |  | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| SMTC configuration#2 |  |  | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | >7 | During T2, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.6.2-3: Cell specific test parameters for inter frequency NR cell re-selection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for GSO test SSC.2 for NGSO test |  |  | SSC.1 for GSO test SSC.2 for NGSO test |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 14 | 14 | 14 | -4 | -infinity | 12 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 14 | 14 | 14 | -4 | -infinity | 12 |
| SS-RSRP Note3 | dBm/SCS | -84 | -84 | -84 | -102 | -infinity | -86 |
| Io | dBm/9.36 MHz | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonIntrasearchP | dB | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.14.1.6.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 66 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to a lower priority cell shall be less than 6 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_inter_enh + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_inter_enh + TSI-NR,

Where:

Tevaluate, NR_inter_enh	See Table 4.2C.2.4-2 in clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

-------------- End of Change <1> --------------

-------------- Start of Change <2> --------------

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

A.14.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2C.2.8, for UE fulfilling low mobility relaxed measurement criterion.

A.14.1.9.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.14.1.9.2-1, A.14.1.9.2-2 and A.14.1.9.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in TS 38.304 [1]. So, Cell 2 and Cell 1 configure the UE as follows:

lowMobilityEvalutation TS 38.331 [2] criterion is configured according to the parameters listed in Table A.14.1.9.2-3;

cellEdgeEvaluation TS 38.331 [2] criterion is not configured;

combineRelaxedMeasCondition TS 38.331 [2] is not configured;

Table A.14.1.9.2-1: Supported test configurations

| Configuration | Description of serving cell |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.9.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell2 | The UE camps on Cell 2 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to Cell 1 |
|  | Neighbour cells |  | 1 | Cell1 |  |
| T1 end condition | Active cell |  | 1 | Cell1 | The UE shall perform reselection to Cell 1 during T1 |
|  | Neighbour cells |  | 1 | Cell2 |  |
| T2 end condition | Active cell |  | 1 | Cell2 | The UE shall perform reselection to Cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1 | Cell1 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
| DRX cycle length |  | s | 1 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1 | Not configured |  |
| T1 |  | s | 1 | 25 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1 | 25 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.9.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| Satellite information |  | 1 | SSC.1 for GSO test SSC.2 for NGSO test |  | NSC.1 for GSO test NSC.2 for NGSO test |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |  |
| Pcompensation | dB | 1 | 0 |  | 0 |  |  |
| Qhysts | dB | 1 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
| Treselection | s | 1 | 0 | 0 | 0 |  | 0 |
| SnonintersearchP | dB | 1 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1 | 50 |  | 50 |  |  |
| SSearchDeltaP | dB | 1 | 3 |  | 3 |  |  |
| TSearchDeltaP | s | 1 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.14.1.9.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to a lower priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a known lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64s, allow 17s for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already detected higher priority cell, which we allow 17s for UE fulfilling low mobility relaxed measurements in the test case.

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

A.14.1.10.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2C.2.8, for UE fulfilling not-at-cell edge relaxed measurement criterion.

A.14.1.10.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.14.1.10.2-1, A.14.1.10.2-2 and A.14.1.10.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE not-at-cell edge as defined in clause 5.2.4.9.2 in TS 38.304 [1]. So, Cell 2 and Cell 1configures the UE as follows:

cellEdgeEvaluation TS 38.331 [2] criterion is configured according to the parameters listed in Table A.14.1.9.2-3;

lowMobilityEvalutation TS 38.331 [2] criterion is not configured;

combineRelaxedMeasCondition TS 38.331 [2] is not configured;

Table A.14.1.10.2-1: Supported test configurations

| Configuration | Description of serving cell |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.14.1.10.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell2 | The UE camps on Cell 2 in the initial phase, it fulfills Not-at-cell edge relaxation measurements criterion, and during T1 period the UE reselects to Cell 1 |
|  | Neighbour cells |  | 1 | Cell1 |  |
| T1 end condition | Active cell |  | 1 | Cell1 | The UE shall perform reselection to Cell 1 during T1 |
|  | Neighbour cells |  | 1 | Cell2 |  |
| T2 end condition | Active cell |  | 1 | Cell2 | The UE shall perform reselection to Cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1 | Cell1 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
| DRX cycle length |  | s | 1 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1 | Not configured |  |
| T1 |  | s | 1 | 20 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1 | 20 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.14.1.10.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| Satellite information |  | 1 | SSC.1 for GSO test SSC.2 for NGSO test |  | NSC.1 for GSO test NSC.2 for NGSO test |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |  |
| Pcompensation | dB | 1 | 0 |  | 0 |  |  |
| Qhysts | dB | 1 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
| Treselection | s | 1 | 0 | 0 | 0 |  | 0 |
| SnonintersearchP | dB | 1 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1 | 50 |  | 50 |  |  |
| SSearchThresholdP | dB | 1 | 50 |  | 50 |  |  |
| SSearchThresholdQ | s | 1 | Not Configured |  |  |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.14.1.10.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling not-at-cell edge relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected lower priority cell for UE fulfilling not-at-cell edge relaxed measurements shall be less than 17s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling not-at-cell-edge relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling not-at-cell-edge relaxed measurements shall be less than 17s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.3-1 in clause 4.2.2.10

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64s, allow 17s for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already higher priority cell, which we allow 17s for UE fulfilling not-at-cell edge relaxed measurements in the test case.

-------------- End of Change <2> --------------